3GPP TSG RAN WG1 #112	                                                 	R1-2300587

Athens, Greece, February27th – March3th, 2023

Agenda item:	9.7.1

Source:		Xiaomi

Title:	Discussion on techniques in spatial and power domains

Document for:	Discussion

Introduction

In RAN1#98 meeting, the WID for network energy savings was concluded in [1], with the following objectives:

The objectives of the work item are the following: - Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2] - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements - Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2] - Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2] - Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3]. - Specify the corresponding RRM/RF core requirements, if necessary, for the above features [RAN4]

In this contribution, we discuss the potential enhancements on spatial and power domain for network energy saving.

Discussion

Spatial domain

In spatial domain, the number of gNB’s spatial elements (e.g. antenna ports, active transceiver chains) can be reduced for energy power saving. For instance, partial spatial elements can be turned off by the gNB when the traffic is not heavy, and vice versa. When the spatial elements are turned off, the antenna ports of CSI-RS mapping to the spatial elements are muted accordingly.

To realize the variation in the number of CSI-RS antenna ports, different CSI-RS resources with different number of antenna ports can be configured according to current specification. To be specific, the UE adopts one CSI-RS resource to perform the measurement based on the configuration. When gNB chooses to turn off parts of the spatial elements, another CSI-RS resource has to be chosen. The UE performs measurement and reports the measurement results based on the configuration associated with the updated CSI-RS resource. However, considering that the traffic may be varied from time to time, the dynamic adaption of spatial elements is more suitable. If adopting different CSI-RS resources to adapt different number of spatial elements, frequently activation and deactivation for the CSI-RS resources occurs, which will increase the signalling overhead. Last but not least, different CSI-RS resources may be configured with different CSI-reportConfig. More CSI-reportConfig may be needed in this case. However, the supported number of CSI-reportConfig is limited due to UE capability, e.g., maximum 4 P/SP/A- CSI-reportConfig currently. The scheduling flexibility may be reduced in this case.

To reduce the signalling and processing overhead for CSI measurement and reporting, and to support the dynamic change of spatial elements, the number of antenna ports for one CSI-RS resource can be changed dynamically, as shown in Figure 1. By supporting one CSI-RS resource to associated different number of spatial elements, the problem of frequent activation and deactivation mentioned above is avoided, which is beneficial for the reduction of CSI measurement and reporting overhead.

Figure 1 The dynamic spatial adaptation scheme

As number of antenna ports for one CSI-RS resource can be changed dynamically, one issue is related to the resources for CSI-RS reception. In current specification, the UE determines the resources for CSI-RS reception based on the mapping rules between CSI-RS sequence and resources elements  k,lp,μ, wherein p is the antenna port, k and l are the index of subcarrier and OFDM symbol, respectively.  Once the number of antenna ports and other parameters are configured by RRC parameters, the mapping rules between CSI-RS sequence and resources elements  k,lp,μ are determined [2].  However, when the number of antennas is dynamically changed, the muting antenna ports without transmitting CSI-RS sequence are transparent to UE. The UE cannot determine the resources for CSI-RS reception with current mechanism.

Observation 1: The resources for CSI-RS reception are unknown for the UE when the number of antenna ports are dynamically changed.

Based on the aforementioned observations, there could be different solutions as follows:

- Option 1: Reuse original mapping configurations for one CSI-RS resource and indicate the antenna ports muting patterns. The potential specification impact for this option is to introduce the indication of muting patterns.  An example is shown is Figure 2, the UE determines the resource mapping by existing mechanism for one CSI-RS resource configured with 8 ports in Figure 2(a). Once a muting pattern is dynamically indicated, for instance, 4 antenna ports with p=3004, … ,3007 are muted, the UE determines the resource mapping for CSI-RS reception in Figure 2(b).

Figure 2(a) 8-port CSI-RS

Figure 2(b) 8-port CSI-RS with 4 ports muted

Figure 2 CSI-RS resource with density=one, nrofPorts=p8, cdm-Type=fd-CMD2,

frequencyDomainAllocation=011110, firstOFDMSymbolinTimeDomain = 3

- Option 2: Configure multiple mapping configurations and indicate one configuration dynamically. To be specific, one CSI-RS resource can be configured with multiple combinations of parameters, for instance, multiple combinations of nrofPorts, density, cdm-Type, etc. The UE determines the resource mapping for CSI-RS reception when one of the configurations is determined.

Proposal 1: Signaling to enable efficient adaptation of resource mapping for the reception of one CSI-RS resource should be further studied.

When the number of antenna ports with one CSI-RS resource changes dynamically, the quantities of the CSI estimation will change accordingly. To reflect the quantity difference, CSI reporting configuration containing multiple antenna ports patterns should be introduced. In this case, the gNB can choose proper MCS for the subsequent scheduling of PDSCHs transmitted by different antenna ports patterns.

Proposal 2: Enhancement to enable CSI reporting with multiple spatial elements patterns should be introduced.

Power domain

In current specification, the power offset values between PDSCH and CSI-RS is semi-statically configured by powerControlOffset for each NZP CSI-RS resource. The UE performs the channel estimation and calculate the CSI based on the power offset. Accordingly, the gNB can choose the preferred precoding for the PDSCH based on the reported CSI. When the dynamic power offset is introduced according to the WID, the UE should adopt the updated power offset for CSI calculation to improve the accuracy of channel estimation. In this case, signalling to enable efficient adaptation of power offset should be further studied.

Proposal 3: Signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS needs further study.

Similar to the spatial element adaptation, the power offset will also influence the reported CSI quantities. Therefore, the CSI reporting configuration containing multiple CSI quantities associated with one CSI-RS resource should be introduced.

Proposal 4: Enhancement to enable CSI reporting with multiple power offsets should be introduced.

Conclusions

In this contribution, we provide the potential scenario and candidate solutions for the network energy saving, the following proposals are made based on our analysis:

Observation 1: The resources for CSI-RS reception are unknown for the UE when the number of antenna ports are dynamically changed.

Proposal 1: Signaling to enable efficient adaptation of resource mapping for the reception of one CSI-RS resource should be further studied.

Proposal 2: Enhancement to enable CSI reporting with multiple spatial elements patterns should be introduced.

Proposal 3: Signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS needs further study.

Proposal 4: Enhancement to enable CSI reporting with multiple power offsets should be introduced.

References

- RP-223540, New WID: Network energy savings for NR, 3GPP TSG RAN Meeting #98-e,	Electronic Meeting, December 12-16, 2022.

- 3GPP TS 38.211, Physical channels and modulation.