3GPP TSG RAN WG1 #112 Meeting	R1-2300609

Athens, Greece, 27 Feb. – 03 March, 2023

Agenda item:		9.2.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Other aspects on ML for positioning accuracy enhancement

Document for:		Discussion and Decision

# Introduction

Rel-18 study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, agreed in [1], includes a use case of positioning accuracy enhancement, and the exact objectives are as follows:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced

In this contribution, we mainly focus on proposals not agreed upon, topics for further study indicated as part of the agreements made part of the RAN1-111 meeting and previous meetings, and new specification impacts analysis. We also discuss representative sub-use cases agreed upon in the 3GPP RAN1#110-bis-e meeting in the specific concepts and aspects related to AI/ML for positioning, which are listed in the following:

Agreement - Study and provide inputs on benefit(s) and potential specification impact, at least for the following cases of AI/ML based positioning accuracy enhancement - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML-assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML-assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML-assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

we discuss the following aspects related to AI/ML for positioning:

- Section 2 recises specific aspects related to data collection. Here we explore some specification impacts on obtaining ground truth, noisy ground truth, and data diversity.

- Section 3 revises specific aspects related to model monitoring and updating. Here we discuss essential specification impact related to model updating and monitoring.

- Section 4 revises the specification impact related to model input and output. Here we discuss aspects of model input-output in terms of positioning accuracy, feature selection, and model input for one-sided and two-sided models.

- Section 5 provides a summary of observations and proposals.

# Specific Aspects related to Data Collection

In this section, we consider the challenges related to data collection and labeling to generate training and test/validation datasets for AI/ML-based positioning. In the following subsections, we focus on the next 3GPP RAN1-111 agreements:

Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, - The following options of entity and mechanisms to generate ground truth label are identified for further study - For direct AI/ML positioning, ground truth label is UE location - PRU with known location - UE generates location based on non-NR and/or NR RAT-dependent positioning methods - LMF generates UE location based on positioning methods - LMF with known PRU location - Note: user data privacy needs to be preserved - For AI/ML assisted positioning, ground truth label is one or more of the intermediate parameter(s) corresponding to AI/ML model output - PRU generates label directly or calculates based on measurement/location - UE generates label directly or calculates based on measurement/location - Network entity generates label directly or calculates based on measurement/location - The following options of entity to generate other training data at least measurement corresponding to model input are identified for further study - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP - Note: other options of entity to generate other training data are not precluded - Note: Existing PRU definition is in 38.305

Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, - The following options of entity and mechanisms to generate ground truth label are identified for further study - For direct AI/ML positioning, ground truth label is UE location - PRU with known location - UE generates location based on non-NR and/or NR RAT-dependent positioning methods - LMF generates UE location based on positioning methods - LMF with known PRU location - Note: user data privacy needs to be preserved - For AI/ML assisted positioning, ground truth label is one or more of the intermediate parameter(s) corresponding to AI/ML model output - PRU generates label directly or calculates based on measurement/location - UE generates label directly or calculates based on measurement/location - Network entity generates label directly or calculates based on measurement/location - The following options of entity to generate other training data at least measurement corresponding to model input are identified for further study - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP - Note: other options of entity to generate other training data are not precluded - Note: Existing PRU definition is in 38.305

Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, study benefits, feasibility and potential specification impact (including necessity) for the following aspects - Request/report of training data - Ground truth label - Measurement corresponding to model input - Associated information of ground truth label and/or measurement corresponding to model input - Assistance signaling and procedure to facilitate generating training data - Reference signal (e.g., PRS/SRS) configuration(s) and configuration identifier - Assistance information, e.g., between LMF and UE/PRU, for label calculation/generation, and label validity/quality condition, etc. - Note1: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed - Note2: Study may consider different entity to generate training data as well as different types of training data when applicable - Note3: study considers both of the following cases when applicable - when the training entity is the same entity to generate training data - when the training entity is not the same entity to generate training data

## On obtaining ground truth label and other training data

As acknowledged in the agreements above, collecting and labeling training data is a lengthy and computationally expensive process due to the following:

- The scarce availability of NR elements (UE or NW entities) that could measure and extract accurate training data (inputs and labels) from NR measurements.

- The scarce radio resources for reporting large amounts of training data, especially between UE and gNB.

Observation-1: A method to overcome the training data scarcity challenge would be to reuse measurements collected during the regular UE operation (e.g., during ongoing positioning sessions) as training data samples for a positioning task. Such measurements are not labeled by default; therefore, new labeling methods must be devised.

The labeling process may be performed at any RAN entity called NR-verifier, which uses the AIML inference's output for solving a different task. If the task outcome is successful, then the NR-verifier concludes that the AIML output is correct. Therefore the inference sample may be labeled with the respective output and may thus become a training sample instead. An NR-verifier could be the LMF, the serving gNB of the target UE, the neighbor gNBs, etc.

Consider the following simplified example: a UE performs AI/ML-based LOS estimation. The UE collects PRS samples from TRP1 and estimates that TRP1 is in LOS. The inference outcome is thus LOS = 1. It transfers this information to the NR-verifier, which, using this information, selects the best TRP1 serving beam B_LOS. If B_LOS coincides with the beam already selected via the standard beam management, then the verifier concludes that the inference outcome LOS = 1 is correct. Following, it labels the respective PRS samples with LOS = 1 and marks them as training data.

We can consider another scenario for direct positioning where an ML model estimates the user position. The positioning information is transferred to the NR-verifier, which uses the user position as assisted information for beam management. If the beam management procedure fails, NR-verifier concludes that the estimated position is not accurate enough, which results in not adding the sample to the training dataset. On the opposite side, a successful beam alignment leads to the conclusion that the estimated position is accurate enough compared to the target accuracy. Thus, the NR-verifier evaluates the estimated position and marks the sample as training data.

Proposal-1: RAN1 to study the possibility of labeling measurements collected during regular UE operation, where such measurements, upon labeling, may be used for training a positioning machine learning task.

## Impact related to noisy ground truth labels

For machine learning, especially in the context of positioning, it is challenging to derive ground truth or the correct/ complete labeling information of the training data without manual intervention. Especially for intermediate features such as the LOS/NLOS indicator. Hence, in most cases, training data is labeled with noisy labels (i.e., approximate or incomplete labels).

Consider that a training sample is made from measurements of NR signals and their corresponding label, consisting of the position of the target device at the time of measurement. For each of these samples, it may be possible to obtain noisy labels (inaccurate position) from several sources like GNSS, RAT, etc. When this is the case, samples with one or several sources of noisy labels should be pre-evaluated before transmission and collection as new training data.

Observation-2: The noisy label evaluation can be assisted by the LMF and can optimize the data collection and accept/reject a sample based on the labeling accuracy. For samples with only one labeling source, the evaluation may depend only on the variance of the position estimation compared to the target accuracy.

Furthermore, the consistency of the obtained labels changes the decision to accept or reject the sample where there is the opportunity to label a sample with multiple positioning sources. To enable the sample evaluation process, a label consistency score (LCS) may be defined by the LMF. The selected LCS metric may have the following properties:

- Providing higher LCS value when positioning estimation(s) has/have higher accuracy.

- In the case of consistency between estimated positions from various sources, LCS values increase proportionally to the number of positioning sources.

Additionally, a threshold for LCS may be signaled by the LMF to enable the sample rejection process. Such threshold may be defined based on the network requirement, e.g., target accuracy, or ML-related parameters, e.g., current collected training dataset size.

One important aspect of successful training is processing noisy ground truth label(s). For samples with only one labeling source, we may use the variance of position estimation weight in the effects of each sample on the training results.

Observation-3: In case of the opportunity to label a sample with multiple positioning sources, one may exploit all the information from different positioning sources and incorporate them in the labels for training the ML model.

Proposal-2: RAN1 to study solutions on how to clean the noisy labels for Case1 and Case 2a as part of data collection.

The noisy ground truth label challenge is valid even considering the use of PRUs for obtaining such information due to the dynamic nature of the environment in which these nodes would be deployed. Challenges related to dynamic LOS blockages, lack of correlation between spatially collocated UEs with respect to LOS/NLOS, etc., provide additional complexity to this issue.

Figure 1 shows an example with three PRUs 1-3 deployed in a network. PRU-1 has a LOS link to the gNB/TRP, PRU-2 has a static blocker which implies that the particular gNB/TRP-PRU link will always be NLOS, and PRU-3 has a mobile block. During the measurement-1 time instance, the link with gNB/TRP is LOS; during measurement-2, the gNB/TRP is NLOS (or potentially even obstructed LOS/NLOS), with the link becoming LOS again during the measurement-3 time instance.

Figure 1 - Practical Deployment Scenario with Challenging Ground Truth Estimation for LOS/NLOS.

From the RAN1 perspective, it is vital to investigate means to obtain/access/derive the correct labels of positioning training data without relying on the manual scheme. Such mechanisms are essential to ensure that the AI/ML-based enhancements investigated as part of the study would closely reflect real-world deployments. It is also important to note that the accuracy of the labeled data would be the main factor determining model performance.

One solution for the noisy ground truth label could be to utilize the known location attribute of PRUs to modify, complete, or correct the noisy label. The measurements, including the noisy label, could be collected from the PRUs and provided to a direct positioning AI/ML model.

Using an iterative approach whereby the loss function between the predicted PRU location and actual/known PRU location is minimized by varying the values within the noisy label could be used to modify or correct the noisy label. The dataset with the corrected labels could be used for further processing, including training of AI/ML models AI/ML assisted positioning methods, where the corrected labels could be used as the output parameter. An overview of overcoming the noisy label is illustrated in Figure 2.

Figure 2 - Overview of the solution for overcoming the noisy label issue.

Another problem related to noisy labels and labeling by PRU is the source(s) of noisy labels. Specifically, a PRU may be able to extract labels for a data point from multiple sources where such sources are all noisy. For example, a positioning measurement may be labeled with a location estimate obtained from either a RAT or a non-RAT (e.g., GNSS) source. Both sources introduce errors to the location estimate. When this is the case, the PRU must decide which source to use for labeling, how to combine sources, and how to eventually correct or complete the combined result.

Observation-4: Answering the noisy label problem is expected to be use-case dependent and may require some assistance from the network. For example, the network may provide a set of rules for label quality evaluation, where the network may indicate one or more rules to reject/accept a sample with one or several noisy labels.

Potential Specification Impact:

The potential specification impact due to the iterative ground truth label correction/modification mechanism would depend on the node where the model training is conducted. For network-side training, label correction/modification could be done without any specification impacts if the network has sufficient information related to the location of the PRUs. For UE-based training, network assistance would be required for label correction/modification since the UEs might not know the PRU locations to implement the iterative mechanism. In this scenario, the UE could request network assistance for the ground truth label determination.

Observation-5: The potential specification impact from noisy ground truth labels during the data collection and model training phase could depend on whether UE-side or network-side training is considered.

Observation-6: For network-side training, label correction/modification could be done without any specification impacts as long as the network has sufficient information related to the location of the PRUs.

Proposal-3: RAN1 to study adaptable training solutions to assist in cleaning noisy labels. Such assistance may be in the form of defining new metrics associated with the label cleanliness level and/or cleanliness thresholds based on which the related training data sample is retained or discarded.

## Impact related to the availability of data diversity for AI/ML model

The performance of the AI/ML model for the positioning use case is highly dependent on how the training is realized beforehand, including the selection of the AI/ML model and the collected labeled data that is used for training. As mentioned in Sec. 2.2 of [2], assuming data availability from uniform or grid-based UE distribution in practical deployments is not feasible.

Considering the positioning use case where the given AI/ML model is deployed either at the UE or Network side, several challenges should be handled to perform efficient AI/ML model training and generalization, for instance:

- The positioning-related dataset is collected at limited spatial and temporal conditions. However, the target positioning AI/ML model may exist in various conditions such as RF environment, location, and time.

- The procedure of data collection performed by the UE or the network could be time-consuming until reaching the targeted training dataset (in terms of size and quality).

- The network can provide additional data for the UE if the network knows the AI/ML model details. However, the quality of these offered additional data in improving model performance is uncertain (e.g., source, context).

To improve the training procedure, we propose to trigger the training only once the training dataset is analyzed and judged to be good enough in accordance with the targeted positioning accuracy (Figure 3). The available training dataset is examined as a spatial process and characterized with the proposed inter-point distance (IPD) metric. The training operation can be realized if the collected training data responds to the pre-established IPD condition. Otherwise, it is necessary to request the collection of additional labeled data, which could be either:

- Synthetic data: generated through a data augmentation procedure which has the advantage of avoiding requesting new data but may include interpolation errors (acceptable to some extent as a function of the targeted positioning accuracy). More details can be found in section 2.1.3.1.

- Real data: given the available labeled data, identify the locations of the additional data to reach the required IPD condition and request them following the conventional data collection scheme. This approach could optimize the data collection by requesting only the ones necessary to achieve the targeted positioning accuracy. More details can be found in section 2.1.3.2.

### Data Augmentation:

In scenarios where the additional dataset cannot be collected from the UEs/PRUs, other synthetically generated data or data augmentation (DA) can be one possible way-forward. DA is a well-known machine learning technique that can increase the generalizability and robustness of the trained data model. However, the quality of the augmented data may be questionable.

Specifically, for AI/ML positioning, DA can help estimate features related to localization, such as LOS/NLOS classification or direct position estimation. For instance, the target device may perform spatial interpolation and report the corresponding quality of DA to the NW. This report aims to address challenges such as the cost of positioning data collection, class imbalance, and data scarcity.

Formally, if a set of data {A} is collected for model training, a second set {B} can be generated synthetically with the DA technique and used to create variations on the same site but for different UE locations (Figure 3). Data set {B} can either be used:

- To enrich the training dataset and realize the training on {A}+{B},

- Perform training on data set {A} and then perform AI/ML model test on data set {B}

Figure 3 - Data augmentation example.

### Inter-Point Distance based Positioning Accuracy Estimation

The training dataset quality is crucial to reach a well-trained ML model, which allows to obtain the targeted accuracy in all locations. However, for the positioning case, the distribution of the collected training set locations in the spatial domain is quite essential: this mainly relates to the inner characteristics of the training data set considering point pattern analysis (e.g., how the points are spatially distributed such as clustered or totally random, the distance between the different points).

So far, no available mechanism allows us to assess the quality of the training set automatically from a spatial process perspective, accounting for its inner attributes – such as environmental attributes including, e.g., clutter density, size, and height.

Intuitively, we can state that there is a close relationship between the following characteristics:

- The spatial distribution attributes of the training data set, such as the distance between the geographical locations associated with the training data points and,

- The achievable localization accuracy with the trained ML model

We must translate this relationship to provide the required training set quality (over the whole area of interest) regarding a targeted location QoS/accuracy. Addressing this problem would have different impacts, depending on whether UE-based or LMF-based/UE-assisted localization mechanisms are utilized, mainly depending on the model training and inference node. We consider the Euclidean distance between the locations associated with the training data points as IPD.

Observation-7: There are no mechanisms to evaluate the dataset quality in the spatial domain and the correlation with the respective scenario attribute, such as clutter density, clutter height, etc.

An overview of the problem scenario is shown in Figure 4. Here we consider a scenario where the environmental attributes in terms of clutter, synchronization/timing errors, etc., could vary. With the network collecting measurements from the UE, building labeled datasets from the measurements and possibly (a) utilizing the dataset for training AI/ML models for positioning – for network/LMF-based model training, inference, and positioning; or (b) sharing the dataset with the UEs for training AI/ML models for positioning – for UE-based model training, inference, and positioning.

Figure 4 - Overview of the problem scenario.

In this particular scenario, it is considered that if clutter density is an environment attribute that is taken into consideration, the amount of dataset with a given Euclidean distance between the locations associated with the training data points would be proportional to the amount of clutter in the scenario. Thus, the Euclidean distance between the samples would be lower for dense or heavy clutter scenarios compared to a medium clutter scenario; a slight change in such challenging environment might cause significant performance differences in positioning accuracy.

Proposal-4: RAN1 to consider inter-point distance (IPD) as a KPI while collecting training data samples and during model training. If the dataset used for model training does not conform to the required IPD metric, data augmentation could be used to generate additional data samples synthetically.

Potential specification impact:

The mechanism for data augmentation is shown in Figure 5. It requires network assistance to ensure the efficient generation of the additional dataset. For example, to perform the data augmentation following procedure may follow:

- UE sends data augmentation requests to the network with an indication mentioning the required percentage of data for given ML model training.

- The network can then assess the request of the UE by directly providing the data augmentation parameters/configurations.

- UE can then perform data augmentation based either on network recommendations.

- Additionally, UE may report feedback messages to the network indicating data augmentation performance as achieved interpolation accuracy.

Figure 5 - Data Augmentation call flow.

Observation-8: The AI/ML model deployed at UE for positioning could be a vendor-specific or proprietary algorithm (e.g., black box).

Observation-9: The challenges related to AI/ML model training related to dataset collection, quality, and required network assistance could be addressed with the help of additional synthetic data or data augmentation.

Observation-10: AI/ML model generalization can be realized on the variations of the dataset on the same site/area using DA techniques for unavailable UE locations.

Proposal-5: RAN1 to consider network-assisted data augmentation solutions as part of data collections for direct and assisted AI/ML positioning.

### Training data availability in the presence of abnormal propagation conditions

Lifecycle management of a positioning ML model requires periodic model updates, where the periodicity depends on the following:

- The inference frequency, i.e., how often the model is used after deployment.

- The type of UE that uses the model. That is because different UEs have different capabilities to collect data and/or update the model accordingly.

- What positioning task is the model solving, e.g., direct positioning AI/ML or AI/ML assisted positioning (LOS detection, location estimation, etc.).

- The availability of training data, i.e., how much, how diverse, and how often training data can be collected.

- The reliability of the training data, i.e., is training data characterizing well the conditions for which the ML model is being trained.

While features 1-3 are intrinsic to the ML function and/or UE type and thus relatively controllable, features 4-5 are extrinsic and typically depend on the propagation and interference conditions in which the UE finds itself.

Thus, if only features 1-3 are accounted for when managing the ML model update, while features 4 and 5 are disregarded, the ML model may be updated to learn a set of peculiar/rare channel conditions that do not represent well the average daily conditions. In other words, the training is done based on a situation that does not represent typical wireless conditions, but an anomaly, leading to an overall unrealistic model that does not generalize well. Thus, the ML model forgets how to deal with average conditions and exhibits poor performance in most cases. This phenomenon is called catastrophic forgetting, and it is one of the caveats of continual learning/updating of ML functions.

Exemplary rare conditions that may lead to a catastrophic forgetting of a positioning ML function are linked to the cases in which a positioning link towards a set of TRPs is temporarily blocked or severely attenuated due to an anomaly in the wireless channel. Such anomalies often occur due to very high speeds and/or the presence of structures that create a Faraday cage/shield around either the positioning receiver or transmitter, like buildings covered in metallic scaffolding, large structures associated with construction sites, outdoor concerts, road work, etc.

Potential specification impact

To manage rare events occurrences during data collection, the following aspects may be necessary:

  - Defining conditions to identify different types of positioning rare events relevant for an ML-positioning function, where such function is either LOS-detection or location estimation.

  - Defining actions to manage each type of rare event by the UE, e.g., whether to discard the measurement, whether to use the measurement to update the model or roll-back a previous model, etc.

Observation-11: Any given positioning measurements that does not represent well the reflectors distribution for that set of TRPs relative to the UE location may be seen as abnormal/rare events. Nevertheless, some rare events may prove useful to the ML positioning function, while others may be entirely detrimental.

Proposal-6: RAN1 to define means to identify/manage abnormal propagation conditions during the data collection.

# Specific Aspects related to Model Update and Monitoring

Solutions addressing this agreement related to model monitoring and update are discussed in this section based on the agreement of 3GPP RAN1-110b-e meeting:

Agreement Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on potential specification impact for the following aspects - Assistance signaling and procedure at least for UE-side model - Report/feedback and procedure at least for Network-side model - Note1: study is applicable to both of the following cases - Model inference and model monitoring at the same entity - Entity to perform the model monitoring is not the same entity for model inference - Note2: other aspects are not precluded

We also present some discussions related to following agreement about model indication:

Agreement Regarding AI/ML model indication[/configuration], to study and provide inputs on potential specification impact at least for the following aspects on conditions/criteria of AI/ML model for AI/ML based positioning accuracy enhancement - Validity conditions, e.g., applicable area/[zone/]scenario/environment and time interval, etc. - Model capability, e.g., positioning accuracy quality and model inference latency - Conditions and requirements, e.g., required assistance signalling and/or reference signals configurations, dataset information - Note: other aspects are not precluded

Another agreement used as a reference in this subsection is the one raised in the 3GPP RAN1-111:

Agreement - Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on feasibility, potential benefits (if any) and potential specification impact at least for the following aspects - At least the following are identified for further study as potential data for calculating monitoring metric - If monitoring based on model output - E.g. , estimated UE location corresponding to model output for direct AI/ML positioning, estimated intermediate parameter(s) corresponding to model output for AI/ML assisted positioning, ground truth label corresponding to model inference output for both direct and AI/ML assisted positioning - If monitoring based on model input - E.g., measurement corresponding to model inference input - Note1: other type of potential data for model monitoring is not precluded - Note2: combination of one or more type of potential data for monitoring is not precluded - If a given type of data is necessary for calculating monitoring metric, study whether and if so - How an entity can be used to provide the given type of data for calculating monitoring metric - Companies are requested to report their assumption of the entity (or entities) used to provide the given type of data for calculating monitoring metric for each case - Potential signalling for provisioning of the given type of data for calculating associated monitoring metric - Potential assistance signaling and procedure to facilitate an entity providing data for calculating monitoring metric - Potential UE-network interaction - E.g., model monitoring decision indication between UE and network

## Model Updating

It is well known that RF imperfections introduce various distortions (e.g., carrier frequency offset, sampling time offset, TX/RX beam offsets, clock offsets and drifts, phase noise, etc.) to the received signal. In data communications, these errors are estimated with the network support (e.g., by frequency, and time offsets are compensated for during synchronization). However, that support is not available in positioning, and as a result, the distortions are absorbed in the total positioning error. These distortions typically depend on the HW limitations of the different antenna configurations and form factors such as ADC resolutions and crystal oscillators.

Observation-12: Considerations related to RF limitations translate into an additional phase rotation and delays of the positioning signal by the RF chain, as observed at the baseband receiver. As a result, a positioning entity (UE, TRP, etc.) hosting the ML positioning function experiences certain RF-based signal distortions which are not considered explicitly or characterized and compensated for when training the model. Such imperfections are different between host-type devices. For example, a PRU or gNB hosting the ML model would require adapting the model to its RF-specific characteristics.

Therefore, there is a need for a framework through which the generic ML positioning model is customized to the specific NR elements host types. Specifically, before the ML model is deployed on a large scale, it is refined (updated) based on at least intrinsic characteristics (e.g., RF limitations) of the NR element types, including target UE, PRU, or gNBs.

One way of enabling the above scenario would be that LMF selects a head NR unit as representative for a given NR element type and thus for a given expected intrinsic distortion range. Next, the model is customized and refined with the help of these head units to compensate for the element-specific distortion.

Proposal-7: RAN1 to consider a framework for positioning, through which a generic ML positioning model can be customized to the specific NR elements host types - including target UE, PRU, or gNBs and their RF chain imperfections.

Potential specification impact:

Such host-type-based refinement may be coordinated by a central unit, e.g., LMF, which may select a set of representatives, i.e., NR head unit, to assist with such model tailoring. For a given expected intrinsic distortion range, the head unit becomes representative for a given NR host type. Then, the model is refined by/with the support of head units so that it is best customized to compensate the element-specific distortion.

To perform the model adaptation, the LMF may provide to head units the generic ML model and request it to perform model refinement. Alternatively, the LMF may request the head unit to collect, timestamp, transfer its training data, and model specific constraints (e.g., the maximum number of inputs and their sampling resolution, depth and type of the neural network). The selection of head units and the type of assistance information exchanged bi-directionally is expected to require standardization by RAN1.

Proposal-8: RAN1 to consider model monitoring to be triggered and/or coordinated by the LMF, including which NR elements should perform the monitoring.

## Impact related to AI/ML model monitoring

Based on the latest RAN1-111 agreements, it is critical to study the potential specification impact of the AI/ML model monitoring for the positioning use case, which includes:

Agreement - Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on feasibility, potential benefits (if any) and potential specification impact at least for the following aspects - At least the following are identified for further study as potential data for calculating monitoring metric - If monitoring based on model output - E.g. , estimated UE location corresponding to model output for direct AI/ML positioning, estimated intermediate parameter(s) corresponding to model output for AI/ML assisted positioning, ground truth label corresponding to model inference output for both direct and AI/ML assisted positioning - If monitoring based on model input - E.g., measurement corresponding to model inference input - Note1: other type of potential data for model monitoring is not precluded - Note2: combination of one or more type of potential data for monitoring is not precluded - If a given type of data is necessary for calculating monitoring metric, study whether and if so - How an entity can be used to provide the given type of data for calculating monitoring metric - Companies are requested to report their assumption of the entity (or entities) used to provide the given type of data for calculating monitoring metric for each case - Potential signalling for provisioning of the given type of data for calculating associated monitoring metric - Potential assistance signaling and procedure to facilitate an entity providing data for calculating monitoring metric - Potential UE-network interaction - E.g., model monitoring decision indication between UE and network

Furthermore, aspects related to data collection as per RAN1-111 agreements for AI/ML model training for AI/ML based positioning:

Agreement - Regarding data collection for AI/ML model training for AI/ML based positioning, - The following options of entity and mechanisms to generate ground truth label are identified for further study - For direct AI/ML positioning, ground truth label is UE location - PRU with known location - UE generates location based on non-NR and/or NR RAT-dependent positioning methods - LMF generates UE location based on positioning methods - LMF with known PRU location - Note: user data privacy needs to be preserved - For AI/ML assisted positioning, ground truth label is one or more of the intermediate parameter(s) corresponding to AI/ML model output - PRU generates label directly or calculates based on measurement/location - UE generates label directly or calculates based on measurement/location - Network entity generates label directly or calculates based on measurement/location - The following options of entity to generate other training data at least measurement corresponding to model input are identified for further study - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP - Note: other options of entity to generate other training data are not precluded - Note: Existing PRU definition is in 38.305

The diverse characteristics of the wireless environment require extraction of the labeled data for training, monitoring, testing, and validation of a given AI/ML model, as also reflected in the latest RAN1-111 agreements, which are summarized in the following:

- How to perform data collection for training, obtain ground truth or other training data for Case 1 to Case 3b

- Model monitoring, testing, and validation

Moreover, when the model is deployed on the UE side, the given AI/ML model is expected to be proprietary (closed model).

Observation-13: The AI/ML model deployed at UE used for positioning could be implementation-specific and may not be shared with other entities, e.g., with the network (LMF).

To overcome the above challenges, the network can collect data to monitor the performance of the AI/ML model. In turn, depending on the outcome, the network may update, re-select or deactivate the AI/ML model. For this purpose, the required data or "labeled data" could be made available at the UE with network assistance to derive the AI/ML model performance.

Model monitoring would also depend on the direct AI/ML positioning and assisted AI/ML positioning. For example, in assisted AI/ML positioning, to increase confidence on the training data and the model output, the intermediate feature extraction shall be tested/validated against the "labeled data" as illustrated in Figure 6. The "labels" represent the ground truth, such as true LOS/NLOS classification or true location of UE. As data labeling is not for free, it typically requires external gears/devices that support an in-field measurement.

The positioning reference unit (PRU) introduced and discussed in TS 38.305 is intrinsically suitable to accommodate real-world measurement and can provide labels for AI/ML models. For example, network assistance can be used to provide the PRU positioning measurement report and their associated true labels to monitor the given AI/ML model without knowing the details of the model at the UE. Moreover, the PRU positioning measurement and their corresponding labeled ground truth can also be used to fine-tune the AI/ML model at the UE.

Figure 6 - Model monitoring for direct AI/ML and AI/ML assisted positioning.

Observation-14: Labeled ground truth data is required to monitor the AI/ML model to increase the model's confidence.

For direct AI/ML positioning, the inference output corresponds to the estimated location; for assisted AI/ML positioning, the inference output corresponds to the intermediate features. Therefore, the ground truth label required for the model monitoring requires corresponding labels for both direct and assisted AI/ML positioning. Similarly, for Case1-Case3, the model can be deployed at the UE, LMF, or gNB to compute the model monitoring metric. Here, the required samples with relevant ground truth must be available on the computing node.

Observation-15: The inference output for direct AI/ML positioning is the estimated location. For assisted AI/ML positioning, the inference output would be the intermediate features (ToA, LOS/NLOS, etc.). Similarly, the model can be deployed at the UE, gNB, and LMF; therefore, the assistance data required for the model monitoring needs to be aligned with the representative sub-use case in which the node model is deployed.

Potential specification impact:

Figure 7 - Mechanism to monitor model performance and trigger possible actions.

The mechanism for model performance monitoring is shown in Figure 7. Here, we consider the scenario where UE is the node responsible for model training and inference. In case the UE moves to a new network coverage area or to a different region within the same network, which possibly might impact the model performance, the UE could request the LMF to share possible data for model monitoring. The LMF could then collect the data from the PRU or other "trusted" devices and share it with the UE. The UE could use the received data to monitor the model performance. The UE could compute and report model monitoring metrics to the LMF, where the details of the metric could depend on whether a direct or AI/ML-assisted positioning method is being implemented. Based on the reported model monitoring performance metric, the LMF could trigger actions such as fine-tuning or updating the model, model deactivation, or switching to classical positioning methods, etc.

Observation-16: In the scenario where the UE moves to a new network coverage area or a different region within the same network, the AI/ML model performance might get impacted and must require model monitoring.

Proposal-9:  For case 1 and case 2a, UE-side model considering a proprietary model, UE may request assistance data from the network (e.g., the LMF) that contain data for monitoring the AI/ML model performance (including samples with corresponding labels).

# Specific Aspects related to Model Input/Output and other assistance data

During the RAN1-109e meeting, the following agreement was made (relevant portion highlighted):

Agreement Companies are encouraged to study and provide inputs on potential specification impact at least for the following aspects of AI/ML approaches for sub use cases of AI/ML for positioning accuracy enhancement. - AI/ML model training - training data type/size - training data source determination (e.g., UE/PRU/TRP) - assistance signalling and procedure for training data collection - AI/ML model indication/configuration - assistance signalling and procedure (e.g., for model configuration, model activation/deactivation, model recovery/termination, model selection) - AI/ML model monitoring and update - assistance signalling and procedure (e.g., for model performance monitoring, model update/tuning) - AI/ML model inference input - report/feedback of model input for inference (e.g., UE feedback as input for network side model inference) - model input acquisition and pre-processing - type/definition of model input - AI/ML model inference output - report/feedback of model inference output - post-processing of model inference output - UE capability for AI/ML model(s) (e.g., for model training, model inference and model monitoring) - Other aspects are not precluded - Note: not all aspects may apply to an AI/ML approach in a sub use case - Note2: the definitions of common AI/ML model terminologies are to be discussed in agenda 9.2.1

A related agreement on the node for model inference and related specification impact was made during RAN1-110:

Agreement Study aspects in terms of potential benefit(s) and requirement(s)/specification impact(s) of AI/ML model training and inference in AI/ML for positioning accuracy enhancement considering at least - UE-side or Network-side training - UE-side or Network-side inference - Note: model inference at both UE and network side is not precluded where proponent(s) are encouraged to clarify their AI/ML approaches Note: companies are encouraged to clarify aspects of their proposed AI/ML approaches for positioning when AI/ML model training and inference are not performed at the same entity

And the following agreement was done in RAN1-111:

Agreement Regarding AI/ML model inference, to study and provide inputs on potential specification impact (including necessity and applicability of specifying AI/ML model input and/or output) at least for the following aspects for each of the agreed cases (Case 1 to Case 3b) in AI/ML based positioning accuracy enhancement - Types of measurement as model inference input - new measurement - existing measurement - UE is assumed to perform measurement as model inference input for Case 1, Case 2a and Case 2b; TRP is assumed to perform measurement as model inference input for Case 3a and Case 3b - Report of measurements as model inference input to LMF for LMF-side model (Case 2b and Case 3b) - For AI/ML assisted positioning, new measurement report and/or potential enhancement of existing measurement report as model output to LMF for UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a) - Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model - New and/or enhancement to existing assistance signaling - Note: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed

Based on the above agreements, it is essential to clarify the need for possible inputs from the network to the UE – in the form of new signaling, especially in scenarios where UE-based model inference is done.

In this section, we focus on possible issues that might arise in the case of UE-based model inference and potential signaling support that the network can provide to the UE. We note that several companies have proposed multiple input types and sizes to solve a given localization task, e.g., location estimation or LOS detection. Thus, depending on the underlying localization method, it has been proposed that the LMF may require measurements of delay profile, power delay profile (PDP), channel impulse response (CIR), received signal phase, RSRP, RSRPP, RSSI, LOS indicator, etc. or any combination of thereof.

## Network-based Feature Selection for UE-based Positioning Accuracy Improvements

The AI/ML algorithms for classifying the channels as LOS or NLOS typically use the fact that the channel measurements or CIRs look different for LOS and NLOS links. Such differences are numerically expressed through the values of relevant features extracted from CIRs. For example, ​NLOS CIRs are usually associated with a higher delay spread, a less peaky CIR shape, a lower strongest-to-mean power ratio, a higher rise time, etc.​ Such features could be measured by signal parameters such as RSRP or derived using measured signals such as RMS delay spread.

However, the radio environment can be very diverse among different scenarios, such as the objects' size, distribution, and material or blockers leading to differences in penetration, reflection, and diffraction of radio signals from such things. Furthermore, available bandwidth, e.g., depending on the capability of the UE, significantly impacts the shape of the CIRs by determining their time resolution and further affecting the features extracted from CIRs.

To better illustrate the idea, let's refer to Figure 8 and Figure 9. Here, the RMS delay spread is a distinguishing channel feature for LOS/NLOS classification when 500 MHz of BW is available in Scenario 1 (Figure 8). At the same time, its distinguishing ability is degraded for 50 MHz or lower BW in Scenario 1. On the other hand, in Scenario 2 (Figure 9), the RMS delay spread is not a good feature for classification, even for 500 MHz of available bandwidth.

Figure 8 - Distribution of RMS delay spread for LOS vs NLOS measurements with different available bandwidths in Scenario 1.

Figure 9 - Distribution of RMS delay spread for LOS vs. NLOS measurements with different available bandwidths in Scenario 2.

It is evident from the above discussion that the optimal selection of channel features to be used for LOS/NLOS classification is highly dependent on the environment setting and the available bandwidth of the channel for the measurements.

Observation-17: LOS/NLOS classification using AI/ML depends on the environmental setting as well as the bandwidth capabilities of the UE.

To mitigate the impact of diverse environmental settings and bandwidth capabilities on the effectiveness of AI/ML-based NLOS classification, the network may adapt the channel features used for the classification based on the identified conditions and capabilities. That is, the network can determine a set of features associated with a given setting (e.g., a specific set of cells) and the bandwidth reported by the UE that is best suitable for LOS/NLOS classification purposes using AI/ML models. For example, the network could signal to the UE that a specific set of features (e.g., Kurtosis, RMS delay spread, or mean excess delay) should be used for the classification.

Observation-18: For optimal NLOS/LOS classification, the channel features used may not be static but dynamically updated based on the identified environmental conditions and UE capabilities.

Nevertheless, environmental conditions may change during a positioning session, meaning that the proper set of channel features should also be updated. In such case, to avoid additional signaling concerning the channel feature updates, the network may provide to the UE not just a single set of channel features but rather a superset (i.e., a list) of channel feature sets. This list of feature sets should apply to multiple environmental scenarios (e.g., based on the cell in which the UE is residing) and the UE bandwidth capabilities and thus may be ranked as such. By obtaining such a ranked feature set list, the UE may be configured in advance with the set of channel features to be used for AI/ML LOS indication.

The outcome of the LOS/NLOS classification at the UE side, using the feature sets, can be then either utilized by the UE itself in the case of UE-based positioning (i.e., Case 1), or it could be reported to the LMF in the case of UE-assisted/LMF-based positioning (i.e., Case 2a).

Similar approach also applies to Case 3a, where the model at the gNB side classifies UL channels as LOS/NLOS, to be indicated to LMF for AI/ML-assisted positioning. For this, gNB may receive assistance data from LMF, indicating the feature sets for classification, which are determined based on the supported bandwidth and computation capabilitites of the gNB, as well as the environment conditions.

LPP and NRPPa protocols can be extended to include this information as part of the assistance data providing the ranked list of channel features to the UE and gNB, respectively.

Proposal-10 The network should be able to assist UE and/or gNB for LOS/NLOS classification by providing a ranked list of channel features, which applies to Case 1, Case 2a, and Case 3a. The ranked list should be based on the UE and/or gNB capabilities in terms of bandwidth, computation, and environmental setting.

## Positioning Accuracy Improvement Under Heavy NLOS Using ML-based Positioning Anchor Selection

One of the challenges in positioning is the selection of positioning anchors (i.e., the nodes that will transmit and/or receive reference signals for positioning) to do positioning measurements. Various factors related to anchors have a direct impact on positioning accuracy. These include the channel quality between the target UE and the anchors (e.g., LOS/NLOS, SINR), geometric arrangement of the anchors with respect to each other and the target UE, the confidence of the anchor locations, relative distance and/or speed between the anchors and the target UE. The anchor selection problem is further complicated when different types of anchors such as gNB/TRP, GNSS, RSU, PRU, or another UE (in case of SL positioning) with different mobility and channel characteristics appear and disappear from the UE surroundings over time.

Common approaches to solve the anchor selection problem rely on various channel metrics between the UEs and anchors, such as LOS/NLOS classification, ToA, etc. Based on such metrics, anchors are then (not) selected if they (do not) satisfy specific criteria, e.g., a channel metric value below/above a threshold. However, such approaches might become inefficient under mobile conditions, where the thresholds must be dynamically adjusted, e.g., according to the varying channel conditions.

To help make efficient decisions on anchor selection under dynamically mobile conditions with a multitude of candidate anchors available, ML methods could be utilized instead.

Considering different use cases, the ML model selecting the anchors may reside either at the i) UE, ii) gNB, or iii) LMF side. Among these options, a gNB-sided model (as in Case 3a) would be restrictive in case we consider that the UE may be mobile. An LMF-sided model would have access to more knowledge in general, e.g., the location of the surrounding anchors, whereas a UE-sided model would be able to work also outside the network coverage and may enable a faster operation. In any case, the model input requires data relating to the properties of potential anchors (e.g., their location, link conditions, etc.). An indicative signal of the positioning QoS resulting from the selected anchor, such as in terms of positioning accuracy, is necessary to train the model.

If the model resides at the UE, the model can be used for UE-based positioning (as in Case 1) and LMF-based positioning (as in Case 2a). If LMF is involved, it would request UE to determine and propose candidate anchors for positioning by providing necessary assistance data (including any potential list of anchors). To train the model, the UE would request data from the network, such as indicative of the positioning accuracy resulting from the anchor selection.

If the model resides at the LMF side, LMF would collect the necessary information from the UEs and/or NG-RAN nodes (Case 2b and Case 3b) to input to the model. The outcome, i.e., the selected anchors, can be used again for UE-based or LMF-based positioning.

Proposal 11: Considering Case 1 and Case 2a, UE may conduct anchor selection for positioning by requesting necessary assistance data from the network (e.g., containing the location of candidate anchors). For such anchor selection task, UE may train an ML model, requesting a signal from the network that indicates the positioning QoS resulting from the selected anchor(s). Similarly, LMF may request UE to do anchor selection by providing necessary assistance data, including any potential list of anchors.

## Model Input types and sizes and One-Sided vs. Two-Sided Models

On the topic of one-sided and two-sided models, the following proposal was discussed during the RAN1-110 meeting:

Proposal 3.2.1-2 For evaluation of AI/ML-based positioning, one-sided model is prioritized over two-sided model.

In the 3GPP RAN1-111 meeting, companies achieved the following agreement:

Agreement For the study of benefit(s) and potential specification impact for AI/ML based positioning accuracy enhancement, one-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI.

One-sided models imply that a single AI/ML model is deployed. This model resides at a single NR element (e.g., UE, TRP, or LMF) with applicable positioning methods, including direct and AI/ML-assisted positioning methods. Most of the solution approaches presented in this study have been considered one-sided models, with the AI/ML model directly indicating the UE location or providing intermediate features used by classical positioning approaches to estimate the UE location.

In the other case, two-sided models imply that there are two AI/ML models, possibly deployed at the UE-side and network-side, with the output of the first model provided as the input for the second model. Such an approach will remove the need to define new reporting metrics, e.g., CIR, and may only require RAN to determine a code (and a code size) to be transferred among models. One key motivation to consider two-sided approaches related to the agreement from RAN1-110b-e meeting is to consider CIR and PDP as possible model inputs.

For one-sided models, RAN1 discusses using CIR and PDP as inputs. Both model inputs require signaling measurements with high dimensionality (which scale linearly with the number of receive antennas, sampling resolution, and a certain number of PRBs), with potentially high reporting overhead. This would imply that with these model inputs, the network would need to collect measurements over significant periods for different PRS bandwidths/carriers and for UEs with a variable number of RX antennas to create a sufficiently diverse training dataset, as well as a systematic collection of CIR/PDP measurements from the UE for model inference – in case of LMF/network-based model inference. Such considerations make it necessary to investigate models that could enable vendor-agnostic training data collection with minimal overhead. The benefits of the two-sided models would be the translation of the PDP/CIR measurements into small measurement reports on one side, which could then be used to reconstruct the large initial measurement on the other side, with little to no reconstruction loss.

Observation-19: The solution approaches presented so far as part of this study have considered one-sided models, with the AI/ML model output directly indicating the UE location or providing intermediate features that are used by classical positioning approaches to estimate the UE location.

Observation-20: Using CIR and PDP with high-dimensionality as model input could cause significant overhead for data collection for model training and inference. It is also unclear how an AIML model for positioning can cope with variable CIR/PDP size and shape.

Proposal-12: RAN1 to consider the impact of both CIR and PDP as model input in terms of over-the-air signaling and assess solutions to enable overhead reduction and improve the quality of the collected data samples.

Furthermore, positioning reference signals may be aggregated over multiple carriers, where each aggregated carrier signal may occupy a different bandwidth, implement a different comb, have a different duration, etc. Hence, a positioning signal can occupy various resource elements from one session to the other, and a machine learning positioning module should be able to cope with this variable signal size. To ensure that the NW may request positioning measurements (for training, inference, testing, etc.) from each UE/TRP in a selected fixed format (size and type) which is agnostic to the size and shape of the positioning reference signals.

In one example, this may be realized by projecting the received signal onto a space indicated by, e.g., the LMF. Specifically, the LMF transfers to the UE a projector/basis and requires the UE to return the results of projecting each received signal onto the selected basis.

For example, a similar approach to that used in the CSI feedback use case may be applied where the UE/TRP is instructed to collect P/SRS samples and encode them in a code H to be used on the NW side. Generating code H may be seen as compressing the received signal, while using code H by the LMF may be equivalent to decompressing the code H.

Furthermore, when the AIML positioning model resides in the positioning receiver (e.g., UE or TRP), the latter should be able to run the AIML model no matter the size and shape of the received signal.

Observation-21: A positioning receiver that uses AI/ML-direct/assisted positioning must ensure that the input to the AI/ML block is of fixed size and shape, regardless of the size and shape of the received positioning signal.

Proposal-13: RAN1 to study solutions that enable the UE to report positioning measurements in a format agnostic to the PRS configuration, including what type of assistance the LMF may provide for enabling agnosticism.

# Conclusion

In this contribution, we make the following observations and proposals:

## Observations

Observation-1: A method to overcome the training data scarcity challenge would be to reuse measurements collected during the regular UE operation (e.g., during ongoing positioning sessions) as training data samples for a positioning task. Such measurements are not labeled by default; therefore, new labeling methods must be devised.

Observation-2: The noisy label evaluation can be assisted by the LMF and can optimize the data collection and accept/reject a sample based on the labeling accuracy. For samples with only one labeling source, the evaluation may depend only on the variance of the position estimation compared to the target accuracy.

Observation-3: In case of the opportunity to label a sample with multiple positioning sources, one may exploit all the information from different positioning sources and incorporate them in the labels for training the ML model.

Observation-4: Answering the noisy label problem is expected to be use-case dependent and may require some assistance from the network. For example, the network may provide a set of rules for label quality evaluation, where the network may indicate one or more rules to reject/accept a sample with one or several noisy labels.

Observation-5: The potential specification impact from noisy ground truth labels during the data collection and model training phase could depend on whether UE-side or network-side training is considered.

Observation-6: For network-side training, label correction/modification could be done without any specification impacts as long as the network has sufficient information related to the location of the PRUs.

Observation-7: There are no mechanisms to evaluate the dataset quality in the spatial domain and the correlation with the respective scenario attribute, such as clutter density, clutter height, etc.

Observation-8: The AI/ML model deployed at UE for positioning could be a vendor-specific or proprietary algorithm (e.g., black box).

Observation-9: The challenges related to AI/ML model training related to dataset collection, quality, and required network assistance could be addressed with the help of additional synthetic data or data augmentation.

Observation-10: AI/ML model generalization can be realized on the variations of the dataset on the same site/area using DA techniques for unavailable UE locations.

Observation-11: Any given positioning measurements that does not represent well the reflectors distribution for that set of TRPs relative to the UE location may be seen as abnormal/rare events. Nevertheless, some rare events may prove useful to the ML positioning function, while others may be entirely detrimental.

Observation-12: Considerations related to RF limitations translate into an additional phase rotation and delays of the positioning signal by the RF chain, as observed at the baseband receiver. As a result, a positioning entity (UE, TRP, etc.) hosting the ML positioning function experiences certain RF-based signal distortions which are not considered explicitly or characterized and compensated for when training the model. Such imperfections are different between host-type devices. For example, a PRU or gNB hosting the ML model would require adapting the model to its RF-specific characteristics.

Observation-13: The AI/ML model deployed at UE used for positioning could be implementation-specific and may not be shared with other entities, e.g., with the network (LMF).

Observation-14: Labeled ground truth data is required to monitor the AI/ML model to increase the model's confidence.

Observation-15: The inference output for direct AI/ML positioning is the estimated location. For assisted AI/ML positioning, the inference output would be the intermediate features (ToA, LOS/NLOS, etc.). Similarly, the model can be deployed at the UE, gNB, and LMF; therefore, the assistance data required for the model monitoring needs to be aligned with the representative sub-use case in which the node model is deployed.

Observation-16: In the scenario where the UE moves to a new network coverage area or a different region within the same network, the AI/ML model performance might get impacted and must require model monitoring.

Observation-17: LOS/NLOS classification using AI/ML depends on the environmental setting as well as the bandwidth capabilities of the UE.

Observation-18: For optimal NLOS/LOS classification, the channel features used may not be static but dynamically updated based on the identified environmental conditions and UE capabilities.

Observation-19: The solution approaches presented so far as part of this study have considered one-sided models, with the AI/ML model output directly indicating the UE location or providing intermediate features that are used by classical positioning approaches to estimate the UE location.

Observation-20: Using CIR and PDP with high-dimensionality as model input could cause significant overhead for data collection for model training and inference. It is also unclear how an AIML model for positioning can cope with variable CIR/PDP size and shape.

Observation-21: A positioning receiver that uses AI/ML-direct/assisted positioning must ensure that the input to the AI/ML block is of fixed size and shape, regardless of the size and shape of the received positioning signal.

## Proposals:

Proposal-1: RAN1 to study the possibility of labeling measurements collected during regular UE operation, where such measurements, upon labeling, may be used for training a positioning machine learning task.

Proposal-2: RAN1 to study solutions on how to clean the noisy labels for Case1 and Case 2a as part of data collection.

Proposal-3: RAN1 to study adaptable training solutions to assist in cleaning noisy labels. Such assistance may be in the form of defining new metrics associated with the label cleanliness level and/or cleanliness thresholds based on which the related training data sample is retained or discarded.

Proposal-4: RAN1 to consider inter-point distance (IPD) as a KPI while collecting training data samples and during model training. If the dataset used for model training does not conform to the required IPD metric, data augmentation could be used to generate additional data samples synthetically.

Proposal-5: RAN1 to consider network-assisted data augmentation solutions as part of data collections for direct and assisted AI/ML positioning.

Proposal-6: RAN1 to define means to identify/manage abnormal propagation conditions during the data collection.

Proposal-7: RAN1 to consider a framework for positioning, through which a generic ML positioning model can be customized to the specific NR elements host types - including target UE, PRU, or gNBs and their RF chain imperfections.

Proposal-8: RAN1 to consider model monitoring to be triggered and/or coordinated by the LMF, including which NR elements should perform the monitoring.

Proposal-9:  For case 1 and case 2a, UE-side model considering a proprietary model, UE may request assistance data from the network (e.g., the LMF) that contain data for monitoring the AI/ML model performance (including samples with corresponding labels).

Proposal-10 The network should be able to assist UE and/or gNB for LOS/NLOS classification by providing a ranked list of channel features, which applies to Case 1, Case 2a, and Case 3a. The ranked list should be based on the UE and/or gNB capabilities in terms of bandwidth, computation, and environmental setting.

Proposal 11: Considering Case 1 and Case 2a, UE may conduct anchor selection for positioning by requesting necessary assistance data from the network (e.g., containing the location of candidate anchors). For such anchor selection task, UE may train an ML model, requesting a signal from the network that indicates the positioning QoS resulting from the selected anchor(s). Similarly, LMF may request UE to do anchor selection by providing necessary assistance data, including any potential list of anchors.

Proposal-12: RAN1 to consider the impact of both CIR and PDP as model input in terms of over-the-air signaling and assess solutions to enable overhead reduction and improve the quality of the collected data samples.

Proposal-13: RAN1 to study solutions that enable the UE to report positioning measurements in a format agnostic to the PRS configuration.

# References

- 3GPP RP-213599, "New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface" 3GPP RAN-94-e.

- 3GPP R1-2212331, "Evaluation on AI/ML for positioning accuracy enhancement," Nokia, Nokia Shanghai Bell, 3GPP RAN1-111.

- 3GPP R1-2209371, "Evaluation on AI/ML for positioning accuracy enhancement," Nokia, Nokia Shanghai Bell, 3GPP RAN1-110bis-e.

- 3GPP R1-2205633, "Final Summary of [109-e-R18-AI/ML-07] Email discussion on evaluation of AI/ML for positioning accuracy enhancement," RAN1-109-e, May 2022.

- 3GPP TS 38.305, "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 17)".