3GPP TSG RAN WG1 #112	   R1-2300644

Athens, Greece, February 27th – March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | XXXX | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on UE resuming a UE initiated COT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - gNB resuming a gNB initiated COT and COT sharing between gNB and UE have been supported in FR 2-2. To achieve similar benefit, UE resuming a UE initiated COT should be supported in FR 2-2 as well. - As there is no consensus to introduce a higher layer parameter to indicate Type 2 /Type 3 LBT in COT resuming at UE, to comply with the regulations in all the regions, for the UE with Type 2 LBT capability, a Type 2 LBT can be applied to UE resuming a UE initiated COT if the gap between the DL transmissions and any previous transmission is at least 8 us.Otherwise, for the UE without Type 2 LBT capbility, Type 1 LBT should be applied to UE resuming a UE initiated COT if the gap between the DL transmissions and any previous transmission is at least 8 us. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add text description in section 4.4.4 in TS 37.213 to desrbile UE resuming a UE intiated COT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE resuming a UE initiated COT cannot be supported in FR2-2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.4.4	Channel access procedures in an initiated channel occupancy

If a gNB/UE initiates a channel occupancy using the channel access procedures described in clause 4.4.1 on a channel, the gNB/UE may transmit a DL/UL transmission(s) that is followed by a UL/DL transmission(s) within the maximum Channel Occupancy Time described in Clause 4.4.1. The followings are applicable to the UL/DL transmission(s):

-	The transmission bandwidth(s) corresponding to the UL/DL transmission(s) shall be within the DL/UL bandwidth part(s) where in the channel occupancy is initiated:

-	Regardless of the duration of the gap between the UL/DL transmission(s) and previous DL/UL transmission(s) on the channel, the UL/DL transmission(s) occurs following the procedures described in Clause 4.4.3; or

-	if the gap between the UL/DL transmission(s) and previous DL/UL transmission(s) on the channel is more than a threshold that is determined by the gNB and is at least 8μs, the UL/DL transmission(s) occurs following the procedures described in Clause 4.4.2. Otherwise, the UL/DL transmission(s) occurs following the procedures described in Clause 4.4.3.

If a gNB initiates a channel occupancy using the channel access procedures described in clause 4.4.1 on a channel, the gNB may transmit a DL transmission(s) on the channel within the maximum Channel Occupancy Time described in Clause 4.4.1 on the channel after the DL transmission(s) initiating the channel occupancy. The followings are applicable to the DL transmission(s):

-	Regardless of the duration of the gap between the DL transmission(s) and any previous transmission(s) corresponding to the channel occupancy initiated by the gNB, the DL transmission(s) occurs following the procedures described in Clause 4.4.3; or

-	if the gap between the DL transmission(s) and any previous transmission(s) corresponding to the channel occupancy initiated by the gNB is more than a threshold that is determined by the gNB and is at least 8μs, the DL transmission(s) occurs following the procedures described in Clause 4.4.2.

If a UE initiated a channel occupancy using the channel access procedures described in clause 4.4.1 on a channel, the UE may transmit a UL transmission(s) on the channel within the maximum Channel Occupancy Time described in Clause 4.4.1 on the channel after the UL transmission(s) initiating the channel occupancy. The followings are applicable to the UL transmission(s):

-	When Type 2 channel access capability is indicated by UE, if the gap between the UL transmission(s) and any previous transmission(s) corresponding to the channel occupancy initiated by the UE is at least 8μs,the UL transmission(s) occurs following the procedures described in Clause 4.4.2. Otherwise, the UL transmission(s) occurs following the procedures described in Clause 4.4.3.

-    When Type 2 channel access is not indicated by UE, if the gap between the UL transmission(s) and any previous transmission(s) corresponding to the channel occupancy initiated by theUE is at least 8μs,the UL transmission(s) occurs following the procedures described in Clause 4.4.1. Otherwise, the UL transmission(s) occurs following the procedures described in Clause 4.4.3.

If a gNB shares a channel occupancy initiated by a UE using the channel access procedures described in clause 4.4.1 on a channel, the gNB may transmit a transmission on the channel that follows a scheduled UL transmission or a configured PUSCH transmission by the UE if the following conditions are satisfied:

-	The DL transmission shall contain transmission to the UE that initiated the channel occupancy and can include non-unicast and/or unicast transmissions where any unicast transmission that includes user plane data is only transmitted to the UE that initiated the channel occupancy.

When a UE is provided cg-COT-SharingList-r17 by higher layers, the UE is configured with a table wherein each row is given by higher layer parameter CG-COT-Sharing-r17. One row of the table is configured for indicating that the channel occupancy sharing is not available and other rows of the table each provides a channel occupancy sharing information. In this case, each configured grant PUSCH transmission includes 'COT sharing information' in CG-UCI as described in [10] that indicates a row index to the table.

If a gNB shares a channel occupancy initiated by a UE using configured grant PUSCH transmission and the UE is configured by cg-COT-SharingList-r17, the gNB may transmit a transmission that follows the configured grant PUSCH transmission by the UE if the following conditions are satisfied:

-	If the gNB determines that the 'COT sharing information' in CG-UCI in slot n indicates a row index that corresponds to a CG-COT-Sharing-r17 that provides channel occupancy sharing information, the gNB can share the UE channel occupancy starting from slot n+O, where O= offset-r17 slots, for a duration of D=duration -r17 slots where duration-r17, and offset-r17 are higher layer parameters provided by CG-COT-Sharing-r17.