3GPP TSG RAN WG1 #112		                                                      R1-2300663

Athens, Greece. February 27th – March 3rd, 2023

Source:	CATT

Title:	Discussion on timing advance management to reduce latency

Agenda Item:	9.12.2

Document for:	Discussion and Decision

Introduction

In RAN #97 e-meeting, a revised WID on further NR mobility enhancements was agreed [1]. In the WID, timing advance management was included in order to reduce handover interruption time. In RAN1 #111 meeting, the following agreements on timing advance management were achieved [2] .

Agreement On mechanism to acquire TA of the candidate cell(s) in Rel-18 LTM, at least support PDCCH ordered RACH. - The PDCCH order is only triggered by source cell - FFS: the details including content of DCI, RACH resource configuration, RAR transmission mechanism, etc. - Note: any other RACH-based solutions are for discussion separately Agreement (Made in RAN1#110b-e) Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility. - FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2) Agreement For PDCCH ordered RACH in LTM, at least the following enhancements are supported - Introduce indication of candidate cell and/or RO of candidate cell in DCI - configuration of RACH resource for candidate cell(s) is provided prior to the PDCCH order - FFS: whether/how to transmit RAR Agreement On whether RAR is needed for PDCCH ordered RACH for a candidate cell in LTM, the following alternatives are considered for further study - Alt 1: RAR is needed - Alt 2: RAR is not needed - Note: If Alt 2 is supported, TA value of candidate cell is indicated in cell switch command - Alt 3: whether RAR is needed can be configured Agreement - TA updating (i.e. re-acquisition of TA) for candidate cell can be triggered by NW. - same triggering mechanism reuse the initial TA acquisition, i.e., PDCCH order triggered RACH in a candidate cell

In this contribution, timing advance managements are discussed with the following outline:  RACH-based solution for TA acquisition is discussed in section 2, which include TA acquisition and TA indication by PDCCH-ordered RACH.

In section 3, RACH-less solution for TA acquisition is discussed with the emphasis on the SRS-based TA acquisition. In section 4, other issues, including the number of TA/TAG that can be maintained by UE and TAT issues are discussed. A brief conclusion is given in Section 5.

RACH-based solution

In RAN1 #111 meeting, it has been agreed that PDCCH ordered RACH is supported to acquire TA of the candidate cell(s) in Rel-18 LTM. For PDCCH ordered RACH, there are still the following issues to be discussed:

- Whether CFRA or CBRA based RACH or both should be supported?

- Whether to receive RAR after triggered PRACH transmission

- If RAR is needed, whether it is received from serving cell or candidate cell

- What’s the TA indication mechanism for the supported CFRA/CBRA based RACH without RAR

In NR Rel-16/17， both CFRA and CBRA are supported to obtain the initial TA through PDCCH ordered RACH, Random Access Preamble index field in the PDCCH order is used to distinguish CBRA and CFRA, as shown in Table 1. When this field takes all 0, a CBRA procedure is triggered, otherwise, the ra_PreambleIndex is used for a CFRA procedure. This procedure can be extended in NR Rel-18 LTM to determine the initial TA of the candidate cell before handover command with the candidate cell index included in the PDCCH order DCI. For CFRA, similarly to those in traditional method, UE will send the preamble with ra_PreambleIndex of the candidate cell to obtain the initial TA of the candidate cell.

The beam used to send the preamble is the receiving beam of the associated SSB, which has been obtained during the beam measurement report,  indicated in the PDCCH order by SS/PBCH Index and the candidate cell index. The candidate cell index is used to indicate which candidate cell the indicated SS/PBCH belongs to, reserved bits(10 bits) in PDCCH order can be considered if the Candidate cell index is less than 10 bits. The RO associated with the SS/PBCH index has been configured to UE through ssb-perRACH-OccasionAndCB-PreamblesPerSSB , considering that one SS/PBCH index might be associated with more than one RO, the ultimate RO to transmit the preamble is determined by the PRACH Mask Index in the PDCCH order. The time domain and frequency domain resource for the PRACH occasion can be determined by the RRC configuration through RACH-ConfigGeneric, the time domain resource for the PRACH occasion can be determined by the same configuration and look up table 6.3.3.2-2 in TS 38.211.

RACH-ConfigGeneric ::=              SEQUENCE {

prach-ConfigurationIndex            INTEGER (0..255), (Time domain resource index for PRACH occasion, used as the index for Table 6.3.3.2-2 in TS38.211 to determine Time domain resource)

msg1-FDM                            ENUMERATED {one, two, four, eight},(Frequency domain resource indication)

msg1-FrequencyStart                 INTEGER (0..maxNrofPhysicalResourceBlocks-1),(Frequency domain resource of PRACH occasion)

zeroCorrelationZoneConfig           INTEGER(0..15),

preambleReceivedTargetPower         INTEGER (-202..-60),

preambleTransMax                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200},

powerRampingStep                    ENUMERATED {dB0, dB2, dB4, dB6},

ra-ResponseWindow                   ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80},

...,

[[

prach-ConfigurationPeriodScaling-IAB-r16    ENUMERATED {scf1,scf2,scf4,scf8,scf16,scf32,scf64}                    OPTIONAL,   -- Need R

prach-ConfigurationFrameOffset-IAB-r16      INTEGER (0..63)                                                       OPTIONAL,   -- Need R

prach-ConfigurationSOffset-IAB-r16          INTEGER (0..39)                                                       OPTIONAL,   -- Need R

ra-ResponseWindow-v1610                     ENUMERATED { sl60, sl160}                                             OPTIONAL, -- Need R

prach-ConfigurationIndex-v1610              INTEGER (256..262)                                                    OPTIONAL  -- Need R

]]

}

The power to send the preamble can be determined by the equation in section 7.4 of TS38.213.

Table 1. Fields of DCI of PDCCH order

| PDCCH order field | Size | Description |
| --- | --- | --- |
| Identifier for DCI formats | 1 bit | Value should be “1” indicating DCI Format 1_0 |
| Frequency domain resource assignment | log2(NRBDL,BWP∙(NRBDL,BWP+1)/2) | NRBDL,BWP is the size of DL BWP. This field is all ones indicating that this is a PDCCH order. |
| Random Access Preamble index | 6 bits | ra_PreambleIndex. If this is all 0 a CBRA procedure is triggered, else the ra_PreambleIndex is used for a CFRA procedure. |
| UL/SUL indicator | 1 bit | If SUL is configured, and ra_PreambleIndex is not all 0, this field indicates UL carrier, else it is reserved. When indicating UL carrier: “0” is for normal uplink and “1” is for SUL. |
| SS/PBCH Index | 6 bits | If ra_PreambleIndex is not all zeros, the SS/PBCH block index used to determine the associated RO to transmit the preamble on, else reserved. |
| PRACH Mask Index | 4 bits | If ra_PreambleIndex is not all zeros, this parameter is used to determine the RO to transmit the preamble on, else reserved. |
| Reserved bits Candidate cell index | 10 bits xxx bits | Index of the candidate cell. |

.

Proposal 1: Support CFRA to obtain TA of the candidate cell(s) in Rel-18 LTM for PDCCH ordered RACH. The RRC configuration of the candidate cell, e.g., the frequency and time domain resource to send the preamble and the association between SS/PBCH index and RACH occasion are pre-configured as legacy.

For CBRA, to obtain the TA of the candidate cell, UE selects a preamble and transmits it to the candidate cell in the PRACH occasion. To determine the preamble to be transmitted, UE can choose a SSB (e.g. the SSB with the maximum RSRP which has been stored during the beam management) and use the association between the SSB index and the preambles used for CBRA, e.g. for SSB index n, 0<n<3, the starting index of the preamble used for CBRA associated with this SSB index is n*64/4. UE will use this association to determine the range of the preamble that can be applied and randomly selects a preamble from this range.  The beam used to send preamble can be determined by the receiving beam of the associated SSB which has been stored during the beam measurement.

Similar to those of CFRA, the time domain resource and frequency domain resource to send the preamble can be determined by the RRC configuration through RACH-ConfigGeneric and look up table 6.3.3.2-2 of TS38.211.

Upon receiving the preamble from UE, candidate cell gNB will estimate the TA and acknowledges the reception of the preamble by sending a random access response (RAR) where the estimated TA will be included. The RAR can be received from the serving cell or the candidate cell and the UE attempts to receive the RAR during the RAR window until the RAR is successfully decoded. For CBRA based RACH, to obtain the TA of the candidate cell, UE has to monitor the information from candidate cell until the 4-step of CBRA is completed since contention resolve of the UE is completed in the fourth step. So RAR must be included for CBRA. During this procedure, the data transmission/reception with serving cell will be interrupted, this data transmission interruption need to be evaluated before the agreement on applying of CBRA.

Proposal 2: Do not support CBRA based RACH for the data transmission interruption with the serving cell due to the 4-step CBRA procedure to obtain TA of the candidate cell.

After receiving the preamble from UE, candidate cell gNB will estimate the TA, in the legacy 2-step or 4-step random access procedure, the TA is included in the RAR and indicated to UE. In the offline discussion after RAN1 #111 meeting, following proposal was made on whether RAR is needed:

Proposal 1: For PDCCH ordered-RACH, whether RAR is needed can be configured/indicated

•     FFS: if reception of RAR is configured/indicated, whether RAR is received from serving cell or target cell

•     if reception of RAR is not configured/indicated, TA value of candidate cell is indicated in cell switch command

•     FFS: RACH resource configuration for candidate cell

•     FFS: signaling for configuration/indication whether RAR needs to be received

If  RAR reception is indicated, whether  RAR is received from serving cell or target cell is an issue to be discussed. If RAR is received from serving cell, which means the RAR from the candidate cell needs to be transferred from candidate cell to the serving cell after UE sends the preamble. Considering the non-ideal backhaul between the serving cell and candidate cell, which varies from 2~50 ms, UE might miss the corresponding RAR reception using the current ra-ResponseWindow.

ra-ResponseWindow                  ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80},

If RAR is received from candidate cell, the PDCCH scheduling RAR need to be decoded. For which the Search space and CORESET of the non-serving cell need to be pre-configured. In addition, considering  that in Rel-17 non-UE dedicated PDCCH/PDSCH can only be transmitted by the serving cell, the UE behavior of receiving RAR need to be redefined.

If reception of RAR is not configured/indicated, UE will not receive RAR in the ra-ResponseWindow as the legacy. The advantage is that the interruption due to receiving RAR will be limited, e.g. at most 2 slots depending on the PRACH format and SCS used. In this case, additional rules are required to terminate the normal RACH procedure, otherwise, upon not receiving RAR from gNB. UE will regard that msg1 is not successfully decoded and will keep transmitting preamble with a ramping up power. In this case, TA value of candidate cell is indicated in cell switch command.

Regarding the signaling for configuration/indication whether RAR needs to be received, higher layer signaling,  e.g. RRC can be used to distinguish the with/without RAR for TA acquisition of candidate cells  from the normal ones.

Observation 1: For PDCCH ordered-RACH, if RAR reception is configured/indicated to be received from serving cell, considering the non-ideal backhaul between serving cell and candidate cell, which might cause UE missing RAR reception in the current ra-ResponseWindow.

Proposal 3: For PDCCH ordered-RACH, if RAR is configured/indicated to be received from non-serving cell, the Search Space and CORESET of the non-serving cell need to be pre-configured. Meanwhile, the UE behavior of receiving RAR need to be re-defined considering that in Rel-17 non-UE dedicated PDCCH/PDSCH can only be transmitted by the serving cell.

Proposal 4: For PDCCH ordered-RACH, if reception of RAR is not configured/indicated, additional rules are required to terminate normal RACH procedure.

Proposal 5:  For PDCCH ordered-RACH, if RAR reception is configured/indicated, TA can be included in RAR, otherwise, TA will be included in cell switch command.

Proposal 6: Regarding the signaling for configuration/indication whether RAR needs to be received, higher layer signaling, e.g. RRC can be used to distinguish the with/without RAR for TA acquisition of candidate cells  from the normal ones.

RACH-less solution

TA of the candidate cell can be also obtained by RACH-less solution. In RAN1 #111 meeting, there are the following alternatives:

- Alt1: SRS based TA acquisition

- Alt2: RACH-less mechanism as in LTE

- Alt3: UE based TA measurement(including UE based TA measurement with one TAC from serving cell

TA of the candidate cell can be obtained by SRS based TA acquisition. In the implementation, the following issues need to be considered:

- SRS resource/resource sets Configuration

In the procedure of SRS based TA acquisition, SRS resource of the candidate cell need to be pre-configured, this can be achieved by associating SRS resource/resource sets with candidate cell, e.g. including additional PCI index in SRS-ResourceSet

SRS-ResourceSet ::=                     SEQUENCE {

srs-ResourceSetId                       SRS-ResourceSetId,

srs-ResourceIdList                      SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId

usage                                   ENUMERATED {beamManagement,codebook,nonCodebook,}

………

candidate cell index                    CandidateCellIndex-r18

………

Proposal 7: For TA acquisition of the candidate cell, SRS based TA acquisition can be considered.

Proposal 8: For SRS based TA acquisition of the candidate cell, SRS resource/resource sets with candidate cell index need to be configured.

- Time and frequency resource to transmit SRS

The time and frequency resource to transmit SRS can be configured in SRS-Resource. For periodic and semi-persistent SRS, periodicity and offset are configured in SRS-Resource. To trigger the transmission of aperiodic SRS, traditional method is to use the SRS_request field(2 bits) in DCI format 0_1. Each 2 bits of SRS request corresponds to one aperiodicSRS-ResourceTrigger which is associated with 1 SRS resource set. This method can be extended to the candidate cell, e.g. configure additional PCI in SRS-ResourceSet and use traditional DCI format 0_1, or alternatively, introduce candidate cell index in triggering DCI format 0_1.  MAC CE can also be considered for the triggering, where aperiodicSRS-ResourceTrigger/SRS-ResourceSet with candidate cell indicator can be included.

Proposal 9: For SRS based TA acquisition of the candidate cell, for periodic and semi-persistent SRS, periodicity and offset of the SRS resource in the candidate cell is pre-configured. To trigger aperiodic SRS transmission, the following methods can be considered:

- Alt1: Use traditional DCI format 0_1 with additional PCI included in SRS-ResourceSet.

- Alt2: Introduce candidate cell index/indicator in the triggering DCI format 0_1.

- Alt3: New MAC CE is designed to include aperiodicSRS-ResourceTrigger/SRS-ResourceSet with candidate cell index.

- Beam used for SRS transmission

In SRS based TA acquisition, the beam used to transmit SRS to candidate cell is another issue to be considered. Considering downlink synchronization with the candidate cell has been completed at this time, a straightforward way is to use SS/PBCH block receiving beam of the same SS/PBCH block index as the one the UE uses to obtain MIB. Alternatively, SRS-SpatialRelationInfo configured in the candidate cell can be activated, candidate cell index can be included in the activated signaling, e.g. MAC CE.

Proposal 10: In SRS based TA acquisition of the candidate cell, the following mechanisms can be considered to determine the beam used for SRS transmission:

- Alt1: SS/PBCH block receiving beam to obtain MIB.

- Alt2: Activated spatial relation configured by SRS-SpatialRelationInfo.

- Power used for SRS transmission

In SRS based TA acquisition, the power to transmit SRS to the candidate cell needs to be determined. In the legacy method, UE determines the SRS transmission power  in SRS transmission occasion i as [3]:

- Power Control for SRS transmission

In SRS based TA acquisition, the power to transmit SRS to the candidate cell needs to be determined. In the legacy method, UE determines the SRS transmission power  in SRS transmission occasion i as [3]:

To determine the SRS transmission power to the candidate cell. The open loop parameter settings (P0, alpha) can be configured by candidate cell and transferred to UE. PL-RS can be CSI-RS/SSB included in the PathlossReferenceRS-Config configured in SRS-ResourceSet or SSB of the non-serving cell.

Close loop power control parameter setting can be determined by setting srs-PowerControlAdjustmentStates to be separateClosedLoop to indicate the SRS power control setting is independent of those of the PUSCH(considering there is no PUSCH transmission at this time). The close loop power control of SRS can be achieved by the TPC field in DCI with candidate cell index.

Proposal 11: To determine the SRS transmission power in SRS based TA acquisition of the candidate cell, PL-RS can be CSI-RS/SSB included in the PathlossReferenceRS-Config configured in SRS-ResourceSet or PSS/SSS of the candidate cell.

Proposal 12: The close loop power control of SRS can be achieved by the TPC field in DCI with candidate cell index. Close loop power control parameter srs-PowerControlAdjustmentStates is configured and set to be separateClosedLoop to indicate the SRS power control setting is independent of those of PUSCH considering there is no PUSCH transmission at this time.

- TA indication for SRS based TA acquisition

Estimated TA of the candidate cell can be transmitted to the serving cell and then the serving cell sends it to the UE in the cell switch command, in which candidate cell index/indicator is included. Each TA can be associated with a candidate cell Index/indicator, an example of the association is shown in Table 2. The association between candidate cell index and candidate cell indicator, which is shown in the first and second line of Table 2, can be indicated to UE by higher layer configurations, e.g. RRC. Meanwhile, the association between candidate cell index/indicator and TAG, as shown in the second and third column of the table, can also be indicated to UE, e.g. through MAC-CE. In the implementation, when a candidate cell indicator is indicated to UE, e.g. in the cell switch command, the associated TA value will be applied to the later uplink transmission.

Table 2. Association between TAG/TA and candidate cell Index/ indicator

| Candidate cell Index(PCI) | PCI-1 | PCI-2 | PCI-3 | PCI-4 |
| --- | --- | --- | --- | --- |
| Candidate cell Indicator | 00 | 01 | 10 | 11 |
| TAG/TA | 1/30 | 2/32 | 3/35 | 4/32 |

Proposal 13: Estimated TA of the candidate cell can be transmitted to the serving cell and then the serving cell sends it to the UE in the cell switch command with the candidate cell index/indicator included.

For Alt2, it can be achieved by implementation and has no additional impact on the current spec.

For Alt3, the reference signal used to estimate the receiving timing difference between serving cell and candidate cell need to be considered, and it also depends on the accuracy of TA. If SSB is used as the reference signal to estimate the downlink receiving difference between serving cell and candidate cell, the accuracy of the downlink receiving difference is not high due to the limited bandwidth of the SSB(20RB). An alternative way is to configure a group of wideband CSI-RS to improve the accuracy of the estimation, in that way the transmission efficiency of the data will be reduced due to the occupancy of the CSI-RS.

Proposal 14:  For UE based TA measurement, i.e. Alt3. the reference signal which could be used to estimate the downlink reference timing difference between serving cell and candidate cell needs to be considered.

Other issues

Issue 1：Number of TA/TAG maintained by UE for the candidate cell

Regarding the number of TA/TAG maintained by UE for the candidate cell, in RAN 1#111 meeting, there were the following proposal:

Proposal 1-3:  For TA management in L1/L2 based mobility, support at least one of candidate cells can be associated with one acquired TA other than the TA used for the serving cell

- FFS: the maximal number of TA associated with candidate cell(s) can be handled by UE

According to our understanding, UE will maintain at least 2 TAs in L1/L2 based mobility, one TA for serving cell and one TA for candidate cell. The maximum number of TA associated with candidate cell(s) can be handled by UE and depends on UE capability.

Proposal 15: For TA management in L1/L2 based mobility, support at least one of the candidate cells can be associated with one acquired TA/TAG other than the TA/TAG used for the serving cell. The maximal number of TA/TAG associated with candidate cell(s) depends on UE capability.

One related issue discussed in proposal 1-3 is whether two TA(s)/TAG(s) for a candidate cell can be configured, In RAN1#111 meeting, there were the following proposals:

Proposal 1-4: On whether two TA(s)/TAG(s) for a candidate cell can be configured, discuss and down-select among the following alternatives:

- Alt1: Support up to two TA(s)/TAG(s) configuration for a candidate cell if it is configured as mDCI MTRP

- Alt2: Don’t support two TA(s)/TAG(s) configuration for a candidate cell if it is configured as mDCI MTRP

In our opinion, whether two TA(s)/TAG(s) for a candidate cell can be configured depends on whether multi-DCI based multi-TRP is considered in L1/L2 mobility, if two TRPs for a candidate cell are considered in L1/L2 mobility, 2 TAGs can be configured as those already agreed in NR Rel-18 FeMIMO. Otherwise, it is preferable 1 TAG configured for 1 TRP.

Proposal 16: Support up to two TA(s)/TAG(s) configuration for a candidate cell if it is configured as mDCI MTRP. i.e. Alt1 is supported.

Issue 2:  TimeAlignmentTimer(TAT) related issue

In NR Rel-16/17,  TimeAlignmentTimer is configured in TAG.

TAG ::=                             SEQUENCE {

tag-Id                              TAG-Id,

timeAlignmentTimer                  TimeAlignmentTimer,

...

}

TimeAlignmentTimer ::=              ENUMERATED {ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity}

gNB will perform close loop TA updating periodically according to the periodicity provided by TimeAlignmentTimer. Both gNB and UE consider that uplink transmission is synchronized in the duration indicated by TAT, as stated in section 5.2 of [4].

- timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells belonging to the associated TAG to be uplink time aligned.

If TAT expires, gNB and UE will think uplink synchronization is destroyed. And the corresponding UE behavior includes flushing HARQ buffers and clearing the PUSCH/PUCCH/SRS transmission for all serving cells as defined in section 5.2 of [4].

In L1/L2 based inter-cell mobility, whether TAT needs to be maintained at UE side is the first issue to be considered. If the TA is only indicated to UE in the cell switch command, no TAT maintenance is needed at UE side, gNB can count the TAT and if gNB finds the expiration of TAT, gNB will initiate a PDCCH ordered RACH to re-acquire the TA.

Proposal 17: Consider whether TAT needs to be maintained at UE side.

Conclusions

In this contribution, we focus on the TA management in L1/L2 based inter-cell mobility, TA acquisition and indication for RACH-based solution as well as RACH-less solution are mainly discussed. We have the following proposals:

Proposal 1: Support CFRA to obtain TA of the candidate cell(s) in Rel-18 LTM for PDCCH ordered RACH. The RRC configuration of the candidate cell, e.g., the frequency and time domain resource to send the preamble and the association between SS/PBCH index and RACH occasion are pre-configured as legacy.

Proposal 2: Do not support CBRA based RACH for the data transmission interruption with the serving cell due to the 4-step CBRA procedure to obtain TA of the candidate cell.

Observation 1: For PDCCH ordered-RACH, if RAR reception is configured/indicated to be received from serving cell, consider the non-ideal backhaul between serving cell and candidate cell, which might cause UE missing RAR reception in the current ra-ResponseWindow.

Proposal 3: For PDCCH ordered-RACH, if RAR reception is configured/indicated to be received from non-serving cell, the Search Space and CORESET of the non-serving cell need to be pre-configured. Meanwhile, the UE behavior of receiving RAR need to be re-defined considering that in Rel-17 non-UE dedicated PDCCH/PDSCH can only be transmitted by the serving cell.

Proposal 4: For PDCCH ordered-RACH, if reception of RAR is not configured/indicated, additional rules are required to terminate normal RACH procedure.

Proposal 5:  For PDCCH ordered-RACH, if RAR reception is configured/indicated, TA can be included in RAR, otherwise, TA will be included in cell switch command.

Proposal 6: Regarding the signaling for configuration/indication whether RAR needs to be received, higher layer signaling, e.g. RRC can be used to distinguish the with/without RAR for TA acquisition of candidate cells  from the normal ones.

Proposal 7: For TA acquisition of the candidate cell, SRS based TA acquisition can be considered.

Proposal 8: For SRS based TA acquisition of the candidate cell, SRS resource/resource sets with candidate cell index need to be configured.

Proposal 9: For SRS based TA acquisition of the candidate cell, for periodic and semi-persistent SRS, periodicity and offset of the SRS resource in the candidate cell is pre-configured. To trigger aperiodic SRS transmission, the following methods can be considered:

- Alt1: Use traditional DCI format 0_1 with additional PCI included in SRS-ResourceSet.

- Alt2: Introduce candidate cell index/indicator in the triggering DCI format 0_1.

- Alt3: New MAC CE is designed to include aperiodicSRS-ResourceTrigger/SRS-ResourceSet with candidate cell index.

Proposal 10: In SRS based TA acquisition of the candidate cell, the following mechanisms can be considered to determine the beam used for SRS transmission:

- Alt1: SS/PBCH block receiving beam to obtain MIB.

- Alt2: Activated spatial relation configured by SRS-SpatialRelationInfo.

Proposal 11: To determine the SRS transmission power in SRS based TA acquisition of the candidate cell, PL-RS can be CSI-RS/SSB included in the PathlossReferenceRS-Config configured in SRS-ResourceSet or PSS/SSS of the candidate cell.

Proposal 12: The close loop power control of SRS can be achieved by the TPC field in DCI with candidate cell index. Close loop power control parameter srs-PowerControlAdjustmentStates is configured and set to be separateClosedLoop to indicate the SRS power control setting is independent of those of PUSCH considering there is no PUSCH transmission at this time.

Proposal 13: Estimated TA of the candidate cell can be transmitted to the serving cell and then the serving cell sends it to the UE in the cell switch command with the candidate cell index/indicator included.

Proposal 14:  For UE based TA measurement, i.e. Alt3. the reference signal which could be used to estimate the downlink reference timing difference between serving cell and candidate cell needs to be considered.

Proposal 15: For TA management in L1/L2 based mobility, support at least one of the candidate cells can be associated with one acquired TA/TAG other than the TA/TAG used for the serving cell. The maximal number of TA/TAG associated with candidate cell(s) depends on UE capability.

Proposal 16: Support up to two TA(s)/TAG(s) configuration for a candidate cell if it is configured as mDCI MTRP. i.e. Alt1 is supported.

Proposal 17: Consider whether TAT needs to be maintained at UE side.

References

- RP-222332: Revised WID on Further NR mobility enhancements, MediaTek, RAN #97-e meeting, September 12th-16th, 2022.

- RAN1 # 111 meeting, Chairman notes.

- 3GPP TS38.213 v17.1.0.

- 3GPP TS38.321 v16.4.0