3GPP TSG RAN WG1 #112		R1-2300671

Athens, Greece, February 27th – March 3rd, 2023

Source: 	CATT

Title:	Evaluation on AI/ML for CSI feedback enhancement

Agenda Item:	9.2.2.1

Document for:	Discussion and Decision

Introduction

In RAN#94-e meeting, AI/ML for NR air-interface was agreed and several objectives were approved in the SID [1]. In previous meetings, evaluation methodology and KPIs for AI/ML based CSI feedback enhancement were discussed, and several agreements had been achieved.

In this contribution, intermediate KPI for rank>1, CSI payload for AI/ML based evaluation, and AI/ML model settings for rank>1 are discussed. Evaluation results for spatial-frequency domain CSI compression using two-sided AI/ML model and CSI prediction are also provided.

Intermediate KPI

In RAN1#109-e, for the evaluation of the AI/ML based CSI feedback enhancement, the intermediate KPIs of GCS/SGCS and/or NMSE were taken as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI. One open issue is how to report GCS/SGCS for rank>1 cases if GCS/SGCS is adopted. The following three methods were provided in RAN1#109-e [2]:

- Method 1: Average over all layers

  - wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples.

- Method 2: Weighted average over all layers

  - Note: Companies to report the formula (e.g., whether normalization is applied for eigenvalues)

- Method 3: GCS/SGCS is separately calculated for each layer (e.g., for K layers, K GCS/SGCS values are derived respectively, and comparison is performed per layer)

In RAN1 #110 meeting, SGCS was adopted[3]. In RAN1#110b-e meeting, it was agreed that at least Method 3 was adopted, FFS whether additionally adopted a down-selected metric between Method 1 and Method 2[4]. For Method 2, the SGCS is calculated as follows:

- Method 2: Weighted average over all layers

SGCS=E1Ni=1NjKλijk=1Kλik||wijHwij ||||wij||  ||wij||2

where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij.

Evaluating intermediate KPI is to evaluate the performance of AI/ML models directly without applying the output of AI/ML model in a system/link level platform, which can save plenty of simulation efforts. However, it is desirable that the intermediate KPI can reflect the performance of the system, i.e., eventual KPI, when AI/ML model is applied. Therefore intermediate KPIs that are consistent with eventual KPIs preferred.

For a given rank in MIMO systems, the contributions of layers to the throughput are different. A layer corresponds to a larger eigenvalue is expected to have higher contribution than a layer corresponds to a smaller eigenvalue. Therefore, for the calculation method of SGCS, Method 2 is preferred than Method 1.

Proposal 1: For the evaluation of AI/ML based CSI feedback enhancement, as the intermediate KPI for rank>1 cases, if a down-selected metric between the following two methods is adopted, Method 2 is supported:

- Method 1: Average over all layers

- Method 2: Weighted average over all layers

SGCS=E1Ni=1NjKλijk=1Kλik||wijHwij ||||wij||  ||wij||2

  - where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij.

CSI payload for evaluation

For the eventual evaluation of AI/ML based CSI compression sub use cases, the throughput gain of AI/ML based feedback over Rel-16 Type II codebook based feedback at the same feedback overhead, or the overhead reduction of AI/ML based feedback over Rel-16 Type II codebook based feedback at the same throughput would be investigated. In order to reveal the performance gain of AI/ML based CSI feedback properly, the overhead for AI/ML based feedback and Rel-16 Type II codebook based feedback should be calculated properly.

There are several alternatives on calculating CSI feedback payload size. In RAN1 #111 meeting, the following two options were discussed[5]:

- Option 1: Payload size is calculated based on the maximum rank.

- Option 2: Payload size is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE.

  - The above-mentioned “CSI payload” is calculated as max allowed bits at the given rank

It is our view that for option 1, the mentioned “maximum rank” is the max allowed bits for the maximum rank. Option 1 is the feedback overhead metric adopted for Rel-16/Rel-17 CSI evaluations. For legacy codebook based CSI feedback, it can be easily obtained based on the configured parameters. For AI/ML based CSI feedback, since the payload for a given rank is always fixed, it also can be easily obtained. For option 1, the payload size of CSI feedback would not be changed with the actual CSI reporting, so the payload sizes for Rel-16 Type II codebook based feedback and AI/ML based CSI feedback respectively can be easily aligned.

There are two alternatives on CSI payload calculation for option 2:

- Option 2a: The “CSI payload” is calculated as each CSI reported payload with a given rank

- Option 2b: The “CSI payload” is calculated as max allowed bits at the given rank

Option 2a can be viewed as using average CSI overhead, i.e., the size of CSI payload is calculated as follows:

C=E1Ni=1NCi

where Ci is the overhead of reported CSI at resource unit i and N is the total number of CSI report resource units. E∙ denotes the average operation over multiple samples.

Due to the change of the number of non-zero coefficients, the bit sizes of CSI reports for a given rank can vary from slot to slot for Rel-16 Type II codebook based feedback. In order to calculate the average CSI overhead, the bit size for each CSI report should be calculated and accumulated.

For option 2b, the average CSI overhead can be calculated with the following formula:

C=j=1KRjCj

where Cj is the maximum allowed bits for rank j, K is the maximum rank of the UE, and Rj is the ratio of CSI reports with rank j among all CSI reports. For Rel-16 Type II codebook based feedback, option 2b can be viewed as a solution assuming the number of non-zero coefficients for each rank is always no less than the value calculated based on parameter K0 configured by gNB, i.e., the overhead of reported CSI is always equal to the maximum allowed bits for a given rank. The complexity of calculating option 2b for Rel-16 Type II codebook based feedback would be much lower than option 2a, since only the distribution of ranks needs to be counted instead of counting bit size for each CSI report.

A comparison of the three options are provided in Table 1. It can be seen that compared to other options, option 1 is simpler for payload size calculation and easier for aligning payload sizes for AI/ML based and Type II codebook based solution. Option 2b can achieve a compromise on complexity and alignment to actual payload size of CSI feedback.

Table 1: Comparison of options for CSI feedback payload size calculation

|  |  | Option 1 | Option 2a | Option 2b |
| --- | --- | --- | --- | --- |
| Complexity of statistics |  | Low | High | Medium |
| Close to the actual payload size of CSI feedback | Rel-16 Type II codebook based solution | Low | High | Medium |
|  | AI/ML based solution | Depend on model design, e.g. rank-specific, layer-common | High | Medium |
| Payload size alignment between AI/ML based solution and Rel-16 Type II codebook based solution |  | High | Low | Medium |

Proposal 2: Align the interpretation on payload size for AI/ML-based CSI compression and the legacy Type II codebook .

Proposal 3: For the CSI payload size calculation for the evaluation of AI/ML-based CSI compression and the legacy TypeII codebook based CSI feedback, one of the following two options is supported:

- Option 1(1st preferred): Payload size is calculated based on the maximum allowed bits for the maximum rank;

- Option 2b(2nd preferred): Payload size is calculated as the weighted average of CSI payload per rank, with the following calculation formula:

C=j=1KRjCj

  - where Cj is the maximum allowed bits for rank j, K is the maximum rank of the UE, and Rj is the ratio of CSI reports with rank j among all CSI reports.

AI/ML model settings for rank>1

For spatial-frequency domain CSI compression using two-sided AI model, AI/ML model settings for rank>1 has been discussed for several meetings, the main controversy is how to identify the options of schemes, i.e., how to judge whether an AI/ML model setting is layer common, layer specific, rank common or rank specific. For the previous classification methods, the procedures of model training and model inference are considered together. For example, in RAN1 #111 meeting, the following proposal was discussed [6]. For the proposal, when identify which option an AI/ML model setting belongs to, model training and model inference are considered together.

Proposal 3.2.3: For the evaluation of the AI/ML based CSI compression sub use cases with rank >=1, companies to report the specific option adopted for AI/ML model settings to adapt to ranks/layers. - Option 1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference. - FFS on the reported complexity and storage - FFS: input/output type - Option 2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference. - FFS: input/output type - Option 3 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference. - FFS on the reported complexity and storage - Note: input/output type is Precoding matrix - Companies to report the setting is layer specific and rank common (For a specific layer, the same model is applied for all rank values), or layer specific and rank specific (For a specific layer, different models are applied for different rank values) - Option 4 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference. - Note: input/output type is Precoding matrix - Companies to report whether the setting is layer common and rank common (A unified AI/ML model is applied for each layer to perform individual inference), or layer common and rank specific (For a specific layer, different models are applied for different rank values) - Other options not precluded.

For AI/ML models in AI/ML model settings with input/output type of precoding matrix, considering whether the input/output of the AI/ML models is of one layer or one rank, the following two types can be considered:

- Layer-level AI/ML model: the input/output of the AI/ML model is precoding matrix of one layer;

- Rank-level AI/ML model: the input/output of the AI/ML model is precoding matrix of one rank.

For an AI/ML model setting with rank-level AI/ML models with input/output type of precoding matrix, since at least the dimensions of AI/ML model input for different ranks are different, only rank specific AI/ML model can be used, i.e., separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference.

For AI/ML model settings with layer-level AI/ML models, it is hard to identify which option an AI/ML model setting belongs to for some specific cases. For example, for an AI/ML model setting, if a set of layer-level AI/ML models are trained with a mixed dataset of data from all layers and all ranks, and different AI/ML models in the set are applied to different rank and/or different layers, it does not belongs to any of the options in the proposal. To better classify AI/ML model settings, we suggest to classifying AI/ML model settings from views of model training and model inference respectively.

From the view of model training, the following four options can be considered:

- Option 1(layer common, rank common): A unified AI/ML model is trained with data from all layers of all ranks.

- Option 2(layer common, rank specific): Separated AI/ML models are trained per rank value with data from all layers of the rank.

- Option 3(layer specific, rank common): Separated AI/ML models are trained per layer value with data from all ranks.

- Option 4(layer specific, rank specific): Separated AI/ML models are trained per layer value per rank value.

Figure 1: AI/ML model setting clarification

Table 2: Comparison of options of AI/ML model settings from the view of training

| Options | Training dataset and number of AI/ML model/AI/ML model sets |
| --- | --- |
| Layer common，Rank common | One or a set of AI/ML models: Trained with mixed dataset of #1, #2, #3, …., #10. |
| Layer common，Rank specific | One or a set of AI/ML models for all layers of Rank 1: Trained with dataset of #1. One or a set of AI/ML models for all layers of Rank 2: Trained with mixed dataset of #2 and #5. One or a set of AI/ML models for all layers of Rank 3: Trained with mixed dataset of #3, #6 and #8. One or a set of AI/ML models for all layers of Rank 4: Trained with mixed dataset of #4, #7, #9 and #10. |
| Layer specific，Rank common | One or a set of AI/ML models for layer 1 for all ranks: Trained with mixed dataset of #1, #2, #3 and #4. One or a set of AI/ML models for layer 2 for all ranks: Trained with mixed dataset of #5, #6 and #7. One or a set of AI/ML models for layer 3 for all ranks: Trained with mixed dataset of #8, #9. One or a set of AI/ML models for layer 4 for Rank 4: Trained with dataset of #10. |
| Layer specific，Rank specific | One or a set of AI/ML models for layer 1 of Rank 1: Trained with dataset of #1. One or a set of AI/ML models for layer 1 of Rank 2: Trained with dataset of #2. One or a set of AI/ML models for layer 2 of Rank 2: Trained with dataset of #3. One or a set of AI/ML models for layer 1 of Rank 3: Trained with dataset of #4. One or a set of AI/ML models for layer 2 of Rank 3: Trained with dataset of #5. One or a set of AI/ML models for layer 3 of Rank 3: Trained with dataset of #6. One or a set of AI/ML models for layer 1 of Rank 4: Trained with dataset of #7. One or a set of AI/ML models for layer 2 of Rank 4: Trained with dataset of #8. One or a set of AI/ML models for layer 3 of Rank 4: Trained with dataset of #9. One or a set of AI/ML models for layer 4 of Rank 4: Trained with dataset of #10. |
| Note: With each dataset, one or a set of AI/ML models can be trained. For example, for an AI/ML model setting with (Layer common，Rank common), it is possible that a set of AI/ML models are trained with the same dataset. Different AI/ML models in the set can be with different payload size, or model backbone, etc. |  |

The comparisons of above options can be seen in Figure 1 and Table 2, wherein in Figure 1, each of the numbered blocks represents a dataset generated with precoding matrixes of its corresponding layer and rank. It can be seen that the number of AI/ML models and the dataset for training can be different for different options.

Similarly, from the view of model inference, AI/ML model settings can be classified into four options:

- Option 1(layer common, rank common): A unified AI/ML model is applied for all layer values and all rank values to perform inference.

- Option 2(layer common, rank specific): Separated AI/ML models are applied per rank value to perform individual inference, while the same AI/ML model is applied for the layers of a specific rank to perform inference.

- Option 3(layer specific, rank common): Separated AI/ML models are applied per layer value to perform individual inference, while the same AI/ML models are applied for the same layer value of all ranks to perform inference.

- Option 4(layer specific, rank specific): Separated AI/ML models are applied per layer value per rank value to perform individual inference.

The comparisons of above options can be seen in Figure 1 and Table 3, wherein in Figure 1, each of the numbered blocks represents an AI/ML model applied for the corresponding layer and rank in the inference phase.

Table 3: Comparison of options of AI/ML model settings from the view of inference

| Options | Applied AI/ML model(s) in the inference phase |
| --- | --- |
| Layer common，Rank common | Same AI/ML model is applied for any layer of any rank, i.e., model #1, model #2, model #3, …, model #10 are the same. |
| Layer common，Rank specific | An AI/ML model is applied for Rank 1: #1. An AI/ML model is applied for all layers of Rank 2, i.e., model #2 and model #5 are the same. An AI/ML model is applied for all layers of Rank 3, i.e., model #3, model #6 and model #8 are the same. An AI/ML model is applied for all layers of Rank 4, i.e., model #4, model #7, model #9 and model #10 are the same. |
| Layer specific，Rank common | An AI/ML model is applied for layer 1 of all ranks, i.e., model #1, model #2, model #3, model #4 are the same. An AI/ML model is applied for layer 2 of all ranks, i.e., model #5, model #6, model #7 are the same. An AI/ML model is applied for layer 3 of all ranks, i.e., model #8, model #9 are the same. An AI/ML model is applied for layer 4 of Rank 4: model #10. |
| Layer specific，Rank specific | An AI/ML model is applied for layer 1 of Rank 1: model #1. An AI/ML model is applied for layer 1 of Rank 2: model #2. An AI/ML model is applied for layer 2 of Rank 2: model #3. An AI/ML model is applied for layer 1 of Rank 3: model #4. An AI/ML model is applied for layer 2 of Rank 3: model #5. An AI/ML model is applied for layer 3 of Rank 3: model #6. An AI/ML model is applied for layer 1 of Rank 4: model #7. An AI/ML model is applied for layer 2 of Rank 4: model #8. An AI/ML model is applied for layer 3 of Rank 4: model #9. An AI/ML model is applied for layer 4 of Rank 4: model #10. |

For AI/ML model settings, if only one AI/ML model is trained with a dataset/mixed dataset as shown in Table 2, not all combinations of options of AI/ML model setting from the view of training and inference are available. For example, if the AI/ML models are trained in a rank common manner, rank specific AI/ML models for inference are impossible. However, if a set of AI/ML model is trained with a dataset/mixed dataset as shown in Table 2, all combinations of an option from the view of model training and an option from the view of model inference are possible. For example, if a set of AI/ML models are trained in a layer common, rank common manner, different AI/ML models from the set used for different layers and/or ranks can be possible.

Table 4 shows available combinations of options of AI/ML model setting from the view of training and inference if only one AI/ML model is trained with a dataset/mixed dataset as shown in Table 2. In Table 4, “Y” represents the combination is available and “N” represents the combination is unavailable.

Table 4: Combinations of options of AI/ML model setting from the view of training and inference

| Training\Inference | Layer common，Rank common | Layer common，Rank specific | Layer specific，Rank common | Layer specific，Rank specific |
| --- | --- | --- | --- | --- |
| Layer common，Rank common | Y | Y | Y | Y |
| Layer common，Rank specific | N | Y | N | Y |
| Layer specific，Rank common | N | N | Y | Y |
| Layer specific，Rank specific | N | N | N | Y |

Proposal 4: For AI/ML based CSI compression sub use cases with rank >=1, companies to report the specific option adopted for AI/ML model settings to adapt to ranks/layers from the view of model training and model inference respectively:

- For model training

  - Option 1(layer common, rank common): A unified AI/ML model is trained with data from all layers of all ranks.

  - Option 2(layer common, rank specific): Separated AI/ML models are trained per rank value with data from all layers of the rank.

  - Option 3(layer specific, rank common): Separated AI/ML models are trained per layer value with data from all ranks.

  - Option 4(layer specific, rank specific): Separated AI/ML models are trained per layer value per rank value.

- For model inference

  - Option 1(layer common, rank common): A unified AI/ML model is applied for all layer values and all rank values to perform inference.

  - Option 2(layer common, rank specific): Separated AI/ML models are applied per rank value to perform individual inference, while the same AI/ML model is applied for the layers of a specific rank to perform inference.

  - Option 3(layer specific, rank common): Separated AI/ML models are applied per layer value to perform individual inference, while the same AI/ML models are applied for the same layer value of all ranks to perform inference.

  - Option 4(layer specific, rank specific): Separated AI/ML models are applied per layer value per rank value to perform individual inference.

Simulation results

CSI compression in spatial-frequency domain

In this section, our simulation results on spatial-frequency domain CSI compression using two-sided AI/ML models are provided, with the simulation assumptions shown in Table 11 in Annex. Unless explicitly stated (e.g. for Section 5.1.4), the simulations assume that AI/ML model for CSI compression are jointly trained and the encoder and decoder are deployed in UE side and network side respectively.

AI/ML model description

In RAN1 #111 meeting, the following agreements were achieved on generalization/scalability over various configurations for CSI compression [5]:

Agreement For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input dimensions of CSI generation part (e.g., different bandwidths/frequency granularities, or different antenna ports), the generalization cases of are elaborated as follows - Case 1: The AI/ML model is trained based on training dataset from a fixed dimension X1 (e.g., a fixed bandwidth/frequency granularity, and/or number of antenna ports), and then the AI/ML model performs inference/test on a dataset from the same dimension X1. - Case 2: The AI/ML model is trained based on training dataset from a single dimension X1, and then the AI/ML model performs inference/test on a dataset from a different dimension X2. - Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of X1, X2,..., Xn, and then the AI/ML model performs inference/test on a single dataset subject to the dimension of X1, or X2,…, or Xn. - Note: For Case 2/3, the solutions to achieve the scalability between Xi and Xj, are reported by companies, including, e.g., pre-processing to angle-delay domain, padding, additional adaptation layer in AI/ML model, etc. - FFS the verification of fine-tuning - FFS other additional cases Agreement For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different output dimensions of CSI generation part (e.g., different generated CSI feedback dimensions), the generalization cases of are elaborated as follows - Case 1: The AI/ML model is trained based on training dataset from a fixed output dimension Y1 (e.g., a fixed CSI feedback dimension), and then the AI/ML model performs inference/test on a dataset from the same output dimension Y1. - Case 2: The AI/ML model is trained based on training dataset from a single output dimension Y1, and then the AI/ML model performs inference/test on a dataset from a different output dimension Y2. - Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of Y1, Y2,..., Yn, and then the AI/ML model performs inference/test on a single dataset of Y1, or Y2,…, or Yn. - Note: For Case 1/2/3, companies to report whether the output of the CSI generation part is before quantization or after quantization. - Note: For Case 2/3, the solutions to achieve the scalability between Yi and Yj, are reported by companies, including, e.g., truncation, additional adaptation layer in AI/ML model, etc. - FFS the verification of fine-tuning - FFS other additional cases Agreement For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input/output dimensions, companies to report which case(s) in the following are evaluated - Case 0 (benchmark for comparison): One CSI generation part with fixed input and output dimensions to 1 CSI reconstruction part with fixed input and output dimensions for each of the different input and/or output dimensions. - Case 1: One CSI generation part with scalable input and/or output dimensions to N>1 separate CSI reconstruction parts each with fixed and different output and/or input dimensions - Case 2: M>1 separate CSI generation parts each with fixed and different input and/or output dimensions to one CSI reconstruction part with scalable output and/or input dimensions - Case 3: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions

In this section, we provide a solution of Case 3 that can achieve the scalability over different input/output dimensions, based on adaptive layers. The solution can be used for various configurations, e.g., various sizes of CSI feedback payloads, various antenna port layouts, various bandwidths, etc.

Figure 2 illustrates the basic structure of the solution when it is used for CSI feedback payload scalability. For the ease of discussion, we call the model as payload-scalable AI/ML model. For the model, transformer based AI/ML model (two transformer encode blocks) is used in both encoder part (EN block) and decoder part (DE block), and fully-connected layers are used for adaptation layers (i.e. down-sampling (DS-x block) and up-sampling (US-x block)). A pair of DS-x block and US-x block corresponds to a payload configuration while EN block and DE block are shared among all payload configurations. The AI/ML model is trained based on training dataset by mixing datasets subject to various sizes of payloads, i.e., it is Case 3 in the above agreement in RAN1 #111 meeting for achieving the scalability over different output dimensions of CSI generation part. In the simulation, a 2-bit uniform quantization is used, and the output of the CSI generation part is before quantization. At the training phase, multi-task learning scheme is used and the loss function is the average SGCS over all payload configurations. At the inference phase, only one branch is activated according to the configured payload. The input data v is eigenvector base on SVD of channel matrix.

Figure 2: Basic structure of payload-scalable AI/ML model

To support variable ports, payload-scalable AI/ML model can be further extended with the above idea as shown in Figure 3 (named as SCsiNet). Fully connected layers are used for linear pre-transforming (LPT-x block) and linear transforming (LT-x block) for the purpose of unifying input/output dimensions and probability distribution of eigenvectors from different port numbers. Similar to payload-scalable AI/ML model, EN block and DE block are shared among all payload and port configurations. The input of SCsiNet is eigenvector from all layers, i.e. ignoring layer index. Note that the proposed SCsiNet can be also extended for other aspects such as bandwidth etc.

Figure 3: Basic structure of SCsiNet, scalable in payloads and ports

The SCsiNet can be viewed as a family of AI/ML models with shared core blocks, or an AI/ML model with scalability fulfilled by multiple additional adaptive layers.

The training parameters of SCsiNet are given in Table 5. The AI/ML model is trained based on training dataset by mixing datasets subject to various number of antenna ports and various size of payloads, i.e., it is Case 3 (i.e., A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions) in the agreement in RAN1#111 meeting. In the simulation, a 2-bit uniform quantization is used, and the output of the CSI generation part is before quantization. The FLOPs and size of AI/ML model for each payload and port configuration are provided in Table 6.

For each AI/ML model, the AI/ML model is trained for a specific payload and port with mixed eigenvectors from all 4 layers. In our simulations, ideal eigenvectors are used at the training phase for both encoder input and ground truth/label. At the inference phase, realistic eigenvectors are used for encoder inputs and SGCS is calculated with reconstructed eigenvectors and ideal eigenvectors.

Table 5: Parameters for AI/ML model training for CSI compression

| Parameter | Value |
| --- | --- |
| Size of training and validation dataset | 40Drops×570UEs×20Samples |
| Size of test dataset | 10Drops×570UEs×20Samples |
| Epoch | 200~300 |
| Batch size | 1024 |
| Learning rate | 10-5~10-3 |
| Optimizer | Adam |
| Loss function | SGCS |
| Input/output format | Eigenvector |
| Quantization | Uniform, 2 bits |

Table 6: FLOPs and size of SCsiNet

| Payload size(bits) |  | 20 | 40 | 60 | 80 | 100 | 120 | 140 | 160 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FLOPs of encoder (M) | 16ports | 9.8 | 9.84 | 9.87 | 9.9 | 9.93 | 9.96 | 9.96 | 10.02 |
|  | 32ports | 9.9 | 9.93 | 9.97 | 10 | 10 | 10.06 | 10.06 | 10.12 |
| FLOPs of decoder (M) | 16ports | 9.8 | 9.84 | 9.87 | 9.9 | 9.93 | 9.96 | 9.96 | 10.02 |
|  | 32ports | 9.9 | 9.93 | 9.96 | 10 | 10.03 | 10.06 | 10.06 | 10.12 |
| Payload (bits) |  | 180 | 200 | 220 | 240 | 260 | 280 | 300 | 320 |
| FLOPs of encoder (M) | 16ports | 10.1 | 10.08 | 10.11 | 10.14 | 10.17 | 10.21 | 10.24 | 10.27 |
|  | 32ports | 10.15 | 10.18 | 10.21 | 10.24 | 10.27 | 10.3 | 10.34 | 10.37 |
| FLOPs of decoder (M) | 16ports | 10.05 | 10.08 | 10.11 | 10.14 | 10.17 | 10.2 | 10.24 | 10.27 |
|  | 32ports | 10.15 | 10.18 | 10.21 | 10.24 | 10.27 | 10.3 | 10.33 | 10.36 |
| #Total parameters of Encoder (M) |  | 2.51 |  |  |  |  |  |  |  |
| #Total parameters of Decoder (M) |  | 2.52 |  |  |  |  |  |  |  |

Evaluation results for scalability over payloads and ports

Figure 4 provides simulation results for SCsiNet in terms of intermediate KPI (SGCS). The scalable SCsiNet provides results from payload = 20, 40 … 320 bits, for both 16 ports and 32 ports.

Figure 4: Intermediate KPI (SGCS) of SCsiNet

It can be seen from this simulation results that compared with a family of layer-common AI/ML models, SCsiNet can achieve a similar performance for all layers. Note that the total number of layer-common AI/ML models should be 2×16=32 to support 20/40/…320-bit payloads and 16/32 ports. This means that the size of SCsiNet is approximately 1/32 as that of the family of layer-common AI/ML models, since all branches of SCsiNet use common transformer blocks (i.e. EN, DE), and the size of transformer blocks is usually much bigger than other blocks (i.e. DS-x, US-x, LT-x, LPT-x) of the AI/ML model.

Observation 1: Compared with a family of layer-common AI/ML models, the scalable AI/ML model (SCsiNet) can achieve a similar performance and can significantly reduce storage memory and model transferring overhead.

Evaluation results for different ranks

For CSI feedback with rank>1, as discussed in Section 4, several options can be considered for AI/ML model training and AI/ML model inference. Different options from the view of AI/ML training have different impacts on training complexity and storage size of the AI/ML model, and the different options from the view of AI/ML inference can have different spec impacts.

For example, if layer common, rank common model setting is adopted at the inference phase, since the same AI/ML model is used for all layers and all ranks, the overhead of CSI feedback is proportional to the number of layers. As have been pointed out in Section 2, in real deployment, the gain of increasing precoding accuracy for high ranks would be much lower than that for low ranks, and the probability of scheduling UEs with high ranks would be lower than that for low ranks. Therefore allocating much higher payloads for high ranks than low ranks is not a good idea. For DL Type II codebook based CSI feedback in NR systems, the overheads of PMI feedback for rank 3, 4 are comparable to that for rank 2. For AI/ML based CSI feedback, the overhead of PMI feedback for rank 3, 4 are also not expected to be much larger than rank 2. Therefore layer specific AI/ML model should be considered.

Proposal 5: For AI/ML based CSI feedback, the overheads of CSI feedback for rank 3 and rank 4 are expected to be comparable to rank 2.

Figure 5, Figure 6, Figure 7 and Figure 8 provide simulation results of 16 ports and 32 ports respectively for rank 1, rank 2, rank 3 and rank 4 based on the SCsiNet proposed in section 0. In the simulation, six CSI feedback payload configurations are considered for 16 ports and 32 ports. The bit distributions among 4 layers are shown in Table 7. The payload of rank 1 is about half of the payload of rank 2/3/4. In the simulation, rank-adaptive scheduling is considered. It can be seen from the simulation results that compared to Rel-16 Type II codebook based CSI feedback, obvious performance gain can be achieved by CSI feedback with the proposed scalable AI/ML model for all ranks:

- Under the same CSI feedback payload, SGCS can be improved by 0.02~0.1 for rank 1, 0.03~0.17 for rank 2, 0.03~0.22 for rank 3 and 0.03~0.18 for rank 4;

- Under the same SGCS, payload can be saved by 40%~60% for rank 1, 50%~60% for rank 2, 30%~60% for rank 3 and 50%~60% for rank 4.

Observation 2: Compared to Rel-16 Type II codebook based CSI feedback, obvious performance gain can be achieved by CSI feedback with proposed scalable AI/ML model for rank=1, 2, 3, 4:

- SGCS can be improved by 0.02~0.22 under the same CSI feedback payload;

- Payload can be saved by 30%~60% bits under the same SGCS.

Table 7: Payload distribution among 4 layers

| Payload size(bits) | Rank1 | Rank2 |  |  | Rank3 |  |  |  | Rank4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Layer1 | Layer1 | Layer2 | Total | Layer1 | Layer2 | Layer3 | Total | Layer1 | Layer2 | Layer3 | Layer4 | Total |
| Config1 | 40 | 60 | 20 | 80 | 40 | 20 | 20 | 80 | 40 | 20 | 20 | 20 | 100 |
| Config2 | 60 | 80 | 40 | 120 | 60 | 40 | 20 | 120 | 60 | 20 | 20 | 40 | 140 |
| Config3 | 80 | 100 | 60 | 160 | 80 | 40 | 40 | 160 | 80 | 40 | 20 | 40 | 180 |
| Config4 | 120 | 160 | 80 | 240 | 100 | 80 | 60 | 240 | 120 | 60 | 40 | 40 | 260 |
| Config5 | 160 | 220 | 100 | 320 | 160 | 120 | 80 | 360 | 160 | 100 | 60 | 60 | 380 |
| Config6 | 240 | 320 | 140 | 460 | 180 | 140 | 120 | 440 | 180 | 140 | 80 | 60 | 460 |

Figure 5: Intermediate results of SCsiNet for rank=1

Figure 6: Intermediate results of SCsiNet for rank=2

Figure 7: Intermediate results of SCsiNet for rank=3

Figure 8: Intermediate results of SCsiNet for rank=4

Based on the proposed SCsiNet, the SLS evaluation results for 16 ports and 32 ports are provided in Figure 9. Rank adaption and FTP model-1 are applied in the simulation. The payload is calculated based on the maximum allowed bits for the maximum rank. It can be seen from the simulation results that compared to Rel-16 Type II codebook based CSI feedback, significant performance gain can be achieved by AI/ML based CSI feedback:

- The throughput can be improved by 3%~11% under the same CSI feedback payload.

Figure 9: Eventual simulation results for rank adaption

Observation 3: Compared to Rel-16 Type II codebook based CSI feedback, 3%~11% throughput improvement under the same CSI feedback payload can be achieved by proposed scalable AI/ML model.

Evaluation results for separate training

For spatial-frequency domain CSI compression using two-sided AI/ML model, there are several challenges for joint training of AI/ML models: If the two-sided AI/ML model is trained at one side and a part of AI/ML model (i.e. encoder or decoder) is transferred to the other side through air interface: 1) Transferring AI/ML model would cause proprietary/privacy problem. 2) Transferred AI/ML model may not match the hardware platform of the other side well. Then low operating efficiency, high power consumption and large operating delay may be incurred. 3) If the AI/ML model is transferred through air interface, a common model representation format (MRF) would be needed to exchange interpretable languages between network and UE, which needs more study within 3GPP at present. If the two-sided model at network side and UE side is joint training respectively, the BP and FP need to be exchanged between network side and UE side, which will increase the DL and UL resource overhead.

Training AI/ML models in two sides separately is an attractive solution that can avoid above problems. In RAN1 #110 meeting, it was clarified that separate training included sequential training starting with UE side training, or sequential training starting with network side training, or parallel training at UE and network [3]. In RAN1 #110bis-e meeting, the following two options of sequential training were concluded to be considered for evaluation [4]:

- Option 1: Sequential training starting with network side (network-first training). For the evaluation, the following procedure is considered:

  - Step1: network side trains the network side CSI generation part (which is not used for inference) and the network side CSI reconstruction part jointly

  - Step2: After network side training is finished, network side shares UE side with a set of information (e.g., dataset) that is used by the UE side to be able to train the UE side CSI generation part

  - Step3: UE side trains the UE side CSI generation part based on the received set of information

- Option 2: Sequential training starting with UE side (UE-first training). For the evaluation, the following procedure is considered:

  - Step1: UE side trains the UE side CSI generation part and the UE side CSI reconstruction part (which is not used for inference) jointly

  - Step2: After UE side training is finished, UE side shares network side with a set of information (e.g., dataset) that is used by the network side to be able to train the CSI reconstruction part

  - Step3: network side trains the network side CSI reconstruction part based on the received set of information

In RAN1 #111 meeting, the following agreement was further achieved on evaluation for separate training [5]:

Agreement For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following evaluation cases for sequential training are considered for multi-vendors - Case 1 (baseline): Type 3 training between one NW part model and one UE part model - Note 1: Case 1 can be naturally applied to the NW-first training case where 1 NW part model to M>1 separate UE part models - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training UE part model is the same or a subset of the dataset for training NW part model - Note 2: Case 1 can be naturally applied to the UE-first training case where 1 UE part model to N>1 separate NW part models - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training NW part model is the same or a subset of the dataset for training UE part model - Companies to report the AI/ML structures for the combination(s) of UE part model and NW part model, which can be the same or different - FFS: different quantization methods between NW side and UE side - Case 2: For UE-first training, Type 3 training between one NW part model and M>1 separate UE part models - Note: Case 2 can be also applied to the M>1 UE part models to N>1 NW part models - Companies to report the AI/ML structures for the M>1 UE part models and the NW part model - Companies to report the dataset used at UE part models, e.g., same or different dataset(s) among M UE part models - Case 3: For NW-first training, Type 3 training between one UE part model and N>1 separate NW part models - Note: Case 3 can be also applied to the N>1 NW part models to M>1 UE part models - Companies to report the AI/ML structures for the UE part model and the N>1 NW part models - Companies to report the dataset used at NW part models, e.g., same or different dataset(s) among N NW part models - FFS: whether/how to report overhead of dataset

Parallel training means the AI/ML model at the UE side and the AI/ML model at the network side are trained separately, with no distinguishable sequential order. In other words, parallel training is order-agnostic training. For example, UE side and network side can use a common dataset from Mobile Communication Open Dataset [7], without knowing the dataset is UE-first or NW-first. One example of parallel training is as follows:

  - UE side trains the UE side CSI generation part with dataset #A;

  - Network side trains the network side CSI reconstruction part based on dataset #B;

  - Dataset #A and dataset #B can be the same or not.

In this section, intermediate evaluations of separate training for rank 1 are provided, with the following 2 cases considered:

- Case 0 in the above agreement in RAN1 #111 meeting;

- Parallel training. For ease of discussion, we denote the case for parallel training as Case 4.

For Case 0, the following sub cases are considered:

- Case 0-1: NW-first sequential training,

  - Case 0-1-1: One UE side CSI generation part + one network side CSI reconstruction part, both the UE side CSI generation part and the network side CSI reconstruction part are transformer based model with 6 layers;

  - Case 0-1-2: One UE side CSI generation part + one network side CSI reconstruction part, the network side CSI reconstruction part is a transformer based model with 6 layers, and the UE side CSI generation part is a transformer based model with 5 layers;

  - Case 0-1-3: One UE side CSI generation part + one network side CSI reconstruction part, the network side CSI reconstruction part is a transformer based model, and the UE side CSI generation part is a ResNet based model.

- Case 0-2: UE-first sequential training

  - Case 0-2-1: One UE side CSI generation part + one network side CSI reconstruction part, both the UE side CSI generation part and the network side CSI reconstruction part are transformer based model with 6 layers;

  - Case 0-2-2: One UE side CSI generation part + one network side CSI reconstruction  part, the network side CSI reconstruction part is a transformer based model with 5 layers, and the UE side CSI generation part is a transformer based model with 6 layers;

  - Case 0-2-3: One UE side CSI generation part + one network side CSI reconstruction part, the UE side CSI generation part is a transformer based model, and the network side CSI reconstruction part is a ResNet based model.

For Case 4, we consider a parallel training with both the UE side CSI generation part and the network side CSI reconstruction part trained with the same dataset of {Channel, target CSI}, where “channel” is used as the input, and “target CSI” is used as output for the training of the UE side CSI generation part; and “target CSI” is used as the input, and “channel” is used as output for the training of the network side CSI reconstruction part. The following sub cases are considered:

  - Case 4-1: One UE side CSI generation part + one network side CSI reconstruction part, both the UE side CSI generation part and the network side CSI reconstruction part are transformer based model with 6 layers ;

  - Case 4-2: One UE side CSI generation part + one network side CSI reconstruction part, the network side CSI reconstruction part is a transformer based model with 6 layers, and the UE side CSI generation part is a transformer based model with 5 layers;

  - Case 4-3: One UE side CSI generation part + one network side CSI reconstruction part, the network side CSI reconstruction part is a transformer based model, and the UE side CSI generation part is a ResNet based model;

For the evaluation for the three cases, eigenvector is adopted as the input of the CSI generation part and the output of the CSI reconstruction part.

The simulation results on separate training are provided in Table 8. For reference, simulation result of joint training based on transformer structure is also provided. For sequential training, we also provided simulation results with dataset in step 2 which is the only half size of dataset in step 1. It can be seen from the simulation results that:

- For separate training, for the cases with one UE side CSI generation parts and one network side CSI reconstruction part, when the network side CSI reconstruction part and the UE side CSI generation part use aligned AI/ML model structure, similar performance can be achieved by separate training and joint training.

- For separate training, for the cases with one UE side CSI generation part and one network side CSI reconstruction part, when the network side CSI reconstruction part and the UE side CSI generation part use the same AI/ML model backbone and different number of layers, compared to joint training, there is minor SGCS degradation(<0.5%) of separate training

- For separate training, for the cases with one UE side CSI generation part and one network side CSI reconstruction part, when the network side CSI reconstruction part and the UE side CSI generation part use different AI/ML model backbones, compared to joint training, some SGCS degradation of separate training can be obtained (2% ~ 6.6%).

- For separate training, for the cases with one UE side CSI generation part and one network side CSI reconstruction part, compared to training the AI/ML model in both sides with the same dataset, minor performance loss can be seen for training the AI/ML model in one side with a dataset of a half size of the dataset for training the AI/ML model in the other side.

- For separate training, similar performance can be achieved by parallel training and sequential training.

Table 8: SGCS for separate training

| CSI feedback payload size(bits) |  | Dataset size: (the NW part model, the UE part model) | 49 | 87 | 242 |
| --- | --- | --- | --- | --- | --- |
| Joint training |  | 360K | 0.681 | 0.720 | 0.816 |
| Case 0-1: Sequential network-first training | Case 0-1-1 | (360K, 360K) | 0.679 | 0.719 | 0.814 |
|  | Case 0-1-2 | (360K, 360K) | 0.676 | 0.716 | 0.814 |
|  | Case 0-1-2 | (360K, 180K) | 0.675 | 0.716 | 0.812 |
|  | Case 0-1-3 | (360K, 360K) | 0.668 | 0.700 | 0.767 |
| Case 0-2: Sequential UE-first training | Case 0-2-1 | (360K, 360K) | 0.680 | 0.720 | 0.815 |
|  | Case 0-2-2 | (360K, 360K) | 0.674 | 0.716 | 0.813 |
|  | Case 0-2-2 | (180K, 360K) | 0.672 | 0.714 | 0.812 |
|  | Case 0-2-3 | (360K, 360K) | 0.667 | 0.698 | 0.774 |
| Case 4: Parallel training | Case 4-1 | (360K, 360K) | 0.680 | 0.720 | 0.819 |
|  | Case 4-2 | (360K, 360K) | 0.677 | 0.718 | 0.813 |
|  | Case 4-3 | (360K, 360K) | 0.667 | 0.687 | 0.762 |

Observation 4: For separate training for AI/ML based CSI compression, compared to joint training,

- Similar performance can be achieved when the UE side CSI generation part and network side CSI reconstruction part use aligned AI/ML model structure (both use transformer based AI/ML model with 6 layers);

- Performance loss is tiny when both the UE side CSI generation part and network side CSI reconstruction part use transformer based AI/ML model, but with different number of layers;

- Performance loss can be obtained (2% ~ 6.6%) when one of the UE side CSI generation part and network side CSI reconstruction part uses transformer based AI/ML model and the other one uses ResNet based AI/ML model.

Observation 5: For sequential separate training, compared to dataset in step 2 has the same size as dataset in step 1, minor performance loss can be seen for dataset CSI reconstruction part has half size of dataset in step 1.

Observation 6: For separate training, similar performance can be achieved by parallel training and sequential training.

Proposal 6: For separate training, parallel training is supported for further studied and evaluation.

Evaluation results for generalization

For spatial-frequency domain CSI compression using two-sided AI model, evaluations on generalization of the AI/ML model for scenarios of UMi, UMa and InH are provided in Table 9. In the simulation, the following cases are considered:

- Case 1: The training dataset for AI/ML model is obtained from scenario #A, and the inference is performed in scenario #A;

- Case 2: The training dataset for AI/ML model is obtained from scenario #A, and the inference is performed in scenario #B;

- Case 3: The training dataset for AI/ML model is obtained from scenario #A and scenario #B (i.e. mixed dataset of scenario #A and scenario #B), and the inference is performed in scenario #A/ scenario #B.

For simplicity, only rank=1 is considered in this simulation.

Table 9: SGCS for generalization

|  |  | Scenario for inference |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | UMa |  |  | UMi |  |  | InH |  |  |
| CSI feedback payload (bits) |  | 49 | 87 | 242 | 49 | 87 | 242 | 49 | 87 | 242 |
| Scenarios for generating dataset for training | UMa | 0.681 | 0.720 | 0.816 | 0.615 | 0.669 | 0.800 | 0.880 | 0.895 | 0.903 |
|  | UMi | 0.685 | 0.733 | 0.834 | 0.646 | 0.706 | 0.840 | 0.880 | 0.897 | 0.914 |
|  | InH | 0.584 | 0.613 | 0.645 | 0.525 | 0.547 | 0.573 | 0.841 | 0.861 | 0.904 |
|  | Mixed UMa & InH(9:1) | 0.664 | 0.701 | 0.805 | / | / | / | 0.881 | 0.900 | 0.923 |
|  | Mixed UMa & InH(8:2) | 0.654 | 0.695 | 0.794 | / | / | / | 0.883 | 0.899 | 0.922 |
|  | Mixed UMa & InH(7:3) | 0.648 | 0.690 | 0.787 | / | / | / | 0.884 | 0.901 | 0.925 |
|  | Mixed UMa & InH(6:4) | 0.581 | 0.628 | 0.783 | / | / | / | 0.880 | 0.900 | 0.925 |

Based on the simulation results, it can be seen that:

- For applying AI/ML model in UMa/UMi, the SGCS difference between training the AI/ML model with dataset of UMa and training the AI/ML model with dataset of UMi is less than 5%.

- For applying AI/ML model in InH, the AI/ML model trained based on a dataset collected in UMa/UMi slightly outperforms the AI/ML model trained based on a dataset collected in InH.

- For applying AI/ML model in UMa, compared to the AI/ML model trained in UMa, the SGCS degradation of the AI/ML model trained in InH can be 14.9%~21%. Training the AI/ML model with mixed data of UMa and InH can alleviate the performance loss.

Observation 7: For the generalization of AI/ML based CSI feedback, the following is observed:

- For applying AI/ML model in UMa/UMi, the performance difference between training the AI/ML model with dataset of UMa and training the AI/ML model with dataset of UMi is small.

- For applying AI/ML model in InH, the AI/ML model trained based on a dataset collected in UMa/UMi slightly outperforms the AI/ML model trained based on a dataset collected in InH.

- For applying AI/ML model in UMa, compared to the AI/ML model trained in UMa, significantly performance loss can be seen by the AI/ML model trained in InH. Training the AI/ML model with mixed data of UMa and InH can alleviate the performance gap.

CSI prediction in time domain

In this section, we provide our initial evaluation results on AI/ML based CSI prediction.

AI/ML model description

The CSI prediction AI /ML model is constructed by ConvLSTM cells. The structure of ConvLSTM cell is shown in Figure 10:

Figure 10: Structure of ConvLSTM cell

Among the structure, σ and tanh represent sigmoid function and tanh function respectively. Ct and ht represents the short-term status and long-term status in the sequence. xt represents the input at time t, and similarly, ft，it，gt, and ot represents the output of classical structure forgetting gate, input gate (memory gate), update gate and output gate in LSTM, respectively. Forgetting gate determines how much long-term state Ct-1 is left based on the input xt from the current moment and the hidden state ht-1 from the previous moment. Update gate uses the input xt from the current moment and the hidden state ht-1 from the previous moment to update the long-term state Ct-1. Input gate determines how many update gate output gt to update the long-term state Ct-1 according to the input xt from the current moment and the hidden state ht-1 from the previous moment. Output gate determines how many Ct to update ht according to the input xt from the current time and the hidden state ht-1 from the previous time.

The ConvLSTM model contains three ConvLSTM cells. It introduces two-dimensional convolution to capture the information of subbands and ports, and uses the output of convolution as the input of the forgetting gate cc_f, input gate cc_i, update gate cc_g and output gate cc_o. Then these values are used to calculate ft，it，gt and ot. Then the long and short term status Ct and ht are updated. The overall structure is shown in Figure 11.

Figure 11: AI/ML based CSI prediction model

The training parameters are shown in Table 10. In our design, we use the CSI in past three moments (i.e. t-2, t-1, t) to predict the CSI in one moment in the future (i.e. t+1). To be more specific, both the input and output are in the format of eigenvectors. During training phase, ideal eigenvectors are used for both model input and model output. At the inference phase, realistic eigenvector is used for model input. The intermediate KPI, i.e. SGCS/NMSE, is calculated based on predicted eigenvectors and ideal eigenvectors.

Table 10: Parameters for AI/ML model training for CSI prediction

| Parameter | Value |
| --- | --- |
| Size of training and validation dataset | 40Drops×570UEs×17Samples |
| Size of test dataset | 10Drops×570UEs×17Samples |
| Epoch | 300 |
| Batch size | 512 |
| Learning rate | 10-5~10-3 |
| Optimizer | Adam |
| Loss function | SGCS |
| Input/output format | Eigenvector |

Evaluation results

For comparison, simple conventional prediction method, i.e. using nearest historical CSI to predict the CSI in next moment, is also evaluated. The simulation results are shown in Table 11. Both SGCS and NMSE are provided for comparison.

Table 11: Intermediate KPI (SGCS/NMSE) of CSI prediction

|  | AI/ML prediction | Nearest historical CSI |
| --- | --- | --- |
| SGCS | 0.8675 | 0.7781 |
| NMSE | -27.9 dB | -5.7 dB |

It can be seen from Table 11 that AI/ML based CSI prediction can obtain better prediction performance, compared to conventional prediction method using nearest historical CSI.

Observation 8: AI/ML based CSI prediction outperforms the baseline using nearest historical CSI.

Conclusions

In this contribution, we provide our views on evaluation and KPIs for AI/ML based CSI feedback. We have the following observations and proposals:

Observation 1: Compared with a family of layer-common AI/ML models, the scalable AI/ML model (SCsiNet) can achieve a similar performance and can significantly reduce storage memory and model transferring overhead.

Observation 2: Compared to Rel-16 Type II codebook based CSI feedback, obvious performance gain can be achieved by CSI feedback with proposed scalable AI/ML model for rank=1, 2, 3, 4:

- SGCS can be improved by 0.02~0.22 under the same CSI feedback payload;

- Payload can be saved by 30%~60% bits under the same SGCS.

Observation 3: Compared to Rel-16 Type II codebook based CSI feedback, 3%~11% throughput improvement under the same CSI feedback payload can be achieved by proposed scalable AI/ML model.

Observation 4: For separate training for AI/ML based CSI compression, compared to joint training,

- Similar performance can be achieved when the UE side CSI generation part and network side CSI reconstruction part use aligned AI/ML model structure (both use transformer based AI/ML model with 6 layers);

- Performance loss is tiny when both the UE side CSI generation part and network side CSI reconstruction part use transformer based AI/ML model, but with different number of layers;

- Performance loss can be obtained (2% ~ 6.6%) when one of the UE side CSI generation part and network side CSI reconstruction part uses transformer based AI/ML model and the other one uses ResNet based AI/ML model.

Observation 5: For sequential separate training, compared to dataset in step 2 has the same size as dataset in step 1, minor performance loss can be seen for dataset CSI reconstruction part has half size of dataset in step 1.

Observation 6: For separate training, similar performance can be achieved by parallel training and sequential training.

Observation 7: For the generalization of AI/ML based CSI feedback, the following is observed:

- For applying AI/ML model in UMa/UMi, the performance difference between training the AI/ML model with dataset of UMa and training the AI/ML model with dataset of UMi is small.

- For applying AI/ML model in InH, the AI/ML model trained based on a dataset collected in UMa/UMi slightly outperforms the AI/ML model trained based on a dataset collected in InH.

- For applying AI/ML model in UMa, compared to the AI/ML model trained in UMa, significantly performance loss can be seen by the AI/ML model trained in InH. Training the AI/ML model with mixed data of UMa and InH can alleviate the performance gap.

Observation 8: AI/ML based CSI prediction outperforms the baseline using nearest historical CSI.

Proposal 1: For the evaluation of AI/ML based CSI feedback enhancement, as the intermediate KPI for rank>1 cases, if a down-selected metric between the following two methods is adopted, Method 2 is supported:

- Method 1: Average over all layers

- Method 2: Weighted average over all layers

SGCS=E1Ni=1NjKλijk=1Kλik||wijHwij ||||wij||  ||wij||2

  - where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij.

Proposal 2: Align the interpretation on payload size for AI/ML-based CSI compression and the legacy Type II codebook .

Proposal 3: For the CSI payload size calculation for the evaluation of AI/ML-based CSI compression and the legacy TypeII codebook based CSI feedback, one of the following two options is supported:

- Option 1(1st preferred): Payload size is calculated based on the maximum allowed bits for the maximum rank;

- Option 2b(2nd preferred): Payload size is calculated as the weighted average of CSI payload per rank, with the following calculation formula:

C=j=1KRjCj

  - where Cj is the maximum allowed bits for rank j, K is the maximum rank of the UE, and Rj is the ratio of CSI reports with rank j among all CSI reports.

Proposal 4: For AI/ML based CSI compression sub use cases with rank >=1, companies to report the specific option adopted for AI/ML model settings to adapt to ranks/layers from the view of model training and model inference respectively:

- For model training

  - Option 1(layer common, rank common): A unified AI/ML model is trained with data from all layers of all ranks.

  - Option 2(layer common, rank specific): Separated AI/ML models are trained per rank value with data from all layers of the rank.

  - Option 3(layer specific, rank common): Separated AI/ML models are trained per layer value with data from all ranks.

  - Option 4(layer specific, rank specific): Separated AI/ML models are trained per layer value per rank value.

- For model inference

  - Option 1(layer common, rank common): A unified AI/ML model is applied for all layer values and all rank values to perform inference.

  - Option 2(layer common, rank specific): Separated AI/ML models are applied per rank value to perform individual inference, while the same AI/ML model is applied for the layers of a specific rank to perform inference.

  - Option 3(layer specific, rank common): Separated AI/ML models are applied per layer value to perform individual inference, while the same AI/ML models are applied for the same layer value of all ranks to perform inference.

  - Option 4(layer specific, rank specific): Separated AI/ML models are applied per layer value per rank value to perform individual inference.

Proposal 5: For AI/ML based CSI feedback, the overheads of CSI feedback for rank 3 and rank 4 are expected to be comparable to rank 2.

Proposal 6: For separate training, parallel training is supported for further studied and evaluation.

References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94-e, December 6th – 17th, 2021.

- Chairman note for RAN1#109-e meeting, Chair, RAN1#109, May 9th – 20th, 2022.

- Chairman note for RAN1#110 meeting, Chair, RAN1#110, Toulouse, France, August 22nd – 26th, 2022.

- Chairman note for RAN1#110bis-e meeting, Chair, RAN1#110bis, October 10th – 19th, 2022.

- Chairman note for RAN1#111 meeting, Chair, RAN1#111, Toulouse, France, November 14th – 18th, 2022.

- R1-2212966, Summary#6 for CSI evaluation of [111-R18-AI/ML], Moderator (Huawei), RAN1#111, Toulouse, France, November 14th – 18th, 2022.

- https://www.mobileai-dataset.com/html/default/yingwen/DateSet/index.html?index=1, Mobile Communication Open Dataset, CAICT.

Annex

Table 12: Simulation assumptions for CSI compression

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Scenario |  | Dense Urban (Macro only), UMi, InH |
| Frequency Range |  | FR1 only |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz |
| Simulation bandwidth |  | 10MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| CSI feedback |  | CSI feedback periodicity: 5 ms, Scheduling delay: 4 ms |
| Traffic model |  | Full buffer, FTP |
| Traffic load (Resource utilization) |  | ~20% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Ideal |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput, SGCS. |
| Baseline for performance evaluation |  | R16 Type II codebook |

Table 13: Simulation assumptions for CSI prediction

Same as Table 12 except for the following rows:

| Parameter | Value |
| --- | --- |
| Evaluation Metric | SGCS, NMSE. |
| Baseline for performance evaluation | Nearest historical CSI |