3GPP TSG RAN WG1 #112                                              R1-2300673

Athens, Greece, February 27th – March 03rd, 2023

Source: 	CATT

Title:	Evaluation methodology and results on AI/ML for beam management

Agenda Item:	9.2.3.1

Document for:	Discussion and Decision

- Introduction

In RAN1#111 meeting, the following agreements were made for evaluation on AI/ML for spatial-domain beam prediction and time-domain prediction [2]:

Agreement The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Note: Companies to report the ratio for dataset mixing - Note: number of the multiple scenarios/configurations can be larger than two - FFS the detailed set of scenarios/configurations - The following case for generalization verification, can be optionally considered by companies: - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance - FFS: Investigate of the feasibility the fine-tuning on the UE/Network side Agreement - For the evaluation of the overhead for BM-Case1, adoption the following metrics: - RS overhead reduction, - Option 1: RS OH reduction%=1-NM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML - where M is the total number of beams (pairs) to be predicted - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Companies report the assumption on additional measurements Agreement - Companies report the pattern of Set B. - Further study the performance with different patterns of set B(s) for fixed Set B (Option 1) and different pre-configured/pre-known patterns of Set B(s) (Option 2A and 2B). Agreement For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, additionally considering - Various Set B of beam(pairs) Agreement At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable - Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample - Option 2: Measurements of specific Rx beam(s) - Option 2a: Measurements of specific Rx beam(s) per model input sample - Option 2b: Measurements of specific Rx beam(s) for all model input sample - FFS how to select the specific Rx beam(s) - Option 3: Measurements of random Rx beam(s) per model input sample - Other options are not precluded and can be reported by companies. Agreement - For generalization performance verification, consider the following - Scenarios - Various deployment scenarios, - e.g., UMa, UMi and others, - e.g., 200m ISD or 500m ISD and others - e.g., same deployment, different cells with different configuration/assumption - e.g., gNB height and UE height - FFS: e.g., Carrier frequencies - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others - Various UE mobility, - e.g., 3km/h, 30km/h, 60km/h and others - Configurations (parameters and settings) - Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions) - Various gNB settings, e.g., DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions) - Various Set B of beam (pairs) - T1 for measurement /T2 for prediction for BM-Case2 - Other scenarios/configurations(parameters and settings) are not precluded and can be reported by companies. Agreement - For the evaluation of the overhead for BM-Case2, adoption the following metrics: - RS overhead reduction, - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme - Companies report the assumption on additional measurements - FFS: Option 3: RS OH reduction%=1-NLM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance - where M is the total number of beams (pairs) to be predicted for each time instance - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction - Companies report the assumption on T1 and T2 patterns - Other options are not precluded and can be reported by companies.

In this contribution, evaluation methodology and KPIs for AI/ML based beam management enhancement are discussed. Simulation results for beam pair and Tx beam prediction are also provided.

- Discussion

Based on the RAN1#110bis e-meeting agreements, for BM-Case1, there are two alternatives and for each alternative, DL Tx beam, DL Rx beam and beam pair prediction can be further studied [1].

Based on the RAN1#111 meeting agreements as follow, for BM-Case1, the selection of pre-configuration pattern and Rx beam can be further studied [2].

Agreement - Companies report the pattern of Set B. - Further study the performance with different patterns of set B(s) for fixed Set B (Option 1) and different pre-configured/pre-known patterns of Set B(s) (Option 2A and 2B). Agreement At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable - Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample - Option 2: Measurements of specific Rx beam(s) - Option 2a: Measurements of specific Rx beam(s) per model input sample - Option 2b: Measurements of specific Rx beam(s) for all model input sample - FFS how to select the specific Rx beam(s) - Option 3: Measurements of random Rx beam(s) per model input sample - Other options are not precluded and can be reported by companies.

In this contribution, we focus on the evaluation of Set B as a subset of Set A, and evaluate the performance of pre-configured pattern and Rx beam selection. Specifically, we consider the performance of training and inference using the same and different pre-configured patterns. We also investigate the case that Tx beam of each sample corresponds to an optimal Rx beam, each sample randomly corresponds to an Rx beam, and all samples use an optimal Rx beam. In the evaluation, we focus on the DL Tx beam prediction and beam pair prediction, where RSRP and beam ID are predicted respectively.

  - Simulation Assumptions

Based on the agreements in the previous meetings [1], the Dense Urban (macro-layer only, TR 38.913) scenario is the basic one for dataset generation and performance evaluation for AI/ML based beam management. According to the simulation assumptions for this scenario for SLS, our simulation assumptions are shown in Table 36 in Annex.

In our simulation for beam pair prediction, Set A contains 128 beam pairs, including 32 gNB DL Tx beams and 4 UE DL Rx beams, while Set B contains 32 beam pairs, including 8 selected gNB DL Tx beams and 4 UE DL Rx beams. During the last meeting, two options (Option A and Option B) were agreed for the definition of Top-1 genie-aided Tx-Rx beam pair. In our simulation, the Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams is used to obtain the Top-1 genie-aided Tx-Rx beam pair.

Proposal 1: For DL Tx-Rx beam pair prediction, Option A is selected for the definition of Top-1 genie-aided Tx beam, i.e., the Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams.

For DL Tx beam prediction, Set A has 32 Tx beams and Set B has 8 Tx beams. The DL Rx beam of UE is fixed with one DL Rx beam selected from 4 DL Rx beams, and the second beam of 4 DL Rx beams is selected as the specific Rx beam. During the last meeting, Option A and Option B were agreed for the definition of Top-1 genie-aided Tx beam. In our simulation, the Tx beam that results in the largest L1-RSRP over all Tx beams with different Rx beam(s) is selected as the Top-1 genie-aided Tx beam. The selection of Rx beam can be done in four ways: each sample corresponds to an optimal Rx beam, each Tx beam of each sample corresponds to an optimal Rx beam, each sample randomly corresponds to an Rx beam, and all samples use an optimal Rx beam.

Proposal 2: For DL Tx beam prediction, Option B is selected for the definition of Top-1 genie-aided Tx beam, i.e., the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s).

- The specific Rx beam is one DL Rx beam selected from all Rx beams; company can report which specific Rx beam it is.

For DL beam pair prediction and DL Tx beam prediction, we use both fixed and random patterns in our simulation. The fixed pattern selection is shown in Figure 1, with 8 Tx beams selected from 32 Tx beams. For the random pattern, we simulate Opt B and Opt C in the RAN1#110b-e meeting agreement. For Opt B, four fixed patterns are pre-configured as shown in Figure 2. During training and inference, Set B is randomly changed among pre-configured patterns. For Opt C, the 8 gNB DL Tx beams are randomly selected from 32 gNB DL Tx beams. For beam pair prediction, all 4 RX beams are used to receive selected DL Tx beams. For Tx beam prediction, the second beam of all 4 DL Rx beams is selected as a specific Rx beam to receive selected DL Tx beams.

Figure 1: Fixed pattern 1 for DL Tx beam

Figure 2: Pre-configured patterns_1 for DL Tx beam

Figure 3: The pre-configured patterns_2 for DL Tx beam

  - Model description

In our simulation, we investigate a ResNet-based AI/ML model. As shown in Figure 4, the model consists of a three-layer full connection structure and two Resblocks. Each Resblock contains a three-layer convolution structure, and residual calculation is conducted between the first layer and the last convolution layer. The structure is repeated between the two Resblocks. The input data is the L1-RSRP (and beam ID) vector, and the output data is the Top-N Tx beam ID and Rx beam ID for beam pair prediction or Tx beam ID for Tx beam prediction.

Figure 4: ResNet based Model

  - KPI

During the previous meetings, the KPIs for evaluating the performance of AI/ML-based beam management were discussed. Based on the agreements made in the previous meetings, the KPIs can be divided into two categories: beam prediction accuracy-related KPIs and system level performance-related KPIs.

In our simulation, we select the following KPIs for beam prediction accuracy:

- Top-1 (%): the percentage of “the Top-1 genie-aided beam is Top-1 predicted beam”

- Top-K/1 (%): the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”. The value of K is 3.

- Average L1-RSRP difference of Top-1 predicted beam.

For DL beam pair prediction and DL Tx beam prediction, Option 1 (exhaustive beam sweeping) is used as the baseline. The model output, i.e., the top-N beam ID, is used for gNB scheduling. Thus, Option 1 is selected for RS overhead reduction in BM-Case1, which is “RS OH reduction%=1-NM”.

We believe that the reduction of RS overhead, as demonstrated by the intuitive comparison with the baseline scheme (exhaustive beam sweeping), is sufficient to reflect the KPI of RS overhead. Therefore, RS overhead reduction alone is sufficient.

For the KPI of RS overhead for BM-Case1, Option 1 is preferred for RS overhead reduction:

- Option 1: RS OH reduction%=1-NM

  - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement

  - where M is the total number of beams (pairs) to be predicted.

  - Beam management procedures

During the discussion in the last meeting, FL summarized the following options for beam management procedure for both BM-Case1 and BM-Case2.

- Option 1: Use AI/ML model to predict Top-1 DL Tx beam in Set A

  - For UE side AI/ML, the Top-1 DL Tx beam can be recommended by UE for DL data transmission

  - For gNB AI/ML, the Top-1 DL Tx beam can be used for DL data transmission

  - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam

- Option 2: Use AI/ML model to predict Top-K Tx beams in Set A

  - For UE side AI/ML, the Top-K Tx beams can be recommended by UE for P2

  - For gNB AI/ML, the Top-K DL Tx beam can be used for P2

  - P2 is used to select Top-1 DL Tx beam for DL data transmission

  - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam

- Option 3: Use AI/ML model to predict Top-1 Tx-Rx beam pair in Set A

  - For UE side AI/ML,

    - the Top-1 Tx-Rx beam pair can be used for DL data reception

    - the Tx beam of the Top-1 can be recommended by UE for DL data transmission

    - P3 procedure is not needed

  - For gNB side AI/ML,

    - the Tx beam of the Top-1 Tx-Rx beam pair can be used for DL data transmission

- Option 4: Use AI/ML model to predict Top-K Tx-Rx beam pairs in Set A

  - For UE side AI/ML,

    - the Top-K Tx-Rx beam pairs can be used for DL data reception

    - the Top-K Tx beams can be recommended by UE for P2

      - P2 is used to select Top-1 DL Tx beam for DL data transmission

    - P3 procedure is not needed

  - For gNB side AI/ML,

    - the Tx beams of the Top-K Tx-Rx beam pairs can be used for P2

      - P2 is used to select Top-1 DL Tx beam for DL data transmission

- Other options are not precluded.

We generally agree with the above options, except for the following two comments:

For Option 2 and Option 4, we think P2 is not mandatory for selecting the Top-1 DL Tx beam. The gNB can always select one DL Tx beam among the Top-K Tx beams, e.g., based on resource allocation, which is similar to the legacy mechanism. In the legacy mechanism, UE can report up to 4 beams and gNB select one of them for DL data transmission.

For Tx-Rx beam pair prediction for gNB side AI/ML, the Rx beam of the selected Tx-Rx beam pair can be recommended by gNB for DL data reception.

Therefore, we propose the following changes to the above options for beam management procedure for both BM-Case1 and BM-Case2.

Proposal 3: For both BM-Case1 and BM-Case2, further study the following options to apply AI/ML in beam management procedure:

- Option 1: Use AI/ML model to predict Top-1 DL Tx beam in Set A

  - For UE side AI/ML, the Top-1 DL Tx beam can be recommended by UE for DL data transmission

  - For gNB AI/ML, the Top-1 DL Tx beam can be used for DL data transmission

  - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam

- Option 2: Use AI/ML model to predict Top-K Tx beams in Set A

  - For UE side AI/ML, the Top-K Tx beams can be recommended by UE for P2

  - For gNB AI/ML, the Top-K DL Tx beam can be used for P2

  - gNB selects one DL Tx beam for DL data transmission using P2 or up to gNB implementation

  - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam

- Option 3: Use AI/ML model to predict Top-1 Tx-Rx beam pair in Set A

  - For UE side AI/ML,

    - the Top-1 Tx-Rx beam pair can be used for DL data reception

    - the Tx beam of the Top-1 Tx-Rx beam pair can be recommended by UE for DL data transmission

    - P3 procedure is not needed

  - For gNB side AI/ML,

    - the Tx beam of the Top-1 Tx-Rx beam pair can be used for DL data transmission

    - the Rx beam of the Top-1 Tx-Rx beam pair can be recommended by gNB for DL data reception

- Option 4: Use AI/ML model to predict Top-K Tx-Rx beam pairs in Set A

  - For UE side AI/ML,

    - the Top-K Tx-Rx beam pairs can be used for DL data reception

    - the Top-K Tx beams of Top-K Tx-Rx beam pairs can be recommended by UE for P2

      - gNB selects one DL Tx beam for DL data transmission using P2 or up to gNB implementation

    - P3 procedure is not needed

  - For gNB side AI/ML,

    - the Top-K Tx beams of the Top-K Tx-Rx beam pairs can be used for P2

      - gNB selects one DL Tx beam for DL data transmission using P2 or up to gNB implementation

    - the Rx beam corresponding with the selected DL Tx beam can be recommended by gNB

    - P3 procedure is not needed

- Other options are not precluded.

- Simulation results

We evaluate the beam pair prediction and DL Tx beam prediction using fixed, random, and pre-configured patterns. The simulation results are shown for Top-1 accuracy, Top-3/1 accuracy, and average Top-1 RSRP difference. Additionally, the generalization performance is investigated.

The options of Set B selection are as follows:

- Option 1: Set B is fixed across training and inference.

- Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference):

  - Opt A: Set B is changed following a set of pre-configured patterns

  - Opt B: Set B is randomly changed among pre-configured patterns

  - Opt C: Set B is randomly changed among Set A beams (pairs)

  - Beam pair prediction

In the evaluation of beam pair prediction, we use the fixed pattern in Figure 1 and a randomly changed pattern of Opt C as shown in Section 2.1. The simulation results of Top-1 accuracy, Top-3/1 accuracy and average Top-1 RSRP difference are presented in Table 1 and Table 2.

Table 1: Evaluation results for BM-Case1 without model generalization for Tx-Rx beam pair prediction with fixed pattern

|  |  |  | CATT |
| --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |
|  |  | Number of beam pairs in Set B | 32 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |
| AI/ML model input/output |  | Model input | RSRP of beam pairs |
|  |  | Model output | The best DL Tx and Rx beam (beam pair) ID |
| Data Size |  | Training | 150,000 |
|  |  | Testing | 40,000 |
| AI/ML model |  | Short model description | ResNet based |
|  |  | Model complexity | 206k |
|  |  | Computational complexity | 0.76M |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9563 |
|  |  | Top-3/1 | 0.9866 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0127 |
|  | System performance | RS overhead Reduction (%) | 75% |

Table 2: Evaluation results for BM-Case1 without model generalization for Tx-Rx beam pair prediction with random pattern

|  |  |  | CATT |
| --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |
|  |  | Number of beam pairs in Set B | 32 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |
| AI/ML model input/output |  | Model input | RSRP of beam pairs |
|  |  | Model output | The best DL Tx and Rx beam (beam pair)ID |
| Data Size |  | Training | 150,000 |
|  |  | Testing | 40,000 |
| AI/ML model |  | Short model description | ResNet based |
|  |  | Model complexity | 104k |
|  |  | Computational complexity | 0.76M |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.1289 |
|  |  | Top-3/1 | 0.3954 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 11.623 |
|  | System performance | RS overhead Reduction (%) | 75% |

We also evaluate the random pattern with the additional input of beam ID. The input is a random pattern of Opt C (Set B is randomly changed among Set A beam pairs) and beam ID. The simulation results are shown in Table 3. Compared with the beam prediction accuracy with the random pattern using L1-RSRP input only, the additional Beam ID input has a significant performance gain.

Table 3: Evaluation results for BM-Case1 without model generalization for Tx-Rx beam pair prediction with random pattern and beam ID

|  |  |  | CATT |
| --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |
|  |  | Number of beam pairs in Set B | 32 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |
| AI/ML model input/output |  | Model input | RSRP of beam pairs with beam ID |
|  |  | Model output | The best DL Tx and Rx beam(beam pair) ID |
| Data Size |  | Training | 150,000 |
|  |  | Testing | 40,000 |
| AI/ML model |  | Short model description | ResNet based |
|  |  | Model complexity | 206k |
|  |  | Computational complexity | 2.84M |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.7004 |
|  |  | Top-3/1 | 0.8700 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 1.4075 |
|  | System performance | RS overhead Reduction (%) | 75% |

For the selection of Set B pattern, we evaluated the pre-configured patterns shown in Figure 2. The input is a random pattern of Opt B, which is randomly changed among the pre-configured patterns during the training phase. Similarly, the pattern is also randomly changed during the inference phase. The simulation results are shown in Table 4.

Table 4: Evaluation results for BM-Case1 without model generalization for Tx-Rx beam pair prediction with pre-configured patterns

|  |  |  | CATT |
| --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |
|  |  | Number of beam pairs in Set B | 32 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |
| AI/ML model input/output |  | Model input | RSRP of beam pairs |
|  |  | Model output | The best DL Tx and Rx beam (beam pair) ID |
| Data Size |  | Training | 150,000 |
|  |  | Testing | 40,000 |
| AI/ML model |  | Short model description | ResNet based |
|  |  | Model complexity | 104k |
|  |  | Computational complexity | 0.84M |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9352 |
|  |  | Top-3/1 | 0.9600 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0223 |
|  | System performance | RS overhead Reduction (%) | 75% |

Observation 1: For Beam pair prediction, using pre-configured patterns for training and inference, the Top-1 accuracy can be greater than 93%.

Proposal 4: For BM-Case1, random pattern of Opt B, i.e., Set B is randomly changed among pre-configured patterns, can be used to improve beam pair prediction accuracy.

  - DL Tx beam prediction

In the evaluation of DL Tx beam prediction, the simulation assumption is described in Section 2.1. According to the discussion in the previous meeting, we evaluated the Top-1 accuracy, Top-3/1 accuracy and the average Top-1 RSRP difference. For DL Tx beam prediction evaluation, Option 2b (Measurements of specific Rx beam(s) for all model input sample) was selected as the Rx beam.

    - Effect of random pattern on prediction performance

In the evaluation of DL Tx beam prediction with random patterns, we use a random pattern of Opt C (Set B is randomly changed among Set A beams). The simulation assumptions are described in Section 2.1. The simulation results of Top-1 accuracy, Top-3/1 accuracy, and average Top-1 RSRP difference are provided in Table 5.

Table 5: Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction with random pattern

|  |  |  | CATT |
| --- | --- | --- | --- |
| Assumptions |  | Number of beams in Set A | 32 |
|  |  | Number of beams in Set B | 8 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |
| AI/ML model input/output |  | Model input | RSRP of beams with beam ID |
|  |  | Model output | The best DL Tx beam ID |
| Data Size |  | Training | 150,000 |
|  |  | Testing | 40,000 |
| AI/ML model |  | Short model description | ResNet based |
|  |  | Model complexity | 103k |
|  |  | Computational complexity | 0.78M |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.1513 |
|  |  | Top-3/1 | 0.4252 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 12.7665 |
|  | System performance | RS overhead Reduction (%) | 75% |

We also evaluated the random pattern with the additional input of beam ID. The input is a random pattern of Opt C (Set B is randomly changed among Set A beam pairs) and beam ID. The simulation results are shown in Table 6. Compared with the random pattern using L1-RSRP input only, the additional Beam ID input has a significant performance gain.

Table 6: Evaluation results for BM-Case1 without model generalization for Tx beam prediction with random pattern with beam ID

|  |  |  | CATT |
| --- | --- | --- | --- |
| Assumptions |  | Number of beams in Set A | 32 |
|  |  | Number of beams in Set B | 8 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |
| AI/ML model input/output |  | Model input | RSRP of beams with beam ID |
|  |  | Model output | The best DL Tx beam ID |
| Data Size |  | Training | 150,000 |
|  |  | Testing | 40,000 |
| AI/ML model |  | Short model description | ResNet based |
|  |  | Model complexity | 103k |
|  |  | Computational complexity | 0.78M |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.5833 |
|  |  | Top-3/1 | 0.7043 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 1.0647 |
|  | System performance | RS overhead Reduction (%) | 75% |

Observation 2: For DL Tx prediction with random pattern, using beam ID as additional information can improve prediction accuracy.

    - Effect of pre-configured pattern on prediction performance

In the evaluation of pre-configured patterns, four different patterns are designed in Section 2.1. For the selection of Set B pattern, we evaluated pre-configured patterns shown in Figure 2. The input is a random pattern of Opt B (Set B is randomly changed among pre-configured patterns). During the training phase, Set B is randomly changed among pre-configured patterns. Similarly, it is also randomly changed during the inference phase. The simulation results are shown in Table 7. Rx beam is selected as Option 2b (Measurements of specific Rx beam(s) for all model input samples). In Table 7, we select the first beam of Rx beam for evaluation.

Table 7: Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction with pre-configured patterns_1 of specific Rx beam

|  |  |  | CATT |
| --- | --- | --- | --- |
| Assumptions |  | Number of beams/ in Set A | 32 |
|  |  | Number of beams in Set B | 8 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |
| AI/ML model input/output |  | Model input | RSRP of beams |
|  |  | Model output | The best DL Tx beam ID |
| Data Size |  | Training | 150,000 |
|  |  | Testing | 40,000 |
| AI/ML model |  | Short model description | ResNet based |
|  |  | Model complexity | 103k |
|  |  | Computational complexity | 0.78M |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9723 |
|  |  | Top-3/1 | 0.9999 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0311 |
|  | System performance | RS overhead Reduction (%) | 75% |

When Option 1 (measurement of the "best" Rx beam with exhaustive beam sweeping for each model input sample) is selected, the simulation results are shown in Table 8.

Table 8: Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction with pre-configured patterns_1 of “best Rx beam”

|  |  |  | CATT |
| --- | --- | --- | --- |
| Assumptions |  | Number of beams/ in Set A | 32 |
|  |  | Number of beams in Set B | 8 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |
| AI/ML model input/output |  | Model input | RSRP of beams |
|  |  | Model output | The best DL Tx beam ID |
| Data Size |  | Training | 150,000 |
|  |  | Testing | 40,000 |
| AI/ML model |  | Short model description | ResNet based |
|  |  | Model complexity | 103k |
|  |  | Computational complexity | 0.78M |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9772 |
|  |  | Top-3/1 | 0.9995 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0095 |
|  | System performance | RS overhead Reduction (%) | 75% |

To evaluate the performance of different pre-configured patterns, we evaluate the DL Tx beam prediction performance when training and inference using the different pre-configured patterns. The pre-configured pattern used for inference is shown in Figure 3, and the pre-configured pattern used for training is shown in Figure 2. The simulation results of Top-1 accuracy, Top-3/1 accuracy and average Top-1 RSRP difference are provided in Table 9.

Table 9: Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction when training and inference use different pre-configured patterns

|  |  |  | CATT |
| --- | --- | --- | --- |
| Assumptions |  | Number of beams/ in Set A | 32 |
|  |  | Number of beams in Set B | 8 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |
| AI/ML model input/output |  | Model input | RSRP of beams |
|  |  | Model output | The best DL Tx beam ID |
| Data Size |  | Training | 150,000 |
|  |  | Testing | 40,000 |
| AI/ML model |  | Short model description | ResNet based |
|  |  | Model complexity | 103k |
|  |  | Computational complexity | 0.78M |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.0856 |
|  |  | Top-3/1 | 0.2918 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.2571 |
|  | System performance | RS overhead Reduction (%) | 75% |

Observation 3: For DL Tx prediction, the prediction accuracy is greatly reduced when using different pre-configured patterns for training and inference.

Proposal 5: The patterns for training and inference should be pre-configured or configured to cover all beams of Set A as much as possible.

    - Effect of Rx beam selection on prediction

In the evaluation of DL Tx beam prediction with a fixed pattern, we have evaluated different Rx beam selection methods and their effect on the prediction accuracy. We used fixed pattern 1 in Figure 1, and the simulation assumptions are described in Section 2.1.

At least for evaluation of the performance of DL Tx beam prediction, consider the following options for Rx beam to provide input for AI/ML model training and/or inference, if applicable:

- Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample.

- Option 2: Measurements of specific Rx beam(s)

  - Option 2a: Measurements of specific Rx beam(s) per model input sample

  - Option 2b: Measurements of specific Rx beam(s) for all model input sample

- Option 3: Measurements of random Rx beam(s) per model input sample

For Option1, for each Rx beam, measure the RSRP of the Tx beam of each sample on this Rx beam, and the Rx beam with the largest RSRP as the Rx beam selected for each Tx beam of each sample. The simulation results of Top-1 accuracy, Top-3/1 accuracy and average Top-1 RSRP difference are provided in Table 10.

Table 10: Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction for option 1

|  |  |  | CATT |
| --- | --- | --- | --- |
| Assumptions |  | Number of beams/ in Set A | 32 |
|  |  | Number of beams in Set B | 8 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |
| AI/ML model input/output |  | Model input | RSRP of beams |
|  |  | Model output | The best DL Tx beam ID |
| Data Size |  | Training | 150,000 |
|  |  | Testing | 40,000 |
| AI/ML model |  | Short model description | ResNet based |
|  |  | Model complexity | 103k |
|  |  | Computational complexity | 0.78M |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9887 |
|  |  | Top-3/1 | 0.9999 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0091 |
|  | System performance | RS overhead Reduction (%) | 75% |

For Option2a, for each Rx beam, sum the RSRP of 32 DL Tx beams contained in each sample, and select the Rx beam with the largest RSRP sum as the Rx beam selected for all Tx beams of this sample. The simulation results of Top-1 accuracy, Top-3/1 accuracy and average Top-1 RSRP difference are provided in Table 11.

Table 11: Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction for option 2a

|  |  |  | CATT |
| --- | --- | --- | --- |
| Assumptions |  | Number of beams/ in Set A | 32 |
|  |  | Number of beams in Set B | 8 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |
| AI/ML model input/output |  | Model input | RSRP of beams |
|  |  | Model output | The best DL Tx beam ID |
| Data Size |  | Training | 150,000 |
|  |  | Testing | 40,000 |
| AI/ML model |  | Short model description | ResNet based |
|  |  | Model complexity | 103k |
|  |  | Computational complexity | 0.78M |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9877 |
|  |  | Top-3/1 | 0.9999 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0018 |
|  | System performance | RS overhead Reduction (%) | 75% |

For Option2b, we select

- The first Rx beam as the Rx beam of all samples;

- For each Rx beam, sum the RSRP of 32 DL Tx beams of all samples, and select the Rx beam with the largest RSRP summation as the best Rx beam of all samples.

The simulation results of selecting the first beam for Top-1 accuracy, Top-3/1 accuracy and average Top-1 RSRP difference are provided in Table 12, and the simulation results of selecting the best beam for Top-1 accuracy, Top-3/1 accuracy and average Top-1 RSRP difference are provided in Table 13.

Table 12: Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction for option 2b with the first Rx beam

|  |  |  | CATT |
| --- | --- | --- | --- |
| Assumptions |  | Number of beams in Set A | 32 |
|  |  | Number of beams in Set B | 8 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |
| AI/ML model input/output |  | Model input | RSRP of beams |
|  |  | Model output | The best DL Tx beam ID |
| Data Size |  | Training | 150,000 |
|  |  | Testing | 40,000 |
| AI/ML model |  | Short model description | ResNet based |
|  |  | Model complexity | 103k |
|  |  | Computational complexity | 0.78M |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9445 |
|  |  | Top-3/1 | 0.9962 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0749 |
|  | System performance | RS overhead Reduction (%) | 75% |

Table 13: Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction for option 2b with the best Rx beam

|  |  |  | CATT |
| --- | --- | --- | --- |
| Assumptions |  | Number of beams/ in Set A | 32 |
|  |  | Number of beams in Set B | 8 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |
| AI/ML model input/output |  | Model input | RSRP of beams |
|  |  | Model output | The best DL Tx beam ID |
| Data Size |  | Training | 150,000 |
|  |  | Testing | 40,000 |
| AI/ML model |  | Short model description | ResNet based |
|  |  | Model complexity | 103k |
|  |  | Computational complexity | 0.78M |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9842 |
|  |  | Top-3/1 | 0.9999 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0202 |
|  | System performance | RS overhead Reduction (%) | 75% |

Observation 4: For DL Tx prediction with fixed pattern the Top-1 accuracy can be greater than 98%.

For Option3, a randomly selected Rx beam is used for each sample. We randomly select one Rx beam as the Rx beam of all 32 DL Tx beams in each sample. The simulation results for Top-1 accuracy, Top-3/1 accuracy and average Top-1 RSRP difference are provided in Table 14.

Table 14: Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction for option 3

|  |  |  | CATT |
| --- | --- | --- | --- |
| Assumptions |  | Number of beams/ in Set A | 32 |
|  |  | Number of beams in Set B | 8 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |
| AI/ML model input/output |  | Model input | RSRP of beams |
|  |  | Model output | The best DL Tx beam ID |
| Data Size |  | Training | 150,000 |
|  |  | Testing | 40,000 |
| AI/ML model |  | Short model description | ResNet based |
|  |  | Model complexity | 103k |
|  |  | Computational complexity | 0.78M |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9523 |
|  |  | Top-3/1 | 0.9971 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0648 |
|  | System performance | RS overhead Reduction (%) | 75% |

Observation 5: The performance of randomly selecting Rx beam is worse than that of selecting the best Rx beam, but selecting the best Rx beam requires additional selection steps, which will increase the complexity of UE implementation.

    - Effect of AL/ML based RSRP prediction

The above evaluations are based on the AI/ML model based beam ID prediction. In this section, we evaluate the AI/ML model based RSRP prediction with fixed pattern. The number of Tx beam in Set A is 32. The number of Tx beam in Set B is 8. The DL Rx beam of UE is fixed with one DL Rx beam selected from 4 DL Rx beams. In our simulation, We select the first Rx beam of 4 DL Rx beams and .We select the first Rx beam as selected Rx beam of all samples, i.e., option 2b (Measurements of specific Rx beam(s) for all model input sample). The simulation assumption is described in Section 2.1. The simulation results of Top-1 accuracy, Top-3/1 accuracy and average Top-1 RSRP difference are shown in Table 15.

Table 15: Evaluation results for BM-Case1 without model generalization for DL Tx beam RSRP prediction for option 2b

|  |  |  | CATT |
| --- | --- | --- | --- |
| Assumptions |  | Number of beams/ in Set A | 32 |
|  |  | Number of beams in Set B | 8 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |
| AI/ML model input/output |  | Model input | RSRP of beams |
|  |  | Model output | The RSRP of DL Tx beam |
| Data Size |  | Training | 150,000 |
|  |  | Testing | 40,000 |
| AI/ML model |  | Short model description | ResNet based |
|  |  | Model complexity | 103k |
|  |  | Computational complexity | 0.78M |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9321 |
|  |  | Top-3/1 | 0.9842 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0667 |
|  | System performance | RS overhead Reduction (%) | 75% |

Observation 6: For DL Tx RSRP prediction with fixed pattern, the prediction accuracy of Top-1 is higher than 93% for DL Tx beam RSRP prediction.

  - Generalization

For the study of generalization, we respectively studied the generalization under different Set B, different scenario configurations, different frequencies, different ISD, and different UE distributions.

    - Different Set B

In the training phase, the size of input set B is 32 beam pairs. In the inference phase, we simulate different sizes of set B as inputs, which are 16, 32 and 64, respectively.

For 16 beam pairs in Set B, they include selected 4 gNB DL Tx beams and 4 UE DL Rx beams. To determine 4 gNB DL Tx beams out of 32 gNB DL Tx beams, we use fixed pattern 2 and random pattern in our simulation. The fixed pattern 2 selection is shown inFigure 5. For 32 beam pairs in Set B, they include selected 8 gNB DL Tx beams and 4 UE DL Rx beams. To determine 8 gNB DL Tx beams out of 32 gNB DL Tx beams, we use fixed pattern 1 and random pattern in our simulation. The fixed pattern 1 selection is shown in Figure 1. For 64 beam pairs in Set B, they include selected 16 gNB DL Tx beams and 4 UE DL Rx beams. To determine 16 gNB DL Tx beams out of 32 gNB DL Tx beams, we use fixed pattern 3 and random pattern in our simulation. The fixed pattern 3 selection is shown in Figure 6.

Figure 5: Fixed pattern 2 selection of DL TX beam

Figure 6: Fixed pattern 3 selection of DL TX beam

Scenario#1: In the training phase, the sizes of input set B is 32 beam pairs. In theinference phase, we simulate different sizes of set B as input, which are 16, 32 and 64 respectively. The simulation results of accuracy related KPIs using fixed pattern, i.e., training using fixed pattern 1 but inference using fixed pattern 1,2,3, respectively, are shown in Table 16. The simulation results of accuracy related KPIs using random pattern of Opt C(Set B is randomly changed among Set A beams pairs) with beam ID are shown in Table 17.

Table 16: Evaluation results for BM-Case1 with different Set B of inference for Tx-Rx beam pair prediction with fixed pattern

|  |  |  | CATT |  |  |
| --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |  |  |
|  |  | Number of beam pairs in Set B | 16 | 32 | 64 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |
| AI/ML model input/output |  | Model input | RSRP of beam pairs |  |  |
|  |  | Model output | The best DL Tx and Rx beam(beam pair) ID |  |  |
| Data Size |  | Training | 150,000 |  |  |
|  |  | Testing | 40,000 |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |
|  |  | Model complexity | 206k |  |  |
|  |  | Computational complexity | 2.84M |  |  |
| Training |  | Number of beam pairs in Set B | 32 |  |  |
| Inference |  | Number of beam pairs in Set B | 16 | 32 | 64 |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.2761 | 0.9563 | 0.8414 |
|  |  | Top-3/1 | 0.455000 | 0.9993 | 0.9710 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 12.4924 | 0.0323 | 4.3980 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |

Table 17: Evaluation results for BM-Case1 with different Set B of inference for Tx-Rx beam pair prediction random pattern and beam ID

|  |  |  | CATT |  |  |
| --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |  |  |
|  |  | Number of beam pairs in Set B | 16 | 32 | 64 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |
| AI/ML model input/output |  | Model input | RSRP of beam pairs with beam ID |  |  |
|  |  | Model output | the best DL Tx and Rx beam(beam pair) ID |  |  |
| Data Size |  | Training | 150,000 |  |  |
|  |  | Testing | 40,000 |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |
|  |  | Model complexity | 206k |  |  |
|  |  | Computational complexity | 2.84M |  |  |
| Training |  | Number of beam pairs in Set B | 32 |  |  |
| Inference |  | Number of beam pairs in Set B | 16 | 32 | 64 |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.4590 | 0.7004 | 0.7709 |
|  |  | Top-3/1 | 0.7695 | 0.8922 | 0.9359 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 4.2358 | 1.4057 | 0.1321 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |

Scenario#2: In the training phase, the sizes of input set B are mixed 16, 32 and 64 beam pairs together. In the inference phase, we simulate different sizes of set B as inputs, which are 16, 32 and 64, respectively. The simulation results of accuracy related KPIs using fixed pattern, i.e., training using fixed pattern 1,2 and 3 together, but inference using fixed pattern 1,2,3, respectively, are shown in Table 18. The simulation results of accuracy related KPIs using random pattern of Opt C(Set B is randomly changed among Set A beams pairs) with beam ID are shown in Table 19.

Table 18: Evaluation results for BM-Case1 with mixed Set B for Tx-Rx beam pair prediction with fixed pattern

|  |  |  | CATT |  |  |
| --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |  |  |
|  |  | Number of beam pairs in Set B | 16 | 32 | 64 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |
| AI/ML model input/output |  | Model input | RSRP of beam pairs |  |  |
|  |  | Model output | The best DL Tx and Rx beam(beam pair) ID |  |  |
| Data Size |  | Training | 150,000 |  |  |
|  |  | Testing | 40,000 |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |
|  |  | Model complexity | 206k |  |  |
|  |  | Computational complexity | 2.84M |  |  |
| Training |  | Number of beam pairs in Set B | 16，32，64 |  |  |
| Inference |  | Number of beam pairs in Set B | 16 | 32 | 64 |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.8560 | 0.9419 | 0.9469 |
|  |  | Top-3/1 | 0.9771 | 0.9977 | 0.9990 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 1.1749 | 0.0835 | 0.0108 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |

Table 19: Evaluation results for BM-Case1 with mixed Set B for Tx-Rx beam pair prediction with random pattern and beam ID

|  |  |  | CATT |  |  |
| --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |  |  |
|  |  | Number of beam pairs in Set B | 16 | 32 | 64 |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |
| AI/ML model input/output |  | Model input | RSRP of beam pairs with beam ID |  |  |
|  |  | Model output | The best DL Tx and Rx beam(beam pair) ID |  |  |
| Data Size |  | Training | 150,000 |  |  |
|  |  | Testing | 40,000 |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |
|  |  | Model complexity | 206k |  |  |
|  |  | Computational complexity | 2.84M |  |  |
| Training |  | Number of beam pairs in Set B | 16,32,64 |  |  |
| Inference |  | Number of beam pairs in Set B | 16 | 32 | 64 |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.7898 | 0.7902 | 0.7914 |
|  |  | Top-3/1 | 0.9025 | 0.9258 | 0.9301 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 1.0579 | 1.0683 | 1.0652 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |

Observation 7: When AI/ML model is trained and inferenced with different Set B, e.g., the Set B of training is 32 and inference is 16, the Top-1 accuracy is degraded.

    - Different scenario configurations

For the research on generalization, in addition to the above research, we have studied different scenario configurations. We use different scenario configurations in the training and inference phase to verify the generalization of different scenario configurations.

Scenario#3: In the training phase, input set B is 32 beam pairs in the Uma scenario. In the inference phase, we use the data set of 32 beam pairs in the Uma scenario and Umi scenario respectively. Simultaneously, we also trained the model that input scenario of Set B is Uma scenario and the mixed scenario of Umi and Uma. In the mixed scenario of Umi and Uma model, the training data is 75K for the Uma scenario and 75K for the Umi scenario. The simulation results of accuracy related KPIs using fixed pattern 1 are shown in Table 20. Table 21 shows the simulation results of KPIs related to the accuracy of using random pattern of Opt C(Set B is randomly changed among Set A beams pairs) with beam ID.

Scenario#4: In the training phase, input set B is 8 Tx beam in the Umi scenario. In the inference phase, we use the data set of 8 Tx beam in the Uma scenario and Umi scenario respectively. Simultaneously, we also trained the model that input scenario of Set B is Uma scenario and the mixed scenario of Umi and Uma. In the mixed scenario of Umi and Uma model, the training data is 75K for the Uma scenario and 75K for the Umi scenario.  The simulation results of accuracy related KPIs using fixed pattern 1 are shown in Table 22. Table 23 shows the simulation results of KPIs related to the accuracy of using random pattern of Opt C(Set B is randomly changed among Set A beams pairs) with beam ID.

Table 20: Evaluation results for BM-Case1 with different scenario for Tx-Rx beam pair prediction with fixed pattern

|  |  |  | CATT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |  |  |  |  |  |
|  |  | Number of beam pairs in Set B | 32 |  |  |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beam pairs |  |  |  |  |  |
|  |  | Model output | The best DL Tx and Rx(beam pair) ID |  |  |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |  |  |
|  |  | Testing | 40,000 |  |  |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |  |  |
|  |  | Model complexity | 206k |  |  |  |  |  |
|  |  | Computational complexity | 0.76M |  |  |  |  |  |
| Training |  | scenario configurations | Uma |  | Umi |  | Umi&Uma （75K for the Uma and 75K for the Umi） |  |
| Inference |  | scenario configurations | Uma | Umi | Uma | Umi | Uma | Umi |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9560 | 0.9491 | 0.9657 | 0.9698 | 0.9662 | 0.9649 |
|  |  | Top-3/1 | 0.9877 | 0.9638 | 0.9599 | 0.9839 | 0.9798 | 0.9854 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0369 | 0.0202 | 0.0358 | 0.2036 | 0.0624 | 0.0694 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |  |  |

Table 21: Evaluation results for BM-Case1 with different scenario for Tx-Rx beam pair prediction with random pattern with beam ID

|  |  |  | CATT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |  |  |  |  |  |
|  |  | Number of beam pairs in Set B | 32 |  |  |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beam pairs with beam ID |  |  |  |  |  |
|  |  | Model output | The best DL Tx and Rx beam(beam pair) ID |  |  |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |  |  |
|  |  | Testing | 40,000 |  |  |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |  |  |
|  |  | Model complexity | 206k |  |  |  |  |  |
|  |  | Computational complexity | 2.84M |  |  |  |  |  |
| Training |  | scenario configurations | Uma |  | Umi |  | Umi&Uma （75K for the Uma and 75K for the Umi） |  |
| Inference |  | scenario configurations | Uma | Umi | Uma | Umi | Uma | Umi |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.7960 | 0.7419 | 0.7211 | 0.7899 | 0.7881 | 0.7862 |
|  |  | Top-3/1 | 0.9622 | 0.9510 | 0.9611 | 0.9745 | 0.9877 | 0.9796 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.2568 | 1.0021 | 1.0611 | 0.5041 | 0.0695 | 0.0599 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |  |  |

Table 22: Evaluation results for BM-Case1 with different scenario for Tx beam prediction with fixed pattern

|  |  |  | CATT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beams in Set A | 32 |  |  |  |  |  |
|  |  | Number of beams in Set B | 8 |  |  |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beams |  |  |  |  |  |
|  |  | Model output | The best DL Tx beam ID |  |  |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |  |  |
|  |  | Testing | 40,000 |  |  |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |  |  |
|  |  | Model complexity | 103k |  |  |  |  |  |
|  |  | Computational complexity | 0.78M |  |  |  |  |  |
| Training |  | scenario configurations | Uma |  | Umi |  | Umi&Uma （75K for the Uma and 75K for the Umi） |  |
| Inference |  | scenario configurations | Uma | Umi | Uma | Umi | Uma | Umi |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9596 | 0.9428 | 0.9336 | 0.9522 | 0.9495 | 0.9468 |
|  |  | Top-3/1 | 0.9989 | 0.9599 | 0.9964 | 0.9867 | 0.9851 | 0.9963 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0225 | 0.4415 | 0.0859 | 0.0358 | 0.0562 | 0.4019 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |  |  |

Table 23: Evaluation results for BM-Case1 with different scenario for Tx beam prediction with random pattern with beam ID

|  |  |  | CATT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beams in Set A | 32 |  |  |  |  |  |
|  |  | Number of beams in Set B | 8 |  |  |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beams with beam ID |  |  |  |  |  |
|  |  | Model output | The best DL Tx beam ID |  |  |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |  |  |
|  |  | Testing | 40,000 |  |  |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |  |  |
|  |  | Model complexity | 103k |  |  |  |  |  |
|  |  | Computational complexity | 0.78M |  |  |  |  |  |
| Training |  | scenario configurations | Uma |  | Umi |  | Umi&Uma （75K for the Uma and 75K for the Umi） |  |
| Inference |  | scenario configurations | Uma | Umi | Uma | Umi | Uma | Umi |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.5421 | 0.5193 | 0.5429 | 0.5354 | 0.5514 | 0.5329 |
|  |  | Top-3/1 | 0.7221 | 0.7200 | 0.7336 | 0.7602 | 0.7501 | 0.7421 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 4.0623 | 3.8288 | 3.5145 | 3.3595 | 3.5646 | 3.1464 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |  |  |

Observation 8: When AI/ML model is trained and inferenced with different scenario configurations, e.g., the Set B of training is Uma and inference is Umi, the Top-1 accuracy can be comparable.

    - Different UE distributions

We also evaluated the generalization of AI/ML models across scenarios with different indoor and outdoor UE distributions, including the proportion of indoor UEs, 0%, 50% and 80%.

Scenario#5: In the training phase, the proportion of indoor UE is 0. In the inference phase, we use the data sets of 32 beam pairs in three configurations, 0%, 50% and 80% indoor UE. In addition, we also trained the models with the UE distribution of Set B is 50% and 80%.The simulation results of accuracy related KPIs using fixed pattern are shown in Table 24, where fixed pattern 1 is used for training. Table 25 shows the simulation results of KPIs related to the accuracy of using random pattern of Opt C(Set B is randomly changed among Set A beams pairs) with beam ID.

Scenario#6: In the training phase, the proportion of indoor UE in scenario1 is 0%. In the inference phase, we use the data sets of 8 Tx beam in three configurations, 0%, 50% and 80% indoor UE. In addition, we also trained the models with the UE distribution of Set B is 50% and 80%..The simulation results of accuracy related KPIs using fixed pattern are shown in Table 26, where fixed pattern 1 is used for training. Table 27 shows the simulation results of KPIs related to the accuracy of using random pattern of Opt C(Set B is randomly changed among Set A beams pairs)with beam ID.

Table 24 Evaluation results for BM-Case1 with different UE distributions for Tx-Rx beam pair prediction with fixed pattern

|  |  |  | CATT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |  |  |  |  |  |  |  |  |
|  |  | Number of beam pairs in Set B | 32 |  |  |  |  |  |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |  |  |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beam pairs |  |  |  |  |  |  |  |  |
|  |  | Model output | The best DL Tx and Rx (beam pair)ID |  |  |  |  |  |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |  |  |  |  |  |
|  |  | Testing | 40,000 |  |  |  |  |  |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |  |  |  |  |  |
|  |  | Model complexity | 206k |  |  |  |  |  |  |  |  |
|  |  | Computational complexity | 0.76M |  |  |  |  |  |  |  |  |
| Training |  | UE distributions | 0 |  |  | 0.5 |  |  | 0.8 |  |  |
| Inference |  | UE distributions | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9445 | 0.9435 | 0.9374 | 0.9432 | 0.9438 | 0.9375 | 0.9370 | 0.9353 | 0.9387 |
|  |  | Top-3/1 | 0.9989 | 0.9986 | 0.9980 | 0.9989 | 0.9984 | 0.9979 | 0.9982 | 0.9981 | 0.9981 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |  |  |  |  |  |

Table 25: Evaluation results for BM-Case1 with different UE distributions for Tx-Rx beam pair prediction with random pattern with beam ID

|  |  |  | CATT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |  |  |  |  |  |  |  |  |
|  |  | Number of beam pairs in Set B | 32 |  |  |  |  |  |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |  |  |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beam pairs with beam ID |  |  |  |  |  |  |  |  |
|  |  | Model output | The best DL Tx and Rx beam (beam pair)ID |  |  |  |  |  |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |  |  |  |  |  |
|  |  | Testing | 40,000 |  |  |  |  |  |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |  |  |  |  |  |
|  |  | Model complexity | 206k |  |  |  |  |  |  |  |  |
|  |  | Computational complexity | 2.84M |  |  |  |  |  |  |  |  |
| Training |  | UE distributions | 0 |  |  | 0.5 |  |  | 0.8 |  |  |
| Inference |  | UE distributions | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.6712 | 0.6680 | 0.6677 | 0.6501 | 0.6515 | 0.6501 | 0.5768 | 0.5804 | 0.5791 |
|  |  | Top-3/1 | 0.9446 | 0.9464 | 0.9462 | 0.9382 | 0.9376 | 0.9377 | 0.8794 | 0.8811 | 0.8796 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 1.0876 | 1.0588 | 1.0742 | 1.2980 | 1.3050 | 1.3132 | 2.7632 | 2.7284 | 2.7445 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |  |  |  |  |  |

Table 26: Evaluation results for BM-Case1 with different UE distributions for Tx beam predictions with fixed pattern

|  |  |  | CATT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beams in Set A | 32 |  |  |  |  |  |  |  |  |
|  |  | Number of beams in Set B | 8 |  |  |  |  |  |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |  |  |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beams |  |  |  |  |  |  |  |  |
|  |  | Model output | The best DL Tx beam ID |  |  |  |  |  |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |  |  |  |  |  |
|  |  | Testing | 40,000 |  |  |  |  |  |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |  |  |  |  |  |
|  |  | Model complexity | 103k |  |  |  |  |  |  |  |  |
|  |  | Computational complexity | 0.78M |  |  |  |  |  |  |  |  |
| Training |  | UE distributions | 0 |  |  | 0.5 |  |  | 0.8 |  |  |
| Inference |  | UE distributions | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9801 | 0.9800 | 0.9807 | 0.9795 | 0.9800 | 0.9817 | 0.9796 | 0.9789 | 0.9819 |
|  |  | Top-3/1 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 | 0.9999 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 | 0.0002 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |  |  |  |  |  |

Table 27: Evaluation results for BM-Case1 with different UE distributions for Tx beam predictions with random pattern with beam ID

|  |  |  | CATT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beams in Set A | 32 |  |  |  |  |  |  |  |  |
|  |  | Number of beams in Set B | 8 |  |  |  |  |  |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |  |  |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beams with beam ID |  |  |  |  |  |  |  |  |
|  |  | Model output | The best DL Tx beam ID |  |  |  |  |  |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |  |  |  |  |  |
|  |  | Testing | 40,000 |  |  |  |  |  |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |  |  |  |  |  |
|  |  | Model complexity | 103k |  |  |  |  |  |  |  |  |
|  |  | Computational complexity | 0.78M |  |  |  |  |  |  |  |  |
| Training |  | UE distributions | 0 |  |  | 0.5 |  |  | 0.8 |  |  |
| Inference |  | UE distributions | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 | 0 | 0.5 | 0.8 |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.7166 | 0.7179 | 0.7195 | 0.7225 | 0.7187 | 0.7181 | 0.5828 | 0.5790 | 0.5795 |
|  |  | Top-3/1 | 0.9283 | 0.9274 | 0.9287 | 0.9284 | 0.9271 | 0.9288 | 0.8606 | 0.8593 | 0.8592 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0558 | 0.0569 | 0.0558 | 0.0602 | 0.0589 | 0.0604 | 3.4826 | 3.5442 | 3.5321 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |  |  |  |  |  |

Observation 9: When AI/ML model is trained and inferenced with different UE distributions, e.g., the Set B of training of UE distributions is 0% and inference is 50%, the Top-1 accuracy can be comparable.

    - Different carrier frequency

We evaluated the generalization under different carrier frequency models, including 20G and 30G.

Scenario#7: In the training phase, the Set B of carrier frequency is 20G. In the inference phase, we use the data sets of 32 beam pairs in two configurations, carrier frequency 20G and 30G. In addition, we also trained the models with the carrier frequency of Set B is 30G,and also use 20G and 30G for inference. The simulation results of accuracy related KPIs using fixed pattern are shown in Table 28, where fixed pattern 1 is used for training. Table 29 shows the simulation results of KPIs related to the accuracy of using random pattern of Opt C(Set B is randomly changed among Set A beams pairs)  with beam ID.

Scenario#8: In the training phase, the Set B of carrier frequency is 20G. In the inference phase, we use the data sets of 8 Tx beam in two configurations, carrier frequency 20G and 30G. In addition, we also trained the models with the carrier frequency of Set B is 30G, and also use 20G and 30G for inference. The simulation results of accuracy related KPIs using fixed pattern are shown in Table 30, where fixed pattern 1 is used for training. Table 31 shows the simulation results of KPIs related to the accuracy of using random pattern of Opt C(Set B is randomly changed among Set A beams pairs)  with beam ID.

Table 28: Evaluation results for BM-Case1 with different carrier frequency for Tx-Rx beam pair predictions with fixed pattern

|  |  |  | CATT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |  |  |  |
|  |  | Number of beam pairs in Set B | 32 |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beam pairs |  |  |  |
|  |  | Model output | The best DL Tx and Rx (beam pair)ID |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |
|  |  | Testing | 40,000 |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |
|  |  | Model complexity | 206k |  |  |  |
|  |  | Computational complexity | 0.76M |  |  |  |
| Training |  | different carrier frequency | 20G |  | 30G |  |
| Inference |  | different carrier frequency | 20G | 30G | 20G | 30G |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9220 | 0.9159 | 0.9026 | 0.9562 |
|  |  | Top-3/1 | 0.9962 | 0.9899 | 0.9782 | 0.9863 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.4590 | 0.0358 | 0.0182 | 0.0201 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |

Table 29: Evaluation results for BM-Case1 with different carrier frequency for Tx-Rx beam pair predictions with random pattern with beam ID

|  |  |  | CATT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |  |  |  |
|  |  | Number of beam pairs in Set B | 32 |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beam pairs with beam ID |  |  |  |
|  |  | Model output | The best DL Tx and Rx beam(beam pair)ID |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |
|  |  | Testing | 40,000 |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |
|  |  | Model complexity | 206k |  |  |  |
|  |  | Computational complexity | 2.84M |  |  |  |
| Training |  | different carrier frequency | 20G |  | 30G |  |
| Inference |  | different carrier frequency | 20G | 30G | 20G | 30G |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.6425 | 0.6430 | 0.5834 | 0.5981 |
|  |  | Top-3/1 | 0.9335 | 0.9321 | 0.8756 | 0.8916 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0239 | 0.0231 | 0.0870 | 0.0752 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |

Table 30: Evaluation results for BM-Case1 with different carrier frequency for Tx beam predictions with fixed pattern

|  |  |  | CATT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of [beams/beam pairs] in Set A | 32 |  |  |  |
|  |  | Number of [beams/beam pairs] in Set B | 8 |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beam pairs |  |  |  |
|  |  | Model output | The best DL Tx beam ID |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |
|  |  | Testing | 40,000 |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |
|  |  | Model complexity | 103k |  |  |  |
|  |  | Computational complexity | 0.78M |  |  |  |
| Training |  | different carrier frequency | 20G |  | 30G |  |
| Inference |  | different carrier frequency | 20G | 30G | 20G | 30G |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9335 | 0.9301 | 0.9063 | 0.9406 |
|  |  | Top-3/1 | 0.9997 | 0.9710 | 0.9795 | 0.9993 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0529 | 0.0326 | 0.1801 | 0.0579 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |

Table 31Evaluation results for BM-Case1 with different carrier frequency for Tx beam predictions with random pattern with beam ID

|  |  |  | CATT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beams in Set A | 32 |  |  |  |
|  |  | Number of beams in Set B | 8 |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beams with beam ID |  |  |  |
|  |  | Model output | The best DL Tx beam ID |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |
|  |  | Testing | 40,000 |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |
|  |  | Model complexity | 103k |  |  |  |
|  |  | Computational complexity | 0.78M |  |  |  |
| Training |  | different carrier frequency | 20G |  | 30G |  |
| Inference |  | different carrier frequency | 20G | 30G | 20G | 30G |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.5658 | 0.5375 | 0.5176 | 0.5857 |
|  |  | Top-3/1 | 0.8726 | 0.8529 | 0.8752 | 0.8760 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0460 | 0.1240 | 0.0237 | 0.0586 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |

Observation 10: When AI/ML model is trained and inferenced with different carrier frequency, e.g., the Set B of training is 20G and inference is 30G, the Top-1 accuracy can be comparable.

    - Different ISD

In the previous simulation settings, the ISD is 200. To verify the impact of different ISD on model generalization, we simulated the model generalization under different ISD, including ISD =200 and ISD =500.

Scenario#9: In the training phase, dataset of 32 beam pairs is generated with ISD 200m. In the inference phase, we use the data sets of 32 beam pairs but ISD is different, 200m and 500m respectively. And in the training phase, input set B still uses the ISD is 500.The simulation results of accuracy related KPIs using fixed pattern are shown in Table 32 , where fixed pattern 1 is used for training. Table 33 shows the simulation results of KPIs related to the accuracy of using random pattern of Opt C(Set B is randomly changed among Set A beams pairs)  with beam ID.

Scenario#10: In the training phase, dataset of 32 beam pairs is generated with ISD 200m. In the inference phase, we use the data sets of 32 beam pairs but ISD is different, 200m and 500m respectively. And in the training phase, input set B still uses the ISD is 500.The simulation results of accuracy related KPIs using fixed pattern are shown in Table 34, where fixed pattern 1 is used for training. Table 35 shows the simulation results of KPIs related to the accuracy of using random pattern of Opt C(Set B is randomly changed among Set A beams pairs)  with beam ID.

Table 32: Evaluation results for BM-Case1 with different ISD for Tx-Rx beam pair predictions with fixed pattern

|  |  |  | CATT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |  |  |  |
|  |  | Number of beam pairs in Set B | 32 |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beam pairs |  |  |  |
|  |  | Model output | The best DL Tx and Rx beam(beam pairs) ID |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |
|  |  | Testing | 40,000 |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |
|  |  | Model complexity | 206k |  |  |  |
|  |  | Computational complexity | 0.76M |  |  |  |
| Training |  | different ISD | 200 |  | 500 |  |
| Inference |  | different ISD | 200 | 500 | 200 | 500 |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9147 | 0.9022 | 0.9035 | 0.9198 |
|  |  | Top-3/1 | 0.9969 | 0.9946 | 0.9939 | 0.9973 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0003 | 0.0005 | 0.0005 | 0.0003 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |

Table 33: Evaluation results for BM-Case1 with different ISD for Tx-Rx beam pair predictions with random pattern with beam ID

|  |  |  | CATT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beam pairs in Set A | 128 |  |  |  |
|  |  | Number of beam pairs in Set B | 32 |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beam pairs with beam ID |  |  |  |
|  |  | Model output | The best DL Tx and Rx beam(beam pairs) ID |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |
|  |  | Testing | 40,000 |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |
|  |  | Model complexity | 206k |  |  |  |
|  |  | Computational complexity | 2.84M |  |  |  |
| Training |  | different ISD | 200 |  | 500 |  |
| Inference |  | different ISD | 200 | 500 | 200 | 500 |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.6135 | 0.6219 | 0.6029 | 0.6194 |
|  |  | Top-3/1 | 0.8791 | 0.9001 | 0.8954 | 0.9165 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0468 | 0.0502 | 0.0479 | 0.0391 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |

Table 34: Evaluation results for BM-Case1 with different ISD for Tx beam predictions with fixed pattern

|  |  |  | CATT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beams in Set A | 32 |  |  |  |
|  |  | Number of beams in Set B | 8 |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beam pairs with beam ID |  |  |  |
|  |  | Model output | The best DL Tx beam ID |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |
|  |  | Testing | 40,000 |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |
|  |  | Model complexity | 103k |  |  |  |
|  |  | Computational complexity | 0.78M |  |  |  |
| Training |  | different ISD | 200 |  | 500 |  |
| Inference |  | different ISD | 200 | 500 | 200 | 500 |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.9814 | 0.9810 | 0.9806 | 0.9802 |
|  |  | Top-3/1 | 0.9999 | 0.9998 | 0.9998 | 0.9998 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0002 | 0.0002 | 0.0002 | 0.0002 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |

Table 35: Evaluation results for BM-Case1 with different ISD for Tx beam predictions with random pattern with beam ID

|  |  |  | CATT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Assumptions |  | Number of beams in Set A | 32 |  |  |  |
|  |  | Number of beams in Set B | 8 |  |  |  |
|  |  | Baseline scheme | Option 1 (exhaustive beam sweeping) |  |  |  |
| AI/ML model input/output |  | Model input | RSRP of beam pairs with beam ID |  |  |  |
|  |  | Model output | The best DL Tx beam ID |  |  |  |
| Data Size |  | Training | 150,000 |  |  |  |
|  |  | Testing | 40,000 |  |  |  |
| AI/ML model |  | Short model description | ResNet based |  |  |  |
|  |  | Model complexity | 103k |  |  |  |
|  |  | Computational complexity | 0.78M |  |  |  |
| Training |  | different ISD | 200 |  | 500 |  |
| Inference |  | different ISD | 200 | 500 | 200 | 500 |
| Evaluation results | Beam prediction accuracy (%) | Top-1 | 0.7109 | 0.7086 | 0.7063 | 0.7095 |
|  |  | Top-3/1 | 0.9302 | 0.9201 | 0.9211 | 0.9300 |
|  | L1-RSRP Diff | Average L1-RSRP diff | 0.0439 | 0.0573 | 0.1287 | 0.0582 |
|  | System performance | RS overhead Reduction (%) | 75% |  |  |  |

Observation 11: When AI/ML model is trained and inferenced with different ISD, e.g., the Set B of training of ISD is 200 and inference is 500, the Top-1 accuracy can be comparable.

Proposal 6: For the study of generalization, we should focus on the influence of different patterns or different Set B for generalization.

- Conclusions

In this contribution, the simulation results for BM-Case1 are given and discussed. The observations and proposals are summarized as follows:

Observation 1: For Beam pair prediction, using pre-configured patterns for training and inference, the Top-1 accuracy can be greater than 93%.

Observation 2: For DL Tx prediction with random pattern, using beam ID as additional information can improve prediction accuracy.

Observation 3: For DL Tx prediction, the prediction accuracy is greatly reduced when using different pre-configured patterns for training and inference.

Observation 4: For DL Tx prediction with fixed pattern, the Top-1 accuracy can be greater than 98%.

Observation 5: The performance of randomly selecting Rx beam is worse than that of selecting the best Rx beam, but selecting the best Rx beam requires additional selection steps, which will increase the complexity of UE implementation.

Observation 6: For DL Tx RSRP prediction with fixed pattern, the prediction accuracy of Top-1 is higher than 93% for DL Tx beam RSRP prediction.

Observation 7: When AI/ML model is trained and inferenced with different Set B, e.g., the Set B of training is 32 and inference is 16, the Top-1 accuracy is degraded.

Observation 8: When AI/ML model is trained and inferenced with different scenario configurations, e.g., the Set B of training is Uma and inference is Umi, the Top-1 accuracy can be comparable.

Observation 9: When AI/ML model is trained and inferenced with different UE distributions, e.g., the Set B of training of UE distributions is 0% and inference is 50%, the Top-1 accuracy can be comparable.

Observation 10: When AI/ML model is trained and inferenced with different carrier frequency, e.g., the Set B of training is 20G and inference is 30G, the Top-1 accuracy can be comparable.

Observation 11: When AI/ML model is trained and inferenced with different ISD, e.g., the Set B of training of ISD is 200 and inference is 500, the Top-1 accuracy can be comparable.

Proposal 1: For DL Tx-Rx beam pair prediction, Option A is selected for the definition of Top-1 genie-aided Tx beam, i.e., the Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams.

Proposal 2: For DL Tx beam prediction, Option B is selected for the definition of Top-1 genie-aided Tx beam, i.e., the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s).

- The specific Rx beam is one DL Rx beam selected from all Rx beams; company can report which specific Rx beam it is.

Proposal 3: For both BM-Case1 and BM-Case2, further study the following options to apply AI/ML in beam management procedure:

- Option 1: Use AI/ML model to predict Top-1 DL Tx beam in Set A

  - For UE side AI/ML, the Top-1 DL Tx beam can be recommended by UE for DL data transmission

  - For gNB AI/ML, the Top-1 DL Tx beam can be used for DL data transmission

  - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam

- Option 2: Use AI/ML model to predict Top-K Tx beams in Set A

  - For UE side AI/ML, the Top-K Tx beams can be recommended by UE for P2

  - For gNB AI/ML, the Top-K DL Tx beam can be used for P2

  - gNB selects one DL Tx beam for DL data transmission using P2 or up to gNB implementation

  - P3 may be needed for Rx beam sweeping for the predicted Top-1 DL Tx beam

- Option 3: Use AI/ML model to predict Top-1 Tx-Rx beam pair in Set A

  - For UE side AI/ML,

    - the Top-1 Tx-Rx beam pair can be used for DL data reception

    - the Tx beam of the Top-1 Tx-Rx beam pair can be recommended by UE for DL data transmission

    - P3 procedure is not needed

  - For gNB side AI/ML,

    - the Tx beam of the Top-1 Tx-Rx beam pair can be used for DL data transmission

    - the Rx beam of the Top-1 Tx-Rx beam pair can be recommended by gNB for DL data reception

- Option 4: Use AI/ML model to predict Top-K Tx-Rx beam pairs in Set A

  - For UE side AI/ML,

    - the Top-K Tx-Rx beam pairs can be used for DL data reception

    - the Top-K Tx beams of Top-K Tx-Rx beam pairs can be recommended by UE for P2

      - gNB selects one DL Tx beam for DL data transmission using P2 or up to gNB implementation

    - P3 procedure is not needed

  - For gNB side AI/ML,

    - the Top-K Tx beams of the Top-K Tx-Rx beam pairs can be used for P2

      - gNB selects one DL Tx beam for DL data transmission using P2 or up to gNB implementation

    - the Rx beam corresponding with the selected DL Tx beam can be recommended by gNB

    - P3 procedure is not needed

- Other options are not precluded.

Proposal 4: For BM-Case1, random pattern of Opt B, i.e., Set B is randomly changed among pre-configured patterns, can be used to improve beam pair prediction accuracy.

Proposal 5: The patterns for training and inference should be pre-configured or configured to cover all beams of Set A as much as possible.

Proposal 6: For the study of generalization, we should focus on the influence of different patterns or different Set B for generalization.

- References

- R1-2208969, Report of RAN1#110bis-e , Online, October 10th – 19th, 2022.

- R1-2211191, Report of RAN1#111meeting, Chair, RAN1#110, Toulouse, France, November 14th – 18th, 2022.

- Annex

Table 36: Simulation assumptions for Dense Urban scenario for Alt.2 of BM-Case1

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz SCS: 120 kHz |
| Deployment | 200m ISD, 2-tier model with wrap-around (19 sites, 3 sectors/cells per site) |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | 3km/h |
| UE distribution | 10 UEs per sector/cell for full buffer traffic 80% indoor ,20% outdoor as in TR 38.901 |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | Antenna setup and port layouts at UE: [1,2,1,4,2,1,1], 2 panels (left, right) |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| BF scheme | DFT codebook |
| BS Tx Power | 40 dBm |
| Maximum UE Tx Power | 23 dBm |
| UE receiver Noise Figure | 10 dB |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |