3GPP TSG RAN WG1 #112			R1-2300728

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:		9.14.1

Source:	China Telecom

Title:	Discussion on PRACH coverage enhancement

Document for:		Discussion

Introduction

In RAN #111 [1], the following agreements were achieved for PRACH coverage enhancements as follows:

Agreement For multiple PRACH transmissions with same Tx beam, support to differentiate at least between multiple PRACH transmissions and single PRACH transmissions. Agreement For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, consider one or multiple of the following options. - Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs. - Option 2: Multiple PRACH are transmitted on separate ROs. - Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs. - Other options are not precluded. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. Agreement For multiple PRACH transmissions with same Tx beam, down-select one option from the following options. - Option 1: gNB can only configure one value for the number of multiple PRACH transmissions. - Option 2: gNB can configure one or multiple values for the number of multiple PRACH transmissions. - FFS: details Agreement - For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt. - Note: whether to support multiple numbers of PRACH transmissions is separately discussed.

In this contribution, we discuss PRACH coverage enhancement.

Discussion on multiple PRACH transmission with same beam

For multiple PRACH transmissions with same beam, it can be deemed as PRACH repetitions on several ROs associated with the same SSB. In this section, we mainly discuss multiple PRACH transmission and detection, indication and determination of the number of PRACH repetitions, RAR window design and RA-RNTI calculation, and some other issues.

  - Multiple PRACH transmissions and detection

Differentiation between legacy single PRACH transmission and multiple PRACH transmissions

In RAN1 #111 meeting, it was agreed to support to differentiate at least between multiple PRACH transmissions and single PRACH transmissions for multiple PRACH transmission. In this way, gNB can know which ROs are utilized to transmit the multiple PRACH transmission, then joint detection can be performed. The following options are agreed to be considered:

- Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs.

- Option 2: Multiple PRACH are transmitted on separate ROs.

- Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs.

For Option 1, no additional RO is needed, it seems to be a good choice in terms of resource utilization. However, we need to review this option from at least two more aspects.

- Firstly, from preamble perspective, the question is: will preamble arrangement be an issue? Currently, preambles have already been divided for variety of usages, e.g., CBRA/CFRA, 2-Setp/ 4-Step RACH procedure, Msg. 3 repetitions and etc. To deal with issue, a feature combination-based mechanism was introduced in Rel-17, i.e., IE FeatureCombination was introduced to indicate a feature or a combination of features to be associated with a set of random access resources (i.e. an instance of FeatureCombinationPreambles). It can be seen that there are still some spare spaces in this IE, which may be used for multiple PRACH transmission. Although may be complex, but from preamble arrangement perspective, Option 1 seems workable.

- Secondly, from RO perspective, the question is: are there enough ROs for a number of PRACH transmissions with same beam? For current spec, each valid RO is associated with at least one SSB, and current SSB to RO mapping only ensures SSBs are mapped at least once to the PRACH occasions within the association period. It doesn’t ensure the number of ROs to be enough for PRACH repetitions, e.g., 2 or 4 PRACH repetitions. Take PRACH format B4 as an example, part of random access configurations for FR2 as described in TS 38.211 is copied and shown in the following table. It can be known that if FDMed ROs is not considered, the number of ROs in a PRACH configuration period is generally between 4~20, where the maximum number is 40. Consider that the maximum FDM factor is 8, with carefully selection of FDM factor and number of SSB per RO, there may be enough ROs for multiple PRACH transmissions. Thus, from RO perspective, Option 1 seems also workable.

From the above analysis, it can be known that although Option 1 is workable, it has some limitations. The mechanism may be a little bit complex and not that flexible. Thus, we suggest to deprioritize Option 1.

Tab. 1. Part of random access configurations for FR2 and unpaired spectrum from TS 38.211

| PRACH Config. Index | Preamble format | nf mod x=y |  | Slot number | Starting symbol | Number of PRACH slots within a 60 kHz slot | number of time-domain RO within a PRACH slot | PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | x | y |  |  |  |  |  |
| 112 | B4 | 16 | 1,2 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 113 | B4 | 16 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 114 | B4 | 8 | 1,2 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 115 | B4 | 8 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 116 | B4 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 1 | 12 |
| 117 | B4 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 |
| 118 | B4 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 119 | B4 | 4 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 120 | B4 | 2 | 1 | 7,15,23,31,39 | 2 | 2 | 1 | 12 |
| 121 | B4 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 |
| 122 | B4 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 |
| 123 | B4 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 124 | B4 | 1 | 0 | 19, 39 | 2 | 2 | 1 | 12 |
| 125 | B4 | 1 | 0 | 17, 19, 37, 39 | 0 | 1 | 1 | 12 |
| 126 | B4 | 1 | 0 | 24,29,34,39 | 2 | 1 | 1 | 12 |
| 127 | B4 | 1 | 0 | 9,19,29,39 | 2 | 2 | 1 | 12 |
| 128 | B4 | 1 | 0 | 9,19,29,39 | 0 | 2 | 1 | 12 |
| 129 | B4 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 1 | 12 |
| 130 | B4 | 1 | 0 | 7,15,23,31,39 | 0 | 2 | 1 | 12 |
| 131 | B4 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 132 | B4 | 1 | 0 | 23,27,31,35,39 | 2 | 2 | 1 | 12 |
| 133 | B4 | 1 | 0 | 9,11,13,15,17,19 | 0 | 1 | 1 | 12 |
| 134 | B4 | 1 | 0 | 3,5,7,9,11,13 | 2 | 1 | 1 | 12 |
| 135 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 |
| 136 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 2 | 1 | 12 |
| 137 | B4 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 2 | 2 | 1 | 12 |
| 138 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 |
| 139 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 1 | 12 |
| 140 | B4 | 1 | 0 | 3, 5, 7, …, 23,25 | 2 | 1 | 1 | 12 |
| 141 | B4 | 1 | 0 | 3, 5, 7, …, 23,25 | 0 | 2 | 1 | 12 |
| 142 | B4 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 1 | 12 |
| 143 | B4 | 1 | 0 | 0, 1, 2,…, 39 | 2 | 1 | 1 | 12 |

Proposal 1: For multiple PRACH transmissions with same beams, to differentiate the multiple PRACH transmissions with single PRACH transmission, deprioritize Option 1, i.e., Multiple PRACH are transmitted with separate preamble on shared ROs.

For Option 2, different ROs are utilized to differentiate the multiple PRACH transmissions from legacy single PRACH transmission. There may be two approaches, the first one is to separate the legacy ROs into two parts, one for legacy PRACH transmission, the other part for multiple PRACH transmissions. However, this may cause some non-back forward compatibility issues.

The second approach is to define some new ROs, the network is able to differentiate the multiple PRACH transmissions with legacy single PRACH transmission based on the time-frequency location of ROs. One possible way is to determine the separate ROs based on a time or frequency offset and legacy PRACH configuration as illustrated in Fig.1. For time domain offset based resource configuration, an IAB-like approach can be a reference [2], e.g., replace the variable y used in nSFN mod x=y of random access configuration table by y=y+Δy mod x where Δy  is the time domain offset. It should be noted that, all the ROs should be within a SSB to RO association period. Moreover, keeping in mind the intention of PRACH repetition is to improve the coverage, thus the repetitions should not overlap in time domain to avoid power sharing among repetitions. Another thing to be considered is the mapping between ROs and different number of repetitions. For example, there are four available ROs for maximum 4 PRACH repetitions, the configured number of PRACH repetitions is within the set of {1, 2, 4}. If the number of PRACH repetitions is determined as 2, then which two of the four ROs will be utilized for repetitions? If they are randomly selected, it will be hard for the network to perform the joint detection. Thus, some rules are suggested to be predefined.

Fig. 1. Illustration of separate ROs based on a time or frequency offset and legacy PRACH configuration

Another possible way is to configure separate PRACH configurations for different number of PRACH repetitions or different CE levels, similar as the PRACH repetition mechanism introduced for BL/CE UEs [3]. However, it may need pretty much spec. effort since a lot of parameters need to be discussed and specified, including separate time domain/frequency domain resources per each number of repetitions, mapping between SSB and ROs and etc.

Proposal 2: For Option 2, i.e., multiple PRACH are transmitted on separate ROs, new ROs are configured for multiple PRACH transmissions.

For Option 3, Partial of the ROs are shared by partial of multiple PRACH transmissions and legacy single PRACH transmission, while other multiple PRACH are transmitted on separate ROs. In this case, separate preambles are still needed to differentiate the multiple PRACH transmissions with legacy single PRACH transmission. If shared preambles are utilized, it may cause some trouble. As illustrated in Fig.2, RO #0 is the shared RO, and RO #1~#3 are the separated ROs. In this case, we assume 4 PRACH transmission is performed and transmitted on RO #0~#3. If shared preamble is used, there may be the following situations:

- Case 1: If gNB detects this preamble on RO #0, it may not know whether this preamble is transmitted via a legacy UE or an enhanced UE. Anyway, gNB will follow the legacy RAR procedure and transmit Msg.2. Thus, the RAR monitoring window needs to start as legacy. In addition, since gNB knows the four ROs are used for multiple PRACH transmission, it can joint detect the preamble across RO #0~RO #3, if it detects the same preamble as that on RO #0. Then, gNB cannot tell whether there is a single PRACH transmission or a multiple PRACH transmissions.

- Case 2: If gNB only detects some preamble across RO #0~RO #3, while doesn’t detect any preamble on RO #0, then it seems no problem, gNB can know this is a multiple PRACH transmissions. The monitoring window can start after the end of multiple PRACH transmissions.

Fig. 2. Illustration of RO configuration for Option 3

Based on the analysis of the above two cases, option 3 may have a requirement on the RAR window design, that is the RAR window needs to have the same start position as legacy and last for a longer period considering gNB may transmit the RAR after the shared RO or after the end of multiple PRACH transmissions.

Based on the above analysis, we have the following proposal.

Proposal 3: For multiple PRACH transmissions with same beams, to differentiate the multiple PRACH transmissions with single PRACH transmission, support Option 2, i.e., multiple PRACH are transmitted on separate ROs.

Differentiation between different number of multiple PRACH transmissions

Another issue would be, whether it is needed to differentiate between different number of multiple PRACH transmissions. If it is needed, then on top of using separate ROs or/and preambles to differentiate between legacy single PRACH transmission and multiple PRACH transmissions, separate ROs or/and preambles are also needed between different number of multiple PRACH transmission, which may make the design too complicate.

Thus, we prefer the design that for different number of multiple PRACH transmissions, they share the ROs and/or preambles. Meantime, some predefined rules may be needed on the selection of ROs for different number of multiple PRACH transmissions to make the RAR monitoring not too complex.

During the discussion in RAN1 #111 meeting, the following FL proposal was discussed:

Proposal 9-new-b For multiple PRACH transmissions with same Tx beam, a term of "RO group" is introduced to facilitate further discussion. For each RACH attempt, UE determines and selects one RO group for multiple PRACH transmissions, where RO(s) for a specific number of multiple PRACH transmissions are configured [, and transmit PRACH in the RO(s) within the RO group]. [Note 1: Determination and selection of RO group (including related configuration) is a separate discussion, including which WG should work on the configuration aspects, details of the signaling structures (if any).] Note 2: Each RO in one RO group is associated with the same SSB(s). Note 3: Whether/How to capture the RO group in the spec. is a separate discussion.

We think the introduced concept of “RO group” will facilitate the discussion of differentiation between different number of multiple PRACH transmissions. Moreover, the concept of “RO group” may also facilitate the discussion of RAR window, where the reference slot for the start of RAR window can be the end of one RO group. There may be at least two approaches for RO groups configuration.

The first one is, the RO groups for different number of multiple PRACH transmissions is separately configured. The ROs within the RO groups for different number of multiple PRACH transmissions can be overlapped or totally non-overlapped.

The second one is, the RO group for large number of multiple PRACH transmissions contains RO group for smaller number of multiple PRACH transmissions. In fact, the second approach can also can be realized by the first one.

Then, from UE side, after it determines the number of multiple PRACH transmission, it can select one corresponding RO group to transmit the PRACHs.

We think the concept of “RO group” may help to align gNB’s and UE’s understanding about where the multiple PRACH transmissions may occur and when the RAR window starts. Thus, we have the following proposal.

Proposal 4: For multiple PRACH transmissions with same Tx beam, introduce a term of "RO group". For each RACH attempt, UE determines and selects one RO group for multiple PRACH transmissions, where RO(s) for a specific number of multiple PRACH transmissions are configured.

Note 1: Determination and selection of RO group (including related configuration) is a separate discussion.

Note 2: Each RO in one RO group is associated with the same SSB(s).

  - Indication and determination of the number of PRACH repetitions

Whether different number of multiple PRACH transmissions is needed?

From our point of view, we think different numbers of multiple PRACH transmissions are needed, because there may be different coverage performance within one cell. We can first determine a set of numbers for multiple PRACH transmission, the network then can configure partial of them based on the network conditions.

Proposal 5: Support that gNB can configure one or multiple values for the number of multiple PRACH transmissions.

Indication and determination of the number of PRACH repetitions

If multiple values for the number of multiple PRACH transmissions are supported and configured. To determine the number of PRACH repetitions, some RSRP thresholds may need to be defined, similar as the mechanism defined for NB-IoT UE and BL/CE UEs in enhanced coverage. Take three RSRP thresholds as an example, namely RSRP0, RSRP1 and RSRP2, where RSRP0 >RSRP1 >RSRP2. Meantime, the network configures a set of PRACH repetitions as {R0, R1, R2}, where R2 >R1 >R0. It should be noted that “R0, R1, R2” can be selected from a larger value set, e.g., {R0, R1, R2, R3, R4, R5…}. Then, if the UE measured RSRP is less than RSRP2, the number of PRACH repetitions is determined as R2; else if the UE measured RSRP is less than RSRP1, the number of PRACH repetitions is determined as R1; else if the UE measured RSRP is less than RSRP0, the number of PRACH repetitions is determined as R0; else, no PRACH repetition is performed, legacy PRACH transmission is followed.

Moreover, as discussed during RAN1 #110-e meeting, some companies propose that multiple PRACH transmission is only applied when the transmission power or number of PRACH retransmissions reaching a threshold. From our point of view, we think this may bring much latency as: UE first performs legacy PRACH transmission, if failed for several times, then it applies multiple PRACH transmission. We think this is not straightforward. We prefer to directly use the SSB-RSRP to determine the number of multiple PRACH transmission.

  - RAR window and RA-RNTI calculation

In current spec., UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a RAR window after the PRACH transmission. If PRACH repetition is enabled, the design of RAR window may need to be modified. Based on the agreement in RAN1 110b-e meeting, two options for RAR monitoring were agreed to be considered. For Option 2, we think there are two sub-options considering different start position of the RAR window as follows:

- Option 1: One RAR window per each PRACH transmission, the RAR window follows the legacy design.

- Option 2: Only one RAR window for all of the multiple PRACH transmissions.

  - Option 2-1: The start position of the window follows legacy design.

  - Option 2-2: The start position of the window is after the last PRACH transmission.

For Option 1, as illustrated in Fig. 3, for each PRACH repetition, there is one corresponding RAR window. Each RAR window is generated following legacy mechanism, and each related RA-RNTI is calculated as legacy as follows [4]:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

If there is some overlapping between two or more RAR windows, e.g., within T1,2, UE may need to detect the DCI scrambled by RA-RNTI0 and RA-RNTI1, respectively, where RA-RNTI0 and RA-RNTI1 are calculated based on the time & frequency position of RO #0 and RO #1. Moreover, based on Option 1, early termination of PRACH repetition can be realized. However, since we already have an agreement to support to differentiate between multiple PRACH transmissions and single PRACH transmissions, we think the benefit of Option 1 seems small.

Fig. 3. Illustration of Option 1

For Option 2, as illustrated in Fig.4, there is only one RAR window which covers all the four repetitions. Currently, the length of RAR window defined in TS 38.331 follows that: lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and lower than or equal to 40 ms when Msg2 is transmitted with shared spectrum channel access. The length of SSB to RO association period is within the set of {10, 20, 40, 80, 160} ms. It can be then seen that a single RAR window with current length is hard to cover all the repetitions. Thus, if Option 2 is selected, the length of RAR window needs to be extended. Moreover, the RA-RNTI calculation needs to be discussed. This issue is also related to the RO configuration for multiple PRACH transmissions as discussed in Section 2.1.

Observation 1: For Option 2-1 (Single RAR window is utilized to cover all the repetitions, the start position of the window follows legacy design), the length of RAR window needs to be extended.

Fig. 4. Illustration of Option 2-1

For Option2-2, as illustrated in Fig.5, there is only one RAR window after the PRACH repetitions. The RAR window design can largely follow the legacy one. Mainly two issues need to be discussed. The first issue is about the start position of RAR window, it can be simply redefined as after the last PRACH repetitions. If the concept of “RO group” is introduced, then the start position of RAR window can be after the corresponding RO group. The second issue is about the RA-RNTI calculation, a pre-defined rule is needed. For example, similar with Option 2-1, we can use the RA-RNTI calculated based on one of the predefined RPACH repetition, e.g., the first or last PRACH repetition.

Fig. 5. Illustration of Option 2-2

Proposal 6: For RAR window and RA-RNTI calculation, slightly prefer Option 2-2 (Single RAR window is utilized to cover all the repetitions, the start position of the window is after the last PRACH transmission).

  - PRACH repetition pattern

In order to improve the coverage performance, PRACH repetitions should not overlap in time domain to avoid power sharing among repetitions.

If the concept of “RO group” is introduced, then there can be ROs w/ different starting RB positions within one RO group. In this way, multiple PRACH transmissions w/ frequency hopping can be realized. We think whether to introduce frequency hopping for multiple PRACH transmissions depends on whether it will increase the blind detection complexity on gNB side. If the blind detection complexity is not increased, we are OK to support frequency hopping for multiple PRACH transmissions.

  - Power control

As discussed in RAN1 #110b-e meeting, two options for power ramping were discussed regarding whether the power ramping is performed during one RACH attempt as follows.

- Option 1: Transmission power ramping is not applied during the multiple PRACH transmissions.

- Option 2: Transmission power ramping can be applied per PRACH transmission during the multiple PRACH transmissions.

From our point of view, it’ll be kind of weird to perform power ramping during one RACH attempt. Although we can understand the intention, but we are not sure whether there will be some technical/implementation problem. Thus, we prefer option 1 that power ramping is not applied during the multiple PRACH transmission within one RACH attempt. The power ramping can be applied between different RACH attempts, e.g., between the initial transmission and re-transmissions, and between retransmissions. Another issue would be whether the same measurement of the same reference signal is utilized to calculate the pathloss for all PRACH transmission within one RACH attempt. From our point of view, we think the pathloss for different PRACH transmission may be different within one RACH attempt, it depends on the time duration of the transmission, but we think at least same reference signal is utilized for pathloss calculation. Thus, we have the following proposal.

Proposal 7: For multiple PRACH transmissions with same beam in one RACH attempt, transmission power ramping is not applied during the multiple PRACH transmissions, same measurement of the same reference signal is utilized to calculate the pathloss for all PRACH transmission within one RACH attempt.

  - SSBs to ROs mapping

For current SSB to RO mapping, it follows the following order:

- First, in increasing order of preamble indexes within a single PRACH occasion

- Second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

- Third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

- Fourth, in increasing order of indexes for PRACH slots

It is known that the number of SS/PBCH block indexes associated with one PRACH occasion is determined by higher layer parameter SSB-PerRACH-OccasionAndCB-PreamblesPerSSB for 4-step RACH. Assume a number N of SS/PBCH block indexes associated with one PRACH occasion is provided, then N can be smaller than 1, equal to or larger than 1.

For N≤1, for example, assume N = 1/4, then it indicates that one SSB is mapped to 4 ROs. To reduce the access latency, the network can configure FDM-ed ROs by Msg1-FDM, for example, configure Msg1-FDM=4. If we consider 4 SSBs, then follows current SSB to RO mapping order, the pattern is illustrated in Fig. 6. Since the mapping to FDMed ROs is prior to TDMed ROs, which may result in less consecutive ROs associated with the same SSB(s) in time domain. Thus, it can be seen that there may be barely TDMed ROs for multiple PRACH transmissions.

Fig. 6. Illustration of current SSB to RO mapping

- For N>1, it indicates multiple SSBs are mapped to 1 RO. If FDMed ROs are configured at the same time, then it requires gNB to detect multiple SSBs in a same time instance. For FR2, since analog beamforming is generally utilized at the gNB side, it’s hard to generate many beam directions in the same time. Thus, this kind of configuration may not practical compares to the previous one.

- Based on the above analysis, it can be known that for multiple PRACH transmissions, a “TDMed prior to FDMed” SSB to RO mapping approach may be more appropriate. Thus, we have the following proposal.

Proposal 8: If new ROs are defined for multiple PRACH transmissions, SSB indexes are mapped to valid PRACH occasions in the following order:

First, in increasing order of preamble indexes within a single PRACH occasion

Second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

Third, in increasing order of indexes for PRACH slots

Fourth, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

  - Retransmission of Multiple PRACH transmissions

The number of PRACH transmissions for PRACH retransmission

For retransmission for multiple PRACH transmissions, if SSB or CSI-RS selected is not changed from the selection in the last random access preamble transmission, we think the number of PRACH retransmissions in one RACH attempt can be equal to or larger than the previous multiple PRACH transmissions. It depends on the configured number of multiple PRACH transmissions. Else, the number of multiple PRACH retransmissions is determined in the same way as first RACH attempt.

On the other hand, in case some UE is under very poor coverage, it continues to perform large number of multiple PRACH transmissions, it will cause continuing interference and increase the possibility of collision. To deal with this issue, we think the maximum number of RACH attempts for multiple PRACH transmissions should be limited and less than that for single PRACH transmission.

Proposal 9: If SSB or CSI-RS selected is not changed from the selection in the previous multiple PRACH transmissions, the number of PRACH retransmissions in one RACH attempt is equal to or larger than that of the previous multiple PRACH transmissions. Else, the number of multiple PRACH retransmissions in one RACH attempt is determined in the same way as first RACH attempt.

Proposal 10: The maximum number of RACH attempts for multiple PRACH transmissions should be limited and less than that for single PRACH transmission.

Power control

As we discussed in Section 2.5, the power ramping can be applied between different RACH attempts. For detailed power calculation for each PRACH retransmissions, we think the power control calculation for coverage enhancement in eMTC can be a starting point as follows:

- Firstly, set PREAMBLE_RECEIVED_TARGET_POWER to: preambleInitialReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep;

- Secondly, set PREAMBLE_RECEIVED_TARGET_POWER to: PREAMBLE_RECEIVED_TARGET_POWER - 10 * log10(numRepetitionPerPreambleAttempt);

- Thirdly, the transmission power of each PRACH transmission is calculated as PPRACH=minPCMAX,PREAMBLE_RECEIVED_TARGET_POWER+PL

- where PREAMBLE_TRANSMISSION_COUNTER is a counter for power ramping; numRepetitionPerPreambleAttempt indicates the number of multiple PRACH transmissions.

Proposal 11: The power for multiple PRACH transmissions and retransmissions can be calculated as follows:

Firstly, set PREAMBLE_RECEIVED_TARGET_POWER to: preambleInitialReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep;

Secondly, set PREAMBLE_RECEIVED_TARGET_POWER to: PREAMBLE_RECEIVED_TARGET_POWER - 10 * log10(numRepetitionPerPreambleAttempt);

Thirdly, the transmission power of each PRACH transmission is calculated as PPRACH=minPCMAX,PREAMBLE_RECEIVED_TARGET_POWER+PL

Discussion on multiple PRACH transmission with different beams

For multiple PRACH transmissions with different beams, we think the first thing needs to be discussed is that whether the multiple PRACH transmission can be associated with different SSBs. Secondly, whether different preambles can be utilized for multiple PRACH transmission. Thirdly, how to determine whether to perform multiple PRACH transmission with the same beam or different beams. In this section, we mainly discuss the above issues.

  - Association between SSB and multiple PRACH transmissions

Multiple PRACH transmissions are associated with the same SSB

Before we discuss the detailed schemes, we need first to consider the scenario that UE may benefit from multiple PRACH transmission with different beams while associated with the same SSB. One potential use case is that beam correspondence is not guaranteed, thus UE need to try different Tx beams.

For multiple PRACH transmission associated with the same SSB, the legacy behavior of cell search is maintained, i.e., UE select one SSB that satisfies a threshold and perform PRACH transmissions on the ROs associated with the selected SSB.

Moreover, PRACH resource configuration and RAR window design can be the same as that of PRACH repetitions with the same beam, it seems no additional spec. effort is needed. However, the calculation of RA-RNTI may need further discussion. Here, we take Option 3 for RAR window design as an instance. As illustrated in Fig. 7, four different beams are utilized for PRACH transmission. Then, the best situation is that the network detects the preamble on one of the ROs and calculated the RA-RNTI based on this RO. In this way, UE needs to try different RA-RNTIs to detect DCI during the RAR window, once UE succeeds, it will know the best beam for UL transmission. In another case, if the network performs joint detection, and utilize a predefined rule to calculate the RA-RNTI, e.g., based on the first PRACH transmission, then UE will not know which beam to use for the subsequent UL transmission.

Fig. 7. Illustration of multiple PRACH transmission associated with the same SSB based on Option 3 for RAR window design

Multiple PRACH transmissions are associated with different SSBs

In this case, UE may select several SSBs instead of one SSB during the cell search procedure. During PRACH transmission, UE transmit a given number of PRACHs on ROs associated with the selected SSBs as illustrated in Fig. 8.

Fig. 8. Illustration of multiple PRACH transmission associated with different SSBs

Considering that there is a possibility that the UE selected SSBs are associated with the same RO, or the ROs associated with the selected SSBs are FDMed, for both of the cases, more discussion is needed about how to perform multiple PRACH transmissions.

In RAN1 #111, it is agreed to study the case that UE uses different TX beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS. For multiple PRACH transmissions with different beams while associated with different SSBs, considering that some modification is needed for cell search procedure and the limited time, we think there is no time for us to discuss this case.

Proposal 12: Make a conclusion that UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs /CSI-RSs is not considered in Rel-18.

Note: the different SSBs/CSI-RSs are not associated with the same RO.

Conclusions

In this contribution, we discuss PRACH coverage enhancement and have following proposals:

Observation 1: If Option 2-1 (Single RAR window is utilized to cover all the repetitions, the start position of the window follows legacy design) is selected, the length of RAR window needs to be extended.

Proposal 1: For multiple PRACH transmissions with same beams, to differentiate the multiple PRACH transmissions with single PRACH transmission, deprioritize Option 1, i.e., Multiple PRACH are transmitted with separate preamble on shared ROs.

Proposal 2: For Option 2, i.e., multiple PRACH are transmitted on separate ROs, new ROs are configured for multiple PRACH transmissions.

Proposal 3: For multiple PRACH transmissions with same beams, to differentiate the multiple PRACH transmissions with single PRACH transmission, support Option 2, i.e., multiple PRACH are transmitted on separate ROs.

Proposal 4: For multiple PRACH transmissions with same Tx beam, introduce a term of "RO group". For each RACH attempt, UE determines and selects one RO group for multiple PRACH transmissions, where RO(s) for a specific number of multiple PRACH transmissions are configured.

Note 1: Determination and selection of RO group (including related configuration) is a separate discussion.

Note 2: Each RO in one RO group is associated with the same SSB(s).

Proposal 5: Support that gNB can configure one or multiple values for the number of multiple PRACH transmissions.

Proposal 6: For RAR window and RA-RNTI calculation, slightly prefer Option 2-2 (Single RAR window is utilized to cover all the repetitions, the start position of the window is after the last PRACH transmission).

Proposal 7: For multiple PRACH transmissions with same beam in one RACH attempt, transmission power ramping is not applied during the multiple PRACH transmissions, same measurement of the same reference signal is utilized to calculate the pathloss for all PRACH transmission within one RACH attempt.

Proposal 8: If new ROs are defined for multiple PRACH transmissions, SSB indexes are mapped to valid PRACH occasions in the following order:

First, in increasing order of preamble indexes within a single PRACH occasion

Second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

Third, in increasing order of indexes for PRACH slots

Fourth, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

Proposal 9: If SSB or CSI-RS selected is not changed from the selection in the previous multiple PRACH transmissions, the number of PRACH retransmissions in one RACH attempt is equal to or larger than that of the previous multiple PRACH transmissions. Else, the number of multiple PRACH retransmissions in one RACH attempt is determined in the same way as first RACH attempt.

Proposal 10: The maximum number of RACH attempts for multiple PRACH transmissions should be limited and less than that for single PRACH transmission.

Proposal 11: The power for multiple PRACH transmissions and retransmissions can be calculated as follows:

Firstly, set PREAMBLE_RECEIVED_TARGET_POWER to: preambleInitialReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep;

Secondly, set PREAMBLE_RECEIVED_TARGET_POWER to: PREAMBLE_RECEIVED_TARGET_POWER - 10 * log10(numRepetitionPerPreambleAttempt);

Thirdly, the transmission power of each PRACH transmission is calculated as PPRACH=minPCMAX,PREAMBLE_RECEIVED_TARGET_POWER+PL

Proposal 12: Make a conclusion that UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs /CSI-RSs is not considered in Rel-18.

Note: the different SSBs/CSI-RSs are not associated with the same RO.

References

- 3GPP RAN1 Chairman’s Notes, RAN1 meeting #111, Toulouse, France, November 14th – 18th, 2022.

- 3GPP TS 38.211 V17.2.0, “NR; Physical channels and modulation”, Jun. 2022.

- 3GPP TS 36.211 V17.1.0, “E-UTRA; Physical channels and modulation”, Mar. 2022.

- 3GPP TS 38.321 V17.1.0, “NR; Medium Access Control (MAC) protocol specification”, Jun. 2022.