3GPP TSG RAN WG1 #112                                                  R1-2300799

Athens, Greece, February 27th – March 3rd, 2023

Source:          ZTE

Title:            Discussion on positioning SRS power control in 38.213

Agenda item:     7.2

Document for:	Discussion and Decision

- Background

In Rel-16, NR wireless-dependent positioning methods were introduced. Positioning SRS is supported for UL and DL&UL positioning methods, such as UL-AOA, multi-RTT, UL-TDOA.

For power control of positioning SRS, the pathloss reference signal including serving cell SSB, neighbor cell SSB and PRS can be selected by RRC signaling. The corresponding description in current 38.213 [1] can be found as follows.

38.213: PLb,f,cqd is a downlink pathloss estimate in dB calculated by the UE, as described in clause 7.1.1 in case of an active DL BWP of a serving cell c, using RS resource indexed qd in a serving or non-serving cell for SRS resource set qs [6, TS 38.214]. A configuration for RS resource index qd associated with SRS resource set qs is provided by pathlossReferenceRS-Pos - if a ssb-IndexNcell is provided, referenceSignalPower is provided by ss-PBCH-BlockPower-r16 - if a dl-PRS-ResourceId is provided, referenceSignalPower is provided by dl-PRS-ResourcePower

However, there are some misalignments with RRC and LPP configuration. In this contribution, we analyze the issues and provide our views to address the problems.

- Discussion

In Rel-16 for positioning SRS power control, the pathloss reference signal can be SSB or PRS. If PRS is configured as the pathloss RS of the positioning SRS resource set by RRC parameter dl-PRS-r16 in 38.331 [2], the power of PRS resource set can be known based on PRS resource set ID and TRP ID informed in dl-PRS-r16.

As shown in 37.355 [3], the power of PRS is configured in PRS resource set basis. Then, after UE gets TRP ID (dl-PRS-ID-r16) and PRS resource set ID (dl-PRS-ResourceSetId-r16) informed in dl-PRS-r16 by RRC, UE can know the PRS transmission power value from the corresponding PRS configuration which is configured by LPP signaling. That is, even PRS resource ID (dl-PRS-ResourceId-r16) is not provided in dl-PRS-r16 by RRC, UE can still get the PRS power value.

As shown in 38.331, dl-PRS-ResourceId in dl-PRS-r16 is optionally provided. However, in the current 38.213, the condition is ‘a dl-PRS-ResourceId is provided’ which is not correct. The simple solution as follows is to replace dl-PRS-ResourceId by dl-PRS-r16 which includes TRP ID and PRS resource set ID.

- if a dl-PRS-r16 is provided, referenceSignalPower is provided by dl-PRS-ResourcePower

The corresponding draft CR can also be found in our companion contribution [4].

Proposal 1: When PRS is pathloss RS of positioning SRS, the PRS power value should be derived from RRC parameter dl-PRS-r16 rather than dl-PRS-ResourceId-r16.

38.331:

SRS-PosResourceSet-r16 ::=                  SEQUENCE {

srs-PosResourceSetId-r16                    SRS-PosResourceSetId-r16,

srs-PosResourceIdList-r16                   SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-PosResourceId-r16

OPTIONAL, -- Cond Setup

resourceType-r16                            CHOICE {

aperiodic-r16                               SEQUENCE {

aperiodicSRS-ResourceTriggerList-r16        SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-1))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL, -- Need M

...

},

semi-persistent-r16                         SEQUENCE {

...

},

periodic-r16                                SEQUENCE {

...

}

},

alpha-r16                                   Alpha                                                      OPTIONAL, -- Need S

p0-r16                                      INTEGER (-202..24)                                         OPTIONAL, -- Cond Setup

pathlossReferenceRS-Pos-r16                 CHOICE {

ssb-IndexServing-r16                        SSB-Index,

ssb-Ncell-r16                               SSB-InfoNcell-r16,

dl-PRS-r16                                  DL-PRS-Info-r16

}                                                                                                      OPTIONAL, -- Need M

...

}

DL-PRS-Info-r16  ::=                SEQUENCE {

dl-PRS-ID-r16                      INTEGER (0..255),

dl-PRS-ResourceSetId-r16           INTEGER (0..7),

dl-PRS-ResourceId-r16              INTEGER (0..63)                                                     OPTIONAL  -- Need S

37.355:

NR-DL-PRS-ResourceSet-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceSetID-r16			NR-DL-PRS-ResourceSetID-r16,

dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16,

dl-PRS-ResourceRepetitionFactor-r16	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,	-- Need OP

dl-PRS-ResourceTimeGap-r16			ENUMERATED {s1, s2, s4, s8, s16, s32, ...}

OPTIONAL,	-- Cond Rep

dl-PRS-NumSymbols-r16				ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-MutingOption1-r16			DL-PRS-MutingOption1-r16			OPTIONAL,	-- Need OP

dl-PRS-MutingOption2-r16			DL-PRS-MutingOption2-r16			OPTIONAL,	-- Need OP

dl-PRS-ResourcePower-r16			INTEGER (-60..50),

dl-PRS-ResourceList-r16				SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF

NR-DL-PRS-Resource-r16,

...

}

- Conclusion

In this contribution, we propose to change positioning SRS power control description in 38.213 as

Proposal 1: When PRS is pathloss RS of positioning SRS, the PRS power value should be derived from RRC parameter dl-PRS-r16 rather than dl-PRS-ResourceId-r16.

- Reference

- 38.213 NR Physical channels and modulation (Release 17) V17.4.0

- 38.331 Radio Resource Control (RRC) protocol specification (Release 16) V16.11.0

- 37.355 LTE Positioning Protocol (LPP) (Release 16) V16.9.0

- R1-2300798 Draft CR on positioning SRS power control in 38.213 ZTE