3GPP TSG RAN WG1 #112	R1-2300828

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.14.1

Source:	NEC

Title:	Discussion on PRACH coverage enhancement

Document for:	Discussion and Decision

1	Introduction

In RAN1#111 discussion [1] of PRACH coverage enhancement, following agreements are achieved.

Agreement For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, consider one or multiple of the following options. - Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs. - Option 2: Multiple PRACH are transmitted on separate ROs. - Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs. - Other options are not precluded. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. Agreement Study at least the following case for multiple PRACH transmissions with different Tx beams. - UE uses different TX beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS - FFS: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs /CSI-RSs, where the different SSBs/CSI-RSs are not associated with the same RO. - Note: not related to decision on CFRA Note: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs/CSI-RSs, where the different SSBs/CSI-RSs are associated with the same RO is not considered. Agreement For multiple PRACH transmissions with same Tx beam, down-select one option from the following options. - Option 1: gNB can only configure one value for the number of multiple PRACH transmissions. - Option 2: gNB can configure one or multiple values for the number of multiple PRACH transmissions. - FFS: details

In this paper, we discuss and give our views on PRACH coverage enhancement.

2	Discussion

2.1	PRACH repetition threshold

PRACH enhancement is used for cell edge UE to improve PRACH coverage. RSRP criteria could be used to determine if the UE needs to apply PRACH enhancement. In legacy PRACH beam determination, RRC configures rsrp-ThresholdSSB, and if RSRP of a beam is larger than this value, the beam can be select. Otherwise, any beam can be selected. When PRACH repetition is configured, a RSRP threshold is configured to determine whether to use PRACH repetition or not. Network configures RSRP threshold (e.g. rsrp-ThresholdSSB-repetetion) in System Information, and if none of SS-RSRP is above rsrp-ThresholdSSB and at least an SSB with SS-RSRP is above rsrp-ThresholdSSB-repetition, select an SSB above rsrp-ThresholdSSB-repetition for PRACH repetition, else select any SSB for PRACH repetition.

In current NR Rel-17, there are many features supported, e.g. small data transmission, msg3 repetition, reduced capability,  etc. To distinguish those features and combination of features, RRC configuration may associate a set of preambles with a feature combination.

PRACH repetition can also be regarded as a feature and can be combined with other features, e.g. a set of preambles associated with combined msg1 repetition and msg3 repetition, another set of preambles associated with combined msg1 repetition and reduced capability request.

RRC can configure different number of PRACH repetitions. It’s the trade-off between detection performance which trends to large number of repetitions and resource efficiency which requires to small number of repetitions. Different number of PRACH repetition can be regarded as different feature. If so, features with different number of PRACH repetition cannot be combined as combination of features.

Proposal 1: For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions. Rel-17 multiple feature combinations method could be the baseline.

2.2	RAR window and RA-RNTI

RA-RNTI is determined based on the PRACH resource (PRACH occasion, RO) selected by the UE. In addition to preamble ID contained in the RAR message, UE can detect the response from network. When PRACH repetition is applied, even if UE transmits PRACH on multiple PRACH occasions, determining only one RA-RNTI from plenty of RA-RNTI(s) based on plenty of PRACH occasions has benefit to reduce PDSCH detection times from UE perspective.

It is proposed to start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion from the end of the last preamble repetition transmission and RA-RNTI is determined based on the index of the first OFDM symbol of the last preamble repetition transmission in a system frame. Starting RAR window from the end of last PRACH repetition can give more processing time for network response.

Usually the RAR window length is less than 10ms in which unique RA-RNTI is applied. As shown by Fig. 1, if RA-RNTI is determined by the first preamble repetition, for shared RO between PRACH repetition and legacy PRACH and for the overlapped RAR window, a DCI scrambled by RA-RNTI may not be distinguished between PRACH repetition RAR and legacy RAR. It may happened when RAR window starts from the end of last PRACH repetition.

It means there may be two different PRACH occasion mapping to one RAR. If it is allowed, additional procedure to determine on which PRACH transmission occasion where RAR is provided by this RAR should be applied, which may have impact on implementation. If the last PRACH repetition occasion is used to determine RA-RNTI, then the corresponding PRACH occasion indicated by RAR in the RAR window is unique and deterministic.

Fig.1 ambiguous DCI in overlapped RAR window due to RA-RNTI calculated on the first occasion

Proposal 2: Start RAR window at the first PDCCH occasion from the end of the last preamble repetition transmission and RA-RNTI is determined based on the index of the first OFDM symbol of the last preamble repetition transmission in a system frame.

2.3	PRACH repetition power control parameter

Since PRACH repetition request higher resources than no repetition, it’s better to reduce the number of retransmission, i.e. to be detected by network as soon as possible to reduce the resources’ load. In such way, a lower number of maximum transmission/retransmission of PRACH repetition and a higher power ramping step of PRACH repetition than the corresponding configurations of no repetition is benefit to achieve the motivation. For shared PRACH occasion between PRACH repetition and PRACH non-repetition, the same target power should be configured and separate/different ramping step and/or maximum transmission could be configured. Because in shared occasion, power difference between repetition and non-repetition would cause higher interference (higher power preamble will cause higher interference to lower power preamble).

Proposal 3: A separate target power and/or power ramping step and/or maximum number of transmission are configured for PRACH repetition.

3	Conclusion

In this contribution, we give our views on PRACH coverage enhancement, and propose that:

Proposal 1: For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions. Rel-17 multiple feature combinations method could be the baseline.

Proposal 2: Start RAR window at the first PDCCH occasion from the end of the last preamble repetition transmission and RA-RNTI is determined based on the index of the first OFDM symbol of the last preamble repetition transmission in a system frame.

Proposal 3: A separate target power and/or power ramping step and/or maximum number of transmission are configured for PRACH repetition.

References

[1]	Draft_Minutes_report_RAN1#111