3GPP TSG RAN WG1 #112	R1-2300858

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.6.1

Source: 	Lenovo

Title: 	UE complexity reduction

Document for:	Discussion and Decision

# Introduction

In RAN1#111 meeting, following conclusions and agreements were made for support of reduced capability NR devices,

Conclusion For UE BB complexity reduction, for broadcast and unicast PDSCH, RAN1 does not assume that the UE post-FFT buffer size per slot is smaller than 20 MHz. Agreement From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). Agreement For UE BB complexity reduction, a UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Agreement For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH Agreement For UE BB complexity reduction, a UE is able to receive a DL assignment in a DCI with a unicast PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. The number of PRB scheduled in DCI is not larger than the maximum number of PRB agreed in previous agreement from 110b-e Agreement For UE BB bandwidth reduction, for PUSCH, down-select between the following options for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), down-select between the following options for the maximum number of PRBs that the UE can process per slot: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS Same option will be selected for both PDSCH and PUSCH. Conclusion For UE BB complexity reduction, broadcast of separate SIB1/OSI (PDSCH) to Rel-18 RedCap UEs is not supported. Agreement - The minimum DL peak rate target (for FD-FDD) is [10] Mbps based on peak data rate calculation according to 38.306. - The same value for X is used for DL and UL

Besides, in RAN#98, regarding whether to take UE peak rate reduction as a standalone feature and the additional separate early identification of Rel.18 RedCap UEs, it was agreed/concluded that

For UE peak rate reduction as a standalone feature Conclusion - Revisit in RAN#99 (no additional discussion in RAN1 in 1Q’23) For additional separate early identification Agreement - Support additional separate early identification(s) (RAN1, RAN2) - How to support is decided by WGs

In this contribution, we discuss details on UE complexity reduction and early indication for Rel.18 RedCap.

# Discussion

## UE bandwidth reduction

One FFS (from RAN1#110b-e) is that for a cell supporting both Rel.17 and Rel.18 RedCap UEs, whether to support an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs. Since the RF BW is kept as 20MHz for Rel.18 RedCap UEs, and the UEs are able to receive a unicast/broadcast PDSCH spanning a bandwidth of more than ~5 MHz, there should be no problem for Rel.18 UEs to receive the signals/channels in the legacy initial BWPs. In addition, the scheduling restriction of PUSCH spanning no more than ~5 MHz for Rel.18 RedCap UEs does not bring much impact on legacy UEs’ scheduling especially if the gNB could early identify the Rel.18 RedCap UEs and given that PUSCH does not need large transmission BW during initial access.

Based on these, there is no strong motivation to introduce separate initial BWPs for Rel.18 RedCap UEs. We support the FL’s proposal in RAN1#111 that up to one separate (RedCap-specific) initial DL/UL BWP can be configured, and we think no new RRC signaling is introduced for configuring initial BWP for Rel.18 UEs.

Proposal 1: Separate initial BWPs for Rel.18 RedCap UEs are not necessary. No new RRC signaling is introduced for configuring initial BWPs for Rel.18 UEs.

Regarding the maximum number of PRBs for PUSCH and PDSCH, 25/11 PRBs for 15/30 kHz SCS (i.e., option 4 in the RAN1#111 agreement) are assumed during the RAN1 evaluations. These values align with RAN4 specified number of RBs for channel BW for 5MHz. Besides, it is noted in [1] that 25/11 PRBs can provide >10Mbps data rate, which meets the peak data requirement of Rel.18 RedCap already. Based on these, and as it was agreed to have same option for both PDSCH (at least for unicast) and PUSCH, we have the following proposals,

Proposal 2: 25/11 PRBs for 15/30 kHz SCS are supported as the maximum number of PRBs that the UE can transmit per slot or per hop for PUSCH

Proposal 3: 25/11 PRBs for 15/30 kHz SCS are supported as the maximum number of PRBs that the UE can process per slot.

## Early identification

We support early identification of Rel.18 RedCap UEs in Msg1 since it is beneficial for Msg2/Msg3 scheduling. Based on RAN1#111 agreement, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH could be larger than the maximum number of unicast PRBs that the UE can process per slot, in which case the UE might need more time to process the RAR PDSCH. Given this, the TDRA for Msg3 scheduling in RAR PDSCH shall take this more RAR processing time into account. It was agreed in RAN1#111 that the Rel.18 RedCap UE could receive the RAR and correspondingly transmits Msg3 only if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is not smaller than a predefined threshold, otherwise the UE behavior is up to UE implementation. In our understanding, such UE implementation-based behavior cannot guarantee the RAR reception performance and the corresponding proper Msg3 scheduling. If Rel.18 RedCap UEs can be early identified in Msg1, the gNB could take the more processing time for RAR reception of Rel.18 RedCap UEs into account and assign enough time between the RAR reception and Msg3 transmission for proper RAR reception and Msg3 transmission.

Similar with Rel.17 RedCap UEs identification, early identification through Msg1 can be optionally configured for Rel.18 RedCap UEs. Then depending on the configurations, Rel.18 RedCap UEs will perform following for random access,

- If early identification through Msg1 is configured for Rel.18 RedCap UEs, Rel.18 RedCap UEs will use the configured preambles for Rel.18 RedCap UEs for random access.

- If early identification through Msg1 is not configured for Rel.18 RedCap UEs, while early identification is configured for Rel.17 RedCap UEs, Rel.18 RedCap UEs will use the configured preambles for Rel.17 RedCap UEs for random access.

- If early identification through Msg1 is not configured for both Rel.18 and Rel.17 RedCap UEs, Rel.18 RedCap UEs will use the preambles for legacy UEs.

Proposal 4: Separate early identification for Rel.18 RedCap UEs in Msg1 is supported.

# Conclusion

In summary, we have following proposals for UE complexity reduction for Rel-18 RedCap:

Proposal 1: Separate initial BWPs for Rel.18 RedCap UEs are not necessary. No new RRC signaling is introduced for configuring initial BWPs for Rel.18 UEs.

Proposal 2: 25/11 PRBs for 15/30 kHz SCS are supported as the maximum number of PRBs that the UE can transmit per slot or per hop for PUSCH

Proposal 3: 25/11 PRBs for 15/30 kHz SCS are supported as the maximum number of PRBs that the UE can process per slot.

Proposal 4: Separate early identification for Rel.18 RedCap UEs in Msg1 is supported.

# References

- TR 38.865, “Study on further NR RedCap UE complexity reduction”.