3GPP TSG RAN WG1 #112                                                                 R1-2300865

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:					    9.1.1.1

Source:	Sony

Title:	Discussion on unified TCI framework extension for multi-TRP

Document for:	Discussion

Introduction

In RAN#94e, the Rel-18 WID of MIMO evolution for downlink and uplink is approved [1]. Relevant objectives of unified TCI framework extension are highlighted as below.

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

For discussion under this agenda item, multiple issues are summarized in FL’s summary [2] based on latest contributions from companies.

- Summary of companies’ views on each of open issues raised by interested companies, where the open issues are categorized as follow: - Issue 1 – General framework for unified TCI extension - Issue 2 – TCI state update and activation - Issue 3 – How to inform UE which indicated TCI state(s) that UE shall apply to target channel/signal - Issue 4 – UL power control for UL MTRP - Issue 5 – PDSCH-CJT - Issue 6 – Beam reporting and beam failure recovery

In this contribution, we mainly present our views on the last issue with	 TRP-specific beam failure recovery (BFR).

TRP-specific BFR

It has already come out an initial agreement in RAN1#110b meeting on TRP-specific BFR [3], where implicit BFD-RS determination and beam update are approved for further study.

Agreement (RAN1#110b) On unified TCI framework extension, study the following enhancements for: - Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP - Enhancement to beam update after NW response to TRP-specific BFR request

Another issue on beam reporting for simultaneous multi-panel transmission (STxMP) was also captured in [2] after continuous discussion in RAN1#111 meeting. In this section, we present our views on the last two issues.

- Issue 6 – Beam reporting and beam failure recovery Table 6-1 Summary for Issue 6 #; Issue; Companies’ view and summary. 6.1; Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP; For S-DCI based MTRP, if a BFD-RS set is not explicitly provided for TRP-specific BFR, BFD-RS for the BFD-RS set is determined from a corresponding indicated joint/DL TCI state - Support: CATT, Docomo, Ericsson, Huawei/HiSilicon, MTK, Qualcomm, vivo, LG,CMCC, Intel - Concern: FL note: Given the support of several companies, Proposal 6.1 is recommended for this issue. Please check and share your view, if any.. 6.2; Enhancement to beam update after NW response to TRP-specific BFR request; Beam update for all channels/signals associated with s failed TRP after NW response to TRP-specific BFR request for the failed TRP - Support: CATT, Docomo, Qualcomm, vivo, ZTE, Google, LG, OPPO, Apple,CMCC - Concern: FL note: Please update your view to above enhancement to TRP-specific BFR. 6.3; Enhancement to beam reporting for STxMP; Question: This issue should be discussed in which Ais? - AI 9.1.1.1 : QC, OPPO, CMCC, Docomo, ZTE, vivo, LG, CATT, Apple, Transsion - AI 9.1.4.1: Ericsson, NEC, Xiaomi, Huawei/HiSilicon, MTK, Google, Docomo FL note: Please update your view to above question..

Enhancement to beam update

As we known, after detection of beam failure and candidate beam report, beam update is followed as NW response to TRP-specific BFR request. Recalling that MAC CE actives at most 8 codepoint of TCI in unified TCI framework, if beam updates directly to one out of the 8 activated codepoint, there would be no need for MAC CE to active new TCI state. From the perspective of simplified signaling, we support new beam indicator (NBI) can be chosen from the codepoint of TCI already activated by MAC CE, thus the procedure on reactivation of new TCI state by MAC CE can be saved.

- : For beam update, NBI can be the activated codepoint of TCI from MAC CE.

Enhancement on beam reporting

On beam reporting for STxMP, since STxMP is under discussion in parallel within AI 9.1.4.1, beam reporting for STxMP could be discussed there, and we prefer more general enhancement on beam reporting in this agenda.

Two-step BFR procedure

In legacy Rel-17, BFR for multi-TRP has been specified with beam failure declaration still based on all RS in q0,0 or q0,1. Since one of the BFD-RS fails for one TRP may also happen, partial BFR is more reasonable for flexible and fast beam update. Noticing this, we propose similar two-step BFR procedure in legacy NR but with partial BFR in multi-TRP scenario.

Alternative 1: Two-step BFR Step 1: UE only report beam failure by reusing legacy Rel-16 SR/LRR on PUCCH. Step 2: UE report failed BFD-RS index and corresponding candidate beam index by newly defined MAC CE on PUSCH.

Compared to legacy Rel-16 MAC CE format in Figure 1(a), Figure 1(b) illustrates the newly defined MAC CE format. Ci indicates the beam failure of i-th SCell. R is reserved bit. AC represents the existence of corresponding 8-bit candidate beam. However, in multi-TRP scenario, SCell index is not needed. Therefore, we can repurpose Ci to indicate BFD-RS index with other fields reused in this case. Taking two TRP and max two BFD-RS per TRP for illustration, only four Ci are needed as Figure 1(b) shows.

(a) Rel-16 MAC CE format 	                                          (b) New MAC CE format in Alternative 1

Figure 1: MAC CE format for beam report in BFR

Considering one specific case with BFD occurs on both BFD-RS in set q0,0 for TRP1 like Figure 2(a), Figure 2(b) gives an example of the newly defined MAC CE format accordingly. After UE send SR/LRR to gNB for BFD notification as well as PUSCH resource request. UE could send MAC CE with format in Figure 2(b) on assigned PUSCH, where C1 and C2 are 1 for the indication of beam failure on both two BFD-RS for TRP1.

(a) Scenario                           	                                              (b) Example

Figure 2: Specific scenario and MAC CE format for beam report in Alternative 1

- : For two-step BFR procedure in Alternative 1, new MAC CE format for beam report can be considered.

In addition, we consider another two-step procedure with failed BFD-RS index reported in the first step.

Alternative 2: Step 1: UE report beam failure and the failed BFD-RS index on PUCCH. - Use SR/LRR like PUCCH containing failed BFD-RS index to gNB. (Explicit) - Link BFD-RS index to SR/LRR resource represented by SchedulingRequestId. (Implicit) Step 2: UE report candidate beam index corresponding to failed BFD-RS index. - Report on modified legacy MAC CE - Report on modified AP-CSI

In Alternative 2, failed BFD-RS index is reported on PUCCH. Two methods with explicit and implicit report can be considered further. In explicit reporting, we can directly add failed BFD-RS index in legacy SR/LRR PUCCH. For implicit report, a link between BFD-RS and SR/LRR resource needs to be pre-configured by RRC, thus the transmitted SR/LRR PUCCH can simultaneously indicate the failed BFD-RS.

When candidate beam index is reported on MAC CE, modified MAC CE format in Figure 3(a) is needed. Figure 3(b) shows the example corresponding to pre-mentioned scenario in Figure 2(a).

(a) Modified MAC CE format                                         (b) Example

Figure 3: Modified MAC CE format for beam report in Alternative 2

When candidate beam index corresponding to failed BFD-RS is reported on AP-CSI, modified AP-CSI format in Figure 4(a) can be considered. Similarly, Figure 4(b) is an example for the scenario in Figure 2(a).

(a) Modified AP-CSI format

| CSI report number | CSI fields |
| --- | --- |
|  | Candidate beam index corresponding to BFD-RS1-0 (6 bits) |
|  | Candidate beam index corresponding to BFD-RS1-1 (6 bits) |
|  | RSRP1 corresponding to candidate beam1 (7 bits) (optional) |
|  | Differential RSRP2 corresponding to candidate beam2(4 bits) (optional) |

(b) Example

Figure 4: Modified AP-CSI format for beam report in Alternative 2

- : For two-step BFR procedure in Alternative 2, modified MAC CE/AP-CSI format for beam report can be considered.

One-step BFR procedure

Besides two-step BFR procedure, one-step BFR procedure for the reduction of latency and signaling overhead also worth for study. In one-step procedure, failed BFD-RS index and corresponding candidate beam index are included together in one AP-CSI report as shown in Figure 5. With same assumption of two TRP and max two BFD-RS for each TRP, failed BFD-RS indication with 2 bits is enough.

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | BFD-RS index1 (2 bits) |
|  | BFD-RS index2 (2 bits) |
|  | … |
|  | Candidate beam index1 (6 bits) |
|  | Candidate beam index2 (6 bits) |
|  | … |
|  | RSRP1 corresponding to candidate beam1, if reported (7 bits) |
|  | Differential RSRP2 corresponding to candidate beam2, if reported (7 bits) |
|  | … |

Figure 5: Modified AP-CSI format for one-step beam report

Similarly, Figure 6 shows the example of AP-CSI format in one-step BFR for scenario in Figure 2(a).

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | BFD-RS index corresponding to BFD-RS1-0 (2 bits) |
|  | BFD-RS index corresponding to BFD-RS1-1 (2 bits) |
|  | Candidate beam index corresponding to BFD-RS1-0 (6 bits) |
|  | Candidate beam index corresponding to BFD-RS1-1 (6 bits) |
|  | RSRP1 corresponding to candidate beam1, if reported (7 bits) |
|  | Differential RSRP2 corresponding to candidate beam2, if reported (7 bits) |

Figure 6: Example of AP-CSI format for one-step beam report

- : For one-step BFR procedure, modified AP-CSI format for beam report can be considered.

Conclusions

Finally, allow us to repeat our proposals to draw attention.

- : For beam update, NBI can be the activated codepoint of TCI from MAC CE.

- : For two-step BFR procedure in Alternative 1, new MAC CE format for beam report can be considered.

- : For two-step BFR procedure in Alternative 2, modified MAC CE/AP-CSI format for beam report can be considered.

- : For one-step BFR procedure, modified AP-CSI format for beam report can be considered.

Reference

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink. 3GPP RANP#94e.

- R1-2212878, Moderator summary on extension of unified TCI framework (Round 2). 3GPP RAN WG1 #111.

- Draft Report of 3GPP TSG RAN WG1 #110bis-e v0.1.0 (Online, 10th – 19th October 2022).