3GPP TSG RAN WG1 #112			R1-2300876

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.4.1.2

Source: 	Sony

Title:	Discussion on physical channel design framework for SL-unlicensed

Document for:	Discussion

Introduction

In RAN#95-e, Rel-18 NR sidelink evolution has been approved and sidelink on unlicensed spectrum will be studied and specified in order to increase bandwidth for sidelink transmission as described in [1]. In RAN1#110-bis, RAN1 discussed and made some agreements [2].

Agreement For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding the location of 1st starting symbol, down-select one of the followings: - Option 1: it is fixed as symbol#0 - Option 2: it is indicated by sl-StartSymbol as in R16 NR SL - Regarding the location of 2nd starting symbol, down-select one of the followings: - Option A: it is a fixed location - FFS the location, e.g., symbol#4, #7, etc. - Option B: it is a (pre-)configured location per resource pool - FFS the details of candidate locations - Note: assume symbol index in a slot starts from #0 Agreement If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - TBS is determined based on a reference symbol length, down-select one of the followings: - Option 1: The reference symbol length is dynamically indicated by Tx UE - Option 2: The reference symbol length is determined based on 1st starting symbol - Option 3: The reference symbol length is determined based on 2nd starting symbol - Option 4: The reference symbol length is (pre-)configured Agreement Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, RAN1 continues studying the following updated alternatives: - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - FFS: value of K3 - Alt 2-1a: each PSFCH transmission occupies 1 interlace, and further apply frequency-domain OCC - FFS: details of FD-OCC, e.g., OCC length, RB-level, RE-level, etc. - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace - FFS: value of K1 - Alt 2-3a: each PSFCH transmission occupies 1 interlace - Alt 2-4a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE uses different cyclic shifts on different PRBs in the interlace - Alt 3-1a: each PSFCH transmission occupies 1 dedicated PRB and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - The above dedicated PRB and common PRBs are within 1 interlace - FFS: value of K2 - Alt 3-2a: each PSFCH transmission occupies 1 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of a RB set - FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth - FFS: whether IBE issue exists and whether/how to address it - Note: in the above descriptions - The dedicated PRB/cyclic shift conveys ACK/NACK information - Note: as previously agreed: to meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS. Agreement Slots with PSFCH symbols only have 1 candidate starting symbol for PSCCH/PSSCH. Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, RAN1 further study the followings: - Option 1: 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool - Option 2: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 3: 1 sub-channel is defined across all RB sets within the resource pool, i.e., 1 sub-channel includes K interlace(s) across all RB sets within the resource pool - Option 4: 1 sub-channel is defined within 1 RB set or 2 adjacent RB sets, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 5: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly across different RB sets within the resource pool, then across different interlaces in the RB set - FFS: whether/how to use intra-cell guardband PRBs Agreement For S-SSB transmission, down-select one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - Option 1-2: Using interlaced RB transmission for PSBCH only, and apply OCB exemption to S-PSS and S-SSS - Option 3-1: Repeat S-PSS/S-SSS/PSBCH N times in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of N (e.g., N=2) - FFS gap of 0 - Option 3-2: Repeat only S-PSS/S-SSS K times in frequency domain, and PSBCH is rate matched. There is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of K - FFS gap of 0 - FFS PSBCH resource - Option 3-3: keep the legacy S-PSS/S-SSS/PSBCH while repeating PSBCH N times in frequency domain and rate-matching PSBCH to S-PSS/S-SSS symbols, and there is a gap between the PSBCH repetition(s) to meet OCB requirements - FFS details, e.g. the length of gap is (pre-)configured or pre-defined, value of N - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - For Option 1-1 and 1-2 above - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set - FFS: whether transient period issue exists and whether/how to address it Agreement For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Regarding Rx UE behaviour, down-select one of the followings: - Option A: The Rx UE always monitors two AGC symbols in such slot - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol - FFS details - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot Agreement To address PSFCH transmission dropping due to LBT failure, RAN1 down-select one of followings, or support the combination of followings: - Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - FFS other details, e.g., HARQ-ACK timeline - Alt 2: PSFCH occasions are dynamically indicated - FFS: Whether/how to handle the case where some TB’s corresponding PSFCH cannot be transmitted within the same or different COT - FFS other details, e.g., dynamically indicate one or more PSFCH transmission(s), container of the indication, etc. - FFS: Whether such PSFCH occasions are within the same or different COT of corresponding PSSCH - FFS: Whether/how to address PSFCH collision if any - FFS: Whether/how to handle the linearly decreased PSFCH capacity Agreement Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings: - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured - FFS details, e.g., value of K, details on gap length, etc. - Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured - Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period - Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number Agreement Regarding additional candidate S-SSB occasions: - In the same S-SSB period, RAN1 further study the followings: - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s) - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions - FFS details Agreement For contiguous RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and PRBs, further study the following options: - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size - FFS: whether/how to use sub-channel(s) which include intra-cell guardband PRBs - FFS: whether/how to handle the case when the number of PRBs of the resource pool cannot be divided by sub-channel size - Option 2 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set according to the sub-channel size - FFS: whether/how to use intra-cell guardband PRBs - FFS: whether/how to handle the case when the number of PRBs of one RB set cannot be divided by sub-channel size - Option 3 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set and/or guardband PRB according to the sub-channel size - FFS: how to use intra-cell guardband PRBs - FFS: how to use the subchannel including PRBs in guardband Agreement Regarding PSFCH transmission under 60 kHz SCS, further study the following alternatives: - Alt 1: Each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs - FFS details - Alt 2: Each PSFCH transmission occupies some dedicated PRBs - FFS details Agreement Regarding S-SSB, RAN1 further study the following: - How to transmit S-SSB when a SL BWP contains multiple RB sets

In this contribution, we discuss our view on physical channel design framework for SL-unlicensed.

Discussion

Slot structure

In RAN1, whether to introduce additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission has been discussed in addition to Rel-16/17 NR SL slot-based transmission. If only slot-based transmission is supported for SL-U transmission, such SL device will reduce transmission opportunity due to other RAT like WiFi which can transmit immediately after LBT success. To solve the issue, SL-U should support additional starting symbol(s) within a slot for PSCCH/PSSCH transmission

At RAN1#110-bis, it was agreed as working assumption to support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission.

At RAN1#111, we made two options on the location of 1st and 2nd starting symbols respectively as follows;

- Regarding the location of 1st starting symbol, down-select one of the followings:

  - Option 1: it is fixed as symbol#0

  - Option 2: it is indicated by sl-StartSymbol as in R16 NR SL

- Regarding the location of 2nd starting symbol, down-select one of the followings:

  - Option A: it is a fixed location

    - FFS the location, e.g., symbol#4, #7, etc.

  - Option B: it is a (pre-)configured location per resource pool

    - FFS the details of candidate locations

For the position of the 1st starting symbol, we don't think that the current specification needs to be changed. Therefore we support the option 1 for the position of the 1st starting symbol.

For the position of the 2nd starting symbol, we think it should be (pre-)configurable such as the 1st starting symbol. But since Rel-16 SL supports more than or equal to 6 symbols PSSCH (not including GP), the position within a slot could be restricted to e.g. one of symbols between 2nd and 8th symbol considering a specification impact.

Proposal 1: positions of the 1st and 2nd starting symbol are (pre-)configured. (i.e. Option 2 and B)

Proposal 2: a candidate position of the 2nd starting symbol can be restricted.

Regarding AGC symbols for slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, we have discussed the number of AGC symbols for the 1st and 2nd starting symbols in terms of Tx UE behaviour and Rx UE behaviour respectively.

Regarding Tx UE behaviour, it was agreed that if PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose.

When more than one RB sets are configured in a resource pool, different PSSCH with different RB sets from other UEs can be multiplexed in the frequency domain. In such case, from the viewpoint of the Rx UE, two AGC symbols should be necessary. However, at least when the Rx UE detects a PSCCH/PSSCH transmission starting from the 1st starting symbol, only one AGC symbol would be monitored. We think the condition whether to monitor 1 or 2 AGC symbol(s) in a slot does not have to be restricted, and it would be up to Rx UE implementation. Therefore, we support that the Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol (Option C).

Proposal 3: the Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol (Option C).

On TBS size determination for slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, we prefer to independently determine the TBS sizes of PSSCH transmission starting from the 1st and 2nd starting symbols considering resource usage efficiency and avoiding the case of over 1 of coding rate for the 2nd starting symbol.

At RAN1#110, four options have been agreed for a down-selection.

- TBS is determined based on a reference symbol length, down-select one of the followings:

  - Option 1: The reference symbol length is dynamically indicated by Tx UE

  - Option 2: The reference symbol length is determined based on 1st starting symbol

  - Option 3: The reference symbol length is determined based on 2nd starting symbol

  - Option 4: The reference symbol length is (pre-)configured

We support that the reference symbol length could be dynamically indicated by Tx UE (Option 1) or (pre-)configured (Option 4). Considering a signalling overhead, we slightly support the (pre-)configuration (Option 4).

Proposal 4: the reference symbol length could be dynamically indicated by Tx UE (Option 1) or (pre-)configured (Option 4).

Enhancements for SL resource allocation mode 2

Procedure in the case of LBT failure for mode 2

In RAN1#109-e, it was agreed to support UE-to-UE COT sharing for NR-U. Though it is still FFS which SL channels/signals can be transmitted from a COT-shared UE, if PSFCH transmission corresponding to PSSCH within the shared COT is supported and LBT is required for the COT-shared UE before transmitting the PSFCH, a procedure in the case of LBT failure is necessary.

At RAN1#110-bis, some companies have proposed to support more than 1 PSFCH occasion per PSCCH/PSSCH transmission. But such design would result in decreasing the PSFCH capacity. Considering a resource efficiency for PSFCH, we propose to support a dynamic indication of the PSFCH resource through a COT sharing information in SCI.

Proposal 5: PSFCH resources are dynamically indicated by COT initiating UE.

Supporting interlaced mapping

In RAN1#109-e, it was discussed that some solutions to meet OCB and PSD requirement for S-SSB transmission in SL-U and agreed to down-select from the following 4 options;

- Option 1: Using interlaced RB transmission

- Option 2: S-SSB multiplexing with other SL transmissions in the same slot

- Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain

- Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth

For NR-U, SSB can be multiplexed with other DL transmissions including DL signals, e.g. CSI-RS, and DL channels, e.g. PDCCH/PDSCH to meet OCB and PSD requirement for SSB in NR-U. On the other hand, for sidelink transmission, S-SSB multiplexing with other SL channels in frequency domain is not beneficial due to a half-duplex restriction.

Considering a specification effort, new structure of S-SSB should be deprioritized at this stage as long as there is any reason.

Therefore, we have thoughted to down-select from either option 1 or 3. At RAN1#110-bis, it was agreed to support either option 1 or 3 and down-select between them. Comparing them, we don’t see any large difference. But since it was agreed that PRB-based interlaced mapping is supported for PSCCH/PSSCH, the same solution might be better to use for S-SSB transmission as well considering specification efforts as long as there are any critical issues. Therefore we slightly support PRB-based interlaced mapping (option 1) for S-SSB transmission in SL-U.

Proposal 6: PRB-based interlaced mapping (option 1) is supported for S-SSB transmission in SL-U.

Conclusion

Proposal 1: positions of the 1st and 2nd starting symbol are (pre-)configured.

Proposal 2: a candidate position of the 2nd starting symbol can be restricted.

Proposal 3: the Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol (Option C).

Proposal 4: the reference symbol length could be dynamically indicated by Tx UE (Option 1) or (pre-)configured (Option 4).

Proposal 5: PSFCH resources are dynamically indicated by COT initiating UE.

Proposal 6: PRB-based interlaced mapping (option 1) is supported for S-SSB transmission in SL-U.

Reference

- RP-220300, “WID revision: NR sidelink evolution,” 3GPP TSG RAN Meeting #95e.

- RAN1 chair’s notes, RAN1#111.