3GPP TSG RAN WG1 #112		  R1-2300918

Athens, Greece, February 27th – March 3rd, 2023

Source:	Panasonic

Title: 	Discussion on subband non-overlapping full duplex

Agenda Item:		9.3.2

Document for:	Discussion

Introduction

A study item “Study on evolution of NR duplex operation” has been agreed for Rel.18 [1]. One key objective is to study subband non-overlapping full duplex (SBFD). This document provides our view on SBFD.

Discussion

SBFD operation

Link direction of subbands

For SBFD operation in legacy DL symbol and legacy flexible symbol, the following agreements were made in RAN1#111.

Agreement For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived - DL receptions within DL subband(s) are allowed in the symbol - Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol Agreement For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs, Option 1: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol - FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol Option 2: - UL transmissions within UL subband are allowed in the symbol - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same - FFS: SBFD aware UE behaviours - FFS: Whether or not signalling of guardband(s) is needed - FFS: Whether or not the symbol can be converted to a DL-only symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously

For SBFD operation in legacy flexible symbol, we don't want to increase type of the operation further. Option 1 would be same as DL symbol with SBFD operation. Option 2 can be same as legacy flexible symbol or new type of the operation. Depending on the configuration of SBFD symbol, in Option 1, flexible symbol can be same as legacy flexible symbol or same as DL symbol with SBFD operation. Option 2 would not have such flexibility.

Proposal 1: For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, support Option 1.

Subband frequency location indication

In RAN1#110bis-e, the following agreement was made.

Agreement For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined.

Regarding FFS in the agreement, it is not clear whether DL/UL subband includes any intra or inter carrier guard band or not. If it follows NR-U definition of RB set, guard band is outside of DL/UL subband. Therefore, we propose to clarify that guard band is outside of DL/UL subband. Figure 1 shows an example of the relationship between subbands and guard bands. In this example, guard band is outside of DL/UL subbands.

The motivation of having explicit guard band configuration is to limit the subband location to a set of candidate locations. This facilitates UE to apply analogue filtering to suppress leakage interference if analogue filter is required. Furthermore, having explicit guard band configured does not necessarily mean that guard band resources cannot be used for resource allocation. For example, in Figure 1, the UE’s DL and UL BWPs can be used to be the same as carrier BW, having both two guard bands included. Then it is up to the scheduler decision whether to use the guard band for DL/UL resource allocation. Furthermore, it would be also allowed that no guard band is configured and guard band is implicitly allocated by scheduler. In summary, the following operation for guard band can be considered.

- Guard band can be explicitly configured via cell specific configuration. To confiture no guard band is also allowed (i.e., to configure guard band with 0 RB).

- Guard band can be implicitly allocated/adjusted by gNB's scheduling via DCI depending on CLI. Not to adjust guard band via DCI is also allowed (i.e., just use cell specific guard band for the simple operation).

How to adjust guard band size gNB's scheduling is also discussed in section 2.2.4.

Proposal 2: Frequency location of subbands types other than UL subband (i.e., DL subband, guard band if supported) can be also explicitly indicated.

Figure 1  Example of subbands and guard bands

For semi-static configuration of subband frequency locations, to reuse RB set configuration can be considered. For RB set, frequency domain resources for each subband can be semi-statically configured by UE-specific RRC signalling (i.e., IntraCellGuardBandsPerSCS). Guard bands between subbands can also be configured. Subbands and guard bands consist of 1 RB or a set of consecutive RBs. Thus, the functionality and the configuration of RB set can be basically reused except that the configuration is sent over UE-specific signalling.

Proposal 3: The legacy semi-static configuration of RB set can be basically reused but the configuration should be sent over SIB.

Subband time location indication

For SBFD operation, gNB needs to be able to configure different transmission directions to different UEs on the same symbol or slot. By Rel-15/16/17 specifications, cell common transmission direction is configured by SIB and different direction to different UEs can be realized by UE-dedicated RRC slot format configuration or a group common PDCCH (DCI 2_0). The following Figure 2 illustrates one example of using UE-dedicated RRC to configure different transmission directions. Note that the existing signalling schemes can only overwrite semi-static Flexible symbol/slot of SIB.

Figure 2  SBFD operation by using legacy slot format configurations

The above existing signalling scheme is said not widely used in existing NR deployment. Taking Japanese 5G deployment as an example, the TDD pattern (in slot) “DDDSUUDDDD” is typically configured by SIB, where S is a special slot containing DL, UL and Flexible symbols and the Flexible symbols are only used for the gap to switch between DL and UL. In spite of the mandatory support of flexible symbols in Rel-15/16/17 specifications, some argued that the sufficient inter-operability test is not available and not achieved.

Therefore, the solution relying solely on semi-static Flexible symbol/slot is not sufficient. New enhanced signalling is needed to enable SBFD operation in Rel-18, while at the same time, to minimize the impact on legacy NR deployment.

Observation 1: It is not sufficient to operate SBFD only over legacy semi-static Flexible symbol/slot. Method to utilize legacy semi-static DL symbol/slot should be studied.

In order not to change the legacy cell-common semi-static slot format, SBFD aware UE can be configured with a new semi-static symbol/slot format where the legacy DL symbol/slot can be overwritten to a UL or Flexible symbol/slot. With such approach, the SBFD operation between legacy UE and SBFD aware UE becomes possible.

The new Rel-18 semi-static slot format can be configured in two ways: in a cell-common manner using a new parameter such as tdd-UL-DL-ConfigurationCommon-r18; or in a UE-dedicated manner reusing the existing parameter tdd-UL-DL-ConfigurationDedicated. Note that for the former case, although it is a cell common signaling, it would not be visible to Rel-15/16/17 UEs (meaning no impact to legacy UE configuration) because it is a newly introduced parameter in SIB. For the latter case, on the other hand, gNB has flexibility to select which UE to configure.

The following figure shows one example of using cell-common signalling, where both parameters, tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationCommon-r18, are broadcast by SIB, configuring slot formats for a periodicity of 5 slots. Rel-15/16/17 UEs recognize tdd-UL-DL-ConfigurationCommon, and obtain the slot format as DDDDU. On the other hand, SBFD aware UEs may recognize both parameters, and obtain a different slot format from tdd-UL-DL-ConfigurationCommon-r18 as DUUUU. As a result, SBFD operation at cell level is made possible, by transmitting DL and receiving UL at the same time over “D->U” symbols. Instead of “D->U” using tdd-UL-DL-ConfigurationCommon-r18, “D->F” is also possible. Then the symbol used for SBFD operation for SBFD aware UEs is F symbol and the transmission direction can be determined dynamic manner based on SFI or dedicated DCI.

Figure 3  SBFD operation by using a new slot format configuration

Proposal 4: Consider to introduce a new semi-static slot format for Rel-18 (and beyond) where the legacy semi-static DL symbol/slot can be re-configured as UL or Flexible symbol/slot.

Transmission/reception enhancements

In RAN1#111, to study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols was agreed as follows. In the following sub-sections, transmission/reception enhancements for some channels are discussed.

Agreement Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following: - PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols - Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols - Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition - Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols - Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered. Examples of potential enhancements include: - Resource allocation in frequency domain including frequency hopping - Resource allocation in time domain - Power domain - Spatial domain FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured.

Configured UL transmission

When the gNB operates with SBFD operation, the available UL frequency resources, as well as the interference situation, are different between a “normal” UL symbol/ slot (containing no subband) and an SBFD symbol/ slot (where the subbands within the frequency band are used to separate the DL and UL transmission directions).

For configured UL transmission such as CG PUSCH, SRS, PUSCH/PUCCH repetition, the transmission occasions may at one instance overlap with UL frequency resources in normal UL symbol/slot, but may at another instance be overlapping with a downlink subband of SBFD symbol/slot. These two cases are shown in the following Figure 4. In the case of configured UL transmission instance overlapping with SBFD slots/ symbols, their resource allocation may deteriorate the performance of DL transmission allocated to the DL subband on the same symbol/ slot (see slots #3 and #4 in Figure 4).

Figure 4  Issue of configured UL transmission resource allocation

It is in principle by current spec that a configured UL transmission can be configured to align the periodicity and symbol offset with that of either normal UL symbol or SBFD symbol. However, such alignment is an additional constraint on the configuration that needs to be taken care of by the gNB and which adds to the complexity of resource allocation. Furthermore, if the SBFD time pattern changes, reconfiguration of configured UL transmission is needed for new alignment, which increases the signaling overhead. Note that this issue does not happen when SBFD operation is carried out in Flexible symbol/slot as configured UL transmission is disabled in Flexible symbol/slots.

Therefore, it is preferable to decouple the configured UL transmission from the configuration/operation of SBFD symbol/slot. This motivates to study methods for UE to validate configured UL transmission instances. For example, the configured UL transmission can be associated with an applicable symbol type (e.g. SBFD symbol vs. normal UL symbol), such that UE will skip the UL transmission instance if it is not overlapping with the applicable symbol type.

To define different symbol types, the combination of legacy cell-common semi-static slot format and the new Rel-18 slot format mentioned in Section 2.1.3 can be utilized. In more details, from Rel-18 UE (and beyond) point of view, the following table provides an example of slot format combinations:

| Legacy cell-common semi-static slot/symbol format | New Rel-18 semi-static slot/symbol format | Conflicting direction? |
| --- | --- | --- |
| D | D | Non-conflicting |
| D | U | Conflicting |
| D | F | Conflicting |
| U | U | Non-conflicting |
| F | D | Non-conflicting |
| F | U | Non-conflicting |
| F | F | Non-conflicting |

Table 1. The conflict symbol relation between legacy and new semi-static slot/symbol format

As shown in the table, the two symbol types can be defined:

- Conflicting symbol direction between legacy semi-static and new Rel-18 semi-static slot format, e.g. D->U and D->F

- Non-conflicting symbol direction between the above two semi-static slot formats, e.g. U->U, F->U, and F->F

For a configured UL transmission, it can be associated with one of the symbol type or both. Since the conflicting and non-conflicting symbol correspond to SBFD-ready and SBFD-forbidden at gNB side, respectively, the configured UL transmission can now be transmitted or dropped in a controlled way.

Proposal 5: Study potential enhancement on configured UL transmission to associate with either normal UL symbol or SBFD symbol.

Dynamic grant PUSCH

For PUSCH scheduled by dynamic grant, the frequency resources can be allocated dynamically. However, the available frequency resources for allocation are different between normal UL symbol and SBFD symbol. Therefore, it is within the scope of the discussion whether the reference resource set can or should be changed between normal UL symbol and SBFD symbol. One possible way is to activate a narrow UL BWP for SBFD symbol and a wide UL BWP for normal UL symbol. The merit would be some savings in the DCI overhead. However, the BWP switching would incur considerable switching delay, if the existing BWP switching definition is reused. The relative long switching delay makes such scheme unattractive. Therefore, the baseline operation would be to use the same FDRA field between SBFD symbol and normal UL symbol. This also provides the benefit of reducing the BD complexity. The UL BWP can be set to the whole carrier BW that is applicable to both SBFD symbol and normal UL symbol. It is up to the scheduling decision to ensure the allocated frequency resources fulfil the restriction of UL subband for SBFD symbols.

In case of PUSCH repetition, since the same FDRA field is applied to all repetition, the resource allocation would have to be limited to within UL subband, if the repetitions occur over both SBFD symbol and normal UL symbol. This operation has the merit of simplicity. Any further optimization for frequency domain allocation needs to be justified.

Proposal 6: For PUSCH with dynamic grant, the same FDRA is used between SBFD symbol and normal symbol.

PDCCH

PDCCH is used to indicate PDSCH reception and PUSCH transmission. If the sufficient chance to receive PDCCH is not maintained due to UL subband in legacy DL symbol or flexible symbol, the performance and latency can be degraded. Therefore, it is necessary to discuss how to allocate PDCCH resource between SBFD symbol and non-SBFD symbol. The following options can be considered.

- Option 1: No change of CORESET regardless of SBFD symbol

  - Option 1a: PDCCH candidate in a CORESET is received if all time/frequency resources of the CORESET is DL

  - Option 1b: PDCCH candidate in a CORESET is received if all time/frequency resources of the PDCCH candidate is DL

  - Option 1c: PDCCH candidate in a CORESET is received if part of time/frequency resources of the PDCCH candidate is DL

- Option 2: Adjustment of CORESET/SS based on the collision with UL subband and guardband

  - Option 2a: CORESET size/location is adjusted to fit only DL resource

  - Option 2b: PDCCH candidate mapping is adjusted to fit only DL resource

- Option 3: Adjustment of CORESET/SS based on the configuration

  - Option 3a: Different CORESET configurations corresponding to SBFD symbol and non-SBFD symbol are introduced

  - Option 3b: Monitoring occasion configuration to fit SBFD symbol time domain location is introduced

Option 1 intends to use the same CORESET between SBFD symbol and non-SBFD symbol. The difference among Option 1a, 1b and 1c is when PDCCH candidate in CORESET can be received. For Option 1a, all PDCCH candidates in the CORESET cannot be received if part of time/frequency resources of the CORESET overlap with UL subband or guardband. For Option 1b, PDCCH candidates overlapping with UL subband or guardband cannot be used. For Option 1c, even if PDCCH candidate overlaps with UL subband or guardband, PDCCH can be received by decoding remaining CCEs or REGs which do not overlap with UL subband and guardband (puncture or rate matching is applied).

For Option 1a/1b/1c, the chance to receive PDCCH seems to be increased in the order of Option 1c, 1b, 1a. However, Option 1a would have sufficient chance to receive PDCCH by configuring multiple CORESETs and applying the dropping rule based on BDs/CCEs.

Figure 5 shows an example of Option 1a with multiple CORESET configurations. In this example, two CORESETs are configured. For CORESET#1, SS#1 is configured. For CORESET#2, SS#2 is configured. The number of BDs for both SS#1 and SS#2 are 44 BDs. The maximum number of BDs is also 44 BDs. The monitoring occasion of SS#1 and SS#2 is the same. For DL slot case (case 1), CORESET#2 is dropped due to dropping rule because total BDs of two CORESETs exceed the maximum number and SS ID for CORESET#2 (i.e., SS#2) is higher than CORESET#1 (i.e., SS#1). For SBFD slot case (case 2), CORESET#1 is dropped (not transmitted) due to overlapping with UL subband. In this case, the chance to receive PDCCH would be the same between DL slot and SBFD slot (i.e., 44 BDs for each).

Figure 5  Option 1a with multiple CORESET configurations

Option 2 intends that the resource of CORESET or PDCCH candidate is implicitly changed when configured CORESET resource overlaps with UL subband or guardband. For option 2a, CORESET size or location is changed (e.g., CCEs which overlaps with UL subband or guardband are not used, then CORESET size is reduced). For option 2b, PDCCH candidate mapping is changed (i.e., some PDCCH candidate indices ms,nCI(L)  are not used to avoid overlapping with UL subband and guardband).

Option 3 intends to introduce new configuration of CORESET and/or SS for SBFD operation. For Option 3a, multiple CORESET configurations are configured and associated with SBFD symbol or non-SBFD symbol. For Option 3b, new monitoring occasion configuration is introduced so that CORESET for DL subbands can only be located on SBFD symbol (e.g., slot level bitmap is used).

From our perspective, Option 2 and Option 3 can increase complexity and require specification effort. Option 1 would be simple and sufficient.

Proposal 7: For PDCCH resource allocation, not to change the CORESET regardless of SBFD symbol can be considered.

PDSCH

For {DUD} subband pattern, DL resources are splitted into two non-contiguous subbands. RA type 0 can be used for non-contiguous resource allocation. However, the granularity of resource allocation for RA type 0 is coarser than RA type 1. RA type 0 cannot be used for fallback DCI. Then, whether/how to allocate PDSCH resource in non-contiguous DL subbands by RA type 1 needs to be considered. In RAN1#111, the following proposal was discussed but not agreed [2].

Proposed Agreement: Study the impact and benefits of potential enhancements for RA type 1 for PDSCH considering non-contiguous DL resources in frequency.

The following options can be considered to allocate non-contiguous DL resources by RA type 1.

- Option 1: Enhanced VRB-to-PRB mapping for SBFD is introduced

- Option 2: Rate matching around UL subband and guard band is used

- Option 3: Actual PDSCH transmission is based on DL subband regardless of resource allocation

For Option 1, VRBs are consist only of PRBs for DL subbands (i.e., the VRBs are indexed only DL subband. UL subband and guardband are excluded). Figure 6 shows an example. In this example, PRBs consist of 30 RBs and VRBs consist of 18 RBs of DL subbands. One potential issue would be that resource fragmentation can be occurred due to different VRB mapping between SBFD aware UE and non-SBFD aware UE.

For Option 2, the existing rate matching techniques are reused (i.e., RBs of UL band and guardband are not used for PDSCH transmission). The definition of TBS determination based on actual resource allocation could need to be clarified.

For Option 3,	PDSCH resource is allocated regardless of subbands, but actual transmission is carried out on DL subband. This option can be similar behavior to Option 2, but the rate matching functionality is not used.

For each option, how to adjust guardband size can be a potential discussion point. Since the required guardband size can be dependent on CLI, the adjustment of guardband size by resource allocation would be beneficial (e.g., guardband size can be increased by not allocating DL resource at edge of DL subband, or guardband size can be reduced by allocating DL resource on guardband).

Proposal 8: Study impact and potential enhancements for RA type 1 for PDSCH.

Figure 6  Enhanced VRB-to-PRB mapping

CSI-RS

In RAN1#110bis-e, to study impact and potential enhancements of CSI-RS resource was agreed as follows.

Agreement (RAN1#110bis-e) Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands.

Potential issues on the current specification would be that CSI-RS resource allocation needs to be contiguous and the smallest configurable number of RBs is 24 or bandwidth of BWP. For {DUD} subband pattern, how to configure frequency domain resource of CSI-RS resource set and CSI reporting need to be considered. The following proposal was discussed in RAN1#111 [2].

Proposed Agreement: For CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subband, consider the following options: - Option 1: Two contiguous CSI-RS resources link to one CSI report - Option 2: Non-contiguous CSI-RS resource - Option 2-1: Non-contiguous CSI-RS resource allocation - Option 2-2: Contiguous CSI-RS resource allocation and non-contiguous CSI-RS resource derived by excluding frequency resources of UL subband and guardband(s) - Option 3: Contiguous CSI-RS resource configuration with overlapping resources with UL subband and guardband(s) - Option 3-1: UE skips a CSI measurement and report for a CSI reporting configuration if any CSI-RS resource collides with UL subband or guardband - Option 3-2: UE skips the subband CSI reporting to a CSI subband colliding with UL subband and guardband(s) - Option 3-3: CSI reporting setting configures that the CSI is not reported for CSI subband(s) colliding with UL subband and guardband(s) - FFS applicability for TRS

Among these options, Option 3-3 can be achieved by the existing specification (i.e., reportFreqConfiguration in CSI-ReportConfig IE is used). If there is no strong motivation, Option 3-3 would be simple and sufficient. From our perspective, to report inaccurate CSI values on UL subband or guardband is also fine.

Proposal 9: For CSI-RS resource allocation, support either of the following.

- CSI reporting setting configures that the CSI is not reported for CSI subband(s) colliding with UL subband and guardband(s)

- CSI is also reported for CSI subband(s) colliding with UL subband and guardband(s) but the reported value can be inaccurate

Other potential enhancements

Timing alignment

NTA,offset can be used for switching gap from UL symbol to DL symbol at gNB. On SBFD symbol, there are DL subband and UL subband in the same symbol. If NTA,offset (and NTA) are set to non-zero value, DL and UL boundaries are not aligned at gNB. Thus, whether/how to align DL and UL boundaries needs to be considered. The following options can be considered.

Option 1: DL and UL timings are aligned regardless of SBFD symbols and normal symbols

Option 2: DL and UL timings are aligned within SBFD symbols, but UL timings between SBFD symbols and normal symbols are not aligned

Option 3: DL and UL timings are not aligned within SBFD symbols, but UL timings between SBFD symbols and normal symbols are aligned

Figure 7 shows an example of each option. In this example, slot#0 and #4 are normal DL slots (i.e., slot consists of normal DL symbols), slot#3 are normal UL slot, slot#1 and #2 are SBFD slots. There are three subbands in SBFD slots. In this figure, gap symbols from DL to UL are omitted. “DL/UL relation” in the figure shows the symbol type and symbol timing at gNB. Whether DL and UL timings are aligned depends on how NTA,offset is applied as described below.

- For option 1, zero value of NTA,offset is applied for both SBFD UL symbols and normal UL symbols.

- For option 2, zero value of NTA,offset is applied for SBFD UL symbols and non-zero value of NTA,offset is applied for normal UL symbols.

- For option 3, non-zero value of NTA,offset is commonly applied for both SBFD UL symbols and normal UL symbols.

For option 1, DL and UL boundaries are aligned in any symbols. However, there is no switching gap from UL to DL (between slot#3 and #4). If switching gap is necessary, it can be realized, e.g., by setting proper NTA, allocating gap symbol by the gNB resource assignment, not receiving some part of symbol by gNB. When the existing deployment is not zero value of NTA,offset  i.e. not small cell, it can have the backward compatibility to legacy deployment. For option 2, DL and UL boundaries are aligned within SBFD symbols. Although switching gap between slot#3 and #4 is available, symbols are overlapped between slot#2 and #3. This ensures to keep existing deployment for normal symbols. For option 3, DL and UL boundaries in SBFD symbols are not aligned. There are overlapping between DL and UL symbols. However, if gap symbols from DL to UL are allocated by the gNB resource allocation, the overlapped symbol is included in gap symbols (as in legacy operation).

Proposal 10: Study how to configure timing advance offset for SBFD symbol and normal symbol

Figure 7  Timing advanced offset with SBFD operation

Interference mitigation schemes

The limitation of UL transmission power can be considered. Assuming the case where DL transmission is interfered by UL transmission of adjacent subband, UL transmission power can affect CLI intensity. According to our simulation result [3], DL performance is degraded depending on UL transmission power in adjacent subband. Thus, it would be beneficial that UL transmission power is differentiated between SBFD symbol and normal UL symbol especially for higher layer configured UL transmission (i.e., the maximum UL transmission power is limited on SBFD symbols).

For coordinated scheduling, to exchange SBFD configurations between gNBs can be considered. In RAN1#110bis-e, whether the exchange of SBFD configurations should be discussed in AI 9.3.2 or AI 9.3.3 was discussed. Based on the guidance from RAN1 chair, the exchange of intended subband configurations for SBFD operation across gNBs is to be handled in AI 9.3.2. The information exchange can include at least the subband configuration and the slot configuration for each subbands. The details of the information need to be discussed based on how to configure subband locations in time and frequency domain. Based on the information of SBFD configurations, gNB can adjust its own scheduling strategy to handle inter-gNB CLI.

Proposal 11: Study potential schemes for interference mitigation such as UL transmission power limitation, and exchange SBFD configurations.

Conclusion

In this contribution, we made the following observation and proposals.

Observation 1: It is not sufficient to operate SBFD only over legacy semi-static Flexible symbol/slot. Method to utilize legacy semi-static DL symbol/slot should be studied.

Proposal 1: For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, support Option 1.

Proposal 2: Frequency location of subbands types other than UL subband (i.e., DL subband, guard band if supported) can be also explicitly indicated.

Proposal 3: The legacy semi-static configuration of RB set can be basically reused but the configuration should be sent over SIB.

Proposal 4: Consider to introduce a new semi-static slot format for Rel-18 (and beyond) where the legacy semi-static DL symbol/slot can be re-configured as UL or Flexible symbol/slot.

Proposal 5: Study potential enhancement on configured UL transmission to associate with either normal UL symbol or SBFD symbol.

Proposal 6: For PUSCH with dynamic grant, the same FDRA is used between SBFD symbol and normal symbol.

Proposal 7: For PDCCH resource allocation, not to change the CORESET regardless of SBFD symbol can be considered.

Proposal 8: Study impact and potential enhancements for RA type 1 for PDSCH.

Proposal 9: For CSI-RS resource allocation, support either of the following.

- CSI reporting setting configures that the CSI is not reported for CSI subband(s) colliding with UL subband and guardband(s)

- CSI is also reported for CSI subband(s) colliding with UL subband and guardband(s) but the reported value can be inaccurate

Proposal 10: Study how to configure timing advance offset for SBFD symbol and normal symbol

Proposal 11: Study potential schemes for interference mitigation such as UL transmission power limitation, and exchange SBFD configurations.

- Reference

- RP-222110, “Revised SID: Study on evolution of NR duplex operation”, CMCC

- R1-2212735, Summary #3 of subband non-overlapping full duplex, Moderator (CATT)

- R1-2204135, “Discussion on evaluation on NR duplex evolution”, Panasonic