3GPP TSG RAN WG1 #112	R1-2300919

Athens, GR, February 27th – March 3rd, 2023

Source:	Sharp

Title:	Two TAs for multi-DCI

Agenda Item:	9.1.1.2

Document for:	Discussion and Decision

Background

In RAN#94e, the working item to enhance both downlink and uplink MIMO operations in Rel-18 was agreed [1].

According to the WID, the following item needs to be studied, and if justified, specified:

- Two TAs for UL multi-DCI for multi-TRP operation

Discussions

  - TAC signaling

For TAC signaling, the following agreement was made [2]:

Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support enhancements related to indicating TAG ID via absolute TA command: - FFS: whether the indication is implicit or explicit - Detailed indication schemes are FFS - This does not preclude indication of two TAG IDs (if supported) - Note: This applies at least to MSGB in case of C-RNTI

Although a TAG is defined as a group of serving cells having the same TA, for Rel-18, a serving cell can belong to two TAGs. However, a TA value from absolute TA command is indicated for the serving cell via MAC while an update TA value is associated with a TAG ID in MAC CE. That is, it is unknown that a TA value from absolute TA command is associated with which TAG. For this reason, we propose that the TAG ID should be included in the absolute TA command.

Proposal 1: TAG ID should be included in the absolute TA command.

  - Initial TAC signaling

For initial TAC signaling, the following agreements were made [2]:

Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support CFRA triggered by PDCCH order for both intra-cell and inter-cell cases. Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, there is no consensus to support enhancements for CBRA triggered by PDCCH order. Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it is up to RAN2 to decide whether there is a need to enhance CBRA procedure to support per TRP UE-initiated RACH procedure. Agreement: Send LS to RAN2 regarding the above conclusion. Final LS in R1-2213004.

We summarize the above agreements as below:

|  | Intra-cell MTRP | Inter-cell MTRP |
| --- | --- | --- |
| CFRA triggered by PDCCH order | Support | Support |
| CBRA triggered by PDCCH order | No enhancement | No enhancement |
| CFRA triggered by UE |  |  |
| CBRA triggered by UE | Up to RAN2 | Up to RAN2 |

CFRA triggered by PDCCH order for inter-cell MTRP

For initial TAC signaling, the following agreements were made:

Working Assumption For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI - the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP at least for inter-cell Multi-DCI. - FFS: for intra-cell Multi-DCI - FFS: whether there are any restrictions needed - FFS: if cross TRP RACH triggering is an optional feature Agreement For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, support a mechanism to determine which PRACH configuration (i.e., RACH configuration corresponding to serving cell PCI or an additional PCI) to be used in the RACH procedure triggered by PDCCH order - FFS: Explicit indication or implicit indication through PDCCH order Agreement For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, support one of the alternatives (down selection to be done in RAN1#111): - Alt 1: PDCCH scheduling RAR will always be received from serving cell there is no need for additional type 1 CSS configuration per additional PCI - Alt 2: In addition to PDCCH scheduling RAR being received from serving cell, reception of PDCCH scheduling RAR from a TRP corresponding to an additional PCI for a RACH procedure associated to the additional PCI is supported additional type 1 CSS configuration per additional PCI needs to be supported

RA triggering

For Rel-17 PDCCH order triggered random access, one of PRACH resources is selected as a set of PRACH occasion and PRACH preamble from RACH-ConfigCommon. However, this PRACH resource is for single-TRP (i.e., TRP#1) for inter-cell MTRP case. In RAN1 110bis e-meeting, it was agreed that an additional RACH configuration is introduced for inter-cell multi-DCI based MTRP operation with two TA enhancement. Furthermore, PRACH resources of the additional RACH configuration are used for TRP#2 because the additional RACH configuration is associated with each configured additional PCI. For these reasons, PDCCH order needs to select RACH-ConfigCommon for TRP#1 or RACH configuration associated with additional PCI for TRP#2.

Observation 1: For RA triggered by PDCCH order in inter-cell MTRP, PDCCH order needs to select RACH-ConfigCommon for TRP#1 or RACH configuration associated with additional PCI for TRP#2.

To select one of RACH configurations, there are two alternatives as the following:

- Alt 1: Explicit indication of one of RACH configurations by PDCCH order

- Alt 2: Implicit indication of one of RACH configurations by PDCCH order

For Alt 1, a new DCI field in PDCCH order indicates a value, and one of RACH configurations can be selected based on the value. For example, when the value is 0, PRACH resources of RACH-ConfigCommon are used for TRP#1 (i.e., perform legacy procedure). When the value is 1~7, PRACH resources of Rel-18 RACH configuration associated with the additional PCI corresponding to the value is used for TRP#2. Furthermore, the current PDCCH order has 12 Reserved bits. Therefore, the new DCI field does not increase DCI overhead.

For Alt 2, CORESET pool index of PDCCH order can be used. For example, if CORESET pool index is 0, RACH-ConfigCommon is used to select PRACH resource for TRP#1. If CORESET pool index is 1, Rel-18 PRACH configuration associated with additional PCI is used to select PRACH resource for TRP#2. However, for Rel-17, up to 7 AdditionalPCIIndex can be configured, and switching between 2 CORESET pool index values cannot identify one of AdditionalPCIIndex. For this reason, we support Alt 1.

Proposal 2: For RA triggered by PDCCH order in inter-cell MTRP, PDCCH order should explicitly indicate one of RACH configurations.

RAR reception

For RAR reception, the UE monitors RAR in type 1 CSS set. In Rel-15, RAR for secondary TAG establishment is monitored in the type 1 CSS set of PCell even if the UE transmits PRACH preamble to SCell. This is because additional type 1 CSS set for SCell has large burden on the UE and UE’s blind decoding. Therefore, for inter-cell MTRP, RAR should be always received from serving cell (i.e., there is no need for additional type 1 CSS set per additional PCI).

Proposal 3: For RA triggered by PDCCH order in inter-cell MTRP, RAR should be always received from serving cell (i.e., there is no need for additional type 1 CSS set per additional PCI).

CFRA triggered by PDCCH order for intra-cell MTRP

For initial TAC signaling, the following agreements were made:

Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support at least one of the following alternatives (down selection to be done in RAN1#111): - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 2: indicate TAG ID as part of PDCCH order - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 4: divide RACH resources into two groups, where for a RACH procedure, if the corresponding RACH resource belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 5: divide preambles into two groups, where for a RACH procedure, if the corresponding preamble belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP - Alt 6: TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order - Alt 7: Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order Note: If Alt 1 or Alt 2 is downselected, then it does not preclude indication of two TAG IDs (if supported)

RA triggering

In our view, for intra-cell mTRP case, additional RACH configuration is unnecessary because the association between SS/PBCH and RACH preamble/PRACH occasion is the same among TPRs.

Proposal 4: For RA triggered by PDCCH order in intra-cell mTRP, additional RACH configuration is unnecessary.

However, the UE needs to identify whether RA triggered by PDCCH order is for second TA acquisition in the serving cell. There are alternatives to indicate second TA acquisition as the following:

- Alt 1: Explicitly indicate TAG ID as part of TA command in RAR

- Alt 2: Explicitly indicate TAG ID as part of PDCCH order

- Alt 3/4/5: Reserve RA resource for two TA acquisition (grouping of SSB or preambles)

- Alt 6: Implicitly indicate TAG ID as CORESETPoolIndex of PDCCH order

- Alt 7: Implicitly indicate TAG ID as TCI state used to receive PDCCH order

Our 1st preference is either Alt 1 or Alt 2, and 2nd preference is Alt 3/4/5. For Alt 3/4/5, there is a little standardization effort such that the number of SSBs/RA resources for each group is defined. Alt 6 has less flexibility than Alt 1/2 because TRP#1 cannot trigger RA procedure toward TRP#2. For Alt 7, since a Rel-16 TCI state for PDCCH is associated with CORESETPoolIndex, it is similar to Alt 6. Furthermore, since a Rel-18 DL/Joint TCI state is not defined yet, Alt 7 should not be supported for sake of progress.

Proposal 5: For RA triggered by PDCCH order in intra-cell MTRP,

- 1st preference: TAG ID should be explicitly indicated as part of PDCCH order or part of TA command in RAR.

- 2nd preference: Two groups of SSB and/or RA resources should be supported.

  - Association between TAG and target UL channel

For association between TAG and target UL channel, the following agreement was made:

Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, the four options agreed in RAN1#110bis-e are refined as below (down-selection of one or a combination of the options to be performed in RAN1#111): - Option 1: Associate TAG to TCI-state/spatial relation - Configure TAG ID as part of UL/joint TCI state or spatial relation - for UL transmission, the TAG ID associated with the UL/joint TCI state or spatial relation is utilized - Option 2: Associate TAG to CORESETPoolIndex - for dynamically scheduled/activated PUSCH, TAG associated with the CORESET pool index of the CORESET carrying the scheduling/activating PDCCH is utilized for UL transmission - for Type 1 CG, P/SP-SRS, and P/SP-PUCCH, coresetPoolIndex is RRC-configured. - FFS: Other signals/channels: AP-SRS, and dynamic HARQ-ACK - Option 3: Associate TAG to SSB group (if such an association is agreed in agenda 9.1.1.2). For a UL transmission, UE adopts the TAG associated with the SSB group such that - if the PL RS is an SSB, then the UE adopts the TAG associated with the SSB group which the PL RS of the UL transmission belongs to - if the PL RS is a CSI-RS, then the UE adopts the TAG associated with the SSB group which the QCL source SSB of the PL RS belongs to - Option 4: TAG association performed as follows: - for dynamically scheduled/activated channels/signals, TAG associated with the CORESET pool index of the CORESET carrying the scheduling PDCCH is utilized for UL transmission - for P/SP UL channels / signals (not scheduled or activated by DCI), TAG ID is RRC-configured.

For Option 1, if the UE supports UL TCI state, it can include TAG (or TAG ID). However, if the UE does not support UL TCI state, association between spatial relations for PUSCH/PUCCH/SRS and TAG needs to be defined. In our view, it has large spec impacts.

For the current spec, the difference between multi-DCI based MTRP and single-DCI based MTRP is specified by using CORESET pool index. Furthermore, two TAs are supported only for multi-DCI based MTRP in AI.9.1.1.2. Therefore, it is reasonable to associate TAG to CORESET pool index (Option 2). For Option 1, even if UL TCI state indicates a TAG ID, there is a case that all CORESETs included in active BWP have the same CORESET pool index and the TAG ID is not used. For this reason, we support Option 2.

Proposal 6: TAG should be associated with CORESET pool index.

  - Overlapping problem

For overlapping problem, the following agreement was made:

Agreement For multi-DCI based multi-TRP operation with two TAs, study how to handle overlapping part between two UL transmissions associated with two TAs, where the study includes: - whether to introduce scheduling restriction in overlapping part - whether to introduce dropping rules - whether specification impact is needed, or if the issue can be handled via implementation - whether to allow overlapped transmission in case the UE supports STxMP transmission (if STxMP feature is agreed in NR Rel-18) Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it cannot always be assumed that both TRPs have knowledge of the overlapping region between transmissions corresponding to the two TAs. - Note: This doesn’t prevent the network from applying scheduling restrictions even if the TRPs have no knowledge of the overlapping region

Figure 1: Overlapping problem.

Even for two TDM PUSCHs, they are potentially overlapped due to different TA values as shown in Figure 1.

In our view, there are two options to solve the overlapping problem as the following:

- Option 1: Introduce scheduling restriction

- Option 2: Introduce dropping rule

In Figure 1, although two PUSCHs are consecutively TDMed, the overlapping problem occurs. That is, from logical-time line perspective, they are not overlapped. For this reason, it is not reasonable to introduce dropping rule based on logical-time line and channel priority logically. If dropping rule is supported for overlapping part, the former part should be always transmitted and the latter part should be reduced.

Proposal 7: For supporting dropping rule for overlapping part, the latter part should be reduced.

Furthermore, we think Option 1 is also reasonable and simple. For example, a gap of two UL transmissions having different TAs shall be larger than N symbols/slots. This N can be determined based on UE capability and/or the maximum difference of the TAs.

Proposal 8: For supporting scheduling restriction, a gap between two UL transmissions having different TA values should be larger than N symbols based on UE capability and/or the maximum difference of the TA values.

However, if scheduling restriction is supported, we think unexpected error case occurs. For example, there is a case that CG-PUSCH with TA#1 is scheduled during TA switching from TA#2 to TA#1. In the case, CG-PUSCH shall be dropped because TA#1 cannot be used. Furthermore, if only supporting dropping rule, high priority channel (e.g., PUCCH with HARQ-ACK) is potentially dropped. Therefore, we support to introduce both scheduling restriction and dropping rule.

Proposal 9: Support both scheduling restriction and dropping rule.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: TAG ID should be included in the absolute TA command.

Proposal 2: For RA triggered by PDCCH order in inter-cell MTRP, PDCCH order should explicitly indicate one of RACH configurations.

Proposal 3: For RA triggered by PDCCH order in inter-cell MTRP, RAR should be always received from serving cell (i.e., there is no need for additional type 1 CSS set per additional PCI).

Proposal 4: For RA triggered by PDCCH order in intra-cell mTRP, additional RACH configuration is unnecessary.

Proposal 5: For RA triggered by PDCCH order in intra-cell MTRP,

- 1st preference: TAG ID should be explicitly indicated as part of PDCCH order or part of TA command in RAR.

- 2nd preference: Two groups of SSB and/or RA resources should be supported.

Proposal 6: TAG should be associated with CORESET pool index.

Proposal 7: For supporting dropping rule for overlapping part, the latter part should be reduced.

Proposal 8: For supporting scheduling restriction, a gap between two UL transmissions having different TA values should be larger than N symbols based on UE capability and/or the maximum difference of the TA values.

Proposal 9: Support both scheduling restriction and dropping rule.

Observation 1: For RA triggered by PDCCH order in inter-cell MTRP, PDCCH order needs to select RACH-ConfigCommon for TRP#1 or RACH configuration associated with additional PCI for TRP#2.

References

- [1]	RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Dec., 2021.

- [2]	RAN1 chair’s note, RAN1#111 meeting, Nov. 2022.