3GPP TSG RAN WG1 #112			                          R1-2300976

# Athens, Greece, February 27th – March 3rd, 2023

Source: 	CMCC

Title:	Discussion on RAN2 LS on SPS configuration for unicast and multicast

Agenda item:	5

Document for:	Discussion & Decision

- Introduction

In the LS from RAN2, two questions about unicast and multicast SPS configuration was proposed [1]. In this contribution, the reply of this LS will be discussed.

- Discussion

In RAN2’s LS, two questions about unicast and multicast SPS configuration are proposed to get RAN1’s feedback.

1. Overall Description: RAN2 discussed unicast and multicast SPS configuration and made following agreements for Rel-17: Check with RAN1 whether NW can configure SPS in one BWP using sps-Config and sps-ConfigMulticastToAddModList-r17 simultaneously. Check with RAN1 whether other indexes than 0 can be used if a single DL SPS configuration for multicast is configured.. In the description for sps-Config (IE for single unicast SPS) in the current RRC spec, there’s a restriction that NW cannot configure SPS in one BWP using sps-Config and sps-ConfigToAddModList (IE for one or multiple unicast SPS) simultaneously: sps-Config UE specific SPS (Semi-Persistent Scheduling) configuration for one BWP. Except for reconfiguration with sync, the NW does not reconfigure sps-Config when there is an active configured downlink assignment (see TS 38.321 [3]). However, the NW may release the sps-Config at any time. Network can only configure SPS in one BWP using either this field or sps-ConfigToAddModList.. Q1: RAN2 would like to ask RAN1’s view on whether similar restriction is required when configuring SPS for both unicast and multicast in one BWP, i.e., network cannot use sps-Config to configure unicast SPS and simultaneously use sps-ConfigMulticastToAddModList-r17 to configure multicast in one BWP. Q2: Another question RAN2 would like to ask RAN1 is that when a single multicast SPS is configured for a group of UEs and there is no SPS configuration for unicast for at least one UE in the group, whether sps-ConfigIndex other than 0 can be configured by network for the single multicast SPS. 2. Actions: To RAN1: ACTION: RAN2 respectfully requests RAN1 to provide feedback to the above two questions.

Q1:

Regarding the first question, we think sps-Config and sps-ConfigMulticastToAddModList-r17 can be configured in the same BWP simultaneously, because these two IEs are used to configure unicast SPS and multicast SPS respectively and the UE capabilities reporting are also independent. Considering the case that one UE supports only one unicast SPS configuration but supports multicast SPS configuration(s), this is the only way to configure both unicast SPS and multicast SPS to this UE.

Proposal 1. Reply the LS to RAN2 on Q1: From RAN1’s perspective, NW can configure SPS in one BWP using sps-Config and sps-ConfigMulticastToAddModList-r17 simultaneously.

In Rel-17 multicast SPS design, in addition to group-common DCI with G-CS-RNTI, the UE-specific PDCCH, i.e., DCI with CS-RNTI can also be used to release the multicast SPS, that is DCI with CS-RNTI can be used to release unicast SPS as well as multicast SPS. For differentiating which SPS configuration to be released, the “HARQ process number” field is used to indicate the SPS configuration index. However, there is no sps-ConfigIndex configuration in sps-Config for single unicast SPS. If sps-Config and sps-ConfigMulticastToAddModList-r17 are configured in the same BWP simultaneously, the unicast SPS index can be defined as zero to differentiate the sps-ConfigIndex configured in sps-ConfigMulticastToAddModList-r17 and “HARQ process number” field can still be used to indicate the SPS index. The related draft CR can be found in [2].

Proposal 2. If both sps-Config and sps-ConfigMulticastToAddModList-r17 are configured in one BWP, the unicast SPS index should be defined as zero for default. The “HARQ process number” field with value zero in DCI with CS-RNTI is used to release the unicast SPS. Endorse the draft CR in R1-2300978.

Q2：

Regarding the second question, we think sps-ConfigIndex other than 0 can be configured by network for the single multicast SPS. Consider the case that a single multicast SPS is configured for a group of UEs and there is no unicast SPS configuration for UE#1, but a single unicast SPS is configured for UE#2 by sps-Config. As the discussion in Q1, because there is no sps-ConfigIndex configuration in sps-Config, if both sps-Config and sps-ConfigMulticastToAddModList-r17 are configured in one BWP, the unicast SPS index should be defined as zero for UE#2. Therefore, only the sps-ConfigIndex other than 0 can be configured for the single multicast SPS for differentiating SPS configuration index when using DCI with CS-RNTI to release SPS configuration for UE#2.

Proposal 3. Reply the LS to RAN2 on Q2: From RAN1’s perspective, sps-ConfigIndex other than 0 can be configured by network for the single multicast SPS.

The consequent spec impact is that the “HARQ process number” filed in DCI format 4_1/4_2 should also be used to indicate the SPS index in SPS release DCI if only single multicast SPS is configured, and the related draft CR can be found in [3].

Proposal 4. The “HARQ process number” field in DCI format 4_1/4_2 is used to indicate the SPS index in SPS activation/release DCI when only single multicast SPS is configured. Endorse the draft CR in R1-2300979.

- Conclusions

In this contribution, the answers of SPS configuration for unicast and multicast from RAN2 LS are discussed, and the following proposals are made.

Proposal 1. Reply the LS to RAN2 on Q1: From RAN1’s perspective, NW can configure SPS in one BWP using sps-Config and sps-ConfigMulticastToAddModList-r17 simultaneously.

Proposal 2. If both sps-Config and sps-ConfigMulticastToAddModList-r17 are configured in one BWP, the unicast SPS index should be defined as zero for default. The “HARQ process number” field with value zero in DCI with CS-RNTI is used to release the unicast SPS. Endorse the draft CR in R1-2300978.

Proposal 3. Reply the LS to RAN2 on Q2: From RAN1’s perspective, sps-ConfigIndex other than 0 can be configured by network for the single multicast SPS.

Proposal 4. The “HARQ process number” field in DCI format 4_1/4_2 is used to indicate the SPS index in SPS activation/release DCI when only single multicast SPS is configured. Endorse the draft CR in R1-2300979.

- References

- R1-2300015 (R2-2213108), LS on SPS configuration for unicast and multicast, RAN2, ASUSTEK

- R1-2300978, Draft CR on SPS release PDCCH validation of DCI with CS-RNTI, CMCC

- R1-2300979, Draft CR on PDCCH validation for single multicast SPS, CMCC