3GPP TSG RAN WG1 #112		                                         R1-2301016

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.8.1

Source: 	CMCC

Title: 	Discussion on side control information and NCR behavior

Document for:	Discussion and Decision

Introduction

In RAN1 #111 meeting, the following aspects of NCR was discussed. And a few agreements have been achieved [1]. The detailed agreements are listed in each specific sections.

Specify the signalling and behavior of the following side control information for controlling the NCR-Fwd [RAN1, RAN2] - Beamforming - UL-DL TDD operation - ON-OFF information Note: Power control aspect will be checked in RAN#98e.

In this contribution, we provide our views on the signalling and behavior of beamforming, UL-DL TDD operation and ON-OFF information.

Discussion

The basic assumptions of network-controlled repeater

In the SI phase of NCR, the basic functions of NCR-MT and NCR-Fwd are defined and captured in the TR 38.876. The details are as below.

Agreement Capture the following model of network-controlled repeater in TR 38.867. - The NCR-MT is defined as a function entity to communicate with a gNB via Control link (C-link) to enable the information exchanges (e.g. side control information). The C-link is based on NR Uu interface. - Note: Side control information is at least for the control of NCR-Fwd - The NCR-Fwd is defined as a function entity to perform the amplify-and-forwarding of UL/DL RF signal between gNB and UE via backhaul link and access link. The behavior of the NCR-Fwd will be controlled according to the received side control information from gNB. Agreement Recommend to capture the following examples of the transmission/reception of C-link and backhaul link by NCR in TR 38.867. - The DL of C-link and DL of backhaul link can be performed simultaneously or in TDM way. - The UL of C-link and UL of backhaul link can be performed in TDM way - Note-1: Multiplexing is under the control of gNB with consideration for NCR capability - Note-2: Simultaneous transmission of the UL of C-link and UL of backhaul link is subject to NCR’s capability

The detailed structure of NCR is illustrated in the Figure 1. The NCR-Fwd part only contains the RF units for amplifying and forwarding. The NCR-MT could take control of the NCR-Fwd and reuse the RF units to receive and transmit the side-control information of gNB. Sharing a same set of RF units between NCR-MT and NCR-Fwd would facilitate the measurement and relieve the effort of the control.

The side-control information could be sent to NCR-MT from gNB. And the NCR-MT should also send feedbacks to the gNB not only for the acknowledgement of the control information, but also the measurement reports, such as CSI or RSRP, to maintain the backhaul links. The protocol stack of NCR-MT could base on current specification to reduce the additional development efforts. The NCR-MT can reuse the NR UE chipset to improve the cost efficiency, which also benefit the commercialization of NCR. Thus, the protocol stack of NCR-MT should contain not only the physical layer but also the RRC and MAC layer to realize both semi-static configuration and dynamic indications.

Figure 1 The basic structure of Network-controlled repeater

Proposal 1:

The protocol stacks of the MT part of NCR should contain RRC, MAC and physical layer to support the semi-static configurations and dynamic indications and feedbacks.

Beamforming

FR1 beam information

Agreement At least for FR2, beam information is beneficial and recommended as the side control information for network-controlled repeater to control the behaviour of NCR at least for access link - FFS: Detailed mechanism of indication. - Note: There are no supporting evaluation results on FR1 at this point to reach similar conclusion

During the SI phase, Only the beam information in FR2 is recommended. But in the WID, both FR1 and FR2 are considered for the coverage extension. From our understanding, the beamforming in FR1 is also beneficial for the coverage. The signaling and mechanism of beamforming in FR2 could be reused for FR1 NCR.

The digital beamforming is used at gNB for FR1. But the NCR cannot decode and regenerate the data for UEs. Then the digital beamforming cannot be used for NCR. Analog beamforming could be used for FR1 NCR. For the AC beam of NCR-Fwd in FR1, current beam index-based indication could be reused. For the DL beamforming of the BH link and C-link, the receiving beams of NCR-Fwd depends on implementation. For the UL beamforming of the BH and C-link, the current mechanisms could be reused.

Proposal 2:

Beamforming should be also supported for FR1 NCR.

Proposal 3：

The beam index based schemes could be reused for FR1 NCR for the AC beam indication.

Proposal 4:

The legacy beamforming mechanism could be used for the BH link and C-link for FR1 NCR.

Beams indication for access link

In RAN1#111 meeting, the periodic and aperiodic beam indication for access link were agreed. The details are as follows.

Agreement For each periodic beam indication for access link, one RRC signalling is used including the information defined by the following: - A list of X (1≤X≤Xmax) forwarding resource, each is defined as {Beam index, time resource} - FFS: The value of Xmax, other details Each time resource is defined by {Starting slot defined as the slot offset in one period, starting symbol defined by symbol offset within the slot, duration defined by the number of symbols} with dedicated field. - The periodicity is configured as part of the RRC signaling for periodic beam indication - The same periodicity is assumed for all time resource(s) in one periodic beam indication. - The reference SCS is configured as part of the RRC signaling for periodic beam indication - The same reference SCS is assumed for all time resource(s) in one periodic beam indication. Agreement For each aperiodic beam indication for access link, one DCI is used with the information defined by Option-1: - Lmax fields are used to indicate the beam information and each field refers to one beam index ; - Note: The bitwidth of this field is determined by the number of beams used for access link. - Tmax fields to indicate the time resource; - Note: A list of time resource is pre-defined by RRC signalling. The bitwidth of this field for time resource indication is determined by the length of list. - FFS: The value of Tmax - Down-select between Tmax=1 or Tmax=Lmax. - FFS: How to define the association between time indication and beam indication Each time resource is defined by {Starting slot defined as the slot offset, starting symbol defined by symbol offset within the slot, duration defined by the number of symbols} with dedicated field.

For the periodic beam indication for access link, the maximum number of the forwarding resources is still open. At least two dimensions could be considered for the maximum forwarding resource number. One is the maximal supported beam number. The other is the number of time domain resources, which is related to the periodicity for the beam indication. Currently, 64 SSBs and corresponding CORESET#0 are supported within one periodicity. The maximal value of X should be larger than 64 and could be the multiples of 64. Considering an extreme case that all the SSBs, CORESET#0 and SIB1 could be forwarded in one RRC signaling, the forwarding resource number should be larger than 3*64. An example number, such as 256, could fulfil this case. Another way is to consider slot number of periodicity and the sub-slot scheduling or transmission within one slot. The maximal forwarding resource number could be the multiple of the slot number in one periodicity or the multiple of the slot number of the maximal periodicity. For the 2nd way, the maximal periodicity should be determined first.

Proposal 5:

Two options could be considered for the determination of the maximal number of forwarding resources in one list.

- Option 1: the multiple of maximum beam (or SSBs) number, such as the multiple of 64.

- Option 2: the multiple of the slot number of the maximum periodicity.

Proposal 6:

The multiple of maximum beam (or SSBs) number is slightly preferred. 256 can be the starting point for the value of Xmax .

Multiple periodic beam indications should be configured to match different channels and traffics. Then the maximal number of the forwarding resources in one list is not that critical. If one list cannot fulfil the capacity, multiple lists with same periodicity could be configured.

Proposal 7:

Multiple RRC configurations for periodic beam indication should be supported.

For the aperiodic beam indication, the value of Tmax was still FFS. The Tmax fields are used to indicate the time resources according to the RRC configurations. Two options need to be down-selected. One is that Tmax equals to 1, which means only one time resource is indicated for the aperiodic beam indication in one DCI. And the 2nd option is that Tmax equals to Lmax, which means multiple beams and corresponding time resources are indicated through one DCI signaling. And if multiple time resources and multiple beam indexes are indicated through one signaling, the association between beam index and the time resources should be defined. In the legacy scheduling in the licensed spectrum, one DCI is associated with one short transmission. And multiple transmissions triggered by one signaling is usually supported through the semi-persistent transmissions. In addition, the HARQ-ACK for PDCCH carrying side-control information is not supported. The gNB cannot get the knowledge whether the DCI carrying aperiodic beam indication is correctly received by NCR-MT. If one DCI indication with multiple beam indications is missed, the impact would be more severe than one DCI with single beam indication. Without the support of HARQ-ACK for DCI carrying side-control information, one DCI indication with multiple beam indication should not supported. On the other side, the semi-persistent beam indication could be used to indicate multiple beam indication. There is no need to support duplicated functions for aperiodic beam indication and semi-persistent beam indication.

Proposal 8:

The value of Tmax should equal to one for aperiodic beam indication in DCI, which means that only one time domain resource would be indicated by the aperiodic beam indication.

The Lmax,for the beam indication could equal to one or larger than one. If the Lmax,equals to one, it means only one beam is indicated over the time resource indicated by T. If the Lmax,larger than one, it means multiple beam transmission over the same time resource should be supported, which depends on NCR capability. And there are few discussions on how to support the simultaneous transmission over multiple access beams. Then the Lmax,should also equal to one at least in Rel-18. The association between time indication and beam indication is straightforward consequently.

Proposal 9:

The value of Lmax,equals to one.

Though the time resource for the aperiodic beam indication is indicated in DCI. There is no conclusion about the subcarrier spacing indication of aperiodic beam indication. The subcarrier spacing of the time domain resources should be configured in the RRC. And to facilitate the scheduling and forwarding, the subcarrier spacing should be aligned to the forwarding resources for the periodic beam indication.

Proposal 10:

The subcarrier spacing of the time resource indicated in DCI should be configured by RRC. In addition, the subcarrier spacing should be aligned to the forwarding resources configured for the periodic beam indication.

The semi-persistent beam indication was not fully discussed in the last meeting. FL’s proposal was provided for further discussion [2].

Proposal 1-1-A-3: For semi-persistent beam indication, one of following option is supported: - Option-1: - A list of Y (1≤Y≤Ymax) beam indication is configured by one RRC signaling - Each beam indication in the list corresponds to a list of Z(1≤Z≤Zmax) forwarding resource, each is defined as {Beam index, time resource} - FFS: The value of Zmax, which refers to the maximum beams indicated in one indication. - One MAC CE is used to select one of Y beam indication from the list - Option-2: - A list of P(1≤P≤Pmax) forwarding resource is configured by one RRC signaling, each is defined as {Beam index, time resource} - One MAC CE is used to select Z forwarding resource(s) from the list - FFS: The value of Z (1≤Z≤Zmax) , where Zmax refers to the maximum beams indicated in one indication.

Two options were provided for down-selection. In Option-1, a list of beam indication is configured by RRC. And each of the Y beam indication also corresponds to a list of forwarding resources. The RRC configured “Y beam indication” in Option-1 is actually a combination of multiple lists of forwarding resources. Strictly speaking, the Option-1 is defining a combination of beam indications or a combination of beam indication lists. The Option-1 could be updated as follows.

- Option-1-updated:

  - A list set of Y (1≤Y≤Ymax) beam indication lists is configured by one RRC signaling(s)

    - Each beam indication list in the list set corresponds to a list of Z(1≤Z≤Zmax) forwarding resource, each is defined as {Beam index, time resource}

    - FFS: The value of Zmax, which refers to the maximum beams indicated in one beam indication list.

  - One MAC CE is used to select one of Y beam indication list from the list set of Y beam indication lists.

Proposal 11:

The Option-1 for semi-persistent beam indication can be updated as below to be more clear.

- Option-1-updated:

  - A list set of Y (1≤Y≤Ymax) beam indication lists is configured by one RRC signaling(s)

    - Each beam indication list in the list set corresponds to a list of Z(1≤Z≤Zmax) forwarding resource, each is defined as {Beam index, time resource}

    - FFS: The value of Zmax, which refers to the maximum beams indicated in one beam indication list.

  - One MAC CE is used to select one of Y beam indication list from the list set of Y beam indication lists.

The Option-2 defines a forwarding resource list in RRC configuration. And one MAC CE are used to select several forwarding resources from the list. The difference between Option-1 and Option-2 are not too much. Option-1 has pre-defined multiple lists of forwarding resources and select one. Option-2 has a forwarding resource pool and the MAC CE are used to select several resources from it. Compared with Option-1, the MAC CE in Option-2 would be larger than Option-1. A small size MAC CE can provide a better coverage and limited overhead for side control information.

Proposal 12:

The updated Option-1 is preferred for the semi-persistent beam indication.

Since multiple beam indication behavior are defined, the confliction between multiple beam indications should be discussed. FL provided a proposal could be considered as a starting point.

Proposal 1-1-A-4: For the priority rule of the beam indication among periodic, semi-persistent and aperiodic indication, one of following option is supported: - Option-1: No conflict is expected on the beam indication from different types of indication. - Option-2: If there is conflict among beam indication from different type of indication, the order of priority is defined as: Aperiodic beam indication > semi-persistent beam indication >periodic beam indication - Option-3: If there is conflict among beam indication from different type of indication, the order of priority is defined as: Periodic beam indication >aperiodic beam indication> semi-persistent beam indication

Periodic beam indication for access link through one RRC signalling is agreed. According to the agreement, multiple periodic beam indication could be delivered through multiple RRC signallings. It is expected that the multiple periodic beam indication should not conflict between each other.

Proposal 13:

The multiple periodic indications through RRC signaling should not conflict between each other.

Periodic forwarding resources should be used for forwarding important channels and reference RSs, such as SSBs, SIB1, CSI-RSs and RACH occasions which are critical for initial access and measurements. Those forwarding resources should not be overridden by semi-persistent and aperiodic beam indications. Since there are no explicit defined forwarding resources for those important signals and channels, it should depend on gNB’s scheduling which has the full knowledge of signals and channels transmission. Then Option 1 with no confliction and option 2 in which aperiodic beam indication has the highest priority is preferred. compared with Option 1, Option 2 has more flexibility. Then option 2 is slightly preferred.

Proposal 14:

For the priority rule of the beam indication among periodic, semi-persistent and aperiodic indication, option 2 is slightly preferred.

- Option-2: If there is conflict among beam indication from different type of indication, the order of priority is defined as: Aperiodic beam indication > semi-persistent beam indication >periodic beam indication

Beams indication for backhaul link

In the last meeting, the indication of backhaul link and the predefined rule for backhaul link determination is agreed.

Agreement The semi-static beam indication for backhaul link is supported as: - If the beam indication framework in Rel-15 is used for NCR-MT - The DL beam is indicated by MAC CE to select one of TCI state ID from the RRC-configured list of beams for C-link - The UL beam is indicated by SRI on C-link via MAC CE. - If the beam indication framework in Rel-17 is used for NCR-MT - The DL and UL beam are indicated by MAC CE to select one of TCI state ID from the RRC-configured list of beams for C-link Agreement The following pre-defined rules are applied to determine the beam for backhaul link: - In the time domain resource with simultaneous downlink reception or uplink transmission in C-link and backhaul link, the beam of backhaul link is the same as the beam of C-link regardless whether there is beam indicated by the dedicated signal for backhaul link. - In the time domain resource without simultaneous downlink reception or uplink transmission in C-link and backhaul link, if the NCR does not support capability with the new signalling for backhaul beam indication or if no beam is indicated for backhaul link by the dedicated signal, - When Rel-15/16 beam indication framework is used for C-link, - The beam determined by QCL assumption for CORESET with the lowest ID and spatial relationship for PUCCH with lowest PUCCH resource ID in the C-link is applied for the DL and UL of backhaul link, respectively. - When Rel-17 beam indication framework (i.e., unified TCI framework) is used for C-link, the indicated unified TCI for C-link DL and UL is applied for the DL and UL of backhaul link, respectively. Otherwise, the beam indicated by the dedicated signalling is applied for backhaul link.

When the C-link and backhaul link works simultaneously, the BH link would follow the beam indication of the C-link. But when C-link and backhaul link does not work at the same time, the backhaul link should follows the signalling of BH beam indication carried by MAC CE. Currently no RRC based BH link beam indication is defined.

On the other side, periodic, semi-persistent and aperiodic beam indication for access link have been defined and would be defined. The periodic access link beam indication is carried in RRC signalling. Theoretically, the periodic access link beam can be indicated through the RRC signalling when the necessary configuration of a repeater is delivered to the NCR-MT. But at this time, the MAC CE for the beam indication of backhaul link is still not received. Though the default behaviors are defined when no BH link beam is indicated, the NCR cannot work properly. It is proposed to defined a RRC based backhaul beam indication which can be configured with the periodic access beam indication simultaneously. In this condition, after the first set of RRC configuration is received, the NCR-Fwd can work properly without the MAC CE delivered backhaul link beam indication. The currently defined MAC CE based backhaul link beam indication could also be used to update the RRC based BH link beam indication.

Proposal 15:

An RRC based backhaul link beam indication should be defined.

Although when BH link and C-link work simultaneously, the C-link and BH link would share a same beam due to the hardware limitation. But the consideration of the scheduling strategy for control link and BH link would be different. The scheduling of C-link could be more flexible and combined with other normal UEs. But the backhaul link transmission of NCR-Fwd would require a high reliability and high SINR for the data throughput.

Besides, the interference situation of NCR-Fwd BH link is different from the C-link of NCR-MT. The NCR-Fwd should receive the signal from one side, amplified internally and transmit it from the other side. Due to the internal delay of the process is in a nano second level, it is possible that the NCR-Fwd transmitting could interfere the NCR-Fwd receiving.

As illustrated in the Figure 2 below, the DL data is transmitted from gNB to NCR through the BH link. In the AC link, the data is forwarded to the UE. It possibly happens that the transmission signals of NCR-Fwd AC link are reflected by other objectives and received by the NCR-Fwd receivers. Then the BH link is interfered by the AC link beams. On the other side, the situation of C-link is different. Since NCR-MT does not need to forward the side control information to other UEs.

There is no transmission from the NCR while it is receiving the DL signals. No self-interference would happen between AC link and C-link. Thus the beams of C-link have less limitation compared with the BH link. However, the self-interference is possible to happen at NCR-Fwd. The self-interference issue should be considered for the beam determination and indication of BH link.

Observation 1:

- For data forwarding, it is possible that the DL reception at through the BH beam could be interfered by the transmission in Access link beam.

- When C-link works alone, there is no interference issue as data forwarding.

Proposal 16:

The self-interference issue should be considered for the beam determination and indication of BH link.

Figure 2 The illustration of self-interference of NCR-Fwd

Figure 3 The multiplexing of side control information and data forwarding

Considering the self-interference between BH beam and AC beam when data is forwarded by NCR-Fwd, the scheduling of certain BH beams and the AC beams simultaneously should be avoided. The coverage of access UEs are provided by the beams of the AC link of NCR. If self-interference happens, the AC beam cannot be suspended. But the beam of BH link could be changed to reduce the self-interference. Currently only one backhaul beam can be indicated through the MAC CE according to the latest agreements. Once self-interference happens between the AC beam and BH link beam, there is no opportunity to reduce the interference. Multiple BH link beams should be supported for the self-interference reduction. And the association between BH link beam and AC link beam should be defined to indicate which BH beam and AC beam should work simultaneously.

Proposal 17:

Multiple BH link beams should be supported for the self-interference reduction.

Beam management for initial access

As the NCR is transparent to the UEs, if a UE want to access the gNB, the NCR should be able to forward the SSB to the access UE and could also forward the PRACH from the UE to the gNB in the proper uplink slots to accomplish the initial access procedure. Unlike the CSI-RS, the beams of SSB from gNB are relatively fixed and pointing to different directions to provide a basic coverage.

Two schemes could be considered for NCR to provide qualified SSBs to the access UEs. The 1st one is as illustrated in Figure 6. The NCR detects the best or qualified SSBs which will be forwarded for the access links. This scheme has less impact to the configuration of gNB or network planning. But the power of the forwarded SSBs varies due to that they are received from different directions and using various receiving beams. But after the synchronization and initial access, the gNB can still use the best beam for CSI-RS and Data forwarding. The 2nd scheme is that the gNB pre-allocates some SSBs from the maximum 64 SSBs for NCR. And those SSBs are transmitted using a same beam or in a same direction at the backhaul link, as illustrated in Figure 7. And at the access link, the NCR would forward the SSBs to the different directions. This scheme could provide the SSBs with similar power. But this scheme requires to add or change the SSB beams of the gNB if a new NCR is introduced. If all the NCR use different SSBs or different SSB indexes, the number of NCR that could be introduced to the gNB is limited. And this also impacts the number of SSBs used by gNB directly serving the UEs. In the practical procedure of network planning and operation, it will bring a large amount of additional workload if we have to update the configuration of gNB to change the SSB pattern every time when a new NCR is introduced.

Figure 6 Scheme 1 using different BH beams

Figure 7 Scheme 2 using a same backhaul beam

Both schemes have its own advantages and drawbacks. The Scheme 1 is easy to deployed and without impact to the gNB’s configuration. But the power strength of the forwarded SSBs are not even. And if the number of the qualified SSBs is small, the NCR coverage would be loss due to not enough qualified SSBs could be forwarded for the access link. The advantage of the Scheme 2 is that the forwarded SSBs have a similar transmit power. But the total number of NCR under one gNB is limited. And the updating of gNB’s configuration for SSB allocation or SSB pattern updates will bring additional work load in the practice. Both schemes have their own using scenarios and should be considered in the following studies.

Proposal 18:

Both schemes have their own usage scenarios and should be considered in the following studies.

- Scheme 1: The NCR only forwarded the qualified SSBs without the change of SSB beams at gNB.

- Scheme 2: A set of SSBs are allocated for NCR using a same directional backhaul link.

UL-DL TDD operation

It was proposed to specify the signalling and behavior of NCR for the UL-DL TDD operations. In the previous meetings, the following agreements have been achieved.

Agreement For the TDD UL/DL configuration of network controller repeater: - At least semi-static TDD UL/DL configuration is needed for network-controlled repeater for links including C-link, backhaul link and access link. - FFS: handling of flexible symbols - Note1: The same TDD UL/DL configuration is always assumed for backhaul link and access link - Note2: The same TDD UL/DL configuration is assumed for C-link and backhaul link and access link if NCR-MT and NCR-Fwd are in the same frequency band. Agreement For the signaling of information on UL-DL TDD configuration, if the NCR-MT can acquire the TDD configuration as legacy UEs or from the OAM, new signaling may not be necessary. - Note 1: The same TDD UL/DL configuration is assumed for C-link and backhaul link and access link if the NCR-MT and the NCR-Fwd are in the same frequency band. - FFS: Other cases where new signaling may be necessary. Agreement For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), following options are considered for the NCR-Fwd on these symbols - Option 1: The NCR-Fwd is expected to be OFF or not forwarding over these symbols - Option 2: The NCR-Fwd will follow the TDD operation determined by NCR-MT, i.e., determined by NCR-MT based on the received SFI indication or scheduling from gNB - Note: It means that no new side control signalling is needed. - Option 3: The NCR-Fwd will follow a new dynamic side control signalling of DL/UL forwarding over these symbols to NCR-Fwd Agreement For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the default behavior of the NCR-Fwd is expected to be OFF or not forwarding over these symbols - FFS: The behavior over these symbol if dynamic DL/UL operation is supported by NCR-MT and/or NCR-Fwd. Conclusion For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the dynamic DL/UL operation of NCR-MT and NCR-Fwd is not supported in Rel-18.

It is common understanding that the NCR-MT could acquire the TDD configurations from the gNB work as a legacy UE. And if the NCR-MT and NCR-Fwd are in the same frequency band, the same TDD UL/DL configuration is assumed for C-link, backhaul link and access link. Then no additional UL-DL TDD configurations are needed for the NCR.

Proposal 19:

The NCR-MT could acquire the TDD configuration from the gNB as a legacy UE. The legacy TDD configurations could be reused for the NCR.

ON-OFF information

In the last meeting, the indication of ON-OFF information was discussed. The related agreements are listed below.

Agreement For FR2, the “ON” state of NCR-Fwd is indicated: - Alt-2: Implicit indication via the beam indication (i.e., if there is beam indication, the NCR is assumed to be ON over the indicated time domain resource associated with corresponding beam(s)) Agreement For FR1, the “ON” state of NCR-Fwd is indicated: Alt-2: Indication via the beam indication (i.e., if there is beam indication, the NCR is assumed to be ON over the indicated time domain resource associated with corresponding beam(s)) - When there is only one beam, the sole purpose of the beam indication is for indicating “ON” state of NCR-Fwd

From the operation point, a semi-static configuration of on-off pattern should be supported for the basic communication between the gNB and access UEs. During the semi-static configured ON resources, the NCR could forward the periodic signals and channels to the access UEs, such as SIB information, SSB, and periodic CSI-RS for initial access and measurements. And in the uplink, the NCR could also forward the periodic signals and channels from UEs, such as SRS, PUCCHs and PRACH. Within the RRC configured resources, some of the resources could be allocated for the forwarding of PDSCH and PUSCH for traffic. If the RRC configured forwarding resources is not enough to fulfill the traffic load, additional forwarding resources could be indicated through MAC CE or DCI based indications.

In the last meeting, it was agreed that the “On” state is indicated through the beam indication. If there is beam indication, the NCR is assumed to be On over the indicated time domain resource associated with corresponding beam(s). According to the agreements, there is no condition or limitation for the On state indication and beam indication. Then, if the RRC, MAC CE and DCI based beam indication are supported, then it should also applicable to the On state indication.

Proposal 20:

If the RRC, MAC CE and DCI based beam indication are supported, then it should also applicable to the On-state indication.

During the discussion in the previous meetings, an explicit off indication was proposed. If the indication of off state is semi-static, it could be redundant since we already have a conclusion that the NCR-Fwd is expected to be off if no indication is received. And since if the on-states are configured for the forwarding resources, then the other resources should be naturally considered as off. Then the necessity of semi-static configured off states is not obvious. Regarding the dynamic indicated off state, the motivation is also not clear. As discussed above, the RRC configured forwarding resources should support a minimal set of resources for reference signals and channels. It is not reasonable to configure a large amount of resources for data forwarding and then using dynamic indications to turn them off. It also means that the configuration of the large amount resources is also not correct.

Observation 2:

- The motivation of semi-static configured off state is not clear, considering that NCR-Fwd is always expected to be OFF.

- The motivation is also not clear that configuring a large amount of redundant resources and then canceling them dynamically.

Proposal 21:

The explicit configuration or indication of OFF state should be deprioritized or not supported.

As discussed above, the gNB could configure the time domain resources that carrying the broadcast and cell-specific signals/channels as On. And the NCR-Fwd would forward those signals and channels to the access UEs. There is no need for NCR-MT to read or first to interpret the content of those signals/channels and then determine whether to forwarding them.

An RRC configuration of the forwarding time domain resources from gNB is straightforward and efficient. Whether the NCR-MT needs to decode or measure the channels and signals depends on the gNB’s configuration as legacy UEs. Since SSBs are carried in 4 symbols and the PDSCH and PUSCH are usually scheduled in slots, both symbol-level and slot-level on-off configuration and indication should be supported.

Proposal 22:

The time domain resources carrying the broadcast and cell-specific signals could be indicated through the RRC configurations of ON-OFF state by gNB.

Proposal 23:

Both symbol-level and slot-level on-off configurations and indications should be supported.

If the semi-static configured time domain resources for forwarding cannot satisfy the traffic requirement, additional resources could be indicated through the dynamic indications. Both semi-static configured and dynamically indicated On state or the time domain resources should be supported for NCR forwarding. For both semi-static or dynamically indicated forwarding occasions, the beam of AC-link of NCR-Fwd should be indicated.

Proposal 24:

Both semi-static configuration and dynamic indication of On-state should be supported for NCR forwarding.

Power control information

In RANP#98 meeting, the note related to power control is removed from the WID and the NCR specific power control will not be supported [3]. Based on this conclusion, no power control will be supported for NCR. It means that all the transmit power including DL and UL depends on NCR’s implementation. But there are some cases need to be discussed for the determination of the transmit power of uplink. Simultaneous UL transmission of C-link and backhaul link are supported based on NCR’s capability. If the NCR does not support simultaneous transmission of C-link and backhaul link, C-link and backhaul link would work in a TDMed manner. The transmit power of C-link in uplink would be determined based on the mechanism of UE’s power control. And the transmit power of backhaul link could base on implementation, such as OAM or other configurations. If the NCR has the capability to support the simultaneous UL transmission of C-link and backhaul link, C-link transmission from NCR-MT and the backhaul forwarding through NCR-Fwd would happen at the same time. To align or compatible with the transmit power of NCR-MT, the NCR-Fwd could adopt a same PSD as NCR-MT for uplink transmission.

Proposal 25:

In case of simultaneous UL transmission of C-link and backhaul link, the NCR-Fwd could adopt a same PSD as NCR-MT for uplink transmission.

Conclusions

In this contribution, we provide our views on the signalling and behavior of beamforming, UL-DL TDD operation and ON-OFF information. The observations and proposals are as below.

Observation 1:

- For data forwarding, it is possible that the DL reception at through the BH beam could be interfered by the transmission in Access link beam.

- When C-link works alone, there is no interference issue as data forwarding.

Observation 2:

- The motivation of semi-static configured off state is not clear, considering that NCR-Fwd is always expected to be OFF.

- The motivation is also not clear that configuring a large amount of redundant resources and then canceling them dynamically.

Proposal 1:

The protocol stacks of the MT part of NCR should contain RRC, MAC and physical layer to support the semi-static configurations and dynamic indications and feedbacks.

Proposal 2:

Beamforming should be also supported for FR1 NCR.

Proposal 3：

The beam index based schemes could be reused for FR1 NCR for the AC beam indication.

Proposal 4:

The legacy beamforming mechanism could be used for the BH link and C-link for FR1 NCR.

Proposal 5:

Two options could be considered for the determination of the maximal number of forwarding resources in one list.

- Option 1: the multiple of maximum beam (or SSBs) number, such as the multiple of 64.

- Option 2: the multiple of the slot number of the maximum periodicity.

Proposal 6:

The multiple of maximum beam (or SSBs) number is slightly preferred. 256 can be the starting point for the value of Xmax .

Proposal 7:

Multiple periodic beam indications should be supported for RRC configuration.

Proposal 8:

The value of Tmax should equal to one for aperiodic beam indication in DCI, which means that only one time domain resource would be indicated by the aperiodic beam indication.

Proposal 9:

The value of Lmax,equals to one.

Proposal 10:

The subcarrier spacing of the time resource indicated in DCI should be configured by RRC. In addition, the subcarrier spacing should be aligned to the forwarding resources configured for the periodic beam indication.

Proposal 11:

The Option-1 for semi-persistent beam indication can be updated as below to be more clear.

- Option-1-updated:

  - A list set of Y (1≤Y≤Ymax) beam indication lists is configured by one RRC signaling(s)

    - Each beam indication list in the list set corresponds to a list of Z(1≤Z≤Zmax) forwarding resource, each is defined as {Beam index, time resource}

    - FFS: The value of Zmax, which refers to the maximum beams indicated in one beam indication list.

  - One MAC CE is used to select one of Y beam indication list from the list set of Y beam indication lists.

Proposal 12:

The updated Option-1 is preferred for the semi-persistent beam indication.

Proposal 13:

The multiple periodic indications through RRC signaling should not conflict between each other.

Proposal 14:

For the priority rule of the beam indication among periodic, semi-persistent and aperiodic indication, option 2 is slightly preferred.

- Option-2: If there is conflict among beam indication from different type of indication, the order of priority is defined as: Aperiodic beam indication > semi-persistent beam indication >periodic beam indication

Proposal 15:

An RRC based backhaul link beam indication should be defined.

Proposal 16:

The self-interference issue should be considered for the beam determination and indication of BH link.

Proposal 17:

Multiple BH link beams should be supported for the self-interference reduction.

Proposal 18:

Both schemes have their own usage scenarios and should be considered in the following studies.

- Scheme 1: The NCR only forwarded the qualified SSBs without the change of SSB beams at gNB.

- Scheme 2: A set of SSBs are allocated for NCR using a same directional backhaul link.

Proposal 19:

The NCR-MT could acquire the TDD configuration from the gNB as a legacy UE. The legacy TDD configurations could be reused for the NCR.

Proposal 20:

If the RRC, MAC CE and DCI based beam indication are supported, then it should also applicable to the On-state indication.

Proposal 21:

The explicit configuration or indication of OFF state should be deprioritized or not supported.

Proposal 22:

The time domain resources carrying the broadcast and cell-specific signals could be indicated through the RRC configurations of ON-OFF state by gNB.

Proposal 23:

Both symbol-level and slot-level on-off configurations and indications should be supported.

Proposal 24:

Both semi-static configuration and dynamic indication of On-state should be supported for NCR forwarding.

Proposal 25:

In case of simultaneous UL transmission of C-link and backhaul link, the NCR-Fwd could adopt a same PSD as NCR-MT for uplink transmission.

References

[1] Chair’s notes RAN1_111, 3GPP TSG RAN WG1 #111, Toulouse, France, November 14th – 18th, 2022

[2] R1-2212785, Summary#3 of discussion on side control information, Moderator (ZTE), 3GPP TSG RAN WG1 #111, Toulouse, France, November 14th – 18th, 2022

[3] Draft Meeting Report_RAN98e, 3GPP TSG RAN Meeting #98-e, Electronic Meeting, December 12-16, 2022