3GPP TSG RAN WG1 #112 R1-2301088 Athens, Greece, February 27th – March 3rd, 2023 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; rev; -; Current version:; 17.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on HARQ-ACK feedback for multicast SPS activation/deactivation. Source to WG:; Lenovo. Source to TSG:; R1. Work item code:; NR_MBS-Core; Date:; 2023-02-15. Category:; F; Release:; Rel-17. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; UE behaviour on HARQ-ACK feedback operation is unclear when UE receives multicast DCI format for activating multicast SPS configuration. According to RAN1#107e agreement, only ACK/NACK based feedback is supported for multicast SPS activation/deactivation. It implies that the HARQ-ACK feedback option for multicast SPS activation/deactivation is not configurable. Agreement For multicast SPS activation/deactivation, only ACK/NACK based feedback is supported. However, the current spec, “The second HARQ-ACK reporting mode is not applicable for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception.”, does not clearly cover the intention that ACK/NACK feedback is reported for SPS activation irrespective of the first HARQ-ACK reporting mode or the second HARQ-ACK reporting mode configured for the G-CS-RNTI. In addition, the amount of required PUCCH resources for DCI format for SPS activation is same to that for the first SPS PDSCH after activation because both need UE-specific PUCCH resource for gNB to identify which UE missed the DCI format for SPS activation. Furthermore, the current spec, “The second HARQ-ACK reporting mode is not applicable for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception.” leads to different HARQ-ACK reporting modes for the first SPS PDSCH after activation and the subsequent SPS PDSCHs for same multicast SPS configuration. RAN1 has no agreement that ACK/NACK feedback for SPS activation is referenced to as ACK/NACK feedback for the first SPS PDSCH after activation. It is necessary to cover the intention to make spec clear.. -. Summary of change:; Add descriptions that “The UE provides HARQ-ACK information for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration.” and delete the descriptions of “The second HARQ-ACK reporting mode is not applicable for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception.”. Consequences if not approved:; HARQ-ACK feedback for SPS activation is not aligned with RAN1 agreements.. Clauses affected:; 18. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:; This is the first version for this CR.. # 18 Multicast Broadcast Services This clause is applicable only for PDCCH receptions, PDSCH receptions, and PUCCH transmissions for MBS on a serving cell. DCI formats with CRC scrambled by G-RNTI for multicast or G-CS-RNTI scheduling PDSCH receptions are referred to as multicast DCI formats and the PDSCH receptions are referred to as multicast PDSCH receptions. DCI formats with CRC scrambled by MCCH-RNTI or G-RNTI for broadcast scheduling PDSCH receptions are referred to as broadcast DCI formats and the PDSCH receptions are referred to as broadcast PDSCH receptions. HARQ-ACK information associated with multicast DCI formats or multicast PDSCH receptions is referred to as multicast HARQ-ACK information. <Unchanged part omitted> In clauses referring to a higher layer parameter value provided by a first or second PUCCH-Config, when applicable a corresponding higher layer parameter value for PUCCH transmissions associated with multicast PDCCH or PDSCH receptions is provided as described in [12, TS 38.331]. In clauses referring to a higher layer parameter value provided by SPS-PUCCH-AN or SPS-PUCCH-AN-List, when applicable a corresponding higher layer parameter value for PUCCH transmissions associated with multicast SPS PDSCH receptions is provided as described in [12, TS 38.331]. In clauses referring to a higher layer parameter value provided by pdsch-HARQ-ACK-Codebook or pdsch-HARQ-ACK-CodebookList, when applicable a corresponding higher layer parameter value for HARQ-ACK codebooks associated with multicast HARQ-ACK information is provided as described in [12, TS 38.331]. A UE monitors PDCCH for scheduling PDSCH receptions or for activation/release of SPS PDSCH receptions for a corresponding SPS PDSCH configuration as described in clause 10.1. The UE provides HARQ-ACK information with ACK value when the UE detects a DCI format indicating activation or release for a multicast SPS PDSCH configuration. A UE can be configured by harq-FeedbackOptionMulticast, for a G-RNTI for multicast or for a G-CS-RNTI, to provide HARQ-ACK information for a transport block reception associated with the G-RNTI for multicast or with the G-CS-RNTI, according to the first HARQ-ACK reporting mode or according to the second HARQ-ACK reporting mode. The UE determines a priority for a PUCCH transmission with multicast HARQ-ACK information according to any HARQ-ACK reporting mode as described in clause 9 for a PUCCH transmission with unicast HARQ-ACK information. For the first HARQ-ACK reporting mode, the UE generates HARQ-ACK information with ACK value when a UE correctly decodes a transport block; otherwise, the UE generates HARQ-ACK information with NACK value, as described in clauses 9 and 9.1 through 9.3. The UE determines a PUCCH or a PUSCH to provide the HARQ-ACK information as described in clause 9.2. For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. <Unchanged part omitted>