3GPP TSG RAN WG1#112		R1-2301100

Athens, February 27th – March 3rd, 2023

Source:	Panasonic

Title: 	Discussion on XR capacity enhancement techniques

Agenda Item:		9.10.1

Document for:	Discussion

# Introduction

The study of XR in NR started in Rel. 17 and continued in Rel. 18, followed by a new work item. The objectives of the work item on XR were defined [1]. The enhancements will be considered to achieve improved network capacity considering the characteristics of the XR traffics. In this contribution, we present several capacity enhancement techniques to support XR in NR more efficiently.

# Characteristics of XR traffics

The traffics for XR applications were studied extensively during the study item phase in Rel. 17 [2]. XR applications impose high data rates both in downlink and uplink. The high data rates are originated from video and pose/control streams. The video streams are considered as quasi-periodic traffics. The video streams typically generate video frames at the rate of 60/90/120 fps. Each frame could arrive over a jitter window. The jitter could span over a [-4,4] ms window or even a larger window. In addition, the frame sizes could be large and vary. In order to carry the video streams in the network, each frame should be divided into smaller IP packets. All the IP packets should be received successfully to reconstruct the frame out of them. For typical XR applications, such as AR/VR, the packet delay budget (PDB) could be 10 ms, while typical cloud gaming applications require PDB of 15 ms. The pose/control streams generate less amount of traffic (e.g., 0.2 Mbps) but with more frequent packets, e.g., 250 fps and the PDB of 10 ms. Moreover, the XR applications would require supporting multi-stream traffics in uplink (UL) and downlink (DL).

# Discussions

The dynamic scheduling could be utilized to deliver XR traffics in DL. The gNB can assign dynamic resources for the XR upon arrival of the traffic. However, the dynamic scheduling for XR UL traffic may increase the latency, as the UE needs to send a scheduling request (SR) and buffer status report (BSR) to be scheduled. Instead, the configured grant (CG) can be utilized to reduce the latency and signaling overhead by reserving periodic radio resources for UL transmissions. However, using the conventional CG for XR video traffics can reduce the system capacity significantly. This is due to the fact that a large number of PUSCH occasions should be reserved to compensate the jitter and varying size of frames, while only a small portion of them would be used by the UE. In this regard, the following enhancements were considered [1]:

- Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE (RAN1);

The rest of this section describes how these enhancements could be envisioned.

## Support of multiple CG PUSCH occasions

To handle the XR traffic with a jitter, a CG configuration can contain several PUSCH occasions. There are two approaches. First approach is multiple CG PUSCH occasions having different time and frequency resource allocations. The other approach is multiple CG PUSCH occasions having same frequency resource allocation but only with different time allocations. As CG PUSCH configurations itself can be already multiple since Release 15, we think the difference of the frequency allocations can be realized by multiple CG PUSCH configurations itself. The handling of the jitter itself does not require to have different frequency allocations for multiple CG PUCH occasions. Therefore, we propose following.

Proposal 1: Multiple CG PUSCH occasions configured by a CG configuration have the same frequency resource allocation.

For CG Type 1, multiple CG PUSCH occasions by a CG configuration could be realized by just having multiple time resources in a rrc-ConfiguredUplinkGrant. Therefore, we don't see the specific need of RAN1 discussion.

For CG Type 2, we see following design options.

Design 1: The time offset(s) compared with the first time resource indicated by the activation DCI is RRC configured.

Design 2: An activation DCI contains multiple Time domain resource assignment fields.

Design 3: An activation DCI contains single Time domain resource assignment fields. An entry in the RRC parameter pusch-TimeDomainAllocationList has multiple time resource allocations, i.e., multiple SLIVs.

The design 3 can be quite similar to an activation DCI in CG PUSCH realized by the DCI for "multi-PDSCH/PUSCH with a single DCI" specified for NR for 52.6GHz to 71 GHz. Such design has flexibility on multiple offsets on the choice of SLIVs and offsets without increasing the DCI size. We propose to take such design as the starting point.

Proposal 2: The baseline design can be an activation DCI contains single Time domain resource assignment fields and an entry in the RRC parameter pusch-TimeDomainAllocationList has multiple time resource allocations.

## Dynamic indication of unused CG PUSCH occasions

In order to improve the system capacity, the UE can send an indication of unused CG resources. Then, the gNB can reschedule the unused resources for other traffics. The indication could be carried as a part of a CG-UCI or separately by a dedicated part of CG PUSCH. To use the UCI indication efficiently, the indication size and mapping could be adjusted according to the CG PUSCH occasion. As an example, we can consider a CG configuration for supporting a video stream that generates video frames with the rate of 60 fps. The CG configuration contains five PUSCH occasions, forming a resource bundle. There is a 2-ms time offset between two consecutive PUSCH occasions within a resource bundle. The indication is used to notify the gNB regarding the unused PUSCH occasions within a resource bundle. Accordingly, the unused indication size is adjusted. The size of UCI indication could be from 1 to 4, depending on the PUSCH occasion. For a given indication, the first bit (left bit) is an indication for not using the next PUSCH occasion. While the last bit (right bit) is an indication for not using the last PUSCH occasion within a resource bundle. Figure 1 illustrates the unused CG indication with variable UCI sizes. If the variable UCI indication size is used, it enables reducing the UCI overhead and obtaining the simplicity of the design. These trade-off needs some further discussion. The coding of UCI can be based on the current UCI encoding scheme.

Proposal 3: The unused CG indication size should be adjusted according to the PUSCH transmission occasion.

Figure 1. illustration of unused CG PUSCH indication with adjustable size.

# Conclusion

In this contribution, we provide our view on techniques for supporting XR applications efficiently. We made following proposals.

Proposal 1: Multiple CG PUSCH occasions configured by a CG configuration have the same frequency resource allocation.

Proposal 2: The baseline design can be an activation DCI contains single Time domain resource assignment fields and an entry in the RRC parameter pusch-TimeDomainAllocationList has multiple time resource allocations.

Proposal 3: The unused CG indication size should be adjusted according to the PUSCH transmission occasion.

# Reference

[1] 	RP-223502, “New WID on XR Enhancements for NR,” RAN#98-e.

[2] 	TR 38.838, “Study on XR (Extended Reality) Evaluations for NR (Release 17),” V17.0.0.