3GPP TSG RAN WG1 #112 		                                   		   	R1-2301101

Athens, Greece, Feb. 27th – March 3rd, 2023

Agenda Item:	9.2.4.1

Source:	InterDigital, Inc.

Title:	Evaluation on AI/ML for positioning accuracy enhancement

Document for:	Discussion and Decision

# Introduction

In RAN#111, the following agreements were made [1].

Agreement Study how AI/ML positioning accuracy is affected by: user density/size of the training dataset. Note: details of user density/size of training dataset to be reported in the evaluation. Agreement For reporting the model input dimension NTRP * Nport * Nt of CIR and PDP, Nt refers to the first Nt consecutive time domain samples. - If N’t (N’t < Nt) samples with the strongest power are selected as model input, with remaining (Nt ‒ N’t) time domain samples set to zero, then companies report value N’t in addition to Nt. It is also assumed that timing info for the N’t samples need to be provided as model input. Agreement For reporting the model input dimension NTRP * Nport * Nt: - If the model input is CIR, then each input value of CIR is a complex number, i.e. it contains two real values, either {real, imaginary} or {magnitude, phase}. - If the model input is PDP, then each input value of PDP is a real value. Agreement At least for model inference of AI/ML assisted positioning, evaluate and report the AI/ML model output, including (a) the type of information (e.g., ToA, RSTD, AoD, AoA, LOS/NLOS indicator) to use as model output, (b) soft information vs hard information, (c) whether the model output can reuse existing measurement report (e.g., NRPPa, LPP). Agreement For AI/ML assisted positioning, evaluate the three constructions: - Single-TRP, same model for N TRPs - Single-TRP, N models for N TRPs - Multi-TRP (i.e., one model for N TRPs) Note: Individual company may evaluate one or more of the three constructions. Agreement For AI/ML assisted approach, study the performance of model monitoring metrics at least where the metrics are obtained from inference accuracy of model output. Agreement For both direct and AI/ML assisted positioning methods, investigate at least the impact of the amount of fine-tuning data on the positioning accuracy of the fine-tuned model. - The fine-tuning data is the training dataset from the target deployment scenario. Agreement For the RAN1#110bis agreement on the calculation of model complexity, the FFS are resolved with the following update: Model complexity to support N TRPs. Single-TRP, same model for N TRPs; PS where PS is the model complexity for one TRP and the same model is used for N TRPs.. Single-TRP, N models for N TRPs; i=1,…NPS,i Where PS,i is the model complexity for the i-th AI/ML model.. Note: The reported model complexity above is intended for inference and may not be directly applicable to complexity of other LCM aspects. Observation Direct AI/ML positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods when the generalization aspects are not considered. - For InF-DH with clutter parameter setting {60%, 6m, 2m}, evaluation results submitted to RAN1#111 indicate that the direct AI/ML positioning can achieve horizontal positioning accuracy of <1m at CDF=90%, as compared to >15m for conventional positioning method. Agreement For AI/ML based positioning, company optionally evaluate the impact of at least the following issues related to measurements on the positioning accuracy of the AI/ML model. The simulation assumptions reflecting these issues are up to companies. - SNR mismatch (i.e., SNR when training data are collected is different from SNR when model inference is performed). - Time varying changes (e.g., mobility of clutter objects in the environment) - Channel estimation error Conclusion Companies describe how their computational complexity values are obtained. - It is out of 3GPP scope to consider computational complexity values that have platform-dependency and/or use implementation (hardware and software) optimization solutions. Observation AI/ML assisted positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods when the generalization aspects are not considered. - For InF-DH with clutter parameter setting {40%, 2m, 2m}, evaluation results submitted to RAN1#111 indicate that the AI/ML assisted positioning can achieve horizontal positioning accuracy of <0.4m at CDF=90%, as compared to >9m for conventional positioning method. - For InF-DH with clutter parameter setting {60%, 6m, 2m}, evaluation results submitted to RAN1#111 indicate that the AI/ML assisted positioning can achieve horizontal positioning accuracy of <1m at CDF=90%, as compared to >15m for conventional positioning method. Note: how to capture the observation(s) into TR is separate discussion. Agreement For AI/ML assisted approach, for a given AI/ML model design (e.g., input, output, single-TRP vs multi-TRP), identify the generalization aspects where model fine-tuning/mixed training dataset/model switching is necessary.

# Evaluation results

In this contribution, we evaluate following parameters on AIML positioning accuracy:

- Size of training dataset

- Training using unobserved RSTD measurements

2.1	Evaluation for size of training dataset

In this subsection, we analyze the impact of size of training dataset and/or user density on direct AIML positioning accuracy. To analyze the impact of training dataset, we generated dataset with 3 different sizes[16000 samples(100%), 8000 samples(50%) and 4000 samples(25%)] and trained a unique AIML model for each sample size of training data. While we changed the size of the training dataset, all other parameters, such as test data size and system level parameters, are kept same among each experiment. To analyze the impact training dataset on different model inputs, we evaluated direct AIML positioning based on RSRP measurements and RSRP+RSTD measurements. We generated this dataset by carrying out System Level Simulations for IIoT scenario. Details of the IIoT scenario parameters are listed in Table A1. Furthermore, all the details related to model input/output and model structure are described in Table A2. Summary of the evaluation assumptions are as follows:

- Inputs:

  - RSRP measurements

  - RSRP + RSTD measurements

- Output: Location information

- Motivation: To evaluate the impact of size of the training dataset

- Model: ResNet

- Model deployed on : UE side

- Parameters swept: size of training dataset

Table 1 Summary of evaluation results of size of training dataset for direct AIML positioning

| Model Input | Model output | (Percentage of training data set without) Label | Settings (drops, clutter param, network synchronization error) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| RSRP | UE position | 0% (default) | {60%, 6m, 2m} , drop A, T1= 0 ns | {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns | 16000 | 4000 | 332 k | 11.37 M FLOPs | 3.35 |
| RSRP | UE position | 0% (default) | {60%, 6m, 2m} , drop A, T1= 0 ns | {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns | 8000 | 4000 | 332 k | 11.37 M FLOPs | 4.25 |
| RSRP | UE position | 0% (default) | {60%, 6m, 2m} , drop A, T1= 0 ns | {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns | 4000 | 4000 | 332 k | 11.37 M FLOPs | 5.00 |
| RSRP + RSTD | UE position | 0% (default) | {60%, 6m, 2m} , drop A, T1= 0 ns | {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns | 16000 | 4000 | 334 k | 11.41 M FLOPs | 1.69 |
| RSRP+RSTD | UE position | 0% (default) | {60%, 6m, 2m} , drop A, T1= 0 ns | {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns | 8000 | 4000 | 332 k | 11.41 M FLOPs | 1.98 |
| RSRP+RSTD | UE position | 0% (default) | {60%, 6m, 2m} , drop A, T1= 0 ns | {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns | 4000 | 4000 | 332 k | 11.41 M FLOPs | 2.46 |

Based on the results presented in Table 1, it is evident that positioning accuracy degrades as we reduce the size of the training dataset. It is also noticeable that the amount of degradation for AIML models trained with RSRP+RSTD measurements is less that AIML models trained with RSRP measurements only, indicating robustness of the AIML models trained with both power and timing measurements.

Observation 1: For direct AIML positioning, from the evaluation results presented with different size of training dataset, 90% horizontal positioning accuracy degrades as we reduce the size of the training dataset.

Observation 2: For direct AIML positioning, when the model input is RSRP measurements, reduction in training dataset size to 50% of the reference size(16000 training samples vs 8000 training samples) results in ~0.9 m horizontal accuracy(90%) loss.

Observation 3: For direct AIML positioning, when the model input is RSRP measurements, reduction in training dataset size to 25% of the reference size (16000 training samples vs 4000 training samples) results in ~1.65 m horizontal accuracy(90%) degradation.

Observation 4: For direct AIML positioning, when the model input is RSRP and RSTD measurements, reduction in training dataset size to 50% of the reference size (16000 training samples vs 8000 training samples) results in ~0.29 m  horizontal accuracy(90%) reduction.

Observation 5: For direct AIML positioning, when the model input is RSRP measurements, reduction in training dataset size to 50% of the reference size (16000 training samples vs 4000 training samples) results in ~0.77 m  horizontal(90%) accuracy reduction.

Observation 6: For direct AIML positioning, degradation in positioning accuracy is higher with RSRP measurement as model input compared to RSRP+RSTD measurement as model input.

2.2 Evaluation for training with unobserved RSTD measurements

In this subsection, the performance results of the AIML assisted positioning is evaluated. In NLOS heavy environment, there is a high likelihood that RSTD measurements obtained by the UE are inaccurate. To overcome this limitation of inaccurate RSTD measurements in NLOS environment, we are proposing to estimate unobserved/ideal RSTD measurements using an AIML model.

During training phase, the UE can obtain its location using GPS/GNSS. Based on its location information, the UE can derive its ideal/unobservable RSTD measurements. During training phase, the UE uses measured RSRP fingerprint as an input and unobserved/ideal RSTD measurements (calculated based on known position information of the UE and position information of the TRP) as the target output of the AIML model. During testing phase, the UE inputs RSRP fingerprint as an input to the AIML model and estimates ideal/unobserved RSTD measurements using the trained AIML model.  By providing predicted unobserved RSTD measurements as an input to the DL-TDOA positioning method, the UE obtains its location.

DL-TDOA

AIML model

RSRP fingerprint

Unobserved
    RSTD

UE  location

Figure 1 AIML assisted positioning with unobserved RSTD measurements

In Table 2, horizontal accuracy results for AIML assisted positioning are presented. To study the impact of training an AIML model with unobserved RSTD compared to estimate RSTD, simulation results from [2] with RSTD prediction are presented for comparison. It should be noted that the AIML model presented in [2] is trained based on measured RSTD as the target metric. The results presented in Table 2 shows that AIML model trained using unobserved/ideal RSTD yield better positioning accuracy that AIML positioning model trained using measured RSTD samples.

Table 2 Evaluation results for AIML assisted positioning by unobserved RSTD prediction

| Model Input | Model output | (Percentage of training data set without) Label | Settings (drops, clutter param, network synchronization error) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| RSRP | (estimated) RSTD | 0% (default) | {60%, 6m, 2m} , drop A, T1= 0 ns | {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns | 16000 | 4000 | 332 k | 11.42 M FLOPs | 15.68 |
| RSRP | (unobserved) RSTD | 0% (default) | {60%, 6m, 2m} , drop A, T1= 0 ns | {60%, 6m, 2m} , drop A’ (unseen), T1= 0 ns | 16000 | 4000 | 332 k | 11.42 M FLOPs | 10.60 |

Observation 7: Positioning based on unobserved RSTD measurements prediction yields ~ 5.08 m better for 90% horizontal positioning accuracy than RSTD estimation.

In addition, we applied DL-TDOA positioning directly on measured RSTD values and achieved 11.60 m  horizontal accuracy for 90%ile UEs.

Observation 8: Positioning based on unobserved RSTD measurements prediction yields ~ 1.0 m better for 90% horizontal positioning accuracy than DL-TDOA positioning method.

# Conclusion.

In this contribution, the following proposals and observations are made.

Observation 1: For direct AIML positioning, from the evaluation results presented with different size of training dataset, 90% horizontal positioning accuracy degrades as we reduce the size of the training dataset.

Observation 2: For direct AIML positioning, when the model input is RSRP measurements, reduction in training dataset size to 50% of the reference size (16000 training samples vs 8000 training samples) results in ~0.9 m horizontal accuracy (90%) loss.

Observation 3: For direct AIML positioning, when the model input is RSRP measurements, reduction in training dataset size to 25% of the reference size (16000 training samples vs 4000 training samples) results in ~1.65 m horizontal accuracy (90%) degradation.

Observation 4: For direct AIML positioning, when the model input is RSRP and RSTD measurements, reduction in training dataset size to 50% of the reference size (16000 training samples vs 8000 training samples) results in ~0.29m  horizontal accuracy(90%) reduction.

Observation 5: For direct AIML positioning, when the model input is RSRP measurements, reduction in training dataset size to 50% of the reference size (16000 training samples vs 4000 training samples) results in ~0.77m  horizontal (90%) accuracy reduction.

Observation 6: For direct AIML positioning, degradation in positioning accuracy is higher with RSRP measurement as model input compared to RSRP+RSTD measurement as model input.

Observation 7: Positioning based on unobserved RSTD measurements prediction yields ~ 5.08 m better for 90% horizontal positioning accuracy than RSTD estimation.

Observation 8: Positioning based on unobserved RSTD measurements prediction yields ~ 1.0 m better for 90% horizontal positioning accuracy than DL-TDOA positioning method.

# Reference

[1] RAN1 Chairman’s note, RAN1#111, November, 2022.

[2] R1-2207094,  Evaluation on AI/ML for positioning accuracy enhancement, InterDigital, Inc., RAN WG1 #110-e, Toulouse, France.

# Appendix

Table A1: IIoT scenario system parameters

| Parameter | Values |
| --- | --- |
| Carrier frequency, GHz | 3.5GHz |
| Bandwidth, MHz | 100MHz |
| Subcarrier spacing, kHz | 30kHz |
| Channel model | InF-DH |
| Hall size | 120(L) x 60(W) m, D – 20 m |
| BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. |
| Room height | 10 m |
| Number of floors | 1 |
| Clutter parameters: {density , height ,size } | InF-DH - {60%, 6m, 2m} |
| UE model parameters |  |
| UE noise figure, dB | 9dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) |
| UE antenna radiation pattern | Omni, 0dBi |
| Network synchronization | Fully synchronized |
| UE/gNB RX and TX timing error | T1= 0 ns |
| UE horizontal drop procedure | Uniformly distributed over entire factory floor |
| UE antenna height | 1.5 m |
| gNB model parameters |  |
| Total gNB TX power, dBm | 24 dBm |
| gNB noise figure, dB | 5dB |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 |
| gNB antenna height | 8 m |

Table A2: Model configuration for direct AI/ML positioning

| Parameter | Values |
| --- | --- |
| Training input measurements | 1. RSRP: Per beam RSRP from multiple TRPs (108 RSRP values, 6 beams per TRP) 2. RSRP + RSTD: Per beam RSRP from multiple TRPs (108 RSRP values, 6 beams per TRP) and per TRP RSTD value (18 RSTD values) |
| Output | UE position |
| Number of TRPs | 18 |
| BS locations | As specified in Table A1 |
| ML model | ResNet (1 Convolutional layer, ‘k’ residual layers, 1 fully connected layer) |