3GPP TSG RAN WG1 #112			R1-2301150

Athens, Greece, February 27th – March 3rd 2023

Source:	Mitsubishi Electric

Title:	On sidelink co-channel coexistence issues

Agenda Item:	9.4.2 - Co-channel coexistence for LTE sidelink and NR sidelink

Document for: 	Discussion/Decision

# Introduction

One of the objectives of the WID about NR sidelink evolution [1] is to:

Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4]

- Reuse the in-device coexistence framework defined in Rel-16 as much as possible

In RAN1#110 [6], the group concluded that TDM-based semi-static resource pool partitioning based on Rel-16/17 specifications is one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs. Yet, this solution, although feasible, is challenging to deploy and unable to adapt to future traffic changes [8], which also leads to important drawbacks in terms of performance. For these reasons, RAN plenary RAN#97 [7] guides RAN 1 to continue the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A.

In RAN1#110b-e and RAN1#111 meetings there was no significant progress on issues such as how to deal with PSFCH overlapping or higher SCS support, although some proposals exist and were thoroughly debated [9], [10].

This contribution discusses the mechanisms for co-channel coexistence between LTE and NR sidelink.

# Background and motivation

Co-channel coexistence studies responds to a clear market need. In its input to the Rel.18 workshop, 5GAA rated the co-channel coexistence studies as a high priority [2]. Since the available ITS frequency spectrum is small, there is a clear need for NR to coexist with LTE. First of all, this enables more efficient spectrum usage and enables NR to access larger spectrum. Second of all, migration from LTE to NR sidelink needs to be considered. When the share of LTE-V2X will decrease, those devices being gradually replaced with NR-V2X UEs, co-channel coexistence opens the way to smooth transition between the two technologies and maximizes deployment flexibility.

Due to the long lifetime of LTE-V2X UEs, the solutions to be designed should be seamless and backwards compatible for LTE users, who wouldn’t need to have knowledge of the coexistence with NR. Changes to LTE-V2X should be avoided. Dynamic resource pool sharing is the only viable solution of ensuring smooth technological transition.

# Discussion

To enable dynamic resource pool sharing, several technical solutions need to be designed to overcome issues described in the following subsections.

## On PSFCH issues

Concerning the AGC issues caused by NR PSFCH transmissions occurring in resources that overlap with resources to be used for LTE SL transmissions, the following proposal [10] was discussed in RAN1#111 without any agreement:

Proposal 1-1 (VII): - For dynamic resource pool sharing, in NR SL resource pools with PSFCH configured and when HARQ-ACK is enabled, based on (pre-)configuration, when PSFCH resources overlap with resources to be used for LTE SL transmissions in the time domain, the NR SL UE - Always avoids transmissions on the PSFCH resources (Alt 1), or - FFS details including whether the TX UE avoid selecting resources for PSCCH/PSSCH transmissions with the overlapping PSFCH resources and/or RX UE does not transmit on the overlapping PSFCH resources. - Does not avoid transmission on the PSFCH resources (Alt 2), or - Conditionally avoids transmissions on a subset of the PSFCH resources. - FFS details of conditions including - a (pre-)configured subset, - the consideration of the LTE RSRP and LTE and/or NR priority, - presence of PSCCH/PSSCH transmission in the same time slot LTE subframe as PSFCH transmission with the same power by the same UE. - FFS for the case when there is an overlapping of time and frequency resources between PSFCH and LTE SL transmission - Introduce additional PSFCH periodicity of [5, 8 and] 10. - Note: Alignment between PSFCH periodicity and LTE logical subframes should be ensured by proper configuration. - FFS: Whether to confine the PSFCH transmission, in the time domain, within the guard symbol of the LTE SL subframe. - FFS details including the conditions.

This proposal was an attempt to reach a compromise solution by mixing all the alternatives on the table. The different candidate solutions seem to have some overlap, and it is unclear whether they are all needed/ whether they all procure some specific gain or advantage. We are in favor of pre-configurability only among complementary alternatives that bring specific gains (e.g. for different use cases etc), but we do not wish to go towards pre-configurability among equivalent designs.

In order to ease the understanding of the relative performance and achievable tradeoffs among different possibilities, we evaluated the following classes of solutions (color code corresponds to the curves on the evaluation results):

- Alt 1: PSFCH collision mitigation by scheduling/ dropping

  - the probability of collision is reduced by preventive PSSCH resource selection at the transmitter and/or PSFCH dropping at the receiver, possibly based on some RSRP conditions

- Alt 2: PSFCH collision mitigation by introducing higher PSFCH periodicity

  - the probability of collision is reduced by concentrating PSFCH transmission in fewer less frequent PSFCH occasions

- Alt 3: Collision-free PSFCH transmission by confining the PSFCH transmission within the guard symbol of LTE transmission

  - Collision is completely suppressed by reusing 60kHz-like PSFCH transmission regardless of the NR PSSCH numerology. PSFCH collision with resources used by LTE transmission is completely avoided by design.

- Alt 4: PSFCH collision avoidance by partial resource overlap (unfeasible in practice, for comparison purposes only)

  - the slots containing PSFCH occasions are pulled out of the LTE resource pool (LTE resource pool is smaller than and included in the NR resource pool)

### Considerations on different possible approaches

#### Alt.1: PSFCH collision mitigation by PSFCH dropping (Alt.1-1) and/or PSSCH resource selection (Alt.1-2)

Concerning the wording of different flavors of Alt1, it is worthy to note that the following two notions are distinct and non-interchangeable:

- time domain overlap between time slots containing PSFCH and LTE subframes

- PSFCH resources overlapping with LTE transmission in the time domain

As an example, let us refer to the existing NR transmissions with 60kHz SCS, which by nature does not cause any PSFCH issue when configured with PSFCH every 4 slots. In the case of that configuration, although the slot with PSFCH at 60kHz overlaps in the time domain with the LTE subframe, there is no collision between PSFCH resource and LTE transmission, as it can be seen from Figure 1. In the following, when we discuss PSFCH collision issues, we refer to PSFCH resources overlapping with LTE transmission in the time domain. This includes effective collision (overlapping frequency resources) and AGC issues (transmission in the same time in different subchannels).

Observation 1: For NR transmission with SCS 60kHz and PSFCH enabled, although the slot with PSFCH at 60kHz overlaps in the time domain with the LTE subframe, there is no collision between PSFCH resource and LTE transmission.

Figure 1 Time alignment of LTE subframe, NR @60kHz slot and NR@15kHz slot

A first proposed way of ensuring that the PSFCH resources do not overlap with LTE SL transmissions is (Alt1-1) PSFCH dropping at the receiver side, with the following possibilities

  - In case of detecting LTE presence in resources overlapping with the PSFCH candidate, the RX UE always drops PSFCH. This systematically penalises NR performance if no further condition is applied.

  - In case of detecting LTE presence in resources overlapping with the PSFCH candidate, the RX UE drops PSFCH under certain conditions. This may bring some tradeoff between NR and LTE performance

A second proposed way of ensuring that the PSFCH resources do not overlap with LTE SL transmissions is (Alt1-2) preventive resource selection at the transmitter side

  - The PSCCH/PSSCH TX UE always avoids selecting resources for PSCCH/PSSCH transmissions with corresponding PSFCH resources that overlap with LTE SL transmissions in the time domain. This systematically penalises NR performance if no further condition is applied.

  - The PSCCH/PSSCH TX UE avoids selecting resources for PSCCH/PSSCH transmissions with corresponding PSFCH resources that overlap with LTE SL transmissions in the time domain under certain conditions. This may bring some tradeoff between NR and LTE performance.

Again, if applied in an absolute manner with pure and simple exclusion, this systematically penalises NR performance only and may lead to excessive exclusion and NR resource starvation and some further conditions (e.g. exclude only under some RSRP/RSSI and/or priority conditions) may improve the situation.

Observation 2: Alt.1-1 and Alt1-2 need to apply resource exclusion under limiting conditions (e.g. exclude only under some RSRP/RSSI and/or priority conditions) in order to avoid excessive dropping/resource exclusion and NR resource starving.

On RSRP-based conditions for Alt.1

The tradeoffs that can be achieved when applying Alt.1 under conditions largely depend on the condition. In subsection 3.1.2 we investigated RSRP-based conditions.

We define a new RSRP threshold RSRPThPSFCH, associated with PSFCH occasions, which is ΔdB away from the RSRP threshold used in the PSCCH/PSSCH resource allocation procedure for candidate exclusion, RSRPTh (there is one such value per subchannel, and values may depend on e.g. priority)

RSRPThPSFCH= RSRPTh+∆(dB)

The PSFCH collision mitigation decision at the transmitter/receiver is based on this value. We proceed in the following manner. For each subchannel i in the set S of subchannels with LTE detection (SCI correctly decoded) in the PSFCH occasion, we compute the RSRP associated to the subchannel, RSRPiPSFCH, and then we determine the index of the subchannel with the most significant value:

i =argmaxi∈SRSRPiPSFCH-RSRPTh, iPSFCH

- For Alt.1-1, if RSRPiPSFCH≥RSRPTh, iPSFCH PSFCH is dropped

- For Alt.1-2, the resource allocation procedure for PSSCH/PSFCH considers, for candidate resource exclusion, both the RSRP measured on PSSCH resources and the one measured on PSFCH resources, that is:

  - If RSRPiPSFCH-RSRPTh, iPSFCH≤RSRP-RSRPTh, resource allocation is based on RSRP and RSRPTh (Rel.16 procedure)

  - If RSRPiPSFCH-RSRPTh, iPSFCH>RSRP-RSRPTh, on RSRP and RSRPTh in Rel.16 resource allocation procedure are replaced by RSRPiPSFCH and RSRPTh, iPSFCH (the RSRP situation on PSFCH resources takes priority in the exclusion process).

In both flavors of Alt.1, the value ∆(dB) controls how much LTE presence in a PSFCH occasion weights in the resource allocation process:

- If ∆→∞, LTE presence is ignored (no avoidance, NR UE never avoids collision)

- If ∆→-∞, LTE presence takes precedence (NR UE always avoids collision)

#### Alt.2: PSFCH collision mitigation by introducing higher PSFCH periodicity

Concerning Alt2, it is conceptually clear how using a periodically repeating set of PSFCH slots pushes LTE UEs out of the concerned slot and reduces, up to a certain point, the collision probability between PSFCH resources and LTE transmissions.

Yet, this approach has certain limitations, which will be further discussed based on simulation results:

- The probability of PSFCH collision with LTE is sometimes reduced, but collisions still exist

  - in some scenarios (e.g. for groupcast option 1) there is no PRR gain

- Increasing the PSFCH period N from 4 to e.g. 10 results in increased latency

- Increasing the PSFCH period N from 4 to e.g. 10 results in PSFCH performance penalty and/or reduced multiplexing capability

On latency and multiplexing capability

PRR performance of this method will be investigated in the performance sub-section. Nevertheless, it should be noted that PRR simulations are not representative of the side effects of such important reduction of the PSFCH occasions.

First of all, having sparse PSFCH occasions has an impact on the latency.

Second, having such sparse PSFCH occasions seriously reduces either the multiplexing capability or the performance of PSFCH. For a PSSCH transmission with LPSSCH subchannels, there are up to F=LPSSCH*Q*Mset= LPSSCH*Q*M/(N*L) different PSFCH associated resources, and this value was dimensioned for PSFCH occasions occurring at most every N=4 slots. Q stands for the number of configured cyclic shift pairs. Highly increasing N cannot be compensated without severe penalty on the multiplexing capability F. This penalty is especially damaging for groupcast option 2 (ACK/NACK-based) transmissions. Or, as it will be seen in the evaluation section, this is the main use case where Alt.2 has a technical interest. In the remainder of the paper, Ftot may also be used to represent the total number of PSFCH resources in a PSFCH occasion (system perspective).

A numerical example with M/L=25 and LPSSCH=1  leads to the numbers in the following table (consistent with the simulation campaign):

Table 1: PSFCH multiplexing capability F depending on the PSFCH periodicity N, for different configured number of cyclic shift pairs Q (M/L=25 and LPSSCH=1)

| F | N=1 | N=2 | N=4 | N=10 |
| --- | --- | --- | --- | --- |
| Q=1 | 25 | 12 | 6 | 2 |
| Q=2 | 50 | 25 | 12 | 5 |
| Q=3 | 75 | 37 | 18 | 7 |
| Q=6 | 150 | 75 | 37 | 15 |

Another numerical example with M/L=12 and LPSSCH=1 (which corresponds to baseline of cases presented in 3.1.1.3) leads to the numbers in the following table (consistent with the toy examples in Figure 3 - Figure 6 to ease the comparison):

Table 2: PSFCH multiplexing capability F depending on the PSFCH periodicity N, for different configured number of cyclic shift pairs Q (M/L=12 and LPSSCH=1)

| F | N=1 | N=2 | N=4 | N=10 |
| --- | --- | --- | --- | --- |
| Q=1 | 12 | 6 | 3 | 2 |
| Q=2 | 24 | 12 | 6 | 3 |
| Q=3 | 36 | 18 | 9 | 4 |
| Q=6 | 72 | 36 | 18 | 8 |

The multiplexing capability (and thus e.g. the group size in groupcast option 2) is severely reduced.

If some irregular PSFCH occasions were to be supported, as proposed in the previous meetings, that would impose to re-define in a non-straightforward manner the mapping between PSSCH slots/subchannels and the associated PSFCH resources. The specification impact was not assessed yet, but it doesn’t seem possible to make this kind of remapping with light or limited changes given the variable and large spacing between successive PSFCH occasions.

Observation 3: Alt.2 has several drawbacks including increased latency, penalized PSFCH multiplexing capability, and potentially significant specification impact to re-define the correspondence between PSSCH resources and PSFCH time/frequency/code sets with irregular spacing.

#### Alt 3: Collision-free PSFCH transmission by confining the PSFCH transmission within the guard symbol of LTE transmission

Previous approaches aim to reducing the probability of having PSFCH transmissions in resources that overlap with LTE SL transmissions in the time domain.

A more radical and effective solution is to suppress the PSFCH issue altogether by confining PSFCH transmission within the guard interval of LTE transmission. Such an approach guarantees that there is no PSFCH issue and that any PSSCH resource not used by LTE can be freely used by NR. The counterpart is to allow “60kHz-like” PSFCH transmission.

A first possible approach would be to use 60kHz subcarrier spacing for PSFCH transmission regardless of the subcarrier spacing used for PSSCH transmission. There is no extra measure to be taken either at the transmitter or at the receiver side. Guard bands are reduced, but remain within the UE capability, since the UE is already supposed to be able to switch or to gain AGC within the time duration of an OFDM symbol with 60kHz SCS. The specification impact is very limited, since 60kHz transmission is already specified.

Observation 4: The specification impact of Alt.3 (PSFCH confined in LTE Guard) is very limited, since 60kHz transmission is already specified.

a

b

c  d

e

f

g

h

Figure 2 – Examples of PSFCH transmission confined, in the time domain, in the guard symbol of LTE transmission; last line is existing Rel.16 60kHz SCS configuration with N=4

Implementation options:

PSFCH symbols can be produced as “natively” 60kHz, which is the most flexible option. This implies that inter-numerology switch needs to be performed during the guard intervals between PSSCH and PSFCH transmissions. The smallest required switching times are equal to the ones assumed by the Rel.16 60kHz specification, but in most cases they can be relaxed (very long guard intervals are usually available when switching to PSSCH to PSFCH transmission; when switching back to PSSCH, solutions like lines c or e in Figure 2 allow 30kHz-like switching times).

Alternatively, this can also be done based on the PSSCH SCS. In implementation, the same result can be obtained by using the same SCS as the PSSCH transmission and applying well known methods such as alternating between used and nulled subcarriers (e.g., an OFDM symbol with 30kHs SCS and where every other subcarrier is nulled is the same as the repetition of two OFDM symbols with 60kHz, among which the first can be used as AGC and the second as PSFCH; an OFDM symbol with 15kHs SCS and where every 4th subcarrier is used is the same as the 4 times repetition of an OFDM symbols with 60kHz, among which two can be discarded/used as guard, and the remaining two can serve as AGC and PSFCH respectively). Configurations like the ones in lines d, g of in Figure 2 can be easily produced this way.

Regardless of how the PSFCH transmission is produced in implementation and of which subcarrier spacing is used for PSSCH transmission, there is no PSFCH issue as long as the PSFCH transmission remains confined, in the time domain, within the guard interval of LTE transmission. The specification impact remains very limited, because 60kHz PSFCH transmission is already specified, and the PSFCH periodicities are equally among the ones already specified.

Observation 5: Alt.3 (PSFCH confined in LTE Guard) can be produced, in implementation, either as “natively 60kHz” or based on the PSSCH SCS, through signal processing methods (comb subcarrier nulling etc…)

Observation 6: For Alt.3 (PSFCH confined in LTE Guard) with PSFCH symbols natively produced in 60kHz SCS, guard bands can often be relaxed with respect to the already specified 60kHz durations.

On latency and multiplexing capability

Alt.3 (PSFCH confined in LTE Guard) does not suffer from increased latency with respect to already existing Rel.16 PSFCH configurations.

When operating PSFCH with a subcarrier spacing k-times higher than the PSSCH SCS, there is of course a reduction by a factor of k in the number of available PRBs in the considered resource pool wrt to the reference case of Rel.16 (e.g. Figure 3). There are obvious mitigation strategies allowing to retrieve the same PSFCH multiplexing capability, e.g.

- Through time domain multiplexing of several PSFCH symbols in the same PSFCH occasion (see Figure 4 and line f of Figure 2)

- Through code domain multiplexing, whenever Q is not already configured to 6 (see Figure 5 and lines e, g of Figure 2)

All of the above can be done with virtually no spec impact (implicit reinterpretation of Q values for example).

Whenever a different benefit may be drawn (e.g. in terms of guard band duration relaxation), other strategies allowing a limited reduction of the multiplexing capability by a maximum factor of 2 can be applied (e.g. reducing the value of Mset as in Figure 6, or limiting PSSCH allocations with HARQ enabled to start only in even (or odd) subchannels, etc…). Multiplexing capability is overall better than in Alt.2. In examples in Figure 3 - Figure 6 both the PSFCH total multiplexing capability Ftot (system perspective) and PSFCH multiplexing capability associated to a transmission (F, representative of the maximum group size when ACK/NACK feedback is enabled) are depicted. These values are to be compared with the ones in Table 2. It is clear that Alt.3 has clearly superior PSFCH multiplexing capability with respect to Alt.2.

| Figure 3: Rel.16-like PSSCH to PSFCH mapping (F=18) | Figure 4: Retrieving the multiplexing capability through time domain multiplexing (F=18) |
| --- | --- |
| Figure 5: Retrieving the multiplexing capability through code domain re-configuration (F=18) | Figure 6: Reduced Mset size/reduced multiplexing capability (can be compensated by doubling Q). (F=9) |

Observation 7: Alt.3 (PSFCH confined in LTE Guard) does not suffer from increased latency with respect to already existing Rel.16 PSFCH configurations.

Observation 8: PSFCH multiplexing capability of Alt.3 (PSFCH confined in LTE Guard) is overall better than in Alt.2 and can in most cases be the same as in Rel.16 corresponding configuration.

Proposal 1: To eliminate the PSFCH issue, confine the PSFCH transmission, in the time domain, within the guard symbol of the LTE transmission.

### Simulated comparative performance

#### PRR comparative performance of Alt.2 and Alt 3

We evaluate the performance of introducing a new PSFCH periodicity of N=10 slots by considering medium or high traffic density. We evaluate groupcast option 2 (ACK/NACK) and groupcast option 1 (NACK only) transmissions.

As benchmark, we consider PSFCH periodicity of N=4 slots, where the NR terminal takes into account the LTE sensing for PSSCH resource allocation only, and does not implement any measure to mitigate the PSFCH issue. As a pure example, we also plot the performance of solutions where PSFCH occasions would be somehow pulled out of the LTE resource pool through a semi-static configuration, which is not feasible in practice. This would guarantee collision-free PSFCH transmission, but the number of resources available for LTE transmission would be reduced (every N-th slot is excluded from the LTE resource pool).

With medium to high traffic, with groupcast option 2 transmission (ACK/NACK) increasing the PSFCH period to N=10 (and thus accepting the increased latency and decreased group size) procures some PRR gain with respect to the baseline configuration with N=4. Yet, Alt.3 (confine PSFCH withing the guard symbol of LTE) performs significantly better without such impact on PSFCH performance.

| Figure 7: LTE PRR/groupcast option 2 | Figure 8: NR PRR/groupcast option 2 |
| --- | --- |
| Figure 9: LTE PRR/groupcast option 2 | Figure 10: NR PRR/groupcast option 2 |

With medium to high traffic, with groupcast option 1 transmission (NACK only) increasing the PSFCH period to N=10 (and thus accepting the increased latency and decreased group size) has no PRR gain with respect to baseline configuration with N=4, where the NR terminal takes into account the LTE sensing for PSSCH resource allocation only, and does not implement any measure to mitigate the PSFCH issue. In this case, there is no interest whatsoever in applying Alt.2. Yet, Alt.3 (confine PSFCH withing the guard symbol of LTE) performs better, procuring a range extension of 10-20m with respect to the baseline, and with virtually no impact on PSFCH performance.

| Figure 11: LTE PRR/groupcast option 1 | Figure 12: NR PRR/groupcast option 1 |
| --- | --- |
| Figure 13: LTE PRR/groupcast option 1 | Figure 14: NR PRR/groupcast option 1 |

The range gains of Alt. 2 and Alt 3 over N=4 baseline for LTE at PRR 95% is summarized in the following table:

Table 3: Communication range gain (in meters) over baseline (N=4 with no PSFCH collision mitigation)

|  | Alt 2 (N=10) | Alt 3 |
| --- | --- | --- |
| Groupcast option 1 (NACK only) High traffic (baseline coverage at PRR 95 % = 611 m) | 0 m | 20 m |
| Groupcast option 1 (NACK only) Medium traffic (baseline coverage at PRR 95 % = 681 m) | 0 m | 10 m |
| Groupcast option 2 (ACK/NACK) High traffic (baseline coverage at PRR 95 % = 425 m) | 70 m | 140 m |
| Groupcast option 2 (ACK/NACK) Medium traffic (baseline coverage at PRR 95 % = 555 m) | 40 m | 110 m |

Observation 9: With groupcast option 2, Alt.2 (N=10) brings some gain with respect to the case where no PSFCH collision mitigation is performed but is significantly outperformed by Alt.3 (PSFCH confined in LTE Guard).

Observation 10: With groupcast option 1, Alt.2 (N=10) has no gain with respect to the case where no PSFCH collision mitigation is performed.

Observation 11: With groupcast option 1, Alt.3 (PSFCH confined in LTE Guard) brings some gain with respect to both Alt. 2 (N=10) and with the case where no PSFCH collision mitigation is performed.

#### PRR performance of Alt.1 family

In the current subsection, we evaluate the performance of different flavors of Alt.1.

As explained in subsection 3.1.1.1 PSFCH collision is avoided based on RSRP criteria and the value ∆(dB) controls how much the LTE presence in a PSFCH slot weights in the resource allocation process and/or in the PSFCH dropping decision:

- If ∆→∞, LTE presence is ignored (no avoidance, NR UE never avoids collision)

- If ∆→-∞, LTE presence takes precedence (NR UE always avoids collision)

- Other values of ∆ perform a tradeoff between sensitivity to LTE presence and impact on NR

As performance benchmark, the curves corresponding to Alt 3 (PSFCH in LTE Guard) and N=4 with no PSFCH collision avoidance strategy (same as in the precedent section) are kept on the figures. It is worth noting that the blue curves for N=4 with no PSFCH collision avoidance strategy are a particular case of Alt.1, where ∆→∞.

Alt.1-1: PSFCH dropping at the RX side

It appears from the simulation results in the following figures that:

- Excessive PSFCH dropping severely degrades NR performance

- Unexpectedly, strict PSFCH dropping also degrades LTE performance. Although AGC problems are avoided for LTE, PSFCH dropping causes NR increased number of retransmissions: the average number of re(transmissions) goes from 1.6 when no PSFCH dropping is performed to around 3.6 when PSFCH is systematically dropped. This causes more collisions at PSSCH level, and the PRR performance of LTE is degraded. The effect is more severe with high traffic density.

- Loose PSFCH dropping does not procure enough gain for LTE, who performs roughly as in the absence of Alt.1.

- Conditional PSFCH dropping based on RSRP criteria is not effective

| Figure 15: LTE PRR/groupcast option 2-Study of Alt.1-1 | Figure 16: NR PRR/groupcast option 2-Study of Alt.1-1 |
| --- | --- |
| Figure 17: LTE PRR/groupcast option 2-Study of Alt.1-1 | Figure 18: NR PRR/groupcast option 2-Study of Alt.1-1 |

Alt.1-2: TX UE avoids selecting resources for PSCCH/PSSCH transmissions with corresponding PSFCH resources colliding with LTE transmission

It appears from the simulation results in the following figures that:

- Excessive resource exclusion degrades NR performance

- Unexpectedly, excessive resource exclusion also degrades LTE performance. Although AGC problems are avoided for LTE, this strategy degrades the quality of the resource used by the NR and also has an effect of concentrating NR transmissions onto resources that it would not have naturally picked. This leads to an increased number of NR retransmissions: the average number of re(transmissions) goes from 1.6 (ignore LTE) to around 2.2 (exclude all PSSCH resource causing PSFCH collision). This causes more collisions at PSSCH level, and the PRR performance of LTE is degraded. The effect is more severe with high traffic density, but Alt.1-2 performs averagely better than Alt.1-1.

- Loose exclusion at TX does not procure enough gain for LTE, who performs roughly as in the absence of Alt.1.

- Conditional Alt.1-2 resource selection based on RSRP criteria is not effective

| Figure 19: LTE PRR/groupcast option 2-Study of Alt.1-2 | Figure 20: NR PRR/groupcast option 2-Study of Alt.1-2 |
| --- | --- |
| Figure 21: LTE PRR/groupcast option 2-Study of Alt.1-2 | Figure 22: NR PRR/groupcast option 2-Study of Alt.1-2 |

Observation 12: PSFCH collision mitigation by PSFCH dropping (Alt.1-1) and/or PSSCH resource selection (Alt.1-2) degrades both NR and LTE PRR performance when excessive dropping/exclusion is performed.

Observation 13: PSFCH dropping at the receiver side (Alt.1-1) has lower performance than avoidance at the transmitter side (Alt.1-2).

Observation 14: Condition-based PSFCH collision mitigation by PSFCH dropping (Alt.1-1) and/or PSSCH resource selection (Alt.1-2) is not effective when the condition is RSRP: no performance improvement with respect to the case where no collision strategy is implemented.

Observation 15: PSFCH collision mitigation by PSFCH dropping (Alt.1-1) and/or PSSCH resource selection (Alt.1-2) are both largely outperformed by Alt.3 (PSFCH in LTE Guard).

Proposal 2: Do not support PSFCH dropping at the receiver side for reducing the time domain overlap between PSFCH transmission and LTE transmissions.

Proposal 3: Further investigate whether other conditions than RSRP may be effective when UE avoids selecting resources for PSCCH/PSSCH transmissions with corresponding PSFCH resources colliding with LTE transmission based on a condition.

#### Consequences of not supporting PSFCH

If a lack of consensus on the way forward on PSFCH issues continues, and the unfortunate issue is that NR PSFCH cannot be supported for Rel.18 coexistence, it is worthy to note that there is a non-negligible associated performance impact.

In this section, we evaluate the case where PSFCH is disabled and replaced by blind two-fold re-transmission. The performance loss on NR is obviously important. It should be noted that, from LTE perspective only, blind re-transmission performs better than Alt.2 with N=10. On the other hand, Alt.3 performs better than blind re-transmission, since the average number of retransmission is lower than 2.

| Figure 23: LTE PRR/groupcast option 2 (with blind 2Tr) | Figure 24: NR PRR/groupcast option 2 (with blind 2Tr) |
| --- | --- |
| Figure 25: LTE PRR/groupcast option 2 (with blind 2Tr) | Figure 26: NR PRR/groupcast option 2 (with blind 2Tr) |

## On device types

In RAN1#109-e, the following decision was made:

For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence,

- For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module.

  - FFS details on how the NR SL module uses this information.

  - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc.

- FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL.

- FFS: Whether/how device type B should be supported.

RAN1#110 made the following working assumption

Co-channel coexistence between LTE SL and NR SL is supported for device type A. Device type A contains both LTE SL and NR SL modules. For device type A, the NR SL module may use the sensing and resource reservation information shared by the LTE SL module.

While this gives some perspective on how a device type A behaves, the group didn’t manage to define device type B. In RAN1#110b-e, it was moreover agreed that

For dynamic resource pool sharing, the NR SL module is expected to use the information shared by the LTE SL module to the NR SL module which is known by NR SL module at the latest T ms prior to slot n.

There is so far no limitation or proposed behavior for the case where the LTE information is outdated or absent. Most simulation results presented so far do not take into account the aging of the information provided by the LTE module. We believe that all NR devices allowed to coexist must be aware of the resources being occupied by LTE transmissions. Whether the NR module acquires this knowledge from a collocated LTE module or by other means is of lesser importance.

Some devices which are not dual module devices (and thus may not qualify as “type A” according to the previous agreements) can equally be supported with a limited (or none) effort on top of what needs to be done for dual module devices. Examples for such devices are NR devices where the NR module is able of detecting the LTE transmission (e.g. blindly, based on RRSI measurements, able of decoding LTE SCI, etc), or where it can obtain sensing information through inter-UE coordination, etc.

Among the simulation results in section 3.5, the results produced in a dynamic scenario picture both the case of a Type A ideal device (having instantaneous access to the full sensing results of the LTE module) and of a very basic Type B NR module (with no PHY tunnel from a dual LTE module), who performs RSSI based exclusion on top of the Rel.17 NR sensing. The performance gap between ideal Type A and simplified Type B is already small and can be further closed if we consider more realistic assumptions for Type A behavior.

Proposal 4: Co-channel coexistence between LTE SL and NR SL is supported for NR devices capable of detecting an LTE SL transmission:	
 	-dual module devices where the NR module uses the sensing and resource reservation information shared by the LTE module (Type A)	
 	-NR devices excluding LTE resources at least based on RSSI measurements (Type B)
 	-FFS other method(s) for the NR module to be aware of resources being occupied by LTE SL.

## On higher subcarrier spacing

In RAN1#110, the following agreement was made:

For co-channel coexistence in Rel-18, dynamic resource pool sharing is studied, with the following constraints:

  - NR SL resource pool is configured with 15 kHz SCS.

    - FFS support of NR SL resource pool configured with higher SCS, including other solutions to overcome the AGC issue caused by the differing SCSs between the NR SL and LTE SL resource pools

One of the main purposes of co-channel coexistence is to ensure smooth and spectrally efficient transition from LTE to NR V2X. It would be beneficial to have a flexible solution allowing to map any NR traffic into a LTE resource pool, without limitations on the subcarrier spacing.

Given that 30 kHz SCS is mandatory for Rel-16/17 UEs, 30kHz subcarrier spacing should remain mandatory for Rel.18 UEs also, including in coexistence scenarios.

Moreover, 60kHz subcarrier spacing has the advantage of easing coexistence with 15kHz LTE transmission, because the PSFCH issue can simply be avoided by proper configuration: NR UEs with 60kHz, if configured with PSFCH every 4 slots, have the PSFCH transmission falling into the guard interval of the LTE transmission and do not cause any PSFCH issue (see Figure 1).

Observation 16: NR transmission with SCS 60kHz and PSFCH every 4 slots can be supported without any specification change and without PSFCH issues.

Proposal 5: For co-channel coexistence with dynamic resource sharing, support NR SL transmissions with SCS of 15kHz, 30kHz and 60kHz.

In the following, we will discuss separately the AGC issue caused by PSSCH transmission with higher SCS, and the PSFCH issue (which occurs for PSFCH transmissions with 15kHz and 30kHz subcarrier spacings).

Concerning the AGC issues caused by PSSCH transmissions, the following proposal [9] was discussed in RAN1#110b-e without any agreement:

Proposal 1-5 (V): - For dynamic resource pool sharing, the following options are studied to resolve the AGC issue in LTE SL UEs if higher SCSs are supported: - Option 1: Use of multi-slot transmissions or slot aggregation, where the NR SL transmissions of higher SCSs occupy all symbols (across multiple time slots) within a LTE SL subframe of 15 kHz at least when the NR SL transmission overlaps an LTE SL transmission. - Option 2: NR SL UE transmits LTE SCIs (SCI format 0 or 1), indicating resources reserved by NR SL transmissions, informing the LTE SL UEs about the resource reservations used by NR SL UEs. - Option 3: NR SL UE uses the information shared by the LTE SL module in its own resource selection procedure to exclude slots overlapping with LTE SL transmissions. - Other options are not precluded, including combination of more than one option.

Concerning Option 1, multi-slot transmissions or slot aggregation are known tools, already considered for SL-U. From AGC perspective, there is no absolute need to occupy all slots overlapping an LTE transmission: at least the first one should be occupied in order to avoid AGC issues. Nevertheless, occupying all the slots renders NR transmission more visible (RSSI-wise) to LTE sensing, at the expense of increased resource granularity for NR.

Concerning Option 2, it has the advantage of a fairer resource balance between LTE and NR, and it avoids systematically sacrificing NR transmission in the favour of LTE transmission. It also allows the LTE modules to avoid NR transmissions based on the priority associated to the NR transmission. On the other hand, such signalling implicitly assumes transmission with slot aggregation only. Nevertheless, more details on the implementation of this solution are needed to clarify the feasibility and specification impact, especially to understand:

- in which time-frequency resources this LTE SCI is transmitted, especially in resource pools where LTE PSSCH and PSCCH are configured contiguous

- how does such a transmission pass the RSRP check made by LTE UEs on LTE PSSCH DMRS

- what is the impact on NR PSSCH/PSCCH design, on NR slots with PSFCH, on TB mapping etc..

Concerning Option 3, the current formulation is rather unclear, but this option does not seem needed.

- If the intention is to exclude only subchannels occupied by LTE (collision), this is not related to higher SCS or to AGC, and this will be solved regardless of the higher SCS aspect by the resource allocation procedure.

- If the intention is to exclude all slots overlapping in the time domain with LTE subframes, and arrive at some sort of TDM-like outcome between LTE and NR, this leads to excessive resource exclusion for NR. In the toy example of Figure 27, with this option, all NR transmissions would compete on 25% of the available resource (pink slots) although 75% of the resource is free and could be occupied by NR transmission with Option 1. This type of excessive exclusion seriously impacts the NR performance due to NR resource starvation and should be avoided.

| LTE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LTE |  |  |  | LTE |  |

Figure 27: Toy example of NR resource starvation with Option 3

Proposal 6: For dynamic resource pool sharing, the following options to resolve the AGC issue in LTE SL UEs if higher SCSs are supported:

  - Option 1: Use of multi-slot transmissions or slot aggregation, where the NR SL transmissions of higher SCSs occupies at least the first slot and possibly all slots within a LTE SL subframe of 15 kHz at least when the NR SL transmission overlaps in the time domain with an LTE SL transmission.

  - FFS Option 2: NR SL UE transmits LTE SCIs (SCI format 0 or 1), indicating resources reserved by NR SL transmissions, informing the LTE SL UEs about the resource reservations used by NR SL UEs.

  - Combination of more than one option is not precluded.

## On combinations of operation modes

In RAN1#109e, the following agreement was made:

For the study of co-channel coexistence solutions in Rel-18, the combination of operational modes Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority.

- FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C).

RAN plenary guidance encouraged RAN1 to continue the work on dynamic resource pool sharing […] with high priority for Type A devices and operating combination A.

Combinations B and C should be equally considered. With respect to the specification support required by Combination A, support of combinations B and C are likely to require very limited (in the case of Combination C, potentially none) extra work.

Proposal 7: For the study of co-channel coexistence solutions in Rel-18, combination of operational modes B and C are also considered.

## Co-channel coexistence with full or partial (Alt.4) resource pool separation between LTE-V2X and NR-V2X

The Rel.16 long term TDM solution based on (semi-)static, (pre)configured time-domain partition between LTE and NR resource pools can be translated to the co-channel coexistence case in a rather straightforward manner. The advantage of such a direct extension is that the specification work is very limited. Since the LTE and NR resources are TDMed, there are no restrictions on numerology or configurations to be used since each RAT uses its own separate resource and Rel.16 framework already handles concurrent operation from UE perspective. In the past RAN1#110 meeting, the group draw the following conclusion

For co-channel coexistence in Rel-18, RAN1 concludes that the TDM-based semi-static resource pool partitioning based on Rel-16/17 specifications is one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs.

  - Note: The LTE and NR resource pools do not overlap in time with each other in the TDM-based semi-static resource pool partitioning.

  - Note 2: Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe

  - FFS: potential enhancements for synchronization can be further investigated

The static solution has the advantage of simplicity and of very limited specification impact. Yet, the static TDM solution cannot address by itself the needs expressed during Rel.18 package discussions, and which motivated the inclusion of co-channel coexistence work.

Indeed, once deployed, the TDM solution becomes practically static and is unable to adapt to the traffic changes between the two RATs. Any evolution of the resource pool configuration is submitted to regulatory changes, and even if such changes occur, it is almost impossible to ensure that all vehicles update their configuration in a timely manner. As further discussed in section 3.5, improper resource pool partitioning between the two RATs severely degrades the system performance. Static TDM partitioning in ITS spectrum cannot respond to the needs of a flexible technology transition path from LTE to NR.

Observation 17: In ITS spectrum, solutions with static TDM partition between LTE and NR pools are unable to adapt to the traffic changes between the two RATs and cannot respond to the need for a flexible technology transition path from LTE to NR.

Observation 18: In ITS spectrum, reconfiguration of static TDM partitions between LTE and NR pools cannot be assumed.

Potential enhancements to the Rel.16 TDM solutions may be further discussed, such as UE to network assistance for better resource pool configuration management (for cases where a network deployment exists). The study of combinations of operation modes B and C can be beneficial to improve the performance of TDM solutions, which may be a more viable options for in-coverage or mixed coverage scenarios.

To overcome some of the technical challenges raised by a fully dynamic approach, some hybrid solutions assuming partial overlap between NR and LTE resource pools were also proposed (e.g. assume that the PSFCH resources of NR would be in a non-overlapping part of the resource pool). For that to be possible, it is assumed there are some slots that are in the NR resource pool and not in the LTE resource pool, that is, there are some conveniently places “gaps” between physical slots in the LTE resource pool. Yet, as explained in [8], by default, an LTE resource pool is configured to occupy contiguous subframes across time based on the bitmap definition, for the number of subchannels configured in the resource pool. That is, in ITS spectrum, the existence of such conveniently placed gaps cannot be assumed, their existence is up to regulatory changes, that implies the same drawbacks and lack of practical feasibility as the full TDM partition (completely separated resource pools). We need to design solutions capable of dealing with fully overlapped resources (same resource pool) between LTE SL and NR SL and that do not rely on such unrealistic assumptions.

Observation 19: In ITS spectrum, the existence of NR resources TDM-ed with resource pools configured for LTE SL transmission cannot be assumed.

In the unlikely case where such conveniently placed gaps would exist or would be enabled by future regulation changes, it is a (pre)configuration issue to place e.g. the PSFCH slots in the non-overlapping parts and there is no specific design or specified solution that is needed to support this. Moreover, simulation results in section 3.1.2, where this option corresponds to Alt.4, show that there is no performance benefit from applying such kind of solutions. Indeed, when applied, Alt.4 reduces the amount of available resources for LTE, which increases the collision probability and decreases the PRR. This penalty is more important than the expected gain in terms of avoiding collisions between NR PSFCH and LTE PSSCH: LTE performance with Alt.4 is worse than doing nothing (no specific measure for PSFCH issue mitigation).

It is therefore necessary to concentrate on solutions dealing with full resource overlap, that are equally applicable to partial overlap, instead of designing solutions for corner cases of partial overlap, which are not applicable to the more general case of full overlap.

Proposal 8: For dynamic resource pool sharing, the design target is the case where LTE-V2X and NR-V2X resource pools are fully overlapped (equivalent to a single resource pool).

# Conclusions

Based on extensive simulation results, the following conclusions appear:

- Alt.3 (confining the PSFCH transmission within the LTE guard) has best overall performance and largely outperforms all other schemes in all tested scenarios.

- Alt.2 (increase the PSFCH periodicity to N=10) procures some gain with groupcast option 2 transmission (ACK/NACK) but lacks to bring any gain with groupcast option 1 transmission (NACK only).

- Alt.1 based on RX PSFCH dropping is detrimental to both LTE and NR performance.

- Alt.1 based on TX avoiding resources with corresponding PSFCH resource colliding with LTE transmission needs more investigation: excessive avoidance is detrimental, and conditional behavior based on RSRP thresholds fail to improve the situation. However, other condition-based strategies were not tested.

The following proposals thus stand:

Proposal 1: To eliminate the PSFCH issue, confine the PSFCH transmission, in the time domain, within the guard symbol of the LTE transmission.

Proposal 2: Do not support PSFCH dropping at the receiver side for reducing the time domain overlap between PSFCH transmission and LTE transmissions.

Proposal 3: Further investigate whether other conditions than RSRP may be effective when UE avoids selecting resources for PSCCH/PSSCH transmissions with corresponding PSFCH resources colliding with LTE transmission based on a condition.

Proposal 4: Co-channel coexistence between LTE SL and NR SL is supported for NR devices capable of detecting an LTE SL transmission:	   -dual module devices where the NR module uses the sensing and resource reservation information shared by the LTE module (Type A)    -NR devices excluding LTE resources at least based on RSSI measurements (Type B)   -FFS other method(s) for the NR module to be aware of resources being occupied by LTE SL.

Proposal 5: For co-channel coexistence with dynamic resource sharing, support NR SL transmissions with SCS of 15kHz, 30kHz and 60kHz.

Proposal 6: For dynamic resource pool sharing, the following options to resolve the AGC issue in LTE SL UEs if higher SCSs are supported:

Proposal 7: For the study of co-channel coexistence solutions in Rel-18, combination of operational modes B and C are also considered.

Proposal 8: For dynamic resource pool sharing, the design target is the case where LTE-V2X and NR-V2X resource pools are fully overlapped (equivalent to a single resource pool).

As a useful reminder, the full set of observations is summarized here-below:

Observation 1: For NR transmission with SCS 60kHz and PSFCH enabled, although the slot with PSFCH at 60kHz overlaps in the time domain with the LTE subframe, there is no collision between PSFCH resource and LTE transmission.

Observation 2: Alt.1-1 and Alt1-2 need to apply resource exclusion under limiting conditions (e.g. exclude only under some RSRP/RSSI and/or priority conditions) in order to avoid excessive dropping/resource exclusion and NR resource starving.

Observation 3: Alt.2 has several drawbacks including increased latency, penalized PSFCH multiplexing capability, and potentially significant specification impact to re-define the correspondence between PSSCH resources and PSFCH time/frequency/code sets with irregular spacing.

Observation 4: The specification impact of Alt.3 (PSFCH confined in LTE Guard) is very limited, since 60kHz transmission is already specified.

Observation 5: Alt.3 (PSFCH confined in LTE Guard) can be produced, in implementation, either as “natively 60kHz” or based on the PSSCH SCS, through signal processing methods (comb subcarrier nulling etc…)

Observation 6: For Alt.3 (PSFCH confined in LTE Guard) with PSFCH symbols natively produced in 60kHz SCS, guard bands can often be relaxed with respect to the already specified 60kHz durations.

Observation 7: Alt.3 (PSFCH confined in LTE Guard) does not suffer from increased latency with respect to already existing Rel.16 PSFCH configurations.

Observation 8: PSFCH multiplexing capability of Alt.3 (PSFCH confined in LTE Guard) is overall better than in Alt.2 and can in most cases be the same as in Rel.16 corresponding configuration.

Observation 9: With groupcast option 2, Alt.2 (N=10) brings some gain with respect to the case where no PSFCH collision mitigation is performed but is significantly outperformed by Alt.3 (PSFCH confined in LTE Guard).

Observation 10: With groupcast option 1, Alt.2 (N=10) has no gain with respect to the case where no PSFCH collision mitigation is performed.

Observation 11: With groupcast option 1, Alt.3 (PSFCH confined in LTE Guard) brings some gain with respect to both Alt. 2 (N=10) and with the case where no PSFCH collision mitigation is performed.

Observation 12: PSFCH collision mitigation by PSFCH dropping (Alt.1-1) and/or PSSCH resource selection (Alt.1-2) degrades both NR and LTE PRR performance when excessive dropping/exclusion is performed.

Observation 13: PSFCH dropping at the receiver side (Alt.1-1) has lower performance than avoidance at the transmitter side (Alt.1-2).

Observation 14: Condition-based PSFCH collision mitigation by PSFCH dropping (Alt.1-1) and/or PSSCH resource selection (Alt.1-2) is not effective when the condition is RSRP: no performance improvement with respect to the case where no collision strategy is implemented.

Observation 15: PSFCH collision mitigation by PSFCH dropping (Alt.1-1) and/or PSSCH resource selection (Alt.1-2) are both largely outperformed by Alt.3 (PSFCH in LTE Guard).

Observation 16: NR transmission with SCS 60kHz and PSFCH every 4 slots can be supported without any specification change and without PSFCH issues.

Observation 17: In ITS spectrum, solutions with static TDM partition between LTE and NR pools are unable to adapt to the traffic changes between the two RATs and cannot respond to the need for a flexible technology transition path from LTE to NR.

Observation 18: In ITS spectrum, reconfiguration of static TDM partitions between LTE and NR pools cannot be assumed.

Observation 19: In ITS spectrum, the existence of NR resources TDM-ed with resource pools configured for LTE SL transmission cannot be assumed.

# Annex: Simulation parameters

The following simulation parameters were used in section 3.5

| Sidelink Frequency | 6 GHz |
| --- | --- |
| BW | 20MHz |
| SCS | 15kHz |
| Number of subchannels | 4 |
| Channel | LOS/NLOSv (from TR 37.885) with 23dBm transmit power |
| Antenna gain | Gt=Gr=3dB |
| UE drop and mobility modeling | Highway, 6 lanes with wraparound Option A with 140 km/h and T=4 (37.375 devices per km) |
| Device type | R18-Type A (dual NR+LTE RAT with NR SPS based also of LTE RAT sensing) R14/15-Type C (mono RAT LTE) |
| Traffic model | Periodic, RRI={20, 50, 100}ms Priority=1 Latency = RRI LTE: 2 blind transmissions NR: Up to 4 non-blind transmissions Probability of reselection = 0.2. Packet size: - NR: @RRI=50ms: 1200/800 bytes - LTE: @RRI=100ms: 300/190 bytes |
| Communication Mode (NR) | Groupcast option 1 (NACK only) or groupcast option 2 (ACK/NACK). |
| PSFCH slots properties | Minimum response latency K=2 and period N=1,2,4 or 10. |
| Recipient set definition | The recipients are all other NR RAT at a distance from the sender smaller than 240 m (=> average of 6, 9). |
| MCS | 1 TB/slot/subchannel: - 16QAM 5/6 with 1200 bytes - 16QAM 3/4 with 800 bytes - QPSK 1/2 for 300 bytes - QPSK1/3 for 190 bytes. |
| RAT number and density | High traffic density: 56 RATs per km, i.e. 18.75 NR RAT/km and 37.375 LTE RAT/km). Global channel load: 750 different TB per s and per km in average. Medium traffic density: 50 RATs per km, i.e. 12.5 NR RAT/km and 37.375 LTE RAT/km). Global channel load: 625 different TB per s and per km in average. |

# References

- RP-220300, WID revision: NR sidelink evolution, 3GPP TSG RAN Meeting #95e.

### RWS-210360, 5GAA input to 3GPP Rel.18 Workshop, 5GAA.

### RWS-21006, On NR-LTE V2X Co-Channel Coexistence, Qualcomm.

### RWS-210244, Enhancements for sidelink operations, LG Electronics.

### RWS-210445, NR sidelink and V2X enhancements, Huawei, HiSilicon.

### Chairman’s notes, RAN1#110, August 2022, Toulouse, France.

### Chairman’s notes, RAN#97-e, electronic meeting.

### RP-222369, Discussion on Co-Channel Coexistence for LTE Sidelink and NR Sidelink, Continental Automotive GmbH, Toyota InfoTechnology Center, Robert Bosch GmbH, Mitsubishi, Volkswagen AG, Deutsche Telekom, Fraunhofer HHI, Fraunhofer IIS/ RAN#97-e, electronic meeting.

### R1-2210298, FL Summary EoM for AI 9.4.2 - Co-Channel Coexistence for LTE and NR Sidelink, RAN1#110b-e.

### R1-2212540, FL Summary EoM for AI 9.4.2 - Co-Channel Coexistence for LTE and NR Sidelink, RAN1#110b-e.