3GPP TSG RAN WG1 #112			R1-2301166

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.1.1.2

Source:	Google

Title:	Discussion on two TAs for multi-DCI

Document for: 	Discussion

- Introduction

In this contribution, we discuss issues of two TA operation and provide our views in the following paragraphs. In Rel-18 MIMO WID [1], 2TA operation has been approved to study and, if justified, specify. In previous meeting(s) [2] [3] [4], RAN1 has agreed to support 2TA operation, including intra- and inter-cell MTRP framework. RAN1 has also agreed 2TA operation is supported for both FR1 and FR2. Agreements from previous RAN1 meetings are shown in the followings. However, there are still some issues left to be technically debated and resolved. We discuss some issues and provide our opinions in below session(s).

RAN1 #109(e) Agreement Enhancement on two TAs for UL multi-DCI for multi-TRP operation is supported in Rel-18. Note 1: whether (1) the network signals two TACs or (2) the network signals one TAC and the UE deriving the second TA can be further studied. Note 2: evaluations can be considered on as-needed basis. Agreement Support two TA enhancement for both intra-cell and inter-cell multi-DCI multi-TRP scenarios in Rel-18. Agreement Enhancements on two TAs for UL multi-DCI for multi-TRP operation are applicable to both FR1 and FR2. Conclusion For multi-DCI multi-TRP operation with two TAs, the decision on the maximum uplink timing difference is left up to RAN4. - Send an LS to RAN4 asking them the maximum uplink timing difference RAN1 can assume between the two TAs for multi-DCI multi-TRP operation. The LS is endorsed in R1-2205593. Agreement Two TA enhancement for uplink multi-DCI based multi-TRP operation are applicable to at least: - TDM based multi-DCI uplink transmission - simultaneous multi-DCI uplink transmission (if simultaneous uplink multi-DCI uplink transmission is supported in Agenda 9.1.4.1) - Note: Whether two TA enhancement is applicable to other schemes is a separate discussion, which is not in the scope of AI 9.1.1.2. RAN1 #110 Agreement: For multi-DCI based multi-TRP operation with two TAs, support configuring two TAGs belonging to a serving cell. Agreement: For multi-DCI based multi-TRP operation with two TAs, study the impact of two TAs for the following: - RACH triggered by PDCCH order in intra-cell MTRP case - RACH triggered by PDCCH order in inter-cell MTRP case - Which might require RACH enhancement as well - UE triggered RACH by CBRA or CFRA in RRC connected mode Further details of enhancements needed (if any) Agreement For multi-DCI multi-TRP operation with two TAs, up to two n-TimingAdvanceOffset value per serving cell is supported. RAN1 #110b(e) Agreement For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG - baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length - as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length - FFS: the maximum Rx timing difference (could be up to RAN4) - Other than UE capability details and relevant configuration, no additional RAN1 specification enhancement specific for this case is expected Agreement Multi-DCI multi-TRP operation with two TAs is supported for Rel-15/16/17 TCI frameworks and unified TCI framework extension discussed in 9.1.1.1 as well as UL beam indication via spatial relation. Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support enhancements related to indicating TAG ID via absolute TA command: - FFS: whether the indication is implicit or explicit - Detailed indication schemes are FFS - This does not preclude indication of two TAG IDs (if supported) - Note: This applies at least to MSGB in case of C-RNTI Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it cannot always be assumed that both TRPs have knowledge of the overlapping region between transmissions corresponding to the two TAs. - Note: This doesn’t prevent the network from applying scheduling restrictions even if the TRPs have no knowledge of the overlapping region Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support at least one of the following alternatives (down selection to be done in RAN1#111): - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 2: indicate TAG ID as part of PDCCH order - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 4: divide RACH resources into two groups, where for a RACH procedure, if the corresponding RACH resource belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 5: divide preambles into two groups, where for a RACH procedure, if the corresponding preamble belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP - Alt 6: TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order - Alt 7: Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order Note: If Alt 1 or Alt 2 is down-selected, then it does not preclude indication of two TAG IDs (if supported)

- Discussion

In order to support 2TA operation, UE is supposed to maintain two TA values at least for a serving cell at the same time. One critical issue is how UE determines which TA value to apply when transmitting an UL channel or RS. In previous RAN1 meetings, a couple of options have been intensively discussed. In RAN1#110b(e), several options are extracted to be down-selected in RAN1#111 as below. However, we failed to have a conclusion/agreement in RAN1#111 and this should be determined in RAN1#112.

Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, the four options agreed in RAN1#110bis-e are refined as below (down-selection of one or a combination of the options to be performed in RAN1#111): - Option 1: Associate TAG to TCI-state/spatial relation - Configure TAG ID as part of UL/joint TCI state or spatial relation - for UL transmission, the TAG ID associated with the UL/joint TCI state or spatial relation is utilized - Option 2: Associate TAG to CORESETPoolIndex - for dynamically scheduled/activated PUSCH, TAG associated with the CORESET pool index of the CORESET carrying the scheduling/activating PDCCH is utilized for UL transmission - for Type 1 CG, P/SP-SRS, and P/SP-PUCCH, coresetPoolIndex is RRC-configured. - FFS: Other signals/channels: AP-SRS, and dynamic HARQ-ACK - Option 3: Associate TAG to SSB group (if such an association is agreed in agenda 9.1.1.2). For a UL transmission, UE adopts the TAG associated with the SSB group such that - if the PL RS is an SSB, then the UE adopts the TAG associated with the SSB group which the PL RS of the UL transmission belongs to - if the PL RS is a CSI-RS, then the UE adopts the TAG associated with the SSB group which the QCL source SSB of the PL RS belongs to - Option 4: TAG association performed as follows: - for dynamically scheduled/activated channels/signals, TAG associated with the CORESET pool index of the CORESET carrying the scheduling PDCCH is utilized for UL transmission - for P/SP UL channels / signals (not scheduled or activated by DCI), TAG ID is RRC-configured.

From observations over previous RAN1 meetings, Option 1 and 2 are most popular options, each of which has a number of supporting companies. Regarding Option 2, it could be suitable for UL channel/RS scheduled dynamically. That is, which TA value to apply for the scheduled UL channel/RS depends on the CORESETPoolIndex configured for the scheduling CORESET. However, before that, network may need to configure or indicate the association between TA value and CORESETPoolIndex firstly. Further, preconfigured UL channel or RS is not configured with CORESETPoolIndex in legacy. For example, configured UL grant, PUCCH reporting periodic CSI or periodic SRS. Therefore, network is supposed to configure CORESETPoolIndex to those preconfigured UL channel or RS. Option 4 may be able to resolve this issue. However, it still has drawback regarding the latency to switch TA value for an UL channel or RS.

On the other hand, Option 1 may provide a more flexible and compatible framework. Which TA value to apply for a PUSCH transmission occasion can be determined based on its indicated UL beam. The effort to achieve the goal is how to associate TA value with UL beam. Here, we should consider both unified TCI, legacy TCI and spatial relation. When considering spatial relation, both PUCCH spatial relation and SRS spatial relation should be taken cared. In our opinions, Option 3 is also feasible. Option 3 can also provide a unified framework to associate with TAG regardless of beam indication framework (Rel-15/16 TCI/spatial relation or Rel-17 unified TCI).

In overall, based on above analysis, we suggesting adopting Option 2. However, Option 3 is also acceptable to us. So, we propose the following proposal.

Proposal 1: For association of TA values with UL channels/RSs, support one of the following options

- Option 1: Associate TAG to TCI-state/spatial relation

- Option 3: Associate TAG to SSB group

In RAN1#110bis (e), there were a couple of agreements related to inter-cell M-TRP, which are shown as below.

Agreement For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, support one of the alternatives (down selection to be done in RAN1#111): - Alt 1: PDCCH scheduling RAR will always be received from serving cell => there is no need for additional type 1 CSS configuration per additional PCI - Alt 2: In addition to PDCCH scheduling RAR being received from serving cell, reception of PDCCH scheduling RAR from a TRP corresponding to an additional PCI for a RACH procedure associated to the additional PCI is supported additional type 1 CSS configuration per additional PCI needs to be supported Agreement For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, support PRACH configuration associated with additional configured PCIs different from the PCI of the serving cell. - FFS: details Agreement For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, support a mechanism to determine which PRACH configuration (i.e., RACH configuration corresponding to serving cell PCI or an additional PCI) to be used in the RACH procedure triggered by PDCCH order - FFS: Explicit indication or implicit indication through PDCCH order

Regarding the PDCCH scheduling RAR, we support that it can be transmitted from a TRP corresponding to an additional PCI. We consider it beneficial especially for mobility case. It can avoid additional message(s) transferring between two TRPs and reduce the latency of receiving the PDCCH scheduling RAR.

On the other hand, as quoted above, RAN1 has already agreed to support PRACH configuration associated with additional configured PCIs different from the PCI of the serving cell. In addition, we agreed to support to indicate which PRACH configuration to be used for RA procedure triggered by PDCCH order. One remaining open issue is how to indicate that by PDCCH order. We suggest going with the simplest way, i.e., to explicitly indicate it in PDCCH order, given that there still some reserved bits in PDCCH order. Above discussions lead to the following proposals.

Proposal 2: For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, support the following:

- Alt 2: In addition to PDCCH scheduling RAR being received from serving cell, reception of PDCCH scheduling RAR from a TRP corresponding to an additional PCI for a RACH procedure associated to the additional PCI is supported

  - Additional type 1 CSS configuration per additional PCI needs to be supported.

Proposal 3: For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, explicit indication in PDCCH order is supported for determining which PRACH configuration to be used in the RACH procedure triggered by PDCCH order.

Besides inter-cell M-TRP, we have the followings for intra-cell M-TRP.

Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support at least one of the following alternatives (down selection to be done in RAN1#111): - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 2: indicate TAG ID as part of PDCCH order - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 4: divide RACH resources into two groups, where for a RACH procedure, if the corresponding RACH resource belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 5: divide preambles into two groups, where for a RACH procedure, if the corresponding preamble belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP - Alt 6: TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order - Alt 7: Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order Note: If Alt 1 or Alt 2 is down-selected, then it does not preclude indication of two TAG IDs (if supported)

For this agreement, we do not think indicating TAG ID in RAR can work. It may involve NBC issue since legacy UE may have issue to decode a PDSCH containing RAR message. We prefer simple way and explicit indication. Therefore, we support Alt 2 to indicate TAG ID directly in PDCCH order. However, if implicitly way is preferred by the group, we can also accept Alt 7. Therefore, we propose the followings.

Proposal 4: For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support either Alt 2 or Alt 7:

- Alt 2: indicate TAG ID as part of PDCCH order

- Alt 7: Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order.

In previous RAN1 meetings, RAN1 also discussed whether a PDCCH order intended for one TRP can be triggered by the other one TRP. Finally, we concluded that at least for inter-cell multi-DCI, cross-TRP PDCCH order can be supported. The related agreements are shown as below.

RAN1 #110b(e) Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support one of the following alternatives in RAN1#111: - Alt 1: PDCCH order sent by one TRP triggers RACH procedure towards the same TRP - note: with Alt 1, PDCCH order sent by one TRP triggering RACH procedure towards another TRP is not allowed - Alt 2: PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP - This does not preclude PDCCH order triggering two RACH procedures for two TRPs RAN1 #111 Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP at least for inter-cell Multi-DCI. - FFS: for intra-cell Multi-DCI - FFS: whether there are any restrictions needed - FFS: if cross TRP RACH triggering is an optional feature

From our perspective, given that we have agreed to support cross-TRP PDCCH order for inter-cell M-DCI case, there is no harm to further support intra-cell M-DCI case. First, it provides NW the scheduling flexibility. Second, it seems from the agreement for determining which TA/TAG is related to a RA procedure, cross-TRP PDCCH order for intra-cell M-DCI is naturally supported from some alternatives. Based on above reasons, we suggest the following.

Proposal 5: For intra-cell multi-DCI, support PDCCH order sent by one TRP can trigger RACH procedure towards either the same TRP or a different TRP.

- Conclusion

According to the above discussion(s), we have the following proposal(s).

Proposal 1: For association of TA values with UL channels/RSs, support one of the following options

- Option 1: Associate TAG to TCI-state/spatial relation

- Option 3: Associate TAG to SSB group

Proposal 2: For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, support the following:

- Alt 2: In addition to PDCCH scheduling RAR being received from serving cell, reception of PDCCH scheduling RAR from a TRP corresponding to an additional PCI for a RACH procedure associated to the additional PCI is supported

  - Additional type 1 CSS configuration per additional PCI needs to be supported.

Proposal 3: For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, explicit indication in PDCCH order is supported for determining which PRACH configuration to be used in the RACH procedure triggered by PDCCH order.

Proposal 4: For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support either Alt 2 or Alt 7:

- Alt 2: indicate TAG ID as part of PDCCH order

- Alt 7: Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order.

Proposal 5: For intra-cell multi-DCI, support PDCCH order sent by one TRP can trigger RACH procedure towards either the same TRP or a different TRP.

# Reference

- RP-213598 New WID: MIMO Evolution for Downlink and Uplink, Samsung

- Draft Report of 3GPP TSG RAN WG1 #109-e v0.3.0 (Online meeting, 9th – 20th May 2022)

- Draft Report of 3GPP TSG RAN WG1 #110 v0.2.0 (Toulouse, France, 22nd – 26th August 2022)

- Draft Report of 3GPP TSG RAN WG1 #110bis-e v0.1.0 (Online, 10th – 19th October 2022)