3GPP TSG RAN WG1 #112			R1-2301247

Athens, Greece, February 27th – March 3rd, 2022

Agenda item:	9.1.2

Source: 	Moderator (Samsung)

Title: 	Moderator Summary on Rel-18 CSI enhancements

Document for:	Discussion and Decision

## Introduction

The scope given in the Rel-18 NR Evolved MIMO WID pertaining to CSI enhancement is as follows:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

## Summary of companies’ views

Proposals to be presented online:

Type-II CJT

| 1.3 | Conclusion 1.C.1: On the Parameter Combination of Type-II codebook refinement for CJT mTRP, there is no consensus on adding a new (not previously agreed) codebook parameter, as well as replacing the legacy parameter L with a new (not previously agreed) parameter. - Note: Since dynamic {Ln} selection was agreed, this implies that the list of supported {Ln} combinations will be discussed separately from the list of supported {pv,} combinations - FFS: Whether/how the list of supported {Ln} combinations can be linked with the list of supported {pv,} combinations without introducing a new (not previously agreed) codebook parameter, e.g. via some UE capability Support/fine: Samsung, MediaTek, ZTE, Intel, Fujitsu, Ericsson, Apple, vivo, CATT, CMCC, NEC, NTT DOCOMO, Nokia/NSB, Lenovo, Spreadtrum, Sharp, Google, Qualcomm, Fraunhofer IIS/HHI, Xiaomi, FL Note: This proposal was discussed offline [1]. This conclusion merely states the fact and the inevitable outcome. Stating that “the conclusion is not acceptable” simply means inability to deal with the reality (albeit unpleasant). Current situation is summarized below: Separate tables for {Ln} and {pv,beta}, not adding a new parameter not previously agreed - Support/fine: Samsung, MediaTek (if resulting in fewer combos), ZTE, Intel, Fujitsu, Ericsson (2nd, with linkage), Apple, vivo, CATT, CMCC, NEC, NTT DOCOMO (with possible linkage, e.g. via Lmax constraint), Nokia/NSB, Replace L with Lmax in a single table (hence adding a new parameter not previously agreed) - Support/fine: MediaTek, OPPO, AT&T, Spreadtrum, Huawei/HiSi, IDC, Intel - Concern (against compromise in RAN1#111 and redundant): vivo, Samsung, ZTE, Nokia/NSB, CMCC, NEC, CATT, Replace L with Ltot in a single table (hence adding a new parameter not previously agreed) - Support/fine: Ericsson, LG, Qualcomm, Xiaomi, IDC - Concern (against compromise in RAN1#111 and redundant): vivo, Samsung, ZTE, Nokia/NSB, CMCC, NEC, CATT |
| --- | --- |
|  | Proposal 1.C.2: On the Type-II codebook refinement for CJT mTRP, only support NL ={2,4} as additional candidate values to NL=1. - FFS: Additional restriction(s) depending on the configured value for NTRP Support/fine: Huawei/HiSi, NTT DOCOMO, Ericsson, MediaTek, vivo, OPPO, ZTE, Lenovo, Apple, NEC, Sharp, Google, CMCC, Fujitsu, Nokia/NSB, Xiaomi, AT&T, Sony, Samsung |
|  | Proposal 1.C.3: On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {Ln} for the higher-layer-configured value of NTRP (FFS by RAN1#112: whether the bracketed permutations are also supported): - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {n} for Rel-17-based refinement are derived from the supported combinations of {Ln} for Rel-16-based refinement NTRP; {Ln} combination. 1; {2}. {4}. {6} (analogous to legacy, only for total # ports =32, rank 1-2, R=1. 2; {2,2}. {2,4}, [{4,2}]. {4,4}. 3; {2,2,2}. {2,2,4} [and its other permutations]. {4,4,4}. 4; {2,2,2,2}. {2,2,2,4} [and its other permutations]. {2,2,4,4} [and its other permutations]. {4,4,4,4}. Support/fine: ZTE, NTT DOCOMO, Samsung, Lenovo, vivo, Nokia/NSB, [Xiaomi], AT&T, FL Note: Just as conclusion, this proposal merely states the fact and the inevitable outcome given Table 1C. Stating that “the conclusion is not acceptable” simply means inability to deal with the reality (albeit unpleasant). |
|  | Proposal 1.C.4: On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {pv,} from where the value of {pv,} is gNB-configured via higher-layer (RRC) signaling: - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {M} for Rel-17-based refinement are derived from the supported combinations of {pv ,} for Rel-16-based refinement pv for layers 1-4; ; Condition(s). {1/8, 1/8, 1/16, 1/16}; ¼; --. ½; --. {1/4, 1/4, 1/8, 1/8}; ¼ (*); --. ½ (*); --. {1/4, 1/4, 1/4, 1/4}; ¾ (*); --. {1/2, 1/2, 1/2, 1/2}; ½; - Only applicable when NTRP≤3 and NL=1 - Optional. (*) Supported by legacy Rel-16 Support/fine: [ZTE], NTT DOCOMO, [Samsung], Lenovo, vivo, AT&T, FL Note: Just as conclusion, this proposal merely states the fact and the inevitable outcome given Table 1D. Stating that “the conclusion is not acceptable” simply means inability to deal with the reality (albeit unpleasant). |
| 1.4 | Proposal 1.D.1: On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, down-select between the following two alternatives (by RAN1#112-bis where n denotes the n-th CSI-RS resource): - Alt1. Prio(,l,m,n)=( k=0n-12Lk) .N.RI.P(m)+N.RI.l(n)+N.n - Note: This implies that CSI-RS resource is designated the highest priority - Alt2. Prio(,l,m,n)=2L’.Qn).RI.N3+2L’.RI. P(m)+RI.l(n)+ - Note: This implies that CSI-RS resource is designated the lowest priority (after FD basis) - Note: L’ denotes the max value of Ln from all selected N CSI-RS resources - FFS: Q(n) maps the index n according to a rule, e.g., Q(n)=n, or Q(n)=0 if n corresponds to strongest TRP/SCI. - Alt3. Replace SD basis index l in legacy Prio calculation with k=0n-12Lk+ln, i.e., SD basis index over all resources: Prio(,l,m,n) = 2Ltot.RI.P(m)+ RI.k=0n-12Lk+RI.l(n)+ FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m Support/fine: Samsung, Lenovo, CATT, NTT DOCOMO, OPPO, MediaTek, Apple, Google, vivo, NEC, LG, Ericsson, ZTE, Spreadtrum, Qualcomm, Fujitsu, CMCC, Nokia/NSB, Xiaomi, Huawei/HiSi, Fraunhofer IIS/HHI, |
| 1.5 | Proposal 1.E.1: On the Type-II codebook refinement for CJT mTRP, regarding CBSR, at least for restricting SD basis selection, the legacy CBSR scheme is fully reused for each of the RRC-configured NTRP CSI-RS resources (resulting in CSI-RS-resource-specific SD beam group restriction) - Joint rank restriction is applied across NTRP CSI-RS resources - Note: Assuming (N1,N2) is configured with CBSR (as in legacy), the configured (N1,N2) value is common for all CSI-RS resources per previous agreement - FFS: Whether amplitude restriction is CSI-RS-resource-common or specific, and soft vs hard restriction - FFS: Whether CBSR can be configured to be off for a CSI-RS resource Support/fine: Huawei/HiSi, ZTE, Spreadtrum, LG, Xiaomi, NEC, Intel, NTT DOCOMO, Nokia/NSB, Lenovo, vivo, OPPO, MediaTek, Apple, LG, ZTE, Sharp, Google, Qualcomm (no soft restriction on amplitude), CMCC, Ericsson (same as Qualcomm), [Samsung], NEC, Fujitsu, AT&T, Sony, |
| 1.2 | Proposal 1.B.1: On the Type-II codebook refinement for CJT mTRP, for mode-1, down select (in RAN1#112) only one from the following schemes - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FD basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources Support/fine: MediaTek, Samsung, Fujitsu, Ericsson, Nokia/NSB, Apple, ZTE, OPPO, vivo, Intel, Qualcomm, LG, CATT, Xiaomi, Huawei/HiSi (Minitial modified), Spreadtrum, NTT DOCOMO, NEC, Google, CMCC, AT&T, Sharp. |

TDCP

| 3.1 | Proposal 3.A: For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows: - Basic feature: Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported - Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported - For Y>1, the phase can be configured to be absent for all the Y delays - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource - TBD: Candidate value(s) for Y>1 - FFS: Value of Dbasic Support/fine: Ericsson, Nokia/NSB, Samsung, ZTE, Huawei/HiSi, Qualcomm, vivo, MediaTek, NTT DOCOMO, NEC, Sharp, [OPPO Y>1 phase off not needed], [Lenovo], Fraunhofer IIS/HHI, [Intel, Y>1 phase off not needed], Xiaomi, AT&T, Sony, Google, [Apple] |
| --- | --- |

Type-II Doppler

| 2.2 | Proposal 2.B.1: For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following: - Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Optional features: - X=1 and the CQI is associated with: - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrix - X=2 and - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - The 2nd CQI is associated with the middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrix - FFS: Whether/how to include CQI overhead reduction for X=2 Support/fine: Samsung, Ericsson, Qualcomm, CATT, Xiaomi, Nokia/NSB, Huawei/HiSi, Lenovo, ZTE, MediaTek, Intel, Fujitsu, Apple, Qualcomm, vivo, NTT DOCOMO, Google, NEC, Fraunhofer IIS/HHI, CMCC, IDC, Sony, Spreadtrum, Sharp, |
| --- | --- |
| 2.4 | Proposal 2.D.1: For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, reuse the following components of the legacy per-coefficient quantization scheme: - Alphabets for amplitude and phase - Quantization of phase and quantization of differential amplitude relative to a reference, reference amplitude (with SCI determining the location of one reference amplitude), where the reference is defined for each layer and each “group” of coefficients Proposal 2.D.2: For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, for each layer: - One (common) SCI (Strongest Coefficient Indicator) applies across all Q selected DD basis vectors - One group comprises one polarization across all Q selected DD basis vectors (Cgroup,phase=1, Cgroup,amp=2) - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported Support/fine: Samsung, MediaTek, ZTE, OPPO, Intel, Fujitsu, Ericsson, Apple, LG, Qualcomm, vivo, CATT, Spreadtrum, Xiaomi, NTT DOCOMO, NEC, Huawei/HiSi, Google, Fraunhofer IIS/HHI, Lenovo, CMCC, IDC, Nokia/NSB, Sharp, Sony, |
| 2.3 | Conclusion 2.C.2: For the Type-II codebook refinement for high/medium velocities, for N4>1, regarding the parameter Q, there is no consensus in supporting additional candidate values FL Note: Need to be finalized on day-1 of RAN1#112 as a potential pre-requisite of Parameter Combination discussion. This is merely stating the fact ad inevitable outcome. Consensus is needed to add a new value |
|  | Proposal 2.C.3: For the Type-II codebook refinement for high/medium velocities, regarding the parameter K (the number of AP-CSI-RS resources for the CMR), optionally support only K=12 as an additional candidate value Support/fine: Huawei/HiSi, ZTE, Ericsson, MediaTek (optional), Nokia/NSB (optional), vivo, Samsung, Sony, |
|  | Conclusion 2.C.4: For the Type-II codebook refinement for high/medium velocities, regarding the parameter δ (in slots), in addition to 0 and 2, no consensus in supporting an additional candidate value FL Note: Need to be finalized on day-1 of RAN1#112 as a potential pre-requisite of Parameter Combination discussion. This is merely stating the fact ad inevitable outcome. - Consensus is needed to add a new value |
|  | Conclusion 2.C.5: For the Type-II codebook refinement for high/medium velocities, regarding the parameter N4 (length of DFT vector, unit-less), there is no consensus in supporting additional candidate values FL Note: Need to be finalized on day-1 of RAN1#112 as a potential pre-requisite of Parameter Combination discussion. This is merely stating the fact ad inevitable outcome. - Consensus is needed to add a new value |
|  | Proposal 2.C.6: For the Type-II codebook refinement for high/medium velocities, regarding the parameter d (in slots), - for P/SP-CSI-RS, support d equal to the periodicity of the CSI-RS resource - for AP-CSI-RS, also support d =1 Support/fine: vivo, Lenovo, Intel, Samsung, MediaTek, DOCOMO, NEC, ZTE, Qualcomm, CMCC, Fraunhofer IIS/HHI, Fujitsu, Nokia/NSB, Sony, Ericsson (support only 2nd bullet), Ericsson, |
|  | Proposal 2.C.1: For the Type-II codebook refinement for high/medium velocities, for N4>2 and Q>1, the selection of Q out of N4 DD basis vectors is indicated by a log2N4-1Q-1-bit indicator in CSI part 2 - Analogous to FD basis selection, DD basis index 0 (representing DC) is always selected. FL Note: This is a simple extension of FD basis selection in Rel-16 eType-II codebook (when N3<19) Support/fine: Lenovo, NTT DOCOMO, CATT, Spreadtrum, Samsung, MediaTek, Apple, ZTE, Google, Qualcomm, CMCC, Ericsson, Fujitsu, Nokia/NSB, Xiaomi, Sony, Not support: Fraunhofer IIS/HHI (want cyclic shift selection), |
| 2.5 | Proposal 2.E.2: For the Type-II codebook refinement for high/medium velocities, - The constraint on the maximum number of non-zero coefficients (NZCs) per-layer (K0) is defined jointly across all Q DD basis vectors. - Also support a constraint on the total number of non-zero coefficients (NZCs) summed across all Q DD basis vectors and across all layers: - Following the legacy specification, the maximum total number is 2K0 Support/fine: Huawei/HiSi, ZTE, Xiaomi, Intel, Samsung, MediaTek, Apple, LG, NEC, Sharp, CMCC, Fraunhofer IIS/HHI, Nokia/NSB, Sony, |
| 2.6 | Conclusion 2.F.1: On the Parameter Combination of Type-II codebook refinement for high/medium velocities, there is no consensus on including another non-UCI Doppler codebook parameter codebook as a variable in the list of supported Parameter Combinations. - Note: This implies that other non-UCI Doppler codebook parameters will be a part of RRC configuration (either explicit or implicit) FL Note: Need to be finalized on day-1 of RAN1#112 as a potential pre-requisite of Parameter Combination discussion. This is merely stating the fact ad inevitable outcome. |

### Issue 1: Type-II codebook refinement for CJT

Table 1A Summary: issue 1

| # | Issue | Companies’ views |
| --- | --- | --- |
| 1.1 | [110bis-e] Agreement On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer: - Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2) - FFS: Amplitude quantization table enhancement - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported - Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111) - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N) - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported - If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook Proposal 1.A.1: On the Type-II codebook refinement for CJT mTRP, revert the following working assumption: - Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111) - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N) - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported FL Note: Just as what we did in RAN1#110bis-e, this has to be decided based on empirical evidence (i.e. SLS results). Per agreement this needs to be concluded in this meeting. Since the WA was made conditioned upon the benefit of Alt3 over Alt1 - If there is no confirmed benefit from Alt3 over Alt1 in the alleged scenarios (inter-site CJT, 500m ISD), the WA should be reverted (hence no support of Alt3). - Otherwise, confirmed as an agreement. The available SLS results are summarized as follows for the alleged “missing” scenarios from Alt3 proponents in RAN1#110bis-e (500m ISD or larger, inter-site CJT): - “Notable” (small in FL perspective) gain: Huawei (2-3% mean UPT), ZTE (0.2-1.2% mean UPT) - No demonstrable gain: Samsung, vivo | Support/fine (want to revert WA): vivo, Samsung, OPPO, MediaTek, Fraunhofer IIS/HHI, Apple, DOCOMO, Intel, AT&T, Nokia/NSB, Ericsson, Sharp, Google, Sony, [Xiaomi], Not support (want to confirm WA): ZTE, Spreadtrum, CATT, LG, Huawei/HiSi, Lenovo, Fujitsu, NEC, |
| 1.2 | Proposal 1.B.1: On the Type-II codebook refinement for CJT mTRP, for mode-1, down select (in RAN1#112) only one from the following schemes - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FD basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources FL Note: This proposal was discussed offline [1]. - Some Alt2/3 proponents argued (albeit unfounded IMO) that Alt1 should be precluded since it is “against previous agreement”. - Some Alt1 proponents argue that Alt2/3 is not aligned with the WID since it is advertised for inter-site CJT where ideal sync/backhaul is nowhere attainable (which is true). - Based on the presented results, it is observed by the FL that all the 3 alternatives perform closely to each other in UPT vs overhead even for inter-site CJT. Current situation (based on the agreement in RAN1#111 to down select from 3 alternatives): - Alt1: MediaTek (1st), Samsung (1st), Intel (2nd), Fujitsu, Ericsson, Nokia/NSB (1st), Apple, IDC, AT&T, NTT DOCOMO, Sharp, - Alt2: ZTE, OPPO, vivo (1st), Intel, Qualcomm (2nd), LG, CATT (2nd), Xiaomi, NEC, Google, Lenovo, CMCC, Huawei/HiSi (with Minitial modified), NTT DOCOMO, - Alt3: ZTE, Huawei/HiSi, CATT, Spreadtrum, Fraunhofer IIS/HHI, - No mode-1 support: Qualcomm (1st), vivo (2nd), Samsung (2nd), Nokia/NSB (2nd), MediaTek (2nd) | Support/fine: MediaTek, Samsung, Fujitsu, Ericsson, Nokia/NSB, Apple, ZTE, OPPO, vivo, Intel, Qualcomm, LG, CATT, Xiaomi, Huawei/HiSi (Minitial modified), Spreadtrum, NTT DOCOMO, NEC, Google, CMCC, AT&T, Sharp. Not support: |
| 1.3 | Conclusion 1.C.1: On the Parameter Combination of Type-II codebook refinement for CJT mTRP, there is no consensus on adding a new (not previously agreed) codebook parameter, as well as replacing the legacy parameter L with a new (not previously agreed) parameter. - Note: Since dynamic {Ln} selection was agreed, this implies that the list of supported {Ln} combinations will be discussed separately from the list of supported {pv,} combinations - FFS: Whether/how the list of supported {Ln} combinations can be linked with the list of supported {pv,} combinations without introducing a new (not previously agreed) codebook parameter, e.g. via some UE capability Support/fine: Samsung, MediaTek, ZTE, Intel, Fujitsu, Ericsson, Apple, vivo, CATT, CMCC, NEC, NTT DOCOMO, Nokia/NSB, Lenovo, Spreadtrum, Sharp, Google, Qualcomm, Fraunhofer IIS/HHI, Xiaomi, FL Note: This proposal was discussed offline [1]. This conclusion merely states the fact and the inevitable outcome. Stating that “the conclusion is not acceptable” simply means inability to deal with the reality (albeit unpleasant). Current situation is summarized below: Separate tables for {Ln} and {pv,beta}, not adding a new parameter not previously agreed - Support/fine: Samsung, MediaTek (if resulting in fewer combos), ZTE, Intel, Fujitsu, Ericsson (2nd, with linkage), Apple, vivo, CATT, CMCC, NEC, NTT DOCOMO (with possible linkage, e.g. via Lmax constraint), Nokia/NSB, Replace L with Lmax in a single table (hence adding a new parameter not previously agreed) - Support/fine: MediaTek, OPPO, AT&T, Spreadtrum, Huawei/HiSi, IDC, Intel - Concern (against compromise in RAN1#111 and redundant): vivo, Samsung, ZTE, Nokia/NSB, CMCC, NEC, CATT, Replace L with Ltot in a single table (hence adding a new parameter not previously agreed) - Support/fine: Ericsson, LG, Qualcomm, Xiaomi, IDC - Concern (against compromise in RAN1#111 and redundant): vivo, Samsung, ZTE, Nokia/NSB, CMCC, NEC, CATT |  |
|  | Question: Please share your view on the supported value(s) of NL NL = 2: - Support/fine: Huawei/HiSi, NTT DOCOMO, Ericsson, MediaTek, vivo, OPPO, Lenovo, AT&T - Not support: NL = 4: - Support/fine: Huawei/HiSi, NTT DOCOMO, Ericsson, MediaTek, vivo, OPPO, ZTE, Lenovo, AT&T - Not support: NL = 8: - Support/fine: CATT, Ericsson, - Not support: MediaTek, Lenovo, OPPO, NL = 16: - Support/fine: CATT, Ericsson, - Not support: Huawei/HiSi, MediaTek, Lenovo, OPPO, Proposal 1.C.2: On the Type-II codebook refinement for CJT mTRP, only support NL ={2,4} as additional candidate values to NL=1. - FFS: Additional restriction(s) depending on the configured value for NTRP Support/fine: Huawei/HiSi, NTT DOCOMO, Ericsson, MediaTek, vivo, OPPO, ZTE, Lenovo, Apple, NEC, Sharp, Google, CMCC, Fujitsu, Nokia/NSB, Xiaomi, AT&T, Sony, Samsung FL Note: The support for NL=1 has been agreed in RAN1#111. Just as conclusion, this proposal merely states the fact and the inevitable outcome. Stating that “the conclusion is not acceptable” simply means inability to deal with the reality (albeit unpleasant). |  |
|  | CF. Table 1C: Question: Share your view on the supported combinations of {Ln} by inputting your view in Table 1C Proposal 1.C.3: On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {Ln} for the higher-layer-configured value of NTRP (FFS by RAN1#112: whether the bracketed permutations are also supported): - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {n} for Rel-17-based refinement are derived from the supported combinations of {Ln} for Rel-16-based refinement NTRP; {Ln} combination. 1; {2}. {4}. {6} (analogous to legacy, only for total # ports =32, rank 1-2, R=1. 2; {2,2}. {2,4}, [{4,2}]. {4,4}. 3; {2,2,2}. {2,2,4} [and its other permutations]. {4,4,4}. 4; {2,2,2,2}. {2,2,2,4} [and its other permutations]. {2,2,4,4} [and its other permutations]. {4,4,4,4}. Support/fine: ZTE, NTT DOCOMO, Samsung, Lenovo, vivo, Nokia/NSB, [Xiaomi], AT&T, FL Note: Just as conclusion, this proposal merely states the fact and the inevitable outcome given Table 1C. Stating that “the conclusion is not acceptable” simply means inability to deal with the reality (albeit unpleasant). |
|  | CF. Table 1D: Question: Share your view on the supported combinations of {pv,} by inputting your view in Table 1D Proposal 1.C.4: On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {pv,} from where the value of {pv,} is gNB-configured via higher-layer (RRC) signaling: - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {M} for Rel-17-based refinement are derived from the supported combinations of {pv ,} for Rel-16-based refinement pv for layers 1-4; ; Condition(s). {1/8, 1/8, 1/16, 1/16}; ¼; --. ½; --. {1/4, 1/4, 1/8, 1/8}; ¼ (*); --. ½ (*); --. {1/4, 1/4, 1/4, 1/4}; ¾ (*); --. {1/2, 1/2, 1/2, 1/2}; ½; - Only applicable when NTRP≤3 and NL=1 - Optional. (*) Supported by legacy Rel-16 Support/fine: [ZTE], NTT DOCOMO, [Samsung], Lenovo, vivo, AT&T, FL Note: Just as conclusion, this proposal merely states the fact and the inevitable outcome given Table 1D. Stating that “the conclusion is not acceptable” simply means inability to deal with the reality (albeit unpleasant). |
| 1.4 | Question: Re UCI omission, please share your view on how to refine the legacy Rel-16 Type-II UCI omission rule upon the addition of CSI-RS resource dimension n - Alt1. CSI-RS resource is the highest priority Prio(,l,m,n)=2Ltot.N.RI.P(m)+N.RI.l(n)+N.n - Alt2. CSI-RS resource is the 2nd highest priority (after layer): Prio(,l,m,n)=2Ltot.N.RI.P(m)+N.RI.l(n)+RI.n+ - Alt3. CSI-RS resource is the lowest priority (after FD-basis): Prio(,l,m,n)=2L’.P(n).RI.Mv+2L’.RI. P(m)+RI.l(n)+ - Alt 4. Replace SD basis index l in legacy Prio calculation with k=0n-12Lk+ln, i.e., SD basis index over all resources: Prio(,l,m,n) = 2Ltot.RI.P(m)+ RI.k=0n-12Lk+RI.l(n)+ Note: Since Ln is per CSI-RS resource, l is replaced by l(n) and L by Ltot (summed across all N CSI-RS resources) Note2: P(n) maps the index n according to whether the CSI-RS n corresponds to SCI, and L’(max value from all N CSI-RS resources) Note: Legacy priority equation is defined as follows: - Priority level definition: If priority levels of two LCCs and are such that Prio(,l2,m2)< Prio(,l1,m1), LCC has a higher priority over - Non-zero LC coefficients and bits of bitmap are prioritized/ordered from high to low priority according to (,l,m) with the same priority function Prio(,l,m) - The priority level is calculated as Prio(,l,m)=2L.RI.P(m)+RI.l+ where P(m) maps the index m according to the following order of the corresponding FD components (if selected): 0, N3-1, 1, N3-2, 2, .... Proposal 1.D.1: On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, down-select between the following two alternatives (by RAN1#112-bis where n denotes the n-th CSI-RS resource): - Alt1. Prio(,l,m,n)=( k=0n-12Lk) .N.RI.P(m)+N.RI.l(n)+N.n - Note: This implies that CSI-RS resource is designated the highest priority - Alt2. Prio(,l,m,n)=2L’.Qn).RI.N3+2L’.RI. P(m)+RI.l(n)+ - Note: This implies that CSI-RS resource is designated the lowest priority (after FD basis) - Note: L’ denotes the max value of Ln from all selected N CSI-RS resources - FFS: Q(n) maps the index n according to a rule, e.g., Q(n)=n, or Q(n)=0 if n corresponds to strongest TRP/SCI. - Alt3. Replace SD basis index l in legacy Prio calculation with k=0n-12Lk+ln, i.e., SD basis index over all resources: Prio(,l,m,n) = 2Ltot.RI.P(m)+ RI.k=0n-12Lk+RI.l(n)+ FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m Support/fine: Samsung, Lenovo, CATT, NTT DOCOMO, OPPO, MediaTek, Apple, Google, vivo, NEC, LG, Ericsson, ZTE, Spreadtrum, Qualcomm, Fujitsu, CMCC, Nokia/NSB, Xiaomi, Huawei/HiSi, Fraunhofer IIS/HHI, FL Note: | ORIGINAL (in question): Alt1: Samsung, Lenovo, CATT, NTT DOCOMO, OPPO, MediaTek, Apple, Google, CMCC, Nokia/NSB, Alt2: Alt3: ZTE, NEC(1st), Spreadtrum, Fraunhofer IIS/HHI, Huawei, HiSilicon Alt3B P(m)=m: Qualcomm Alt 4: vivo, NEC(2nd), LG, Ericsson, Xiaomi, |
| 1.5 | Question: Re CBSR, please share your view on how to refine the legacy Type-II CBSR upon the addition of CSI-RS resource dimension - Alt1. Legacy is used for each of the NTRP CSI-RS resources - Alt2. Newly designed CBSR considering all N CSI-RS resources jointly Proposal 1.E.1: On the Type-II codebook refinement for CJT mTRP, regarding CBSR, at least for restricting SD basis selection, the legacy CBSR scheme is fully reused for each of the RRC-configured NTRP CSI-RS resources (resulting in CSI-RS-resource-specific SD beam group restriction) - Joint rank restriction is applied across NTRP CSI-RS resources - Note: Assuming (N1,N2) is configured with CBSR (as in legacy), the configured (N1,N2) value is common for all CSI-RS resources per previous agreement - FFS: Whether amplitude restriction is CSI-RS-resource-common or specific, and soft vs hard restriction - FFS: Whether CBSR can be configured to be off for a CSI-RS resource FL Note: This is the current situation - Alt1: Huawei/HiSi, ZTE, Spreadtrum, LG, Xiaomi, NEC, Intel, NTT DOCOMO, Nokia/NSB, Fujitsu, - Alt2: Samsung (joint rank restriction across resources), | Support/fine: Huawei/HiSi, ZTE, Spreadtrum, LG, Xiaomi, NEC, Intel, NTT DOCOMO, Nokia/NSB, Lenovo, vivo, OPPO, MediaTek, Apple, LG, ZTE, Sharp, Google, Qualcomm (no soft restriction on amplitude), CMCC, Ericsson (same as Qualcomm), [Samsung], NEC, Fujitsu, AT&T, Sony, Not support: CATT (no CBSR for CJT) |

Table 1B Type II CJT: summary of observation from SLS

| Company | SLS results |  |  |
| --- | --- | --- | --- |
|  | Issue # | Metric | Observation |
| Huawei/HiSi | 1.2 | Mean UPT Gain vs overhead, 5% UPT Gain vs overhead | Alt2/3 outperforms Alt1 with about 2~4% gain at mean UPT and up to 8% gain at 5% UPT. |
|  | 1.3 | Mean UPT Gain vs overhead | Table 5. Selected parameter combinations for Rel-16-based CJT codebook. Index; Lmax; pv (v=1,2); pv (v=3,4); beta. 1; 6; 1/8; 1/16; 1/2. 2; 8; 1/8; 1/16; 3/4. 3; 8; 1/4; 1/8; 1/2. 4; 10; 1/8; 1/16; 1/4. 5; 10; 1/4; 1/8; 1/2. 6; 12; 1/8; 1/16; 1/2. 7; 12; 1/4; 1/8; 1/2. 8; 12; 1/2; 1/4; 1/2. 9; 14; 1/2; 1/4; ¼. Table 7. Selected parameter combinations for Rel-17-based CJT codebook Index; αmax; M; beta. 1; 1.25; 1; 0.5. 2; 1. 5; 1; 0.5. 3; 1.75; 1; 0.5. 4; 2; 1; 0.75. 5; 2.5; 1; 0.75. 6; 1.75; 2; 0.5. 7; 2; 2; 0.5. 8; 2.75; 2; 0.5. |
|  | 1.1 | Mean UPT Gain vs overhead, 5% UPT Gain vs overhead | For inter-site CJT with large inter-site distance, Alt 3 (Cgroup,amp=2N) has better performance compared to Alt1 (Cgroup,amp=2). (2% mean UPT gain) |
|  | Other issue (RX info FB) | Mean UPT Gain vs overhead, 5% UPT Gain vs overhead | The full channel feedback for CJT codebook by per-RX reporting can provide 5~10% gain for mean UPT and 18~35% gain for 5% UPT respectively. |
| ZTE | 1.2 | Avg UPT gain vs overhead, cell-edge UPT gain vs overhead | It can be observed the average and cell-edge UPT gains of Alt2 (and Alt3) over Alt1 while considering report overhead. (1% avg UPT gain) |
|  | 1.1 | Avg UPT gain vs overhead, cell-edge UPT gain vs overhead | We observe that 0.2%~1.2% mean UPT gain and 2.2%~12.1% cell edge UE gain can be achieved using Alt 3 compared with Alt1 |
|  | 1.3 | Avg UPT gain vs overhead | - For {L1, ..., LNTRP}, at least of the following entries should be supported. SD-basis comb. index; L1; L2 (if reported); L3 (if reported); L4 (if reported). 1; 2; 2. 2; 2; 2; 2. 3; 2; 2; 2; 2. 4; 4; 4. 5; 4; 4; 4. 6; 4; 4; 4; 4. 7; 6; 6. 8; 6; 6; 6. - For {pv, beta}, at least of the following entries should be supported. {pv, beta} comb. index; Pv; beta. v = {1,2}; v = {3,4}. 1; 1/8; 1/16; 1/2. 2; 1/8; 1/16; 3/4. 3; 1/4; 1/8; 1/2. 4; 1/4; 1/8; 3/4. 5; 1/4; 1/4; 3/4. 6; 1/2; 1/4; 1/2. 7; 1/2; 1/2; 1/2. 8; 1/2; 1/2; 3/4. |
|  | Other issue (RX info FB) | Avg UPT | It can be observed that, through additionally reporting Rxx information, the reporting of receiving side information can bring a significant performance gain. (10% UPT gain) |
| vivo | 1.1 | Mean SE vs payload (FULL BUFFER traffic) | Alt3 shows a negligible performance improvement over Alt1 for the scenario with 500m ISD and for the high payload case of the scenario with 200m ISD. Combining the payload and the SE gain, Alt1 outperforms Alt 3. |
|  | 1.2 | Mean SE vs payload (FULL BUFFER traffic) | There are several observations shown below for Mode 1 Wf selection schemes, - Alt 1 has slightly lower performance compared to Alt2 and Alt3 especially in high-payload region - Performance difference among the alternatives is small. - Layer-common Alt 1 has slightly lower performance compared to layer-specific Alt 1. No performance gain can be observed by introducing O3 for Alt 1 Wf selection in Mode 1. |
|  | Other issue (w.r.t. R) | Mean SE vs payload | Some performance gains can be obtained for a larger R. However, there is a large increase on PMI payload as well. The performance-overhead curve of R=4 is not superior over R=2 |
| Samsung | 1.3 | Average UPT gain vs overhead | On (pv,beta) pv, β=18,18, 18,14,18,12, 14,14, 14,12, 12,14 are good candidates to yield better UPT vs overhead trade-off performance than other combinations of pv, β. Index; pυ; β. v∈{1,2}; v∈{3,4}. 1; 1/8; 1/16; 1/8. 2; 1/8; 1/16; 1/4. 3; 1/8; 1/16; 1/2. 4; 1/4; 1/8; 1/4. 5; 1/4; 1/8; 1/2. 6; 1/2; 1/4; 1/4. On {Ln} - Ln=(4,4,2) (and its permutations) perform worse than Ln=(4,2,2), hence they are good candidates to be removed from parameter combination for Ln when NTRP=3. - Ln=(4,4,4,2) (and its permutations) perform worse than Ln=(4,4,2,2), hence they are good candidates to be removed from parameter combination for Ln when NTRP=4. |
|  | 1.2 | Average UPT gain vs overhead | - Mode 1 with Alt 2 per-TRP SVD (the advocated lower complexity benefit for Alt2) incurs ~4% UPT loss (for the same PMI overhead) over Mode 2. - Overall, Mode 2 and Mode 1 with Alt 1 and Alt 2 using joint-SVD operation yield similar performance. - Mode 1 with Alt 2 needs additional UE processing to find per-TRP FD basis vectors for the case of joint-SVD operation, compared to Mode 2 or Mode 1 with Alt 1 – thereby resulting in higher UE complexity - With TRP-common q3, Mode 1 Alt1 performs slightly better than Alt2, and Mode 1 Alt 1 and Mode 2 perform similarly. - With TRP-specific q3 (additional spec impact to be needed), the performance of the both Mode 1 Alt1 and Alt2 can be improved and they yield a small gain (~2% average UPT gain) over Mode 2. Regardless, Mode1 Alt1 and Alt2 perform similarly. |
|  | 1.1 | Average UPT gain vs overhead | - There is no benefit of Alt3 over Alt1 shown in our SLS results for both mode 1 and mode 2 cases even in the inter-site inter-cell scenarios. |
| Qualcomm | 1.2 | Average throughput gain, cell-edge throughput gain, | For Type-II-CJT, mode-2 and mode-1 CB Alt2/3 have similar performance, while mode-1 CB Alt1 can have performance loss at smaller number of selected FD bases (M). |
| MediaTek | 1.2 | Avg UPT Gain | At low to medium ISDs (500m), Alt 2 does not provide any performance benefit over Alt 1. This is because, post SVD of mTRP channel, Alt 1 can better select a common FD basis set for coherent transmission, while Alt 2 favours individual TRP precoders to be better by TRP specific FD basis selection, thereby suffering in coherent transmission performance. As the ISD becomes larger, it becomes more probable for a UE to be served by a single dominant TRP, so that Alt 2 gives a slightly better performance than Alt 1 in certain limited parameter combinations. - Considering UE implementation, feedback overhead, and commonality with Mode 2, Alt 1 is a preferred choice for Mode 1 codebook FD bases selection - FD bases selection Alt 2 does not provide a consistent performance benefit over Alt 1, even in high ISD scenarios. |
|  | 1.1 | Avg UPT Gain | Quantization Alt 3 does not provide consistent performance benefit over Alt 1, even in high ISD scenarios. |
| Nokia | 1.2 | Mean UE SE, Cell-edge SE | In terms of throughput performance, Alt 1 shows about 1.3% and 3.5% gain in mean and cell edge throughput, respectively, over Alt 2/Alt 3, when using integer offsets, i.e., no oversampling. This gain increases significantly with fractional offsets, i.e., with oversampling, and is about 19% and 47% in mean and cell edge throughput, respectively, with an oversampling factor O4=4. |

Table 1C Type II CJT: Supported combinations for {Ln} (assume Rel-16-based, Rel-17 follows per RAN1#111 agreement)

| NTRP | {Ln} combination | Company views |
| --- | --- | --- |
| 1 | {2} | Support/fine: ZTE, NTT DOCOMO, Samsung, CMCC Not support: |
|  | {4} | Support/fine: ZTE, NTT DOCOMO, Samsung, CMCC Not support: |
|  | {6} | Support/fine: ZTE, NTT DOCOMO, CMCC Not support: Samsung |
| 2 | {2,2} | Support/fine: ZTE, NTT DOCOMO, Samsung Not support: |
|  | {2,4}, {4,2} | Support/fine: NTT DOCOMO, Samsung Not support: |
|  | {2,6}, {6,2} | Support/fine: Not support: Samsung |
|  | {4,4} | Support/fine: ZTE, NTT DOCOMO, Samsung Not support: |
|  | {4,6}, {6,4} | Support/fine: Not support: Samsung |
|  | {6,6} | Support/fine: ZTE Not support: Samsung |
| 3 | {2,2,2} | Support/fine: ZTE, NTT DOCOMO, Samsung Not support: |
|  | {2,2,4} and its other permutations | Support/fine: Samsung, NTT DOCOMO Not support: |
|  | {2,2,6} and its other permutations | Support/fine: Not support: Samsung |
|  | {2,4,4} and its other permutations | Support/fine: NTT DOCOMO Not support: Samsung |
|  | {2,4,6} and its other permutations | Support/fine: Not support: Samsung |
|  | {2,6,6} and its other permutations | Support/fine: Not support: Samsung |
|  | {4,4,4} | Support/fine: ZTE, NTT DOCOMO, Samsung Not support: |
|  | {4,4,6} and its other permutations | Support/fine: Not support: Samsung |
|  | {4,6,6} and its other permutations | Support/fine: Not support: Samsung |
|  | {6,6,6} | Support/fine: ZTE Not support: Samsung |
| 4 | {2,2,2,2} | Support/fine: ZTE, NTT DOCOMO, Samsung Not support: |
|  | {2,2,2,4} and its other permutations | Support/fine: NTT DOCOMO, Samsung Not support: |
|  | {2,2,2,6} and its other permutations | Support/fine: Not support: Samsung |
|  | {2,2,4,4} and its other permutations | Support/fine: Samsung, NTT DOCOMO, Samsung Not support: |
|  | {2,2,4,6} and its other permutations | Support/fine: Not support: Samsung |
|  | {2,2,6,6} and its other permutations | Support/fine: Not support: Samsung |
|  | {2,4,4,4} and its other permutations | Support/fine: Not support: Samsung |
|  | {2,4,4,6} and its other permutations | Support/fine: Not support: Samsung |
|  | {2,4,6,6} and its other permutations | Support/fine: Not support: vivo, Samsung |
|  | {2,6,6,6} and its other permutations | Support/fine: Not support: vivo, Samsung |
|  | {4,4,4,4} | Support/fine: ZTE, Samsung Not support: |
|  | {4,4,4,6} and its other permutations | Support/fine: Not support: vivo, Samsung |
|  | {4,4,6,6} and its other permutations | Support/fine: Not support: vivo, Samsung |
|  | {4,6,6,6} and its other permutations | Support/fine: Not support: Lenovo, vivo, Samsung |
|  | {6,6,6,6} | Support/fine: Not support: Samsung, Lenovo, vivo |

Table 1D Type II CJT: Supported combinations for (pv,) (assume Rel-16-based, Rel-17 follows per RAN1#111 agreement)

| pv for layers 1-4 |  | Company views |
| --- | --- | --- |
| {1/8, 1/8, 1/16, 1/16} | 1/8 | Support/fine: Samsung, vivo Not support: ZTE |
|  | ¼ | Support/fine: Samsung, NTT DOCOMO, vivo, CMCC Not support: ZTE |
|  | ½ | Support/fine: ZTE, Samsung, NTT DOCOMO, CMCC Not support: |
|  | ¾ | Support/fine: ZTE, NTT DOCOMO, CMCC Not support: Samsung, vivo |
| {1/4, 1/4, 1/8, 1/8} | 1/8 | Support/fine: vivo Not support: Samsung, ZTE |
|  | ¼ (*) | Support/fine: Samsung, NTT DOCOMO, CMCC Not support: ZTE |
|  | ½ (*) | Support/fine: ZTE, Samsung, NTT DOCOMO, CMCC Not support: |
|  | ¾ (*) | Support/fine: ZTE, NTT DOCOMO, CMCC Not support: Samsung |
| {1/2, 1/2, 1/4, 1/4} [111] Agreement On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter pv, support the additional value of pv=1/2 for v=1,2,3,4 with the following condition: Only to be used in combination with other parameter value(s) to limit the increase in PMI overhead comparable to the maximum overhead of the legacy Rel-16/17 Type-II codebooks (exact parameter combination(s) FFS) | 1/8 | Support/fine: Not support: Samsung, ZTE |
|  | ¼ | Support/fine: Samsung Not support: ZTE |
|  | ½ (*) | Support/fine: ZTE, NTT DOCOMO, CMCC Not support: Samsung |
|  | ¾ | Support/fine: Not support: Samsung |
| {1/4, 1/4, 1/4, 1/4} | 1/8 | Support/fine: Not support: Samsung |
|  | ¼ | Support/fine: Not support: Samsung |
|  | ½ | Support/fine: Not support: Samsung |
|  | ¾ (*) | Support/fine: ZTE, NTT DOCOMO, CMCC Not support: Samsung |
| {1/2, 1/2, 1/2, 1/2} | 1/8 | Support/fine: Not support: Samsung |
|  | ¼ | Support/fine: Not support: Samsung |
|  | ½ | Support/fine: ZTE, NTT DOCOMO, Google, CMCC Not support: Samsung |
|  | ¾ | Support/fine: ZTE, Google Not support: Samsung |

(*) Supported by legacy Rel-16 Type-II

Table 1E Type II CJT: List of UCI parameters (to date)

Rel-16 based: UCI parameter list

| Parameter | UCI | Details/description | Status |
| --- | --- | --- | --- |
| # NZ coefficients | Part 1 | RI ({1,…, RIMAX}) and KNZ,TOT (the total number of non-zero coefficients summed across all the layers and all N CSI-RS resources, where KNZ,TOT {1,2,…, 2K0} are reported in UCI part 1 | Complete |
| Wideband CQI | Part 1 | Same as R15 | Complete |
| Subband CQI | Part 1 | Same as R15 | Complete |
| CSI-RS resource selection bitmap | Part 1 | NTRP-bit bitmap to indicate the UE recommendation of N CSI-RS resources - Non-existent if the value of N is RRC-configured to NTRP | Complete |
| Indication of number of SD basis vectors {L1, ..., LNTRP} | Part 1 | UE recommendation selecting one of the NL RRC-configured value combinations (log2(NL)-bit indicator) - Non-existent if NL=1 | Complete |
| N Bitmap(s) per layer | Part 2 | RI=1-2: for layer l and CSI-RS resource n, size-2LnM For RI=3-4, bitmaps, each with size-2LnMi (i=0,1,…, RI-1, where i denotes the i-th layer and n denotes the n-th CSI-RS resource) are reported in UCI part 2 | Complete |
| Strongest coefficient indicator (SCI) | Part 2 | RI=1: A log2KNZ-bit indicator for the strongest coefficient index l*,m*,n* RI>1: See Table below | Complete |
| SD basis subset selection indicator for each of the N CSI-RS resources | Part 2 | SD basis subset selection indicator is a log2N1N2Ln-bit indicator for n=0,1,…,N–1. Details follow Rel.15 | Complete |
| FD basis subset selection indicator | Part 2 | Mode-1: TBD (down-selection from 3 alternatives) Mode-2: See Table below | Mode-1 TBD Mode-2 complete |
| LC coefficients: phase | Part 2 | Quantized independently across layers | Complete |
| LC coefficients: amplitude | Part 2 | Alt1 (agreed): Quantized independently across layers (including a reference amplitude for weaker polarization, for each layer) Alt3 (WA): Quantized independently across layers (including 2N-1 reference amplitudes for 2N-1 (polarization, CSI-RS resource) pairs excluding the pair of (polarization, CSI-RS resource) associated with the SCI, for each layer) | WA on Alt3 support needs to be confirmed |
| SD oversampling (rotation) factor q1, q2 | Part 2 | Values of q1,n, q2,n follow Rel.15, reported per CSI RS resource | Complete |

| SCI and FD basis subset selection indicator |  |
| --- | --- |
| SCI for RI>1 | Per-layer SCI defined across N CSI-RS resources, where SCIi is a log22n=0N-1Ln–bit (i=0,1,…,(RI-1)) indicator. The location (index) of the strongest LC coefficient for layer i before index remapping is (li*,mi*), SCIiindicates li*, and mi* is not reported |
| Index remapping | For layer i, the index mi of each nonzero LC coefficient cli,mi is remapped with respect to mi* to mi such that mi*=0. The FD basis index kmi associated to each nonzero LC coefficient cli,mi is remapped with respect to kmi* to kmi such that kmi*=0. The sets cli,mi≠cli*,0 and kmi≠0 are reported. Informative note (for the purpose of reference procedure): The index li,mi of nonzero LC coefficients is remapped as (li,mi)→(li,mi-mi*modMi). The codebook index associated with nonzero LC coefficient index li,mi is remapped as kmi→kmi-kmi*modN3. |
| Combinatorial indicator for N3≤19 | log2N3-1Mi-1 bits |
| Combinatorial indicator for N3>19 | log2N3'-1Mi-1 bits |
| Minitial | Reported in UCI part 2, , bits |

Rel-17 based: UCI parameter list

| Parameter | UCI | Details/description | Status |
| --- | --- | --- | --- |
| # NZ coefficients | Part 1 | RI ({1,…, RIMAX}) and KNZ,TOT (the total number of non-zero coefficients summed across all the layers and all N CSI-RS resources, where KNZ,TOT {1,2,…, 2K0} are reported in UCI part 1 | Complete |
| Wideband CQI | Part 1 | Same as R15 | Complete |
| Subband CQI | Part 1 | Same as R15 | Complete |
| CSI-RS resource selection bitmap | Part 1 | NTRP-bit bitmap to indicate the UE recommendation of N CSI-RS resources - Non-existent if the value of N is RRC-configured to NTRP | Complete |
| Indication of number of SD basis vectors {L1, ..., LNTRP}, where Ln=alphan*PCSI-RS/2 | Part 1 | UE recommendation selecting one of the NL RRC-configured value combinations (log2(NL)-bit indicator) - Non-existent if NL=1 | Complete |
| N Bitmap(s) per layer | Part 2 | For layer l and CSI-RS resource n, size-2LnM, or (K1,nM where Ln=K1,n/2) | Complete |
| Strongest coefficient indicator (SCI) | Part 2 | For layer l: A log22Mn=0N-1Ln-bit indicator for the strongest coefficient index | Complete |
| SD basis subset selection indicator for each of the N CSI-RS resources | Part 2 | SD basis subset selection indicator is a log2N1N2Ln-bit indicator for n=0,1,…,N–1, where Ln=alphan*PCSI-RS/2. Details follow Rel.15 | Complete |
| FD basis subset selection indicator | Part 2 | Mode-1: TBD (down-selection from 3 alternatives) Mode-2: a log2(N-1) bit indicator only if N>M=2, where N∈{2,4} is configured with the higher-layer parameter valueOfN, when M=2. | Mode-1 TBD Mode-2 complete |
| LC coefficients: phase | Part 2 | Quantized independently across layers | Complete |
| LC coefficients: amplitude | Part 2 | Alt1 (agreed): Quantized independently across layers (including a reference amplitude for weaker polarization, for each layer) Alt3 (WA): Quantized independently across layers (including 2N-1 reference amplitudes for 2N-1 (polarization, CSI-RS resource) pairs excluding the pair of (polarization, CSI-RS resource) associated with the SCI, for each layer) | WA on Alt3 support needs to be confirmed |

Table 2 Additional inputs: issue 1

| Company | Input |
| --- | --- |
| Mod V0 | Please share your inputs on each of the issues and, if applicable, proposals in TABLE 1A Also check TABLE 1A/B/C/D/E and update your views |
| ZTE | Initial input: Correct our preference for Issue 1.4 by adding Alt 3. To report the entire CSI for dominant TRP firstly (e.g., a CSI-RS n corresponding to strongest TRP/SCI has P(n)=0) in order to guarantee MU-MIMO performance for a TRP firstly, and then the rest TRPs (one by one). Otherwise, we may have insufficient DD-bases for each of serving TRPs while performing ZF/SLNR precoding refinement in MU-MIMO. Some further input for our preference for Table 1D. |
| Lenovo | Issue 1.1: Support the working assumption Issue 1.2: Support Alt2 of Proposal 1.B.1 Issue 1.3: Agree with Conclusion 1.C.1. Based on RAN1#111 agreement, since the network would configure NL combinations of number of beams, it would be very hard to associate each of the NL combinations with a distinct (pv, β) combination value. Instead, we propose to associate (pv, β) with Ltot, so as to ensure the overall complexity is managed (Note: this does not imply reporting Ltot, and is only used for configuration combination selection). We also support Proposal Conclusion 1.C.2. Given that the UE is expected to select N out of NTRP TRPs after the configuration with the NL beam number combinations, the UE may need to check up to NL.(2NTRP-1) hypotheses of joint beam/TRP combinations. Our preference is not to support NL=8,16 Issue 1.4: We prefer Alt1 Issue 1.5: We support Proposal 1.E.1, the same problem was discussed in Rel-17 NCJT discussions and it was concluded to support the legacy design per resource. We don’t see clear motivation to deviate from that for CJT |
| CATT | Issue 1.1: Support to confirm the working assumption. The gain of results from Huawei and ZTE is sufficient to support the Alt3 in addition to Alt1. Issue 1.B.1: Alt1 shall be precluded first as it contradicts the agreed definition of mode-1, that FD basis are independently selected for different TRPs. [Mod: This is not a common understanding and IMO Alt1 is a viable option for mode-1] “No mode-1 support” shall not be an option of this discussion as we have agreed to support both mode-1 and mode-2. [Mod: If companies cannot agree on the scheme for mode-1, no support for mode-1 becomes a natural outcome. This is nothing new in RAN1] Issue 1.3: As long as the candidate value of NL is agreed, all combinations of {Ln} can be supported. We don't have to explicitly give the list of supported {Ln}. It is up to gNB to configure the combinations. Regarding the UE capability, the total number of spatial beams across all TRPs or maximum number of spatial beams across all TRPs can be considered to address the concern of UE complexity during UE feature discussion. Issue 1.4: Support Alt1. Issue 1.5: CBSR is not very useful for C-JT operation. Even though the beam direction of each TRP is restricted, it is difficult to tell what is finally restricted after coherent combining from multiple TRPs. Do not support CBSR for C-JT is also an option. |
| NTT DOCOMO | Conclusion 1.C.1 FL captured our view well. Yes, we’d like to consider some restriction on possible linkage configuration between {Ln} and (pv, β) with separate tables. Proposal 1.C.2 Support. Table 1C/1D Our preference added. Issue 1.4 We prefer Alt1. |
| vivo | Issue 1.1 Support to revert the WA. At least results from three companies (vivo, Samsung and MTK) suggest no gain is observed for Alt 3. Issue 1.2 Support Alt 2. - Based on multiple companies’ evaluation, Alt 1 does not provide gain over Alt 2. In some cases, Alt 1 even has a bit performance loss. - We also observe that Mode 2 generally gives higher gain than Mode 1. Issue 1.3 Added our preference. - We have concern to have too large L_tot, as it requires high UE complexity. - We are wondering whether it is needed to mention permutation for Ln values, as how to configure CSI-RS resources to match the Ln values is totally up to gNB’s implementation. Issue 1.4 We added a new alternative. In our understanding, since the difference among different CSI-RS resources is mainly SD basis selection. Hence the most straight-forward extension of legacy priority rules is to handle CSI-RS resources in SD basis vectors, i.e., to replace SD basis vector l in legacy Pri calculation with k=0n-12Ln+ln, i.e., SD basis index over all CSI-RS resources. Issue 1.5 Support Alt 1. |
| OPPO | Issue 1.1 1.2: support. Issue 1.3: We can be fine with the conclusion though we think RRC parameter Lmax is a better solution. However, to address the issue of UE complexity with large value of Ltot, the maximum number of beams across all CSI-RS resources Lmax should be reported via UE capability. Regarding the value of N_L, We support the values of 2,4 considering UE complexity. Issue 1.4: Prefer Alt.1. Issue 1.5: Fine with the proposal 1.E.1. |
| Spreadtrum | Issue 1.1: Since there are multiple companies show that performance gain of Alt3 can be observed, we prefer to support Alt3. Issue 1.2: Our first preference is Alt3 from previous agreement. Now that Alt3 is ruled out due to lack of majority support, we can accept Alt2. For Alt1, we have similar view as CATT that Alt1 is not valid. In our understanding, for mode 1, ‘independent FD basis selection across N TRP’ means that the selection procedure is independent, rather than the distribution of selected FD basis. Issue 1.3: We are fine with Conclusion 1.C.1. In addition, the linkage between the list of supported {Ln} combinations and the list of supported {pv,} combinations should be carefully designed considering UE complexity. Issue 1.4: We think TRP level UCI omission can be considered to achieve additional TRP selection (the metric is according to reporting overhead rather than channel quality). So we prefer Alt3 added by ZTE in principle. The definition of P(n) can be further discussed. Issue 1.5: Support Alt1 for simplicity. |
| Samsung | Issue 1.1 Support to revert the WA. In our view, since the SCI is across TRPs, coefficients associated with different TRPs need to be normalized with respect to the strongest coefficient across all TRPs, implying that the W2 coefficients after normalization in some sense shall already capture the power level of different TRPs. This is regardless of the scenarios or ISD values. We have verified this via simulation results from the previous meeting, wherein we show that there is no gain (and additional overhead/complexity) with Alt3 over Alt1 in inter-site scenarios with small (200m) as well as large (500m) ISDs. Issue 1.2 Support Alt1 only, and can’t accept Alt2/3, if Alt1 is precluded, we rather prefer not to support mode 1, due to the following technical reasons: - Performance: Mode 1 with any alternative (Alt1/2/3) is worse than Alt2. The performance can be improved and brought closer to mode 2 (if joint SVD is performed), but mode 2 remains superior. This observation is valid for all scenarios of interests agreed in EVM (including inter-cell, inter-site with large ISDs). - Complexity: Alt2/3 has higher UE complexity than Alt1 due to multiple vs one Wf calculation. - Overhead: Alt2/3 incurs more overhead Alt1 - Among Alt1/2/3, Alt1 is the only alternative which makes sense technically. - Among mode1/2, mode 2 is sufficient. If mode 1 has to be supported, we can only accept Alt1, i.e., the best among the three alts for mode 1. Otherwise, we can live without mode 1. - Final point: the WID assumes perfect synchronization among TRPs. Strictly speaking, we are not sure we can even discuss Alt2/3 since it assumes otherwise. Issue 1.3: OK with conclusion 1.C.1 Re N_L value: - Ntrp=1: N_L = 1 is sufficient (just like legacy). Why would a gNB configure multiple N_L values, e.g. L=2 and 6 to a UE? [Mod: Added FFS on further restriction] - Ntrp>1: we prefer to keep it as small as possible, but at the same time, we don’t prefer to sacrifice the UPT performance. Also, this value can be discussed after the {Ln} is agreed, because then only we will know how many candidate combinations are supported. - But, we are OK to discuss/list the candidate values We don’t support any {Ln} combination including 6, due to either requiring large UE computational complexity (large-size of SVD is needed)/huge feedback overhead or allowing {Ln} value configurations, which could be ineffective in performance. We select best candidates of (pv, beta)= 18,18, 18,14,18,12, 14,14, 14,12, 12,14 considering - the overhead regime of interest (<= 1000 bits) - good UPT-vs-overhead performance for any NTRP=1,…,4 and Ln combinations Hence, we support those (pv, beta) combinations. Also, we suggest to prune out Ln=(4,4,2) (and its permutations) for NTRP=3 and Ln=(4,4,4,2) (and its permutations) for NTRP=4 due to worse performance than other combinations of Ln. Issue 1.5 Our initial view on CBSR is that at least rank should be jointly restricted (TRP-common). Re the beam group + amplitude restriction based on legacy, we should be careful about the max overhead, which is as follows. - Vector group restriction: B1 = 11 bits For per-TRP restriction, max 44 bits (for 4 TRPs) - Amplitude restriction: B2 = 2N1N2 per vector group 8N1N2 for 4 groups. For per-TRP restriction, max 512 bits (for 4 TRPs). This overhead is very large (4x of legacy). The 44 bits for vector group restriction is perhaps OK, but 512 bits for amplitude restriction is too much and perhaps is not needed. Besides, amplitude restriction is an optional feature even in legacy due to overhead (max 128 bits). So, we think amplitude restriction can be TRP common. In summary, we prefer - RI and amplitude restrictions: TRP common - Beam group restriction: per TRP |
| MediaTek | Issue 1.1 Support to revert the WA. As shown in our and number of other companies results we do not see any performance benefits from Alt 3. Issue 1.2 We support Alt 1 only any other Alt will result in more complex codebook design without any performance benefits. Agree with the comments made by Samsung that it seems we will be diverging from the WID problem statement once we start going Alt 2/3 solution. Issue 1.3 Conclusion 1.C.1: We are fine in principle; we strongly believe we should go for the solution which can allows for flexible UE capability reporting, as all the parameters mentioned in the table can have direct impact on UE complexity. Proposal 1.C.2: Support Issue 1.4 Support Alt. 1 Issue 1.5 Proposal 1.E.1: We believe at least RI restriction should be considered jointly across TRPs. |
| Apple | Issue 1.1 One compromise could be to make Alt1 basic feature. Issue 1.2 If we need to choose between Alt1 and Alt2, Alt 1 is our preference. Issue 1.3 We are fine with Proposal 1.C.2 Issue 1.4 Alt1 Issue 1.5 We are fine with Proposal 1.E.1 in principle, but we think Proposal 1.E.1 needs some further clarification, i.e., independent CBSR configuration or common CBSR. |
| LG | Issue 1.1 Do not support FL’s proposal. In case of MTRP scheme, cell edge gain should be focused. Issue 1.3 Linking {Ln} and {p, beta} via UE capability is one possible approach, but using Ltot instead of L to indicate {p, beta} combination is straight-forward extension of legacy and we prefer to use Ltot. It is true that this parameter is not agreed previously but it was discussed in UE’s {Ln} reporting sense, so here Ltot should be discussed separately from previous discussion. Issue 1.4 Support Alt. 4. Issue 1.5 We also have similar view with MTK. Common RI restriction should be applied across TRPs. |
| ZTE | Issue 1.1 Support to confirm the WA. Based on our evaluation result, we observe performance gain of Alt3 over Alt1, especially for cell-edge UE. Technically speaking, - Alt3 can provide a more appropriated amplitude reference for each TRP, and as a consequently, under a given K0, both accuracy and the number of available NZC(s) can be increased clearly. - On the other case, although Alt1 is to provide polarization-specific reference for NZC amplitude, we may have the opposite observation for dual polarization for different TRP(s). - For instance, for a strongest TRP, NZC corresponding to +45-degrees polarization may have a strongest amplitude compared with -45 degree, but for other TRPs, NZC amplitude compared with -45 degree polarization may be stronger. It means that, if using Alt1, the quantization error may be much more severe than legacy case of single TRP. For moving forward this issue, we may be open to have Alt3 only for mode-1 (dedicated for inter-site case). Then, another compromise one is raised by Apple. Issue 1.2 Support. We prefer Alt2 based on our observation from SLS evaluation results. Regarding introducing fractional/oversampling parameters for Alt1, we observe marginal performance gains from our simulation results. Issue 1.3 Support. We prefer to have NL=4 from spec perspective. The maximum number of total L for N_TRP CSI-RS can be up to UE capability. Issue 1.4 Support Alt3. Specifically speaking, we prefer to add the TRP-dimension after FD-dimension, as the most significant dimension. - NZ coefficient(s) corresponding to the main/primary TRP provide the dominant power(s). Once CSI omission occur, we still can perform a subset of serving TRPs well. Then, the rest TRPs can be used for serving other UEs, like soft-switching. Issue 1.5 Support Proposal 1.E.1. Table 1C/1E: We think the following criterion should be considered while selecting parameter combination: - The low-bound in UPT performance in CJT mTRP should be greater than that in the worst case in sTRP eTypeII, i.e., PC1. - The upper bound of bit overhead in CJT CSI is ~3000 bit considering the typical payload of UCI in PUSCH; - The average bit overhead of CJT-CSI rather than worst case should be considered for a given parameter set, due to large variation of report overhead in terms of UE-assisted TRP/{L1, ..., LNTRP} selection and rank distribution |
| NEC | Issue 1.1: We prefer to confirm the working assumption. It’s around 50% that weaker polarizations are different for different TRPs, so applying one reference amplitude for same polarization across TRPs (Alt 1) is not suitable for cases when the power gap is large. If companies think CJT gain mainly derives from the case when power difference is small across polarizations/TRPs, then we think the reference amplitude quantization table can be updated (the FFS part in previous agreement “FFS: Amplitude quantization table enhancement”), for example, no need of 3 bits to cover (0 ~ -21dB). 1 or 2 bits may be sufficient, as the differential amplitude coefficient is sufficient to reflect the amplitude difference. Proposal 1.B.1: Support the proposal, and Alt 2 preferred. Conclusion 1.C.1: Support, and we also support Ln combinations linked with list of {pv, beta}. Conclusion 1.C.2: Support. Issue 1.4: We also prefer to prioritize CSI-RS associated with SCI firstly, and we are also fine with Alt 4 added by vivo. Our position added in the table. Issue 1.5: Firstly, we’d like to clarify that RI restriction will be separately discussed or covered by the proposal? Considering UE determined TRP selection and Ln combination, we also prefer at least RI restriction should consider TRP combinations, for example, single-TRP hypothesis and multi-TRP hypothesis which is similar as NCJT. Another possible case is that for multi-TRP hypothesis and with larger total value of Ln, higher rank may be restricted due to large overhead, while for single-TRP or smaller total value of Ln, the restriction may not be needed. And we support CBSR for CJT. In addition, considering UE determined TRP selection, the power limitation may be different for different hypothesis, at least for single-TRP and multi-TRP hypothesis. |
| vivo2 | Correct a typo in Alt 4 of Issue 1.4. |
| Sharp | Issue 1.1: We support proposal 1.A.1 Issue 1.2: We prefer Aly. 1 Issue 1.3: We are OK with the conclusion 1.C.1 and proposal 1.C.2. Issue 1.5: We are fine with proposal 1.E.1. |
| Google | Issue 1.1: OK with current proposal. Issue 1.2: OK with current proposal. Issue 1.3: OK with the conclusion, and our view on the parameters were provided in the table. Issue 1.4: Support Alt1. Issue 1.5: Support current proposal |
| Qualcomm | Issue 1.2 Fine with Proposal 1.B.1 Maybe this issue has been discussed quite a bit, but still want to express our view: 1. Theoretical-wise, we understand the motivation of Alt1 as, to exploit TRP-relative delay with the FD index offset, but we failed to find an algorithm with physical meaning to do it – since FD selection is after SVD (as agreed layer-specific), the offset no longer represent TRP-relative delay 2. UPT v.s. overhead perspective, - For larger M, mode-2 and mode-1 can have similar UPT (under same K0) – mode-2 can work well without a need of FD offset - For smaller M (e.g. M=2), where mode-2 v.s. mode-1 is more controversial regarding distributed TRP scenario with larger ISD, the overhead of Alt1 v.s. Alt2 is marginal - E.g. for N3=13 and M=2, offset need 4 bits, while log2N3M=7 bits – marginal difference as only 3 bits Issue 1.3 We don’t have strong view regarding limiting NL value. According to current trend to discuss {Ln}-combo as part of ParamCombo, we think this is more like a standard-effort issue, rather than a UE-complexity issue. Our original view was, to simply order the power of all beams across all TRPs, and only define {Ltot,pv,β} in standard with no NL limitation, and no need to exhaustively scanning all {{Ln}+NTRP}-combos, which is with the least standard efforts. Besides, I also doubt whether it is possible to scan exhansively (not limited to 20MHz with 52 PRBs, not limited to certain deployment, consider this feature’s future usage) But given the current situation, we can compromise to Conclusion 1.C.1 In another aspect, we do have opinion on the value of n=1NLn – this has much higher impact than NL regarding UE-complexity, in our view. Besides, large n=1NLn may bring too large UCI overhead, or result in very small M (e.g. M=1 via small pv=1/16) if we try to reduce the overhead – note that M=1 can be just wideband PMI and not meaningful for Type-II. Will provide more specific view on limiting the value of n=1NLn probably in a later round. Issue 1.4 In our view, since W2 coefficients are only divided into two groups (group 1 and 2 in CSI part 2), only the lowest-priority dimension (outermost indexing) is meaningful Therefore, we think Alt1/2/4 does not differ much – all these three Alts use (permuted) FD as lowest priority - Although, Alt1/2 seems a little weird, by using TRP-dimension (n, an intermediate SD) inner than SD itself (l) We mainly have some implementation concern regarding FD permutation. According to our info, both infra and UE product people suffered much to get aligned for Rel-16 FD permutation – not workable for the first few rounds of testing. Since for Alt3, FD permutation is not meaningful, our preference is Alt3’: Alt3 without FD permutation Issue 1.5 We prefer no soft restriction for W2 power: Alt1 without soft CBSR Rank restriction across TRPs is OK (fine with Alt2 for rank restriction) |
| CMCC | Issue 1.2: Support Alt2 as the SLS results of Alt 1 don’t shown enough performance gain over Alt2, even some sources shown some performance loss. Issue 1.3: Support Proposal 1.C.2 and add our preference in Table 1C and 1D. Issue 1.4: Prefer Alt1. Issue 1.5: Support Proposal 1.E.1. Since the selection of N out NTRP is determined by UE, new CBSR cross all the N resources may not be easy to design. |
| Ericsson | Issue 1.4 We prefer Alt 4. As mentioned by vivo, this is the simplest extension as we can reuse the existing formula and just change l in legacy Pri calculation with k=0n-12Ln+ln. For Alt 1 and Alt 2, since they both have the CSI-RS index (i.e., TRP index) n, how should the CSI-RSs be indexed? Is it the correct understanding that the strongest TRP corresponds to n=0, the next strongest TRP corresponds to n=1, …, and the weekest TRP corresponds to n=3? we did a small calculation of the priority function with N3 =10, Rank =2, Mv = 2, Ln = 2, and N=2. Our calculations are in the excel sheet. It seems Alt 3 results in some duplicate priority values for different rows (we have colored the duplicated priority values). Could other companies verify these calculations? If this is verified, then Alt 3 may have some problem. Issue 1.5 We are ok with using legacy approach for each of the NTRP CSI-RS resources. But we have a question about soft amplitude restriction. In Rel-16 eType-II CB, soft beam restriction is applied where a threshold can be configured for a beam such that its associated coefficients amplitudes average over all selected FD basis vectors does not exceed the threshold. The average amplitude is with respect to the strongest coefficient. This makes sense for single TRP. In CJT scenario, the strongest coefficient and the restricted beams can be associated to different TRPs and the different TRPs don’t share power. In the example below, beam ‘d’ is the strongest beam in TRP1. In this example, the practical use of restricting beam ‘a’ in TRP2 with respect to the strongest coefficient in TRP 1 is unclear. Hence, we don’t think soft amplitude restriction is needed. We support using legacy approach for each of the NTRP CSI-RS resources with hard restriction only. |
| Mod V23 | Revised proposals based on comments and added new proposals |
| Fraunhofer IIS/HHI | Issue 1.1: Support to revert the WA. Issue 1.2: Technically, Alt 2/Alt 3 is a better solution for Mode-1. As shown by many companies, the throughput-overhead trade-off is better with Alt 2/Alt 3. Mode 1 and Mode 2 have been supported for inter-site scenarios and intra-site scenarios, respectively. Therefore, the discussion shall be aimed at finding a right solution for the Wf reporting rather than focusing on the discussion to not support Mode-1. [Mod: We have discussed this for 3-4 meetings and interested companies don’t seem to converge. That’s the reality. If you really think you can do a much better as a moderator on this topic, please feel free to start an offline amongst interested companies and let me know the outcome :-)] Issue 1.3: Fine with the conclusion. Proposal 1.D.1: We think TRP index should have lower priority. If omission occurs, gNB can get full information of the precoder coefficients associated with at least a subset of TRPs with Alt 1, whereas with Alt 1, gNB can only get a subset of precoder coefficients of all TRPs which may result in an insufficient CJT transmission. Therefore, we prefer Alt 2. |
| Fujitsu | Issue 1.1: We still prefer to confirm the working assumption, but we accept Alt 3 is an optional feature. Issue 1.2: Fine with the proposal. Issue 1.3: In our opinion, a predefined table without RRC configuration is enough for Ln combination. In addition, gNB can match the CSI-RS of each TRP with different L by its implement, so NL=2,4 is sufficient instead of considering the order of TRP. Issue 1.4: Support. Issue 1.5: We prefer Alt 1. Table 1E Regarding the reporting on number of NZC in Part I, there is no conclusion on reporting scheme for KNZ. The agreement we only achieved is that one constraint 2K0 on the total number of NZCs summed across TRPs and layers. Thus, our suggestion is to update the table as follow, Parameter; UCI; Details/description; Status. # NZ coefficients; Part 1; RI ({1,…, RIMAX}) and, Alt 1: KNZ,TOT (the total number of non-zero coefficients summed across all the layers and all N CSI-RS resources, where KNZ,TOT {1,2,…, 2K0} are reported in UCI part 1 Alt 2: KNZ,i (the total number of non-zero coefficients summed across all the layers for each CSI-RS resource, where KNZ,TOT {1,2,…, 2K0} are reported in UCI part 1; Complete TBD. [Mod: We have 2 agreements on this issue: [110bis-e] Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, the constraint on the maximum number of non-zero coefficients (NZCs) per-layer (K0) is defined jointly across all N CSI-RS resources - TBD: the constraint on the total number of NZCs across all layers [110bis-e] Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, also support a constraint on the total number of non-zero coefficients (NZCs) summed across all layers: - Following the legacy specification, the maximum total number is 2K0 From the above 2 agreements it’s quite clear the natural implication is what you call Alt1 (already captured in Table 1E). This is no longer a TBD issue. I hope we don’t need to discuss every natural implication of preious agreements ] |
| Intel | Issue 1.1: We are fine with the proposal to revert the WA. Issue 1.2: We are OK with the proposal as a one step further. Issue 1.3: It is hard to discuss the number of combinations (NL) without understanding of the total number of selected SD basis vectors (LTOT). LTOT defines the UE complexity as well as the number of possible combinations. If LTOT is small (e.g. equal to 6) then we can support indication of all the combinations. So, in our view it would be better to agree the LTOT at least as assumption for RAN1 discussion (i.e. it doesn’t mean that LTOT will be explicitly introduced in RAN1 spec or as RRC parameter). [Mod: This was actually discussed in the last meeting and the proponents of semi-static Ln couldn’t accept this Ltot/Lmax even for discussion purposes The conclusion is an inevitable outcome.] Issue 1.4: We don’t have strong view at this point, further discussion is needed. We can agree on the list of alternatives for further discussion. To simplify, we can omit some alternatives. Issue 1.5: Support the proposal, except RI restriction. RI restriction should be joint since there is only one RI for CJT. [Mod: Joint RI restriction is already included ] |
| Nokia/NSB | Proposal 1.C.2 Support Proposal 1.C.3 Fine Proposal 1.D.1 Fine. Our preference is Alt 1 for simplicity |
| Fraunhofer IIS/HHI | Issue 1.2: Agreement For the Rel-18 Type-II codebook for CJT mTRP, support the following two modes: - Mode 1: Per-TRP/TRP-group SD/FD basis selection which allows independent FD basis selection across N TRPs / TRP groups. Example formulation (N = number of TRPs or TRP groups): - Mode 2: Per-TRP/TRP group (port-group or resource) SD basis selection and joint/common (across N TRPs) FD basis selection. Example formulation (N = number of TRPs or TRP groups): - Striving for the two modes to share commonality in detailed designs such as parameter combinations, basis selection, TRP (group) selection, reference amplitude, W2 quantization schemes. - FFS: Depending on the decision on SCI design, whether additional per-TRP/TRP-group amplitude scaling and/or co-phase is needed or not, and whether they are a part of W2s Agreement For the Rel-18 Type-II codebook for CJT mTRP based on the Rel-16 Type-II codebook, SD basis and FD basis are separate, each fully reusing the legacy Rel-16 DFT-based design If consensus is not reached for Wf reporting for mode-1 (Proposal 1.B.1), then based on the above two agreements, shouldn’t the legacy based independent FD basis selection be the default solution? [Mod: No, since there are 3 alternatives agreed to be studied and there is no agreement on “default”. I appreciate your sudden interest in this topic and your trying to “advise” the moderator. I await your own evaluation … in the meantime check my previous response on your previous comment (scroll up)] |
| Xiaomi | Proposal 1.A.1 Our first preference is to support Alt 3 for the case of large amplitude difference among MTRPs. But if the performance gain of Alt 3 is negligible, We can accept this proposal. Proposal 1.B.1 Support and prefer Alt 2 Conclusion 1.C.1 Support Proposal 1.C.2 Support it since small number of combinations can reduce UE complexity. Proposal 1.C.3 We prefer to not support much larger Ltot or large difference among L for different TRPs. Proposal 1.D.1: Support. We share similar with vivo and Ericsson, and Alt3 is preferred. Proposal 1.E.1 Support. |
| Mod V31 | No change in content to proposals. @ALL: please keep in mind that a conclusion of no consensus is based on a fact that there is no consensus. Therefore, the inevitable outcome is that the feature is not supported. Whether you support or don’t support (accept or cannot accept) the conclusion doesn’t change the fact and reality. I hope this utterly simple principle is crystal clear to everyone. |
| AT&T | Proposal 1.A.1: Support to revert the WA. Proposal 1.B.1: Support Alt1 (preference) & Alt2 Conclusion 1.C.1: If we look at the {Ln} Table (Issue 1.3) and add the parameter of Lmax/Ltot n=1NLn≤Lmax, n=1NLn=Ltot. Then use Lmax/Ltot as a linkage parameter between the {Ln} and the β/pv tables, we think this a better way to link between the two tables (similar to Huawei/HiSi, R1-2205881) since it is consistent with legacy, Lmax/Ltot is proportional to the NZC, can reduce redundancy, & can better optimize the combinations. We did not agree on the parameters Lmax/Ltot in RAN1#111, however this is a separate discussion on how to link between the{Ln}and the β/pv tables. NTRP; {Ln} combination; Lmax/Ltot. 1; {2}. {4}. {6} (analogous to legacy, only for total # ports ≥32. 2; {2,2}; 4. {2,4},{4,2}; 6. {4,4}; 8. 3; {2,2,2}; 6. {2,2,4},{2,4,2},{4,2,2}; 8. {4,4,4}; 12. 4; {2,2,2,2}; 8. {2,2,2,4},{2,2,4,2},{2,4,2,2},{4,2,2,2}; 10. {2,2,4,4},{2,4,2,4},{2,4,4,2},{4,4,2,2},{4,2,4,2},{4,2,2,4}; 12. {4,4,4,4}; 16. Lmax/Ltot; pv for layers 1-4; . {1/8, 1/8, 1/16, 1/16}; ¼. ½. {1/4, 1/4, 1/8, 1/8}; ¼ (*). ½ (*). {1/2, 1/2, 1/4, 1/4}; ½ (*). {1/4, 1/4, 1/4, 1/4}; ¾ (*). {1/2, 1/2, 1/2, 1/2}; ½. [Mod: Linkage will be discussed later. As of now the conclusion is the best we can do to progress. Please check the situation and read my comment Mod V31. I promise we will discuss linkage once we complete the tables for Ln and (pv,beta)] Proposal 1.C.2: Support Proposal 1.C.3: we are fine Proposal 1.C.4: we are fine Proposal 1.E.1: we are fine |
| ZTE | Proposal 1.C.4: Our concern on 1st and 3rd combinations is much relevant to performance gains over sTRP. Although we do a lot of homework about that, we still hardly identify that. Then, for progress, we can live with having the whole candidates in the table be approved directly, as a compromise. From our evaluation, we do not identify report overhead issues at least for up to N_TRP=3, even for Pv=1/2 for all rank 1~4. [Mod: Thanks for being flexible, let’s wait for Samsung to respond] Proposal 1.D.1: Thanks for good comments from E///. Good catch. There is a typical for Alt2, 2L’.P(n).RI.N3. Then, if my understanding is correct, QC’s comment is much relevant to P(n). Agree, for avoiding ambiguities, we may use a different function title, like Q(n). Based on above, we have the following suggestion for update. - Alt2. Prio(,l,m,n)=2L’.Q(n).RI.N3+2L’.RI. P(m)+RI.l(n)+ - Note: This implies that CSI-RS resource is designated the lowest priority (after FD basis) - Note: L’ denotes the max value of Ln from all selected N CSI-RS resources - FFS: Q(n) maps the index n according to a rule, e.g., Q(n)=n, or Q(n)=0 if n corresponds to strongest TRP/SCI. [Mod: OK] Then, regarding Alt1 and Al3, if our understanding is correct, if having only two subsets of bitmap and NZCs as usual, the performance of above may be quite similar due to the fact that both of them are just to omit a half of the bitmap and NZCs corresponding to higher value of P(m), i.e., FD-bases. The difference for Alt2 is to remove the half corresponding to TRPs with lowest power. |
| Huawei, HiSilicon | Issue 1.2. For the following arguments, we don’t think ideal sync-backhaul can result in the same delay profiles between TRPs, they are different things. For different locations of TRPs (D-MIMO or inter-site cells), even they have ideal backhaul and sync, there’s high probability that they have different delay profiles to UEs. In such cases, Alt 2/3 can better represent the channel better and have better performances. - Some Alt1 proponents argue that Alt2/3 is not aligned with the WID since it is advertised for inter-site CJT where ideal sync/backhaul is nowhere attainable (which is true). Issue 1.3, we believe some linkage between Ln combinations and {pv, β} is needed, and the Ltot or Lmax is a better way to reduce the table size and save efforts on discussion of parameter combinations. [Mod: Check Mod V31 comment and the FFS of the proposal and my comment to AT&T. We will discuss linkage once the tables are complete.] We support proposal 1.C.2, and we have no strong view on NL=8, open to further discuss it considering performance, flexibility and UE complexity. We don’t support proposal 1.C.3, we believe Ln=6 should also be included for N_TRP={2, 3, 4}, especially for large #CSI-RS ports. Otherwise, there’s possibility CJT performance may even be worse than single TRP transmission. From the previous comments, it is Ltot which affects the complexity, instead of Ln. For example, {6,2,2} with Ltot=10 can have lower UE complexity than {4,4,4} with Ltot =12. Therefore, we don’t see the reason to remove Ln = 6 for CJT as legacy single-TRP has had it. It is more reasonable to restrict the value of Ltot/Lmax. And gNB can configure the NL {Ln} combinations for candidate values {2,4,6} within the Ltot/Lmax restriction. [Mod: Read the proposal again. It says “at least”. Meaning we can still discuss whether other combinations are adopted. This is the best we can do considering that the previous agreement on supporting dynamic Ln selection makes the codebook much more convoluted and complex.] Issue 1.4. For proposal 1.D.1, we prefer Alt 3 that the CJT CSI can fallback to single-TRP transmission with UCI omission. Similar comments as Ericsson and ZTE, from the range of P(m) below from legacy priority equation, it seems the equation for Alt 3 should be as following. Prio(,l,m,n)=2L’.P(n).RI.N3Mv+2L’.RI. P(m)+RI.l(n)+ The priority level is calculated as Prio(,l,m)=2L.RI.P(m)+RI.l+ where P(m) maps the index m according to the following order of the corresponding FD components (if selected): 0, N3-1, 1, N3-2, 2, .... Issue 1.5. For proposal 1.E.1, if we “fully reuse” legacy CBSR scheme, does it mean that gNB also has the flexibility to not configure CBSR for a certain CSI-RS resource? We think this should also be supported since with multi-TRP cooperating, some TRP may not have strong interference to neighbour TRPs as it’s also serving UEs in neighbour TRPs. [Mod: Good point. I tend to agree that it should be supported as well. Let’s hear from other companies.] |
| Mod V36 | Some edits. Repeating myself since what I said on V31 doesn’t seem to go through for some … @ALL: please keep in mind that a conclusion of no consensus is based on a fact that there is no consensus. Therefore, the inevitable outcome is that the feature is not supported. Whether you support or don’t support (accept or cannot accept) the conclusion doesn’t change the fact and reality. I hope this utterly simple principle is crystal clear to everyone. |
| Qualcomm | Issue 1.4 Thanks Ericsson for the good catch, and thanks ZTE, Huawei and FL for some fix work. Indeed FD permutation is a main concern from our side, and I bet this can be a lengthy discussion, e.g. for TRP-common or TRP-specific FD index remapping, if Rel-16-analogous FD permutation is kept :) Therefore, we’d like to add one FFS to proposal 1.D.1 FFS FD permutation P( ) as Rel-16-analogous, or no permutation i.e. P(m)=m. BTW, to me, Alt1 of proposal 1.D.1 is like we additionally have SD permutation/interlacing, besides FD [Mod: OK] |
| Mod V42 | Some edits per QC comment Repeating myself since what I said on V31 doesn’t seem to go through for some … @ALL: please keep in mind that a conclusion of no consensus is based on a fact that there is no consensus. Therefore, the inevitable outcome is that the feature is not supported. Whether you support or don’t support (accept or cannot accept) the conclusion doesn’t change the fact and reality. I hope this utterly simple principle is crystal clear to everyone. |
| Qualcomm | Thanks FL for the quick response to Proposal 1.D.1 One minor additional request: Can we have the new added FFS applying to all 3 Alts, not only under Alt2? Thanks [Mod: Done] |
| Sony | Issue 1.1: We are okay to revert WA. Issue 1.3: Okay to support proposal 1.C.2. Issue 1.5: Can support proposal 1.E.1. |
| Samsung | Proposal 1.C.2: since our suggestion “N_L=1 for N_trp=1” is FFS. We should make it clear by adding the red text as shown bullet. Proposal 1.C.2: On the Type-II codebook refinement for CJT mTRP, only support NL ={2,4} as additional candidate values to NL=1. - Note: NL=2,4 is applicable only for NTRP >1 - FFS: whether NL=2 is applicable for NTRP =1 - FFS: Additional restriction(s) depending on the configured value for NTRP [Mod: This is not needed since this proposal is only about the values of N_L. In fact, the note I added earlier (to accommodate your earlier comment) is not needed either. The last FFS is sufficient. So I will not add your suggested bullet and will also remove the note – since it states the obvious (we don’t need to state all the obvious facts in notes)] Proposal 1.C.3: - Re 3rd combination (N_TRP=1 and {6}), we can accept it for progress since it is supported in legacy. However, there seems to be a typo, it should be “=” since we don’t have >32 port CSI-RS. Besides, there are 2 other conditions in legacy, as shown below. - “(analogous to legacy, only for total # ports ≥ =32, rank 1-2, and R=1” [Mod: Thanks for being flexible. OK since it’s the same as legacy] Proposal 1.C.4: re the last 3 combinations C5, C6, C7, - We repeat our concern on overhead. Since the UPT vs overhead trade-off is the metric agreed in EVM, we should definitely maximize the performance, but at the same time keep the overhead as small as possible so that it can fit into the allocated UL resource for CSI report. Because otherwise (overhead is so high that the CSI can’t be fit into the allocated resources) UCI omission will occur too frequently, considering CSI report required from multiple UEs for MU-MIMO, which we don’t want since UCI omission is an emergency scheme for addressing a “rare event” not to address a “frequent event”. For the 7 candidates , the overhead of bitmap + 2K0 NZ coefficients is shown below is shown below (for Ln=4 NSB=19). Considering max Rel.16 overhead (which is around 1000), we can observe that the last three combinations exceed Rel16 overhead bounds for all Ntrp values, and the 4th combination exceeds Rel16 overhead bounds for Ntrp=3,4. - We don’t need both C5 and C7 (considering they are identical for rank1-2, which is mandatory, note rank 3-4 is optional in legacy). We suggest to remove C5 and keep C7, since C7 perhaps is supported strongly by more companies. - So, we suggest to - remove C5 - reduce overhead at least for C7 by restricting NTRP≤3 and NL=1. Besides, this combination is optional Proposal 1.C.4: On the Type-II codebook refinement for CJT mTRP, support at least the following combinations of {pv,}: - FFS by RAN1#112: whether other combinations can be supported - For C7, restrict the overhead by restricting NTRP≤3 and NL=1. Besides, this combination is optional Ntrp=2; Ntrp=3; Ntrp=4. pv for layers 1-4; ; ; ; rank 2; rank 4; rank 2; rank 4. C1; {1/8, 1/8, 1/16, 1/16}; ¼; 264; 240; 396; 360; 528; 480. C2; ½; 432; 352; 648; 528; 864; 704. C3; {1/4, 1/4, 1/8, 1/8}; ¼ (*); 440; 360; 660; 540; 880; 720. C4; ½ (*); 720; 528; 1080; 792; 1440; 1056. C5; {1/2, 1/2, 1/4, 1/4}; ½ (*); 1440; 880; 2160; 1320; 2880; 1760. C6; {1/4, 1/4, 1/4, 1/4}; ¾ (*); 1000; 1160; 1500; 1740; 2000; 2320. C7; {1/2, 1/2, 1/2, 1/2}; ½; 1440; 1760; 2160; 2640; 2880; 3520. [Mod: Thanks for being flexible. OK] Proposal 1.E.1 - One issue we need to discuss is (N1,N2) value. In legacy CBSR, (N1,N2) is configured jointly with CBSR. If (N1,N2) is the same (common) for TRPs, and CBSR is TRP-specific, then we need to include this a restriction, i.e., (N1,N2) is the same for TRPs. How can be FFS. - Minor comment on wording, since legacy CBSR restricts 4 beam groups and (optionally) amplitude value for each beams, we suggest clarify it, as shown below in red. - Re HW question “does it mean that gNB also has the flexibility to not configure CBSR for a certain CSI-RS resource?” our understanding is yes, gNB may or may not configure CBSR for each CSI-RS resource independently. Proposal 1.E.1: On the Type-II codebook refinement for CJT mTRP, regarding CBSR, at least for restricting SD basis selection beam groups, the legacy CBSR scheme is fully reused for each of the RRC-configured NTRP CSI-RS resources (resulting in CSI-RS-resource-specific SD basis selection beam group restriction) - Joint rank restriction is applied across NTRP CSI-RS resources - Assuming (N1,N2) is configured jointly with CBSR (as in legacy), then configured (N1,N2) value is the same for all CSI-RS resources. FFS: details - FFS: Whether amplitude restriction is CSI-RS-resource-common or specific, and soft vs hard restriction [Mod: We have agreed that (N1,N2) is common for all CSI-RS resources. Strictly speaking this is not needed, but it’s fine to add since (N1,N2) is inside the CBSR IE – just to avoid future contradiction] |
| Mod V48 | Revised some proposals (per Qualcomm’s comment and Samsung’s compromise) |
| Mod V53 | No change other than editorial clarification in 1.C.4 that (pv,beta) is higher-layer configured from the supported combinations, and Rel-16+Rel-17 based relationship (proposals 1.C.3 and 1.C.4) |
| Mod V56 | Minor change adding FFS (from some offline comments) re Rel-16 vs Rel-17 for proposals 1.C.3 and 1.C.4 |
| Samsung | In our revised tdoc uploaded yesterday, we further analyze the performance of Mode 1 Alt1/Alt2 by optimizing the oversampling factor O3 for FD basis selection. The new results consider the cases of TRP-common q3 for Mode 1 and TRP-specific q3 for Mode 1, respectively. Note that TRP-common q3 for Mode 1 can be spec-transparently implemented but TRP-specific q3 for Mode 1 needs additional spec impact. As seen in the SLS results, we can make the additional observations as follows: Observation A1: - With TRP-common q3, Mode 1 Alt1 performs slightly better than Alt2, and Mode 1 Alt 1 and Mode 2 perform similarly. - With TRP-specific q3 (additional spec impact to be needed), the performance of the both Mode 1 Alt1 and Alt2 can be improved and they yield a small gain (~2% average UPT gain) over Mode 2. Regardless, Mode1 Alt1 and Alt2 perform similarly. The new results additionally support our previous argument on Mode 1 Alt1. Figure 1: Average UPT gain vs overhead w.r.t. different FD basis selection offset methods, i.e., Mode 1 with Alt1 and Alt2, and Mode 2 in inter-site inter-cell scenario, TRP-common q3 and O3=4 Figure 2: Average UPT gain vs overhead w.r.t. different FD basis selection offset methods, i.e., Mode 1 with Alt1 and Alt2, and Mode 2 in inter-site inter-cell scenario, TRP-specific q3 and O3=4 Also, we have added new observations in Table 1B for your reference. |
| Mod V61 | No change |

### Issue 2: Type-II codebook refinement for high/medium UE velocities (with time/Doppler-domain compression)

Table 3A Summary: issue 2

| # | Issue | Companies’ views |
| --- | --- | --- |
| 2.1 | [109-e] Agreement The work scope of Type-II codebook refinement for high/medium velocities includes refinement of the following codebooks, based on a common design framework: - Rel-16 eType-II regular codebook - Rel-17 FeType-II port selection (PS) codebook FFS: Whether to prioritize/down-select from the two Proposal 2.A: The Rel-18 Type-II codebook refinement for high/medium velocities comprises refinement of the following codebooks: - Refinement of the Rel-16 eType-II regular codebook - Refinement of the Rel-17 FeType-II port selection (PS) codebook, based on the same design details as the Refinement of the Rel-16 eType-II regular codebook, except for the supported set of parameter combinations - Time-/Doppler-domain reciprocity is not assumed Conclusion 2.A.2: For the Rel-18 Type-II codebook refinement for high/medium velocities, there is no consensus on supporting the refinement based on the Rel-17 FeType-II port selection (PS) codebook. Therefore, only the refinement based on the Rel-16 eType-II regular codebook is supported. FL Note: This proposal has been discussed since RAN1#110 and needs conclusion. Since many companies voice concern on supporting the refinement based on Rel-17 FeType-II PS, conclusion 2.A.2, merely stating the fact, is inevitable. | Down-select to only Rel-16 eType-II or Rel-16 with higher priority: Apple, DOCOMO, MediaTek, ZTE, NEC, Xiaomi, Samsung, Lenovo, Intel (if Rel-17, no DD reciprocity), Xiaomi. Qualcomm, Apple, DOCOMO, Ericsson, Nokia/NSB, LG, Spreadtrum, CMCC, vivo, OPPO, Google, Sharp, Sony, Proposal 2.A: - Support/fine: Huawei/HiSi, Fraunhofer IIS/HHI - Concern on Rel-17: vivo, LG, NTT DOCOMO, Spreadtrum, Qualcomm, Lenovo, Nokia/NSB, OPPO, Google, Xiaomi, |
| 2.2 | [111] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, down-select (by RAN1#112) one from the following alternatives: - Alt1. The CQI is associated with the entire duration of the CSI reporting window and all the N4 W2 matrices - Alt2A. The CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Alt2B. The CQI is associated with the first/earliest d slots of the CSI reporting window and the first/earliest one of the N4 W2 matrices Note: The N4 W2 matrices represent the combining coefficients before DD compression at the UE, or after DD de-compression at the gNB [111] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, decide by RAN1#112 whether including X>1 CQIs in one sub-band and one CSI reporting instance are supported - If supported, also decide the value(s) of X and the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots) Proposal 2.B.1: For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following: - Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Optional features: - X=1 and the CQI is associated with: - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrix - X=2 and - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - The 2nd CQI is associated with the middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrix - FFS: Whether/how to include CQI overhead reduction for X=2 FL Note: This proposal has been discussed OFFLINE [1] Current situation is summarized as follows: Add X=2: - Support: Samsung, Ericsson, Qualcomm (only 2A, optional), CATT, Xiaomi, Nokia/NSB, Huawei/HiSi, Lenovo, Fraunhofer IIS/HHI, NEC, CMCC, - Not support: ZTE, MediaTek, Intel, Fujitsu, Apple, Qualcomm, vivo, OPPO Slot/W2 ref: - Alt1: Samsung, OPPO, Ericsson, Xiaomi, Nokia/NSB, Huawei/HiSi, Lenovo, Sharp - Alt2A: ZTE, Intel, Fujitsu, LG, Qualcomm, vivo, OPPO, CMCC - Alt2B: Intel | Support/fine: Samsung, Ericsson, Qualcomm, CATT, Xiaomi, Nokia/NSB, Huawei/HiSi, Lenovo, ZTE, MediaTek, Intel, Fujitsu, Apple, Qualcomm, vivo, NTT DOCOMO, Google, NEC, Fraunhofer IIS/HHI, CMCC (tend to agree with Google), IDC, Sony, Spreadtrum, Sharp, Not support: |
| 2.3 | CF. Table 3C on DD basis selection indicator Proposal 2.C.1: For the Type-II codebook refinement for high/medium velocities, for N4>2 and Q>1, the selection of Q out of N4 DD basis vectors is indicated by a log2N4-1Q-1-bit indicator in CSI part 2 - Analogous to FD basis selection, DD basis index 0 (representing DC) is always selected. FL Note: This is a simple extension of FD basis selection in Rel-16 eType-II codebook (when N3<19) | Support/fine: Lenovo, NTT DOCOMO, CATT, Spreadtrum, Samsung, MediaTek, Apple, ZTE, Google, Qualcomm, CMCC, Ericsson, Fujitsu, Nokia/NSB, Xiaomi, Sony, Not support: Fraunhofer IIS/HHI (want cyclic shift selection), |
|  | [111] Agreement For the Type-II codebook refinement for high/medium velocities, for N4>1, regarding the parameter Q, at least Q=2 is supported. - FFS: Whether Q=3 and/or Q=4 are also supported as other candidate value(s), as well as the supported Parameter Combination(s) Conclusion 2.C.2: For the Type-II codebook refinement for high/medium velocities, for N4>1, regarding the parameter Q, there is no consensus in supporting additional candidate values FL Note: Need to be finalized on day-1 of RAN1#112 as a potential pre-requisite of Parameter Combination discussion. This is merely stating the fact ad inevitable outcome. - Consensus is needed to add a new value | Also support Q=3: - Support/fine: ZTE, Fraunhofer IIS/HHI, CATT, NEC, Ericsson, Huawei, HiSilicon - Not support: Samsung, MediaTek Also support Q=4: - Support/fine: ZTE, Lenovo, NEC, Fraunhofer IIS/HHI, Huawei, HiSilicon - Not support: Samsung, MediaTek |
|  | [111] Agreement For the Type-II codebook refinement for high/medium velocities, the parameter K (the number of AP-CSI-RS resources for the CMR) is gNB-configured via higher-layer (RRC) signalling at least from the following set of candidate values: {4, 8} - FFS: If additional candidate value(s) of K are supported, e.g. 5, 12, 16, also taking into account other use cases (e.g. for training filter coefficients, prediction or performance monitoring) and TDD Proposal 2.C.3: For the Type-II codebook refinement for high/medium velocities, regarding the parameter K (the number of AP-CSI-RS resources for the CMR), optionally support only K=12 as an additional candidate value Support/fine: Huawei/HiSi, ZTE, Ericsson, MediaTek (optional), Nokia/NSB (optional), vivo, Samsung, Sony, Not support: FL Note: Need to be finalized on day-1 of RAN1#112 as a potential pre-requisite of Parameter Combination discussion. This is merely stating the fact ad inevitable outcome. - Consensus is needed to add a new value | Also support K=5: - Support/fine: ZTE - Not support: Xiaomi, Samsung, Also support K=10: - Support/fine: Huawei/HiSi - Not support: Samsung, Also support K=12: - Support/fine: Huawei/HiSi, ZTE, Ericsson, MediaTek (optional), Nokia/NSB (optional), vivo, Samsung - Not support: Also support K=16: - Support/fine: - Not support: Xiaomi, Samsung, MediaTek |
|  | [111] Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter δ (in slots), support the additional value of 2 - FFS (by RAN1#112): For the last supported additional value, down select between 1, 3, 4, and 5 Conclusion 2.C.4: For the Type-II codebook refinement for high/medium velocities, regarding the parameter δ (in slots), in addition to 0 and 2, no consensus in supporting an additional candidate value FL Note: Need to be finalized on day-1 of RAN1#112 as a potential pre-requisite of Parameter Combination discussion. This is merely stating the fact ad inevitable outcome. - Consensus is needed to add a new value | Also support =1: - Support/fine: Intel, Samsung, MediaTek, Ericsson, - Not support: CATT, Fraunhofer IIS/HHI, Also support  =3: - Support/fine: Xiaomi, MediaTek, - Not support: Samsung, Fraunhofer IIS/HHI, Also support  =4: - Support/fine: vivo, Xiaomi, CATT, - Not support: Samsung, Fraunhofer IIS/HHI, Also support =5: - Support/fine: ZTE, Intel, - Not support: Samsung, Fraunhofer IIS/HHI, |
|  | [111] Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter N4 (length of DFT vector, unit-less), support 8 as an additional candidate value - FFS (by RAN1#112): Whether any of the following additional candidate values are supported: 3, 5, 16, 32 - The candidate values supported by UE are reported via UE capability (details can be discussed in UE feature). Conclusion 2.C.5: For the Type-II codebook refinement for high/medium velocities, regarding the parameter N4 (length of DFT vector, unit-less), there is no consensus in supporting additional candidate values FL Note: Need to be finalized on day-1 of RAN1#112 as a potential pre-requisite of Parameter Combination discussion. This is merely stating the fact ad inevitable outcome. - Consensus is needed to add a new value | Also support N4=3: - Support/fine: - Not support: ZTE, Xiaomi, CATT, Intel, Also support N4=5: - Support/fine: ZTE - Not support: Xiaomi, CATT, Intel, Also support N4=16: - Support/fine: Fraunhofer IIS/HHI, NEC, Samsung, - Not support: ZTE, Xiaomi, CATT, Intel, MediaTek Also support N4=32: - Support/fine: NEC, - Not support: ZTE, CATT, Intel, MediaTek |
|  | [111] Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter d (in slots), - Support at least the following candidate value: - If the configured CMR is AP-CSI-RS, this candidate value is the configured value of m parameter - FFS: If the configured CMR is P/SP-CSI-RS - FFS: Whether in the above two cases, the number of slots between the last CSI-RS occasion no later than the legacy reference resource and the starting of WCSI window shall be integer multiples of d slots. - FFS: Whether additional candidate value(s) of d are supported, e.g. d<m, d>m, d=1 If more than one candidate values of d are supported, the value of d is gNB-configured via higher-layer (RRC) signalling Proposal 2.C.6: For the Type-II codebook refinement for high/medium velocities, regarding the parameter d (in slots), - for P/SP-CSI-RS, support d equal to the periodicity of the CSI-RS resource - for AP-CSI-RS, also support d =1 Support/fine: vivo, Lenovo, Intel, Samsung, MediaTek, DOCOMO, NEC, ZTE, Qualcomm, CMCC, Fraunhofer IIS/HHI, Fujitsu, Nokia/NSB, Sony, Ericsson (support only 2nd bullet), Ericsson, Not support: FL Note: Need to be finalized on day-1 of RAN1#112 as a potential pre-requisite of Parameter Combination discussion - Consensus is needed to add a new value | For AP-CSI-RS, also support d<m: - Support/fine: - Not support: For AP-CSI-RS, also support d>m: - Support/fine: - Not support: For AP-CSI-RS, also support d=1: - Support/fine: ZTE, Samsung, Ericsson, - Not support: Xiaomi For P/SP-CSI-RS, support d=periodicity - Support/fine: vivo, Lenovo, Xiaomi, Intel, Samsung, MediaTek, CMCC, NTT DOCOMO, NEC, Ericsson - Not support: For P/SP-CSI-RS, support other values than d=periodicity - Support/fine: Ericsson (1,2), - Not support: Samsung |
| 2.4 | CF. Table 3C on LC coefficients and SCI Proposal 2.D.1: For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, reuse the following components of the legacy per-coefficient quantization scheme: - Alphabets for amplitude and phase - Quantization of phase and quantization of differential amplitude relative to a reference, reference amplitude (with SCI determining the location of one reference amplitude), where the reference is defined for each layer and each “group” of coefficients Proposal 2.D.2: For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, for each layer: - One (common) SCI (Strongest Coefficient Indicator) applies across all Q selected DD basis vectors - One group comprises one polarization across all Q selected DD basis vectors (Cgroup,phase=1, Cgroup,amp=2) - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported FL Note: The above proposals were proposed in RAN1#111 but some companies (e.g. Fraunhofer, Intel, Lenovo) requested more time to look into this. This issue needs to be concluded in this meeting. The above proposals have also been discussed OFFLINE [1] | Proposal 2.D.1: - Support/fine: Samsung, MediaTek, ZTE, OPPO, Intel, Fujitsu, Ericsson, Apple, LG, Qualcomm, vivo, CATT, Spreadtrum, Xiaomi, NTT DOCOMO, NEC, Huawei/HiSi, Google, Fraunhofer IIS/HHI, Lenovo, CMCC, IDC, Nokia/NSB, Sharp, Sony, - Not support: Proposal 2.D.2: - Support/fine: Samsung, MediaTek, ZTE, OPPO, Intel, Fujitsu, Ericsson, Apple, LG, Qualcomm, vivo, CATT, Spreadtrum, Xiaomi, NTT DOCOMO, NEC, Huawei/HiSi, Google, Fraunhofer IIS/HHI, Lenovo, CMCC, IDC, Nokia/NSB, Sharp, Sony, - Not support: |
| 2.5 | CF. Table 3C on bitmap Proposal 2.E.1: For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select one from the following alternatives: - Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv - Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector. - Alt4. A bitmap that includes bits associated with the set of {(f1, s1, t1)} with d(f1, s1, t1)<=d, where d is the threshold that can be configured by gNB, df1, s1,t1= minf1-f0, Mv-f1-f0+ mins1-s0, L – s1-s0+ mint1-t0, Q – t1-t0. s0, f0 and t0 denotes a reference SD basis index and a reference FD basis index and a reference DD basis index associated with SCI, respectively. FL Note: The above proposal has also been discussed OFFLINE [1]. This is the current situation: - Alt1: Samsung, MediaTek, ZTE, OPPO, Intel, Fujitsu, Apple, LG, CATT, Spreadtrum, Nokia/NSB, Google, CMCC, IDC, NEC (1st) - Alt2: -- - Alt3A: MediaTek, Fraunhofer IIS/HHI, OPPO, Ericsson, Qualcomm, CATT (2nd), Xiaomi, Huawei/HiSi, Lenovo, NEC (2nd) - Alt3B: MediaTek, - Alt3C: Qualcomm, - Alt4: vivo, | Support/fine: Samsung, MediaTek, ZTE, OPPO, Intel, Fujitsu, Ericsson, Apple, LG, CATT, Spreadtrum, Nokia/NSB, Lenovo, Google, Fraunhofer IIS/HHI, Qualcomm, Xiaomi, NTT DOCOMO, NEC (after Q is finalized), Huawei/HiSi, Fraunhofer IIS/HHI, CMCC, IDC, Sharp, Sony, vivo, Not support: |
|  | CF. Table 3C on the constraint on total number of NZCs Proposal 2.E.2: For the Type-II codebook refinement for high/medium velocities, - The constraint on the maximum number of non-zero coefficients (NZCs) per-layer (K0) is defined jointly across all Q DD basis vectors. - Also support a constraint on the total number of non-zero coefficients (NZCs) summed across all Q DD basis vectors and across all layers: - Following the legacy specification, the maximum total number is 2K0 FL Note: The proposal is analogous (in principle) to the one agreed for CJT. There is no reason to adopt a different principle. | Support/fine: Huawei/HiSi, ZTE, Xiaomi, Intel, Samsung, MediaTek, Apple, LG, NEC, Sharp, CMCC, Fraunhofer IIS/HHI, Nokia/NSB, Sony, Not support: |
| 2.6 | Question: Re Parameter Combination, should any of the new non-UCI Doppler codebook parameters be included as a variable in the list of supported Parameter Combinations (in addition to L, pv, and )? If so, which one? - If a new non-UCI Doppler codebook parameter is not included as a variable in the list of supported Parameter Combinations, it will be a part of RRC configuration (either explicit or implicit) - If there is no consensus in adding new parameter(s), the default would be “no” Conclusion 2.F.1: On the Parameter Combination of Type-II codebook refinement for high/medium velocities, there is no consensus on including another non-UCI Doppler codebook parameter codebook as a variable in the list of supported Parameter Combinations. - Note: This implies that other non-UCI Doppler codebook parameters will be a part of RRC configuration (either explicit or implicit) FL Note: Need to be finalized on day-1 of RAN1#112 as a potential pre-requisite of Parameter Combination discussion. This is merely stating the fact ad inevitable outcome. | No (default): Samsung (Q is separate) , ZTE, MediaTek, LG, Google, CMCC, Ericsson, Nokia/NSB, vivo, Yes: Huawei/HiSi (Q), Lenovo (Q) |

Table 3B Type II Doppler: summary of observation from SLS

| Company | SLS results |  |  |
| --- | --- | --- | --- |
|  | Issue # | Metric | Observation |
| Issue # 2.1 |  |  |  |
| Issue # 2.2 |  |  |  |
| Huawei/HiSi | X>1 | CDF of CQI overhead | - Observation 13: Reporting CQIs with nonuniform intervals has much lower overhead than reporting CQIs with uniform intervals. |
| ZTE | X=2, CQI ref | SLS: UPT | - Average gain of UTP is shown in Figure 5. Then, based on evaluation results, some limited gains of other CQI schemes can be observed compared with Alt2A |
| OPPO | X=2, CQI ref | SLS: UPT vs overhead | Observation 1: - X=1 CQI corresponded to the first and last slot is as good as Alt1 with X=1. - X=2 CQIs corresponded to the first and middle slot does not show clear gain over one CQI corresponded to the first slot. - X=2 CQIs corresponded to the first and second half of reporting window show clear gain over one CQI corresponded to the first slot. |
| Vivo | X>1, CQI ref | SLS: UPT | - it is observed that there is marginal throughput gain for Alt1 and X>1 CQIs comparing to Alt 2A |
| Fraunhofer IIS/HHI | X>1 | SLS: UPT | - In our evaluations, it has been found that for small WCSI values, a single CQI is sufficient, whereas for larger WCSI values as shown in Table 1, at least 2 two CQI values are required |
| CATT | X>1, CQI ref | SLS: UPT | Observation-1: - 2 CQIs in a CSI report have some performance gain compared with 1 CQI. - More than 2 CQIs in a CSI report have marginal performance gain compared with 2 CQIs. |
| Intel | X>1 | SLS: UPT vs overhead | Observation 3: - Up to 1% for average UE throughput and up to 5% for cell-edge UE throughput performance gains can be observed for X = 4 CQIs comparing to X = 1 CQI |
| Samsung | X>1, CQI Ref | SLS: UPT vs overhead | Observation 11: - Alt1 outperforms Alt2A and Alt2B, i.e., achieves up to 2% and 2.5% avg. UPT gain, respectively. - Alt1 outperforms Alt2A and Alt2B regardless whether OLLA is ON or OFF - Among the three alternatives {1st, middle, last} for one slot, last slot is the best, i.e., achieves ~up to 2% avg. UPT gain over 1st or middle slots. - Among the three alternatives {(1st, middle), (middle, last), (1st, last)} for two slots, (middle, last) or (1st, last) is better than (1st, middle), i.e., achieve ~up to 3% avg. UPT gain over (1st, middle). Observation 12: X=2 CQIs achieves better UPT vs overhead trade-off than X=1 CQI, i.e. up to 1.5%% gain in avg. UPT |
| Qualcomm | X>1, CQI ref | SLS: UPT | Observation 2: For Type-II-Doppler, more future CQI report (with Alt1) does not bring UPT gain (over Alt2A), under MMSE-based channel extrapolation. Observation 3: For Type-II-Doppler, CQI defined with entire WCSI (Alt1) has the highest UE complexity regardless of X=1 or X>1. |
| Ericsson | X>1, CQI ref | SLS: UPT vs overhead | - We see minor to no difference between Alt2A/B in our interpretation as both methods results in that only the first W_2 is used in the CQI computation - As seen in the results, Alt1 clearly shows improved mean UTP gain for the ideal predictor for N_4 ≥ 8 and already at N_4≥ 4 for cell-edge UTP (10-30% gain). Compared to the ideal channel predictor, using a simple auto-regressive (AR) predictor (described in detail in an earlier contribution) with K=8, m=1, results in much lower gains in general, while it is still clear that Alt1 yields better performance than Alt2A - As seen in the table, there are potential throughput gains, up to 10-20% at cell-edge, with X= 2 or 4 compared to the agreed value of X=1. Note that the increased CQI overhead is not included in the mean CSI overhead gains presented in the table |
| MediaTek | X>1, CQI ref | SLS: UPT | - Observation 1: For association of X=1 CQI, Alt 2A/2B can provide up to 1% UPT gain over Alt 1. - Observation 3: The avg. UPT gain considering X=2 and X=4 CQIs is 1~1.5 % over X=1, Alt 1. The UPT gain is <1 % considering Alt 2A/B. |
| Issue # 2.3 |  |  |  |
| Huawei/HiSi | K, m | SLS: UPT | - Observation 11: For both R17 FeTypeII codebook and R16 eTypeII codebook, it can be observed that with larger value of K, the performance gain will be larger. - Observation 12: For different values of m with same values of CSI-RS resource and CSI feedback period, it can be observed that smaller value of m decreases performance gain compared with larger value of m |
| ZTE | Q | SLS: UPT vs overhead | - In terms of average UPT, the cases of Q=3, 4 and 5 can bring significant performance gains over the baseline of Q=2. The cases of Q=3, 4 can achieve a good trade-off between throughput performance and report overhead as shown in red box - Then, based on evaluation results, some limited gains of other CQI schemes can be observed compared with Alt2A |
| Vivo | Delta | NMSE vs delay | - Assuming parameter N4=8 for reporting window is indicated in the current case, the predictive accuracy of the last four PMIs cannot be guaranteed in above figure, it will cause a negative impact on the first four slots with good prediction performance when CSI compression is carried out based on all of N4=8 PMIs. |
| Fraunhofer IIS/HHI | Q | SLS: UPT | - Observation 1: The throughput gain of the enhanced Type II CB increases with increasing number of DD components for different WCSI values |
| CATT | Q, N4 | SLS: UPT | - Based on the simulation results, there is almost no throughput gain of Q=4 over Q=3. Comparing Q=3 and Q=2, it is observed that Q=3 has 2.07% gain at 10km/h UE velocity. - Comparing the average throughput of different N_4 with Rel-16 under same overhead, it is observed that N_4=8 can achieve best throughput gain. There is no throughput gain when N_4=16, because the accuracy of channel prediction deteriorates when N_4 becomes large |
| Intel | d | SLS: UPT | Observation 1: - The performance gains observed for decreased DD unit size d = P_RS/2 comparing to DD unit size equal to the CSI-RS periodicity d = P_RS are not significant |
| Samsung | Q | SLS: UPT vs overhead | Observation 13: - UPT vs overhead trade-off for Q>2 is worse than that for Q=2 - Weak coefficients increase overhead, but don’t provide UPT gain ( beta can be small) |
| Ericsson | Q, d, K | SLS: UPT, overhead | - Q=3 provides performance gains over Q=2 across different prediction windows - In particular, [𝑁4=16, d=1] has ~4% higher mean UTP than [𝑁4=8, d=2] and ~12 % higher than [𝑁4=4, d=4] for the same level of CSI OH. It is also clear that d=1 (red) yields higher throughout than d=2 (yellow) or d=4 (purple) for similar level of CSI OH in general - Based on the results above, we see that there are some potential gains for the parameter combinations [K,m] = [12,1] and [K,m] = [12,2]. |
| MediaTek | N4, d, Q | SLS: UPT vs overehad | - Observation 5: For the same N_4, reducing the value of d reduces CSI reporting window size, thereby increasing feedback overhead. - Observation 6: For the same W_CSI, reducing the value of d increases the amount of DD compression, thereby degrading performance. - Observation 7: Q=3,4 can provide an avg. UPT gain of 3~4 % compared with Q=2, however, the feedback overhead incurred increases by 30~50 %. |
| Nokia/NSB | d,N4 | SLS: UPT | We may notice that for high speed at 30km/h we must choose T_(CSI-RS)= 5ms which means a very dense P/SP-CSI-RS scheme. Nonetheless the gain is remarkably high, i.e., 9% for mean UE Thrp, 26% for UE cell-edge Thrp vs. ZoH scheme. On the other hand, for the configurations with 10 and 20km/h we have relaxed the T_(CSI-RS) with 20 and 10ms respectively. However, as we may observe, the attainable gain is highly impacted by the time length of T_(CSI-RS) and W_CSI. |
| Issue # 2.4 |  |  |  |
| Issue # 2.5 |  |  |  |
| Huawei/HiSi | bitmap | SLS: UPT, overhead (separate analysis) | - Observation 9: With similar performance, Alt 3A, Alt3B, Alt3C has 37.5%, 25% and 25% reduction of bitmap size, respectively, compared with Alt 1 for R16 eTypeII codebook. - Observation 10: With similar performance, Alt 3A, Alt3B, Alt3C has 62.5%, 17% and 17% reduction of bitmap size, respectively, compared with Alt 1 for R17 FeTypeII codebook |
| ZTE | 2K0 constraint | SLS: UPT vs overhead | - It can observe that, Alt1b outperforms Alt1a, and the performance gain can be improved with the increase of report overhead - Alt1a. the number of NZCs is upper bounded per DD basis vectors - Alt1b. the number of NZCs is upper bounded across all DD basis vectors |
| OPPO | Bitmap | SLS: UPT vs overhead | - Observation 2: Alt3A can save 10% overhead without UPT loss for Q=2. |
| Vivo | Bitmap | SLS: UPT vs overhead | - For NZC bitmap design of Type II Doppler CSI, Alt 4 achieves almost same performance (<1% gap) as Alt 1 with considerable overhead reduction (20-40 bits). |
| Fraunhofer IIS/HHI | Bitmap | SLS: UPT vs overhead | - Observation 5: For larger number of SD-DD/SD-FD pairs. Alt 3B/3C results in a similar throughput to that of Alt 1, however with only a minor feedback reduction. - Observation 6: For L = 4 and M =4, Alt 3A results in 24 bits and 56 bits overhead reduction per layer for Q = 2 and Q = 3 DD components, respectively with performance close to that of Alt 1. |
| CATT | Bitmap | SLS: UPT vs overhead | - Based on the simulation results, it is observed that Alt3A has best performance compared with Alt3B and Alt3C under same bitmap overhead - When Q=2, the bitmap overhead of Alt1 is 64bits. The bitmap overhead of Alt3A with S=4 is 40bits which incurs 37.5% overhead reduction, and the average throughput has only 0.58% loss compared with Alt1. Therefore, S=4 is enough for Q=2. When Q=3, the bitmap overhead of Alt1 is 96bits. The bitmap overhead of Alt3A with S=6 is 60bits which incurs 37.5% overhead reduction, and the average throughput has only 1.3% loss compared with Alt1. Therefore, S=6 can be used when Q=3 |
| Intel | Bitmap | SLS: UPT vs overhead | Observation 2: - Performance degradation of up to 0.8% in average UE throughput and up to 2% for cell-edge UE throughput is observed for Alt3A comparing to Alt1. - 48 bits can be saved for configurations with M = 4 and 84 bits for configuration with M = 7 for Alt3A comparing to Alt1 |
| Samsung | Bitmap | SLS: UPT vs overhead | Observation 14: there is no noticeable difference between Alt1 and Alt3A/3B/3C for the bitmap; - Alt1 with lower β value (i.e., μβ) than legacy can be used to achieve similar performance vs overhead as Alt3A/3B/3C with legacy β - When compared with Alt1, - there is no gain with Alt3A/3B/3C in high overhead regime (>400 bits), and - there is very small (<1%) gain with Alt3A/3B/3C in low overhead regime (<400 bits) |
| Qualcomm | Bitmap | SLS: UPT, overhead | Observation 4: For Type-II-Doppler, 2-stage (2LM+QS)-bit bitmap (Alt3C) achieves same average throughput as 2LMQ-bit 3D bitmap, while overall feedback overhead can be reduced about 5.8% to 9.9% (reduced from 973 bits, to 917 or 877 bits |
| Ericsson | Bitmap | SLS: UPT, overhead | Bitmap alternative Alt1 with reporting of only non-empty DD bitmaps is close to Rel-16 Type-II implementation in complexity and is a simpler reporting format |
| MediaTek | Bitmap | SLS: UPT vs overhead | Observation 8: NZC indication by Alt 3A/3B can yield the same UPT gain as Alt 1 with 20~30 % feedback overhead saving. |
| Issue # 2.6 |  |  |  |

Table 3C Type II Doppler: List of UCI parameters (to date)

Rel-16 based: UCI parameter list

| Parameter | UCI | Details/description | Status |
| --- | --- | --- | --- |
| # NZ coefficients | Part 1 | RI ({1,…, RIMAX}) and KNZ,TOT (the total number of non-zero coefficients summed across all the Q selected DD basis and across all the layers, are reported in UCI part 1 | TBD |
| Wideband CQI | Part 1 | Same as R15 | Complete |
| Subband CQI | Part 1 | Same as R15 for X=1 | Complete for X=1 (support for X=2 is TBD) |
| Q Bitmap(s) per layer | Part 2 | Details TBD | Down-select from alts on TBD |
| Strongest coefficient indicator (SCI) | Part 2 | Legacy (Rel16) for N4=1 (not sure this has been agreed) TBD for N4>1 For legacy: - RI=1: A log2KNZ-bit indicator for the strongest coefficient index l*,m*,d* - RI>1: See Table 2 above | TBD |
| SD basis subset selection indicator | Part 2 | SD basis subset selection indicator is a log2N1N2L-bit indicator. Details follow Rel.15 | Complete |
| FD basis subset selection indicator | Part 2 | Details follow Rel.15 (Table 2 above) | Complete |
| DD basis subset selection indicator (per layer), if N4>1 | Part 2 | Details TBD | TBD |
| LC coefficients: phase | Part 2 | Legacy (Rel16) for N4=1, and TBD for N4>1 | Complete for N4=1 TBD for N4>1 |
| LC coefficients: amplitude | Part 2 | Legacy (Rel16) for N4=1, and TBD for N4>1 | Complete for N4=1 TBD for N4>1 |
| SD oversampling (rotation) factor q1, q2 | Part 2 | Values of q1, q2 follow Rel.15 | Complete |

Table 4 Additional inputs: issue 2

| Company | Input |
| --- | --- |
| Mod V0 | Please share your inputs on each of the issues and, if applicable, proposals in TABLE 3A Also check TABLE 3A/B/C and update your views |
| Lenovo | Issue 2.1: Support Conclusion 2.A.2 Issue 2.2: - We are fine with supporting two modes of CQI reporting, e.g., X=1 and X=2. - Our preference is to discuss later whether one of the two modes is optional, unless some vendors have concerns with supporting X=2. [Mod: Indeed there are a handful.] - Is it possible to add FFS on whether each of the X CQI values is mapped to a given slot or a given window, e.g., as follows: - Alt1: each of the X CQI values is associated with a given slot - Alt2: each of the X CQI values is associated with a configured time window, e.g., X=Q=2, where each CQI is associated with a window of d slots each [Mod: The current proposal is the best possible compromise from companies’ views. I am sure you’ll come to the same conclusion if you review all of them] Issue 2.3: We support Proposal 2.C.1, Conclusion 2.C.3 and Conclusion 2.C.5 Issue 2.5: We are fine with Proposal 2.E.1, prefer Alt3A Issue 2.6: Support including Q as part of the parameter combination list |
| CATT | Issue2.2: We support to have X = 2, as performance gain can be observed compared with X = 1. Regarding Slot/W2 ref: Our preference is Alt1. Issue 2.3: Ok with 2.C.1. We do see performance gain with Q = 3, and support to have Q = 3. Regarding proposal 2.C.4, we don’t support the proposal. We don’t support =1, as it does not perform very different from =0 and =2. |
| NTT DOCOMO | We added our view on Proposal 2.C.1and d value for P-/SP-CSI-RS. |
| Spreadtrum | Issue 2.1: Support Conclusion 2.A.2. Issue 2.2: We support X=1 and the CQI is associated with the first slot as basic feature. For the two optional features X=2, we are also fine If there’s majority support. Issue 2.3: Support Proposal 2.C.1. Issue 2.4: Support both Proposal 2.D.1 and Proposal 2.D.2. Issue 2.5: Support Alt1. In our views, further overhead reduction may not be needed since the PMI overhead is mainly affected by the number of reported NZCs. |
| Samsung | Issue 2.2: based on our study, we see gains with both X=2 (over X=1) and Alt1 (over Alt2A/2B) for CQI calculation. In the absence of these gains, the gain of Type II Doppler codebook with shrink to a tiny number (not good for an enhancement like this, if the is tiny, e.g. <5%). We do understand the concern on UE complexity, but it is up to UE implementation which can’t be quantified via simulation. We did some simulation (as shown in out Tdoc) on two slots, and observed additional gain over Alt2A, which can reduce the performance issue (mentioned above). So, for progress, we can accept two slots for CQI calculation. We are not convinced that two slots should be optional. But, we can move on if proposal 2.B.1 can be agreed, as it is (without any changes). Issue 2.3: - proposal 2.C.1: minor correction, for N4=2, there is no need for DD basis indication since N4=Q=2. So, “for N4>1…” can be replaced with “for N4>2…”. [Mod: OK] - Conclusion 2.C.2: support. The UPT vs overhead tradeoff of Q=3,4 is worse than that for Q=2 - Conclusion 2.C.3: support - proposal 2.C.4: support - Conclusion 2.C.5: among the 4 candidates, N4=16 is at least needed, at least for low UE speed (e.g. 3-10kmph). N4=8 for these speeds is too small. So, we suggest to include it. [Mod: Yes, that’s your preference, but check the support vs not support list. As said conclusion merely states the fact and one cannot pick and choose which conclusion to support ]. Issue 2.5 - Proposal 2.E.1: support Alt1, there is no benefit with Alt3A since Alt1 with a lower beta can achieve the same performance overhead tradeoff. Issue 2.6: we prefer separate Q from parameter combination, otherwise, we will have to go through the same simulation campaign as we are currently doing for CJT, necessary for CJT, but not for Doppler in our view. So, the separate Q can make our life much easy. |
| MediaTek | Issue 2.1: - Conclusion 2.A.2: Support Issue 2.2: - Proposal 2.B.1: Even though we have showed through simulation that X>1 is not delivering any performance benefits we are fine with supporting the compromise proposal. Issue 2.3: - Proposal 2.C.1: Agree with the comment made by Samsung, “for N4>1…” can be replaced with “for N4>2…” or simply mention for cases where N4>Q. - Conclusion 2.C.2, 2.C.3, 2.C.4, 2.C.5: Support Issue 2.5: - Proposal 2.E.2: Support Issue 2.6: - Agree with Comment made by Samsung, Q can be listed separately. |
| Apple | Issue 2.1: Slightly prefer Conclusion 2.A.2 Issue 2.2: We are fine with Proposal 2.B.1 Issue 2.3: We are fine with Proposal 2.C.1 We are fine with Conclusion 2.C.2, 2.C.3, 2.C.5 We are fine with 2.C.4 Issue 2.4: We are fine with Proposal 2.D.1 and Proposal 2.D.2 Issue 2.5: We are fine with Proposal 2.E.1, slightly prefer Alt 1 We are fine with Proposal 2.E.2, but we think the last bullet is “across all layers” [Mod: Good catch, thanks] |
| LG | Issue 2.1: Support Conclusion 2.A.2 Issue 2.2: We also support X=N4 to fully achieve potential gain of CSI prediction and it can be an optional feature considering UE side complexity. [Mod: A handful of UE vendors and 1 infra vendor already had issue with X=2 but are willing to compromise. X=N4 is unacceptable to them. Sorry] Issue 2.3: Our 1st preference is to support d<periodicity considering high velocity UE but we can live with d=periodicity in case of P/SP CSIRS. Issue 2.4: We are fine with Proposal 2.D.1 and Proposal 2.D.2 Issue 2.5: Proposal 2.E.2: Further clarification is needed. Is this K0=β2LMor K0=β2LMQ? In our view, the former is not sufficient since it does not consider additional DD basis dimension. If it is the later, we are fine with Q=2 but prefer to put FFS Q>2, if supported. [Mod: Analogous to legacy, the equations you have above are K0,MAX, not K0. Yes, K0,MAX would be the second one] Issue 2.6: Same view with Samsung and MTK. |
| ZTE | Issue 2.2: Although we do not identify clear benefits of having X=2 and two slots (first + last), we can live with that for compromise. Then, for X=2, we think that we need to be very careful how to allocate the second WB/SB-CQIs into CSI part1/2, while considering marginal/low benefits. BTW, we prefer the original version from the FL for X=2: “middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrix” Issue 2.3: C.1: Support the modification from Samsung. C.2: Not support. As a compromise, we are wondering whether we may only support Q=3 additionally. [Mod: As I said to Samsung, the conclusion is just a fact and inevitable. Whether you support it or not doesn’t change the fact] C.3: Not our preference, but we can live with the conclusion. C.4: Not our preference, but we can live with that, if we can have clear common understanding that there is no restriction that there should be the integer multiples of d slots for the time domain duration between the last CSI-RS occasion no later than the legacy reference resource and the starting of WCSI window. C.5: Not our preference, but we can live with that. Then, our preference is corrected. Issue 2.4: Support both proposals Issue 2.5: Support Alt1 in proposal 2.E.1 and proposal 2.E.2. Issue 2.6: We share the same views with Samsung. In our views, the number of Q, if having candidate value of 3, is much relevant to time-domain channel correlation (Doppler-domain), rather than Delay domain. |
| NEC | Issue 2.1: We also slightly prefer Conclusion 2.A.2. And we are fine to follow majority view if any. Proposal 2.B.1: Support. Proposal 2.C.1: We are fine. And we’d like to clarify whether DD basis remapping analogous FD basis remapping is needed or not in spec? Conclusion 2.C.2: We prefer to support Q=3 and Q=4. Conclusion 2.C.3: Fine with the conclusion. Proposal 2.C.3: Fine with the proposal. Conclusion 2.C.5: We prefer to support N4= 16. Regarding parameter d: We support d= periodicity for P/SP CSI-RS, and our position added in the table. Proposal 2.D.1 and 2.D.2: Support. Proposal 2.E.1: Support the proposal, and we are fine with Alt 1 or Alt 3A, while for Alt 3A, if it is supported, we think it should depend on the configuration, or in other words, in some configurations, Alt 3A can be supported for overhead reduction, and in some other configurations, Alt 1 should be supported. For example, at least when M=1, we don’t think it’s typical case to select S<Q DD basis firstly. Proposal 2.E.2: Support the proposal. Issue 2.6: We are fine either way, while this can be discussed after Q value finalized. If only Q=2 supported, seems no need to discuss this. |
| Sharp | Issue 2.1: We support conclusion 2.A.2 Issue 2.2: We support FL proposal 2.B.1 Issue 2.3: We are fine with all conclusions Issue 2.4: We support proposal FL proposal 2.D.1 and 2.D.2 Issue 2.5: We are OK with proposal 2.E.1 and 2.E.2 Issue 2.6: We are open to discuss this issue. |
| Google | Issue 2.1: Support Conclusion 2.A.2 Issue 2.2: Although we do not see the need for >1 CQI, if there is no other concern, we can be fine with Proposal 2.B.1 Issue 2.3: OK with 2.C.1/2/3/4/5. Issue 2.4: Support Proposal 2.D.1 and Proposal 2.D.2 Issue 2.5: OK with Proposal 2.E.1/2 Issue 2.6: We slightly prefer “No” |
| Qualcomm | Issue 2.2 Fine with Proposal 2.B.1, and prefer middle slot for X=2, mainly for possible RAN4 test feasibility Issue 2.3/2.4/2.5 Fine with all the proposals/conclusions. |
| CMCC | Issue 2.3: Support all the proposals and conclusions. For the d value for P/SP-CSI-RS, support d = periodicity. Issue 2.4: Support Proposal 2.D.1 and 2.D.2 Issue 2.5: Support Proposal 2.E.1 and prefer Alt1. The total overhead much rely on the LC coefficient reporting, and if the maximum value of Q is only 2, then this optimization over NZCs locations bitmap seems overdesign. Support Proposal 2.E.2. Issue 2.6: Slightly prefer “no”, if there are only two candidate values for Q. |
| Ericsson | Issue 2.2 We are support FL’s Proposal 2.B.1. For X=2, we prefer the 2nd CQI to be associated with the middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrix. This is a natural choice as this represents the ‘sample-hold’ CQI report with X=2 samples. Issue 2.3: We support Proposal 2.C.1. Regarding Conclusion 2.C.2, we see that some companies have shown gain with Q=3. We think agreeing Q=3 only may be a middle ground (no need to Q=4). Would the companies with concern for adding Q>2 be ok with this? Regarding Conclusion 2.C.3, we see some potential benefits to adding K=12. But we can live with the conclusion. On d value for P/SP-CSI-RS, we have some questions for the proponents of d=periodicity. Below table shows the possible prediction window lengths WCSI: CSI-RS periodicity (slots); WCSI = d N4 (slots). N4=1; N4=2; N4=4; N4=8. 4; 4; 8; 16; 32. 5; 5; 10; 20; 40. 8; 8; 16; 32; 64. 10; 10; 20; 40; 80. 16; 16; 32; 64; 128. 20; 20; 40; 80; 160. 32; 32; 64; 128; 256. 40; 40; 80; 160; 320. 64; 64; 128; 256; 512. 80; 80; 160; 320; 640. 160; 160; 320; 640; 1280. 320; 320; 640; 1280; 2560. 640; 640; 1280; 2560; 5120. If d=CSI-RS periodicity, then for different periodicities, we will have very large WCSI lengths (e.g., for a 20 slot CSI-RS periodicity, WCSI can be as large as 80 slots). It is unclear how accurate the CSI prediction will be for such large WCSI. Then, with the proposal of d=CSI-RS periodicity, to limit the prediction window length, one would have to reduce the CSI-RS periodicity which will increase the CSI-RS overhead. Setting d=1, 2 decouples WCSI from CSI-RS periodicity and will make type II Doppler codebook applicable to a larger range of CSI-RS periodicities. MediaTek results simulate the case with CSI-RS periodicity of 4 slots with CSI reporting periodicity of 4ms. Based on MediaTek’s results (blue curve in Figure 3): - (d, N4, Q) = (1, 4, 2) achieves the best performance at 30% mean throughput gain - (d,N4, Q) = (2,4, 2) achieves around 19% mean throughput gain - (d,N4, Q) = (4,4, 2) shows a loss of 2-3% These results show different performance overhead tradeoffs and it is not conclusive that d=4 is better than d=1. Also, in MediaTek’s results CSI reporting periodicity is much smaller than WCSI = dN4 is the last two cases. With (d, N4, Q) = (4, 4, 2), not sure why one would report CSI every 4 slots while WCSI = 16 slots. In Ericsson results, we simulated CSI-RS periodicity of 5 slots with CSI reporting periodicity of d*N4 slots (same as prediction window length). Our results show: d; N4=4; N4=8. 1; 15% Avg gain (~78% overhead reduction); 34% Avg gain (~58% overhead reduction). 2; 15% Avg gain (~78% overhead reduction); 29% Avg gain (~58% overhead reduction). 4; 10% Avg gain (~78% overhead reduction); 22% Avg gain (~58% overhead reduction). Our results show that for a similar level of overhead, Avg throughput can significantly drop as d is increased. Hence, d=1 offers a much better performance overhead tradeoff than d=4. We see that Intel and Nokia also have some results. We suggest to make a decision on this issue of d value for P/SP-CSI-RS based on the available results. According to our results, we see notable performance degradation with increasing d. So, we cannot accept agreeing d=CSI-RS periodicity, and prefer a solution that includes d=1 and/or 2. Perhaps, one way to resolve this is to have UE capabilities for the value of d? Issue 2.6: We prefer to keep Q parameter separate from the (L, pv,  |
| Mod V23 | Revised proposals based on comments and added new ones |
| Fraunhofer IIS/HHI | Issue 2.1: Support. Issue 2.2: Many companies have shown significant gains using X≥2 compared to X =1. Therefore, at least X = 2 should be a basic feature as well. [Mod: UE vendors and 1 infra vendor have issues making X=2 basic. This is the best we can achieve in terms of compromise. Else without CQI this codebook is useless and might as well be down-scoped in March …] Proposal 2.C.1: In Rel. 16, the FD basis vector associated with the SCI is cyclically shifted to FD basis 0. So, for the reporting of the selected FD components, as FD component 0 is always selected, the remaining M-1 components are reported. So far, we haven’t discussed anything about related to cyclic shifting of the DD basis vector associated with the SCI. Therefore, we prefer having more discussion on this proposal. Conclusion 2.C.2: Not support. We do not think it a good idea to support only a single value of Q for different values of N4. Companies have shown significant gains when using Q > 2 DD components. Supporting a single value of Q restricts the codebook capability to work under different practical scenarios. Therefore, at least one another value of Q should be supported. [Mod: Check my comment below “Mod V31” regarding what no consensus means] Proposal 2.C.4: Not support. δ=0 and δ=2 are already sufficient in our view. [Mod: Applying what no consensus means (an utterly simple principle you – and other – seem to be struggling with for some reason), I have changed this proposal to conclusion of no consensus to your avour] Conclusion 2.C.5: we have observed gains even for N4 = 20 especially for low speed Ues. Therefore, at least N4 = 16 shall be supported. [Mod: Check Mod V31 comment] Proposal 2.C.6: Support Proposal 2.D.1/2.D.2: Support Proposal 2.E.1: Support. Support Alt 3A. For the comparison of different alternatives, the value of β should be fixed. There is no point in comparing Alt 1 and Alt 3A with varying β. Proposal 2.E.2: Support |
| Fujitsu | Issue 2.1: Support conclusion 2.A.2 Issue 2.2: Fine with Proposal 2.B.1 Issue 2.3: Fine with all conclusions Issue 2.4: Fine with all proposals Issue 2.5: OK with proposal 2.E.1 and 2.E.2 |
| Intel | Issue 2.2: We prefer to support the middle slot of the CSI reporting window for the 2nd CQI since the last slot has the worst prediction error. So CQI calculated at the middle may better represent the actual channel quality. [Mod: It’s now middle slot only] Issue 2.3: Proposal 2.C.1: We don’t think that any discussion is needed before Q > 2 agreed. For Q = 2 this proposal is reduced to indication of one index (i.e. there is no combination involved). [Mod: true] Conclusion 2.C.2, Conclusion 2.C.3 and Conclusion 2.C.5: We are OK with the conclusions. Proposal 2.C.4: In our view value of 1 is needed for delta. And, in addition to that we can further extend the range. We don’t see any problem to support larger number of values for delta since this parameter is mainly to assist UE/gNB alignment of slots used for prediction (CSI reporting window). It doesn’t have direct impact on overhead and its impact on UE complexity strongly depends on the prediction algorithm. For some algorithms there is no additional computation/complexity to support larger delta. Regarding DD unit size (d), based on our simulation results, CSI-RS periodicity and CSI periodicity has greater impact on the performance. So, we don’t see the need of values other than d = m/CSI-RS periodicity. Issue 2.5: Proposal 2.E.2: The same comment as Apple – ‘across all layers’ in the last sub-bullet. [Mod: done] |
| Nokia/NSB | Proposal 2.C.1 Support Conclusion 2.C.2 Fine Conclusion 2.C.3 Fine Proposal 2.C.4 Fine Proposal 2.C.5 Fine Proposal 2.C.6 We have similar observations as Ericsson based on SLS results, i.e. for P/SP CSI-RS we observe some prediction gain for values of d=1,2. One possible compromise for P/SP-CSI-RS may be to support both d=period and one value d=1, i.e. add P/SP to the second bullet point - for P/SP-CSI-RS, support d equal to the periodicity of the CSI-RS resource - for AP/P/SP-CSI-RS, also support d =1 [Mod: OK] Proposal 2.E.2 Support Conclusion 2.F.1 Support |
| Fraunhofer IIS/HHI | Correcting our position on Issue 2.1: Conclusion 2.A.2: Not support |
| Xiaomi | Conclusion 2.A.2: Support Proposal 2.B.1: Support. We share same view with intel. Proposal 2.C.1: Support Conclusion 2.C.2: If Q>2 can achieve better tradeoff between performance and overhead. We can support. Conclusion 2.C.3: Support Proposal 2.C.4: We do not see there are much difference between δ =1 and δ =0,2. In order to provide more schedule or processing time of gNB, a larger value, e.g., δ =4 can be considered. Conclusion 2.C.5: Support Proposal 2.C.6: Not support. For lager period of P-CSI-RS, d=1 will need more computation complexity, since more channel predication are required at UE side. Proposal 2.D.1 and 2.D.2: Support Proposal 2.E.1: Support and prefer Alt3A Proposal 2.E.2: Support Conclusion 2.F.1: It can be discussed after the value Q is determined. |
| MediaTek | Re Ericsson: The 4ms reporting periodicity is only for the baseline Rel-16 eType II precoder. Through this we wanted to assess the benefit of Rel-18 compared to more frequent reporting using Rel-16 precoder. Rel-18 precoder is reported every dN4 slots, same as your understanding. Furthermore, the feedback overhead for dN4 slots is normalized by 4 slots to ensure a fair comparison between Rel-16 and Rel-18 precoders and obtain the overhead gain in the x-axis. We can see that d=1 can offer better performance, however, the feedback overhead per 4 slots would be ~80% higher compared to Rel-16. The key takeaway from our experiment is the following trade-offs: - Constant N4: d reduces →WCSI reduces → frequent CSI reporting → feedback overhead increases (blue curve) - Constant WCSI: d reduces →N4 increases → DD compression increases for a given Q → performance degrades (orange curve) In both cases, we see that smaller value of d can cause worse performance-overhead tradeoff. Besides this, the biggest concern on smaller d value is the UE complexity of CSI interpolation every slot or every 2 slots. |
| Mod V31 | Proposal 2.C.4 changed to conclusion 2.C.4 due to no consensus. @ALL: please keep in mind that a conclusion of no consensus is based on a fact that there is no consensus. Therefore, the inevitable outcome is that the feature is not supported. Whether you support or don’t support (accept or cannot accept) the conclusion doesn’t change the fact and reality. I hope this utterly simple principle is crystal clear to everyone. |
| Ericsson | On Issue 2.5, We support Proposal 2.E.1 We’d like to update our position. We support Alt 3A. Ericsson can be removed from Alt 1. |
| Huawei, HiSilicon | Issue 2.1, we support proposal 2.A. We don’t see any technical reason to not support Rel-17 based enhancement, and due to the commonality between Rel-16 & Rel-17 based enhancements, there’s marginal efforts to support both. Proposal 2.B.1 Fine with Proposal 2.B.1. For X=2, considering smaller performance gain and doubled CQI feedback overhead, we prefer add a FSS considering CQI overhead reduction for X=2. [Mod: OK] Conclusion 2.C.2: Don’t support. Only a single value Q=2 is too restrictive for different practical scenarios and companies shows performance gain for Q>2. We think at least another value need support. [Mod: Read my comments below. Whether you support the conclusion of no consensus or not doesn’t change the fact/reality that additional Q will not be supported] Conclusion 2.C.3: We hope companies could further consider larger K values from performance point of view. From our SLS, there’s a good performance gain with more CSI-RS occasions. Conclusion 2.C.4, we prefer to have more values for , but we can be fine with it. Proposal 2.E.1 Support the proposal. We support Alt3A for bitmap overhead reduction and no obvious performance loss. Conclusion 2.F.1 This issue can be discussed after the value Q is determined. |
| Mod V36 | Some edits. Repeating myself since what I said on V31 doesn’t seem to go through for some … @ALL: please keep in mind that a conclusion of no consensus is based on a fact that there is no consensus. Therefore, the inevitable outcome is that the feature is not supported. Whether you support or don’t support (accept or cannot accept) the conclusion doesn’t change the fact and reality. I hope this utterly simple principle is crystal clear to everyone. |
| Vivo | Issue 2.2 OK with Proposal 2.B.1 as a compromise, and prefer middle slot for X=2. Issue 2.3 - OK with proposal 2.C.1. - OK with conclusions 2.C.2 and 2.C.5. - On 2.C.3, we echo Huawei’s comment on requesting companies to further consider larger K values, as UE needs this burst to calculate both the time domain filter for prediction and perform channel prediction based on this calculated filter, if only one RS set is supported for channel measurement. If no larger value is supported, gNB needs to configure multiple RS sets and transmits another associated RS burst to UE so that UE can calculate and update the time domain filter used for prediction. Therefore, if we have to make a conclusion, we propose to have the following change. Conclusion 2.C.3: For the Type-II codebook refinement for high/medium velocities, regarding the parameter K (the number of AP-CSI-RS resources for the CMR in the resource set triggered for reporting the CSI), there is no consensus in supporting additional candidate values [Mod: Modified to 2.C.3 to support K=12] - On 2.C.4, as we have agreed to support one additional value, this one value should be finalized. - Our first preference is to have a larger value (e.g., delta=4) as delta=2 is already supported. For example, if we have delta=4, gNB can configure two N4=2 reports, each with delta = 2 and delta = 4. This can even achieve better performance than configure an N4=4 report with delta, as the prediction performance of the first two DD units is not impacted by the prediction loss of the last two DD units during compression. - But we can compromise to another value like delta=1. The report slot is usually an UL slot. To make sure CQI reference slot is a valid DL slot and reduce the possibility that the predicted CSI is dropped, we think it is needed to have multiple delta>0 values so that gNB can have the flexibility to configure the CQI reference slot to a valid DL slot. [Mod: As I said (see below), no consensus implies no support regardless of the previous agreement. This may be what will happen to mode-1 as well. We are applying the same principle] - On the issue for d value for P/SP CSI-RS, as we have agreed to support P/SP CSI-RS for CSI prediction, this issue needs to be finalized. We support to use CSI-RS periodicity as the d value, which is more friendly to UE given UE does not need to perform interpolation. Further, based on our evaluation, it can also achieve large performance gain compared with legacy eType II without prediction. We are also okay to support P/SP CSI-RS periodicity as a basic feature and a smaller value as an optional feature. Issue 2.5 - We support to remove the brackets for Alt 4, as we observe gain on overhead reduction for Alt 4 (about 40 bits with <1% SE difference). [Mod: Done due to the reasoning below] - Based on our observation on multiple companies’ results (cf. [8], [27]) showing overhead-UPT curves for Q=2 and N4=4, with similar overhead reduction (about 40-50 bits), Alt 3A can suffer about 5%-10% performance loss whereas Alt 4 only has <1% loss compared with Alt 1. - Alt 3A itself is not fully clear to us. - To understand Alt 3A better, we are wondering whether the first bitmap is layer-specific or layer-common, or it is next-level of details to be studied? - As indicated by the WID, DD basis should be decoupled from SD and FD basis. However, based on our understanding of Alt 3A, it actually couples DD and FD basis, and the first bitmap actually selects the best joint SD and FD basis vectors. This somehow violates WID. [Mod: IMO your assessment is valid. Note that we also have the following agreement as the corollary of the WID: [110bis-e] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, support the following codebook structure where N4 is gNB-configured via higher-layer signaling: - For N4=1, Doppler-domain basis is the identity (no Doppler-domain compression) reusing the legacy W1, W2, and Wf, e.g. W1W2(Wf)H - For N4>1, Doppler-domain orthogonal DFT basis commonly selected for all SD/FD bases reusing the legacy W1 and Wf, e.g. W1W2Wf⨂WdH - Only Q (denoting the number of selected DD basis vectors) >1 is allowed - TBD (by RAN1#110bis): whether rotation is used or not - FFS: identical or different rotation factors for different SD components - FFS: Whether Q is RRC-configured or reported by the UE Note: Detailed designs for SD/FD bases including the associated UCI parameters follow the legacy specification FFS: Whether one CSI reporting instance includes multiple W2 and a single W1 and Wf report. I agree with you that the net effect of Alt3A runs against the agreement since DD basis seems to be independently selected for each SD/FD basis combo. But let’s hear from other companies… ] Issue 2.6 We support to configure Q separately if multiple Q values are supported. |
| MediaTek | Re vivo, Issue 2.5: - The first bitmap of size MQ to select delay Doppler pairs is definitely layer specific, similar to legacy SD-FD bitmap of Rel-16, or the Alt 1 being discussed currently. Some companies propose combinatorial selection of S pairs out of MQ pairs. However, for our simulation and feedback overhead calculation, we used S = μMQ pairs with bitmap selection. - We don’t think Alt 3A couples DD-FD basis, and neither does it violate the WID. The SD, FD, DD selection are all independent of each other, and the reduction is only during NZC selection phase. In fact, Alt 3A is a special case of Alt 1, when the NZC reside only in specific columns of the overall bitmap of size 2L x MQ. Alt 3A just tells which columns are these. [Mod: Thanks. My assessment is more aligned with vivo. But noted.] |
| Mod V42 | Changed conclusion 2.C.3 to proposal 2.C.3 due to change of views Repeating myself since what I said on V31 doesn’t seem to go through for some … @ALL: please keep in mind that a conclusion of no consensus is based on a fact that there is no consensus. Therefore, the inevitable outcome is that the feature is not supported. Whether you support or don’t support (accept or cannot accept) the conclusion doesn’t change the fact and reality. I hope this utterly simple principle is crystal clear to everyone. |
| Sony | Issue 2.1: Okay with conclusion 2.A.2. Issue 2.2: We can support Proposal 2.B.1. Issue 2.3: Support Proposal 2.C.1 and are okay with conclusions 2.C.2, 2.C.5. Issue 2.4: Support FL’s proposals 2.D.1, 2.D.2. Issue 2.5: Support FL’s proposals 2.E.1, 2.E.2. |
| Samsung | Conclusion 2.C.2: support Proposal 2.C.3: although we don’t think K=12 is needed, so we can live with it as long as it is optional Conclusion 2.C.5: we still prefer 16. But, given the situation, OK with the conclusion Proposal 2.C.6: re d=1 for P/SP, it requires per-slot interpolation (in addition to prediction), which adds to UE complexity. Besides, d=1, the CSI reporting shortens to N4 <=8 (based on agreements thus far), which is very small, at least for low mobility UE. Finally, we are not sure it can bring noticeable gain since we don’t per-slot measurement (we only have per-p slot measurement, p=period). Considering we don’t see any strong reason to support. However, for progress, we can OK with it, if we can reduce the UE complexity by limiting the max value of #csi-rs ports to 8 (max value of #CSI ports / min periodicity of P/SP = 32/4=8). Proposal 2.C.6: For the Type-II codebook refinement for high/medium velocities, regarding the parameter d (in slots), - for P/SP-CSI-RS, support d equal to the periodicity of the CSI-RS resource - for P/SP/AP-CSI-RS, also support d =1 - for P/SP, max #csi-rs ports is limited to 8. [Mod: Since we removed P/SP, this is no longer needed] Proposal 2.E.1: - @Fraunofer, we don’t think different beta for Alt1 and Alt3A is not a fair comparison. Our metric to support any parameter value is based on UPT vs overhead. - Agree with vivo that Alt3A is not according to the agreement (please see below) and WID wording (please see below) since it corresponds to joint (FD, DD) basis. So, we should perhaps remove it. - @Mediatek: - We don’t think Alt 3A couples DD-FD basis, and neither does it violate the WID. - Response: don’t we select S pairs of (FD,DD) basis vectors jointly from a 2D grid of MQ basis vector pairs? Then, WID please see below. - The SD, FD, DD selection are all independent of each other, and the reduction is only during NZC selection phase. - Response: We can also view Alt3A as a scheme which does layer-specific joint (FD,DD) basis selection via the size MQ bitmap, it is somewhat similar to M’<=M reporting discussed during Rel.16 (I am sure some people remember that). The actual bitmap for NZC is size 2LS, here S corresponds to joint (FD,DD) basis pairs. - In fact, Alt 3A is a special case of Alt 1, when the NZC reside only in specific columns of the overall bitmap of size 2L x MQ. Alt 3A just tells which columns are these - Response: this (Alt3A) is then a simple optimization of Alt1, which is not necessary since the same performance can be achieved by using lower beta in Alt1 (and follow bitmap design) [110bis-e] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, support the following codebook structure where N4 is gNB-configured via higher-layer signaling: - For N4=1, Doppler-domain basis is the identity (no Doppler-domain compression) reusing the legacy W1, W2, and Wf, e.g. W1W2(Wf)H - For N4>1, Doppler-domain orthogonal DFT basis commonly selected for all SD/FD bases reusing the legacy W1 and Wf, e.g. W1W2Wf⨂WdH [Comment1: reusing legacy means Wf has to be separate, and bitmap should include M FD basis vectors for NZC selection for each SD beam, which may not be the case with Alt3A) - Only Q (denoting the number of selected DD basis vectors) >1 is allowed - TBD (by RAN1#110bis): whether rotation is used or not - FFS: identical or different rotation factors for different SD components - FFS: Whether Q is RRC-configured or reported by the UE Note: Detailed designs for SD/FD bases including the associated UCI parameters follow the legacy specification [Comment2: both SD/FD and associated UCI parameters should follow legacy] FFS: Whether one CSI reporting instance includes multiple W2 and a single W1 and Wf report. WID - Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - [Comment 3: Joint basis is not codebook refinement, and it certainly is a modification of the FD basis.] - UE reporting of time-domain channel properties measured via CSI-RS for tracking [Mod: Noted that Samsung’s observation is aligned with vivo] |
| Mod V48 | Minor revision on 2.C.6 |
| Fraunhofer IIS/HHI | @Samsung: please find our response inline. Samsung: @Fraunofer, we don’t think different beta for Alt1 and Alt3A is not a fair comparison. Our metric to support any parameter value is based on UPT vs overhead. Fraunhofer: Using different beta for Alt 1 and Alt 3A is not a fair comparison. Companies have shown that using same values of beta for Alt and Alt 3A results in throughput values close to that of Alt 1 but with a feedback reduction of 24 bits per layer on an average. Your observation on using different values of beta holds not only for Alt 3A but also for Alt 1. If different values of beta have no impact on the throughput, then your observation proves the fact that the angle-delay-Doppler precoder is sparse and a smaller beta value is sufficient for this codebook. This is the whole point of Alt 3A, which tries to reduce the feedback overhead of the bitmap by reporting only the dominant columns of the bimap associated with the FD-DD components. Samsung: Agree with vivo that Alt3A is not according to the agreement (please see below) and WID wording (please see below) since it corresponds to joint (FD, DD) basis. So, we should perhaps remove it. Fraunhofer: Your observation becomes valid only if UE reports FD and DD components jointly using a log2N3N4-1S-1-bit indicator. I hope you are aware that the M FD components are reported using legacy log2N3-1M-1-bit indictor and the Q DD components are reported using log2N4-1Q-1-bit indicator (See proposal 2.C.1). Therefore, as the FD and DD components are not reported jointly, Alt 3A does not violate the agreement in any way . As MediaTek clearly explained, dominant FD-DD pairs are calculated only during the NZC phase after calculating the precoder from the M FD and Q DD components Samsung: @Mediatek: - Mediatek: We don’t think Alt 3A couples DD-FD basis, and neither does it violate the WID. - Samsung: don’t we select S pairs of (FD,DD) basis vectors jointly from a 2D grid of MQ basis vector pairs? Then, WID please see below. - Fraunhofer: Selection of S pairs from a 2D grid of N3N4 basis vector pairs violate the WID - Mediatek: The SD, FD, DD selection are all independent of each other, and the reduction is only during NZC selection phase. - Samsung: We can also view Alt3A as a scheme which does layer-specific joint (FD,DD) basis selection via the size MQ bitmap, it is somewhat similar to M’<=M reporting discussed during Rel.16 (I am sure some people remember that). The actual bitmap for NZC is size 2LS, here S corresponds to joint (FD,DD) basis pairs. - Fraunhofer: S pairs are indicated via MQ bitmap and not N3N4 bitmap. So, it doesn’t violate the WID. - Mediatek: In fact, Alt 3A is a special case of Alt 1, when the NZC reside only in specific columns of the overall bitmap of size 2L x MQ. Alt 3A just tells which columns are these. - Response: this (Alt3A) is then a simple optimization of Alt1, which is not necessary since the same performance can be achieved by using lower beta in Alt1 (and follow bitmap design) - Fraunhofer: Although we have repeated it many times, let me repeat it here again for your convenience. Using a smaller beta value is not the point here. This codebook exploits sparsity in angle-delay-Doppler domain, and I hope there is no doubt accepting this fact. Basically, smaller values of beta are completely fine to achieve satisfactory throughput gains compared to Rel. 16 eType-II codebook. The whole point of Alt 3A is not about using either smaller or larger beta values. It’s about discarding or reducing the 2LMQ-KNZ zeros in the bitmap after NZC selection to reduce the feedback overhead associated only with the BITMAP. For L=4, M =4, Q = 2, the bitmap size is 64 bits which can be brought down to 40 bits with Alt 3A with the same performance as Alt 1and an overhead saving of 24 bits per layer. [110bis-e] Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, support the following codebook structure where N4 is gNB-configured via higher-layer signaling: - For N4=1, Doppler-domain basis is the identity (no Doppler-domain compression) reusing the legacy W1, W2, and Wf, e.g. W1W2(Wf)H - For N4>1, Doppler-domain orthogonal DFT basis commonly selected for all SD/FD bases reusing the legacy W1 and Wf, e.g. W1W2Wf⨂WdH [Comment1: reusing legacy means Wf has to be separate, and bitmap should include M FD basis vectors for NZC selection for each SD beam, which may not be the case with Alt3A) Fraunhofer: Reporting of Wf is neither related to Alt 1 nor Alt 3A. Regardless of Alt 1 or Alt 3A, Wf is separately indicated using log2N3-1M-1 indicator. If S≥M then the bitmap comprises M FD basis vectors for each SD beam which addresses your concern. No company who provided simulation results wants S to be less than M . - Only Q (denoting the number of selected DD basis vectors) >1 is allowed - TBD (by RAN1#110bis): whether rotation is used or not - FFS: identical or different rotation factors for different SD components - FFS: Whether Q is RRC-configured or reported by the UE Note: Detailed designs for SD/FD bases including the associated UCI parameters follow the legacy specification [Comment2: both SD/FD and associated UCI parameters should follow legacy] Fraunhofer: yes SD/FD and associated UCI parameters follow legacy for both Alt 1 and Alt 3A. FFS: Whether one CSI reporting instance includes multiple W2 and a single W1 and Wf report. WID - Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - [Comment 3: Joint basis is not codebook refinement, and it certainly is a modification of the FD basis.] - Fraunhofer: As explained above, there is no use of joint basis for Alt 3A . Bases on your thinking, one can also say that the joint basis argument is valid for Alt 1. For Alt 1, a bitmap of size 2LM is reported for each DD component which means that for each slice, all M FD components are always associated with a DD component. Does this also mean that UE reports joint FD-DD basis? I don’t think your argument is valid here. - UE reporting of time-domain channel properties measured via CSI-RS for tracking @Vivo: Pls find our response inline Issue 2.5 - We support to remove the brackets for Alt 4, as we observe gain on overhead reduction for Alt 4 (about 40 bits with <1% SE difference). - Based on our observation on multiple companies’ results (cf. [8], [27]) showing overhead-UPT curves for Q=2 and N4=4, with similar overhead reduction (about 40-50 bits), Alt 3A can suffer about 5%-10% performance loss whereas Alt 4 only has <1% loss compared with Alt 1. - Fraunhofer: We empathize with you regarding Alt 4. Unfortunately, no other company simulated your preference, and we cannot really talk about the simulation results of Alt 4. Since we and other companies simulated Alt 1 and Alt 4, we can compare and share our views only on Alt 1 and Alt 3A. Companies who evaluated Alt 3A aimed to achieve performance close to that of Alt 1 due to which overhead reduction of approximately 37.5% is observed. I think performance reduction of 75% is too optimistic in our opinion. Not only Alt 3A, there is no wonder even Alt 1 will also suffer performance reduction if number of NZCs are drastically reduced like you did. BTW Alt 3A must be compared with baseline i.e., Alt 1. - Alt 3A itself is not fully clear to us. - To understand Alt 3A better, we are wondering whether the first bitmap is layer-specific or layer-common, or it is next-level of details to be studied? - Fraunhofer: Please see response from Mediatek. - As indicated by the WID, DD basis should be decoupled from SD and FD basis. However, based on our understanding of Alt 3A, it actually couples DD and FD basis, and the first bitmap actually selects the best joint SD and FD basis vectors. This somehow violates WID. - Fraunhofer: Please see my elaborate response to Samsung [Mod: Noted that you disagree with vivo/Samsung and agree with MediaTek on this matter] |
| Mod V53 | No change |
| Nokia/NSB | Issue 2.5 Proposal 2.E.1 In our view Alt 3A is inconsistent with a previous agreement (see highlighted text below) which allows the NZC locations in SD-FD to be different for different DD basis vectors. For example, when Mν=1, Alt 3A results in Q identical bitmaps (the MνQ bitmap must be an all-one vector in this case or else some of the Q bases are useless because the corresponding bitmaps would be empty). Besides, with Alt3A if a pair of DD basis vectors are set to ‘1’ for a given FD basis vector, in the MνQ bitmap, the same locations of NZCs are forced for the two DD bases, but some of these NZCs may be small or even quantised to 0, which is a problem for the amplitude reporting of the NZCs. The principle of free selection of the NZCs has been adopted consistently for all Type-II codebooks and we see no strong reason why Type-II-Doppler should be an exception. Agreement [RAN1#111] For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, support the following: - Q different 2-dimensional bitmaps are introduced for indicating the location of the NZCs, where the qth (q=1,…., Q) 2-dimensional bitmap corresponds to qth selected DD basis vector - The number of selected DD basis vectors is denoted as Q - This implies that for each layer, the location of NZCs in SD-FD can be different for different selected DD basis vectors. FFS: Further overhead reduction on bitmap(s) FFS: Whether the number of NZCs is upper bounded across all DD basis vectors or per DD basis vector [Mod: Noted that you agree with vivo’s assessment as opposed to MTK’s] |
| Fraunhofer IIS/HHI | @Nokia/NSB Please find our response inline. Nokia: In our view Alt 3A is inconsistent with a previous agreement (see highlighted text below) which allows the NZC locations in SD-FD to be different for different DD basis vectors. For example, when Mν=1, Alt 3A results in Q identical bitmaps (the MνQ bitmap must be an all-one vector in this case or else some of the Q bases are useless because the corresponding bitmaps would be empty). FHG: We don’t think Alt 3A is inconsistent with the previous agreement. In the previous agreement, further feedback overhead reduction is FFS and it doesn’t preclude any potential overhead reduction scheme. Nokia: For example, when Mν=1, Alt 3A results in Q identical bitmaps (the MνQ bitmap must be an all-one vector in this case or else some of the Q bases are useless because the corresponding bitmaps would be empty). FHG: This is a corner case. So far only Q = 2 is agreed. For Mν=1 and Q = 2, there are only 2 FD-DD pairs and there is no need to further reduce the feedback overhead. UE can simply only a single bitmap of bitmap of size 2LS, where S = Q and the MvQ bitmap may be redundant in this case. BTW, when Mν=1, we do not understand why Alt 3A results in Q identical bitmaps? Perhaps you can elaborate it? Nokia: Besides, with Alt3A if a pair of DD basis vectors are set to ‘1’ for a given FD basis vector, in the MνQ bitmap, the same locations of NZCs are forced for the two DD bases, but some of these NZCs may be small or even quantised to 0, which is a problem for the amplitude reporting of the NZCs. FHG: This is not true. With Alt3A if a pair of DD basis vectors are set to ‘1’ for a given FD basis vector, in the MνQ bitmap, the same locations of NZCs are not forced for the two DD bases. Nokia: The principle of free selection of the NZCs has been adopted consistently for all Type-II codebooks and we see no strong reason why Type-II-Doppler should be an exception. FHG: See our detailed response to Samsung. Free selection also applies here. UE freely selects M FD and Q DD components and from the selected MvQ FD-DD pairs, UE freely selects S FD-DD pairs associated with dominant energy. If selection of S pairs out of MvQ FD-DD pairs results in no throughput loss in addition to significant feedback overhead saving of 37.5% per layer, we do see a strong reason to study it |
| Mod V56 | No change |
| Nokia/NSB | @Fraunhofer, please find below some further comments/answers FHG: This is a corner case. So far only Q = 2 is agreed. For Mν=1 and Q = 2, there are only 2 FD-DD pairs and there is no need to further reduce the feedback overhead. UE can simply only a single bitmap of bitmap of size 2LS, where S = Q and the MvQ bitmap may be redundant in this case. BTW, when Mν=1, we do not understand why Alt 3A results in Q identical bitmaps? Perhaps you can elaborate it? For example, for Mν=1 and Q=2, in Alt 3A, the FD-DD selection bitmap must be [1 1], whereas the other bitmap of size 2LS=2L is, for example for L=2, [1 0 1 0]. This means that, according to the previous agreement, the 2LMν bitmap for the first DD basis is [1 0 1 0], and the bitmap for the second DD basis is also [1 0 1 0]. It is not possible for a UE to choose different locations of the NZCs for the two DD bases. FHG: This is not true. With Alt3A if a pair of DD basis vectors are set to ‘1’ for a given FD basis vector, in the MνQ bitmap, the same locations of NZCs are not forced for the two DD bases. As shown in the example above, if a pair of DD bases are set to ‘1’ for a given FD basis, in the MνQ bitmap, the locations of the NZCs for that FD basis are the same for the two DD bases, because there is a single column in the 2LS bitmap indicating these positions. Some of these NZCs may be small or even quantised to zero because they are not freely selected based on their magnitude |
| Mod V61 | No change |

### Issue 3: TRS-based reporting of time-domain channel properties (TDCP)

Table 5A Summary: issue 3

| # | Issue | Companies’ views |
| --- | --- | --- |
| 3.1 | [111] Agreement For the Rel-18 TRS-based TDCP reporting, down select only one of the following alternatives by RAN1#112: - AltA.1 (Doppler spread) as described in R1-2210523 - AltA.2 (Doppler shift): A UE is configured to report the Doppler shifts corresponding to the M strongest peaks of the wideband Doppler spectrum, for each of the N≥1 configured TRS resources - A TDCP report can be configured with N periodic TRS resources (e.g., N=2 with one TRS resource per TRP) - Parameter M is RRC configured with candidate values TBD, e.g. M=1,2,3,… - Wideband Doppler spectrum is calculated from the wideband time correlation function, given, as an example, by t,τ=ct,τct,0 , where ct,τ=n=0N-1hnt+τ∙hn*t and hn is the channel for subcarrier n. - AltB (TD correlation profile) as described in R1-2210523 Down-selection is to done based on, at least, the (single-)user throughput (LLS) performance comparison among the alternatives assuming: - Three special cases of an agreed use case (companies can select only one or more): aiding gNB to determine switching between Type-I and Rel-16 eType-II codebooks, or to determine SRS periodicity in the UL-SRS reciprocity-based precoding scheme; or aiding the gNB implementation in CSI prediction for TDD - In their simulations on switching between Type-I and Rel-16 eType-II codebooks, companies should state how to calculate the metric for the determination and how to set the threshold, and what the UE reports. - In their simulations on UL-SRS reciprocity-based precoding scheme, companies should state how to set the SRS periodicity based on the reported metrics, and what the UE reports; and the results should be displayed in terms of user throughput vs SRS overhead - In their simulations on CSI prediction for TDD, the results should be the correlation between real channel and predicted channel, and what the UE reports; aided by the reported metric. - Other scenarios of the agreed use cases can optionally be simulated - Based on the agreed EVM for sTRP and mTRP Note: Different alternatives may or may not apply to different use cases FFS: The need for a measure of confidence level in the TDCP report, and/or UE behaviour when the quality of TDCP measurement is not sufficiently high FFS: TDCP parameter(s) signalled with respect to each alternative Proposal 3.A: For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows: - Basic feature: Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported - Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported - For Y>1, the phase can be configured to be absent for all the Y delays - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource - TBD: Candidate value(s) for Y>1 - FFS: Value of Dbasic FL Note: Most simulation results address use case 1, showing that AltB is either superior or performs similarly to AltA.2 (while AltA.1 performs the worst) with ONLY one lag. For use case 2, the main concern against AltB is the inability of the NW to reconstruct Doppler profile due to the absence of phase. In addition, >1 lags would be required to derive an accurate Doppler profile. All results assume wideband. The above compromise proposal fully accounts the available results (AlB is superior for use case 1) while addressing the main concern from the opponents. This is the current situation: AltA.1 (Doppler spread): - Support/fine: Google, LG, OPPO, Mavenir, Apple (1st pref), IDC, Spreadtrum, NEC (2nd pref), - Concern: Ericsson, Nokia/NSB (New) AltA.2 (Relative Doppler shift): - Support/fine: ZTE, CATT, Huawei/HiSi, - Concern: Ericsson, Nokia/NSB AltB (TD correlation profile): - Support/fine: Samsung, Ericsson, MediaTek, vivo, Qualcomm, DOCOMO, OPPO, Sharp, Lenovo, Apple (2nd pref), IDC, NEC (1st pref), CEWiT, Fraunhofer IIS/HHI, Intel, Xiaomi, Nokia/NSB - Concern: Nokia/NSB, Google | Support/fine: Ericsson, Nokia/NSB, Samsung, ZTE, Huawei/HiSi, Qualcomm, vivo, MediaTek, NTT DOCOMO, NEC, Sharp, [OPPO Y>1 phase off not needed], [Lenovo], Fraunhofer IIS/HHI, [Intel, Y>1 phase off not needed], Xiaomi, AT&T, Sony, Google, [Apple] Not support: |

Table 5B TDCP: summary of observation from simulation

| Company | SLS results |  |  |
| --- | --- | --- | --- |
|  | Issue # | Metric | Observation |
| Huawei/HiSi | 3.1 | Doppler shift, Correlation vs SNR Use case: SRS periodicity No throughput results per RAN1#111 agreement | Observation 14: SRS could not provide accurate Doppler shift information Observation 15: Due to the common feature of Doppler profile among gNB antennas, TRS could provide sufficient Doppler shift information even if it is single port. Observation 16: A “common Doppler profile” of multiple delay paths is a satisfying depict of the Doppler profile. As observed in Figure 18, prediction with pure amplitude of channel correlation information is even worse than that without any prediction, as it provides a wrong channel correlation info due to the lack of phase information. On the contrary, prediction with Doppler shift information demonstrates a way higher channel correlation, as it is a better representative of channel variation property. |
| ZTE | 3.1 | LLS: CIR, UPT vs speed, Doppler shift/correlation vs speed at different SNR Use case: SRS periodicity | It is observed from Figure 6 that multiple paths are merged into one peak due to the poor resolution, thereby Doppler spread can not be accurately estimated It is clearly observed from Figure 8 that Alt A.2 outperforms Alt B with higher throughput. Therefore, it we conclude that Alt A.2 is much more robust to noise than Alt B. The physical reason behind is that Alt A.2 selects strong or high-SNR peaks among the Doppler spectrum and reports the corresponding Doppler shifts, and thereby reduces the noise effects It is observed that Y = 4 performs worst, at some points the throughput is even lower than the random-SRS-periodicity case. Besides, Y = 16 performs best, but the gain against Y = 8 is very limited. |
| Google | 3.1 | Square cosine symmetry (SCS) vs Doppler shift SCS vs correlation No throughput results per RAN1#111 agreement | Observation 1: Perfect Doppler shift can assist the gNB to determine whether the CSI could be outdated or not. Observation 2: Channel correlation cannot provide direct information to determine whether the CSI is outdated or not. |
| Vivo | 3.1 | Autocorrelation vs lag No throughput results per RAN1#111 agreement | The Figure 10 shows the relationship between temporal correlation at different lags and maximum doppler shift based on Bessel function assumption |
| Xiaomi | 3.1 | Doppler shift estimation error vs speed/SNR No throughput results per RAN1#111 agreement | Observation 1: Only periodic TRS resource with larger period cannot obtain better Doppler estimation performance. Observation 2: Periodic TRS resource and aperiodic TRS resource are jointly configured in one period can achieve better Doppler estimation performance. Observation 3: Even only reporting partial time domain correlation amplitudes can achieve similar performance with all time domain correlation amplitudes reporting. Observation 4: Doppler shift estimation performance of both AltB and AltA2 for different speeds at different SNRs. While AltA1 can achieve better performance at low speed, e.g., smaller than 10 Km/h. |
| CATT | 3.1 | LLS: TP vs SNR Use case: gNB prediction | It can be observed from the simulation results that compared with baseline (without gNB-side CSI prediction), single Doppler reporting provides slight performance gain, and obvious performance gain can be achieved with multiple Doppler reporting |
| Mavenir | 3.1 | LLS: RMS vs SNR, STD vs SNR, Doppler spread vs speed, UPT vs SNR Use case: SRS periodicity | Observation 5. With the increasement of UE velocities, RMS & STD tend to increase as well. Observation 6. Doppler spread is hard to estimate on shorter lag (shorter than 1 TRS slot) over TDL-A channel under long delay spread. Observation 7. The longer delay spread results in worse accuracies of estimated Doppler. Observation 8. 4-symbol lag within single TRS burst has illustrated the worst measured accuracy, particular in lower SNR. Observation 9. Different lags between TRS pairs would take distinct performance with changing UE velocity, particularly in SNR0dB. Observation 11. Higher Doppler scenario means higher time-varying channel, SRS should be scheduled more frequently to overcome aging issue; but lower SRS overhead is enough to get better throughput when channel is under lower Doppler. |
| Samsung | 3.1 | SLS: UPT vs speed LLS: BLER vs speed Use case: T1-T2 switch | Observation 16: based on SLS evaluations, - AltB and AltA.2 achieve similar performance, and AltA.1 is worse than AltA.2 and AltB - AltB is simpler, easier to calculate/implement, and test; also, Doppler profile can be obtained from the TD correlation Observation 17: based on LLS evaluations, - The BLER performance is almost same for AltA.2 and AltB away from switching threshold speed. - At switching speed, AltB performs slightly better than AltA.2. - The number of lags required for AltA.2 computation is higher than AltB |
| Ericsson | 3.1 | LLS: UPT vs speed Use case: T1-T2 switch | - For the use case of using TDCP reporting for Type I vs Type II CSI switching, Alt B outperforms Alt A.1 and Alt A.2. - For the use case of using TDCP reporting for Type I vs Type II CSI switching, Alt B performs close to ideal mode switching (i.e., always selecting the mode which gives the best throughput).Alt B is robust against phase jumps between two TRS bursts separated by a delay value  since Alt B only makes use of the amplitude of the TD-correlation. - Alt A.2 is very sensitive to phase jumps between two TRS bursts and does not perform better than random mode switching (i.e., randomly choosing Type I or Type II without any TDCP report). - Alt A.2 single peak (M=1) is very sensitive to frequency errors at the UE and/or gNB, while Alt B is very robust as it measures and reports the amplitude of the TD-correlation. |
| MediaTek | 3.1 | LLS: UPT Use case: T1-T2 switch | - Observation 11: Alternatives B and A1 both achieve good performance for the codebook switching use case at all UE speeds. - Observation 12: Alternative A1 has much higher computational complexity compared to that of B for while delivering similar performance |
| Nokia, NSB | 3.1 | LLS: SE Use case: T1-T2 switch | - For method B only single parameter is important for the performance, which is lag power threshold. Number of lags used is not important for Type-I vs Type-II switching. - In Method B for low UE speeds smaller thresholds work better for lower speeds and higher threshold values work better for higher speeds with switch point at around 38 kmph. - For Method B higher values of lag power threshold show better performance across different UE speeds. - Depending on the use case optimal lag power threshold value can be different. - Quantization settings can influence the accuracy of the reporting values and as a consequence use case performance. |

Table 6 Additional inputs: issue 3

| Company | Input |
| --- | --- |
| Mod V0 | Please share your inputs on each of the issues and, if applicable, proposals in TABLE 5A Also check TABLE 5A/B and update your views |
| Lenovo | We are OK with the compromise to report the phase as an optional feature to address comments of some companies that the Doppler spectrum is not recoverable without phase information, however we prefer if the support of Y>1 is not optional. Reporting one lag value would not provide sufficient characterization of the Doppler profile, especially the TRS-based reporting has been agreed to be aperiodic only Proposal 3.A: For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband amplitude/phase of the time-domain correlation profile with Y≥1 lag(s) as follows: - Basic feature: Y≥1, only wideband quantized amplitude is reported - Optional feature: Y≥1, wideband quantized amplitude and phase for each lag are reported - For Y>1, the phase can be configured to be absent for all the Y lags - TBD: Whether the value of Y is configurable or following the lags from the configured TRS resource - Note: Y=1 is supported TBD: Candidate value(s) for Y>1 [Mod: Please check response from Ericsson. Also other companies. For Type-1/2 switching Y=1 amplitude only is sufficient. This has been demonstrated by all companies providing results. If you have some empirical evidence suggesting otherwise, we can discuss] |
| Ericsson | Regarding Lenovo’s comment about making Y>1 as the basic feature, it should be emphasized that for Alt B, throughput results form at least 4 companies (Ericsson, Samsung, Nokia, MediaTek) show that reporting normalized wideband autocorrelation amplitude for a single lag performs the best for the Type I vs Type II codebook switching use case. In Alt B, a threshold for switching between Type I and Type II is directly applied on the wideband quantized amplitude on a single lag at the gNB side. As Alt B does not rely on Doppler profile, a single lag is shown to be enough for the Type I vs Type II codebook switching use case at least based on 4 company simulation results. Also, we think it would not be possible to calculate Doppler power spectrum with only wideband amplitudes for Y>=1 lags (i.e., one would need both the wideband amplitudes and phases for Y>=1 lags). The idea behind the compromised proposal from the FL is to support Y=1 lag as the basic feature which can be used for at least Type I vs Type II codebook switching use case (Y=1 is much simpler in terms measurement complexity for the UE). To allow for the possibility of computing Doppler spectrum at the gNB side, the FL’s proposal also allows reporting wideband amplitudes and phase of Y>=1 lags. The current proposal is a good compromise considering measurement complexity, throughput performance results, and possibility to allow computing Doppler power spectrum at the gNB side. Hence, we are supportive of current Feature lead proposal 3.A by having - Y=1 with wideband amplitude only as basic feature, and - Y>=1 with wideband amplitude and phase as an optional feature. |
| NTT DOCOMO | Support. Agree with Ericsson that the current proposal is a good compromise. |
| OPPO | On the FL’s proposal, we support the basic feature (Y=1, only wideband quantized amplitude is reported). On the optional feature, we can understand the motivation of main bullet (Y≥1, wideband quantized amplitude and phase for each lag are reported), which is a compromise among different options. However, we don’t know why phase reporting is optional for Y>1. Without the reporting, how to estimate the DPS at gNB would be an issue. Can the proponents clarify the motivation not to report the phase information? [Mod: Please check ZTE response. Overall, the scheme looks more unified if switching off the phase for Y>1 is allowed. In addition, it may be more future proof if TDCP is enhanced in later releases] |
| Samsung | Same view as E///, we also think Y=1 with WB amp only should the basic feature since it is sufficient for the main use case (T1-T2 CB switch). For other use cases, we can optionally have Y>=1 with (amp, phase). We should not mix two use cases into one. The current proposal is clean and reasonable. |
| MediaTek | Support FL proposal as is. We agree with the comments made by Ericsson that Y=1 can deliver satisfactory performance hence, it is sufficient for the basic feature. |
| ZTE | Support FL proposal as is, for progress, although it is not definitely our preference. Regarding OPPO’s comment, from our perspective, reporting both amplitude and phase is to accommodate Doppler-domain spectrum estimation, no doubt, but, if considering the lack of long-term phase consistency for some UEs, the amplitude-only for multiple lags becomes a tradeoff solution for estimating above (although some loss) and considering UE implementation complexity, and supports the case of determining SRS periodicity for TDD, for instance. |
| NEC | Support Proposal 3.A. |
| Sharp | Support Proposal 3.A. We have the same view as Ericsson. |
| Google | From our simulation results, we cannot see a good use case for channel correlation with regard to SCS/SE. In addition, currently there is no former definition of “lag”. We think this needs to be clarified first. Secondly, for a UE with >1 Rx, should the UE report 1 channel correlation per Rx, or 1 channel correlation averaged across the Rx? Similar to other metrics, e.g., SS-RSRP/CSI-RSRP in 38.215, this needs to be clarified as well. |
| Mod V23 | No revision on proposal. @Proponents: Please address Google’s comments. |
| Fujitsu | We still prefer the Doppler shifts reporting for both use case in order to a unified framework. [Mod: the proposal accommodates this since with Y>1 and phase on the gNB can reconstruct Doppler profile] |
| Intel | Considering the situation, we support proposal in general. One thing that may need further clarification is whether we need flexibility to turn on and off reporting of phase for Y>1. Based on the comment from ZTE, it seems UE capability is needed to determine if long-term phase consistency is not supported by the UE. In case UE don’t have this capability, gNB can ignore reported phases or gNB can configure Y=1. But, we are not sure if we really need to turn off phase reporting. |
| Fraunhofer IIS/HHI | Support proposal 3.A which is a good compromise and a way to further progress on this topic. |
| Xiaomi | We are fine with Y=1 wideband quantized amplitude is reported as a basic feature. According to simulation results provided by some companies, it is suitable to use case of Type I and Type II switching. However, it cannot be used to determine the period of SRS in TDD system, i.e., the second use case. In our tDoc, the simulation results shows Y>1 wideband quantized amplitude can also obtain accurate Doppler shift estimation by using Alt B. We adopt expression (1) to calculate Doppler shift. MinτAt,τ-J0(2πfdτ) (1) where τ denotes the lag or interval of two CSI-RS resources which belong to same or different TRS bursts, e.g., τ=4, 14, 18,…, OFDM symobls. The values τ depends on the configuration of TRS burst. J0 is Bessel function. Note that the calculation of expression (1) is implemented at gNB side. UE just reports Y>1 quantized amplitudes corresponding to different values τ. In the proposal, according to understanding, the motivation of reporting Y≥1, wideband quantized amplitude and phase for each lag is used to obtain Doppler shift by calculating Doppler power spectrum.. However, Doppler shift can also be obtained through the calculation of expression (1) according to the reported multiple amplitudes corresponding to different τ, which does not need to calculate Doppler power spectrum. Therefore, Y>1, wideband quantized amplitude reporting should also be supported as a basic feature, which is applicable to the use case of SRS period determination. For the convenience, the comparison results on 60 km/h at different SNR are given as following for the three schemes. Other simulation results on different speed can be found in our tDoc. @Google: In our view, the lag can be defined as interval of two CSI-RS resources which belong to same or different TRS bursts. [Mod: Given majority view of AltB, the compromise of having Y>1 as optional is the best we can do. I hope you understand.] |
| Mod V31 | Changed “lag” to “delay” to avoid ambiguity per Google comment. “Delay” has been defined in R1-2210523 |
| Ericsson | Regarding Google’s comments: - Regarding the first comment, the use cases and the alternatives to evaluate were agreed in the last meeting. Also, throughput was agreed as the metric to use in evaluations. We have throughput results from four companies showing the benefit of channel correlation reporting for the agreed use case of Type 1 and Type 2 switching. - Regarding how to handle case with >1 Rx, we don’t think we need to report 1 channel correlation per Rx. One channel correlation should be sufficient. Regarding whether the channel correlations are averaged across Rx’s, this may be a UE-implementation related detail. We think it should be discussed at a later stage taking into account input from UE/chipset vendors. |
| AT&T | We joined late and want to support proposal 3.A with (Y =1) as a basic feature & optional for (Y>1) |
| Mod V36 | No change @Google: I assume Ericsson has addressed your question? |
| Google | We can compromise to accept the channel correlation for sake of progress, but it looks the proposal needs some revision. - The UE complexity depends on 1) number of delays; 2) the value of the delay – intra-slot or inter-slot within a burst or cross-burst. Large delay would require large buffer. So, we think we should make Y=1 and delay = 4 symbols (intra-slot) as basic feature, and other configurations can be optional. - Since in current proposal the gNB can configure the UE to report phase for Y>1, why the gNB cannot configure the UE to report phase for Y=1? Actually, the phase is more useful for intra-slot correlation as the gNB can keep continuous phase. For cross-burst case, it is hard to keep phase continuity. Proposal 3.A: For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows: - Basic feature: Y=1 and delay=4 symbol, wideband quantized amplitude is reported - Optional feature: Y=1 with delay other than 4 symbols and Y≥1, wideband quantized amplitude and phase for each delay are reported - For Y=1 and Y>1, the phase can be configured to be absent for all the Y delays - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource - TBD: Candidate value(s) for Y>1 [Mod: Thanks Yushu for the compromise. Would it be ok to leave this as FFS since this is a next level detail issue] |
| vivo | OK with proposal 3.A as it is. |
| Xiaomi2 | We understand that the current proposal is a comprise result for all companies, and we can live with it for specification progress. The calculated Y≥1 wideband quantized amplitude/ phase are associated with TRS resources configuration or delays τ. For example, even when Y=1, two or more periodic and/or aperiodic TRS resources can be configured to obtain the wideband quantized amplitude. For a mobile speed of UE, more TRS burst configurations can help to improve the accuracy of amplitude, since more TRS bursts can be used to reduce the noise impact. Similarly, the values τ has impact on the wideband amplitude as well. Therefore, how to configure TRS resources and/or delays τ need to further study. [Mod: Thanks very much for your understanding] |
| Mod V42 | No change |
| Ericsson | Thank you to Google and Xiaomi for the compromise. Regarding the suggested delay value for Y=1 in basic feature by Google, we ran some quick simulations. As shown in the figure below, seems most of the gain is lost when the delay value is 4 symbols or 1 slot compared to 5 slot delay. We may need some careful study on the exact delay value so that the use cases agreed give meaningful gains for the basic feature. |
| Sony | Our view is also that the FL’s proposal represents a good compromise between Alt B and enabling the gNB to compute the Doppler power spectrum. We therefore support it. |
| Google | We are open to further study the delay values, but we think we should not always say Y=1 with any value is basic feature. But Y=1 with delay > 14 symbols should be optional, since this requires cross-burst correction. How about the following change? Proposal 3.A: For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows: - Basic feature: Y=1 and delay<=14 symbol, wideband quantized amplitude is reported - Optional feature: Y=1 with delay>14 symbols and Y≥1, wideband quantized amplitude and phase for each delay are reported - For Y=1 and Y>1, the phase can be configured to be absent for all the Y delays - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource - TBD: Candidate value(s) for Y>1 [Mod: Thanks for being flexible. IMO your suggestion seems reasonable Let’s see how the proponents respond] |
| Mod V48 | @Proponents (e.g. Ericsson ): Please check Google’s suggestion if this is fine for you. |
| Ericsson | Thanks Google for the flexibility. Based on some offline discussion with Google, the following may be a reasonable WF? Proposal 3.A: For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows: - Basic feature: Y=1 and delay<= X symbols, wideband quantized amplitude is reported - Optional feature: Y=1 with delay> X symbols and Y≥1, wideband quantized amplitude and phase for each delay are reported - For Y=1 and Y>1, the phase can be configured to be absent for all the Y delays - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource - TBD: Candidate value(s) for Y>1 - FFS: value of X [Mod: OK this seems reasonable. I have revised the proposal. Thanks, Yushu, for being flexible.] |
| Mod V50 | Revised 3.A based on the inputs from Google and Ericsson |
| Google | Thank you for the update. We are fine with the latest version from Ericsson. [Mod: Again, thanks Yushu for your flexibility] |
| Mod V53 | No change in content except editorial on notation from X to D_basic (since we have used X in Doppler for # CQIs) to avoid confusion |
| Mod V56 | No change |
| Apple | More clarification is needed. What is the quantized amplitude? Is it normalized or non-normalized? Different UE has different AGC and ADC design, how can we design an amplitude report unless it is normalized? |
| Ericsson | @Apple: According to the first agreement where we defined, Alt B it is a normalized amplitude (R1-2210523). So in this case, it should be quantized normalized amplitude. We suggest the following revision: Proposal 3.A: For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows: - Basic feature: Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported - Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported - For Y>1, the phase can be configured to be absent for all the Y delays - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource - TBD: Candidate value(s) for Y>1 - FFS: Value of Dbasic [Mod: OK] |
| Mod V61 | Added “normalized” per Ericsson’s suggestion to address Apple’s question |

# References

| 1 | R1-2301248 | Summary of OFFLINE discussion on Rel-18 MIMO CSI | Moderator (Samsung) |
| --- | --- | --- | --- |
| 2 | R1-2300095 | CSI enhancement for coherent JT and mobility | Huawei, HiSilicon |
| 3 | R1-2300158 | Enhanced CSI for CJT and Medium/High UE Velocities | InterDigital, Inc. |
| 4 | R1-2300183 | CSI enhancement for high/medium UE velocities and CJT | ZTE |
| 5 | R1-2300205 | Discussion on CSI enhancement for high/medium UE velocities and coherent JT | Spreadtrum Communications |
| 6 | R1-2300250 | CSI enhancement for high/medium UE velocities and coherent JT | OPPO |
| 7 | R1-2300391 | On CSI Enhancement | Google |
| 8 | R1-2300438 | Further discussion on CSI enhancement for high-medium UE velocities and coherent JT | vivo |
| 9 | R1-2300502 | CSI enhancements for medium UE velocities and coherent JT | Fraunhofer IIS, Fraunhofer HHI |
| 10 | R1-2300512 | Discussion of CSI enhancement for high speed UE and coherent JT | Lenovo |
| 11 | R1-2300524 | Potential CSI enhancement for high/medium UE velocities and coherent JT | LG Electronics |
| 12 | R1-2300547 | Discussion on CSI enhancement for high/medium UE velocities and CJT | xiaomi |
| 13 | R1-2300652 | Discussion on CSI enhancement | CATT |
| 14 | R1-2300741 | Views on CSI enhancement for coherent-JT and mobility | Fujitsu |
| 15 | R1-2300812 | Discussion on CSI enhancement | NEC |
| 16 | R1-2300866 | More considerations on CSI enhancement for high/medium UE velocities and CJT | Sony |
| 17 | R1-2300900 | Discussion on CSI enhancement | Mavenir |
| 18 | R1-2300934 | On CSI enhancements | Intel Corporation |
| 19 | R1-2300984 | Discussion on CSI enhancement for high/medium UE velocities and CJT | CMCC |
| 20 | R1-2301219 | Views on CSI Enhancements for CJT | AT&T |
| 21 | R1-2301249 | Views on CSI enhancements | Samsung |
| 22 | R1-2301331 | Views on Rel-18 MIMO CSI enhancement | Apple |
| 23 | R1-2301397 | CSI enhancements for medium UE velocities and Coherent-JT | Qualcomm Incorporated |
| 24 | R1-2301479 | Discussion on CSI enhancement | NTT DOCOMO, INC. |
| 25 | R1-2301525 | CSI enhancements | SHARP Corporation |
| 26 | R1-2301526 | On CSI enhancements for NR MIMO evolution | Ericsson |
| 27 | R1-2301573 | CSI Enhancements | MediaTek Inc. |
| 28 | R1-2301644 | CSI enhancement for high/medium UE velocities and CJT | Nokia, Nokia Shanghai Bell |