3GPP TSG RAN WG1 #112			R1-2301255

Athens, Greece, February 27th– March 3rd, 2023

Agenda item:	9.2.2.1

Source:	Samsung

Title:	Views on Evaluation of AI/ML for CSI feedback enhancement

Document for:	Discussion and Decision

- Introduction

In RAN#94-e, Rel-18 new study item on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” is endorsed. One of the objective of the study item [1] is the following:

*** text omitted*** Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels *** text omitted*** For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

In this contribution, we will provide our views on the evaluation of AI/ML for CSI feedback enhancement.

- Evaluation on AI/ML for CSI feedback enhancement.

Based on the part of SID [1], captured above, the evaluation methodologies (EVMs) and KPIs for the use cases under consideration will be studied. In this regard, this contribution discusses EVMs and KPIs for one of the use cases, namely, CSI feedback enhancement. Moreover, we consider four sub-use cases under CSI feedback enhancement. A brief description of the sub-use cases, which are described in details in [2], is given below.

- CSI prediction:  This sub-use case considers the prediction of CSI in time domain. An AI/ML solution located at either the UE or gNB performs CSI prediction based on a set of inputs, e.g. past CSI measurements/reports.

- Frequency-domain CSI extrapolation: This sub-use case considers extrapolation of CSI in the frequency domain.  An AI/ML solution located at either the UE or gNB performs CSI prediction in one frequency band based on a set of inputs, e.g. CSI measurements/reports for another frequency band.

- Spatial-frequency domain CSI compression: This sub-use case envisions compression of the CSI feedback in spatial and frequency domains based on a two-sided model. One side of the model located at the UE encodes the CSI to its compressed representation. Another side of the model located at the gNB decodes and reconstructs the received compressed CSI feedback. The pair of encoder and decoder, hence, is referred to as an auto-encoder (AE).

- Temporal-spatial-frequency-domain CSI compression: This sub-use case considers the compression of CSI feedback in spatial, frequency, and time domains. As this sub-use case considers the three aforementioned compression domains, it can be loosely considered as a combination of the above three sub-use cases.

Note: sub-use cases (b) and (d) are presented in the Appendix section as they are not selected as representative sub-use cases in this study item.

  - Remaining Issue on General Aspects of Evaluation Methodologies

RAN1 has made a good progress in coming up with a common evaluation methodology for the evaluation of two-sided model based CSI compression and one-sided model based CSI prediction. In RAN1#109-e, EVMs based on SLS as a baseline and LLS as optional are agreed for evaluation of AI/ML-based CSI feedback enhancement. The tables for the agreed evaluation parameters are included in the Appendix section. Moreover, as shown below, it was agreed that companies can consider performing intermediate evaluation on AI/ML model to derive the intermediate KPIs. In our view, the intermediate evaluation with properly selected intermediate KPIs can be used to compare the performance advantages and complexity requirements of potential sub use cases. This greatly simplifies the evaluation efforts and allows the study to consider a diverse set of sub use cases. Eventual evaluation with system level performance metrics will assist in drawing accurate conclusions on the benefits of AI/ML for CSI feedback enhancement and for possible recommendations of normative projects. With this in mind, in the below, we provide our views on the remaining issues on the evaluation methodologies for AI/ML based CSI feedback enhancement.

Agreement For the performance evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline - Link level simulation is optionally adopted Agreement For the evaluation of the AI/ML based CSI feedback enhancement, companies can consider performing intermediate evaluation on AI/ML model performance to derive the intermediate KPI(s) (e.g., accuracy of AI/ML output CSI) for the purpose of AI/ML solution comparison.

### 2.1.1. Reference for Inference Complexity Analysis

RAN1 made the below agreements regarding inference complexity. In agenda item 9.2.1, RAN1#110b-e clarified inference complexity includes complexity of pre- and post-processing. Moreover, in RAN1#109, FLOPs was adopted as a measure of computational complexity. However, while reporting the complexity of pre- and post-processing, the reference input and output format of the pre-processing and post-processing operations, respectively, could be ambiguous. For example, raw channel matrices or eigenvectors can be considered as input for pre-processing. In order to clarify this point we propose the following:

Proposal #1: For the evaluation of the AI/ML based CSI feedback enhancement, while reporting the computational complexity of pre-processing and post-processing the following are considered.

- Estimated raw channel matrix per each  frequency unit as an input for pre-processing

- Precoding vectors per each  frequency unit as an output of post-processing

Note: frequency unit can be set to 4 RB or 2 RB for 15KHz SCS and 30KHz SCS, respectively.

Agreement RAN1#110b-e (under agenda item 9.2.1) The following are additionally considered for the initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Clarification on inference complexity - Note: Inference complexity includes complexity for pre- and post-processing. - LCM related complexity and storage overhead - Storage/computation/latency for training data collection. - Storage/computation/latency for training and model update - Storage/computation/latency for model monitoring. - Storage/computation/latency for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation. - FFS: Power consumption, latency (e.g., Inference latency) Agreement RAN1#109 For the evaluation of the AI/ML based CSI feedback enhancement, Floating point operations (FLOPs) is adopted as part of the ‘Evaluation Metric’, and reported by companies. Agreement RAN1#110 For the evaluation of the AI/ML based CSI compression sub use cases, the capability/complexity related KPIs, including FLOPs as well as AI/ML model size and/or number of AI/ML parameters, are to be reported separately for the CSI generation part and the CSI reconstruction part.

### 2.1.2. Alignment on CSI Payload Size

Another remaining issue is alignment on CSI payload for the comparison of AI/ML based CSI and baseline schemes. One option is to align on the payload size taking in to account the variable payload size supported by the baseline schemes, e.g., different parameter combinations, nonzero coefficients reporting, etc. for Rel-16 Type II CB. While admittedly, this provides the fairest comparison, it is restricting to the AI/ML model design as all companies should strive to fixed payload size if this approach is adopted.

Companies can simply calculate the payload size as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE. The CSI payload can be the maximum payload size of a particular reported rank. Alternatively, it is also possible to count the reported payload bits at each CSI report based on Rel-16 and report the average. Obviously, the latter allows for more accurate comparison as the actually used payload size will be reported while the earlier may help for calibration.  As both views are valid, we propose the following:

Proposal 2: For the CSI payload size calculation for AI/ML-based CSI compression as well as the legacy TypeII codebook, the payload size is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE.

Note: Whether the CSI payload is calculated as each CSI reported payload with a given rank or as max allowed bits at the given rank can additionally be reported.

  - Evaluation of Training Collaboration Types for Two-sided Models

The following were agreed for the evaluation of Type 2 and Type 3 training collaboration of two sided models.

Agreement For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), the following evaluation cases are considered for multi-vendors, - Case 1 (baseline): Type 2 training between one NW part model to one UE part model - Case 2: Type 2 training between one NW part model and M>1 separate UE part models - Companies to report the AI/ML structures for the UE part model and the NW part model - FFS Companies to report the dataset used at UE part models, e.g., whether the same or different dataset(s) are used among M UE part models - Case 3: Type 2 training between one UE part model and N>1 separate NW part models - Companies to report the AI/ML structures for the UE part model and the NW part model - FFS Companies to report the dataset used at NW part models, e.g., whether the same or different dataset(s) are used among N NW part models - FFS N NW part models to M UE part models - FFS different quantization/dequantization methods between NW and UE - FFS: whether/how to evaluate the case where the input/output types and/or pre/post-processing are not aligned between NW part model and UE part model - FFS: companies to report the training order of UE-NW pair(s) in case of M UE part models and/or N NW part models - FFS: whether/how to report overhead Agreement For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following cases are considered for evaluations: - Case 1 (baseline): Aligned AI/ML model structure between NW side and UE side - Case 2: Not aligned AI/ML model structures between NW side and UE side - Companies to report the AI/ML structures for the UE part model and the NW part model, e.g., different backbone (e.g., CNN, Transformer, etc.), or same backbone but different structure (e.g., number of layers) - FFS different sizes of datasets between NW side and UE side - FFS aligned/different quantization/dequantization methods between NW side and UE side - FFS: whether/how to evaluate the case where the input/output types and/or pre/post-processing are not aligned between NW part model and UE part model

Fig. 1 Two different training orders on the Type 2 training for one UE part and N network part models.

One of the FFS points for the evaluation of Type 2 training for multi-vendor training case, i.e., M>1 UE part models and/or N>1 network part models, is the training order. For example, as shown in Fig. 1, two training orders can be considered wherein in one the training is performed in a single training session or multiple sessions. These two orders of training for Type 2 may have different practical implications, e.g., it is increasingly difficult to collaborate with multiple vendors simultaneously, either over the air-interface or offline training setup.

Proposal#3:  For the evaluation of Type 2 training, for multi-vendors training Case 2, i.e., one NW part model and M>1 separate UE part models, and Case 3, i.e., one UE part model and N>1 separate NW part models, evaluate and compare the following two training scenarios:

Scenario 1: training in a single session

Scenario 2: training in multiple sequential training sessions, i.e, one UE part and one network part at each training session.

  - Evaluation Metrics

Agreement RAN1#109-e For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI - For GCS/SGCS, - FFS: how to calculate GCS/SGCS for rank>1 - FFS: whether GCS or SGCS is adopted - FFS other metrics, e.g., equivalent MSE, received SNR, or numerical spectral efficiency gap. Agreement RAN1#110-e For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, between GCS and SGCS, SGCS is adopted Agreement RAN1#110b For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, at least Method 3 is adopted, FFS whether additionally adopt a down-selected metric between Method 1 and Method 2. - Method 1: Average over all layers - Method 2: Weighted average over all layers SGCS=E1Ni=1NjKλijk=1Kλik¦¦wijHwij ¦¦¦¦wij¦¦ ¦¦wij¦¦2 where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the jth output vector of the output CSI of resource unit i. N is the total number of resource units. E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij. - Method 3: SGCS is separately calculated for each layer (e.g., for K layers, K SGCS values are derived respectively, and comparison is performed per layer)

Another remaining issue is details on intermediate KPIs. For example on how to calculate SGCS for rank>1, for methods were discussed

- Method 1: Average over all ranks

- Method 2: Weighted average over all ranks

- Method 3: SGCS is separately calculated for each rank (e.g., for K ranks, K SGCS values are derived respectively, and comparison is performed for per rank)

In our view Method 2 is reasonable as it closely emulates the expected system level throughput. In order to forge this alignment between intermediate KPIs and the eventual system performance metrics, the weights can be set as the singular values of the channel’s covariance matrix. It is to be noted that such properties of intermediate KPI may mean it can be used for other purposes than evaluation such as model monitoring. Thus method 2 can be additionally adopted.

Proposal #4: For the evaluation of the AI/ML based CSI feedback enhancement, adopt SGCS for rank>1 as weighted average over all ranks

SGCS=E1N×Ki=1NjK(λij)j=1K(λij)||wijHwij ||||wij||  ||wij||2

where λij is an eigenvalue of the channel covariance matrix corresponding to wij.

Note: wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples.

Furthermore, some discussion were made in the last RAN1 meeting whether to consider additional intermediate metrics. This includes Realized relative SNR (RRSNR), Chordal distance, numerical spectral efficiency gap, normalized expected directional gain (NEDG). The main motivation is to emulate the DL throughput better. As an example RR SINR and NEDG capture the residual interlayer interference. In particular, consider the below formulation for RR SINR

RRSNRK≔ E1NK i=1Nj=1K(λij)i=1Nj=1KHiwij2wij2

where λij is an eigenvalue of the channel covariance matrix corresponding to wij.

Note: Hi is the channel matrix at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over.

Consider the channel matrix Hi=UiΣiWiH  where Ui=[ui1ui2…]  and W=[wi1wi2…] are the right and left-side eignvector matrices of Hi while Σi=diag(λi1λi2…). Then, Hiwij=k=1Kuik(λik)2(wik)Hwij . Here, it can be noted that RRSNR captures the signal power from the j-th layer projected on the subspace spanned by the other eigenvectors too, i.e.,  (wik)Hwij  for k≠j. However, if the receiver (UE) applies the left-side eigenvectors to receive the transmission for different layers, i.e., (uij)HHiwij=(λik)2(wij)Hwij to receive the j-th layer, this leaking energy is suppressed (nullified). Besides, for MU-MIMO, the gNB may apply inter-layer interference nulling, e.g., by applying zero-forcing, after reconstructing the reporting precoder. Thus, in MU-MIMO scenario, when the gNB do not directly apply the reconstructed precoder, RRSNR does not give practical advantage in terms of emulating the DL MU-MIMO as compared to SGCS.

Observation #1: RRSNR as intermediate KPI may be advantageous on providing better emulation of the MU-MIMO DL throughput performance than GCS in some receiver assumptions. If the UE performs inter-layer interference nulling, this advantage is lost.  This observation holds to other proposed intermediate KPIs such as numerical spectral efficiency gap and NEDG.

Observation #2: In MU-MIMO scenario, when the gNB does not directly apply the reconstructed precoder, RRSNR does not give a practical advantage in terms of emulating the DL MU-MIMO throughput performance than SGCS. This observation holds to other proposed intermediate KPIs such as numerical spectral efficiency gap and NEDG.

Proposal #5: For the evaluation of the AI/ML based CSI feedback enhancement, deprioritize discussion on additional intermediate KPIs.

  - Evaluation for CSI prediction

In RAN1#111, AI/ML based CSI prediction using UE side model is selected as one of the representative sub-use case. Moreover, a working assumption regarding the baseline for evaluation of CSI prediction sub-use case is also take as follows.

Agreement Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. Note: Continue evaluation discussion in 9.2.2.1. Note: RAN1 Defer potential specification impact discussion at 9.2.2.2 until the RAN1#112b-e, and RAN1 will revisit at RAN1#112b-e whether to defer futher till the end of R18 AI/ML SI. Note: LCM related potential specification impact follow the high level principle of other one-sided model sub-cases. Working Assumption For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies. - Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example - It does not imply any restriction on future specification for CSI prediction - FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction

As it is indicated in the above working, assumption, one of the baselines for evaluation of AI/ML based CSI prediction is level x AI/ML based CSI prediction. One of the differentiating factors between the level y/z based CSI prediction and level x based baseline is the LCM assistance that could be given to the UE for the gNB. For example, if models are not generalizable across certain sets of scenarios/configurations, e.g., UE velocities, gNB may give LCM assistance in order to activate/deactivate/select etc. the appropriate AI/ML based functionality. Thus, it is crucial to identify which scenarios and configurations models for AI/ML based CSI prediction can generalize to or not.

To make proper evaluation of level y/z based AI/ML CSI prediction as compared to level x based CSI prediction, RAN1 may additionally consider relevant scenarios and configuration that would affect model generalizability. In this regard, we propose UE speed to be considered as one scenario to evaluate model generalizability in CSI prediction sub-use case.

Agreement RAN1#110 For CSI enhancement evaluations, to verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects as a starting point: - Various deployment scenarios (e.g., UMa, UMi, InH) - Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10) - Various carrier frequencies (e.g., 2GHz, 3.5GHz) - Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc. - Companies to report the selected scenarios for generalization verification Agreement For CSI enhancement evaluations, to verify the generalization/scalability performance of an AI/ML model over various configurations (e.g., which may potentially lead to different dimensions of model input/output), the set of configurations are considered focusing on one or more of the following aspects as a starting point: - Various bandwidths (e.g., 10MHz, 20MHz) and/or frequency granularities, (e.g., size of subband) - Various sizes of CSI feedback payloads, FFS candidate payload number - Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports) - Other aspects of configurations are not precluded, e.g., various numerologies, various rank numbers/layers, etc. - Companies to report the selected configurations for generalization verification - Companies are encouraged to report the method to achieve generalization over various configurations to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc. Conclusion RAN1#110 If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for evaluation, - 100% outdoor UE is assumed for UE distribution. - FFS: whether to add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles - UE speed is assumed for evaluation with 10, 20, 30, 60, 120km/h - Note: Companies to report the set/subset of speeds - 5ms CSI feedback periodicity is taken as baseline, while other CSI feedback periodicity values can be reported for the EVM

Proposal#6: For the evaluation of AI/ML based CSI prediction sub use case, the following evaluation cases are considered,

- Case 1: The AI/ML model is trained based on training dataset from one UE speed A, and then the AI/ML model performs inference/test on a dataset from the same UE speed A

- Case 2: The AI/ML model is trained based on training dataset from one UE speed A, and then the AI/ML model performs inference/test on a different dataset than UE speed A, e.g., UE speed B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple UE speeds including UE speed A and a different dataset than UE speed A, e.g., UE speed B, and then the AI/ML model performs inference/test on a dataset from a single UE speed from the multiple UE speeds, e.g.,  UE speed A, UE speed B.

  - Note: Companies to report the ratio for dataset mixing

- Note: UE speed includes 10Kmphr, 20kmphr, 30kmphr, 60kmphr and 120kmphr

- Preliminary Evaluation Results

  - CSI prediction

Recent research results from both academia and industry indicate that AI-based CSI prediction strategies can significantly reduce prediction error to achieve better performance as compared to non-predictive CSI report supported Rel-15-17.  Many of these results utilize deep learning techniques to learn the temporal channel correlations (and, in some instances, spatial-frequency channel correlations).

Our evaluation results were generated using the following parameters:

Table 1: Simulation parameters

| Parameter | Configuration |
| --- | --- |
| Scenario | UMA 3D |
| Frequency | 2 GHz |
| ISD | 200 m |
| UE distribution (Indoor,Outdoor) | 100% |
| UE speed (Indoor,Outdoor) | (10-120kmphr) |
| gNB antenna config | 32 ports [8,8,2,1,1,2,8], (dH,dV)=(0.5,0.8) |
| UE antenna config | 4 ports [1,2,2,1,1,2,2], (dH,dV)=(0.5,0.5) |
| BS Tx power | 41 dBm |
| BS antenna height | 25 m |
| UE antenna height | 1.5 m |
| UE antenna gain | 5 dBi |
| UE noise figure | 9 dB |

A CSI measurement and application timeline in the figure below is considered in our evaluation. A CSI measured from K=3 CSI-RS resources are used for CSI prediction. The CSI prediction is made to two application instances. Application instance 1 and Application instance 2 which are 10ms and 25ms after the last CSI-RS resource used for measurement, respectively.

Figure 1: CSI measurement and predicted CSI application timeline

An LSTM based AI/ML model illustrated in the figure below is used for CSI prediction.

Figure 2: AI/ML model for CSI prediction

The first set of results in Table 1 and Table 2 show the performance of CSI prediction with the sample-and-hold baseline (CSI measurement and reporting from the latest CSI-RS resource) for the two CSI application instances shown in Figure 1. The SGCS is computed with respect to the ground truth CSI at the corresponding application instance. One observation from these results is that the AI/ML based CSI prediction yields higher SGCS across all UE speeds. Yet, another observation is that the SGCS gain is different over UE speeds and application instances. This suggests that the gNB can actively monitor the UE’s scenario, e.g., speed, and configure the number of CSI prediction instances and prediction window accordingly. For example, the gNB may configure the UE to report CSI for multiple instances over a longer prediction window for low mobility UEs. This may have advantage in terms of reducing CSI overhead as opposed to frequent (short prediction window) reporting. On the other hand, for higher UE speeds, the gNB may configure the UE to predict CSI for a single instance and shorter prediction window just to compensate the delay due to CSI measurement and reporting.

Table 1: SGCS comparison between predicted CSI and sample-and-hold (SGCS computed with respect to the groud-truth CSI at the application instance

| Application Instance 1 (10ms) ¦ SGCS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type↓ ¦UE speed→ | 10 kmph | 20 kmph | 30 kmph | 40 kmph | 60 kmph | 120 kmph |
| No-prediction | 0.8793 | 0.7458 | 0.6905 | 0.5806 | 0.4367 | 0.3773 |
| Model generalization in time | 0.9570 | 0.8527 | 0.7725 | 0.7045 | 0.5745 | 0.4727 |
| Actual % prediction gain | 8.84% | 14.33% | 11.88% | 21.34% | 31.55% | 25.28% |

Table 2: SGCS comparison between predicted CSI and sample-and-hold (SGCS computed with respect to the ground-truth CSI at the application instance

| Application Instance 2 (25ms) ¦ SGCS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type↓ ¦ Dataset→ | 10 kmph | 20 kmph | 30 kmph | 40 kmph | 60 kmph |
| No-prediction | 0.7174 | 0.4705 | 0.3955 | 0.3716 | 0.3682 |
| Model test gcs | 0.7871 | 0.5815 | 0.4907 | 0.4531 | 0.4176 |
| Actual % prediction gain | 9.72% | 23.59% | 24.07% | 17.99% | 13.42% |

Observation 1-1: Gains in terms of SGCS are observed from AI/ML-based CSI prediction as compared to the sample-and-hold baseline over various UE speeds and prediction window (application instances).

- The observed gains are different across UE speeds

- The observed gains are different across different offset values between the CSI measurement and the predicted CSI application instance

In the below we present our evaluation results for model generalization with respect to UE speed. Here dataset of the same size are generating for various UE speeds, i.e., 10, 20, 30, 40, 60 and 120kmphr, according to the agreed SLS table in the Appendix. Then the following two cases are evaluated:

- Case 1: The AI/ML model is trained based on training dataset from one UE speed A, and then the AI/ML model performs inference/test on a dataset from the same UE speed A

- Case 2: The AI/ML model is trained based on training dataset from one UE speed A, and then the AI/ML model performs inference/test on a different dataset than UE speed A, e.g., UE speed B

Table 4 and Table 5 present results for Case 1 (diagonal elements) and Case 2 (off-diagonal elements). From the two tables it can clearly be observed that there is a severe degradation when the training dataset and the dataset for inference are drawn from different UE speeds. In particular, an AI/ML model trained for CSI prediction at high UE speeds performs below the baseline when applied to a lower speed scenario.

Table 3: Model generalization evaluation with respect to UE speed (diagonal entries=Case 1, off-diagonal entries=Case 2)

| Prediction - UE speed generalization Application instance 1 (10ms) ¦ % SGCS gains over sample-and-hold |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Train↓∕Test→ | 10 kmph | 20 kmph | 30 kmph | 40 kmph | 60 kmph | 120 kmph |
| 10 kmph | 8.8% | 8.0% | 1.5% | 11.4% | 7.4% | -2.1% |
| 20 kmph | -11.6% | 14.3% | 5.0% | 10.6% | 16.7% | -1.1% |
| 30 kmph | -27.2% | 1.6% | 11.9% | 15.9% | 25.2% | 0.3% |
| 40 kmph | -36.1% | -14.1% | 1.4% | 21.3% | 24.8% | 0.9% |
| 60 kmph | -51.2% | -29.2% | -19.5% | -5.6% | 31.6% | 8.9% |
| 120 kmph | -58.3% | -44.2% | -39.9% | -29.2% | -3.4% | 25.3% |

Table 4: SGCS comparison between predicted CSI and sample-and-hold (SGCS computed with respect to the groud-truth CSI at the application instance)

| Prediction - UE speed generalization Application instance 2 (25ms) ¦ % SGCS gains over sample-and-hold |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Train↓∕Test→ | 10 kmph | 20 kmph | 30 kmph | 40 kmph | 60 kmph |
| 10 kmph | 9.7% | 25.8% | 10.1% | 3.9% | 0.4% |
| 20 kmph | -27.2% | 23.6% | 22.8% | 9.1% | 2.1% |
| 30 kmph | -42.7% | -3.3% | 24.1% | 18.2% | 1.6% |
| 40 kmph | -56.1% | -17.1% | 7.3% | 21.9% | 2.9% |
| 60 kmph | -49.7% | -22.8% | -7.3% | 0.1% | 13.4% |

Observation 1-2: Severe performance degradation in terms of SGCS is observed when the AI/ML model is trained based on training dataset from one UE speed A, and then the AI/ML model performs inference/test on a different dataset than UE speed A, e.g., UE speed B

- The degradation is sever when UE speed B < UE speed A.

The above results suggest it may be beneficial if some form of model LCM assistance is provided to the UE so that the UE may activates the appropriate AI/ML functionality, i.e., CSI predictor. As an example, RAN1 may identify the different ranges of UE speeds which can be generalized by a single AI/ML model. Then AI/ML based CSI prediction over each of these ranges can be specified as one particular AI/ML based functionality. UE can also indicate which functionalies it supports. Accordingly, gNB may adjust the CSI measurement and reporting configuration, e.g., measurement and prediction window.

  - Spatial-frequency-domain CSI Compression

The training dataset and inference dataset is obtained from the SLS following the Rel-16 SLS assumptions (see Table 3-1).

Table 3-1. Assumption on dataset generation

| Parameter | Value |
| --- | --- |
| Scenario | UMa and UMi (7 macrocell sites and 3 sectors per site), InH |
| Carrier Frequency | 2GHz |
| Inter-BS distance | 500m for UMa and 200m for UMi |
| Simulation bandwidth | 10MHz (52 RB, 13 subbands) |
| UE distribution | 80% indoor, 20% outdoor |

Figure 3-1. Block diagram of ViT-AE based CSI compression

For the compression and reconstruction of CSI, the autoencoder (AE) consisting of the encoder for the compression task and the decoder for the reconstruction task is used (see Figure 1). As an AI model in the encoder and decoder, vision transformer (ViT) [3], a useful model to exploit the self-attention mechanism on 2D input data, is used with some modifications. In our evaluations, uniform quantization is used for the quantizer/de-quantizer and GCS is used as a loss function. The system-level channel data for training is generated from 512 drops and 420 UEs per simulation drop, and thus 215,040 training samples are obtained. For the inference dataset, 8400 samples are generated from 20 drops. Further, the ViT-based AE compresses and reconstructs the rank 1 eigenvector for 13 subbands. As an intermediate AI performance metric, the generalized cosine similarity (GCS) is used. The detailed hyper-parameters are shown in Table 3-2.

Table 3-2. Hyperparameters on CSI compression training

| Parameter | Value |
| --- | --- |
| Batch size | 256 |
| Number of epochs | 1000 with early stopping |
| Optimizer | Adam |
| Initial learning rate | 0.001 |
| Learning rate schedule | 0.96 at every 10 epochs |
| Training/validation split | 80% / 20% |

In Section 3.2.2, we evaluate the AI performance for three cases as agreed in RAN1#109-e:

- Case 1: (co-scenario evaluation) the training dataset is constructed by training inputs from a single configuration#A/scenario#A, and the testing/inference is performed for the same configuration#A/scenario#A.

- Case 2: (cross-scenario evaluation) the training dataset is constructed by training inputs from a single configuration#A/scenario#A, and the testing/inference is performed for a different single configuration#B/scenario#B.

- Case 3: (mixed-scenario evaluation) the training dataset is constructed by mixing training inputs from multiple configurations/scenarios, and the testing/inference is performed for a single configuration/scenario.

Note that we can verify the generalization capability through the cross-scenario evaluation in case 2 and mixed-scenario evaluation in case 3.

### 3.2.1 Evaluation results for co-scenario, cross-scenario, and mixed-scenario training/inference

In this section, we provide the initial simulation results for AI-based spatial-frequency domain CSI compression for co-scenario, cross-scenario, and mixed-scenario.

3.2.1.1 Performance-related KPI

[Case 1] In the co-scenario evaluation, the training dataset and the inference dataset are constructed from a single configuration/scenario. In these evaluations, the number of feedback bits is 338 in AI approaches while eType II CB uses 338 bits for UMa and 272 bits for UMi and InH. The GCS performances for different three scenarios are provided in Table 3-3.

Table 3-3. GCS performances for UMa, UMi, and InH scenarios (GCS).

|  | UMa | UMi | InH |
| --- | --- | --- | --- |
| ViT-based AE | .9376 | .9580 | .9530 |
| CNN-based AE | .8992 | .9101 | .9336 |
| eType II CB for ParamCombination=6 | .9348 | .9418 | .9399 |

Observed from Table 3-3, the performance of AI-based CSI compression varies with the channel scenario. For example, the GCS performance of ViT-based AE changes from 0.9376 to 0.9580. Whereas, the performance variation of eType II CB is relatively small. Note that, since the training and inference datasets are drawn from the same configuration/scenario, the AI-based results (i.e., ViT-based AE and CNN-based AE) could be used as an approximate upper bound in each scenario. These GCSs are used to measure the generalization capability in the subsequent cases.

[Case 2] In the cross-scenario evaluation, configuration/scenario of training dataset and that of inference dataset is different. Thus, through these evaluations, it can be investigated whether the AI model trained with a certain configuration/scenario can generalize the unseen configuration/scenario or not. Note that the eType II CB is free from the performance degradation caused by the cross-scenario since it is not a data-driven approach. In order to measure the generalization, we need to determine a performance metric. For example, in our evaluations, a degree of the performance degradation over the co-scenario performance (in a unit of percentage) can be used. If the degree of the performance degradation is marginal, this implies that the trained AI model generalizes two configurations/scenarios well. The evaluation results of cross-scenario evaluation are in Table 3.

Table 3-4. Cross-scenario performance for UMa, UMi, and InH (GCS).

| Inference Training | UMa (0.9348 for eType II) | UMi (0.9418 for eType II) | InH (0.9399 for eType II) |
| --- | --- | --- | --- |
| UMa | .9376 | .9563 (-0.2%) | .9008 (-5.4%) |
| UMi | .9314 (-0.6%) | .9580 | .9085 (-2.7%) |
| InH | .8858 (-5.5%) | .9064 (-5.4%) | .9530 |

In Table 3-4, we observe that the degree of GCS degradation in UMa and UMi scenarios is much smaller than that in InH scenario. For example, when the AI model is trained with the UMa dataset, the GCS for UMi and InH is degraded by -0.2 % and -5.4 %, respectively (see the second row in Table 5). Compared to eType II, the performance degradation between UMa and UMi is tolerable, implying that the AI model trained with UMa or UMi can be used for the cross-scenario. It is worth noting that, from these evaluations, the generalization capability among various configuration(s)/scenario(s) can be identified. Then, the identified generalization capability can be used to determine the way of dataset construction in the training. For example, compared to the generalization from UMa to UMi, it is difficult to generalize from UMa to InH. Hence, when we construct the mixed-scenario dataset (see Case 3 below), it would be better to include the InH dataset rather than UMi dataset.

Observation 2-1: The following observation were made for generalization performance across deployment scenario

- AI model generalizes well from UMa to UMi and vice versa.

- It is relatively difficult to generalize from UMa or UMi to InH.

3.2.1.2 Capability-related KPI

As a capability-related KPI, the number of model parameters (e.g., weights and biases) and the number of FLOPs are considered. In Table 7, we provide the capability-related KPI for ViT-based AE and eType II codebook-based PMI feedback.

Table 3-5 Capability-related KPI for ViT-based AE and eType II codebook-based PMI feedback

|  | The number of model parameters |  | The number of FLOPs |  |
| --- | --- | --- | --- | --- |
|  | UE | gNB | UE | gNB |
| ViT | 183,283 | 185,208 | 4.2×106 | 4.2×106 |
| CNN | 142,273 | 142,940 | 8.5×105 | 8.5×105 |
| eType II | - |  | 1.6×105 | - |

As shown in Table 3-5, the AI approach only requires the model parameters to run the AI model. Further, since the decoder at gNB reconstructs the CSI in the AI approach, the additional FLOPs are required. We observe that the number of FLOPs to perform the AE operations is much larger than eType II codebook-based PMI feedback. For example, at the UE side, the number of FLOPs for ViT and CNN are around 26 times and 5 times as large as that for eType II codebook, respectively. Even, including the FLOPs for the AE decoder operation, the number of FLOPs for ViT is around 53 times as large as that for eType II (10 times for CNN-based AE). In this respect, the effectiveness of the AI approach at the cost of these large amount of computational complexities can be investigated. Note that the number of FLOPs for ViT is around 5 times as large as that for CNN while the performance of ViT has around 6% gain over the CNN. When using the AI approach, the model selection can be performed using the trade-off between the performance and complexity.

Observation 2-2: The number of FLOPs to perform the AE operations is much larger than eType II.

### 3.2.2. Evaluation for NW-first Type-3 Training

Aiming at evaluating the feasibility of Type 3 training, RAN1#111 made the below agreement. It is crucial that a UE keeps few models as UE’s computational and storage resources are limited. In particular, it is infeasible if UE has to keep multiple models to operate with multiple network vendors. It is to be noted that multiple models could be required to cover the different scenarios and configurations that cannot be generalized by a single AI/ML model.

Agreement For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following evaluation cases for sequential training are considered for multi-vendors - Case 1 (baseline): Type 3 training between one NW part model and one UE part model - Note 1: Case 1 can be naturally applied to the NW-first training case where 1 NW part model to M>1 separate UE part models - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training UE part model is the same or a subset of the dataset for training NW part model - Note 2: Case 1 can be naturally applied to the UE-first training case where 1 UE part model to N>1 separate NW part models - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training NW part model is the same or a subset of the dataset for training UE part model - Companies to report the AI/ML structures for the combination(s) of UE part model and NW part model, which can be the same or different - FFS: different quantization methods between NW side and UE side - Case 2: For UE-first training, Type 3 training between one NW part model and M>1 separate UE part models - Note: Case 2 can be also applied to the M>1 UE part models to N>1 NW part models - Companies to report the AI/ML structures for the M>1 UE part models and the NW part model - Companies to report the dataset used at UE part models, e.g., same or different dataset(s) among M UE part models - Case 3: For NW-first training, Type 3 training between one UE part model and N>1 separate NW part models - Note: Case 3 can be also applied to the N>1 NW part models to M>1 UE part models - Companies to report the AI/ML structures for the UE part model and the N>1 NW part models - Companies to report the dataset used at NW part models, e.g., same or different dataset(s) among N NW part models - FFS: whether/how to report overhead of dataset

In light of the above, we provided the evaluation of Case 3 below.

- Case 3: For NW-first training, Type 3 training between one UE part model and N>1 separate NW part models

  - Note: Case 3 can be also applied to the N>1 NW part models to M>1 UE part models

  - Companies to report the AI/ML structures for the UE part model and the N>1 NW part models

  - Companies to report the dataset used at NW part models, e.g., same or different dataset(s) among N NW part models

FFS: whether/how to report overhead of dataset

Here we assumed network first Type 3 training based on two AI/ML model backbone structures, i.e., CNN and transformer (TF).

- Two network part models (decoders) are trained with their respective nominal encoders.

- The dataset for target CSI and latent space are shared by the two networks, i.e., (v1,z1) and (v2,z2) are provided to the UE

- UE trains its CSI generation part (encoder) by mixing the provided data set.

Figure. Network first Type-3 training for two network parts and single UE part.

The details of the network and UE part models is summarized below

Table 3-6 Details of the UE part and the two NW part models

|  | Enc | Enc* | Dec |
| --- | --- | --- | --- |
| NW#1 | - | TF w/ 2 layers | TF w/ 2 layers |
| NW#2 | - | CNN | CNN |
| UE#1 | TF w/ 2 layers | - | - |

As a benchmark (BM) for evaluation we have taken the following:

BM-1: Single-encoder and single-decoder (Type 1 training)

Moreover, each of the two networks provided 50,400 data samples, i.e., UE received 100,800 data samples. The following table summarizes our evaluation results.

Table 3-7 Results summary for NW first, Type-3 training of 2 NW parts and a single UE part.

|  |  | UE-enc | Enc* | NW-dec | SGCS |
| --- | --- | --- | --- | --- | --- |
| BM 1 (Type 1) | UE-NW#1 | TF (2) | - | TF (2) | .8303 |
|  | UE-NW#2 | TF (2) | - | CNN | .7748 |
| BM 2 (Type 3) | UE-NW#1 | TF (2) | TF (2) | TF (2) | .8359 |
|  | UE-NW#2 | TF (2) | CNN | CNN | .7476 |
| Test | UE-NWs | TF (2) | TF (2) | TF (2) | .5189 (-37.9%) |
|  |  | TF (2) | CNN | CNN | .4490 (-39.9%) |

It can easily be observed that the performance of the two networks degrade significantly if the two networks happen to be from different model backbone. In fact, 37.9% and 39.9% performance degradations were observed from BM-1 for TF-TF and TF-CNN pairing.

Observation 2-3: In NW-first training, i.e., Type 3 training between one UE part model and N>1 separate NW part models, severe performance degradation is observed when the multiple NW parts are from different backbone.

- Conclusion

In this contribution the following observations are made:

Observation #1: RRSNR as intermediate KPI may be advantageous on providing better emulation of the MU-MIMO DL throughput performance than GCS in some receiver assumptions. If the UE performs inter-layer interference nulling, this advantage is lost.  This observation holds to other proposed intermediate KPIs such as numerical spectral efficiency gap and NEDG.

Observation #2: In MU-MIMO scenario, when the gNB does not directly apply the reconstructed precoder, RRSNR does not give a practical advantage in terms of emulating the DL MU-MIMO throughput performance than SGCS. This observation holds to other proposed intermediate KPIs such as numerical spectral efficiency gap and NEDG.

Observation 1-1: Gains in terms of SGCS are observed from AI/ML-based CSI prediction as compared to the sample-and-hold baseline over various UE speeds and prediction window (application instances).

- The observed gains are different across UE speeds

- The observed gains are different across different offset values between the CSI measurement and the predicted CSI application instance

Observation 1-2: Severe performance degradation in terms of SGCS is observed when the AI/ML model is trained based on training dataset from one UE speed A, and then the AI/ML model performs inference/test on a different dataset than UE speed A, e.g., UE speed B

- The degradation is sever when UE speed B < UE speed A.

Observation 2-1: The following observation were made for generalization performance across deployment scenario

- AI model generalizes well from UMa to UMi and vice versa.

- It is relatively difficult to generalize from UMa or UMi to InH.

Observation 2-3: In NW-first training, i.e., Type 3 training between one UE part model and N>1 separate NW part models, severe performance degradation is observed when the multiple NW parts are from different backbone.

And the following proposals are provided:

Proposal #1: For the evaluation of the AI/ML based CSI feedback enhancement, while reporting the computational complexity of pre-processing and post-processing the following are considered.

- Estimated raw channel matrix per each  frequency unit as an input for pre-processing

- Precoding vectors per each  frequency unit as an output of post-processing

Note: frequency unit can be set to 4 RB or 2 RB for 15KHz SCS and 30KHz SCS, respectively.

Proposal 2: For the CSI payload size calculation for AI/ML-based CSI compression as well as the legacy TypeII codebook, the payload size is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE.

Note: Whether the CSI payload is calculated as each CSI reported payload with a given rank or as max allowed bits at the given rank can additionally be reported.

Proposal#3:  For the evaluation of Type 2 training, for multi-vendors training Case 2, i.e., one NW part model and M>1 separate UE part models, and Case 3, i.e., one UE part model and N>1 separate NW part models, evaluate and compare the following two training scenarios:

Scenario 1: training in a single session

Scenario 2: training in multiple sequential training sessions, i.e, one UE part and one network part at each training session.

Proposal #4: For the evaluation of the AI/ML based CSI feedback enhancement, adopt SGCS for rank>1 as weighted average over all ranks

where  is an eigenvalue of the channel covariance matrix corresponding to

Note:  is the jth eigenvector of the target CSI at resource unit i and K is the rank.  is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.   denotes the average operation over multiple samples.

Proposal #5: For the evaluation of the AI/ML based CSI feedback enhancement, deprioritize discussion on additional intermediate KPIs.

Proposal#6: For the evaluation of AI/ML based CSI prediction sub use case, the following evaluation cases are considered,

- Case 1: The AI/ML model is trained based on training dataset from one UE speed A, and then the AI/ML model performs inference/test on a dataset from the same UE speed A

- Case 2: The AI/ML model is trained based on training dataset from one UE speed A, and then the AI/ML model performs inference/test on a different dataset than UE speed A, e.g., UE speed B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple UE speeds including UE speed A and a different dataset than UE speed A, e.g., UE speed B, and then the AI/ML model performs inference/test on a dataset from a single UE speed from the multiple UE speeds, e.g.,  UE speed A, UE speed B.

  - Note: Companies to report the ratio for dataset mixing

- Note: UE speed includes 10Kmphr, 20kmphr, 30kmphr, 60kmphr and 120kmphr

- Appendix

Table 5-1 SLS-based EVM

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FFS |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

Table 5-2 LLS-based EVM

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| Delay spread | 30ns or 300ns |
| Channel estimation | Realistic channel estimation algorithms (e.g. LS or MMSE) as a baseline, FFS ideal channel estimation |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |

## Frequency-domain CSI Extrapolation:

The benefits of AI-based CSI prediction can be applied to extrapolation in other domains (e.g. frequency, space; we use “extrapolation” to refer to those domains).  For example, a gNB can configure a UE to send it CSI reports for an inactive bandwidth part (BWP).  The UE can use received DL CSI-RS on an active BWP and then perform AI-based CSI extrapolation to infer CSI on the inactive BWP.  The gNB can then decide whether to configure the UE to switch to the inactive BWP, depending on the CSI reports for the active and inactive BWPs.

The problem that we are considering, along with the neural network architecture that we developed to address it, is shown in Fig. 2-1.  In particular, the objective is to extrapolate 72+N CSI-RS observations to CSI on the 288 SCs in the region in yellow.  In this case, if N > 0, then some interpolation is performed to infer the 288 SCs in the region in yellow (hence, we refer to our approach as “extra(inter)-polation”).

Fig. 2-1: AI-based CSI frequency extra(inter)-polator

Our evaluation results are shown in Fig. 3, where we compare the performance of our AI-based CSI frequency extra(inter)-polator (which we denote “TestNet”) with an ideal 2-D Wiener filter and an AI-based channel estimator (which we denote “EDSR”) [8].  For both of the latter approaches, we placed CSI-RS on every 4th subcarrier in the region in yellow in Fig. 2.  These results were generated using the following parameters:

- UE speed of 5 km/h

- Delay spread of 100 ns

- TDL-C (NLoS) channel model

- Carrier frequency of 3.5 GHz

- gNB has Nt = 1 transmit antenna and Nr = 1 receive antenna

- K = 48 resource blocks.

Fig. 2-2: AI-based CSI frequency extra(inter)-polation error

The parameters of the neural network architecture that we developed in this case are shown in Figures 4 and 5.

Fig. 2-3: Parameters of AI-based CSI frequency extra(inter)-polator

Fig. 2-4: Parameters of “Basic Block” in AI-based CSI frequency extra(inter)-polator

The hyper-parameters that we used for model training are shown in Table 3.

Table 2-1: Hyper-parameters for model training

| Parameter | Value |
| --- | --- |
| Initial batch size | 16 |
| Batch size schedule | 2 at (60, 120, 180, 240, 300) epochs |
| Number of epochs | 300 |
| Optimizer | Adam |
| Initial learning rate | 0.001 |
| Learning rate schedule | 0.1 at (100, 200, 250) epochs |
| Training/testing split | 80% / 20% |

Observation 2-1: AI-based CSI frequency extrapolation can be enhanced by utilizing additional CSI-RS observations in the band to be extrapolated, which amounts to AI-based CSI frequency extra(inter)-polation.

## Temporal-spatial-frequency-domain CSI compression

In the following the performance of temporal-spatial-frequency domain compression is provided. As it is detailed in [9.2.2.2] and depicted in Fig. 3.4.1 below, two approaches are considered, namely, Approach 1 gNB-side prediction and UE-side prediction. Thus, this sub use case can be considered as joint CSI prediction and compression.

Fig. 4-1.  Approach 1 gNB-side prediction vs. Approach 2  UE-side prediction

The performance is evaluated according to the agreed LLS-based evaluation [9] with the parameters selected as shown in Table 4.1.

Table 4-1 Parameters for LLS-based evaluation

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD , OFDM |
| Carrier frequency | 2GHz |
| Bandwidth | 10MHz |
| Subcarrier spacing | 15kHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C |
| UE speed | 3kmhr, 10km/h, |
| Delay spread | 300ns |
| Channel estimation | ideal |
| Rank per UE | Rank 1 |
| CSI-RS resources | Periodic with 5ms |
| CSI report | 4ms delay between measurement and report |

First, let us evaluate the compressibility of time-correlated CSI. For this the setups shown in Fig. 4-2 are considered. Two setups are depicted in the figure, i.e., suitable for (a) aperiodic burst CSI-RS resources and (b) periodic and semi-persistent CSI-RS resources.

Fig. 4-2. Set up to test time-domain compressibility of CSI

For aperiodic CSI-RS resources, the UE compresses CSI from  measurements after receiving a burst of CSI-RS resources at different time instants. In particular, per each layer, M eigenvectors matrices, denoted as  V1,V2,⋯VM,  are considered.  for  corresponds to the  m-th measurement, m=1,2,⋯,M, and consists of eigenvectors for the subbands, i.e., Vm=vm,1vm,2⋯vm,Ns. The autoencoder in Fig. 4-2. (a) compresses these measurements to generate the codeword   which potential can be partitioned to M sub-codewords C=[c1c2⋯cM. The CSI for the M measurements can then be reported in either a single CSI report or in multiple CSI reports.

For, periodic and semi-persistent CSI-RS resources, as shown in Fig. 3.4.2 (b), a CSI corresponding to a single measurement can be reported at a time. Then the decoder considers the codewords reported for the past  measurements, i.e., cm-M+1,⋯,cm-1and concatenate them with current codeword to reconstruct the CSI. This way the time-domain correlation can still be exploited for spatial-frequency-time domain CSI compression even if the UE reports a CSI for a single measurement at a time.

The detailed AI/ML model and training configuration for the autoencoder is depicted in Fig. 3.4.3. A Bi-LSTM based neural network is employed at both the encoder and decoder. At the encoder, the input layer dimension is Ns×2Nt due to concatenation of real and imaginary precoder parts of Ns eigenvectors. Further, the first and second hidden layers are of dimension Ns×8Nt. The third Bi-LSTM layer generates a P dimensional codeword per subband such that the total output dimension is Ns×P. This is converted in to a 1D sequence of length NsP using a reshape layer. Finally, a 1-bit Quantizer converts them into a discrete sequence in the set {-1,1}.

The decoder input layer concatenates the current and past M-1 measurement reports. The following reshaping layer converts the input in to Ns×PM dimension. This is further processed by two Bi-LSTM layers of dimension Ns×16Nt. The eigenvectors are then reconstructed by a third Bi-LSTM layer by generating a Ns×2Nt output sequence.

| AE Training configuration |  |
| --- | --- |
| Dataset size | 300000 |
| Number of epochs | 150 |
| Loss function | Cosine similarity |
| Optimizer | Nadam |
| Learning rate | 0.001 |
| Train test split | 80%-20% |

Fig. 4-3. Set up to test time-domain compressibility of CSI

Fig. 4-4. shows the performance of spatial-frequency-time-domain compression in terms of GCS with respect to various payload sizes. The performance of the baseline scheme, i.e., Rel-16 eType II codebook, with parameter combination 1-8 as specified TR 38.214 is also provided. The payload size is determined by the payload of a single layer PMI for the 8 parameter combinations for Rel-16 eType II codebook. The measurements are 5ms apart and parameters listed in Table 3.4.1 are considered. One observation is that adding time-domain in the compression domain achieves a significant advantage in terms of overhead reduction. As an example when M=5 measurements are considered, to achieve GCS=0.85, over 185% payload size reduction is achieved by the autoencoder over Rel-16 CB. On the other hand, when the payload size is fixed to 157 bits per layer over 7% gain is observed in terms of GCS. This is significant as compared to the 57% payload overhead reduction achieved by an AI-based spatial-frequency-domain CSI compression.

Fig. 4-4. Performance of spatial-frequency-time-domain compression with variable number of measurements (M=1, 3, 5)

Observation 4-1: Adding the time-domain in the CSI compression domains, i.e., CSI compression in spatial-frequency-time domains, achieves a higher compression, i.e., further reduction in CSI report overhead, as compared to spatial-frequency domain CSI compression.

Table 4.1 Complexity in terms of mega-FLOPs and model size

| Index | Payload | Rel-16 CB complexity # of FLOPs (Mega-FLOPs) For paramCombination-r16=1-8. | Bi-LSTM model complexity (# of FLOPs (Mega-FLOPs) |  |  |  | Model Size (# of parameters) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | UE side | Encoder | Decoder |  |  | Encoder | Decoder |
|  |  |  |  | M=1 | M=3 | M=5 |  |  |
| 1 | 55 | 0.032 | 44.5 | 171.7 | 172.5 | 173.4 | 17,16,404 | 66,15,616 |
| 2 | 84 | 0.032 | 44.6 | 171.9 | 173.2 | 174.4 | 17,20,698 | 66,23,808 |
| 3 | 100 | 0.087 | 44.7 | 172 | 173.5 | 175 | 17,22,872 | 66,27,904 |
| 4 | 157 | 0.087 | 45 | 172.5 | 175.1 | 177.6 | 17,34,012 | 66,48,384 |
| 5 | 214 | 0.087 | 45.2 | 172.9 | 176.4 | 179.8 | 17,43,248 | 66,64,768 |
| 6 | 272 | 0.087 | 45.5 | 173.4 | 177.6 | 181.9 | 17,52,772 | 66,81,152 |
| 7 | 230 | 0.163 | 45.3 | 173.1 | 176.7 | 180.3 | 17,45,602 | 66,68,864 |
| 8 | 564 | 0.163 | 47 | 175.8 | 185 | 194.1 | 18,13,124 | 67,75,360 |

Table 3.4.1 provides the complexity comparison of the aforementioned autoencoder-based CSI compression with respect to Rel-16 CB in terms of mega-FLOPs. Two observations can be made from the table. There is a multiple order of increase on computational complexity is incurred by AI-based CSI compression as compared to PMI derivation based on Rel-16 CB. However, in order to have the full picture of the requirements for AI-based CSI compression, the impact of such increase in computational complexity on inference latency shall be studied.  This is particularly important as the level of parallelization allowed for the tasks in AI-based CSI compression and in legacy codebooks based CSI computation might be different. Additional observation is that the increase in computational complexity is invariant with respect to the number of time-domain measurements as the complexity per measurement remains the same at the encoder.

Observation 4-2: AI-based CSI compression incurs a multiple order of increase in the computational complexity (measured in terms of number of FLOPs) as compared to CSI computation based on Rel-16 eType II codebook.

- The increase in FLOPs is invariant with respect to the number of time-domain CSI measurements considered in the compression, i.e., same for spatial-frequency-domain and spatial-frequency-time-domain compression.

- The model size remains in the same range for spatial-frequency-domain and temporal-spatial-frequency -domain compression

In the following we provide the performance for joint prediction and compression. In this regard, as shown in Fig. 4-5., we considered a UE-side prediction based on the (a) eigenvectors and (b) full-channel matrix.

Fig. 4-4. UE-side prediction: eigenvector-based and channel matrix-based prediction

The performance of the two approaches (Approach 2-1 and Approach 2-2) is provided in Fig. 4-5 in terms of GCS vs. prediction delay. The performance is evaluated based on the parameters list in the Table 3.4.1 with UE speed set to 10km/hr. As a baseline, a Rel-16 CB based reporting with no prediction (sample-and-hold) is considered. Moreover, the two payload sizes per layer are considered, i.e., 272 bits and 564 bits. The GCS is computed between the recovered (predicted and compressed) CSI and the ground truth value (genie-aided CSI). For the baseline method, the GCS is simply computed between the reported CSI and the ground truth at a certain prediction delay value. The first and obvious observation is that the GCS performance degrades as the prediction delay increases. However, this degradation is severe (higher slop) if no prediction is applied. In this regard, joint compression and prediction out performs the baseline by 32% at prediction delay of 20ms. Another observation is that most of the gain is attributed to the CSI prediction aspect compared to the compression aspect as the gain from increasing the payload size from 272 to 564 is less significant ( 2%) as opposed to the 32% gain from compression and prediction.

Fig. 4-5. Performance for UE-side prediction: eigenvector-based and channel matrix-based prediction

Observation 4-3: Significant gain is observed for UE-side joint CSI prediction and compression as compared to Rel-16 CB reporting without prediction.

- Most of the performance gain is attributed to CSI prediction as compared to compression.

- The prediction based on full channel matrices outperforms prediction based on eigenvectors.

In the following the performance of  gNB-side prediction (Approach 1) and UE-side prediction (Approach 2) are presented. It can be noticed from Fig. 4-5 that the eigenvectors-based prediction does not work well as compared to full-channel matrix based  prediction. One challenge for Approach 1 is that the overhead to make the full-channel matrix H available at the gNB. Note here that the full-channel matrix can be decomposed as  H=UΣVH for the right eigenvectors in V where SVDHHH=VΣ2VH and left eigenvectors U given as  SVDHHH=UHΣ2U. Conventionally, the right eigenvectors in V are sent to gNB via CSI feedback. Even if sending the full-channel matrix H which is Nr×Nt per reporting subband as CSI feedback the reporting overhead is hindering. However, the gNB is usually interested in channel information corresponding to few dominant layers (low rank). In this case, the UE may share channel matrix corresponding to the few dominant vectors, e.g.,  Hl=ulvlH for layers, l=1,2...   Note here that H=lλlHl .

- gNB-side prediction with availability of u and V vectors at the gNB

- Payload 564 bits

- Payload =272 bits

Fig. 4-6.  Performance for gNB-side prediction (with the availability of left and right eigenvectors U and V. i.e., Hl=uvlH) paylaoad size set to 274,  time-domain compression for M=3 past and future time instants for UE-side and gNB-side, respectively, are compressed and reported).

In Figure 4-6 (b) and (c), it can be noticed that gNB-side prediction performs well with comparable performance as UE-side prediction, if the UE-side eigenvectors are available at the gNB side. Note here the same payload size is used to feedback the {v}  and {u+v} vectors, in the UE-side prediction and gNB-side prediction, respectively. In Fig. 4-6 (b), the relatively high payload size, i.e., 564 bits, ensured the accurate delivery of the left-side and right-side eigenvectors; therefore, performance is dictated by the prediction performance rather than by feedback reconstruction performance. Additionally,  Fig. 4-6 (c), provides the result for moderate payload size, i.e., 272bits. Herein, even if the performance of UE-side prediction and gNB-side prediction degraded from the one based on uncompressed full-channel matrix, the degradation is not substantial. Thus, both UE-side and gNB-side prediction asymptotically achieved the prediction without (ideal) compression.

Observation 4-4: gNB-side prediction performs well as UE-side prediction if the UE-side eigenvectors are available at the gNB side.

- The UE-side eigenvectors can be compressed and reported with overhead that scales up with the reported rank.

# References

[1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Dec, 2021.

[2] R1-2203898, Samsung, “Representative sub use cases for CSI feedback enhancement” 3GPP TSG RAN1#109-e

[3] A. Dosovitskiy, L. Beyer, A. Kolesnikov, D. Weissenborn, X. Zhai, T. Unterthiner, M. Dehghani, M. Minderer, G. Heigold, S. Gelly, J. Uszkoreit, N. Houlsby “An Image is Worth 16x16 Words: Transformers for Image Recognition at Scale,” Proc. Int. Conf. Learning Representations (ICLR), May 2021.

[3] R1-2203896, Samsung, “General aspects of AI ML framework and evaluation methodology” 3GPP TSG RAN1#109-e

[4] TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, V17.0.0 (2022-03)

[5] R1-2007151, Samsung, “Moderator summary for multi-beam enhancement: EVM”, 3GPP TSG RAN1 #102-e

[6] R1-2007151, Fraunhofer IIS, Fraunhofer HHI, “MIMO Enhancements for Rel.-18”, 3GPP TSG RAN Rel.-18Workshop

[7] R1-2203897, Samsung, “Views on evaluation of AI/ML for CSI feedback enhancement” 3GPP TSG RAN1#109-e

[8] L. Li, H. Chen, H.H. Chang, and L. Liu, “Deep residual learning meets OFDM channel estimation”, IEEE Wireless Communications Letters, vol. 9, no. 5, pp. 615-618.

[9] Chair's notes RAN1#109-e

[10] Chair's notes RAN1#110

[11] Chair's notes RAN1#111