3GPP TSG RAN WG1 #112	R1-2301277

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.7.2

Source:	Samsung

Title:	Enhancements on cell DTX/DRX mechanism

Document for:	Discussion and decision

- Introduction

A WI on network energy saving for NR was endorsed in [1]. One objective is to specify enhancement on cell DTX/DRX mechanism as following.

Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3]

- Note: No change for SSB transmission due to cell DTX/DRX.

- Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

This contribution discusses how to support cell DTX/DRX mechanism.

- Definition of cell DTX/DRX

The definition of cell DTX/DRX was discussed during the SI phase. Although cell DTX/DRX is a definition from gNB’s side, UE behaviour should be clarified/defined when configured with cell DTX/DRX in the WI phase. For active durations, the common understanding is that UE behaviour is the same as legacy. For non-active durations, the following examples were discussed and captured in the TR 38.864 [2]

-	Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods.

-	Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals)

-	Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS).

-	Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

In addition, there is a note in the WID indicates that there is no change for SSB transmission due to cell DTX/DRX. There can be three interpretations for the note.

- Interpretation 1: SSB transmission is independent of cell DTX, i.e., SSB can be transmitted during non-active durations.

- Interpretation 2: The durations for SSB transmission are considered as active durations of cell DTX.

- Interpretation 3: gNB configuration/indication ensures that the durations for SSB transmission are always within cell DTX active durations. The active durations of cell DTX/DRX is not determined/indicated by the SSB symbols.

Interpretation 1 is the simplest solution with no/minimal spec impact. The SSB transmission follows legacy and is not impacted by cell DRX/DRX.

Interpretation 2 is also a simple solution with marginal spec impact. For Interpretation 2, the SSB transmission also follows legacy, the difference from Interpretation 1 is that the SSB symbols implicitly configures the additional active durations. The latency can be thus reduced and gNB can have a larger probability to go to sleep. An example is given in Figure 1 for illustration. For Interpretation 1, the active/non-active durations are determined based on the agreed parameters periodicity, start slot/offset, on duration, the PDCCH is assumed not to be monitored by a UE during non-active durations. For Interpretation 1, UE will not monitor the PDCCH#1, if gNB would transmit the PDCCH, gNB needs to defer the PDCCH #1 to a PDCCH MO on the active symbols, e.g., PDCCH#2. On the contrary, for Interpretation 2, the SSB symbols will be taken as active durations and the UE will monitor the PDCCH on the SSB symbols, in this case, gNB can schedule PDCCH#1 and the increased latency for Interpretation 1 can be avoided. In addition, if there is no other DL transmission in the 2nd cycle, gNB can go to sleep for the active durations, the network energy saving gain can be thus increased by reducing the active RF durations. To reduce UE monitoring PDCCH in the 2nd cycle, dynamic indication of on duration in the scheduling DCI can also be considered.

Figure 1

Interpretation 3 requires that SSB transmission always in the active durations when configured with cell DTX, for example, it can be achieved through alignment SSB transmission cycle and cell DTX cycle.

Proposal 1: Down-select from the following three options for interpretation of ‘No change for SSB transmission due to cell DTX/DRX’.

- Option 1: SSB transmission is independent of cell DTX, i.e., SSB can be transmitted during non-active durations.

- Option 2: The durations for SSB transmission are considered as active durations for the determination of the active durations of cell DTX.

- Option 3: gNB configuration/indication ensures that the durations for SSB transmission are always within cell DTX active durations.

In addition, the 2nd Note in the WID requires to avoid the impact to IDLE/INACTIVE UEs. PDCCH monitoring in CORESET0, SIB1 reception and PRACH transmission should follow the same rules of SSB transmission to avoid the impact of initial access.

Proposal 2: PDCCH monitoring in CORESET 0, SIB1 reception and PRACH transmission follow the same rule of SSB transmission when configured with cell DTX/DRX.

The other channels/signals received/transmitted for a UE in RRC_CONNECTED can be divided into two types according to whether there is an associated PDCCH or DCI format. Channels/signals configured by higher layer signalling, e.g., PDCCH, SPS PDSCH, SR and CG PUSCH, should only be allowed to be received/transmitted during active durations similar as PDCCH monitoring for UE DRX. There can be three options for a channel/signal with a DCI format.

Option 1: The reception/transmission of a channel/signal with a DCI format is independent of cell DTX/DRX, i.e., UE can receive/transmit channels/signals with a DCI format during non-active durations of cell DTX/DRX.

Option 2: The symbols of the reception/transmission of a channel/signal with a DCI format are considered as active for the determination of the active durations of cell DTX/DRX, respectively.

Option 3: The reception/transmission of a channel/signal with a DCI format is not allowed during non-active durations of cell DTX/DRX.

Option 1 is similar as Interpretation 1. Dynamic scheduling follows legacy without considering cell DTX/DRX states. Whether to schedule a PDSCH/PUSCH in a non-active duration is totally up to gNB. UE simply follows the indication of DCI.

Option 2 is similar as Interpretation 2. The only difference from Option 1 is that Option 2 implicitly indicates the PDSCH/PUSCH symbols as active. For the similar reason as SSB transmission, Option 2 could help reduce latency as well as provide network energy saving gain compared with Option 1 with the cost of additional spec impact.

Option 3 requires additional restrictions on dynamic scheduling and may result in latency issue. It is agreed by RAN2 that cell DTX/DRX can be configured by higher layer signaling, e.g., periodicity, start slot/offset, on duration, the active durations determined by the semi-static configuration may not be suitable for all the traffic for a serving cell. For example, as shown in Figure 2, gNB receives an SR for a latency sensitive traffic and can transmit a PDCCH with a UL grant in the first active duration. If the PUSCH transmission is not allowed during non-active durations, gNB has to schedule the PUSCH in the next active duration with the cost of increased latency.

Figure 2

Proposal 3: The reception/transmission of a channel/signal with a DCI format is not impacted by cell DTX and cell DRX, respectively.

- FFS: Whether the symbols of the reception/transmission of a channel/signal with a DCI format are considered as active for the determination of the active durations of cell DTX/DRX, respectively.

- Configuration/indication of cell DTX/DRX

The following agreements were made in RAN2#120 meeting.

Agreements

1 	Clarify previous agreement to: periodic cell DTX/DRX pattern is configured by UE-specific RRC.   Periodic cell DTX/DRX can be activated/deactivated by L1/L2 signalling and UE-specific RRC signaling.

2 	Capture in TR 38.864 that both UE specific and common L1/L2 signalling can be considered for at least activating/deactivating the cell DTX/DRX pattern, per the agreement in 119b-e.

3	Cell DTX and Cell DRX modes can be configured and operated separately (e.g. one RRC configuration set for DL and the other set for UL).  Cell DTX/DRX can also be configured and operated together.

4	It is up to NW whether legacy UEs can access cells with Cell DTX/DRX

5	Cell DTX/DRX can be configured per serving cell and can be applicable for different cells in CA.  No additional RAN2 impacts or enhancements are foreseen.

6	Whether to support multiple Cell DTX/DRX configurations can be discussed later in the normative phase.

7	At least the following parameters can be configured per cell DTX/DRX configuration: periodicity, start slot/offset, on duration.  Details related to UE behaviour can be discussed during WI phase

8	From RAN2 perspective DTX/DRX is feasible

This section discusses the detailed mechanisms of the configuration/indication of cell DTX/DRX.

Semi-static configuration of cell DTX/DRX

It has been agreed by RAN2 that Cell DTX and Cell DRX modes can be configured and operated separately or jointly. Separate operation can provide the best flexibility, it can be up to the gNB to configure the same or different parameters for cell DTX and cell DRX. An example is given in Figure 3 for different cycles for cell DTX and cell DRX.

Figure 3

A simple implementation for joint operation of cell DTX/DRX is that Cell DTX and DRX share the same higher layer parameters, e.g., same periodicity, start slot/offset, on duration. An example is given in Figure 4.

Figure 4

Proposal 4: For joint operation of cell DTX and DRX, a set of higher layer parameters applies to both cell DTX and DRX

- The set of parameters at least includes periodicity, start slot/offset, on duration.

gNB may need to change the cell DTX/DRX configuration based on the arriving traffic. With one configuration for cell DTX/DRX, gNB needs to reconfigure the cell DTX/DRX configurations for all the UEs in a cell if gNB wants to change the cell DTX/DRX configuration. In order to save the energy consumption for the transmitting reconfiguration parameters, multiple configurations for cell DTX/DRX can be provided with L1/L2 signalling activating one of the configurations. Part of the set of parameters for the multiple configurations can be common, and a default configuration for cell DTX/DRX can be further studied.

Proposal 5: Support one or multiple configurations for cell DTX/DRX provided by a higher layer signaling and a fast cell DTX/DRX activation/deactivation mechanism via L1/L2 signaling.

MAC CE activation/deactivation of cell DTX/DRX

When multiple configurations of cell DTX/DRX are provided, a MAC CE can provide indication on the activation/deactivation information of the multiple configurations. For example, the UE can receive a PDSCH that includes a MAC CE indicating one or multiple sets of configurations of cell DTX/DRX are activated.

Moreover, since the RRC reconfiguration procedure typically takes longer delay, parameters for configurations of cell DTX/DRX can be updated by MAC CE. For example, the MAC CE can include at least one of a periodicity, a start slot/offset, or an on-duration for each of the activated set of configurations of cell DTX/DRX.

Proposal 6: Support MAC CE based activation/deactivation and parameter update of configurations for cell DTX/DRX.

Dynamic indication of the configuration of cell DTX/DRX

After RRC configuration, L1 signaling can be used to activate/deactivate/modify the DTX/DRX pattern to ensure the QoS of all the traffic. A cell/group common DCI format can be used for signaling overhead reduction as well as to align DRX among multiple UEs in the cell.

There can be three options for the indication.

Option 1: Dynamically indicates whether the onDuration timer is started for the next one for more cell DTX/DRX cycles.

Option 2: Dynamically indicates which cell DTX/DRX configuration is activated if multiple cell DTX/DRX configurations are provided.

Option 3: Dynamically indicates whether a next period is active or non-active.

Option 4: Dynamically indicates parameter update for the set of configuration of cell DTX/DRX.

Option 1 is similar as the wake-up indication for UE DRX, the simplest solution could be using 1 bit to indicate whether to start the onDuration timer for the next cell DTX/DRX cycle. It can also indicate multiple cell DTX/DRX cycles to save the L1 signaling overhead.

Option 2 can be used for the case of multiple configurations for cell DTX/DRX as discussed in Section 3.1. If a UE is provided with multiple configurations, one of the configurations can be activated by the cell/group common DCI format. Option 2 can joint operate with Option 1. For example, the DCI can indicate both a configuration for cell DTX/DRX and whether to start the onDuration timer of the next cycle for the configuration.

Option 3 provides best flexibility and can work independent of the higher layer parameter set of {periodicity, start slot/offset, on duration}. An example is given in Figure 5 for illustration. A period of 4 slots is indicated by the DCI and a 4 bits bitmap is used with each bit indicates active or non-active of one slot in the period. In this case, slot n is indicated as active and slot n+1, n+2 and n+3 are indicated as non-active.

Figure 5

Option 4 allows the DCI format to override the parameters in the set of configurations for the cell DTX/DRX.

In addition to the four options, dynamic indication of at least one cell DTX/DRX timer can also be considered based on RAN2’s progress.

It has also been agreed by RAN2 that UE specific L1 signalling can be considered for at least activating/deactivating the cell DTX/DRX pattern. A simple solution could be reusing the existing DCI formats scheduling PDSCHs/PUSCHs. Both explicit indication with a new field and implicit indication can be considered. For explicit indication of UE-specific DCI format, in order to minimize the bit of the new field, the indication can be restricted to the scheduled cell. For implicit indication, the symbols of the dynamic scheduled PDSCH/PUSCH can be used as active durations of the scheduled cell.

Proposal 7: Support a cell/group common or UE-specific DCI format to activate/deactivate/modify the configured cell DTX/DRX, the following options or the combinations of the options can be considered.

- Option 1: Dynamically indicates whether the onDuration timer is started for one or more cell DTX/DRX cycles

- Option 2: Dynamically indicates which cell DTX/DRX configuration is activated if multiple cell DTX/DRX configurations are provided.

- Option 3: Dynamically indicates whether a next period is active or non-active.

- Option 4: Dynamically indicates parameter update for the set of configuration of cell DTX/DRX.

- FFS: Dynamically indicates the cell DTX/DRX timers.

- FFS: Implicit indication of cell DTX/DRX.

When configured to monitor the cell/group common DCI format to activate/deactivate/modify the configured cell DTX/DRX, it is possible that UE may miss the DCI format. To ensure UE and gNB to have the same understanding on the cell DTX/DRX state, HARQ-ACK feedback should be supported for the cell/group common DCI format. UE behaviour should also be clarified in case of DCI missing. Considering the UE behaviour can depend on the detailed design of the DCI format, this issue can be discussed later when the contents of the DCI format become clear.

Proposal 8: Support HARQ-ACK feedback for the cell/group common DCI format to activate/deactivate/modify the configured cell DTX/DRX.

- FFS: The UE behaviour if the DCI format is missed.

- Joint operation of cell DTX/DRX and UE DRX

Cell DTX/DRX aims for reducing the network energy saving, the configuration needs to consider all the UEs in a cell. For example, if there is traffic arrival for a single UE, the cell should be in active duration and there will be data transmission between the network and the UE while the other UEs in the cell may not have to monitor the PDCCH. In this case, UE DRX can be configured to avoid unnecessary PDCCH monitoring during cell active durations. When configured with both cell DTX/DRX and UE DRX, the active time of cell DTX/DRX and UE DRX would not be exactly the same, otherwise, there is no need to configure the UE DRX when cell DTX/DRX is configured, UE can simply follow cell DTX/DRX.

There are four cases as shown in Figure 6.

Case 1：The duration is determined as active for both cell DTX/DRX and UE DRX.

Case 2：The duration is determined as non-active for both cell DTX/DRX and UE DRX.

Case 3：The duration is determined as active for cell DTX/DRX but non-active for UE DRX.

Case 4：The duration is determined as non-active for cell DTX/DRX but active for UE DRX.

For Case 1, UE behaviour is the same as neither cell DTX/DRX nor UE DRX is configured.

For Case 2, UE should follow cell DTX/DRX to avoid unnecessary reception/transmission, e.g., SPS PDSCH, SR.

For Case 3, UE should follow UE DRX to avoid unnecessary PDCCH monitoring.

For Case 4, UE can either follow cell DTX/DRX to avoid unnecessary reception/transmission or UE can follow UE DTX/DRX considering the gNB may not always dynamically indicates the cell DTX/DRX state or UE may miss the DCI indication for cell DTX/DRX.

Figure 6

If Option 2 is adopted for Case 4 and Proposal 4-1 is adopted for the other cases, a UE may not be able to monitor the PDCCH if case 1 is not ensured by gNB. The alignment for cell DTX/DRX and UE DRX should be considered at least to ensure there would be a period determined as active for both cell DTX/DRX and UE DRX for PCell. For example, a simple solution could be aligned the starting time of the cycle for cell DTX/DRX and UE DRX for PCell.

Cell DTX/DRX is configured per cell while the UE DRX is common for all the configured cells of a MAC entity. Alignment cell DTX/DRX and UE DRX is not necessary as long as the UE is ensured to be able to monitor the PDCCH. It would complicate the spec as well as adding additional restriction on the cell DTX/DRX configurations, as a result, the network energy saving gain may degrade.

Proposal 9: Study whether/how to align the starting time of cycles of cell DTX/DRX and UE DRX for PCell.

- FFS: SCell

- Joint operation of cell DTX/DRX and other collision handling

When configured with cell DTX/DRX, it can be defined by the specification or configured by higher layer signalling that UE may not be able to transmit a semi-static configured PUCCH during non-active period. If there is another overlapping PUCCH, whether to first perform multiplexing or cancelation should be clarified. An example is given in Figure 7 for illustration. A PUCCH with SR overlaps with a PUCCH with CSI, the PUCCH with SR is within the active duration while the PUCCH with CSI overlaps with the non-active duration. If the UE first determines not to transmit the CSI due to the cell DRX, the SR could be transmitted. However, if the UE first performs UCI multiplexing, the SR will be multiplexed in the PUCCH with CSI and will not be transmitted. UE behaviour needs to be clarified in such case.

Figure 7

Proposal 10: Define the UE behaviour to support the joint operation of cell DTX/DRX and other collision handling.

- Joint operation of cell DTX/DRX and NES techniques in spatial/power domain

Typically, a BFR (beam failure recovery) procedure includes four steps: beam failure detection, candidate beam identification, BFR request and UL/DL adjustment based on the identified candidate beam.

For the first two steps, the UE behavior of beam failure detection and/or candidate beam identification in the case of cell DRX should be considered. For example, a safe way is only use SSBs for beam failure detection or candidate beam identification in cell DTX inactive time.

For the third step, a discussion point is whether UE can transmit BFR request in the case of cell DRX. Due to its urgency and the importance, BFR request should be allowed in DRX inactive time.

Proposal 11: Support BFR procedure enhancement for the impact of cell DTX/DRX operation.

In the parallel agenda item 9.7.1, the spatial and power domain enhancement for NES is under discussion. The goal of spatial and power enhancement is to convey spatial and/or power parameters to UE for flexible CSI acquisition or beam management. These parameters may be associated with their applicable time domain resource or CC(s). Considering that cell DTX/DRX indication may also be associated with time domain resource or CC(s), the interaction of them should be considered.

Basically, there are two different ways for the interaction: 1) joint indication; 2) separate indication. For joint indication, cell DTX/DRX and spatial/power parameters are indicated with same signaling with potential combinations between parameters. This way can save signaling overhead but with less flexibility. For separate indication, cell DTX/DRX and spatial/power parameters are indicated independently. This way is more flexible but requires more signaling. In the case of separate indication, implicit determination of cell DTX/DRX time can be considered. For example, if UE receives spatial/power parameters indication with corresponding applicable time domain resource, the time domain resource can be regarded as cell DTX active time.

Proposal 12: Support joint operation of cell DTX/DRX and NES spatial/power domain techniques.

- Alignment of multiple UEs’ DRX

For a UE configured with UE DRX, the UE is configured with a ON duration within a DRX cycle. In practice, the gNB usually configures the same DRX cycle among UEs and multiple ON durations to distribute UEs in time domain. From the gNB perspective, DRX can also achieve NW energy savings if the gNB can always align the OFF periods for all UEs in a same location. However, such scenario may not always happen in practical implementations.

In Rel-16, a DCI format 2_6 was introduced to wake up UEs for a next ON duration. As illustrated in Figure 8, some ON durations can be cancelled by Rel-16 WUS due to no traffic for the associated UEs, e.g., onDuration#2 and onDuration#4 are cancelled. In this scenario, the gNB may need to wake up multiple times for the non-contiguous ON durations, e.g., onDuration#1 and onDuration#3, which introduces additional transition energy to switch between ON and OFF on the gNB side.

Figure 8

Based on the above discussion, it is beneficial to introduce dynamic adaptation on DRX per group of UEs or per cell for NW energy savings. For example, as shown in Figure 9, dynamically control the start of DRX ON durations from multiple UEs such that the gNB may not need to have unnecessary transition duration within the DRX cycle, and the ON durations for multiple UEs can be aligned or consecutive.

Figure 9

Proposal 13: Support cell specific or UE-group specific indication on UE’s DRX cycle to align multiple UE’s ON durations.

- Conclusion

The contribution discusses cell DTX/DRX related procedure and the proposals are summarized below.

Proposal 1: Down-select from the following three options for interpretation of ‘No change for SSB transmission due to cell DTX/DRX’.

- Option 1: SSB transmission is independent of cell DTX, i.e., SSB can be transmitted during non-active durations.

- Option 2: The durations for SSB transmission are considered as active durations for the determination of the active durations of cell DTX.

- Option 3: gNB configuration/indication ensures that the durations for SSB transmission are always within cell DTX active durations.

Proposal 2: PDCCH monitoring in CORESET 0, SIB1 reception and PRACH transmission follow the same rule of SSB transmission when configured with cell DTX/DRX.

Proposal 3: The reception/transmission of a channel/signal with a DCI format is not impacted by cell DTX and cell DRX, respectively.

- FFS: Whether the symbols of the reception/transmission of a channel/signal with a DCI format are considered as active for the determination of the active durations of cell DTX/DRX, respectively.

Proposal 4: For joint operation of cell DTX and DRX, a set of higher layer parameters applies to both cell DTX and DRX

- The set of parameters at least includes periodicity, start slot/offset, on duration.

Proposal 5: Support one or multiple configurations for cell DTX/DRX provided by a higher layer signaling and a fast cell DTX/DRX activation/deactivation mechanism via L1/L2 signaling.

Proposal 6: Support MAC CE based activation/deactivation and parameter update of configurations for cell DTX/DRX.

Proposal 7: Support a cell/group common or UE-specific DCI format to activate/deactivate/modify the configured cell DTX/DRX, the following options or the combinations of the options can be considered.

- Option 1: Dynamically indicates whether the onDuration timer is started for one or more cell DTX/DRX cycles

- Option 2: Dynamically indicates which cell DTX/DRX configuration is activated if multiple cell DTX/DRX configurations are provided.

- Option 3: Dynamically indicates whether a next period is active or non-active.

- Option 4: Dynamically indicates parameter update for the set of configuration of cell DTX/DRX.

- FFS: Dynamically indicates the cell DTX/DRX timers.

- FFS: Implicit indication of cell DTX/DRX.

Proposal 8: Support HARQ-ACK feedback for the cell/group common DCI format to activate/deactivate/modify the configured cell DTX/DRX.

- FFS: The UE behaviour if the DCI format is missed.

Proposal 9: Study whether/how to align the starting time of cycles of cell DTX/DRX and UE DRX for PCell.

- FFS: SCell

Proposal 10: Define the UE behaviour to support the joint operation of cell DTX/DRX and other collision handling.

Proposal 11: Support BFR procedure enhancement for the impact of cell DTX/DRX operation.

Proposal 12: Support joint operation of cell DTX/DRX and NES spatial/power domain techniques.

Proposal 13: Support cell specific or UE-group specific indication on UE’s DRX cycle to align multiple UE’s ON durations.

References

- RP-223540, “New WID: Network energy savings for NR”

- 3GPP TR 38.864, “Study on network energy savings for NR”