3GPP TSG-RAN WG1 Meeting #112	R1-2301281

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.9.2

Source:	Samsung

Title:	On multi-carrier UL Tx switching

Document for:	Discussion

# Introduction

The Rel-18 WID on Multi-carrier enhancements has as second objective to study and if necessary, specify UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations.

One remaining open issue is the handling of the potential ambiguity issue for the switching period location. In this contribution we provide our views and recommendation on the FL updated proposed agreement 6 [2].

# Background

For Rel-16 and Rel-17 UL Tx Switching, the switching period location is configured by RRC using parameter uplinkTxSwitchingPeriodLocation-r16 in ServingCellConfig. The parameter indicates whether the location of UL Tx switching period is configured in this UL carrier in case of inter-band UL CA, SUL, or (NG)EN-DC. In the case of (NG)EN-DC, the network always configures this field to TRUE for the NR carrier, i.e., the UL switching period always occurs on the NR carrier. RRC also configures parameter uplinkTxSwitchingCarrier which indicates that the configured carrier is carrier1 or carrier2 for dynamic UL Tx switching. In the case of (NG)EN-DC, the network always configures the NR carrier as carrier 2. In the case of inter-band UL CA or SUL, for dynamic UL Tx switching between 2 bands with 2 UL carriers or 3 UL carriers, the network configures the UL carrier(s) on one band as carrier1 and the UL carrier(s) on the other band as carrier2. This field is set to the same value for the carriers on the same band. For Rel-16 and Rel-17 UL Tx Switching, the switching period is then located on the band and serving cell carrier where uplinkTxSwitchingPeriodLocation-r16 is configured by RRC.

For Rel-16 and Rel-17 UL Tx Switching, the RRC configured switching period location is strictly speaking only then needed when the gNB doesn’t provide a scheduling gap larger than reported switching period. In addition, parts of the UE switching behavior are left unspecified in Rel-16 and Rel-17. For example, the UE implementation can choose the symbols on which to switch if the allowed configured switching period is longer than the number of symbols required for the switch. For 14 symbols allowed for the switch but only 4 symbols required by the UE, the exact symbol locations where the UE then executes the switch are not specified.

For Rel-18 UL Tx Switching and when only the existing Rel-16 RRC signaling is available, it would then be in principle possible that more than one band pair configured for switching has uplinkTxSwitchingPeriodLocation-r16 set to TRUE by RRC or more than one band pair can be configured with uplinkTxSwitchingPeriodLocation-r16 set to FALSE. Ambiguity arises for the UE because the location of the switching period, i.e., the victim carrier, may then be possibly allowed for different bands and serving cell carriers. When 3 or 4 bands are configured for Rel-18 switching, the switching period location needs to be determined by the UE for one band and serving cell carrier. This functionality is not supported by the Rel-16 and Rel-17 signaling. Therefore, some additional configuration or rule is necessary for Rel-18 UL Tx Switching.

# Discussion

Handling of the switching period location was discussed in RAN1#110bis-e (FL updated proposed agreement 4.2.1 with Options 0-5). As indicated in their LS reply [3], RAN4 agreed to reuse the Rel-16/Rel-17 approach where RRC configures the switching period on one of the bands for each switching band pair but leaving further details for Rel-18 UL Tx Switching scenarios to RAN1.

Issue 4: Location of switching period

- For single-TAG case, RAN4 agreed to reuse the Rel-16/17 approach (i.e., semi-static configuration of switching period on one of the band for each switching band pair) and discuss further details for Rel-18 Tx switching scenario in RAN1.

- Meanwhile, RAN4 has not concluded on the switching period location for 2-TAG case, with further discussions ongoing.

The following alternatives were identified at the end of RAN1#111 (FL updated proposed agreement 6):

FL updated proposed agreement 6

Down-select a solution for the ambiguity issue on switching period location from following alternatives.

- Alt.1: switching period location is configured per band pair

  - If there are multiple bands configured with switching period location as TRUE in the bands involved in a switching, the switching period location is determined to highest carrier frequency among the bands configured with switching period location as TRUE

- Alt.2: switching period location is configured per band pair, and the priority list of bands is also configured

  - If there are multiple bands configured with switching period location as TRUE in the bands involved in a switching, the switching period location is determined to the band with lowest priority among bands configured with switching period location as TRUE

- Alt.3: gNB configures “switching-from band” or “switching-to band”

  - If gNB configures “switching-from band” as switching period location, switching period is located on band(s) where preceding transmission is performed

- Alt.4: gNB configures switching period location per switching case

  - In 3 bands case, gNB configures switching period location for each of switching case pair such as {A - B}, {A - C}, {B - C}, {A+B - C}, {A+C - B}, {B+C - A}

  - In 4 bands case, gNB configures switching period location for each of switching case pair such as {A - B}, {A - C}, {A - D}, {B - C}, {B - D}, {C - D}, {A+B - C}, {A+B - D}, {A+C - B}, {A+C - D}, {A+D - B}, {A+D - C}, {B+C - A}, {B+C - D}, {B+D - A}, {B+D - C}, {C+D - A}, {C+D - B}, {A+B – C+D}, {A+C – B+D}, {A+D – B+C}

- Alt.4-rev: gNB configures switching period location per configured 3 or 4 bands

  - In 3 bands case, gNB configures switching period location for each of switching case pair such as {A, B}, {A, C}, {B, C}, {A, B, C}

  - In 4 bands case, gNB configures switching period location for each of switching case pair such as {A, B}, {A, C}, {A, D}, {B, C}, {B, D}, {C, D}, {A, B, C}, {A, B, D}, {B, C, D}, {A, B, C, D}

- Alt.5: gNB configures priorities to each carrier/band.

  - The UE determines the switching period location on the band that is not with the highest priority.

When considering Alt.1-Alt.5, we note that similar to Rel-16 and Rel-17, the RRC configured switching period location is strictly speaking only then needed when the gNB doesn’t provide a scheduling gap larger than reported switching period also for Rel-18 UL Tx Switching in the 3 or 4 band case. The UE then does not perform UL transmission in the scheduling gap.

Another consideration is that flexibility should be given to the gNB to configure the switching period location according to needs of the deployment. If there are multiple band pairs configured with the switching period location set to TRUE in the bands involved in switching, any fixed rule such as the switching period location is then determined to be the lowest carrier (or the highest carrier) among the bands configured with switching period location as TRUE can potentially prove cumbersome in future deployments. The lowest band/carrier would likely be better in terms of coverage and range and the highest carrier allow for larger NR carrier bandwidth. It is not safe to assume that one or the other should be preferred and such a choice should be left to the operator and should remain configurable. Alt.1 should therefore be avoided.

We see similar limitations arising out of Alt.3 where the switching period is (always) located on band(s) where the preceding transmission is performed. The previous carrier/band must then always absorb the switching gap when needed and no future flexibility exists to configure the UE behaviour by the network.

Using Alt.2 and Alt.5, a priority list of carrier/bands is configured for the UE. The UE then determines the switching period location as the carrier/band with lowest configured priority either from the multiple configured band pairs or from the list of carriers/bands. Using the RRC configured priority list, the UE needs to determine the carrier/band absorbing the switching period based on the supported as by UE capability and the resulting configured switching cases. While such an approach is possible, we think such a solution is overly complex when compared to a straightforward extension of the existing Rel-16/Rel-17 RRC signalling according to Alt.4 or Alt.4-rev where the gNB explicitly configures the switching period location per switching case or per configured 3 or 4 bands. Further signalling design aspects should be discussed in RAN2.

- Proposal 1: For Rel-18 UL Tx Switching in the 3 or 4 bands case, the switching period location is determined by the UE using Alt.4 or Alt.4-rev where the gNB configures switching period location per switching case or per configured 3 or 4 bands.

# Conclusion

In this contribution, we made the following proposal:

- Proposal 1: For Rel-18 UL Tx Switching in the 3 or 4 bands case, the switching period location is determined by the UE using Alt.4 or Alt.4-rev where the gNB configures switching period location per switching case or per configured 3 or 4 bands.

# References

RP-222251 “Revised SID on Multi-carrier enhancements”; RAN#96

R1-2212879 “Summary#3 of discussion on multi-carrier UL Tx switching scheme”; NTT DoCoMo

R4-2217741 “LS on Rel-18 UL Tx switching”; RAN4

R1-2212061 “On multi-carrier UL Tx switching”; Samsung