3GPP TSG RAN WG1 #112	                     	  R1-2301302

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.5.1.3

Source: 	ASUSTeK

Title:  	Discussion on resource allocation for SL PRS

Document for:	Discussion and Decision

- Introduction

According to WID [1], objective of Expanded and Improved NR Positioning includes sidelink positioning:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]:

  - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1].

    - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum.

    - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified.

  - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2].

  - Specify support of resource allocation for SL PRS:

    - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1].

      - For resource allocation mechanism for SL PRS in Scheme 2:

        - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1].

        - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1].

    - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1].

      - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

  - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1].

  - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2].

In this contribution, we discuss some aspects for SL PRS resource allocation.

- Discussion

According to TS 38.859 [2], some positioning methods using sidelink measurements are identified for possible introduction, such as RTT-type solutions using SL, SL-TDOA, SL-AoA and SL-AoD. To support such sidelink measurements, a new sidelink reference signal for SL positioning/ranging, referred to as SL PRS, will be introduced. For former two timing-based positioning methods, larger bandwidth for SL PRS is required for higher accuracy positioning, e.g., 10, 20, 40 and 100 MHz in FR1 are considered. Given the bandwidth requirement, dedicated resource pool for SL PRS is one direction of avoiding potential backward compatible issues on legacy Rel-16/17 UEs. In frequency domain, SL PRS supports comb-N design in frequency domain pattern, that different RE offsets (< N) will correspond to different non-overlapped SL PRS resources with same SL PRS bandwidth. In time domain, it possibly supports multiple SL PRS occasions in a slot [3].

Moreover, sub-channel-based resource allocation is applied for legacy PSSCH. With regards to bandwidth of SL PRS, there are two alternatives. Alt.1 is that bandwidth of SLPRS can be same or smaller than that of the resource pool, which means that sub-channel-based resource allocation can be also applied for SL PRS in dedicated resource pool. Alt.2 is that bandwidth of SL PRS shall be same as that of the resource pool. Different bandwidth requirement of positioning/ranging may utilize SL PRS in different dedicated resource pool. From our aspect, Alt.2 may induce more resource pool partition in a carrier and require further resource pool selection procedure. In comparison, Alt.1 can provide more flexibility and efficiency on resource utilization. To mitigate possible SL PRS resource blocking, some candidate starting sub-channels can be restricted for different bandwidths of SL PRS transmission, which restriction is similar as SRS resource design.

Proposal 1:  For dedicated resource pool, bandwidth of SL-PRS can be same or smaller than that of the resource pool.

Regarding resource allocation for SL PRS, two schemes are introduced.

In scheme 1 corresponding to network-centric SL PRS resource allocation, the network will allocate SL PRS resource(s) via SL PRS grant. The main issue is to design such SL PRS grant, since granularity of SL PRS resource allocation is a little different from PSSCH, e.g., SL PRS resource requires further scheduling information of RE offset and SL PRS occasion. To differentiate SL grant for PSSCH and SL PRS grant for dedicated resource pool, either RNTI or resource pool index can achieve such differentiation. Regarding whether to include HARQ process, NDI, PUCCH resource indication, it will depend on further study on whether and how to support SL PRS retransmission.

Proposal 2:  In scheme 1, introduce SL PRS grant for allocating SL PRS resource(s).

In scheme 2 corresponding to UE autonomous SL PRS resource allocation, it shall support at least sensing-based resource allocation to avoid resource collision. Following principle of legacy sidelink, TX UE needs to provide sidelink control information for indication/reserving SL PRS resources, which can assist RX UE in SL PRS measurement and also assist RX UE in performing its sensing-based resource allocation. To avoid design complexity, it is better to also keep legacy sidelink principle such that each SL PRS transmission is associated with a SCI transmission in the same slot. Thus, PSCCH which carries SCI associated with SL PRS transmission(s) shall be included in dedicated resource pool for SL positioning.

The sensing-based resource allocation in scheme 2 can be introduced mostly based on sidelink mode 2 design. One difference is that PSSCH applies sub-channel based resource allocation, while SL PRS resources are differentiated in RE level, which expects to suffer higher interference due to Doppler frequency shift. It is FFS on how to deal with such interference issue in scheme 2.

Observation 1:  SL PRS resource differentiation in RE level will suffer higher interference due to  Doppler frequency shift.

Proposal 3:  In scheme 2, sensing-based SL PRS resource allocation is introduced mostly based on sidelink mode 2 design, and further considers interference issue due to resource differentiation in RE level.

- Conclusion

In this contribution, we have following proposals about SL PRS resource allocation:

Proposal 1:  For dedicated resource pool, bandwidth of SL-PRS can be same or smaller than that of the resource pool.

Proposal 2:  In scheme 1, introduce SL PRS grant for allocating SL PRS resource(s).

Observation 1:  SL PRS resource differentiation in RE level will suffer higher interference due to Doppler frequency shift.

Proposal 3:  In scheme 2, sensing-based SL PRS resource allocation is introduced mostly based on sidelink mode 2 design, and further considers interference issue due to resource differentiation in RE level.

- Reference

- RP-223549, “New WID on Expanded and Improved NR Positioning”

- TS 38.859 V18.0.0, “Study on expanded and improved NR positioning; (Release 18)”

- R1-2301300, “Discussion on sidelink positioning reference signal”, ASUSTeK