3GPP TSG RAN WG1 #112		R1-2301330

Athens, Greece, February 27th – March 3rd, 2023

Source: 	Apple Inc.

Title:                     Views on Two TAs for mDCI mTRP

Agenda item:	9.1.1.2

Document for:	Discussion and Decision

# 1 Introduction

In the RAN 111 meeting, the following was agreed to enable two TAs for mDCI mTRP [1]:

Working Assumption For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI - the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support CFRA triggered by PDCCH order for both intra-cell and inter-cell cases. Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP at least for inter-cell Multi-DCI. - FFS: for intra-cell Multi-DCI - FFS: whether there are any restrictions needed - FFS: if cross TRP RACH triggering is an optional feature Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, there is no consensus to support enhancements for CBRA triggered by PDCCH order. Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it is up to RAN2 to decide whether there is a need to enhance CBRA procedure to support per TRP UE-initiated RACH procedure. Agreement: Send LS to RAN2 regarding the above conclusion. Final LS in R1-2213004.

In this contribution, we discuss the open issues related to supporting two timing advance (TA) values for UL multi-DCI for multi-TRP operation.

# 2. PDCCH Order CFRA Procedure

  - 2.1 Intra-Cell mDCI Case

In RAN1 110bis meeting, how to identify the TA/TAG for intra-cell multi-DCI case was discussed and a couple of alternatives were listed in the summary [3] for further study as follows:

Alt 1: include TAG ID as part of TA command in RAR Alt 2: indicate TAG ID as part of PDCCH order Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. Alt 4: divide RACH resources into two groups, where for a RACH procedure, if the corresponding RACH resource belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. Alt 5: divide preambles into two groups, where for a RACH procedure, if the corresponding preamble belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP Alt 6: TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order Alt 7: Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order

The current PDCCH order DCI consists of a couple of information fields for CFRA procedure including SSB index, RA preamble index and PRACH mask index etc. For intra-cell mTRP, upon receiving the PDCCH order, UE can transmit the preamble in a UL beam direction derived based on the associated SSB. In our view, the association between SSB and TRPs can be transparent to UE. The TAG ID for TA indicated by a RAR can be explicitly indicated in the RAR payload by repurposing the reserved bit (i.e., Alt.1). Given the simplicity and good extensivity to be used in other cases, e.g., inter-cell mTRP, we have preference for this alternative.

Proposal 1:

- Support to included TAG ID as part of TA command in RAR to enable two TAs in intra-cell mTRP use case.

  - 2.2 Inter-Cell mDCI Case

In RAN1 111 meeting, it was agreed to support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP at least for inter-cell Multi-DCI. A couple of issues remain open and need to be addressed to complete the design.

The first FFS aspect of PDCCH order CFRA is how to indicate the target TRP to determine the corresponding PRACH resource associated with the SSB index indicated by the PDCCH order. In Rel-17 ICBM framework, each candidate cell is assigned with a dedicated additional PCI index. It is quite straightforward to include the ‘additional PCI Index’ in the PDCCH order to inform UE which candidate cell the CFRA is triggered for.

Proposal 2: Confirming the following working assumption

- For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI

  - the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI

Proposal 3:

- Support to explicitly indicate the ‘additional PCI index’ in the PDCCH order to identify the target cell.

How to transmit RAR was extensively discussed in the past meetings without consensus. Two options exist, one is to transmit from the target cell (Alt.1) and the other is from the serving cell (Alt.2). Our preference is Alt.1 i.e., the associated RAR is transmitted by non-serving cell TRP, which is mainly motivated to mitigate the excessive delay caused by the non-idea backhaul between two TRPs in multi-DCI scenario. To receive RAR from target cell, it is desirable to provide the Type-1 CSS configuration of the 2nd TRP together with CFRA configuration.

Proposal 4:

- Support to transmit RAR from the target cell in inter-cell mDCI mTRP.

- Type-1 CSS configuration of the target cell can be provided for each configured additional PCI.

# 3. Maintenance of Uplink Timing Advance Values

Another FFS aspect identified in the RAN1 110 meeting is how to associate a TAG to an individual UL transmission towards one of two TRPs. Two alternatives were identified as follows:

Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, downselect one of the options in RAN1#110bis-e - Option 1: Associate TAG to TCI-state/spatial relation - Option 2: Associate TAG to CORESETPoolIndex - Option 3: Associate TAG to DL RS group. For a UL transmission, UE adopts the TAG associated with the DL RS group to which the PL RS of the UL transmission belongs. - Option 4: Associate TAG to target UL channels/RSs directly for semi-static UL channels/RSs (e.g. P CSI PUCCH, P SRS, CG PUSCH), and further discuss how to associate TAG to dynamic UL channels/RSs(e.g. via associating TAG to CORESETPoolIndex additionally, etc.)

For Rel-16 mDCI mTRP, ‘CORESETPoolIndex’ value serves as a virtual TRP-ID for PDCCH/PDSCH scheduling, one TRP is associated with CORESETPoolIndex =0 and the other is associated with CORESETPoolIndex = 1. As the TAG is operated on a per serving cell/per UL panel basis, instead of per beam level, it is straightforward to go with Opt.2 i.e., associating each TAG to a particular TRP by levering the value of ‘CORESETPoolIndex’ in Rel-16. Note that, it is allowed that multiple UL TCI-states associated with a non-serving cell are activated. Consequently, Opt.1 may require UE to maintain more than one TAGs for a non-serving cell. Typically, UE maintains the TO/FO tracking loop for timing management for each active TCI-state, instead of the unified TCI-state indicated by the DCI. As one consequence, the  Opt.1 requests UE to store the estimated DL reference timing in a buffer memory for each activated TCI-state, which maybe used for the later UL scheduling. This clearly implies an increased cost and complexity at UE side.

During the offline discussions before RAN1 112 meeting, concerns on support of Rel-17 mTRP PUSCH/PUCCH repetition schemes were raised for Opt.2. Our understanding is that Rel-17 mTRP PUSCH/PUCCH repetition is designed assuming sDCI mTRP use case, where two joint/UL TCI-states were indicated by a single DCI. Even in Rel-17, PUSCH/PUCCH repetition is not supported for mDCI mTRP. The focus of Rel-18 enhancement is to extend the Rel-17 unified TCI-framework from sTRP to mTRP, instead of introducing a PUSCH/PUCCH repetition scheme for mDCI mTRP.

Observation 1:

- Rel-17 mTRP PUSCH/PUCCH repetition is specified assuming sDCI mTRP and irrelevant for TAG management discussion for mDCI mTRP in Rel-18.

Proposal 5:

- Adopt Opt.2 for TAG association with uplink transmissions, i.e., each TAG is associated with one of the CORESETPoolIndex values.

- For Type-1 CG, P/SP/AP-SRS, P/SP-PUCCH and PUCCH for dynamic HARQ-ACK, the corresponding CORESETPoolIndex value is provided using RRC configuration.

# 4. Handling Overlapped Uplink Transmissions

With two TAs, one issue identified in RAN1 110 meeting is that two UL transmissions on a same CC towards two TRPs maybe overlapped in time. A variety of potential solutions were identified in RAN1 110 meeting to address this problem as follows [2]:

Agreement For multi-DCI based multi-TRP operation with two TAs, study how to handle overlapping part between two UL transmissions associated with two TAs, where the study includes: - whether to introduce scheduling restriction in overlapping part - whether to introduce dropping rules - whether specification impact is needed, or if the issue can be handled via implementation - whether to allow overlapped transmission in case the UE supports STxMP transmission (if STxMP feature is agreed in NR Rel-18)

It should be obvious that this is not a problem for UE supports STxMP transmission. For mTRP use case with ideal backhaul, it is feasible to assume that the overlapping transmissions can be avoided by proper network implementation as the two TAs are known at gNB side. However, it is quite challenging for non-ideal backhaul case where timely coordination is not possible.

In RAN1 110bis meeting, the following was concluded for this issue [3]:

Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it cannot always be assumed that both TRPs have knowledge of the overlapping region between transmissions corresponding to the two TAs. - Note: This doesn’t prevent the network from applying scheduling restrictions even if the TRPs have no knowledge of the overlapping region

In [5], RAN4 provides response on the maximum uplink timing difference as follows:

For a UE capable of supporting Receive Time Difference (RTD) > CP, MRTD/MTTD value for FR1 is 33/34.6 µs and MRTD/MTTD value for FR2 is 8/8.5 µs. For a UE not capable of supporting RTD>CP, MTTD is within (CP + M1 µs) for FR1 and MTTD is within (CP + M2 µs) for FR2. Where M1 and M2 are FFS in RAN4.

Observation 2:

- For UE support RTD> CP, the maximum MTTD is 34.6 us for FR1 and 8.5us for FR2 respectively.

Although introducing a dropping rule is a candidate solution, it requires huge standard efforts to discuss the timeline for dropping deprioritized channels (e.g., UCI multiplexing). We therefore prefer to leave for gNB implementation to avoid the overlapping. To assist network in the determination of overlapping part, some UE-assist information report can be considered e.g., the UL transmission timing difference between two TRPs can be reported to gNB to facilitate the setting of scheduling restriction on overlapping part.

Proposal 6:

- Prefer to define scheduling restriction in overlapping part.

- Consider introducing UE-assist information to report the timing difference between two TAGs for overlapping part determination.

# 5. Conclusion

In this contribution, we have presented our views on two TAs for mDCI mTRP. Based on the discussions in the previous sections, we observe and propose the following:

Proposal 1:

- Support to included TAG ID as part of TA command in RAR to enable two TAs in intra-cell mTRP use case.

Proposal 2: Confirming the following working assumption

- For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI

  - the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI

Proposal 3:

- Support to explicitly indicate the ‘additional PCI index’ in the PDCCH order to identify the target cell.

Proposal 4:

- Support to transmit RAR from the target cell in inter-cell mDCI mTRP.

- Type-1 CSS configuration of the target cell can be provided for each configured additional PCI.

Observation 1:

- Rel-17 mTRP PUSCH/PUCCH repetition is specified assuming sDCI mTRP and irrelevant for TAG management discussion for mDCI mTRP in Rel-18.

Proposal 5:

- Adopt Opt.2 for TAG association with uplink transmissions, i.e., each TAG is associated with one of the CORESETPoolIndex values.

- For Type-1 CG, P/SP/AP-SRS, P/SP-PUCCH and PUCCH for dynamic HARQ-ACK, the corresponding CORESETPoolIndex value is provided using RRC configuration.

Observation 2:

- For UE support RTD> CP, the maximum MTTD is 34.6 us for FR1 and 8.5us for FR2 respectively.

Proposal 6:

- Prefer to define scheduling restriction in overlapping part.

- Consider introducing UE-assist information to report the timing difference between two TAGs for overlapping part determination.

# References

- RAN1 111 Meeting Chairman notes

- RAN1 110 Meeting Chairman notes

- RAN1 110 bis Meeting Chairman notes

- RAN2 120 Meeting Chairman notes

- R1-2210817, Reply LS on MTTD for multi-DCI multi-TRP with two TAs, RAN4, RAN1#111, Toulouse, November 2022