3GPP TSG RAN WG1 #112			R1-2301368

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.11.4

Source:	Apple

Title:	Discussion on improved GNSS operations for IoT NTN

Document for:	Discussion/Decision

# Introduction

The Release-18 IoT NTN enhancement work item was approved in RAN #94e meeting [1] and updated in RAN #96e meeting [2].

One of the RAN1 objectives is to study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption.

The following agreements were reached in RAN1#111 meeting [3].

Agreement For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap - FFS details of gap configuration The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement - FFS based on configured timing

In this contribution, we provide our views on improved GNSS operations for a new position fix.

# Discussion

One remaining issue from last RAN1 meeting is the details of gap configuration if eNB aperiodically triggers the UE to perform GNSS measurement. The configured measurement gap should base on UE reported GNSS position fix time duration, and UE shall perform GNSS measurement within the configured gap. After the aperiodic measurement is triggered, the starting time of measurement gap needs to be determined which is relative to the timing of MAC CE signaling reception. For instance, if the triggering signaling is received in subframe n, the measurement will be starting at subframe n+k. where the k subframe at least covers signaling processing time and GNSS measurement preparing time.

Proposal 1: For aperiodic triggered GNSS measurement, after the triggering signaling is received in subframe n, UE performs the GNSS measurement after n+[k]ms. GNSS measurement gap is configured according to UE reported GNSS position fix time.

Another issue is how UE perform GNSS measurement without receiving eNB triggering signalling.

Timer based periodic GNSS measurement can be considered. Depending on the UE reported GNSS position fix time duration for measurement and GNSS validation duration, eNB determines the starting time of aperiodic GNSS measurement, then triggers the GNSS measurement. In case the UE reported GNSS measurement assistance information is the same as last report, or no assistance information is reported, eNB doesn’t need to trigger the aperiodic GNSS measurement, UE could perform the measurement according to the configured periodic GNSS measurement timer. If the aperiodic GNSS measurement is trigger by eNB before the periodic GNSS measurement timer is expired, UE performs the measurement according to the triggering signaling, at the same time periodic GNSS measurement timer is restarted from the starting point of measurement gap. In this case, there is no additional UE power consumption for periodic measurement, and the signaling overhead is saved. An example of periodic GNSS measurement timer configuration is showing in Figure1.

Figure1: example of periodic GNSS measurement timer

Proposal 2: RAN1 considers supporting periodic GNSS measurement timer for UE to re-acquire GNSS position for long connection times

- if UE does not receive eNB triggered GNSS measurement until the periodic GNSS measurement timer is expired, UE performs GNSS measurement periodically

- Otherwise, periodic GNSS measurement timer is restarted from the starting subframe of the measurement gap according to eNB triggering singalling

The timer of periodic GNSS measurement is determined based on UE reported validation duration according to UE moving velocity and/or satellite assistance information. The measurement gap is based on the reported for warm fix or cold fix. These two parameters could be updated during the long RRC connection. As the aperiodic GNSS measurement is triggered via the MAC CE, the periodic GNSS measurement timer information could be carried in MAC CE as well to adapt GNSS measurement gap and GNSS measurement timer changing. In this way, aperiodic GNSS measurement and periodic GNSS measurement are triggered together, aperiodic measurement can be considered as the first measurement of periodic GNSS measurement. In another word, every aperiodic GNSS measurement triggering could update the measurement gap and/or the timer of periodic GNSS measurement.

Proposal 3: For timer based periodic GNSS measurement, GNSS measurement gap and GNSS measurement periodicity are configured via aperiodic GNSS measurement triggering MAC CE.

# Conclusion

In this contribution, we provided our views on improved GNSS operations for IoT NTN. Our proposals are as follows:

Proposal 1: For aperiodic triggered GNSS measurement, after the triggering signaling is received in subframe n, UE performs the GNSS measurement after n+[k]ms. GNSS measurement gap is configured according to UE reported GNSS position fix time.

Proposal 2: RAN1 considers supporting periodic GNSS measurement timer for UE to re-acquire GNSS position for long connection times

- if UE does not receive eNB triggered GNSS measurement until the periodic GNSS measurement timer is expired, UE performs GNSS measurement periodically

- Otherwise, periodic GNSS measurement timer is restarted from the starting subframe of the measurement gap according to eNB triggering singalling

Proposal 3: For timer based periodic GNSS measurement, GNSS measurement gap and GNSS measurement periodicity are configured via aperiodic GNSS measurement triggering MAC CE.

# References

- RP-213596, “New WID on IoT NTN enhancements,” Dec. 2021.

- RP-221806, “Revised WID on IoT NTN enhancements,” Jun. 2022.

- R1-2300001, “Report of RAN1#111 meeting”, ETSI MCC