| 3GPP TSG-RAN WG1 Meeting #112 Athens, Greece, February 27th – March 3rd, 2023 |  | R1-2301442 |
| --- | --- | --- |

Agenda item:	9.14.2

Source: 	Qualcomm Incorporated

Title: 	Power-domain enhancements

Document for:	Discussion/Decision

# Introduction

In this document we present our views on power-domain enhancements targeted at improving 5G-NR coverage. The R18 WID provides the following guidelines on these enhancements:

- Study and if necessary specify following power domain enhancements - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1) Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

Section 2 presents our views on techniques for reducing MPR/PAPR to enable higher power transmissions and Section 3 presents our views on enhancements targeted at realizing higher power transmissions in CA and DC.

# Enhancements to reduce MPR/PAPR

From previous discussions, it was agreed to focus on DFT-S-OFDM waveforms with QPSK modulation. Following this guidance, in the following sections, we focus on DFT-S-OFDM waveforms with pi/2 BPSK or QPSK modulations and present performance results using the two non-transparent schemes currently under study for MPR/PAPR reduction: (a) tone reservation, and (b) FDSS with BWE.

Before we proceed further, we restate a few observations regarding MPR. Transmit power at the UE is broadly governed by a UE’s power class and the various power reductions that a UE is allowed. The power reduction could be to (a) accommodate waveform types (MPR), (b) meet tight emissions requirements (A-MPR) and (c) for power management (P-MPR). Focusing on MPR, the RAN4 specification provides the following table (for a power class 3 UE) that outlines the set of allowed power backoffs for waveforms depending on the number of allocated RBs and their location in the band, as well as the modulation used.

Table 1 MPR Table from 38.101-1

There are many constraints imposed on a UE transmission, including limits on ACLR, SEM, IBE, EVM, etc. A UE may not always be able to synthesize and transmit a waveform at its maximum power based on its power class while also adhering to all the RAN4 constraints. The values in this table are chosen so that a UE can synthesize the required waveforms without having to violate the various RAN4 restrictions on ACLR, IBE, EVM and SEM.

The maximum allowed power back off values are used by the UE to calculate the lower bound on its PCMAX,f,c value. In other words, these numbers represent the maximum amount of backoff a UE is allowed, and a real-world UE may or may not use the maximum allowance when setting its transmit power for a transmission, i.e., the UE is not precluded from self-enhancement for waveforms that have non-zero MPR. There is however a limit to the self enhancement possible, which is the same PCMAX_H identified as the upper limit for 0-dB MPR waveforms.

Observation 1: MPR values only provide a lower bound on PCMAX,f,c. Not all UEs may make use of the allowed reduction in transmit power.

Focusing on DFT-S-OFDM waveforms, when inner RB allocations are used, it is required to transmit the lower order modulations such as pi/2 BPSK and QPSK at full power without any power back off; a back off of up to 1 dB may be applied to outer and edge RB allocations. As one proceeds to higher order modulations, significant power back offs may be applied.

The following figure shows the split between inner RB allocations and outer RB allocations as a function of start RB location and RB length. If the length of RB allocation is less than half the bandwidth and the entire allocation is sufficiently away from the edge of the BW (distance to the edge from either end of the allocation must be at least as long as half the allocation length), then such an allocation is called an inner allocation. Outer allocations constitute a vast majority of the non-inner allocations. Certain exceptional allocations that are right on the edge and have a very small allocation are termed as edge allocations.

Figure 1 Inner, outer, and edge RB allocations for 100 Mhz BW with 30 kHz SCS.

Given the focus on coverage enhancements, if we are to zoom into the region with RB allocations of length 1-64, we notice that a vast majority of such allocations are inner RB allocations as illustrated in the figure below.

Figure 2 Inner, outer, and edge RB allocations for 100 Mhz BW with 30 kHz SCS and LCRB < 64.

It can be seen that close to 80-85% of the allocations in this range fall in the inner region. Note that there is no MPR allowance in this region.  We think that R18 coverage enhancements must narrow its focus to this subset of allocations so that a true coverage gain is realized.

Proposal 1: For enhancements to reduce MPR/PAPR, prioritize inner RB allocations with small RB allocations, for e.g., 1-32 RBs.

For R18 coverage enhancements, we think it is useful to focus on waveforms with 0-dB MPR allowance. Note that for a cell-edge UE, it can be assumed that the network has already exercised all its options to increase transmit power, including scheduling waveforms with no MPR allowance (‘0 MPR waveforms’). An FR1 UE has virtually no room for any further self-enhancement for such waveforms because of proximity between the UE’s actual UL power and the upper limit on transmitted power for the power class itself, PCMAX_H. A meaningful increase in UL coverage is only possible if this impediment to power increase is removed or loosened.

Proposal 2: For enhancements to reduce MPR/PAPR, prioritize mechanisms that allow a 0-dB MPR waveform to be transmitted at a transmit power exceeding the maximum power associated with the UE power class.

## Transparent Schemes for PAPR Reduction

In the last RAN1 meeting (R1-111) it was agreed that transparent waveform shaping techniques to reduce PAPR and consequently MPR can serve as a benchmark to assess the benefit of non-transparent waveform shaping techniques such as FDSS with BWE and tone reservation. Suggested techniques include frequency domain spectrum shaping w/o spectrum extension, transparent peak cancellation, or schemes based on clipping and filtering.

Towards this, we present results on MPR using the raw waveform and compare it to a transparent peak cancellation (TrPC) scheme that leverages the data tones to construct a peak cancelling kernel that is then used to reduce the PAPR of the raw waveform. In effect, such a scheme trades off the relaxed EVM requirement for a QPSK DFT-S-OFDM waveform for better PAPR characteristics. Once the peak cancelling kernel is constructed, SCR-TR algorithm [3] is used to cancel the peaks in the waveform.

Figure 3 Transparent peak cancellation using data tones for constructing a peak cancelling kernel.

Note that such an approach incurs no additional cost to the network and no additional resources are used.

Using such an approach, raw waveforms are shaped to reduce their PAPR characteristics and then passed through an RF simulator to determine the resulting MPR. A PC2 PA model is used and the maximum output power at which the EVM, ACLR, IBE and SEM constraints are all met determines the maximum transmit power for that waveform and consequently its MPR. In the following figures, MPR and the potential power boost using this transparent technique are plotted.

Figure 4 MPR and Power boost values using Transparent Peak Cancellation (TrPC). Every data point corresponds to an allocation with a given start RB and given length (LCRB).

As seen in Figure 4, for inner RB allocations, 0 dB MPR suffices, while for outer RB allocations, < 1 dB of MPR is required. Further, it is seen that for inner RB allocations, it is in fact possible to transmit at a power higher than PC2 power class. It appears that a power boost of up to 1.6 dB is possible for inner RB allocations with small number of RBs.

Caveats on power boost

It is non-trivial for a PC2 UE to transmit at powers greater than 26 dBm. Several RF challenges arise and the overall feasibility and reliability of such a UE warrants scrutiny. In particular, the additional input current to a PA and its implication on the design of the RF transceiver, thermal issues, reliability issues around RF components that have to sustain larger powers need to be examined closely.

Observation 2: For a PC2 UE, using transparent techniques such transparent peak cancellation, a power boost of about 1-1.5 dB seems feasible for inner RB allocations. 1 dB MPR on outer RB allocations may still be required, primarily due to large RB allocations spanning more than 50% of the entire BW. Note that these results do not take other RF implementation challenges into account.

Any non-transparent scheme will need to out-perform transparent techniques such as TrPC to be considered for specification in 5G NR. Since there is no SNR loss associated with transparent techniques, power boost directly translates to coverage gain. The following table summarizes the coverage gain possible for inner and outer RB allocations for the 10 scenarios identified for further study:

Table 2 Coverage gain using transparent techniques

| Case | No spectrum extension |  | With spectrum extension |  |  |  | Δ SNR | Inner power boost | Net cov gain | Outer power boost | Net cov gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #PRBs | MCS | #PRBs before extension | #PRBs after extension | MCS | Spectrum extension factor |  |  |  |  |  |
| 1 | 16 | 7 | 14 | 16 | 8 | 1/8 | 0 | 1.5 | 1.5 | 0 | 0 |
| 2 | 32 | 8 | 28 | 32 | 9 | 1/8 | 0 | 1.2 | 1.2 | 0 | 0 |
| 3 | 8 | 0 | 6 | 8 | 1 | 1/4 | 0 | 1.5 | 1.5 | 0 | 0 |
| 4 | 8 | 6 | 6 | 8 | 8 | 1/4 | 0 | 1.5 | 1.5 | 0 | 0 |
| 5 | 40 | 2 | 30 | 40 | 3 | 1/4 | 0 | 1 | 1 | 0 | 0 |
| 6 | 40 | 6 | 30 | 40 | 8 | 1/4 | 0 | 1 | 1 | 0 | 0 |
| 7 | 16 | 0 | 10 | 16 | 2 | 3/8 | 0 | 1.5 | 1.5 | 0 | 0 |
| 8 | 32 | 2 | 20 | 32 | 4 | 3/8 | 0 | 1.2 | 1.2 | 0 | 0 |
| 9 | 8 | 2 | 6 | 8 | 3 | 1/4 | 0 | 1.5 | 1.5 | 0 | 0 |
| 10 | 24 | 0 | 18 | 24 | 1 | ¼ | 0 | 1.2 | 1.2 | 0 | 0 |
| 11 | 4 | 1 | 2 | 4 | 4 | 1/2 | 0 | 1.5 | 1.5 | 0 | 0 |
| 12 | 6 | 0 | 4 | 6 | 2 | 1/3 | 0 | 1.5 | 1.5 | 0 | 0 |
| 13 | 10 | 3 | 8 | 10 | 4 | 1/5 | 0 | 1.5 | 1.5 | 0 | 0 |

## Tone Reservation

In the following sections we discuss the basics of tone reservation and then present results on net coverage gains using tone reservation.

### Basic principles of tone reservation

Reducing the PAPR (Peak to Average Power Ratio) of the waveform is a common method for achieving higher transmission powers leading to lower MPRs. The input back-off required for meeting the RAN4 restrictions is generally smaller for the reduced-PAPR waveform. This translates to a higher transmission power as well as higher PA efficiency.

As shown in the figure below, classical clip-and-filter techniques take the ideal baseband waveform as input and gently modify the waveform so that it is better suited as input to the PA.

Figure 5 Keys steps in waveform generation and transmission

The classical techniques are transparent in nature, and they tend to take advantage of the relaxed EVM requirements for lower modulation order waveforms to reduce a waveform’s PAPR and other characteristics such as ACLR, IBE and SEM. One common issue with these approaches is that is rather difficult to control the amount of clipping noise that gets generated as part of this process.

Figure 6 Clipping noise generated by classical clip and filter approaches

As an alternative to classical clip & filter techniques, it is desirable to have better control over the clipping or shaping noise and redirect it to a certain designated portion of the band. This motivates us to consider schemes such as tone reservation. Among the many well-known PAPR reduction techniques, “tone reservation” stands out due to its simplicity and efficient usage of the available resources. This technique dedicates a subset of available sub-carriers to absorb the clipping or shaping noise generated when reducing the PAPR of the waveform. These subcarriers are referred to as Peak Reduction Tones (PRTs). To preserve the EVM, there is no overlap between the PRTs and the subcarriers used for transmitting the data.

The principle of tone reservation aims to use the PRTs to construct a compensating waveform, which when added to the raw waveform results in a lower-PAPR waveform. This compensating waveform is constructed by identifying peaks to cancel in the original waveform. The compensating waveform only uses the PRTs and thus leaves the data tones unaltered. Several efficient algorithms exist for this purpose. For example, one well known algorithm is by J. Tellado [4] that iteratively cancels the peaks in the original waveform.

Figure 7 Peak cancelling waveform constructed using reserved tones. An all-ones kernel is used to generate the waveform.

There are many options for the location of PRTs in reference to the data tones. For single carrier waveforms such as DFT-S-OFDM, tones may be reserved immediately adjacent to the RBs allocated to a UE and this is referred to as sideband reservation.

Sideband tone reservation is illustrated in Figure 8, where two UEs are allocated dedicated data tones for their uplink transmission and in addition have some tones on either side of their allocation reserved as PRTs. Note that when using algorithms such as those in [3], the power expended by a UE within the PRTs is quite negligible in comparison to the overall transmit power.

Figure 8 Sideband tone reservation with tones shared between two Ues

On the receiver side, the gNB can discard the PRTs and process only the data tones allocated to a UE to recover the uplink payload. Thus, this approach to increasing uplink transmit power has very minimal impact on gNB receiver design.

Observation 3: With sideband tone reservation, gNB does not have to change its receiver design. Further, link-level performance of the waveform remains unchanged.

### PAPR Reduction Using Tone Reservation

To illustrate the effectiveness of these principles, we consider a UE that is allocated 8 RBs for PUSCH transmission. In addition to these RBs, the UE is also provided with 1 RB of PRTs on either side of its allocation, as shown in the figure below.

Figure 9 Tone reservation with 1 RB PRT on either side of allocation

Focusing on such an allocation we consider a DFT-S-OFDM with QPSK modulation and highlight the improvements in PAPR due to tone reservation in Figure 10. The SCR-TR algorithm with 10 iterations is used for PAPR reduction. As seen in the figures, significant PAPR reduction can be achieved depending on the desired target PAPR. The Power Spectral Density (PSD) of the PRTs is 15 dB lower than the PSD of the data tones. Therefore, the observed PAPR gains come at a negligible price in terms of energy.

Figure 10 CCDF of PAPR of the DFT-s-OFDM QPSK waveform before and after PAPR reduction for an 8 RB allocation with 2 RB tone reservation

### Potential Power Boost Using Tone Reservation

Using such an approach, raw waveforms are shaped using non-transparent tone reservation to reduce their PAPR characteristics and then passed through an RF simulator to determine the resulting MPR. In the following figures, MPR and the potential power boost using non-transparent tone reservation are plotted. As seen in Figure 11, for inner RB allocations, a potential boost of 1-1.5 dB is possible. Outer RB allocations see a reduced gain of about 0.2-0.6 dB. Some outer RB allocations with large number of RBs continue to have an MPR of about 0.2 dB.

Figure 11 MPR and potential power boost using tone reservation. 25% excess BW is assumed.

Caveats on power boost

It is non-trivial for a PC2 UE to transmit at powers greater than 26 dBm. Several RF challenges arise and the overall feasibility and reliability of such a UE warrants scrutiny. In particular, the additional input current to a PA and its implication on the design of the RF transceiver, thermal issues, reliability issues around RF components that have to sustain larger powers need to be examined closely.

Observation 4: For a PC2 UE, when a BW expansion factor of 25% is allowed, tone reservation can result in a potential power boost of about 1-1.5 dB for inner RB allocations and 0.2 to 0.6 dB for certain outer RB allocations. Note that these results do not take other RF implementation challenges into account.

### Coverage Gain Using Tone Reservation

Using the potential power boost presented in the previous section, we aim to assess the net coverage gain that one can realize using non-transparent tone reservation.

To motivate such an analysis, let’s use the example with 8 RBs for data and 2 excess RBs. In this example, a total of 10 RBs are used to transmit a TB size corresponding to 8 RBs. Rather than use these RBs to shape the waveform and reduce PAPR, the reserved tones could have been used for data transmission and reducing the coding rate, i.e., for the same TB size, a lower MCS could have been chosen as shown in Figure 12. It is expected that for the same payload size, the SNR required will be lower when all the RBs are used for data transmission. Thus, there exists a trade-off between transmit power gains using the reserved tones and lower operating SNR due to lower coding rate; the net-benefit of non-transparent tone reservation is then the difference between these two metrics.

Figure 12 Baseline for assessing gains from tone reservation. Values used are for illustration only.

In the following table, we carry out such an analysis for the set of 10 scenarios agreed in the last meeting for calibration. We use link performance curves to assess the additional SNR required when tone reservation is used compared to the case where all RBs are used towards legacy PUSCH transmission. The simulation parameters are summarized in Table 4.

Table 4 Simulation Parameters for link-level evaluations

| Parameter | Value |
| --- | --- |
| Carrier frequency and scenario | 4GHz (Urban) |
| Channel BW | 100MHz (for 4GHz) |
| SCS | 30kHz (for 4GHz) |
| Channel model | TDL-C 300ns for FR1 Urban |
| UE speed | 3km/h |
| Waveform | DFT-s-OFDM |
| Modulation | QPSK |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 4 |
| Number of DMRS symbols | 2 |
| Number of PUSCH data symbols | 12 |

Table 5 Coverage gain using tone reservation

| Case | No spectrum extension |  | With spectrum extension |  |  |  | Δ SNR | Inner power boost | Net cov gain | Outer power boost | Net cov gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #PRBs | MCS | #PRBs before extension | #PRBs after extension | MCS | Spectrum extension factor |  |  |  |  |  |
| 1 | 16 | 7 | 14 | 16 | 8 | 1/8 | 0.6 | 1.5 | 0.9 | 0 | -0.6 |
| 2 | 32 | 8 | 28 | 32 | 9 | 1/8 | 0.6 | 1.4 | 0.8 | 0 | -0.6 |
| 3 | 8 | 0 | 6 | 8 | 1 | 1/4 | 0 | 1.5 | 1.5 | 0 | 0 |
| 4 | 8 | 6 | 6 | 8 | 8 | 1/4 | 0.9 | 1.5 | 0.6 | 0 | -0.9 |
| 5 | 40 | 2 | 30 | 40 | 3 | 1/4 | 0.3 | 1.5 | 1.2 | 0 | -0.3 |
| 6 | 40 | 6 | 30 | 40 | 8 | 1/4 | 0.9 | 1.5 | 0.6 | 0 | -0.9 |
| 7 | 16 | 0 | 10 | 16 | 2 | 3/8 | 0.2 | 1.5 | 1.3 | 0.2 | 0 |
| 8 | 32 | 2 | 20 | 32 | 4 | 3/8 | 0.3 | 1.5 | 1.2 | 0.2 | -0.1 |
| 9 | 8 | 2 | 6 | 8 | 3 | 1/4 | 0 | 1.5 | 1.5 | 0 | 0 |
| 10 | 24 | 0 | 18 | 24 | 1 | 1/4 | -0.2 | 1.5 | 1.7 | 0 | 0.2 |
| 11 | 4 | 1 | 2 | 4 | 4 | 1/2 | 0.1 | 1.5 | 1.4 | N/A | -- |
| 12 | 6 | 0 | 4 | 6 | 2 | 1/3 | 0.4 | 1.5 | 1.1 | 1.4 | 1.0 |
| 13 | 10 | 3 | 8 | 10 | 4 | 1/5 | 0.1 | 1.5 | 1.4 | 0 | -0.1 |

Observation 5: Based on the set of results presented in Table 5, the net coverage gain due to tone reservation varies from 0.5 to 1.5 dB for inner RB allocations. No coverage gain is expected for outer RB allocations.

Next, we compare the above results with the coverage gain possible using transparent techniques and assess the net value of tone reservation. The results are presented in the following table.

Table 6 Coverage gain comparison between transparent techniques and tone reservation

| Case | No spectrum extension |  | With spectrum extension |  |  |  | Net cov gain | Net cov gain | Net cov gain | Net cov gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #PRBs | MCS | #PRBs before extension | #PRBs after extension | MCS | Spectrum extension factor | TrPC Inner Alloc | TR Inner Alloc | TrPC Outer Alloc | TR Outer Alloc |
| 1 | 16 | 7 | 14 | 16 | 8 | 1/8 | 1.5 | 0.9 | 0 | -0.6 |
| 2 | 32 | 8 | 28 | 32 | 9 | 1/8 | 1.2 | 0.8 | 0 | -0.6 |
| 3 | 8 | 0 | 6 | 8 | 1 | 1/4 | 1.5 | 1.5 | 0 | 0 |
| 4 | 8 | 6 | 6 | 8 | 8 | 1/4 | 1.5 | 0.6 | 0 | -0.9 |
| 5 | 40 | 2 | 30 | 40 | 3 | 1/4 | 1 | 1.2 | 0 | -0.3 |
| 6 | 40 | 6 | 30 | 40 | 8 | 1/4 | 1 | 0.6 | 0 | -0.9 |
| 7 | 16 | 0 | 10 | 16 | 2 | 3/8 | 1.5 | 1.3 | 0 | 0 |
| 8 | 32 | 2 | 20 | 32 | 4 | 3/8 | 1.2 | 1.2 | 0 | -0.1 |
| 9 | 8 | 2 | 6 | 8 | 3 | 1/4 | 1.5 | 1.5 | 0 | 0 |
| 10 | 24 | 0 | 18 | 24 | 1 | 1/4 | 1.2 | 1.7 | 0 | 0.2 |
| 11 | 4 | 1 | 2 | 4 | 4 | 1/2 | 1.5 | 1.4 | 0 | -- |
| 12 | 6 | 0 | 4 | 6 | 2 | 1/3 | 1.5 | 1.1 | 0 | 1.0 |
| 13 | 10 | 3 | 8 | 10 | 4 | 1/5 | 1.5 | 1.4 | 0 | -0.1 |

Based on the above comparisons, we make the following observation:

Observation 6: Tone reservation is not well-suited for outer-RB allocations. For inner-RB allocations, compared to transparent techniques, tone reservation brings marginal gains for certain specification allocations.

## FDSS with bandwidth expansion for DFT-S-OFDM

In the last meeting the following agreements on FDSS with BW expansion were made:

Agreement - For RAN1 link-level performance evaluation of MPR/PAR reduction solutions involving the use of Tx spectrum shaping filter, companies are encouraged to use at least the following spectrum shaping filter configuration for calibration purpose: - 2-tap, e.g., (1 0.28), 3-tap, e.g., (0.335 1 0.335), and (0.28 1 0.28) - Truncated RRC (0.5, 0.1667) - There is no restriction to use other spectrum shaping filter coefficients in simulations, e.g., [1 0.28]. - Note: the above does not have spec impact. Agreement - The following non-transparent solutions for MPR/PAR reduction are currently under discussion in RAN1. - Frequency domain spectrum shaping w/ spectrum extension - Tone reservation w/ spectrum extension - In addition, transparent schemes, for instance but not limited to frequency domain spectrum shaping w/o spectrum extension or schemes based on clipping and filtering, are also being evaluated to serve as a benchmark to assess the benefits of non-transparent solutions. Companies are allowed to use any transparent transmission scheme of their choice. Agreement - At least the symmetric spectrum extension option for frequency domain spectrum shaping with spectrum extension (FDSS-SE), are considered for studying MPR/PAR reduction enhancements in Rel-18. Conclusion - It is RAN1 understanding that: - Performance comparison based on net gain results combining transmitter and receiver performance is performed by RAN4. - No final decision would be taken by RAN1 on which MPR/PAR reduction solution, will be specified in Rel-18, if any, since this is RAN4’s responsibility. - It does not preclude RAN1 specification impact Agreement - For the study of the PAPR/CM of DMRS when considering tone reservation as candidate enhancement for MPR/PAR reduction in Rel-18, RAN1 to consider at least the case that PRTs are added to the DMRS symbols (in the sideband). The case of PRTs not added to DMRS symbols can be used as a benchmark. Agreement - Study the PAPR/CM[/OBO] of DMRS with FDSS-SE, e.g., the following solutions: - Option 1 - Based on low PAPR Type 1 DMRS sequence: - 1-a: A DMRS sequence is generated considering the number of PRBs in the inband + extension. The sequence length depends on the number of PRBs in the inband + extension. - 1-b A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. The sequence is then cyclically extended to span the PRBs in the extension. - 1-c A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. DMRS extension is applied similar to data to span the PRBs in the extension. - Option 2 - Based on low PAPR type 2 DMRS sequence - Variances like those of Option 1 can be referred - Option 3 – For in-band DMRS lengths 6/12/18/24 symbols, DMRS sequence is obtained by DFT transformation of low PAPR sequence type 1. Then the sequence is extended to span the PRBs in the extension in the same way as data extension. - Note: Other solutions can be studied. Comparison with the three solutions above is encouraged. Sequence with different density between in-band and extension can be studied

In the following sections we discuss the basics of FDSS with bandwidth expansion and then present results on net coverage gains using FDSS.

### Basics of FDSS with bandwidth expansion

Unlike how tone reservation uses excess bandwidth, FDSS treats the excess bandwidth as part of total allocated bandwidth and uses it towards pulse shaping for DFT-S-OFDM. The core ideas draw upon classical approaches to single-carrier transmissions with root raise cosine pulses with a roll-off factor less than 1.

This is a two-step process, as outlined in Figure 13. Assuming an M-point DFT is used to generate the DFT-S-OFDM waveform, the first step takes the output of the M-point DFT and cyclically extends it to K additional tones. In the second step, this cyclically extended output is then multiplied with frequency-domain coefficients corresponding to a pulse shaping filter. The output of this operation is then mapped to M+K contiguous tones.

Figure 13 FDSS with bandwidth expansion

The choice of the filter/pulse used to realize FDSS depends on the spectrum flatness constraints imposed on a UE’s transmit waveform and on the amount of PAPR reduction that is desired. RAN4 defined a new set of spectrum flatness constraints for pi/2 BPSK waveforms to enable FDSS without BW expansion. A similar exercise may be required here where appropriate spectrum flatness conditions are developed for this new class of waveforms. In what follows, we reuse the spectrum flatness constraints defined for pi/2 BPSK waveforms and consider 3-tap filters that conform to this requirement.

Observation 7: Filter design for FDSS is likely to depend on spectrum flatness requirements, target PAPR, RB allocation and amount of excess bandwidth. The choice of filter is likely to determine transmit power gains and link-level performance.

### PAPR Reduction Using FDSS

To illustrate the effectiveness of FDSS with BW expansion, we consider a UE that is allocated 8 RBs for PUSCH transmission. In addition to these RBs, the UE is also provided with 1 RB of excess BW on either side of its allocation.

Focusing on such an allocation we consider a DFT-S-OFDM with QPSK modulation and note that when FDSS is introduced along with bandwidth expansion, PAPR reduces significantly. This is illustrated in Figure 14.

Figure 14 PAPR of DFT-S-OFDM with bandwidth expansion and FDSS. 3-tap filter conforming to RAN4 spectrum flatness constraints for pi/2 BPSK waveforms is used for FDSS.

As noted in the section on tone reservation, PAPR is just one component of a waveform that determines the transmit power. We will once again have to carefully analyze how these PAPR gains translate to transmit power gains while complying with the various RAN4 constraints.

### Link-level performance and gNB receiver design

One crucial aspect that differentiates FDSS with bandwidth expansion from tone reservation is the overall impact on link-level performance and potential need to enhance the gNB receiver to cope with FDSS.

There are two aspects to take note of in this context.

- For any FDSS scheme with bandwidth expansion, it is unlikely that the gNB will have complete knowledge of the exact pulse shaping filter used by the UE. The UE may be given the freedom to design its own filters subject to certain spectrum flatness conditions.

- How a gnB receiver handles the excess BW plays an important role. A simple gNB receiver may discard the excess bandwidth and exclusively focus on the allocated bandwidth. Such a receiver may stay ignorant to FDSS at the UE. Although such a design is simple and appealing, it comes at the expense of link-level performance since the power expended by a UE in the excess bandwidth is not used optimally by the gNB.

In Figure 15 , we present link-level performance results for the case of a 8 RB data allocation with 2 excess RBs, spanning a total of 10 RBs. The simulation parameters are listed in Table 7 . Similar trends are observed for other cases.

Table 7 Simulation Parameters for link-level performance comparison of FDSS with BWE

| Simulation Parameters |  |
| --- | --- |
| #Antennas at UE | 1 |
| #Antennas at gNB | 4 |
| Channel | TDL-C-300ns |
| Waveform | DFT-S-OFDM, QPSK |
| FDSS mode | Cyclic extension on one side |
| #data symbols | 12 |
| #DMRS symbols | 2 |
| Filter used by UE | 3-tap filter [-0.28 1 -0.28] |
| gNB knowledge of filter | No |
| gNB receiver | MMSE equalization in freq domain + combining across all tones OR combining across data tones only |
| gNB DMRS processing | MMSE channel estimation assuming data and DMRS undergo filtering. Representative filter is used. |

Figure 15 Baseline comparison to assess value FDSS with 8 RB data allocation and 2 excess RBs.  The case with excess RBs is compared against the case where excess RBs are used towards data allocation. QPSK and pi/2 BPSK baselines are included.

Compared to QPSK waveforms as baseline, it is seen that FDSS require an additional SNR of about 0.7 dB assuming the gNB receiver is setup to process the excess BW. The gap grows by additional 0.2 dB if we assume the gNB receiver does not process the excess BW.

Similar to the case of tone reservation, the potential power boost delivered by FDSS+BWE will have to overcome the loss in link-level performance for it provide a coverage gain. This analysis is presented in Section 2.3.5.

Further, unlike tone reservation, FDSS with BWE will likely require hardware changes to gNB receiver as well as UE transmitter. This overall impact must be factored in when deciding whether to specify these non-transparent schemes for coverage enhancement or not.

### DMRS Design for FDSS with BWE

Current 5G NR specification provides three kinds of DMRS sequences for PUSCH transmissions:

- PN sequence mapped to QPSK modulation and used for CP-OFDM

- Low PAPR sequence Type 1: Based on Zadoff-Chu sequence or CGS for DFT-S-OFDM PUSCH

- Low PAPR sequence Type 2: Based on Gold sequence mapped to pi/2-BPSK for DFT-S-OFDM PUSCH

Of the three options, only the second and third can be a potential DMRS for PUSCH with FDSS and BWE. We make the following observations on DMRS based on low PAPR sequence Type 1 and Type 2:

Low-PAPR sequence Type 1 is generated in the frequency domain and waveform is generated using steps similar to that of CP-OFDM. For RB allocations greater than 4, a Zadoff Chu sequence is used with cyclic extension for allocations greater than 5 RBs and truncation when number of RBs allocated is 5. For RB allocations between 1 to 4, computer-generated sequences (CGS) are specified; a total of 30 sequences are provided for each of the 4 cases. By the nature of its construction, the resulting waveform has a constant PSD, and this may be a desirable property from a channel estimation standpoint at the receiver. A comb-2 pattern with OCC is used to generate a total of 4 orthogonal ports.

Low-PAPR sequence Type 2 is generated in time domain and goes through the usual steps of generating a DFT-S-OFDM waveform. For RB allocations greater than 4, a Gold sequence generator is used to generate a pi/2 BPSK sequence. Computer-generated sequences are used when RB allocations are between 1 to 4. The sequence mapping in time domain is such that a 2-comb structure in frequency domain is generated. A total of 4 ports are possible, with 2 ports per comb. The 2 ports per comb use different sequences and are therefore not perfectly orthogonal.

For FDSS with BWE, there are two key aspects to consider:

- Ensuring that the DMRS design has a PAPR that is less than or equal to that of the data waveform

- Assessing the impact of time domain vs frequency domain design of DMRS (Type 1 vs Type 2)

The above two considerations will eventually determine the DMRS design that it to be paired with FDSS with BWE.

#### PAPR Comparisons of DMRS with FDSS and BW Expansion

They key question of DMRS design for FDSS with BW expansion revolves around how the excess BW is handled and how the DMRS sequence is extended to span this bandwidth. For Type 2 DMRS since the sequence is generated in time domain, it is straightforward to mimic the same steps as data and extend the sequence cyclically in frequency domain. For Type 1 DMRS, since the sequence is generated directly in frequency domain, it is not entirely clear how best to extend it across the excess BW. Three potential solutions were identified:

- Type 1 DMRS sequence is mapped to RBs and then the RBs are cyclically extended

- Type 1 DMRS sequence is chosen assuming no excess BW and the chosen sequence is cyclically extended to span the excess BW, i.e. cyclic sequence extension, or

- Type 1 DMRS is directly generated to span the entire allocation, including the excess BW, thereby not requiring any further extension.

The PAPR of the waveforms resulting from the three approaches is plotted in the figures below. Its clear that Option (b) works best for Type 1 DMRS when allocations are large enough to use ZC sequences. Type 1 DMRS with CGS does not perform well with Option (b) --- if Type 1 DMRS is to be used with small allocations, a new set of sequences will be required. Notice also that Type 2 DMRS mirrors the operations followed for a data waveform and consequently has comparable or better PAPR. Thus Type 2 DMRS is also a good candidate DMRS for FDSS and BWE. This is summarized in the following proposal.

Figure 16 PAPR of DMRS waveforms with FDSS and BWE

Proposal 3: If FDSS with BWE is specified and it is agreed to be supported using low-PAPR Type 1 DMRS, at least for RB allocations > 4 RB, consider generating the DMRS sequence using a ZC sequence that is cyclically extended to span the excess RBs before being mapped to tones. - Note: This method of cyclic extension for DMRS symbol differs from that of data symbols. - FFS: Design of low-PAPR Type 1 DMRS for < 5 RB allocations with FDSS and BWE.

#### Link-level Performance Comparisons between Type 1 and Type 2 DMRS

As mentioned earlier, Type 1 and Type 2 DMRS differ in how the power is distributed in frequency domain. Type 1 DMRS distributes power uniformly, while Type 2 DMRS, due to its time-first design, results in a non-uniform distribution. Thus, there could be some concerns around the performance of realistic channel estimation using Type 2 DMRS compared to Type 1 DMRS. It is prudent to examine whether there is any significant performance difference in link performance when using two DMRS types. From Figure 17, Figure 18, and Figure 19, it can be seen that there is little to no performance difference when using either of the two DMRS types.

Figure 17 Baseline comparison to assess value FDSS with a total of 16 RBs with 2 excess RBs.  The case with excess RBs is compared against the case where excess RBs are used towards data allocation. Results with Type 1 and Type 2 DMRS are presented.

Figure 18 Baseline comparison to assess value FDSS with a total of 8 RBs with 2 excess RBs.  The case with excess RBs is compared against the case where excess RBs are used towards data allocation. Results with Type 1 and Type 2 DMRS are presented.

Figure 19 Baseline comparison to assess value FDSS with a total of 40 RBs with 10 excess RBs.  The case with excess RBs is compared against the case where excess RBs are used towards data allocation. Results with Type 1 and Type 2 DMRS are presented.

Based on these results, we make the following proposal:

Proposal 4: If FDSS with BWE is specified, existing 5G NR Type 2 (pi/2 BPSK) DMRS can be reused with similar bandwidth expansion and FDSS as data symbols. - For ≥5 RB allocations, another alternative is to reuse Type 1 (ZC-based) DMRS with cyclic extension of the ZC sequence followed by FDSS.

### Coverage Gains Using FDSS with BW Expansion

#### Potential Power Boost Using FDSS

Using FDSS with BW expansion, raw waveforms are shaped using non-transparent tone reservation to reduce their PAPR characteristics and then passed through an RF simulator to determine the resulting MPR. In the following figures, MPR and the potential power boost using FDSS with 25% excess BW are plotted. As seen in Figure 20, for inner RB allocations, a potential boost of 1.8 dB is possible. Outer RB allocations see gains of about 1.2-1.8 dB. Remarkably, no allocations require any MPR.

Figure 20 MPR and potential power boost using FDSS with BWE. 25% excess BW is assumed.

Caveats on power boost

It is non-trivial for a PC2 UE to transmit at powers greater than 26 dBm. Several RF challenges arise and the overall feasibility and reliability of such a UE warrants scrutiny. In particular, the additional input current to a PA and its implication on the design of the RF transceiver, thermal issues, reliability issues around RF components that have to sustain larger powers need to be examined closely.

Observation 8: For a PC2 UE, when a BW expansion factor of 25% is allowed, non-transparent FDSS can result in a potential power boost of about 1.8 dB for inner RB allocations and 1.2 to 1.8 dB for outer RB allocations. No MPR is required for any inner or outer waveform. Note that these results do not take other RF implementation challenges into account.

#### Net Coverage Gain

Using the potential power boost presented in the previous section, we aim to assess the net coverage gain that one can realize using FDSS with BWE. In the following table, we carry out such an analysis for the set of 10 scenarios agreed in the last meeting for calibration. We use link performance curves to assess the additional SNR required when FDSS is used compared to the case where all RBs are used towards legacy PUSCH transmission

Table 7 Coverage gain using FDSS and BWE

| Case | No spectrum extension |  | With spectrum extension |  |  |  | Δ SNR | Inner power boost | Net cov gain | Outer power boost | Net cov gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #PRBs | MCS | #PRBs before extension | #PRBs after extension | MCS | Spectrum extension factor |  |  |  |  |  |
| 1 | 16 | 7 | 14 | 16 | 8 | 1/8 | 1.9 | 1.2 | -0.7 | 0.5 | -1.4 |
| 2 | 32 | 8 | 28 | 32 | 9 | 1/8 | 2.1 | 1.2 | -0.9 | 0.5 | -1.6 |
| 3 | 8 | 0 | 6 | 8 | 1 | 1/4 | 0.5 | 1.7 | 1.2 | 1 | 0.5 |
| 4 | 8 | 6 | 6 | 8 | 8 | 1/4 | 1.8 | 1.7 | -0.1 | 1 | -0.8 |
| 5 | 40 | 2 | 30 | 40 | 3 | 1/4 | 0.7 | 1.7 | 1 | 1 | 0.2 |
| 6 | 40 | 6 | 30 | 40 | 8 | 1/4 | 1.7 | 1.7 | 0 | 1 | -0.7 |
| 7 | 16 | 0 | 10 | 16 | 2 | 3/8 | 0.5 | 1.8 | 1.3 | 0.3 | -0.2 |
| 8 | 32 | 2 | 20 | 32 | 4 | 3/8 | 0.9 | 1.8 | 0.9 | 0.3 | -0.6 |
| 9 | 8 | 2 | 6 | 8 | 3 | 1/4 | 0.5 | 1.7 | 1.2 | 1 | 0.5 |
| 10 | 24 | 0 | 18 | 24 | 1 | 1/4 | 0.1 | 1.7 | 1.6 | 1 | 0.9 |
| 11 | 4 | 1 | 2 | 4 | 4 | 1/2 | 0.8 | 1.4 | 0.6 | N/A | -- |
| 12 | 6 | 0 | 4 | 6 | 2 | 1/3 | 0.7 | 1.7 | 1.0 | 1 | 0.3 |
| 13 | 10 | 3 | 8 | 10 | 4 | 1/5 | 0.8 | 1.7 | 0.9 | 1 | 0.2 |

Observation 9: Based on the set of results presented in Table 7, the net coverage gain due to FDSS varies from 0.5 to 1.2 dB for inner RB allocations. A loss is noted in certain scenarios with high code rates. Similar observations with smaller gains (< 0.5 dB) are noted for outer RB allocations.

Next, we compare the above results with the coverage gain possible using transparent techniques and assess the net value of FDSS. The results are presented in the following table.

Table 8 Coverage gain comparison between transparent techniques and tone reservation

| Case | No spectrum extension |  | With spectrum extension |  |  |  | Net cov gain | Net cov gain | Net cov gain | Net cov gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #PRBs | MCS | #PRBs before extension | #PRBs after extension | MCS | Spectrum extension factor | TrPC Inner Alloc | Non-trans. FDSS Inner Alloc | TrPC Outer Alloc | Non-trans. FDSS Outer Alloc |
| 1 | 16 | 7 | 14 | 16 | 8 | 1/8 | 1.5 | -0.7 | 0 | -1.4 |
| 2 | 32 | 8 | 28 | 32 | 9 | 1/8 | 1.2 | -0.9 | 0 | -1.6 |
| 3 | 8 | 0 | 6 | 8 | 1 | 1/4 | 1.5 | 1.2 | 0 | 0.5 |
| 4 | 8 | 6 | 6 | 8 | 8 | 1/4 | 1.5 | -0.1 | 0 | -0.8 |
| 5 | 40 | 2 | 30 | 40 | 3 | 1/4 | 1 | 1 | 0 | 0.2 |
| 6 | 40 | 6 | 30 | 40 | 8 | 1/4 | 1 | 0 | 0 | -0.7 |
| 7 | 16 | 0 | 10 | 16 | 2 | 3/8 | 1.5 | 1.3 | 0 | -0.2 |
| 8 | 32 | 2 | 20 | 32 | 4 | 3/8 | 1.2 | 0.9 | 0 | -0.6 |
| 9 | 8 | 2 | 6 | 8 | 3 | 1/4 | 1.5 | 1.2 | 0 | 0.5 |
| 10 | 24 | 0 | 18 | 24 | 1 | 1/4 | 1.2 | 1.6 | 0 | 0.9 |
| 11 | 4 | 1 | 2 | 4 | 4 | 0.5 | 1.5 | 0.6 | 0 | -- |
| 12 | 6 | 0 | 4 | 6 | 2 | 0.33 | 1.5 | 1.0 | 0 | 0.3 |
| 13 | 10 | 3 | 8 | 10 | 4 | 0.2 | 1.5 | 0.9 | 0 | 0.2 |

Observation 10: FDSS with BWE provides marginal gains compared to transparent techniques for certain specific inner and outer RB allocations. In some cases, a net loss in coverage of more than 0.5 dB is observed. In particular, scenarios with high coding rate seem to see a large impact --- excess BW is noted to have a detrimental impact on performance as it further increases the required coding rate.

Based on these observations, we make the following proposal on non-transparent techniques for MPR/PAPR optimization:

Proposal 5: For RB allocations that are of interest to coverage enhancements with DFT-S-OFDM waveforms and QPSK modulation, it is suggested that transparent techniques such as peak cancelation be prioritized over non-transparent techniques such as tone reservation and FDSS with BW expansion. In particular, study mechanisms required to enable a UE to transmit at a power exceeding its power class.

# Enhancements to Realize High Power Transmission in CA and DC

## Introduction

In RAN1-110bis meeting, it was discussed whether any information exchange between UE and gNB can help facilitate high power transmissions in CA and DC. The following agreement was made in this regard:

Agreement For enhancements to realize increasing UE power high limit for CA and DC, RAN1 can study based on RAN4’s input Whether RAN1 enhancements to information exchange between UE and gNB are needed to improve scheduling and network performance when using higher power CA/DC. FFS how to realize such information exchange, e.g., signalling enhancement, and what is the spec impact.

In the subsequent meeting, in RAN1-111, the above agreement was further refined to identify three potential enhancements that might be worth studying:

Agreement - At least the following enhancements to information exchange between UE and gNB to facilitate higher power transmissions in CA and DC can be considered for study. Enhanced signaling, if necessary and subject to RAN4’s input, to allow: - Determination at gNB of power class change at the UE - Increased awareness at gNB of energy/power availability at the UE, e.g., a budget. - More informative PHR to be sent from UE to gNB, which may include, e.g., P-MPR related information, power headroom for carrier configured for DL but not UL, power class change indication. - More effective scheduling decisions in the context of UL CA, e.g., best band combination, preferred carrier for servicing uplink, adaptive load sharing across sharing, - Other options are not precluded.

While we wait for further input from RAN4 on high power transmissions for CA and DC, in the following sections we first investigate the issue of power class change at the UE and then move on to motivate how additional signalling on energy/power availability between UE and gNB might be important to facilitate high power transmissions in CA and DC.

## On Power-class Fallback at the UE

### Background

As per the current 5G NR specification, a PC2 UE is allowed to indicate (via capability) the maximum duty cycle up to which it is able to sustain PC2 power levels for uplink transmissions. For FR1 bands, the maximum duty cycle is indicated via the maxUplinkDutyCycle-PC2-FR1 parameter in the capability report. If this parameter is not reported by a PC2-capable UE, it is assumed to be 50%. When uplink scheduling configured or received by the UE is such that this requirement is violated, then the UE is allowed to fallback to operating as a PC3 UE.

It is noted that exactly when this fallback occurs is not very clear to the gNB as the UE needs to consider the percentage of uplink symbols transmitted in a certain evaluation period and determine whether it can continue to operate as a PC2 UE, but the exact evaluation period is not known to the gNB. For example, a UE averaging over 1 frame may behave differently compared to a UE averaging over 2 frames, etc.

While the above behavior is presented in a non-CA context, it also carries over to HPUE operation in CA mode, i.e., when UE indicates HigherPowerLimitCADC, the per-band-combination power class no longer applies and the UE is governed by the individual power classes of each band, provided ∆Ppowerclass is set to zero. However, determining the value of ∆Ppowerclass is tied to whether the duty cycling requirements are met or not. Thus, it is argued that the value of ∆Ppowerclass for a band at a given time may not be known to the gNB.  In such a case, the individual power classes of each band are determined by the UE based on the duty cycling requirements being met or not and whether higher power-limit capability is actually used or not may not be known to the gNB.

### Discussion

While the issue at hand is rather clear, what exactly is addressed and how it is addressed needs careful consideration. Towards this, we make a distinction between two types of reporting that a UE may undertake. A UE could merely report the value of ΔPpowerclass at any given point in time as part of PHR. Or alternately, the UE could make this reporting more valuable to the gNB by reporting how long the value of the ΔPpowerclass is likely to remain unchanged, or report conditions under which a power class downgrade can be avoided (for e.g., by reporting sustainable duty cycle to continue operating as a PC2 UE).

To further elaborate, note that PHR already provides the network with the instantaneous value of Pcmax, and the instantaneous headroom to that Pcmax. Adding ∆Ppowerclass to this report would not bring any useful non-predictive information to the network. For example, a fallback to default power class after passage of a time interval with estimated high RF exposure would be reflected as a lower Pcmax value in the PHR, while a report informing the network that ∆Ppowerclass of 3 dB is applicable conveys the same information but is in fact less granular.

Observation 11: ∆Ppowerclass reporting does not seem to provide the network with any novel information that would help with reliable prediction of the UE’s near-future UL power capability. It is more useful for the network to know how long a UE is capable of operating at a certain power class.

Further, not all UEs may use the power-class fallback mechanism. P-MPR provides a more nuanced approach to handle uplink tx power. A UE can use P-MPR to manage its ability to support a certain duty cycle, and avoid an abrupt change in power class. P-MPR may offer a more nuanced view of current state of UE transmitter compared to ∆Ppowerclass.

An actionable piece of information that might prove to be more valuable to the gNB is to let the gNB know of the duty cycle that a UE is able to sustain so as to prevent triggering a power class fallback. gNB can then take appropriate action to either prevent a fallback or be prepared for a fallback in case duty cycle requirements cannot be met. We make the following proposal:

Proposal 6: Consider introducing signaling from UE to gNB to indicate the sustainable duty cycle over a certain duration that would prevent triggering a power class fallback at the UE.

## Increased awareness at gNB of energy/power availability at the UE

### Background and Motivation

A commercial 5G-NR UE typically supports communications in several RF bands and is capable of being used globally across any 5G-NR network. The actual bands or band combinations of operation are determined by the spectrum holdings of an operator. A UE’s ability to support multiple bands presents a great opportunity to improve uplink coverage by efficiently using this ability to schedule uplink transmissions across different bands.

Typically, a UE uses a dedicated set of power amplifiers to support transmission in a band. The maximum power a UE can transmit in a band is governed by its power class. When operating across more than one band, although the UE can theoretically deliver maximum power across each of the bands in a band combination, the maximum total power a UE can deliver is governed by the power class associated with that band combination.

For example, a UE may declare itself as a PC3 (maximum output power of 23 dBm) UE in band X and band Y, and as a PC3 UE for the band combination X & Y. The PC3 declaration in band combination X & Y is driven less so by UE capability and more because the RAN4 specification only allows certain power classes for a particular band combination. This unnecessarily limits such a UE from transmitting at the combined maximum power for that band combination.

Recent changes in RAN4 have tried to address this issue. In the R17 WI on increasing UE power high limit for CA/DC, for band combinations where one band supports up to PC3 power class and another band supports up to PC2 power class, the total power associated with the band combination is assumed to be the sum of the individual power classes [5][6]. A new capability signalling is introduced per band combination is signal support for this relaxation. Under certain regulatory environments, there may be a limit on total output power. For such scenarios, Pemax,CA needs to be configured such that the total power limit is imposed. A similar relaxation for other power class pairings across bands is likely to be discussed.

Commercial cellular network deployments have not seen UL-CA being deployed widely and this could be attributed to the sum-power constraint that is imposed across band combinations. When this constraint is in place, the network is less incentivized to enable simultaneous transmissions across multiple bands in uplink. Anytime simultaneous transmissions are required, power sharing across bands kicks in, and primary uplink band can see a drop in uplink transmit power. The relaxation in the RAN4’s R17 WI aims to precisely address this situation.

As part of R18 WID on coverage enhancements, we would like to further facilitate smoother UL-CA operation. It is worth considering the various aspects a gNB needs to take into account to enable UL-CA across different bands. For example, how a gNB selects the band combination to configure for UL-CA or even when to switch a user from a single carrier configuration to UL-CA warrant further investigation. Further, to make UL-CA more effective, the focus should not be limited to increasing uplink transmit power. A more nuanced approach to multi-carrier scheduling also becomes critical in determining how a gNB services a UE’s uplink traffic by adaptive load sharing across multiple carriers.

An important component of enabling simultaneous transmissions across multiple bands is a good understanding of the regulatory constraints that the UE is required to conform to when engaging in uplink transmissions. Regulatory constraints on total RF exposure experienced by a user require a UE to determine safe transmit power levels for each band of operation. When a UE is close to exceeding the allowed RF exposure levels, the UE recomputes the maximum allowed transmit power for each of the bands in use and uplink transmissions may see a change in transmit power (Alternately, a UE may have a fixed limit on its transmit power so that it never exceeds the RF exposure limits.). Since these operations are transparent to the gNB, this can then impact a gNB’s perception of link quality, influence link adaptation, and subsequently cause uplink throughput to fluctuate.  A better understanding of these constraints and its impact on UE behaviour can help gNBs adopt more nuanced scheduling decisions.

Depending on the band combinations (FR1+FR1, FR1+FR2), two types of RF exposure limits can come into play. The first set uses a metric called specific absorption rate (SAR) and is applied to sub-6GHz bands and the second metric is called power density (PD) and is applied to mmW bands. Both these metrics take a time-averaged approach to limiting RF exposure, i.e., RF exposure as measured using these metrics and averaged over a certain time window must stay below a certain specified threshold. In other words, the regulations govern averaged RF exposure and not instantaneous RF exposure (there may be some exceptions to this in specific regions).  The time-averaged nature of these constraints is a rather important aspect when enabling high-power transmissions at the UE. It suggests that while  a UE may not be able to transmit at full power at all times across all bands, the UE is capable of delivering higher total transmit power is short bursts of time.

The goal in R18 coverage enhancements is to place scenarios with increased uplink transmit power in CA/DC on a more solid footing by providing additional signalling support to better facilitate a common understanding between UE and gNB. We propose to introduce energy headroom reporting to aid simultaneous high-power transmissions across multiple bands.

Note that some of these enhancements are not limited to CA/DC operation in uplink and are in general useful for any high-power UE where RF exposure limits may come into play due to simultaneous transmissions across different technologies (Wi-Fi, Bluetooth, etc).

The broad objectives to focus on for this sub-agenda is summarized in the following proposal:

- increasing awareness of power or energy budget available at the UE for each carrier/band,

- aiding the selection of the best band combination for UL CA, and

- aiding scheduling policy when UE is configured with multiple bands in UL CA, for e.g., selecting preferred carrier for servicing uplink, or adaptive load sharing across carriers.

In the following sections, we present results to show that high power transmissions in CA/DC are beneficial to a cell-edge UE. This is followed by a discussion on regulatory aspects of SAR/MPE compliance and aspects related to energy headroom reporting. We then present results to show how such signalling could be beneficial.

### System-level Simulations Illustrating the Benefits of High-Power UL-CA Without Sum-Power Constraint

To assess the benefit of allowing a higher transmit power in UL-CA, we present system-level simulations to quantify the amount of gains that can be realized, and the type of UEs that are likely to benefit the most. We consider UL-CA across a TDD and a FDD carrier, with the TDD carrier limited to 26 dBm (PC2) of transmit power and the FDD carrier limited to 23 dBm of transmit power. The combined power limit is set to either 26 dBm or to 27.8 (~ 28 dBm) dBm to understand the impact of relaxing the sum-power constraint applied to this band combination. The TDD carrier is assumed to be centered at 3.5 GHz with a total of 100 MHz, while the FDD carrier is centered at 2 GHz with a total of 20 MHz. An Urban Macro layout is assumed with an ISD of 500m. Additional simulation parameters are listed in the table below.

| Parameters | Values for FDD carrier | Values for TDD carrier |
| --- | --- | --- |
| Carrier frequency | 2GHz | 3.5GHz |
| Bandwidth | 20MHz | 100MHz |
| Subcarrier spacing | 15KHz | 30KHz |
| Deployment | Urban macro co-located with ISD = 500m (2 Tier, 19 gNBs, with total of 57 sectors) |  |
| BS transmit power | 46dBm for 10MHz |  |
| BS ant height | 25m |  |
| BS noise figure | 5dB |  |
| BS ant element gain | 8dBi |  |
| BS ant config | (M,N,P,Mg,Ng;Mp,Np)= (2,8,2,1,1;1,1) | (M,N,P,Mg,Ng;Mp,Np)= (8,4,2,1,1;1,1) |
| Downtilt | 102 deg |  |
| UE ant height | 1.5m |  |
| UE noise figure | 9dB |  |
| UE ant element gain | 0dBi |  |
| UE ant config | (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1) | (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1) |
| Thermal noise | -174dBm/Hz |  |
| Min BS-UE distance | 35m |  |
| UE distribution | Uniform distribution with Outdoor (20%) and indoor (80%) (10 UEs per sector, total 570 UEs) |  |
| UE transmit power (Option 1) | Up to 23dBm | Up to 26dBm |
|  | Total transmit power across two carriers is up to 26dBm |  |
| UE transmit power (Option 2) | Up to 23dBm | Up to 26dBm |
|  | Total transmit power across two carriers is NOT limited (i.e., up to 28dBm) |  |
| Fractional TPC | alpha = 0.8 | alpha = 0.8 |
| UL-MIMO | 1L | 1L or 2L |
|  | Totally up to 3 layers |  |
| TDD config |  | DDDSUDDSUU with S={10:2:2} |
| Traffic model | - Full-buffer (10 UEs/sector) |  |
| UL scheduler | - PF across UEs |  |

Figure 20 List of parameters used in system-level simulations to illustrate benefit of high power transmissions in uplink CA

Figure 21 Impact of increase uplink transmit power on cell-edge throughput

The CDF of uplink mean throughput in plotted in Fig. X for two scenarios: one where the sum power is limited to 26 dBm and the other where the sum-power can go up to 28 dBm. It can be seen that when sum-power limit is increased, the throughput of cell-edge UEs increases. In particular, for 5th percentile UEs, the total uplink throughput increases by a factor of 3. As expected, cell-center UEs do not benefit much from the increase in total transmit power.

Observation 12: Higher-power limit for inter-band UL-CA benefits the cell-edge UEs by reducing the impact of power sharing across simultaneous transmissions over multiple carriers and enhances the UE throughput.

### RF Exposure Basics: SAR and PD

A UE has to conform to certain regulatory requirements that govern the total RF exposure experienced by a user. RF exposure is categorized into two main categories. The first is called specific absorption rate (SAR) that applies to all RF exposure that occurs below 6 GHz and the other is called power density (PD) that applies to all RF exposure that occurs above 6 GHz. SAR is measured in units of Watts/kg and is intended to monitor the amount of power absorbed by a certain volume of tissue. PD is measured in Watts/cm2 and is intended to monitor the amount of power incident on the surface of the tissue.

SAR is used as the metric for the RF exposure when considering sub-6 GHz bands, while power density is used as a metric to address exposure from mmW bands. International Commission on Non-Ionizing Radiation Protection (ICNIRP) and FCC place restrictions on the maximum permissible exposure (MPE) on human body. However, the limit on SAR and PD cannot be counted separately. The individual SAR and PD values are normalized by the respectively regulatory limits and this normalized sum is required to be less than 1 as seen in the equation below:

i=100 kHz6 GHzSARiSARlim⁡+j=6 GHz300 GHzPDjPDlim ≤ 1

In the above equation, SARi refers to the time-averaged SAR in an RF band i and is given by

SARi=1TSARt-TSARtSARiτdτ,

where SARi(τ) is the instantaneous SAR in band i. Similarly, PDj refers to the time-averaged PD in band j and is given by

PDj=1TPDt-TPDtPDjτdτ,

where PDj(τ) is the instantaneous PD in band j. Note that TSAR and TPD are the respective time windows for averaging. SARlim⁡ is the total regulatory limit for SAR, and PDlim⁡ is the total regulatory limit for PD. SAR and PD calculations should include transmission across all frequencies and technologies (Wi-Fi, Bluetooth, etc). PD is typically measured over a 4 cm2 area and is limited to 1 mW/cm2 over a time averaging window of 4 seconds (this is applicable to frequencies between 24 GHz to 42 GHz). SAR is typically limited to 1.6 W/kg measured over 1g of tissue and is averaged over a window of 360 seconds. Some of the above measurement criteria may differ from one region to another and the numbers indicated above are meant to be representative.

Time averaging is a rather important aspect of complying with RF exposure limits. A UE’s transmissions are constantly monitored and a sliding window is used for computing the amount of RF exposure incurred within a certain time window and the the amount of RF exposure that can be allowed for a set of upcoming slots.

Observation 13: Regulatory constraints on RF exposure are determined by time-averaged RF exposure and not by instantaneous RF exposure.

This is illustrated in Figure 22 where at a certain time instant, the UE computes to total amount of exposure that occurred over a period of time in the past leading up to that instance in time and forecasts the amount of RF exposure that can then be allowed for a period of time in the immediate future.

Figure 22 Time-averaged RF exposure

Once the UE determines the available RF exposure for a certain period of time (for e.g. 500 ms) starting from current time, the UE can then use this exposure limit in a flexible manner while adhering to the power control settings, i.e., the UE can choose to allow a uniform amount of exposure spread out over the entire period of time or allow increased exposure in the initial portion, followed by a decreased amount of exposure in the remaining portion. These choices then determine the amount of transmit power available to transmit uplink data/control.

The following figures illustrates how a UE can translate the available RF exposure to instantaneous transmit power levels. As can be seen, the UE may be able to boost its power to a high-power level for a fraction of time located anywhere within the period of interest.

Figure 23 Forecasting energy availability and its impact on instantaneous power

Observation 14: A UE can determine the allowed RF exposure for a short duration of time in the future. The UE then translates this allowed RF exposure to instantaneous transmit power based on the transmission requirements in uplink.

A UE’s ability to temporarily boost its transmit power varies over time and depends on the set of active transmission bands. To enable concurrent transmissions across different bands in CA, its beneficial for the gNB to better understand a UE’s ability to transmit at high power across different bands at different points in time. In this context, it is pertinent to look at the existing mechanisms in the 5G NR specification that convey information on uplink transmit power to the gNB.

Focusing on power headroom reporting, a UE can report the instantaneous transmit power used to transmit a PUSCH transmission. The report consists of power headroom (6 bits), PCMAX,f,c (6 bits), and P-MPR (2 bits). Power headroom reflects the amount of additional transmit power a UE can deliver in reference to the current PUSCH power level, while PCMAX,f,c  reflects the maximum power a UE can deliver at that instance. P-MPR reporting is only enabled for FR2 bands. As it can be seen, this report only provides an instantaneous snapshot of the set of parameters that governed a particular PUSCH transmission. It carries no information on a UE’s ability to support future uplink transmission at this or higher power level. As a consequence, if a UE reports a low power headroom or low PCMAX, gNB often doesn’t know if this likely to change soon or if its going to be persistent and warrant gNB to change its scheduling strategies to accommodate this.

Figure 24 Single Cell PHR MAC CE with the following fields: 6 bit PH field, 6 bit P_CMAX field, 1 bit (P) to indicate whether MPE is being reported, and 2 bits to indicate MPE.

Observation 15: Existing PHR mechanism only carries information on instantaneous power levels used by a UE.

Observation 16 : For FR1 carriers, existing PHR mechanism does not allow a UE to report the applied power backoff due to RF exposure requirements.

In addition to this, as part of UE capability reporting, a UE is allowed to report maximum UL duty cycle a UE can support, i.e., maximum average percentage of symbols during a certain evaluation period that can be scheduled for uplink transmission. For non-CA operation, this reporting is per band and applicable to all power classes in FR2 and PC1.5 power class in FR1. For inter-band CA operation with PC2 power class, this reporting is per band.  This reporting is however static in nature and cannot be revised by a UE after initial call setup procedure is completed.

The above observations highlight the lack of existing mechanisms for a UE to report time varying transmit power availability to the gNB. The key goal of this discussion is to motivate the need for such signaling so that the gNB can adopt more nuanced scheduling decisions that factor in energy/power availability at the UE. These aspects become even more important for realizing high power transmission in CA/DC.

Observation 17: Time-averaged RF exposure constraints play an important role in realizing the objectives of the Release 17 RAN4 work on “Increasing UE power high limit for CA and DC”.

### Proposed Enhancements

As stated earlier, our overall goal for UE and gNB to have a better understanding of power/energy availability at the UE to facilitate more efficient UL CA operation. Towards this we think considering techniques or signalling mechanisms that provide a more nuanced view of RF exposure and allied aspects to the gNB are useful to consider.

Proposal 7: Introduce signaling to allow UE to report aspects related to power management and RF exposure.

We elaborate further in the next few sections.

#### Enhancements to Power Headroom Reporting

To address some of the drawbacks highlighted in the earlier section, we propose some changes to power headroom reporting that can facilitate smoother, more efficient UL-CA.

First we propose that P-MPR be reported for FR1 carriers as well. This offers the gNB some indication of the transmit power constraints currently in play at the UE. While it doesn’t quite indicate how this may change in the future, its nevertheless a useful piece of information for the gNB to factor into its overall scheduling framework.

Proposal 8: Enhance the current power headroom reporting framework to allow a user to also report P-MPR (via MPE field) for FR1 carriers.

Second, it is noticed that in 5G NR deployments, it is common for a UE to be configured with DL-CA across carriers in different bands, but for uplink, the UE is typically configured just a smaller subset of carriers. This puts the UE in a situation where the UE is monitoring and making measurements across different carriers to ensure downlink reception stays robust, but these measurements are not put to any good use for uplink in these carriers.

Figure 25 UE configured with multiple carriers in downlink and a single carrier in uplink.

At times it may happen that the UE may be in a unique position to determine that scheduling uplink transmissions in a carrier that is different from the carrier configured for uplink might actually be better for the network. This could be due to UE’s knowledge of regulatory constraints on RF exposure in each carrier and its ability to deliver a certain  amount of power on a particular carrier. There is however no existing mechanism for a UE to convey this information back to the gNB. Rather than have a gNB determine the carrier for uplink purely using DL measurements (e.g., RSRP, RSRQ, SINR, etc), it might make sense for a UE to also report the amount of power available in each carrier so that the gNB can then put both these inputs together to determine the better carrier to use in uplink. This then motivates the need to introduce additional signaling that lets gNB be aware of power available in an unconfigured uplink carrier through power headroom reporting. We make the following proposal:

Proposal 9: Enhance the current power headroom reporting framework to allow a user to report power headroom for a carrier that is configured for downlink but not for uplink (i.e., no active uplink BWP).

#### Energy Headroom Reporting

As highlighted in the previous sections, the time varying nature of RF exposure motivates introducing a new mechanism for a UE to report time varying transmit power availability to the gNB. To accomplish this, we propose that a UE report the amount of energy available periodically back to the gNB. This can be realized via energy headroom report or energy availability report (EAR/EHR) that accompanies a PHR or is independently transmitted back to the gNB.

Proposal 10: Introduce MAC-CE signalling to allow UE to report energy headroom for each of the bands in a CA/DC configuration given to the UE. - FFS: signalling details, including, periodicity, reporting triggers, relation to PHR, how to handle multiple bands, reference power, etc.

Figure 26 Energy headroom reporting per carrier

To illustrate the benefits of such signaling, we present simulation results to show that uplink data transfers can be optimized if gNB is aware of energy availability at the UE. Towards this, Figure 27 is intended to show that scheduling policies dictated by uplink duty cycle can have a large impact on rate of data transfer when a UE has a limited energy budget. This figure considers a single carrier link between a UE and a gNB and illustrates the impact of uplink scheduling policies on the total amount of data transferred over a window of 500 ms. In particular, the UE is assumed to have a fixed amount of energy available in each 500 ms window and we focus on average data transferred in a window of 500ms as the amount available energy varies at the UE. The figure shows how the choice of duty cycle can play a role in determining the number of uplink grants required to transfer a certain amount of payload. For lower duty cycles, UE can put more power towards each grant, thus boosting spectral efficiency and decreasing the number of grants it takes to transfer a certain amount of data. If uplink capacity maximizing is the only criterion, then water filling solution dictates that the gNB uses 100% duty cycle and UE uniformly distributes its energy across the slots. However, when the required data rates are well below capacity, there is room to optimize scheduling strategy to cut down on the number of grants, thereby reducing overhead and latency. Since uplink traffic is usually bursty in nature, we think these observations provide valuable insight for real-world deployments.

Observation 18: Lower duty cycles allow power boosting for uplink transmissions under a fixed energy budget at the UE. This results in faster data transfers using fewer grants.

Figure 27 Rate of data transfer as a function of uplink duty cycle for a energy limited UE.

With this as a motivating example, we now consider a 2-carrier UL-CA scenario and illustrate the value of a gNB being aware of energy availability at the UE. In  we illustrate how a gNB that is aware of energy availability at a UE can tailor its scheduling policy to dynamically select the rate at which a UE is scheduled within a certain time frame. The scenario assumes 2 CCs with different pathloss values and varying amounts of energy availability in each carrier. The energy availability in each carrier is assumed to be reported by the UE once every 500 ms. The gNB then tailors its scheduling policy based on (a) reported EHR, (b) pathloss, and (c) a desired average SNR threshold. Under this framework, if a UE reports a large EHR, gNB is more inclined to send more grants to this UE during this time period, while a small EHR triggers fewer grants, with the overall goal of ensuring the average SNR experienced per grant stays above a certain threshold. Aggressive SNR targets result in lower duty cycles, while relaxed SNR targets make sense for capacity maximization and result in higher duty cycles.

It can be seen from Figure 28 that such a framework allows a gNB to fine tune its scheduling policy based on EHR input from the UE. This results in the gNB being able to sustain a certain user perceived throughput with fewer grants than would be necessary otherwise.

Figure 28 Static scheduling vs. dynamic scheduling that uses energy availability at the UE to adapt uplink duty cycle. Duty cycle can be chosen based on low/medium/high SNR targets. High targets are well suited for quick transfer of small payloads.

### Case Study: Energy-aware Scheduling for TDD+FDD Switched UL

In this section we take the case of switched UL using a TDD and a FDD band and illustrate how a network can incorporate energy availability at a UE to help the gNB make better decisions on switching between the two carriers for uplink.

We consider a setup where a UE is configured for uplink over a TDD and a FDD carrier. Switched uplink is assumed --- the UE can transmit only on one of the two bands but not both. The max power on the TDD band is assumed to 26 dBm, while the max power on the FDD band is assumed to be 23 dBm. Further, it is assumed that due to RF exposure, the UE has to occasionally back off its tx power and set its P-MPR to a non-zero value. For the sake of simplicity, we assume that the P-MPR for uplink in the TDD band is always set to zero, but the P-MPR for the FDD band varies over time. The UE is assumed to change its P-MPR once every 100 ms. Only a small variation in power is assumed (1-2 dB). Thus, while the max power on TDD band remains steady, the max power on the FDD band varies over time.

Assuming a DDDSU slot pattern for TDD, once every 5 slots, the gNB needs to determine if it wishes to engage the UE in uplink over the TDD band or the FDD band. Multiple factors go into this decision making, including, pathloss, loading, traffic patterns, fairness, etc. To these factors, we introduce energy awareness --- i.e., gNB knows the amount of power a UE can transmit at in the next 100 ms. gNB scheduler then incorporates this information and determines expected link quality and chooses the most appropriate carrier.

In the absence of energy awareness, a gNB needs to make assumption on a UE’s ability to deliver high power in uplink and may not always have an accurate idea of the prevailing conditions at the UE. This can then lead to more retransmissions and inefficient use of uplink resources.

Detailed simulation assumptions are listed in the tables below.

Table 9 System  Simulation Parameters

Table 10 System Simulation Parameters

The uplink throughput achieved using such an approach is plotted in Figure 29 where it is seen that using energy-aware scheduling UL throughput improves by about 10% at the median compared to a scheduling strategy that is unaware of energy availability at the UE.

Figure 29 Uplink throughput with and without energy-aware scheduling

Observation 19: Using EHR, a gNB can adopt energy-aware scheduling that tailors its scheduling policy based on (a) reported EHR, (b) pathloss, and (c) a desired average UL reception SNR threshold, and other network parameters such as loading and traffic patterns. Such an approach is shown to improve the overall network throughtput in uplink.

# Conclusion

This contribution discusses some potential techniques for enhancing coverage in both FR1 and FR2. In particular, the following observations and proposals have been made:

On MPR/PAPR optimization:

Observation 1: MPR values only provide a lower bound on PCMAX,f,c. Not all UEs may make use of the allowed reduction in transmit power.

Proposal 1: For enhancements to reduce MPR/PAPR, prioritize inner RB allocations with small RB allocations, for e.g., 1-32 RBs.

Proposal 2: For enhancements to reduce MPR/PAPR, prioritize mechanisms that allow a 0-dB MPR waveform to be transmitted at a transmit power exceeding the maximum power associated with the UE power class.

On transparent schemes for MPR/PAPR optimization:

Observation 2: For a PC2 UE, using transparent techniques such transparent peak cancellation, a power boost of about 1-1.5 dB seems feasible for inner RB allocations. 1 dB MPR on outer RB allocations may still be required, primarily due to large RB allocations spanning more than 50% of the entire BW. Note that these results do not take other RF implementation challenges into account.

On tone reservation

Observation 3: With sideband tone reservation, gNB does not have to change its receiver design. Further, link-level performance of the waveform remains unchanged.

Observation 4: For a PC2 UE, when a BW expansion factor of 25% is allowed, tone reservation can result in a potential power boost of about 1-1.5 dB for inner RB allocations and 0.2 to 0.6 dB for certain outer RB allocations.  Note that these results do not take other RF implementation challenges into account.

Observation 5: Based on the set of results presented in Table 5, the net coverage gain due to tone reservation with 25% excess BW varies from 0.5 to 1.5 dB for inner RB allocations. No coverage gain is expected for outer RB allocations.

Observation 6: Tone reservation is not well-suited for outer-RB allocations. For inner-RB allocations, compared to transparent techniques, tone reservation brings marginal gains for certain specification allocations.

On FDSS with BW expansion

Observation 7: Filter design for FDSS is likely to depend on spectrum flatness requirements, target PAPR, RB allocation and amount of excess bandwidth. The choice of filter is likely to determine transmit power gains and link-level performance.

Observation 8: For a PC2 UE, when a BW expansion factor of 25% is allowed, non-transparent FDSS can result in a potential power boost of about 1.8 dB for inner RB allocations and 1.2 to 1.8 dB for outer RB allocations. No MPR is required for any inner or outer waveform. Note that these results do not take other RF implementation challenges into account.

Proposal 3: If FDSS with BWE is specified and it is agreed to be supported using low-PAPR Type 1 DMRS, at least for RB allocations > 4 RB, consider generating the DMRS sequence using a ZC sequence that is cyclically extended to span the excess RBs before being mapped to tones.

- Note: This method of cyclic extension for DMRS symbol differs from that of data symbols.

- FFS: Design of low-PAPR Type 1 DMRS for < 5 RB allocations with FDSS and BWE.

Proposal 4: If FDSS with BWE is specified, existing 5G NR Type 2 (pi/2 BPSK) DMRS can be reused with similar bandwidth expansion and FDSS as data symbols.

- For ≥5 RB allocations, another alternative is to reuse Type 1 (ZC-based) DMRS with cyclic extension of the ZC sequence followed by FDSS.

Observation 9: Based on the set of results presented in Table 7, the net coverage gain due to FDSS varies from 0.5 to 1.2 dB for inner RB allocations. A loss is noted in certain scenarios with high code rates. Similar observations with smaller gains (< 0.5 dB) are noted for outer RB allocations.

Observation 10: FDSS with BWE provides marginal gains compared to transparent techniques for certain specific inner and outer RB allocations. In some cases, a net loss in coverage of more than 0.5 dB is observed. In particular, scenarios with high coding rate seem to see a large impact --- excess BW is noted to have a detrimental impact on performance as it further increases the required coding rate.

Proposal 5: For RB allocations that are of interest to coverage enhancements with DFT-S-OFDM waveforms and QPSK modulation, it is suggested that transparent techniques such as peak cancellation be prioritized over non-transparent techniques such as tone reservation and FDSS with BW expansion. In particular, study mechanisms required to enable a UE to transmit at a power exceeding its power class.

On enhancements to realize high power uplink transmissions in CA and DC

Observation 11: ∆Ppowerclass reporting does not seem to provide the network with any novel information that would help with reliable prediction of the UE’s near-future UL power capability. It is more useful for the network to know how long a UE is capable of operating at a certain power class.

Proposal 6: Consider introducing signaling from UE to gNB to indicate the sustainable duty cycle over a certain duration that would prevent triggering a powerclass fallback at the UE.

Observation 12: Higher-power limit for inter-band UL-CA benefits the cell-edge UEs by reducing the impact of power sharing across simultaneous transmissions over multiple carriers and enhances the UE throughput.

Observation 13: Regulatory constraints on RF exposure are determined by time-averaged RF exposure and not by instantaneous RF exposure.

Observation 14: A UE can determine the allowed RF exposure for a short duration of time in the future. The UE then translates this allowed RF exposure to instantaneous transmit power based on the transmission requirements in uplink.

Observation 15: Existing PHR mechanism only carries information on instantaneous power levels used by a UE.

Observation 16: For FR1 carriers, existing PHR mechanism does not allow a UE to report the applied power backoff due to RF exposure requirements.

Observation 17: Time-averaged RF exposure constraints play an important role in realizing the objectives of the Release 17 RAN4 work on “Increasing UE power high limit for CA and DC”.

Proposal 7: Introduce signaling to allow UE to report aspects related to power management and RF exposure.

Proposal 8: Enhance the current power headroom reporting framework to allow a user to also report P-MPR (via MPE field) for FR1 carriers.

Proposal 9: Enhance the current power headroom reporting framework to allow a user to report power headroom for a carrier that is configured for downlink but not for uplink (i.e., no active uplink BWP).

Proposal 10: Introduce MAC-CE signaling to allow UE to report energy headroom for each of the bands in a CA/DC configuration given to the UE.

- FFS: signaling details, including, periodicity, reporting triggers, relation to PHR, how to handle multiple bands, reference power, etc.

Observation 18: Lower duty cycles allow power boosting for uplink transmissions under a fixed energy budget at the UE. This results in faster data transfers using fewer grants.

Observation 19: Using EHR, a gNB can adopt energy-aware scheduling that tailors its scheduling policy based on (a) reported EHR, (b) pathloss, and (c) a desired average UL reception SNR threshold, and other network parameters such as loading and traffic patterns. Such an approach is shown to improve the overall network throughtput in uplink.

# Reference

- R1-2008624, Baseline FR1 coverage performance, RAN1 103-e, October 26th – November 13th, 2020.

- R1-2008625, Baseline FR2 coverage performance, RAN1 103-e, October 26th – November 13th, 2020.

- J. Tellado, Multicarrier Modulation With Low PAR: Applications to DSL and Wireless, 2000

- R1-2006977, Potential coverage enhancement techniques for PUSCH, 3GPP TSG RAN1 WG1, Meeting 102-e, Qualcomm.

- R4-2210767, Increasing the maximum power limit for inter-band UL CA, RAN4 103-e, May 9th – 20th, 2022.

- R4-2210768, Increasing the maximum power limit for inter-band UL DC, RAN4 103-e, May 9th – 20th, 2022.