3GPP TSG RAN WG1 #112		    	 R1-2301445

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.16.1

Source:	Qualcomm Incorporated

Title:	NR support for dedicated spectrum less than 5MHz for FR1

Document for:	Discussion and Decision

# Introduction

Rel-18 work on NR support for dedicated spectrum less than 5MHz for FR1 has the following objectives for dedicated FDD spectrum in FR1 [1]:

- Identify and specify necessary changes to NR physical layer with minimum specification impact to operate in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN1]:

  - Restrict to subcarrier spacing of 15kHz and the use of normal cyclic prefix.

  - For SSB:

    - Reuse PSS/SSS specification without puncturing.

    - PBCH based on current design

  - Identify and specify necessary minimum changes to PDCCH, CSI-RS/TRS, PUCCH, and PRACH for functional support based on existing design, without optimization.

- Specify necessary RAN4 requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN4], including in bands n100, n8, n26 and n28:

  - Specify system parameters (including channel and sync rasters) for the associated dedicated spectrum.

  - Minimize impact on RF requirements:

    - Reuse 5 MHz channel bandwidth at least for FRMCS use case (assuming co-located NR and GSM-R with same operator).

    - Specify the required RF requirements for optional 3 MHz channel bandwidth in bands n100, n8, n26 and n28.

  - Specify RRM requirements while minimizing specification impact to support operation in dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz.

In this document, we focus on the RAN1 aspects and discuss the potential enhancement for the dedicated spectrum less than 5MHz.

# Discussion

# General framework of NR dedicated spectrum less than 5MHz

In previous RAN1 meeting, there are the following agreements:

Agreement

In an LS to RAN4, in addition to reuse 5 MHz channel bandwidth, RAN1 suppose only 3 MHz channel bandwidth is supported, and would like to get RAN4 responses on the maximum transmission bandwidth (the number of PRBs) for this channel BW.

Agreement

RAN1 would like to ask RAN4 if finer sync. raster for the 3MHz and/or 5MHz channel bandwidth is feasible, as well as any input from RAN1 for RAN4’s answer to this question.

Agreement

Before getting RAN4 responses, RAN1 assume maximum transmission bandwidth, 15RBs or 16RBs for 3 MHz channel BW for evaluation and analysis.

Note: include agreement into the LS

Agreement

Before getting RAN4 responses, RAN1 assume that the UE could know which RBs are used for SSB transmission after PSS/SSS is detected for evaluation and analysis.

Note: it does not mean indication signaling is needed.

Note: include this agreement into the LS

As clarified in WID, there are two cases to be considered:

- Case 1: Reuse 5 MHz channel bandwidth at least for FRMCS use case (assuming co-located NR and GSM-R with same operator).

- Case 2: Specify the required RF requirements for optional 3 MHz channel bandwidth in bands n100, n8, n26 and n28.

In Europe, the (E)R-GSM bands in n100 will be re-farmed for FRMCS to share 5.6MHz x 2 by NR railway (NR-R) and GSM-R. NR-R is deployed with collocated towers for GSM-R without guard bands, the UE performance is not affected by unwanted emission outside the 3.6MHz but within the 5MHz (as long as the relative transmit power between the two channels is not too high). In this case, the nominal channel bandwidth and BWP=5MHz can be reused and RAN1’s task is to limit the resource allocation / apply puncturing for “soft refarming”. If there is the bandwidth of 3.6MHz in the dedicated spectrum for FRMCS, the SSB of 20RBs can be reused without any puncturing. The subset of “Bandwidth allocations” can be further studied based on input from owners of spectrum. Considering digital filtering may still be needed before FFT-processing, the options for NR bandwidth allocation need to be down-selected and determined based on the requirement of spectrum holders as well as the RAN4 inputs on the minimum frequency separation from NR allocation to the closest GSM-R carrier. If the bandwidth smaller than 3.6MHz can be allocated for NR-R, Case 2 can be considered to allocate 3MHz for NR-R and remaining bandwidth for GSM-R. The other use cases for Case 2 are the respective dedicated spectrum for public safety in n26, n28 and smart grid in n8. For this case, a new channel bandwidth of 3MHz needs to be specified for NR. The detailed SSB pattern needs to consider the guard band per side as well as PSS/SSS location considering the impact of the sync raster, which will be dependent on RAN4 discussion. From RAN1 perspective, at least for now, we can assume SSB patterns with 15RBs or 16RBs to assess the impact on the PBCH link budget due to puncturing of 5RBs or 4RBs, where PSS/SSS is reused and fully confined within the restricted bandwidth. Whether to maximize the bandwidth up to 15RBs or 16RBs within 3MHz needs RAN4 evaluation on the guard band requirement. Considering the bandwidth of 15PRBs with SCS=15kHz is already supported in LTE, we slightly prefer 15RBs.

Proposal 1: RAN1 to support the following two Cases

- Case 1 for 5MHz channel BW: max transmission BW is 20RBs and SSB is reused without puncturing.

- Case 2 for 3MHz channel BW: max transmission BW is 15RBs or 16RBs, which is up to RAN4 feedback.

As discussed in previous meeting, the UE needs to know which RBs are used for SSB transmission after PSS/SSS with no blind detection. If a finer sync raster is designed for 3MHz channel bandwidth, the new sync and legacy sync raster should have the distance large enough against the oscillator frequency offset. It is possible for UE to differentiate the PBCH pattern without puncturing or with puncturing based on the frequency band and the sync raster.

Proposal 2: The early indication for Case 1 and Case 2 can be dependent on frequency band and sync raster

- For n8, n26 and n28, only Case 2

- For n100, whether it is Case 1 or Case 2 can be determined by the sync raster.

# Enhancement for NR dedicated spectrum less than 5MHz

## SSB

In previous RAN1 meeting, there are the following agreements on SSB for dedicated spectrum:

Agreement

For transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth, a subset of PRBs of 20-PRB PBCH are used for PBCH transmission if the transmission BW of a channel is less than 20PRBs.

- FFS which PRBs are used and how to use the PRBs

- Note: PRBs for PSS/SSS are not punctured.

Agreement

Study whether and how to recover PBCH detection performance for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered,

- Opt.1: Power boosting

- Opt.2: Multiple PBCH receptions

- Opt.3: PBCH remapping

- Opt.4: PBCH payload reduction

- Opt.5: PBCH rate matching around the punctured PRBs

- Opt.6: no enhancement specified

As discussed in Sect. 2.1, PBCH puncturing is needed for Case 2 to restrict the transmission within 3MHz. For the SSB pattern with punctured PBCH, the number of punctured PRBs depends on the RAN4 agreement on channel bandwidth. RAN1 can decide the location of punctured PRB taking into account the RAN4 sync raster design. In our view, it is preferable to define only one SSB pattern with PBCH puncturing for 3MHz channel bandwidth, and the PBCH puncturing should be done with PRB granularity. To minimize the implementation impact, the legacy PBCH/DMRS RE mapping should be reused, with no PBCH rate matching around the punctured PRBs.

Proposal 3:

- Define only one SSB pattern with PBCH puncturing for 3MHz channel bandwidth, taking RAN4 sync raster design into consideration

  - The PBCH puncturing is done with PRB granularity.

  - Reuse legacy PBCH/DMRS RE mapping and no PBCH rate matching around the punctured PRBs.

For evaluation, we assume the SSB patterns with 15PRBs or 16PRBs as illustrated in Figure 3, where PSS/SSS is reused without puncturing and edge RBs of the PBCH is punctured. We evaluated the link level performance degradation due to puncturing of PBCH. The assumption is that PBCH is transmitted on 900MHz with 4 transmit antennas at gNB side and 2 receive antennas at UE side, and ideal synchronization is assumed based on legacy PSS/SSS. In order to compensate the loss due to punctured RBs, we consider power boosting to increase the EPRE of the remaining PBCH RBs by 1.25dB for 5RB punctured and 0.97dB for 4RB punctured, respectively (such that the same total power is used for the whole PBCH in both cases).

- 15RBs												(b) 16RBs

Figure 3 PBCH puncturing

- AWGN channel											(b) TDL-C channel

Figure 4 PBCH performance

Table 1 summarizes the PBCH performance for AWGN and TDL-C channels. The PBCH puncturing results in power loss and coding gain loss of around 2~3dB. With the power boosting, the loss can be minimized to less than 0.5dB loss in AGWN channel and around 1dB loss in TDL-C channel.

Table 1 Required SINR for PBCH detection

- AWGN channel

|  | Required SINR at BLER=1% | Power boosting |
| --- | --- | --- |
| PBCH of 20RBs (baseline) | -10.5dB | 0 |
| PBCH of 16RBs | -9.2dB (+1.3dB) | 0 |
| PBCH of 15RBs | -8.7dB (+1.8dB) | 0 |
| PBCH of 16RBs | -10.1dB (+0.4dB) | +0.97dB |
| PBCH of 15RBs | -10.0dB (+0.5dB) | +1.25dB |

- TDL-C channel (100ns)

|  | Required SINR at BLER=1% | Power boosting |
| --- | --- | --- |
| PBCH of 20RBs (baseline) | -3.3dB | 0 |
| PBCH of 16RBs | -1.6dB (+1.7dB) | 0 |
| PBCH of 15RBs | -1.1dB (+2.2dB) | 0 |
| PBCH of 16RBs | -2.5dB (+0.8dB) | +0.97dB |
| PBCH of 15RBs | -2.3dB (+1.0dB) | +1.25dB |

Observation 1: For BW of 3MHz, PBCH puncturing may result in 1~2dB loss, but power boosting on the remaining RBs can minimize the loss to 0.5~1.0dB.

Proposal 4: Support power boosting on remaining PBCH/DMRS for 3MHz channel BW and keep same EPRE for SSS and PBCH/DMRS.

## CORESET0

In previous RAN1 meeting, there are the following agreements for the CORESET0 for dedicated spectrum:

Agreement

For CORESET#0 configuration for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth, following options are for study,

- Opt.1: Existing configuration table for 15kHz SCS, 5MHz minimum channel BW (i.e., table 13-1 in TS38.213) is reused for configuration

- Opt.2: A new CORESET#0 configuration table is to be introduced for the configuration.

Agreement

Study whether and how to recover PDCCH detection performance of CORESET#0 for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered,

- Opt.1: Power boosting

- Opt.2: Non-interleaved CCE-to-REG mapping

- Opt.3: A new interleaver to ensure PDCCH is fully mapped in the spectrum

- Opt.4: New aggregation level(s) for fit in the spectrum

- Opt.5: PDCCH rate matching

- Opt.6.: no enhancement specified

In the following, we discuss the CORESET0 design from the aspects of resource mapping, puncturing pattern, PDCCH candidates and configuration indication.

### 1) Resource mapping for CORESET0

Legacy CORESET0 requires min 24RBs with bandwidth of 4.32MHz. Therefore, for both Case 1 and Case 2, the bandwidth for CORESET0 needs to be restricted. For legacy 5MHz with 24RBs, CORESET0 has CCE with REG-bundling size of L=6 and interlever with interleaverSize R=2, defined as

fx=rC+c+nshift mod NREGCORESETL
x=cR+r
r=0,1,…,R-1
c=0,1,…,C-1
C=NREGCORESET(LR)

However, the interleaver with R=2 cannot be directly applied to 15-RB CORESTE0 or 2-symbol 16-RB CORESET0. To minimize the specification impact, we propose to reuse the CCE-to-REG mapping and apply no interleaving to CORESET0. Then, we only need to determine how to puncture the 24-RB CORESET0 to fit into dedicated spectrum.

Figure 5 and Figure 6 show the punctured CORESET0 for Case 1 with 20RBs and Case 2 with 15RBs, respectively, assuming no interleaving and RB offset relative to the first RB of SSB after puncturing is 0 to maximize the available CCEs and AL for PDCCH in punctured CORESET0. If RB offset=2 or 4RBs, the starting RBs of larger AL will be punctured, which results in a smaller number of PDCCH candidates.

Considering the limited TUs for this WI, RAN1 should be cautious about introducing new interleaving or new aggregation level for PDCCH for the dedicated spectrum, which may require more specification impact and additional test cases.

Figure 5 CORESET0 for 5MHz channel bandwidth with max 20RBs

Figure 6 CORESET0 for 3MHz channel bandwidth with max 15RBs

Proposal 5: For 5MHz and 3MHz channel bandwidth with less than 5MHz transmission bandwidth, CORESET0 uses

- no interleaving

- RB offset=0

- L=6 for REG bundle size

### 2) Puncturing pattern for CORESET0

In case of 20RBs for 5MHz channel bandwidth (Figure 5), AL=4 and 8 can be supported. However, in case of only 15RBs for 3MHz (Figure 6), part of the REG-bundle 7 in case of 3-symbol PDCCH will be out of the max transmission bandwidth. In order to support large AL, instead of indicating 15-RB CORESET0, gNB can allocate 16RBs for CORESET0 but puncture one or two RBs to limit the transmission within 15RBs. The puncturing pattern may be RB-level or REG-bundle-level. As illustrated in Figure 7, if precoding is wideband across all REGs, Alt1 can be applied; but if precoding is within a REG-bundle, Alt2 is needed considering the legacy NR does not support partial precoding within a REG-bundle and the UE also may not be able to do partial REG-bundle channel estimation.

Alt1: RB-level puncturing

Alt2: REG-bundle-level puncturing

Figure 7 Puncturing pattern for CORESET0 in 3MHz channel bandwidth

In Figure 8, we compare 3-symbol PDCCH detection performance with AL=4 and 8 assuming no puncturing and puncturing based on Alt1/2, respectively. The assumption is that PDCCH is transmitted on 900MHz with 4 transmit antennas at gNB side and 2 receive antennas at UE side and ideal synchronization is assumed based on legacy PSS/SSS. Table 2 summarizes the required SINR at BLER=1% for PDCCH detection.

- AL=8														(b) AL=4

Figure 8 PDCCH performance for CORESET puncturing with 3-symbol case

Table 2 Required SINR for 3-symbol PDCCH detection

- AL=8

| AL=8 | Channel estimation | Required SINR at 1% BLER |
| --- | --- | --- |
| w/o puncturing (baseline) | Within a REG-bundle | -4.12dB |
| w/ REG-bundle puncturing (RB index=14, 15) | Within a REG-bundle | -3.28dB (+0.84dB) |
| w/o puncturing | Across all REGs | -4.70dB (-0.58dB) |
| w/ RB puncturing (RB index=15) | Across all REGs | -4.25dB (-0.13dB) |

- AL=4

| AL=4 | Channel estimation Precoding | Required SINR at 1% BLER |
| --- | --- | --- |
| w/o puncturing (baseline) | Within a REG-bundle | -0.14dB |
| w/ REG-bundle puncturing (RB index=14, 15) | Within a REG-bundle | 1.57dB (+1.71dB) |
| w/o puncturing | Across all REGs | -0.25dB (-0.11dB) |
| w/ RB puncturing (RB index=15) | Across all REGs | 0.49dB (+0.63dB) |

Observation 2: If 16RBs are allocated to CORESET0 for BW of 3MHz with max 15RBs,

- Compared with legacy AL=8 (assuming no puncturing and channel estimation within a REG-bundle), AL=4 with no puncturing has around 4dB loss.

- Compared with legacy Al=8, 1RB puncturing for AL=8 with channel estimation across all REGs can achieve similar performance, which is ~1dB better than 1REG-bundle puncturing for AL=8 with channel estimation within a REG-bundle.

- Compared with legacy Al=4, 1RB puncturing for AL=4 with channel estimation across all REGs only has loss of 0.63dB, which is ~1dB better than 1REG-bundle puncturing for AL=4 with channel estimation within a REG-bundle.

Therefore, even for max 15RBs transmission bandwidth, we propose to support the AL=8 for 3-symbol CORESET by indicating the 16-RB CORESET0 and the 1RB punctured to minimize the PDCCH performance loss. For AL=1 and 2, if the CCE has the punctured RB, the corresponding PDCCH candidate cannot be used due to the high coding rate over the limited remaining RBs after puncturing. The allocation of using the punctured PDCCH candidate for AL=1 and 2 can be avoided by gNB scheduling.

Proposal 6: If 3MHz channel bandwidth has max transmission bandwidth of 15RBs, CORESET0 can be configured with 16RBs, 3 symbols assuming the same precoding across all REGs to support AL=4 or 8 with 1RB puncturing.

- AL=1 or 2 with puncturing is not supported.

### 3) PDCCH candidates in CORESET0

Regarding the PDCCH candidate monitoring, it should be based on the existing hashing punction, where the number of CCEs NCCE,p is counted using the indicated RB number for CORESET0, instead of the maximum transmission bandwidth. For the 3MHz channel bandwidth with max 15RBs transmission bandwidth, as shown in Figure 6.

- In case of 2-symbol CORESET0 with 15RBs, the number of CCEs NCCE,p is equal to 5 and the candidate number per aggregation level L is equal to NCCE,pL, i.e., 1 for AL=4, 2 for AL=2 and 5 for AL=1

- In case of 3-symbol CORESET0 with 16RBs, the number of CCEs NCCE,p is equal to 8 and the candidate number per aggregation level L is equal to NCCE,pL, i.e., 1 for AL=8, 2 for AL=4, 4 for AL=2 and 8 for AL=1.

For the 5MHz channel bandwidth with max 20RBs transmission bandwidth, AL=4 and 8 for 3-symbol CORESET can be supported without puncturing. For the PDCCH candidate monitoring, as shown in Figure 5,

- In case of 2-symbol CORESET0 with 18RBs, the number of CCEs NCCE,p is equal to 6 and the candidate number per aggregation level L is equal to NCCE,pL, i.e., 1 for AL=4, 2 for AL=2 and 3 for AL=1

- In case of 3-symbol CORESET0 with 20RBs, the number of CCEs NCCE,p is equal to 10 and the candidate number per aggregation level L is equal to NCCE,pL, i.e., 1 for AL=8, 2 for AL=4, 5 for AL=2 and 10 for AL=1.

Proposal 7: Reuse hashing function for PDCCH candidates assuming the number of CCEs counted based on the configured RB number for CORESET0 for the 3MHz and 5MHz channel bandwidth as

| Number of RBs NRBCORESET | Number of SymbolsNsymbolCORESET | Offset (RBs) | Interleaving | Precoding Granularity | Puncturing | Number of CCEs |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 2 | 0 | No | allContiguousRBs | - | 5 |
| 16 | 3 | 0 | No | allContiguousRBs | RB index =15 | 8 |
| 18 | 2 | 0 | No | allContiguousRBs | - | 6 |
| 20 | 3 | 0 | No | allContiguousRBs | - | 10 |

### 4) Indication of CORESET0 configuration

RAN1 needs to discuss how to use MIB/PBCH to indicate the CORESET0 configuration for dedicated spectrum less than 5MHz. The existing entries in Table 13-1 of TS38.213 for 5MHz with 24RBs cannot be directly applied. RAN1 needs to consider new entries/table for the dedicated frequency bands with 3MHz channel bandwidth and 5MHz channel bandwidth.

Table 13-1: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {15, 15} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of SymbolsNsymbolCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 2 |
| 2 | 1 | 24 | 2 | 4 |
| 3 | 1 | 24 | 3 | 0 |
| 4 | 1 | 24 | 3 | 2 |
| 5 | 1 | 24 | 3 | 4 |
| 6 | 1 | 48 | 1 | 12 |
| 7 | 1 | 48 | 1 | 16 |
| 8 | 1 | 48 | 2 | 12 |
| 9 | 1 | 48 | 2 | 16 |
| 10 | 1 | 48 | 3 | 12 |
| 11 | 1 | 48 | 3 | 16 |
| 12 | 1 | 96 | 1 | 38 |
| 13 | 1 | 96 | 2 | 38 |
| 14 | 1 | 96 | 3 | 38 |
| 15 | Reserved |  |  |  |

On the other hand, the dedicated spectrum can only be accessed by new UEs. It would be a problem if legacy UEs (e.g., with good SINR) can detect the SIB1 transmitted within the restricted bandwidth and send PRACH to access the dedicated spectrum. Therefore, early indication in MIB/PBCH is needed to avoid legacy UEs to monitor the CORESET0 in dedicated spectrum with unnecessary power consumption.

In MIB/PBCH, the parameter kSSB is used to indicate the SSB subcarrier offset in terms of 15kHz SCS relative to lowest subcarrier of common resource blocks for SSB, with 4-bit MSB indicated by ssb-SubcarrierOffset and 1-bit LSB in PBCH payload. As defined in legacy NR,

- If kSSB=0~23, legacy UE will determine the CORESET0 for an associated Type0-PDCCH CSS set based on Table 13-1~15 in TS38.213.

- If kSSB=24~30, the UE may determine the nearest (in the corresponding frequency direction) GSCN of a second SSB block having a CORESET for an associated Type0-PDCCH CSS set as NGSCNrefernce+NGSCNoffset. NGSCNrefernce is the GSCN of the first SSB and NGSCNoffset is the GSCN offset provided by Table 13-16 for FR1.

- If kSSB=31, the UE determines that there is no SS/PBCH block having an associated Type0-PDCCH CSS set within a GSCN range [NGSCNrefernce-NGSCNstart,NGSCNrefernce+NGSCNend. NGSCNstart and NGSCNEnd are respectively determined by CORESET0/SS0 in pdcch-ConfigSIB1.

Table 13-16: Mapping between the combinationof k_SSB and 
controlResorceSetZero and searchSpaceZero in pdcch-ConfigSIB1 to NGSCNoffset for FR1

| kSSB | 16×controlResourceSetZero +searchSpaceZero | NGSCNoffset |
| --- | --- | --- |
| 24 | 0, 1, …, 255 | 1, 2, …, 256 |
| 25 | 0, 1, …, 255 | 257, 258, …, 512 |
| 26 | 0, 1, …, 255 | 513, 514, …., 768 |
| 27 | 0, 1, …, 255 | -1, -2, …, -256 |
| 28 | 0, 1, …, 255 | -257, -258, …, -512 |
| 29 | 0, 1, …, 255 | -513, -514, …., -768 |
| 30 | 0, 1, …, 255 | Reserved, Reserved, …, Reserved |

But for the dedicated spectrum with less than 5MHz, the SSB with RB-level puncturing will be fit into the channel bandwidth and the subcarrier offset from subcarrier 0 in common frequency resource block in the dedicated spectrum to subcarrier 0 of SSB after puncturing is equal to 0, i.e., kSSB=0. Therefore, kSSB can be predefined as 0 for new UEs. The MIB/PBCH can indicate kSSB=30 (reserved in Table 13-16) to make legacy UEs ignore this cell because there is no corresponding CORESET0. The new UEs, capable of accessing the dedicated spectrum with less than 5MHz, will use a new entries/table for the CORESET0 with transmission bandwidth <24RBs. For example, Table 13-1B is illustrated for mapping the CORESET0 index to the CORESET0 configuration parameters for 3MHz and 5MHz channel bandwidth, respectively.

Table 13-1B: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {15, 15} kHz for the frequency bands with minimum channel bandwidth 3 MHz or 5 MHz given in [5, TS38.101-1] and kSSB=30

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of SymbolsNsymbolCORESET | Offset (RBs) | Interleaving | Precoding Granularity | Puncturing | Number of CCEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 15 | 2 | 0 | No | allContiguousRBs | - | 5 |
| 1 | 1 | 16 | 3 | 0 | No | allContiguousRBs | RB index =15 | 8 |
| 2 | 1 | 18 | 2 | 0 | No | allContiguousRBs | - | 6 |
| 3 | 1 | 20 | 3 | 0 | No | allContiguousRBs | - | 10 |
| 4~15 | Reserved |  |  |  |  |  |  |  |

Proposal 8: The dedicated spectrum can only be accessed by dedicated UEs.

- Legacy UEs will ignore the cell by indicating kSSB=30 by MIB/PBCH.

- New UEs capable of accessing the dedicated spectrum will assume kSSB=0 and use Table 13-1B to indicate the CORESET0 configuration for 3MHz and 5MHz channel bandwidth.

Table 13-1B: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {15, 15} kHz for the frequency bands with minimum channel bandwidth 3 MHz or 5 MHz given in [5, TS38.101-1] and kSSB=30

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of SymbolsNsymbolCORESET | Offset (RBs) | Interleaving | Precoding Granularity | Puncturing | Number of CCEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 15 | 2 | 0 | No | allContiguousRBs | - | 5 |
| 1 | 1 | 16 | 3 | 0 | No | allContiguousRBs | RB index =15 | 8 |
| 2 | 1 | 18 | 2 | 0 | No | allContiguousRBs | - | 6 |
| 3 | 1 | 20 | 3 | 0 | No | allContiguousRBs | - | 10 |
| 4~15 | Reserved |  |  |  |  |  |  |  |

## CSI-RS/TRS

As indicated in the WID, RAN1 needs to identify and specify necessary minimum changes to PBCH, PDCCH, CSI-RS/TRS, PUCCH, and PRACH for functional support based on existing design, without optimization.

Legacy CSI-RS is configured with 4n RBs, The UE shall expect that NCSI-RSBW≥min(24, NBWPsize). For Case 1 with nominal BWP=5MHz, the min number of RBs for CSI-RS is 24RBs, which will be out of the BW 3.6MHz. In Rel-16, flexible TRS BW can be configured as 5~10MHz if BWP=10MHz based on UE capability [2]. Similar concept can be extended to define flexible TRS with size of 20RBs within BWP=5MHz based on UE capability.

For Case 2 with newly defined BWP=3MHz, the number of RBs for CSI-RS is NBWPsize=NBWPsize, and there is no need to introduce optional flexible TRS for this case.

Proposal 9: RAN1 to support

- For Case 1 with nominal BWP=5MHz, introduce UE capability of supporting flexible CSI-RS/TRS with 20RBs.

- For Case 2 with BWP=3MHz, CSI-RS/TRS can be the same as the maximum transmission bandwidth.

## PUCCH

Conclusion

No enhancements are needed for PUCCH to support transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth,

- FFS: the necessity for PUCCH FH disabling.

In NR, the PUCCH with dedicated PUCCH-Config can be configured within startingPRB and secondHopPRB after frequency hopping within the UL BWP. However, the PUCCH resources before dedicated PUCCH-Config is based on cell-specific pucch-ResourceCommon, which is defined in Table 9.2.1-1 of TS38.213.

For the PUCCH transmission for msg4, the PRB index in the first hop is  and the PRB index in the second hop is. For Case 2 with BWP=3MHz, there is no issue to reuse the legacy frequency hopping of PUCCH (assuming a BWP of 3MHz is used). However, for Case 1 with nominal UL BWP=5MHz, the PUCCH transmission with frequency hopping may be out of the restricted BW.

Considering we do not expect a significant gain of frequency hopping in BW less than 5MHz, PUCCH frequency hopping for msg4 can be disabled. Since legacy UEs will not access the dedicated spectrum, disabling for PUCCH FH for msg4 can be predefined when new UEs accessing 20RBs with nominal 5MHz and no need to use the Rel-17 RedCap flag in SIB1 for msg4.

Table 9.2.1-1: PUCCH resource sets before dedicated PUCCH resource configuration

| Index | PUCCH format | First symbol | Number of symbols | PRB offset | Set of initial CS indexes |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 2 | 0 | {0, 3} |
| 1 | 0 | 12 | 2 | 0 | {0, 4, 8} |
| 2 | 0 | 12 | 2 | 3 | {0, 4, 8} |
| 3 | 1 | 10 | 4 | 0 | {0, 6} |
| 4 | 1 | 10 | 4 | 0 | {0, 3, 6, 9} |
| 5 | 1 | 10 | 4 | 2 | {0, 3, 6, 9} |
| 6 | 1 | 10 | 4 | 4 | {0, 3, 6, 9} |
| 7 | 1 | 4 | 10 | 0 | {0, 6} |
| 8 | 1 | 4 | 10 | 0 | {0, 3, 6, 9} |
| 9 | 1 | 4 | 10 | 2 | {0, 3, 6, 9} |
| 10 | 1 | 4 | 10 | 4 | {0, 3, 6, 9} |
| 11 | 1 | 0 | 14 | 0 | {0, 6} |
| 12 | 1 | 0 | 14 | 0 | {0, 3, 6, 9} |
| 13 | 1 | 0 | 14 | 2 | {0, 3, 6, 9} |
| 14 | 1 | 0 | 14 | 4 | {0, 3, 6, 9} |
| 15 | 1 | 0 | 14 |  | {0, 3, 6, 9} |

Proposal 10: RAN1 to support disabling PUCCH frequency hopping for msg4 at least for Case 1 with nominal BWP=5MHz.

## PRACH

Conclusion

- No enhancements are required for PRACH to operate NR on transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth.

  - Note: PRACH formats and configurations not fitting into the transmission BW are not applicable

Agreement

- Short PRACH formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS are supported for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth.

The PRACH preamble formats can be used in less than 5MHz are:

- Short PRACH has 2.16MHz bandwidth

- Long PRACH with format 0/1/2 uses 1.08MHz bandwidth.

Within the spectrum less than 5MHz, the uplink capacity of initial access would be very restricted. If long PRACH format is configured and one PRACH occasion has 1.08MHz (equivalent to 6RBs with 15kHz SCS), it is beneficial to support up to 3 FDM PRACH occasions within 20RBs. Note that the current RRC signalling only supports 1, 2, 4 or 8 FDM PRACH occasions.

Proposal 11: RAN1 to support 3-FDM PRACH occasions with 1.25kHz SCS for Case1 with max transmission bandwidth of 20RBs.

# 3. Conclusion

Based on the discussion, we have following proposals:

Proposal 1: RAN1 to support the following two Cases

- Case 1 for 5MHz channel BW: max transmission BW is 20RBs and SSB is reused without puncturing.

- Case 2 for 3MHz channel BW: max transmission BW is 15RBs or 16RBs, which is up to RAN4 feedback.

Proposal 2: The early indication for Case 1 and Case 2 can be dependent on frequency band and sync raster

- For n8, n26 and n28, only Case 2

- For n100, whether it is Case 1 or Case 2 can be determined by the sync raster.

Proposal 3:

- Define only one SSB pattern with PBCH puncturing for 3MHz channel bandwidth, taking RAN4 sync raster design into consideration

  - The PBCH puncturing is done with PRB granularity.

  - Reuse legacy PBCH/DMRS RE mapping and no PBCH rate matching around the punctured PRBs.

Proposal 4: Support power boosting on remaining PBCH/DMRS for 3MHz channel BW and keep same EPRE for SSS and PBCH/DMRS.

Proposal 5: For 5MHz and 3MHz channel bandwidth with less than 5MHz transmission bandwidth, CORESET0 is using

- no interleaving

- RB offset=0

- L=6 for REG bundle size

Proposal 6: If 3MHz channel bandwidth has max transmission bandwidth of 15RBs, CORESET0 can be configured with 16RBs, 3 symbols assuming the same precoding across all REGs to support AL=4 or 8 with 1RB puncturing.

- AL=1 or 2 with puncturing is not supported.

Proposal 7: Reuse hashing function for PDCCH candidates assuming the number of CCEs counted based on the configured RB number for CORESET0 for the 3MHz and 5MHz channel bandwidth as

| Number of RBs NRBCORESET | Number of SymbolsNsymbolCORESET | Offset (RBs) | Interleaving | Precoding Granularity | Puncturing | Number of CCEs |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 2 | 0 | No | allContiguousRBs | - | 5 |
| 16 | 3 | 0 | No | allContiguousRBs | RB index =15 | 8 |
| 18 | 2 | 0 | No | allContiguousRBs | - | 6 |
| 20 | 3 | 0 | No | allContiguousRBs | - | 10 |

Proposal 8: The dedicated spectrum can only be accessed by dedicated UEs.

- Legacy UEs will ignore the cell by indicating kSSB=30 by MIB/PBCH.

- New UEs capable of accessing the dedicated spectrum will assume kSSB=0 and use Table 13-1B to indicate the CORESET0 configuration for 3MHz and 5MHz channel bandwidth.

Table 13-1B: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {15, 15} kHz for the frequency bands with minimum channel bandwidth 3 MHz or 5 MHz given in [5, TS38.101-1] and kSSB=30

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of SymbolsNsymbolCORESET | Offset (RBs) | Interleaving | Precoding Granularity | Puncturing | Number of CCEs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 15 | 2 | 0 | No | allContiguousRBs | - | 5 |
| 1 | 1 | 16 | 3 | 0 | No | allContiguousRBs | RB index =15 | 8 |
| 2 | 1 | 18 | 2 | 0 | No | allContiguousRBs | - | 6 |
| 3 | 1 | 20 | 3 | 0 | No | allContiguousRBs | - | 10 |
| 4~15 | Reserved |  |  |  |  |  |  |  |

Proposal 9: RAN1 to support

- For Case 1 with nominal BWP=5MHz, introduce UE capability of supporting flexible CSI-RS/TRS with 20RBs.

- For Case 2 with BWP=3MHz, CSI-RS/TRS can be same as the maximum transmission bandwidth.

Proposal 10: RAN1 to support disabling PUCCH frequency hopping for msg4 at least for Case 1 with nominal BWP=5MHz.

Proposal 11: RAN1 to support 3-FDM PRACH occasions with 1.25kHz SCS for Case1 with max transmission bandwidth of 20RBs.

# References

- RP-222645 New WID on NR support for dedicated spectrum less than 5MHz for FR1, 3GPP TSG RAN Meeting #97e Electronic Meeting, 12th Sep 2022 - 16th Sep 2022

- RP-201333, “Final Proposal for Flex_BW”, 3GPP TSG RAN Meeting #88e Electronic Meeting, June 29 - July 3, 2020.