3GPP TSG RAN WG1 #112			R1-2301463

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.16.1

Source:	KT Corp.

Title:	On enhancements to operate NR on dedicated spectrum less than 5 MHz

Document for:	Discussion and decision

1. Introduction

Some targets of the specification of dedicated spectrum less than 5MHz are presented as applications like the evolved CII (Critical infrastructure industry) in US and FRMCS (Future Railway Mobile Communication System) in Europe [2]. The object of this specification especially for RAN1 is derived in [2] as follows:

- Identify and specify necessary changes to NR physical layer with minimum specification impact to operate in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN1]:

  - Restrict to subcarrier spacing of 15kHz and the use of normal cyclic prefix.

  - For SSB:

    - Reuse PSS/SSS specification without puncturing.

    - PBCH based on current design

  - Identify and specify necessary minimum changes to PDCCH, CSI-RS/TRS, PUCCH, and PRACH for functional support based on existing design, without optimization.

During the RAN1 #111 meeting, some directions have been agreed for modifying structure of common signals already specified in NR for the dedicated spectrum, especially targeting 3MHz. For SSB, following agreements are made in [1]:

Agreement

For transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth, a subset of PRBs of 20-PRB PBCH are used for PBCH transmission if the transmission BW of a channel is less than 20PRBs.

- FFS which PRBs are used and how to use the PRBs

- Note: PRBs for PSS/SSS are not punctured.

Agreement

Study whether and how to recover PBCH detection performance for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered,

- Opt.1: Power boosting

- Opt.2: Multiple PBCH receptions

- Opt.3: PBCH remapping

- Opt.4: PBCH payload reduction

- Opt.5: PBCH rate matching around the punctured PRBs

- Opt.6: no enhancement specified

In this contribution, we focus on the issue of designing new SSB structure especially requiring bandwidth less than 20 PRBs.

2. On consideration of legacy UE

The considered band for the dedicated spectrum less than 5MHz is FDD bands less than 1GHz, especially 100, 8, 26, and 28 [1]. Many operators, including us, are using some of these bands for LTE services, and considering NR refarming to use as a camping cell driven by their outstanding capacities for coverage. From this situation, we cannot help considering the cases of legacy UE attempting connection with SSBs in dedicated spectrum less than 5MHz, and vice versa. This indicates our first proposal.

Proposal 1: The situations that legacy UEs attempting to access SSBs for the <5MHz bandwidth and evolved UEs operating at the <5MHz bandwidth attempting to access legacy SSBs is considered for the specification.

Alt. 1-1: The resulting degradations of SSB detection performances and predictable maloperations are studied.

Alt. 2-2: Avoid the situations adopting exclusive design for SSBs for the <5MHz bandwidth.

To avoid the cross-accessing cases, for example, a different sync raster or a new PSS sequence are required which are tough changes and usually not preferred. For this reason, many parts of the next chapter assume that legacy users can access the SSB designed for the dedicated spectrum less than 5MHz.

3. Design of SSB for the dedicated spectrum less than 5MHz

As the strong candidates of the number of PRBs of SSB, 16 and 15 are studied for 3MHz channel bandwidth. In our understanding, the whole candidates of the number of PRBs is from 12 to 16, since if less, PSS/SSS need to be punctured and 17 exceeds 3MHz. Many companies have already analyzed the SNR performance with punctured SSBs, ensuring the result that using more PRBs for SSB leads better performance. Considering UEs in which cannot receive additionally sent PBCH blocks, our proposal for the number of PRBs is based in these results and the formal proposal.

Proposal 2: The number of PRBs for SSBs for the <5MHz bandwidth is 16, or as large as possible within the guardband limitation.

On the other hand, we are considering the possibilities of band operation as overriding, that is, the dedicated 3MHz spectrum is contained in the larger serving bandwidth. In this case, some technical enhancements to arise SNR of legacy UE accessing the SSB will be considered. One simple method is sending PBCH outside the guard band of 3MHz, that is, the puncturing is only performed for PRBs in the guardband. See Fig. 1 and 2.

Figure 1. Example of Puncturing (16PRB case, GB < 120kHz)

Figure 2. Example of Puncturing (15PRB case, GB < 300kHz)

The difference of detection performances between (a) and (b) is not significant [3] for both cases. Moreover, (b) can send more PBCH blocks than (a), as described in (a’) and (b’) after possible extension with larger serving bandwidths for legacy UEs.

Proposal 3: The available topmost or bottommost PRBs including PSS/SSS blocks are used for the SSBs for the <5MHz bandwidth.

To boost the PBCH detection performance, we prefer Option 2 or 3 with additional PBCH transmission. In option 1 the downlink power determined by PA is usually not changed frequently, especially to larger than common operation. Option 4, 5 will make the legacy UE impossible to receive the MIB.

Proposal 4: Opt. 2 is adopted to recover PBCH detection performance.

Alt. 4-1: Opt. 3 with additional PBCH transmission is adopted.

4. Conclusion

We give our proposals in this contribution as follows:

Proposal 1: The situations that legacy UEs attempting to access SSBs for the <5MHz bandwidth and evolved UEs operating at the <5MHz bandwidth attempting to access legacy SSBs is considered for the specification.

Alt. 1-1: The resulting degradations of SSB detection performances and predictable maloperations are studied.

Alt. 2-2: Avoid the situations adopting exclusive design for SSBs for the <5MHz bandwidth.

Proposal 2: The number of PRBs for SSBs for the <5MHz bandwidth is 16, or as large as possible within the guardband limitation.

Proposal 3: The available topmost or bottommost PRBs including PSS/SSS blocks are used for the SSBs for the <5MHz bandwidth.

Proposal 4: Opt. 2 is adopted to recover PBCH detection performance.

Alt. 4-1: Opt. 3 with additional PBCH transmission is adopted.

5. References

[1] RAN1 111 Chariman’s Note, Final.

[2] RP-222645, Revised WID: NR support for dedicated spectrum less than 5MHz for FR1.

[3] R1-2212247, On dedicated spectrum less than 5 MHz, MediaTek Inc.