3GPP TSG RAN WG1 #112				R1-2301476

Athens, Greece, February 27th – March 3rd, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on remaining issues regarding Rel-17 RAN1 UE features topics 1

Agenda Item:	8.16.1

Document for: 	Discussion and Decision

- Introduction

The updated RAN1 UE features list for Rel-17 NR after RAN1#111 has been endorsed in [1]. In this contribution, we present our views regarding remaining issues on Rel-17 RAN1 UE features topics 1, i.e., for NR_MBS.

- Discussion on NR_MBS

  - FGs 33-3-3a/3b

| 33. NR_MBS | 33-3-3a | FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | 1. Support of FDM-ed Type-1 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH 2. Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast on PUCCH or PUSCH with max number X of G-RNTIs | 33-3-2, at least one of {33-2a, 33-4, 33-5-1a, 33-5-1f} | Yes |  | FFS | Per BC | N/A | N/A |  | Note1: FDM-ed Type-1 HARQ-ACK codebook is generated by concatenating the Type-1 sub-codebook for unicast and the Type-1 sub-codebook for multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {1, 2, 3, 4} with X no larger than max number of G-RNTIs of FG33-2e Note: the value of X should be common across FG33-2a, 33-3-3a and 33-3-3b if reported | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-3b | Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | 1. Support of Mode 2 TDM-ed Type-1 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH 2. Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast on PUCCH or PUSCH with max number X of G-RNTIs | 33-2a or 33-4 or 33-5-1a or 33-5-1f | Yes |  | FFS | Per BC | N/A | N/A |  | Note1: Mode 2 TDM-ed Type-1 HARQ-ACK codebook is generated based on the union TDRA tables from unicast and multicast and the union of k1 sets from unicast and multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {1, 2, 3, 4} with X no larger than max number of G-RNTIs of FG33-2e Note: the value of X should be common across FG33-2a, 33-3-3a and 33-3-3b if reported | Optional with capability signalling |

The remaining issue for FGs 33-3-3a/3b is what is mentioned in the column of ‘Consequence if the feature is not supported by the UE’. For this issue, necessity to add some text there seems to be unclear in the first place. There are a lot of FGs for MBS without any text in the column. Just removing ‘FFS’s would be sufficient.

Proposal 1: Remove ‘FFS’ from FGs 33-3-3a/3b.

  - FGs 33-5-1/1a/1f/1g

| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast on PCell | 1. Support one SPS group-common PDSCH configuration for multicast 2. Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH | 33-2 | Yes |  |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | 1. Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, [SPS group-common PDSCH activation, and SPS release PDCCH] 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AN-List configuration from unicast SPS | 33-5-1 | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1b | DCI-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling via DCI format 4_2 | 33-5-1a, 33-5-1i | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1g | DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling via DCI format 4_2 | 33-5-1f, 33-5-1i | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |

For FGs 33-5-1/1a, it seems that the discussion point is not UE capability perspective but exact specification-based UE behavior. For HARQ feedback enabling/disabling and SPS activation/release, the following description can be found in 38.213.

A UE can be configured per G-RNTI for multicast or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI for multicast or G-CS-RNTI and pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI for multicast or a G-CS-RNTI, the UE provides HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_1 associated with the G-RNTI or the G-CS-RNTI, and determines whether or not to provide the HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_2 based on an indication by the multicast DCI format 4_2 associated with the G-RNTI for multicast or the G-CS-RNTI [4, TS 38.212]. If a UE is provided pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information, the UE does not expect to be provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI.

Based on the above text with red color, especially ‘for respective PDSCH receptions’, the correct UE behavior can be interpreted as whether HARQ feedback for activation (i.e., for initial SPS PDSCH after activation) is determined by the enabling/disabling signaling and HARQ feedback for release PDCCH is always performed regardless of the enabling/disabling signaling. If this interpretation is common among companies, activation part should be maintained in FG 33-5-1a while release part should be moved to FG 33-5-1.

Proposal 2: Update FGs 33-5-1 and 33-5-1a as follows.

| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast on PCell | 1. Support one SPS group-common PDSCH configuration for multicast 2. Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH 3. ACK/NACK-based HARQ-ACK feedback for SPS release associated with G-CS-RNTI | 33-2 | Yes |  |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | 1. Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, [and SPS group-common PDSCH activation (i.e., initial SPS PDSCH after activation), and SPS release PDCCH] 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AN-List configuration from unicast SPS | 33-5-1 | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |

For FG 33-5-1f, we do not see any reason to adopt different reporting types between ACK/NACK based feedback (FG 33-5-1a) and NACK-only based feedback (FG 33-5-1f). ‘per BC’ should be supported as in FG 33-5-1a.

For FG 33-5-1g, from similar reason to the above, ‘per band’ should be supported as in FG 33-5-1b.

Proposal 3: The reporting type of FG 33-5-1f is per BC.

Proposal 4: The reporting type of FG 33-5-1g is per band.

  - FG 33-5-1e

| 33. NR_MBS | 33-3-1 | Dynamic Slot-level repetition for group-common PDSCH | 1. Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast. | 33-2 | Yes |  |  | Per UE | Yes | Yes |  | Candidate values for X is: {8, 16} This FG is reported for TN and licensed. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-1a | Dynamic Slot-level repetition for group-common PDSCH for NTN and unlicensed | 1. Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast for NTN and unlicensed | 33-2 | Yes |  |  | Per band | N/A | N/A |  | Candidate values for X is: {8, 16} This FG is reported for NTN and unlicensed | Optional with capability signalling |
| 33. NR_MBS | 33-5-1e | Dynamic Slot-level repetition for SPS group-common PDSCH for multicast | Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast. | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate values for X is: {8, 16} | Optional with capability signalling |

For dynamic slot-level repetition, we do not see any reason to adopt different reporting types between dynamic scheduling (FG 33-3-1) and SPS (FG 33-5-1e). For FG 33-5-1e, ‘per UE’ should be supported with FDD/TDD and FR1/FR2 differentiations as in FG 33-3-1. In addition, as we did for FGs 33-3-1/33-3-1a, TN/NTN differentiation and licensed/unlicensed differentiation can be done by adding separate FG for NTN/unlicensed with per-band.

Proposal 5: The reporting type of FG 33-5-1e is per UE with FDD/TDD and FR1/FR2 differentiations, and the note “This FG is reported for TN and licensed” is added.

Proposal 6: Following FG is added.

| 33. NR_MBS | 33-5-1k | Dynamic Slot-level repetition for SPS group-common PDSCH for multicast for NTN and unlicensed | Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast for NTN and unlicensed. | 33-5-1 | Yes |  |  | Per band | N/A | N/A |  | Candidate values for X is: {8, 16} This FG is reported for NTN and unlicensed | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - FG 33-5-1i

| 33. NR_MBS | 33-2f | Dynamic multicast with DCI format 4_2 | Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast | 33-2 | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1i | Multicast SPS scheduling with DCI format 4_2 | 1.Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling 2. Retransmission scheduled by DCI format 4_2 with CRC scrambled with G-CS-RNTI | 33-5-1 | Yes |  |  | FFS | FFS | FFS |  |  | Optional with capability signalling |

According to the same logic as above, ‘per band’ should be supported for FG 33-5-1i as in FG 33-2f.

Proposal 7: The reporting type of FG 33-5-1i is per band.

  - FG 33-5-2

| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration on PCell | 1. Support up to 8 SPS group-common PDSCH configuration per CFR for multicast 2. Support M>=1 activated SPS group-common PDSCH configurations per CFR for multicast 3. The total number of SPS configurations for both multicast and unicast is no larger than 8 in a BWP of a serving cell, and activated SPS group-common PDSCH configurations is no larger than M. 4. The total number of SPS configurations for both multicast and unicast in a cell group is no larger than 32 | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate value set for M is {1, 2, …, 8} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The reporting type of FG for support of up to 8 SPS configurations for unicast (FG 12-2) is per band; correspondingly the reporting type of FG 33-5-2 should also be per band.

Proposal 8: The reporting type of FG 33-5-2 is per band.

  - FG 33-8-1

| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming | 1. Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits 2. Support of shared PUCCH resource configurations with unicast 3. One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits 4. One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI | 33-2a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4a | NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission | 1. Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: a) Up to 4 TBs with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate PUCCH resource configurations from unicast 3. Single TB with NACK-only feedback transmitted in PUCCH 4. up to 4TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits | 33-4 | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-8-1 | PUCCH resource configuration for multicast feedback for dynamically scheduled multicast | Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations | 33-2a | Yes |  |  | [Per band or per FSPC] | [No] | [No] |  |  | Optional with capability signalling |

We do not see any necessity to make the reporting type of FG 33-8-1 finer granularity than per BC. Besides, at least typically separate PUCCH-config would be configured for NACK-only feedback since PUCCH resource for NACK-only feedback would be shared among UEs. FGs 33-4/4a are per BC; thus, the same granularity should be reasonable.

Proposal 9: The reporting type of FG 33-8-1 is per BC.

  - FG 33-9

| 33. NR_MBS | 33-9 | Supporting unicast PDCCH to release SPS group-common PDSCH | Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH | 33-5-1, 12-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The process of checking if it is a DCI for SPS release would be band-independent. Reporting type per UE without FDD/TDD and FR1/FR2 differentiations would be sufficient.

Proposal 10:  The reporting type of FG 33-9 is per UE without FDD/TDD and FR1/FR2 differentiations.

  - FGs 33-5-1h/33-10

| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | Max number of G-CS-RNTIs for SPS multicast | 33-5-1 | Yes |  |  | Per UE | [Yes] | Yes |  | Reporting type of FGs 33-5-1h is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-10 | Support group-common PDSCH RE-level rate matching for multicast | 1) Support of SP ZP-CSI-RS for group-common PDSCH RE-mapping patterns 2) Support of P ZP-CSI-RS for group-common PDSCH RE-mapping patterns 3) Support p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast same as or different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config Note 1: The total number of semi-persistent ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16 4) Support of AP ZP-CSI-RS for group-common PDSCH RE-mapping patterns | 2-33a, 33-2 | Yes |  |  | Per UE | [Yes] | Yes |  | Reporting type of FG 33-10 is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 | Optional with capability signalling |

It was agreed that details on reporting type for FGs 33-5-1h and 33-10 are up to RAN2 discussion, and in the latest 38.306, we can see the following corresponding capabilities. Just to clean up the UE feature list to avoid any potential confusion, the following proposals are submitted. Note that although it is basically not necessary to update RAN1 UE features list according to RAN2 specifications, the update here would be better in order to avoid unnecessary confusion due to remaining brackets and texts in the note.

| maxNumberG-CS-RNTI-r17 Defines maximum number of G-CS-RNTIs for SPS multicast. For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. A UE supporting this feature shall also indicate support of sps-Multicast-r17. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| re-LevelRateMatchingForMulticast-r17 Indicates whether the UE supports group-common PDSCH RE-level rate matching for multicast, comprised of the following functional components: - Supports SP ZP-CSI-RS for group-common PDSCH RE-mapping patterns; - Supports P ZP-CSI-RS for group-common PDSCH RE-mapping patterns; - Supports p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast same as or different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config; - Supports AP ZP-CSI-RS for group-common PDSCH RE-mapping patterns. For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. A UE supporting this feature shall also indicate support of dynamicMulticastPCell-r17. A UE supporting this feature in FR1 bands shall also indicate support of pdsch-RE-MappingFR1-PerSymbol or pdsch-RE-MappingFR1-PerSlot. A UE supporting this feature in FR2 bands shall also indicate support of pdsch-RE-MappingFR2-PerSymbol or pdsch-RE-MappingFR2-PerSlot. NOTE: The total number of semi-persistent ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16. | Band | No | N/A | N/A |

Proposal 11:  FGs 33-5-1h and 33-10 are updated as described in the latest 38.306.

- The reporting type is per band.

- The text in Note column is updated as the following.

  - For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands.

- Conclusion

In this contribution, we presented our views regarding remaining issues on Rel-17 RAN1 UE features topics 1. Based on the discussion in this contribution, following proposals were made.

Proposal 1: Remove ‘FFS’ from FGs 33-3-3a/3b.

Proposal 2: Update FGs 33-5-1 and 33-5-1a as follows.

| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast on PCell | 1. Support one SPS group-common PDSCH configuration for multicast 2. Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH 3. ACK/NACK-based HARQ-ACK feedback for SPS release associated with G-CS-RNTI | 33-2 | Yes |  |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | 1. Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, [and SPS group-common PDSCH activation (i.e., initial SPS PDSCH after activation), and SPS release PDCCH] 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AN-List configuration from unicast SPS | 33-5-1 | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |

Proposal 3: The reporting type of FG 33-5-1f is per BC.

Proposal 4: The reporting type of FG 33-5-1g is per band.

Proposal 5: The reporting type of FG 33-5-1e is per UE with FDD/TDD and FR1/FR2 differentiations, and the note “This FG is reported for TN and licensed” is added.

Proposal 6: Following FG is added.

| 33. NR_MBS | 33-5-1k | Dynamic Slot-level repetition for SPS group-common PDSCH for multicast for NTN and unlicensed | Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast for NTN and unlicensed. | 33-5-1 | Yes |  |  | Per band | N/A | N/A |  | Candidate values for X is: {8, 16} This FG is reported for NTN and unlicensed | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 7: The reporting type of FG 33-5-1i is per band.

Proposal 8: The reporting type of FG 33-5-2 is per band.

Proposal 9: The reporting type of FG 33-8-1 is per BC.

Proposal 10:  The reporting type of FG 33-9 is per UE without FDD/TDD and FR1/FR2 differentiations.

Proposal 11:  FGs 33-5-1h and 33-10 are updated as described in the latest 38.306.

- The reporting type is per band.

- The text in Note column is updated as the following.

  - For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands.

References

- Moderators (AT&T, NTT DOCOMO, INC.), R1-2212895, Updated RAN1 UE features list for Rel-17 NR after RAN1 #111