3GPP TSG-RAN WG1 Meeting #112	R1- 2301557

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.9.2

Source:	Ericsson

Title:	Multi-carrier UL Tx switching

Document for:	Discussion, Decision

# Introduction

In the RAN#98-e plenary, the status report of the Multi-carrier enhancements WI reported 70% completion level and additional TU for RAN1#112 was agreed to complete the WI. Although the remaining issues for UL TX switching are minor issues, it was understood that the allocated additional time unit at this meeting can be used for addressing these issues. Based on companies’ views, the Moderator summary in RAN#98-e plenary lists the following topics for discussions [1]:

- For Multi-carrier UL Tx switching scheme

  - Details on solution for potential ambiguity issue on switching period location

  - Details on potential issues related to timeline for UL Tx switching

  - Working assumption on minimum separation time between two UL Tx switching

In this contribution, we share our view on the above topics.

# Discussion

## Switching period location

In UL Tx switching, the uplinkTxSwitchingPeriod indicates the minimum switching gap that is indicated by the UE capability when a Tx chain switching occurs between carriers/bands to perform a UL transmission starting at time T0  as denoted in clause 6.1.6 of [3].

During the switching gap, the UE may omit transmissions on the UL carrier(s)/band(s), if any. Alignment between the UE and gNB on the band where the switching occurs prior to transmission at time T0, would assist the gNB to schedule accordingly to ensure empty/blank symbols on that carrier/band with consideration to system performance. This goal is achieved by the RRC parameter uplinkTxSwitchingPeriodLocation.  See for example Figure 1 showing the related description in [4].

Figure 1: Excerpt from [4] to illustrate how the switching period location is applied in case of two carriers.

RAN4 has made the following agreement [5] with respect to switching period location to reuse existing signalling for each switching band pair, as described above.

Issue 4: Location of switching period - For single-TAG case, RAN4 agreed to reuse the Rel-16/17 approach (i.e., semi-static configuration of switching period on one of the band for each switching band pair) and discuss further details for Rel-18 Tx switching scenario in RAN1. - Meanwhile, RAN4 has not concluded on the switching period location for 2-TAG case, with further discussions ongoing.

It was discussed during the last RAN1 meeting that in case of switching across 3 or 4 bands, there may ambiguity in determining switching period location in some scenarios if only configuration per band pair is applied. The reason is that multiple choices to determine the switching period location would be possible that causes mis-alignment between UE and gNB.

For example, consider the following scenario for 3-bands (A, B, C).  For each of the band pairs, a band is configured for the location of the switching gap.

  - Band A for (A, B) pair

  - Band A for (A, C) pair

  - Band C for (B, C) pair

Consider a UL transmission is scheduled to start at time T0 and the UE has to switch the Tx chain(s) are switched “from” band A and B “to” band C, or vice-versa to perform the transmission. The UE may omit the transmission during the switching gap as described earlier, however it is not clear from the pair configurations above whether band A, or band C should be used to locate the switching gap.

This problem was discussed during the last meeting. The discussion was concluded by listing the following solutions to resolve the ambiguity [2]:

Updated proposed agreement 6 Down-select a solution for the ambiguity issue on switching period location from following alternatives. - Alt.1: switching period location is configured per band pair - If there are multiple bands configured with switching period location as TRUE in the bands involved in a switching, the switching period location is determined to highest carrier frequency among the bands configured with switching period location as TRUE - Alt.2: switching period location is configured per band pair, and the priority list of bands is also configured - If there are multiple bands configured with switching period location as TRUE in the bands involved in a switching, the switching period location is determined to the band with lowest priority among bands configured with switching period location as TRUE - Alt.3: gNB configures “switching-from band” or “switching-to band” - If gNB configures “switching-from band” as switching period location, switching period is located on band(s) where preceding transmission is performed - Alt.4: gNB configures switching period location per switching case - In 3 bands case, gNB configures switching period location for each of switching case pair such as {A - B}, {A - C}, {B - C}, {A+B - C}, {A+C - B}, {B+C - A} - In 4 bands case, gNB configures switching period location for each of switching case pair such as {A - B}, {A - C}, {A - D}, {B - C}, {B - D}, {C - D}, {A+B - C}, {A+B - D}, {A+C - B}, {A+C - D}, {A+D - B}, {A+D - C}, {B+C - A}, {B+C - D}, {B+D - A}, {B+D - C}, {C+D - A}, {C+D - B}, {A+B – C+D}, {A+C – B+D}, {A+D – B+C} - Alt.4-rev: gNB configures switching period location per configured 3 or 4 bands - In 3 bands case, gNB configures switching period location for each of switching case pair such as {A, B}, {A, C}, {B, C}, {A, B, C} - In 4 bands case, gNB configures switching period location for each of switching case pair such as {A, B}, {A, C}, {A, D}, {B, C}, {B, D}, {C, D}, {A, B, C}, {A, B, D}, {B, C, D}, {A, B, C, D} - Alt.5: gNB configures priorities to each carrier/band. - The UE determines the switching period location on the band that is not with the highest priority.

In our view, a solution that reuses the existing procedures is preferrable. Moreover, configuration based solution as compared to rule-based solution provides scheduling flexibility to the NW and results in improved system operation. Therefore, with respect to the alternatives above, we assess the above solutions as the following:

- Alt. 1, Alt. 2 and Alt. 5 suggest “rule-based” solutions.

- Alt. 3 suggests “rule-based” solution to some extent where the rule is provided by configuration that would imply to follow a certain behaviour. Moreover, it suggests a different mechanism as compared to legacy where in legacy for a band-pair, the location period is the same carrier/band whether the switch is “from” or “to” a carrier/band. This difference may cause additional complexity both at gNB and UE. Moreover, this solution relies on the preceding transmissions and hence, is subject to error in case of misdetection of the previous transmissions.

- Alt. 4 and Alt.4-rev are configuration based solutions and follow the legacy procedures. Alt.4-rev achieves the same goal as Alt.4 with reduced signalling and complexity. The main goal of providing the switching gap location is proper planning of empty/bank symbols to accommodate Tx switching at the UE on the indicated bands. The same band can be used for this purpose for any of the switching cases {A+B - C}, {A+C - B} and {B+C - A}. Therefore Alt.4-rev is a preferred solution.

Based on the above discussion, we propose the following:

The switching period location per configured 3 or 4 bands is determined by configuration, as the following (Alt. 4-rev):

    - In 3 bands case, the switching period location is configured for each of switching case pair such as {A, B}, {A, C}, {B, C}, {A, B, C}

    - In 4 bands case, gNB configures switching period location for each of switching case pair such as {A, B}, {A, C}, {A, D}, {B, C}, {B, D}, {C, D}, {A, B, C}, {A, B, D}, {B, C, D}, {A, B, C, D}

## Switching separation time

The following agreement was made during the last meeting regarding the switching separation time.

Agreement Following restrictions are applied for Rel-18 UL Tx switching across 3 or 4 bands. - The UE does not expect to perform more than one uplink switching within a reference slot based on µUL = max(µUL, 1, µUL, 2, µUL, 3) in case of 3 bands, µUL = max(µUL, 1, µUL, 2, µUL, 3, µUL, 4) in case of 4 bands, where µUL, 1, µUL, 2, µUL, 3, µUL, 4 are SCSs of active UL bandwidth parts of the bands in the band combination - If there are two consecutive intra-band carriers in one band, µUL, 1 = max(µUL, 1-1, µUL, 1-2), where µUL, 1-1 and µUL, 1-2 are SCSs of active UL bandwidth parts of the carriers in the band - (working assumption) If two uplink switching are triggered and result in UL transmissions on more than 2 bands within any two consecutive reference slots, then the time duration between the end of all transmission(s) prior to the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time - The minimum separation time is a sum of X us and the switching gap required for the second uplink switching. - X us is subject to UE capability with a value set of {0us, 500us}

The description of the minimum separation time in case two UL switches across two consecutive reference slots was endorsed as working assumption. Concerns were raised due to the ambiguity that the description could cause.  In our view the working assumption should be revised to accurately reflect the intention. We suggest incorporating the following clarifications in the working assumption for this purpose:

- Order the two UL transmissions that have triggered UL Tx switching by using the corresponding starting times.

- Clarify that all transmissions before the first switch and after the second switch are UL transmissions.

- Clarify the bands corresponding to all transmissions before the first switch and after the second switch.

Therefore, we propose to revise the working assumption as the following, and then confirm:

Confirm the Working assumption on the switching separation time with incorporation the following changes:

    - (working assumption) If two uplink switching are triggered and result in two UL transmissions on more than 2 3 or 4 bands with corresponding starting times T0,1 and T0,2 with T0,1≤T0,2 within any two consecutive reference slots, then the time duration between the end of all UL transmission(s) prior to T0,1 the first uplink switching and the start of all UL transmission(s) after T0,2 the second uplink switching within the two reference slots on the 3 or 4 bands is expected to be not less than a minimum separation time

## Timeline for UL Tx switching

It was discussed in previous meeting whether the current timeline requirements with respect to UL Tx switching is sufficient for UL Tx switching across 3 or 4 bands.

RAN4 has agreed to adopt the following conservative approach [6]  when the indicated switching period capability differs among the band-pairs. Considering the RAN4 agreement on the baseline UE assumption, the current timeline accommodates the needed time to perform switching for any scenario.

Issue 2: Ambiguity issue when two Tx chains are switched between two different band pairs For Rel-18 UL Tx switching among 4 bands, when switching from 1T+1T on band A and B to 1T+1T on band C and D is performed, and it is not clear whether UE performs Tx switching {from band A to C + B to D} or {from band A to D + B to C}, RAN4 agreed that: - As baseline UE assumption, no need to resolve the ambiguity issue of the switching pattern for each Tx chain and determine the switching gap based on the worst case by default, i.e., neither of the two Tx chains is expected to be used for transmission during the maximum of the four switching periods, i.e., max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}. Note: Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C are the switching periods reported by the UE for band pair A&C, B&D,A&D and B&C, respectively.

Therefore, we propose the following:

Further restriction on timeline for UL Tx switching is not supported.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The switching period location per configured 3 or 4 bands is determined by configuration, as the following (Alt. 4-rev):

	In 3 bands case, the switching period location is configured for each of switching case pair such as {A, B}, {A, C}, {B, C}, {A, B, C}

	In 4 bands case, gNB configures switching period location for each of switching case pair such as {A, B}, {A, C}, {A, D}, {B, C}, {B, D}, {C, D}, {A, B, C}, {A, B, D}, {B, C, D}, {A, B, C, D}

Proposal 2	Confirm the Working assumption on the switching separation time with incorporation the following changes:

	(working assumption) If two uplink switching are triggered and result in two UL transmissions on more than 2 3 or 4 bands with corresponding starting times T0,1 and T0,2 with T0,1≤T0,2 within any two consecutive reference slots, then the time duration between the end of all UL transmission(s) prior to T0,1 the first uplink switching and the start of all UL transmission(s) after T0,2 the second uplink switching within the two reference slots on the 3 or 4 bands is expected to be not less than a minimum separation time

Proposal 3	Further restriction on timeline for UL Tx switching is not supported.

# References

RP-223557		Moderator’s summary for discussion [98e-15-R18-Multicarrier]; Moderator (NTT DOCOMO, INC.)

R1-2212894	Summary#4 of discussion on multi-carrier UL Tx switching scheme; Moderator (NTT DOCOMO, INC.)

3GPP TS 38.214		 NR; Physical layer procedures for data

3GPP TS 38.101-1	NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

R1-2210819	LS on Rel-18 UL Tx switching; RAN4

R1-2300029	LS on Rel-18 UL Tx switching; RAN4

# Appendix

## WGs outcomes

### RAN1#109-e: Agreements and conclusions

Conclusion

EN-DC cases are out of scope for Rel-18 UL Tx switching

Conclusion

UL only cell cases are out of scope for Rel-18 UL Tx switching

RAN1 Observation

Four contributions (R1-2203136, R1-2204724, R1-2204909, R1-2205131) from three companies show their evaluation results on UL Tx switching across 3 or 4 bands at RAN1#109-e meeting.

- All evaluation results show the performance gain of UL Tx switching across 4 bands compared with UL Tx switching across 2 bands, assuming TDD bands with different TDD UL/DL configurations are included in 4 bands.

  - Evaluation results in R1-2203136 show the performance gain of UL Tx switching across 4 bands compared with UL Tx switching across 3 bands.

  - Evaluation results in R1-2204724 show that the performance gain of UL Tx switching across 4 bands compared with UL Tx switching across 2 bands depends on achievable switching period, and the longer switching period for UL Tx switching across 4 bands compared with UL Tx switching across 2 bands leads to reduction of the performance gain. Other evaluation results did not consider the impact of longer switching period for UL Tx switching across 4 bands compared with UL Tx switching across 2 bands.

  - Evaluation results in 5131 observe that the gain highly depends on the scheduling mechanism.

  - The range of performance gains shown in four contributions varies depending on the simulation assumptions.

Agreement

Companies are encouraged to investigate pros and cons of following possible mechanisms for dynamic Tx carrier switching across the configured bands, and RAN1 strives for the down-selection at RAN1#110

- Alt.1: Dynamic Tx carrier switching can be across all the supported switching cases by the UE and based on the UL scheduling, i.e., via UL grant and/or RRC configuration for UL transmission

- Alt.2: NW indicates 2 bands out of the configured bands (3 or 4 bands) via DCI or MAC-CE, and dynamic Tx carrier switching between indicated bands is same as Rel-17

- Alt.3: One anchor band is selected among configured bands (3 or 4 bands), and dynamic Tx carrier switching can be performed only from the anchor band to a non-anchor band and from a non-anchor band to the anchor band

- Note: Other mechanisms are not precluded

Agreement

Send LS to RAN4 to ask their feedback on the potential increase of switching period and complexity in the case of UL Tx switching across 3 or 4 bands

- In the LS, observations based on the evaluation results and alternative switching mechanisms discussed in RAN1 are captured for the information to RAN4

- In the LS, RAN1 also asks RAN4 feedback on whether following assumption can be considered as baseline UE assumption/behavior even in case of the UL Tx switching across 3 or 4 bands

  - When one of the two Tx chains is triggered to switch from one band to another band, another Tx chain which is in any of bands is also not expected to be used for transmission during the switching period

LS is endorsed in R1-2205502.

Conclusion

If Rel-18 UL Tx switching is supported, following assumption is applied for Rel-18 UL Tx switching across up to 3 or 4 bands

- Only when the two Tx chains are linked to one NR band, the 2-ports UL transmission on the NR band is possible

RAN1 Observation

Following proposals to address the concern on UE/gNB complexity increase or scheduling restriction due to UL Tx switching across larger number of bands compared with Rel-16/17 are identified in contributions submitted at RAN1#109-e, and companies are encouraged to investigate pros and cons of the proposals so that one or some of them may be down-selected after the down-selection of the mechanism for dynamic Tx carrier switching across the configured bands

- UE can report the supports of only some of concurrent UL cases (combinations of 2 bands for concurrent UL transmissions)

- Switching across 0/1/2 ports is supported only for 2 configured bands out of 3 or 4 configured bands and other bands support switching across 0/1 port only

- Only switching across 0/1 port is supported across all configured bands when 3 or 4 bands are configured

- Prioritization rules between uplink carriers are specified

- No restriction on the UEs choice of MIMO capability on any of the bands/CCs involved in the UL Tx switching band combination is introduced

- After one RF state switch, the next RF state switch must occur after 14 symbols or later (FFS: which SCS is assumed for the symbol duration)

- Note: Other solutions are not precluded

- Note: each proposal assumes certain mechanism for dynamic Tx carrier switching across the configured bands, and hence some or all of the proposals may not be necessary depending on the down selection of the mechanism for dynamic Tx carrier switching across the configured bands

Conclusion

It is RAN1’s understanding that RAN4 should lead the discussion on UL Tx switching with multiple TAGs for both 2 bands case and more than 2 bands case

- For further discussion in RAN1 with regards to UL Tx switching with multiple TAGs, it will be discussed only if triggered by RAN4

- If it is decided to support UL Tx switching with multiple TAGs, it is RAN1's working assumption that the number of TAGs should be limited to up to 2

RAN1 Observation

Following possible switching configurations can be considered, and RAN1 may discuss if any of the following switching configurations need to be supported after making some progress on the discussion on the switching mechanism

- For 3 bands case

  - Switching configuration.3-1: all the 3 bands support up to 2Tx

  - Switching configuration.3-2: only 1 band out of 3 bands support up to 2Tx

  - Switching configuration.3-3: only 2 bands out of 3 bands support up to 2Tx

- For 4 bands case

  - Switching configuration.4-1: all the 4 bands support up to 2Tx

  - Switching configuration.4-2: only 1 band out of 4 bands support up to 2Tx

  - Switching configuration.4-3: only 2 bands out of 4 bands support up to 2Tx

  - Switching configuration.4-4: only 3 bands out of 4 bands support up to 2Tx

- Note: The Spec should not restrict which Tx chain is fixed or switched across certain bands.

### RAN1#110: Agreements and conclusions

Working Assumption

- If Rel-18 UL Tx switching is supported, following switching mechanism is considered as baseline for the Rel-18 UL Tx switching across 3 or 4 bands

  - Alt.1: Dynamic Tx carrier switching can be across all the supported switching cases by the UE and based on the UL scheduling, i.e., via dynamic grant and/or RRC configuration for UL transmission

- RAN1 will support one or more of following complexity reduction options, considering at least the potential additional preparation time, additional interruption time, and RF complexity for certain switching cases/patterns, if Rel-18 UL Tx switching is supported based on Alt.1, and companies are encouraged to investigate options with striving for down-selection at RAN1#110bis-e.

  - Option 1: UE is allowed to support only some of concurrent UL cases (band pairs)

    - FFS: at least one band pair should be supported as in Rel-17

    - FFS: for both 3 and 4 bands cases or only for 4 bands case

    - FFS: potential capability/RRC signaling

  - Option 2: UE is allowed to support 2 ports transmission only on some of bands out of configured bands for UL Tx switching

    - FFS: at least two bands should support up to 2 Tx as in Rel-17

    - FFS: for both 3 and 4 bands cases or only for 4 bands case

    - FFS: for both switched UL and dual UL cases or only for dual UL case

    - FFS: whether/how to reuse or extend existing capability/RRC signaling

  - Option 3: UE is allowed with more preparation procedure time (or interruption time) only for some specific switching cases/patterns

    - FFS: specific switching cases/patterns where more preparation procedure time (or interruption time) is necessary, e.g., switching patterns not existed in Rel-17

    - FFS: how long preparation procedure time and/or interruption time is necessary, and whether RAN4 involvement is necessary

    - FFS: whether/how to report/indicate the specific switching cases/patterns and/or value(s) of preparation procedure time (or interruption time)

    - FFS: what is the definition of preparation procedure time or interruption time, including whether interruption happens during the preparation procedure time and whether it includes switching period

    - FFS: whether/how long minimum interval between two succeeding UL Tx switching is necessary

  - Option 4: UE is allowed to support only some of band pairs for tx switching

    - FFS: at least one band pair should be supported as in Rel-17

    - FFS: for both 3 and 4 bands cases or only for 4 bands case

    - FFS: for switched UL and/or dual UL

    - FFS: potential capability/RRC signaling

  - Other options are not precluded

### RAN1#110bis-e: Agreements and conclusions

Agreement

If Rel-18 UL Tx switching for 3 or 4 bands with dual UL is supported, UE is allowed to support only some of band pairs for concurrent UL transmission based on UE capability

- The supported band pair for concurrent transmission requires the support of UL CA on the corresponding band pair(s) by the UE

- Details on the UE capability such as how to report the support of dual UL and the supported band pair(s) for concurrent UL transmission are further discussed

- Details on the gNB configuration/indication such as how to indicate the band pair(s) UE should expect for concurrent UL transmission are further discussed

- Note: UE is also allowed to support all band pairs for concurrent transmission, and the design of Rel-18 UL Tx switching for 3 or 4 bands with dual UL does not impose any restriction

Agreement

If Rel-18 UL Tx switching for 3 or 4 bands is supported, UE is allowed to support only some of band(s) for up to 2 ports UL transmission based on UE capability

- Further down-select from the following alternatives

  - Alt.1: no restriction for both switched UL and dual UL and for both 3 bands and 4 bands

  - Alt.2: at least one band should support up to 2 ports UL transmission for both switched UL and dual UL and for both 3 bands and 4 bands

  - Alt.3: at least two bands should support up to 2 ports UL transmission for both switched UL and dual UL and for both 3 bands and 4 bands

- Details on the UE capability such as whether existing per-FS UL-MIMO capability can be reused or not are further discussed

- Details on the gNB configuration/indication such as whether/how to additionally indicate 2 ports UL transmission mode for a band/cell are further discussed

- Existing MIMO mechanism for MIMO mode indication should be reused

- Note: UE is also allowed to support all bands for up to 2 ports UL transmission, and the design of Rel-18 UL Tx switching for 3 or 4 bands does not impose any restriction

Agreement

If Rel-18 UL Tx switching for 3 or 4 bands is supported, following is considered as baseline.

- Existing conditions where the switching period is required can be reused for Rel-18 UL Tx switching with 3 or 4 bands when only two bands are involved in a switching

- New conditions where the switching period is required should be introduced for Rel-18 UL Tx switching with 3 or 4 bands when more than two bands are involved in a switching

  - For dual UL, following new conditions are considered

    - When the UE is to transmit a 1-port or 2-port transmission on one uplink carrier on one band (1st band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (2nd and 3rd band)

    - When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 2T on a carrier on another band (3rd band)

    - When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on one of the bands and another different band (1st or 2nd band, and 3rd band)

    - When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (3rd and 4th band)

  - FFS for switched UL and/or for the case with complexity reduction option 1 or 2

  - FFS the same or different switch period for existing conditions and new conditions

Conclusion

No consensus in RAN1 on complexity reduction option 3

Agreement

- Consider following alternatives for UE capability reporting about the supported UL Tx switching options

  - Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination

  - Alt.2: report {switchedUL, dualUL, both} for the band combination and report supported band pair for concurrent transmission for the band combination

- Consider following alternatives for gNB configuration regarding dual UL

  - Alt.1: configure {switchedUL, dualUL} in CellGroupConfig

  - Alt.2: configure {switchedUL, dualUL} for each band pair (combination of serving cells?)

  - Alt.3: at least configuration of supported band pair (combination of serving cells) for concurrent transmission

  - Alt.4: No configuration of supported band pair (combination of serving cells) for concurrent transmission, i.e., UE just assumes as it reports

Working Assumption

Specify UL Tx switching schemes across up to 4 bands in Rel-18

Working Assumption

If Rel-18 UL Tx switching for 3 or 4 bands is supported, both Switched UL and Dual UL are supported

Agreement

Confirm the following working assumption made at the RAN1#110 meeting.

Working Assumption

If Rel-18 UL Tx switching is supported, following switching mechanism is considered as baseline for the Rel-18 UL Tx switching across 3 or 4 bands

- Alt.1: Dynamic Tx carrier switching can be across all the supported switching cases by the UE and based on the UL scheduling, i.e., via dynamic grant and/or RRC configuration for UL transmission

Working Assumption

At least for dual UL, reuse existing RRC parameter {oneT, twoT} via uplinkTxSwitching-DualUL-TxState to solve the issue on ambiguous switching state at least for following cases

- Case#1 of the issue: two Tx chains are currently associated with band A, and next transmission is 1 port transmission on band B, but there are multiple possible switching cases where 1P on band B is supported

  - if twoT is indicated, both of two Tx chains are switched to band B

  - if oneT is indicated, one Tx chain is switched to band B while another Tx chain remains on band A

- Case#2 of the issue: two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, but there are multiple possible switching cases where 1P on band C is supported

  - if twoT is indicated, both of two Tx chains are switched to band C

  - if oneT is indicated, one Tx chain is switched to band C while how to determine the associated band for another Tx chain is FFS

    - Alt.1: based on gNB’s configuration/indication e.g., new RRC parameter

    - Alt.2: based on predefined rule

    - Other alternative is not precluded

- FFS for other potential cases

Agreement

Ask RAN2 to consider following alternatives for UE capability reporting about the supported UL Tx switching options

  - Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination

  - Alt.2: report {switchedUL, dualUL, both} for the band combination and report supported band pair for concurrent transmission for the band combination

    - Note：If there is no report on the supported band pair(s) for concurrent transmission while the UE reports “dualUL” or “both” for the band combination, gNB may assume that the UE supports concurrent transmission on all the band pairs within the band combination

  - Alt.3: report {dualUL} for each band pair in the band combination

    - Note: Within the band combination, the UE shall be capable of being operated in switched UL mode for all band pairs

Agreement

Ask RAN2 to consider following alternatives and specify gNB configuration

  - Alt.1: configure {switchedUL, dualUL} for all serving cells (i.e., for the band combination)

  - Alt.2: configure {switchedUL, dualUL} for combination(s) of serving cells (i.e., for each band pair in the band combination)

  - Alt.3: configure {switchedUL, dualUL} for all serving cells (i.e., for the band combination), and configure combination(s) of serving cells (i.e., as supported serving cell pair(s) for each band pair in the band combination) for concurrent transmission

Working assumption

Study the following alternatives for the minimum separation time between two UL Tx switchings for Rel-18 UL Tx switching schemes across up to 3 or 4 bands, and decide in RAN1#111 whether/which of the following alternatives is needed

- Alt.1: define 14 symbols based on a SCS (FFS on SCS) as minimum separation time between two UL Tx switchings

- Alt.2: define that no more than one uplink Tx switching within a reference slot based on a SCS (FFS on SCS)

- Alt.3: define X slots as minimum separation time between two UL Tx switchings where 3 bands are involved in total, and define Y slots as minimum separation time between two UL Tx switchings where 4 bands are involved in total, where X and/or Y is no less than 1 (FFS on X,Y, FFS reference SCS for the slots in case of multiple SCSs across carriers or expressed in unit of micro second)

- Alt.4: report the minimum separation time for different switching cases

- Other alternative is not precluded

- FFS: Applicable cases for the restriction

- Note: Companies are encouraged to provide detailed numbers of minimum separation time

Agreement

Consider following alternatives on the supported switching cases (Tx chain states) for each scenario

- Scenario#1: For switched UL, if UE supports up to 2 ports UL transmission on all the bands in the band combination,

  - Alt.1-1: only switching cases (Tx chain states) with 2T are assumed

    - In case of 3 bands, 3 switching cases ({2T,0T,0T}, {0T,2T,0T}, {0T,0T,2T}) are assumed

    - In case of 4 bands, 4 switching cases ({2T,0T,0T,0T}, {0T,2T,0T,0T}, {0T,0T,2T,0T}, {0T,0T,0T,2T}) are assumed

  - Alt.1-2: switching cases (Tx chain states) with 1T-1T can also be assumed

    - FFS: detailed switching cases to be assumed

- Scenario#2: For switched UL, if UE supports up to 2 ports UL transmission only on some of the bands,

  - Alt.2-1: for the band where 2 ports UL transmission is not supported, switching cases (Tx chain states) with 1T-1T can be assumed

    - FFS: detailed switching cases to be assumed with different number of bands supporting up to 2 ports UL transmission

  - Alt.2-2: only switching cases (Tx chain states) with 2T are assumed

    - Assumed switching cases are same as Scenario#1

  - Alt.2-3: switching cases (Tx chain states) with 1T-1T can also be assumed

    - FFS: detailed switching cases to be assumed

- FFS: Scenario#3: For dual UL, if UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission on all the bands in the band combination,

  - Alt.3-1: corresponding switching case(s) with 1T-1T for the band pair(s) are not assumed

    - FFS: if UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission only on some of the bands

  - Alt.3-2: corresponding switching case(s) with 1T-1T for the band pair(s) are assumed

    - Assumed switching cases are same as the case where UE supports dual UL for all band pairs in the band combination

Agreement

LS on UE capability and gNB configuration for UL Tx switching across 3 or 4 bands in Rel-18 is endorsed. Final LS in R1-2210724.

### RAN1#111: Agreements and conclusions

Agreement

For switched UL, if UE supports up to 2 ports UL transmission on all the bands in the band combination, only switching cases (Tx chain states) with 2T are assumed

- Conclusion: In case of 3 bands, 3 switching cases ({2T,0T,0T}, {0T,2T,0T}, {0T,0T,2T}) are assumed

- Conclusion: In case of 4 bands, 4 switching cases ({2T,0T,0T,0T}, {0T,2T,0T,0T}, {0T,0T,2T,0T}, {0T,0T,0T,2T}) are assumed

- Based on the assumption, the switching gap is required for every UL transmission with changing transmitting band from preceding transmission in this scenario

Agreement

For switched UL, if UE supports up to 2 ports UL transmission only on some of the bands in the band combination, only switching cases (Tx chain states) with 2T are assumed

- Based on the assumption, the switching gap is required for every UL transmission with changing transmitting band from preceding transmission in this scenario

Agreement

For dual UL, if a UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission on all the bands in the band combination, corresponding switching case(s) with 1T-1T for the band pair(s) where concurrent transmission is not supported are not assumed

Agreement

For dual UL, if UE supports concurrent transmission on all band pairs and supports up to 2 ports UL transmission on all the bands in the band combination, all possible switching cases with 1T-1T and 2T are assumed

- In case of 3 bands, 6 switching cases ({2T,0T,0T}, {0T,2T,0T}, {0T,0T,2T}, {1T, 1T, 0T}, {1T, 0T, 1T}, {0T, 1T, 1T}) are assumed

- In case of 4 bands, 10 switching cases ({2T,0T,0T,0T}, {0T,2T,0T,0T}, {0T,0T,2T,0T}, {0T,0T,0T,2T}, {1T,1T,0T,0T}, {1T,0T,1T,0T}, {1T,0T,0T,1T}, {0T,1T,1T,0T}, {0T,1T,0T,1T}, {0T,0T,1T,1T}) are assumed

Agreement

For dual UL, if UE supports up to 2 ports UL transmission only on some of the bands in the band combination, corresponding switching case(s) with 2T for the band where up to 2 ports transmission is not supported are assumed

- If the UE does not support concurrent transmission on specific band pair(s) in the band combination, corresponding switching case(s) with 1T-1T for the band pair(s) where concurrent transmission is not supported are not assumed

Agreement

Following new conditions are applicable to dual UL only (i.e., not applicable to switched UL)

- When the UE is to transmit a 1-port or 2-port transmission on one uplink carrier on one band (1st band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (2nd and 3rd band)

- When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 2T on a carrier on another band (3rd band)

- When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on one of the bands and another different band (1st or 2nd band, and 3rd band)

- When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (3rd and 4th band)

Agreement

Following working assumption is confirmed with updates.

Working Assumption

At least for dual UL, reuse existing RRC parameter {oneT, twoT} via uplinkTxSwitching-DualUL-TxState to solve the issue on ambiguous switching state at least for following cases

- Case#1 of the issue: two Tx chains are currently associated with band A, and next transmission is 1 port transmission on band B, but there are multiple possible switching cases where 1P on band B is supported

  - if twoT is indicated, both of two Tx chains are switched to band B

  - if oneT is indicated, one Tx chain is switched to band B while another Tx chain remains on band A

- Case#2 of the issue: two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, but there are multiple possible switching cases where 1P on band C is supported

  - if twoT is indicated, both of two Tx chains are switched to band C

  - if oneT is indicated, one Tx chain is switched to band C while how to determine the associated band for another Tx chain is FFS

    - Alt.1: based on gNB’s configuration/indication e.g., new RRC parameter

    - Alt.2: based on predefined rule

    - Other alternative is not precluded

- FFS for other potential cases

Agreement

In Case#2 where two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, if oneT is indicated via uplinkTxSwitching-DualUL-TxState, one Tx chain is switched to band C and associated band for another Tx chain is determined by new RRC parameter which is down-selected from following alternatives.

- An associated band is configured for each band so that another Tx chain is associated with the configured band (as associated band for the transmitting band)

  - E.g., associated band for each transmitting band is configured as {B for A}, {A for B}, {A for C} and {C for D}.

    - When 1 port transmission on band C is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band B is switched to band C while another Tx chain associated with band A remains unchanged (because band A is associated band for band C)

    - When 1 port transmission on band D is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band A (or B) is switched to band D while another Tx chain associated with band B (or A) is switched to band C (because band C is associated band for band D)

If there is one band where concurrent transmission with any other band is not supported, NW does not configure an associated band for the band. In such case, even if oneT is configured, UE performs switching as twoT is configured when 1 port transmission on the band is scheduled

Agreement

There is no restriction on number of bands supporting up to 2 ports UL transmission for both switched UL and dual UL and for both 3 bands and 4 bands.

- It is up to UE capability to support 2 ports UL transmission on none/some/all of the 3 or 4 bands

- Note: UE with only 1 Tx chain is not expected to perform UL Tx switching (no spec impact)

Agreement

Confirm the following working assumption.

Working Assumption

Specify UL Tx switching schemes across up to 4 bands in Rel-18

Agreement

Following restrictions are applied for Rel-18 UL Tx switching across 3 or 4 bands.

- The UE does not expect to perform more than one uplink switching within a reference slot based on µUL = max(µUL, 1, µUL, 2, µUL, 3) in case of 3 bands, µUL = max(µUL, 1, µUL, 2, µUL, 3, µUL, 4) in case of 4 bands, where µUL, 1, µUL, 2, µUL, 3, µUL, 4 are SCSs of active UL bandwidth parts of the bands in the band combination

  - If there are two consecutive intra-band carriers in one band, µUL, 1 = max(µUL, 1-1, µUL, 1-2), where µUL, 1-1 and µUL, 1-2 are SCSs of active UL bandwidth parts of the carriers in the band

- (working assumption) If two uplink switching are triggered and result in UL transmissions on more than 2 bands within any two consecutive reference slots, then the time duration between the end of all transmission(s) prior to the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time

  - The minimum separation time is a sum of X us and the switching gap required for the second uplink switching.

  - X us is subject to UE capability with a value set of {0us, 500us}

## RAN plenary outcomes

### RAN#96 approvals

The WID was approved to be revised as the following Error! Reference source not found.[1]:

1. Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1] - Identify the maximum number of cells that can be scheduled simultaneously - Consider both intra-band and inter-band CA operation - Consider both FR1 and FR2 - The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling 2. Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4] - UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4) - UE capability and RRC configuration related signalling (RAN2) - Note: strive for RAN1/2 design agnostic with the number of bands, i.e., common design between 3 and 4 bands - Note: no additional TAG is introduced for UL transmission on a carrier without corresponding DL carrier - Note: this objective does not target to extend the SUL framework to support more than 1 SUL for 1 NUL - Note: Extension of TX switching for 2 bands to multiple TAG configurations is included in the scope. The work is limited to RAN4. - Switching time and other RF aspects, and RRM requirements for above UL Tx switching schemes across up to 3 or 4 bands (RAN4) - Note: Prioritize UL Tx switching across up to 3 bands is to be addressed first and then that for up to 4 bands can also be addressed

The following proposal was agreed Error! Reference source not found.Error! Reference source not found.:

RAN provides following guidance to RAN1/2/4. - If Rel-18 UL Tx switching is supported, - RAN1/2/4 shall focus on defining necessary mechanisms and requirements for UL Tx switching across 3 or 4 different bands in Q3 2022 - Inter-band UL-CA Option 1 (i.e., switched UL) and Option 2 (i.e., dual UL) without SUL band - Inter-band UL CA Option 1 (i.e., switched UL) for {SUL band + corresponding NUL band} + 1 or 2 other NUL band(s) - UL CA framework where UL CA is performed between NULs according to current RAN4 specifications should not be changed - Note: switching across any band in this scenario is not precluded - Intra-band two contiguous aggregated carriers within one non-SUL band out of 3 or 4 bands - Further check additional scenarios in RAN#97e, e.g., - {SUL band + corresponding NUL band} + {SUL band + corresponding NUL band} - Simultaneous transmission across 2 bands in {SUL band + corresponding NUL band} + 1 or 2 other NUL band(s) (excluding simultaneous transmission between SUL and corresponding NUL) - Mechanisms/requirements should not introduce restrictions on what were already supported in current specifications for UL Tx switching

### RAN#97e approvals

Following proposals were agreed:

- Clarify that the number of TAGs is limited to up to 2 for both 2 bands switching and more than 2 bands switching cases

- Apply the proposed WID update in RP-222251

- Capture following conclusion in the meeting report of RAN#97-e.

  - Conclusion: for the work on UL Tx switching with 2 TAGs, RAN1/2 discussion can be triggered by RAN4 LS. No RAN1 spec impact is expected.

### RAN#98e approvals

The followings were endorsed:

- Conclusion:

  - It is confirmed by RAN/RAN1 chairs that additional 1 TU is allocated to Rel-18 MC enh WI for RAN1 in Q1 2023 (RAN1#112 meeting) by using one of RAN1 reserved TUs.

- SR in RP-223164 with 70% completion.

- The updated SR with TU excel sheet in RP-223557