3GPP TSG RAN WG1 #112		R1-2301573

Athens, Greece, February 27th–March 3rd, 2023

Agenda Item: 9.1.2

Source: MediaTek Inc.

Title: CSI enhancement

Document for: Discussion and Decision

# Introduction

In the RAN#94e meeting, the working item “NR MIMO evolution for downlink and uplink” for Rel-18 is approved. The objectives for DL CSI enhancement include [1]

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows:

-	Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis

-	UE reporting of time-domain channel properties measured via CSI-RS for tracking

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

- Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

- Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this technical document, we share our study results and views based on the agreements from the RAN1#111 meeting [2] and the RAN1#110bis-e meeting [3].

# CSI enhancement for high/medium UE velocities

## CQI association and Multiple CQIs

In RAN1#111 meeting, we have the following agreements on the slot association for X=1 CQI and the issue of X>1 CQIs [2]

Agreement For the Type-II codebook refinement for high/medium velocities, in one CSI reporting instance, for a given CQI sub-band, at least support including one CQI - FFS: The association of the CQI with PMI(s) and/or slot(s) within one duration of CSI reporting window WCSI - FFS: The support for including more than one CQIs Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, down-select (by RAN1#112) one from the following alternatives: - Alt1. The CQI is associated with the entire duration of the CSI reporting window and all the N4 W2 matrices - Alt2A. The CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Alt2B. The CQI is associated with the first/earliest d slots of the CSI reporting window and the first/earliest one of the N4 W2 matrices Note: The N4 W2 matrices represent the combining coefficients before DD compression at the UE, or after DD de-compression at the gNB. Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, decide by RAN1#112 whether including X>1 CQIs in one sub-band and one CSI reporting instance are supported - If supported, also decide the value(s) of X and the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots)

The slot association of CQI(s) for the various alternatives is illustrated in Figure 1.

Figure 1 CQI association in time domain

The first issue is to decide about the slot association of X=1 CQI. Alt 1 is associated with the entire CSI reporting window and all the N4 precoding matrices. In this alternative, the channel and precoder towards the end of the window would suffer from poor prediction performance. Further, the inter-cell interference, which cannot be predicted, would be outdated from the one measured during the latest CSI-RS occasion. Because of these two issues, the CQI calculated according to Alt 1 would suffer in performance. To achieve good prediction performance and accurate interference measurement, it is better if the CQI is associated to the slots closer to the start of the window. The same is observed in the simulation results in Figure 2.

Figure 2 UPT performance of various CQI association alternatives

For this simulation, we use ideal channel prediction at the UE to avoid the errors of a particular prediction algorithm influence the DD codebook compression and decompression processes. The traffic load is maintained at 40 % RU, to minimize MU-MIMO transmission, since MU-CQI is typically recalculated while pairing (also see below). The remaining simulation assumptions are provided in Appendix II. Since Alt 2A/2B can give a better performance, we support the association of X=1 CQI with the first slot or the first d slots of the CSI reporting window. However, since the UE prediction algorithms would typically predict the channel in slots of integral multiples of the CSI-RS occasions, the UE would need to perform CSI interpolation to obtain per slot channel matrix in Alt 2A. This interpolation can be avoided in Alt 2B, making UE implementation potentially simpler.

Observation 1: For association of X=1 CQI, Alt 2A/2B can provide up to 1% UPT gain over Alt 1.

Proposal 1: For association of X=1 CQI, support Alt 2B, where the CQI is associated with the first/earliest d slots of the CSI reporting window and the first/earliest one of the N4 W2 matrices

The next issue is to decide whether X>1 CQIs are needed and their corresponding slot association. The main argument for X>1 CQIs is to account for changes in the channel over the CSI reporting window. However, we note the following issues for the use of multiple CQIs in real-world networks:

- The CQI reported by the UE considering SU-MIMO transmission is not directly usable after MU pairing and the gNB recalculates/adjusts the reported CQI for MU-MIMO transmission

- MCS adaptation by outer loop link adaptation (OLLA) can manage the mismatch between time domain PMI and CQI, if any.

- The feedback overhead incurred for higher X can be non-negligible, especially with subband CQIs for each time domain CQI.

Further, the simulation results in Figure 2 show that X=2 and X=4 CQIs can achieve 1~1.5 % UPT gain over X=1, Alt 1. The UPT gain is <1 % considering Alt 2A/B. For this UPT gain, the increase in CQI feedback overhead is not justified and it is beneficial to consider CQI overhead reduction by considering differential CQI in time domain.

Observation 2: Multiple time domain CQIs reported by the UE may not be usable in real world networks because of MU CQI adjustment and OLLA

Observation 3: The avg. UPT gain considering X=2 and X=4 CQIs is 1~1.5 % over X=1, Alt 1. The UPT gain is <1 % considering Alt 2A/B.

### 2.1.1 CQI feedback overhead reduction

TS38.214 defines the concept of wideband reference CQI and subband differential CQI in the frequency domain, with the wideband CQI being reported using 4 bits and the differential CQIs using 2 bits each. The subband CQIs are then derived by gNB as

csbn=cwb+cdiffn		n=0,…N3-1,

where csbn is the CQI in subband n, cwb is the 4-bit wideband reference CQI, and cdiffn is the 2-bit differential CQI in subband n. The CQI feedback overhead in the absence of such differential CQI is 4N3 bits considering 4-bit subband CQIs. This is reduced to 4+2N3 bits with the differential formulation. The rationale for this CQI feedback overhead reduction is that the subband CQIs do not change significantly from the wideband CQI.

With X CQIs in time domain, the CQI overhead is X4+2N3 bits. However, since the time-domain CQIs are not expected to vary significantly from the reference TD unit, it is possible to use a similar rationale as in the frequency domain to reduce the CQI feedback overhead. Further, in order to aid omission of UCI in capacity constrained PUSCH, the most relevant CQI(s) can be included in CSI part 1, and the remaining time/frequency domain CQI(s) are included in CSI part 2. We describe three such feedback overhead reduction alternatives in the following.

Alternative 1

Denote the CQI in subband n=0,…N3-1 of time unit x=0,…X-1 as c(x)[n].

A single 4-bit reference CQI is reported for the entire time-frequency grid as cwb(0,…X-1)=c.  Then, 2-bit differential CQIs cdiff(x)n are reported for all time-frequency units, such that the CQIs for all time frequency units can be reconstructed by the gNB as

c(x)n=c+cdiff(x)n, x=0,…X-1,n=0,…N3-1

The CQI feedback overhead of this method is 4+2N3X bits, of which

- Part 1 CSI contains the reference CQI c (4 bits)

- Part 2 CSI contains the differential CQIs cdiff(x)n (2N3X bits)

Alternative 2

A 4-bit wideband reference CQI cwb(0) is reported for CQI time unit 0. Then, subband CQIs of time unit 0 are reporting using 2-bit differential CQIs cdiff(0)n relative to reference CQI, such that the CQIs for all subbands of time unit 0 can be reconstructed by the gNB as

c(0)n=cwb(0)+cdiff(0)n, n=0,…N3-1

Further, wideband CQI of remaining (X-1) units are reported using 2-bit differential CQIs cdw(x) relative to reference CQI, such that the gNB reconstructs the wideband CQIs of remaining (X-1) units as

cwb(x)=cwb(0)+cdw(x), x=1,…X-1

Finally, subband CQIs of remaining (X-1) units are reported using 2-bit differential CQIs cdiff(x)n relative to wideband CQIs of respective time units, such that the gNB reconstructed subband CQIs are

c(x)n=cwb(x)+cdiff(x)n, x=1,…X-1,n=0,…N3-1

The CQI feedback overhead of this method is 4+2N3+2X-1+2X-1N3 bits, of which

- Part 1 CSI contains CQIs of time unit 0, i.e., c, cdiff(0)n,n=0,…N3-1 (4+2N3 bits)

- Part 2 CSI contains differential CQIs of remaining time units, i.e., cdw(x), cdiff(x)n, x=1,…X-1,n=0,…N3-1(2X-1+2X-1N3 bits)

Alternative 3

This alternative is a more aggressive reduction scheme compared to Alt 2, wherein we assume that the time domain variations in the CQI are captured by the differential CQIs cdw(x), x=1,…X-1, so that the subband differential CQIs of other time units need not be reported. This is described below.

A 4-bit wideband reference CQI cwb(0) is reported for CQI time unit 0. Then, subband CQIs of time unit 0 are reporting using 2-bit differential CQIs cdiff(0)n relative to reference CQI, such that the CQIs for all subbands of time unit 0 can be reconstructed by the gNB as

c(0)n=cwb(0)+cdiff(0)n, n=0,…N3-1

Further, wideband CQI of remaining (X-1) units are reported using 2-bit differential CQIs cdw(x) relative to reference CQI, such that the gNB reconstructs the wideband CQIs of remaining (X-1) units as

cwb(x)=cwb(0)+cdw(x), x=1,…X-1

Since cdw(x) captures the time domain behavior of CQI, the subband CQIs of remaining time units are reconstructed by the gNB, without additional feedback, as

c(x)[n]=c(0)n+cdw(x), x=1,…X-1,n=0,…N3-1

The CQI feedback overhead of this method is 4+2N3+2X-1 bits, of which

- Part 1 CSI contains CQIs of time unit 0, i.e., c, cdiff(0)n,n=0,…N3-1 (4+2N3 bits)

- Part 2 CSI contains differential CQIs of remaining time units, i.e., cdw(x), x=1,…X-1 (2X-1 bits)

The CQI feedback overhead of these alternatives is shown in Table 1. From the above analysis, we have the following observations and proposals.

Observation 4: With X>1 CQIs in the time domain, the subband CQI feedback overhead increases considerably for Rel-18 Type II codebook.

Proposal 2: For the Rel-18 Type-II codebook refinement for high/medium velocities, investigate methods to reduce CQI feedback overhead for X>1 time-domain CQIs

Table 1 CQI feedback overhead

| N3 | X | Rel-16 eType II CQI feedback overhead | Rel-18 Type II CQI feedback overhead |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | No enhancement | Alt 1 | Alt 2 | Alt 3 |
| 19 | 4 | 42 | 168 | 156 | 162 | 48 |
| 19 | 2 | 42 | 84 | 80 | 82 | 44 |
| 14 | 4 | 32 | 128 | 116 | 122 | 38 |
| 14 | 2 | 32 | 64 | 60 | 62 | 34 |
| 7 | 4 | 18 | 72 | 60 | 66 | 24 |
| 7 | 2 | 18 | 36 | 32 | 34 | 20 |

Proposal 3: For the Rel-18 Type-II codebook refinement for high/medium velocities, reduce multiple CQI feedback overhead using the concepts of reference CQI and differential CQIs

Proposal 4: For the Rel-18 Type-II codebook refinement for high/medium velocities, aid UCI omission of multiple time domain CQIs by including reference CQI(s) and certain differential CQI(s) in CSI Part 1 and including remaining CQIs in CSI Part 2.

## Codebook structure

In RAN1#111 meeting, we have the following agreements regarding the codebook structure [2]:

Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter N4 (length of DFT vector, unit-less), support 8 as an additional candidate value - FFS (by RAN1#112): Whether any of the following additional candidate values are supported: 3, 5, 16, 32 - The candidate values supported by UE are reported via UE capability (details can be discussed in UE feature). Agreement For the Type-II codebook refinement for high/medium velocities, for N4>1, regarding parameter Q, decide in RAN1#112 whether to support the additional values of 3 and/or 4 Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, - The definition and supported values for each of the SD/FD codebook parameters follow the legacy specification. - FFS: The supported parameter combinations considering SD, FD, and DD codebook parameters - For N4=1, the legacy quantization is fully reused. Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, for N4=1, one CSI reporting instance includes a single per layer, a single , and a single Conclusion For the Rel-18 Type-II codebook refinement for high/medium velocities, when N4>1, there is no consensus in introducing rotation factor (selected for each SD basis vector) as an additional PMI component Agreement For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, support the following: - Q different 2-dimensional bitmaps are introduced for indicating the location of the NZCs, where the qth (q=1,…., Q) 2-dimensional bitmap corresponds to qth selected DD basis vector - The number of selected DD basis vectors is denoted as Q - This implies that for each layer, the location of NZCs in SD-FD can be different for different selected DD basis vectors. FFS: Further overhead reduction on bitmap(s) FFS: Whether the number of NZCs is upper bounded across all DD basis vectors or per DD basis vector

There are the following issues for discussion regarding the codebook structure:

- Remaining values of N4

- The size of the DD/TD unit d for periodic CSI-RS

- Additional values of Q

- NNZC bitmap reduction methods

Regarding the values of N4, it is obvious that longer CSI reporting window size will require the prediction to happen long after the latest CSI-RS occasion. This affects prediction performance. Further, reducing TD unit size d to reduce the size of CSI reporting window WCSI=dN4 will require the UE to predict/interpolate the channel every few slots, increasing complexity. Therefore, larger values of N4, i.e., N4=16,32 should be avoided.

Proposal 5: For the Type-II codebook refinement for high/medium velocities, do not support N4=16,32.

For the next two issues, we provide simulation results in Figure 3. In this simulation, we set the CSI-RS periodicity to 4 slots, which is the lowest possible, and CSI reporting periodicity is also set to be 4 slots. The baseline for performance comparison is Rel-16 eType II with parameter combination 1. The same parameter combination is also considered for Rel-18 Type II codebook. For comparison, we normalized the feedback overhead of Rel-18 Type II PMI by the reporting periodicity, i.e., 4 slots, and consequently the x-axis represents the feedback overhead gain achieved by Rel-18 Type II codebook over Rel-16 eType II codebook. The desired operation regime of Rel-18 Type II codebook is the first quadrant, with the operating points lying as away from the origin as possible (implying that the Rel-18 codebook achieves a considerable UPT gain with a reduced feedback overhead compared to Rel-16 codebook).

Figure 3 UPT gain vs. feedback overhead gain for TD unit size and Q values

The blue line represents constant N4=4, with d=1,2,4 and Q=2. It can be seen that as the value of d reduces, the UPT gain increases, however, the feedback overhead gain becomes a negative quantity. This shows that as the value of d reduces, the size of the CSI reporting window WCSI reduces and the feedback overhead increases significantly. The red line represents constant WCSI=8 slots, with d=1,2,4 and Q=2. Here, it is seen that as the value of d reduces, the value of N4 increases, thereby increasing the amount of DD compression for the value of Q=2. This degrades the performance. Finally, the grey line represents constant WCSI=8 slots, with d,N4,Q=1,8,4, (2,4,3) and (4,2,2). It is seen that Q=3,4 can increase the performance by 3~4 % compared with Q=2, however, the feedback overhead incurred increases by 30~50 %. In our opinion, this increase in overhead does not justify the little performance gain.

Observation 5: For the same N4, reducing the value of d reduces CSI reporting window size, thereby increasing feedback overhead.

Observation 6: For the same WCSI, reducing the value of d increases the amount of DD compression, thereby degrading performance.

Observation 7: Q=3,4 can provide an avg. UPT gain of 3~4 % compared with Q=2, however, the feedback overhead incurred increases by 30~50 %.

Proposal 6: For the Type-II codebook refinement for high/medium velocities, support d= CSI-RS periodicity.

Proposal 7: For the Type-II codebook refinement for high/medium velocities, do not support additional values of Q other than Q=2.

Finally, we discuss the NNZC bitmap overhead reduction methods. In the last meeting, the following alternatives were agreed regarding bitmap(s) for indicating the locations of the NZCs:

- Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e., the size of the bitmap for each selected DD basis vector is 2LMv

- Alt2. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design and further compressed using source-coding (e.g Huffman code)

- Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector.

- Alt3B: A single 2-dimensional bitmap of size 2LQ to report the selected S pairs of SD components and DD basis vector and a single 2-dimensional bitmap of size MS for indicating the location of the NZCs, where each row corresponds to a selected FD basis vector and each column corresponds to one of the selected S pairs of SD component and DD basis vector.

- Alt3C: A single 2-dimensional bitmap of size 2LM to report the selected S pairs of SD component and FD basis vector and a single 2-dimensional bitmap of size QS for indicating the location of the NZCs, where each row corresponds to a selected DD basis vector and each column corresponds to one of the selected S pairs of SD component and FD basis vector.

- Alt4. A bitmap that includes bits associated with the set of {(f1, s1, t1)} with d(f1, s1, t1)<=d, where d is the threshold that can be configured by gNB, df1, s1,t1= minf1-f0, Mv-f1-f0+ mins1-s0, L - s1-s0+ mint1-t0, Q - t1-t0. s0, f0 and t0 denotes a reference SD basis index and a reference FD basis index and a reference DD basis index associated with SCI, respectively.

We compare the performance of Alt 1 and the three versions of Alt 3 by simulations, and the results are shown in Figure 4. The details of NZC selection for each of these alternatives is described in the following.

Figure 4 UPT performance of NZC bitmap indication methods (μ=0.75 (top) and μ=0.5 (bottom))

In Alt 1, Q bitmaps of size 2LM, and a total of K0=β×2LMQ NZCs per layer are fed back, with the restriction of 2K0 NZCs across all layers.

In Alt 3A, S=μMQ delay-Doppler pairs are indicated using a bitmap of size MQ. Subsequently, a total of K0=β×2LS NZCs per layer are fed back, with the restriction of 2K0 NZCs across all layers. An illustration of Alt 3A is shown in Figure 5.

Figure 5 NZC indication by Alt 3A

In Alt 3B, S=μ2LQ beam-Doppler pairs are indicated using a bitmap of size 2LQ. Subsequently, a total of K0=β×MS NZCs per layer are fed back, with the restriction of 2K0 NZCs across all layers.

In Alt 3C, S=μ2LM beam-delay pairs are indicated using a bitmap of size 2LM. Subsequently, a total of K0=β×QS NZCs per layer are fed back, with the restriction of 2K0 NZCs across all layers.

Figure 4 shows the results for μ=0.75 and μ=0.5. Parameter combinations 1~4 of Rel-16 eType II are used for performance evaluation, and as the baseline for UPT gain. It is seen that Alt 3A/3B can achieve better UPT-overhead tradeoff compared to Alt 1. Although the overall range of UPT gain is around 5% less with Alt 3 variants compared to Alt 1, they can achieve the same gain as Alt 1 with 40~50 bits of overhead saving. The UPT-overhead tradeoff provided by Alt 3C is similar to Alt 1, since it is equivalent of selecting beam-delay pairs as in Rel-16 with a smaller β. Alt 3A also becomes a natural choice when considering the codebook structure W1W2Wf⨂WdH. Therefore, we are open to support Alt 3A.

Observation 8: NZC indication by Alt 3A/3B can yield the same UPT gain as Alt 1 with 20~30 % feedback overhead saving.

Proposal 8: For the Type-II codebook refinement for high/medium velocities, support NZC indication by Alt 3A, wherein S delay-Doppler pairs are selected first, and 2LS NZCs are selected for feedback subsequently.

## CSI measurement/reporting

In RAN#111 meeting, we have the following agreements regarding various parameters involved in CSI measurement and reporting [2]:

Agreement For the Type-II codebook refinement for high/medium velocities, the parameter K (the number of AP-CSI-RS resources for the CMR) is gNB-configured via higher-layer (RRC) signalling at least from the following set of candidate values: {4, 8} - FFS: If additional candidate value(s) of K are supported, e.g. 5, 12, 16, also taking into account other use cases (e.g. for training filter coefficients, prediction or performance monitoring) and TDD Agreement For the Type-II codebook refinement for high/medium velocities, the parameter m (offset between two AP-CSI-RS resources for the CMR, in slots) is gNB-configured via higher-layer (RRC) signalling from the following set of candidate values: {1, 2} - FFS: Whether 4, 5, 8, 12, and/or 16 are also supported as other candidate value(s) Agreement For the Type-II codebook refinement for high/medium velocities, the parameter δ (in slots) is gNB-configured via higher-layer (RRC) signalling from a set of the following candidate values: - First candidate value: δ=0, - 2 additional non-zero values of parameter δ are supported - FFS: the non-zero value(s), to be selected from 1, 2, 3, 4, 5, 6, and 8 Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter δ (in slots), support the additional value of 2 - FFS (by RAN1#112): For the last supported additional value, down select between 1, 3, 4, and 5 Conclusion For the Type-II codebook refinement for high/medium velocities, regarding the parameter m (offset between two AP-CSI-RS resources for the CMR, there is no consensus in supporting additional candidate value(s).

Regarding the value of K, to have a reasonable CSI-RS overhead and buffer size of UE, K cannot be arbitrarily large. Also, we note that irrespective of how large a value of K is configured, UE would use only as many CSI-RS occasions as its implementation allows. Therefore, our preference is not to support K>10.

Regarding the value of δ, large values are not preferable as the prediction performance degrades if the starting position of the reporting window is further away from the last CSI-RS occasion. Therefore, our preference is to support δ=1,3.

Proposal 9: For the Type-II codebook refinement for high/medium velocities, do not support K>10.

Proposal 10: For the Type-II codebook refinement for high/medium velocities, support δ=1,3.

## Time-domain channel properties (TDCP)

In RAN1 #111, we reached the following agreements regarding the TDCP metric selection [2].

Agreement For the Rel-18 TRS-based TDCP reporting, down select only one of the following alternatives by RAN1#112: - AltA.1 (Doppler spread) as described in R1-2210523 - AltA.2 (Doppler shift): A UE is configured to report the Doppler shifts corresponding to the M strongest peaks of the wideband Doppler spectrum, for each of the N≥1 configured TRS resources - A TDCP report can be configured with N periodic TRS resources (e.g., N=2 with one TRS resource per TRP) - Parameter M is RRC configured with candidate values TBD, e.g. M=1,2,3,… - Wideband Doppler spectrum is calculated from the wideband time correlation function, given, as an example, by At,τ=c(t,τ)c(t,0), where ct,τ=n=0N-1hnt+τ hn*t and hn is the channel for subcarrier n. - AltB (TD correlation profile) as described in R1-2210523 Down-selection is to done based on, at least, the (single-)user throughput (LLS) performance comparison among the alternatives assuming: - Three special cases of an agreed use case (companies can select only one or more): aiding gNB to determine switching between Type-I and Rel-16 eType-II codebooks, or to determine SRS periodicity in the UL-SRS reciprocity-based precoding scheme; or aiding the gNB implementation in CSI prediction for TDD - In their simulations on switching between Type-I and Rel-16 eType-II codebooks, companies should state how to calculate the metric for the determination and how to set the threshold, and what the UE reports. - In their simulations on UL-SRS reciprocity-based precoding scheme, companies should state how to set the SRS periodicity based on the reported metrics, and what the UE reports; and the results should be displayed in terms of user throughput vs SRS overhead - In their simulations on CSI prediction for TDD, the results should be the correlation between real channel and predicted channel, and what the UE reports; aided by the reported metric. - Other scenarios of the agreed use cases can optionally be simulated - Based on the agreed EVM for sTRP and mTRP Note: Different alternatives may or may not apply to different use cases FFS: The need for a measure of confidence level in the TDCP report, and/or UE behaviour when the quality of TDCP measurement is not sufficiently high FFS: TDCP parameter(s) signalled with respect to each alternative

To summarize the previous agreements, three possible alternatives for TDCP are identified for down-selection. The first one, i.e., Alt A1, is doppler-spread based, the second one, i.e., Alt. A2, is doppler shift based, while the third one, i.e., Alt. B, is auto-correlation based. Three possible uses cases for TDCP are also identified, including DL codebook switching, which we will focus on next.

In this document, we share our view as to which of the three metrics is best suited for the identified use cases, while focusing on DL codebook switching. First, let us start with some definitions we shall use in this section, before delving into further details.

fc≝ Carrier frequency

c≝ speed of light

τ≝ Time lag used for CSI auto-correlation calculation

A(τ)≝ Normalized auto-correlation function

Ts≝ Auto-correlation sampling time

fs≝ Auto-correlation sampling frequency

NTRS≝ Number of TRS bursts

fd≝ Dopper frequency

Δfd≝ Doppler frequency resolution

fd,max≝ Maximum unambiguous Doppler frequency

v≝ UE speed

Δv≝ UE speed resolution

To perform simulations for TRS-based TDCP, we need to identify appropriate TRS parameters that would help accomplish the goals of CSI estimation to be used for TDCP calculations. To that end, we provide the following discussions on “maximum resolvable doppler frequency” and “doppler shift resolution”.

Maximum Resolvable Doppler frequency

Since the auto-correlation function is quantized, there exist a maximum doppler frequency that can be resolved. This is referred to as the maximum unambiguous (i.e., resolvable) doppler frequency, and is denoted as fd,max. Any doppler frequency higher than this value, will result in aliasing --- a situation we want to avoid.

The value of fd,max can be derived using the Nyquist criterion, where

fd,max=fs2=12Ts,

We can treat Ts or fs as design parameters, which we want to choose such that a certain fd,max is achieved. Hence, we need fs to satisfy

fs≥2fd,max=2vmaxfcc

⟺ Ts≤c2vmaxfc

For example, if fc=3.5 GHz and vmax=120 Km/h, then Ts≤ 1.2857 ms.

Assuming subcarrier spacing of 30KHz, this means that we need to sample the channel at least once every 2 time slots (i.e., Ts=1 ms). If we violate this criterion, then we will not be able to take correct decision regarding doppler shift or UE speeds. To show an example of the seriousness of this issue, consider a UE moving at 120 Km/h and carrier frequency fc=3.5 GHz. We use CDL-A channel model and 20 TRS bursts with periodicity of 5 slots, i.e., (Ts=2.5 ms). Then, we plot the doppler spectrum (top) and the auto-correlation function A(τ) (bottom) as follows:

Figure 6. 20 TRS bursts with 5 slot separation.

The figure above depicts a maximum peak at a doppler shift of approximately 40 Hz.

Note also that the maximum doppler shift we expect to get is =±fcvc=±388.88 Hz.  On the other hand, the above figure shows a maximum doppler shift above -200 and below 200 Hz. This is yet another indication that the above figure with Ts=2.5 ms is not suitable to correctly predict the doppler spectrum.

Next, we repeat the same experiment and only change the TRS periodicity to 2 slots (i.e., Ts=1 ms). This results in the following figure, which is drastically different from the previous one.

Figure 7. 20 TRS bursts with 2 slot separation.

Now, we see a maximum peak of the doppler spectrum occurring at a doppler shift of approximately -325 Hz, and another lower peak at approximately 250 Hz. For completeness, we also show the result if we kept the TRS periodicity at Ts=1 ms but only used 8 TRS bursts (instead of 20). This results in the following figure.

Figure 8. 8 TRS bursts with 2 slot separation

The disadvantage of using 8 TRS bursts instead of 20 is the lower resolution of the obtained doppler spectrum. Nonetheless, the figure still predicts a maximum peak at a doppler shift of approximately  -333 Hz and a second peak at 266 Hz.

Observation 9: To avoid doppler spectrum aliasing, the auto-correlation sequence must be sampled at time lags with a period ≤c2vmaxfc .

Observation 10: For Periodic TRS-based TDCP calculation, TRS bursts should allow autocorrelation estimation with time lag periodicity upper bounded by c2vmaxfc, where vmax is the maximum considered UE speed, fc is the carrier frequency and c is the speed of light.

Doppler frequency Resolution

Recall that the number of TRS bursts is denoted by NTRS. This means that the auto-correlation function we can calculate has length N≝ 2NTRS-1. This can be decomposed to NTRS-1 positive time lags and NTRS-1 negative time lags, in addition to the zero lag. With sampling frequency fs, we get a doppler resolution of

Δfd=fsN

We can translate this equation into a UE speed resolution as follows:

Δfd=fc Δvc=fsN=1TsN

⟺ Δv=c fc Ts N

Let us consider a numerical example to make better sense of this. Let NTRS = 8, fc=3.5 GHz, and Ts=1ms. Then, we can calculate N=2×8-1=15, which gives respective doppler shift and UE speed resolutions of

Δfd= 66.667 Hz,

Δv=5.714 m/s=20.571 Km/H.

This value of UE speed resolution has low fidelity which makes it difficult to distinguish UE speeds with difference below 20.57 Km/h.

The TDCP Metrics

Recall that three alternatives are identified, namely A1, A2 and B. Based on each alternative, we come up with a singular value that we can compare to a threshold. The result of that comparison determines whether Type I Single-Panel codebook is to be used, or Type II (Rel16) codebook will be used for data precoding in the downlink channel. We calculate the three metrics as follows:

A1: This alternative is straight forward to calculate, despite its high computational complexity compared to other alternatives, since the result is already a single quantity that represents the predicted doppler spread. We follow the procedure described in R1-2210523 as agreed in RAN1 #111. The model we use for the Doppler Spectrum is Jake’s model, which has an auto-correlation function in the shape of a scaled zero-order Bessel function of the first kind. We normalize the obtained auto-correlation sequence with respect to the zero-lag, and we do the same normalization for the TRS-based auto-correlation sequence. This calculation is performed per delay-tap, and for each tap we solve a minimization problem, which results in the best matching doppler shift. We use an exhaustive search solution over discretized doppler shift values with a step size of 1Hz.

A2: Again, we follow the agreement in RAN1 #111, and we use the wide-band auto-correlation function to calculate the doppler spectrum. However, once the doppler spectrum is obtained, we still have a choice as to what value of M (the number of peaks) for which the corresponding doppler shifts are to be reported. We choose M=2, so that the gNB can try to predict the value of the doppler spread as the absolute difference between those reported doppler shifts. This results in a single value that can be compared to a threshold.

B: This alternative is perhaps the simplest since it only requires a direct feedback for the wideband auto-correlation function. Specifically, we choose to feedback the absolute value of the auto-correlation function at the first non-zero time lag.

Simulations

We conduct simulations using carrier frequency at fc=3.5 GHz and use TRS bursts with time separation of 2 slots. Note here that while periodicity 2 is not a possible option for periodic TRS, this can be achieved using Aperiodic TRS instead. Note also that this is necessary to overcome aliasing issue we mentioned earlier for UE speeds of up to 120 Km/h.

We run a precursory simulation that helps us choose appropriate threshold values for all three TDCP metrics. Based on our chosen values, we then run another set of simulations to compare the achieved performance under each of the TDCP metric alternatives.

In the following figure, we use NTRS=8 bursts, and we plot the average throughput in units of Mbps vs. the UE speed (in Km/h). The curves for Type I single-panel and E-Type II use fixed codebooks, while those curves named “CB switching X” dynamically switch codebooks using the TDCP metric X ∈ {A1, A2, B}.

Figure 9. TDCP metric comparison.

Our results indicate the only alternatives B and A1 have decent performance, while alternative A2 struggles to optimally switch codebooks. The reason behind alternative A2’s limitation is the low doppler frequency resolution (66.66 Hz) and as a result it almost always ends up choosing E-Type II even when Type I is a better option. For the remaining two alternatives, B shows slightly better performance at lower UE speeds (3,5,10 Km/h) while A1 shows slightly performance for intermediate UE speeds (20 and 30 Km/h). For higher speeds of 60 and 90 Km/Hr, both alternatives are equally good and they both reach the performance of the Type I codebook, which is the best codebook alternative at those speeds.

Observation 11: Alternatives B and A1 both achieve good performance for the codebook switching use case at all UE speeds.

Observation 12: Alternative A1 has much higher computational complexity compared to that of B for while delivering similar performance.

Proposal 12: For TDCP reporting, use Alt. B (autocorrelation-based reporting as described in R1-2210523) as report quantity.

# CSI enhancement for coherent JT

In this section, we continue the discussion on CSI enhancements for mTRP CJT.

## FD bases selection and reporting for Mode 1 codebook

In RAN1#111 meeting, the following agreements were made regarding frequency domain (FD) bases selection in Mode 1 codebook for Coherent-JT [2]:

Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, study and down select (no later than RAN1#112) only one from the following schemes: - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: where is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource with , and is commonly selected across N CSI-RS resources - Alt2. independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) - Alt3. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: where is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource with , and is independently selected across N CSI-RS resources For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FS basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

In Table 2, we compare the three alternatives of FD basis selection and reporting against three criteria, namely – deployment scenarios of interest, UE implementation, and feedback overhead.

Table 2 Comparison of FD basis selection methods

| Criterion | Alt 1 | Alt 2 | Alt 3 |
| --- | --- | --- | --- |
| Scenarios of interest | Suitable in certain symmetrical scenarios (e.g. co-site largely spaced multi-panel) having similar FD bases distribution across TRPs. | Suited for most practical scenarios without any relation between per TRP FD bases | Suitable for high N3 and/or low pv when the dominant FD bases across TRPs are disjoint |
| UE Implementation | Preferable for implementation, since common FD bases can be found across TRPs (can be reused for Mode 2) and best offset may be energy detected. | FD bases computation may need to be repeated for each TRP | Does not offer significant implementation advantage compared to Alt 2 |
| Feedback overhead | Low | High | Slightly lower than Alt 2 |

It is seen that Alt 1 would perform best when dominant FD bases are same across TRPs, but are separated by offsets (e.g. related to non-uniform inter-antenna distance and/or frequency offset across TRPs – as in co-site multi-panel deployments), and Alt 3 is expected to perform best when there are a small number of FD basis (small pv) disjoint across TRPs (different windows). Given that Alt 1/3 are only useful in specific scenarios, Alt 2 would be a natural choice considering diverse deployments. However, considering UE implementation and feedback overhead, Alt 1 has the potential to maintain commonality with Mode 2 codebook, which was the original intention of Mode 1 codebook.

Further, we also compared the performance of Alt 1 and Alt 2 by simulations in Figure 10. The simulation settings are presented in Appendix III. We do not assume any dynamic TRP selection, such that it is always 4-TRP transmission. The best combination of SD bases (L1,L2,L3,L4) is chosen by the UE. The set of dominant FD bases are found per layer, post the SVD of the mTRP channel, similar to FD bases selection in Rel-16 eType II precoder.

Figure 10 Performance comparison of Alt 1 and Alt 2 feedback schemes

It is seen from the results that at low to medium ISDs (500m), Alt 2 does not provide any performance benefit over Alt 1. This is because, post SVD of mTRP channel, Alt 1 can better select a common FD basis set for coherent transmission, while Alt 2 favours individual TRP precoders to be better by TRP specific FD basis selection, thereby suffering in coherent transmission performance. As the ISD becomes larger, it becomes more probable for a UE to be served by a single dominant TRP, so that Alt 2 gives a slightly better performance than Alt 1 in certain limited parameter combinations.

Based on the above arguments and results, we make the observations and proposals.

Observation 13: Considering UE implementation, feedback overhead, and commonality with Mode 2, Alt 1 is a preferred choice for Mode 1 codebook FD bases selection

Observation 14: FD bases selection Alt 2 does not provide a consistent performance benefit over Alt 1, even in high ISD scenarios.

Proposal 13: On the Type-II codebook refinement for CJT mTRP, for mode-1, support Alt 1.

## On W2 quantization alternatives

In RAN1 #110bis-e meeting, the following agreement was made regarding W2 coefficient quantization for Rel-18 codebook [3]:

Agreement On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer: - Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2) - FFS: Amplitude quantization table enhancement - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported - Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111) - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N) - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported - If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook

Two meetings ago, it was agreed to support quantization Alt 1 – with Cgroup,phase=1and Cgroup,amp=2 for the Rel-18 Type II codebook for CJT. However, there were extensive discussions on Alt 3 – which differs from Alt 1 by having Cgroup,amp=2N, i.e., each polarization of each TRP having its own amplitude reference. It was argued that such amplitude reference would improve performance in geographically distant TRP deployments.

However, amplitude variations across distant TRPs are implicitly taken care in the precoder computation from the joint mTRP channel and consequently in quantization Alt 1. Having separate amplitude references would artificially boost the number of NZC, thereby increasing overhead but not contributing meaningfully to performance improvement. Moreover, following the rationale of multi-TRP transmissions to ensure uniform coverage to all UEs, as long as the UE is close to one or two TRPs in the coordinating set, a single reference amplitude is enough to reap the benefit of CJT.

In order to support our arguments, we provide simulation results comparing the performance of Alt 1 and Alt 3 in Figure 11. The simulation scenario and settings are as explained earlier. From the results, we observe that Alt 3 cannot provide consistent performance benefit over Alt 1. Further, the cost of this little performance benefit has to be borne by the increased overhead of feeding back multiple reference amplitudes. Therefore, supporting quantization Alt 3 is not necessary.

Observation 15: Quantization Alt 3 does not provide consistent performance benefit over Alt 1, even in high ISD scenarios.

Proposal 14: For Rel-18 Type II codebook for CJT, revert the working assumption on quantization Alt 3.

Figure 11 Performance comparison of quantization Alt 1 and Alt 3

## On parameter candidate values

In RAN1 #111, the following agreement was made regarding parameter values for Rel-18 codebook [2]

Agreement On the Type-II codebook refinement for CJT mTRP, regarding the SD basis selection, for a configured value of NTRP, a set of NL combinations of values for {L1, ..., LNTRP} is gNB-configured via higher-layer (RRC) signaling - When NL>1, the selected combination of values for {L1, ..., LNTRP} is reported in CSI part 1 using an indicator, selected from the NL configured combinations - NL =1 is one of the supported candidate values - FFS: Other supported value(s) of NL, and its respective UE capability - FFS: The supported combinations of values for {L1, ..., LNTRP} - Following the legacy design, the SD basis selection for the n-th (n=1,...,N) selected CSI-RS resource is indicated in CSI part 2 using a combinatorial indicator selected from a set of codepoints where, for Rel-16-based refinement PCSI-RS = 2*N1N2. - The supported candidate values for each of the Ln parameters include the legacy candidate values, i.e. {2,4,6} for Rel-16-based refinement, and - for Rel-17-based refinement, the gNB configures a set of N_L combinations for {alpha1, ..., alphaNTRP} where FFS: Whether the set of NL combinations of values for {L1, ..., LNTRP} can be implicitly derived Following the legacy design, for all the selected N CSI-RS resources, the SD basis oversampling group for each CSI-RS resource is indicated in CSI part 2 using an indicator selected from a set of O1O2 codepoints.

Regarding configuration of a separate table for {L1, ..., LNTRP}

Given that there are three candidate values for Ln (i.e., 2,4,6) and the previous agreement to support dynamic TRP selection by the UE, the list of combinations for {L1, ..., LN} and N=1,…NTRP would be prohibitively high for specifying a table. To arrive at a generic, reduced number of combinations requires performing extensive simulations in different scenarios under various ISDs and eliminating scenario-specific and similar UPT-overhead yielding combinations. Further, such performance results would highly depend on the specific combination of pv and β, implying a linkage between the combinations of {L1, ..., LN} and (pv,β). Also, as in Rel-16, the aim of Rel-18 parameter combination should be to specify a combination table in increasing order of performance-overhead, which also implies a linkage.

Therefore, in our opinion, the discussion on the (reduced set of) combinations {L1, ..., LN} cannot happen independent of (pv,β), and there needs to be a link between these two combinations even if they are specified independently.

Observation 16: Given the three candidate values for Ln and the previous agreement to support dynamic TRP selection by the UE, the list of combinations for {L1, ..., LN} and N=1,…NTRP would be prohibitively high for specifying a table.

Observation 17: Extensive simulations in different scenarios under various ISDs are required to arrive at a generic, reduced number of combinations of {L1, ..., LN}.

Observation 18: Performance-overhead of mTRP CJT highly depends on the specific combinations of {L1, ..., LN} and (pv,β), implying a link between them.

Proposal 15: Regarding parameter combination framework for Rel-18 Type II codebook for mTRP CJT, further study is needed

- To arrive a generic, reduced set of combinations of {L1, ..., LN} for potential specification

- To specify a linkage between the combinations of {L1, ..., LN} and (pv,β).

- To specify a list of parameter combinations in increasing order of performance-overhead.

Regarding the value of NL

The UE must select a specific combination {L1, ..., LN} of SD bases among NL combinations configured by the gNB. This selection is not trivial and may require the UE to do capacity computations for all combinations to arrive at the best one. Therefore, a restriction or UE capability on the value of NL is needed. Our preference is NL≤4.

Proposal 16: To limit UE complexity of selecting the best SD basis combination, we prefer NL≤4.

# Conclusion

In summary, based on the above discussion we have the following observations and proposals:

Observation 1: For association of X=1 CQI, Alt 2A/2B can provide up to 1% UPT gain over Alt 1.

Observation 2: Multiple time domain CQIs reported by the UE may not be usable in real world networks because of MU CQI adjustment and OLLA

Observation 3: The avg. UPT gain considering X=2 and X=4 CQIs is 1~1.5 % over X=1, Alt 1. The UPT gain is <1 % considering Alt 2A/B.

Observation 4: With X>1 CQIs in the time domain, the subband CQI feedback overhead increases considerably for Rel-18 Type II codebook.

Observation 5: For the same N4, reducing the value of d reduces CSI reporting window size, thereby increasing feedback overhead.

Observation 6: For the same WCSI, reducing the value of d increases the amount of DD compression, thereby degrading performance.

Observation 7: Q=3,4 can provide an avg. UPT gain of 3~4 % compared with Q=2, however, the feedback overhead incurred increases by 30~50 %.

Observation 8: NZC indication by Alt 3A/3B can yield the same UPT gain as Alt 1 with 20~30 % feedback overhead saving.

Observation 9: To avoid doppler spectrum aliasing, the auto-correlation sequence must be sampled at time lags with a period ≤c2vmaxfc .

Observation 10: For Periodic TRS-based TDCP calculation, TRS bursts should allow autocorrelation estimation with time lag periodicity upper bounded by c2vmaxfc, where vmax is the maximum considered UE speed, fc is the carrier frequency and c is the speed of light.

Observation 11: Alternatives B and A1 both achieve good performance for the codebook switching use case at all UE speeds.

Observation 12: Alternative A1 has much higher computational complexity compared to that of B for while delivering similar performance.

Observation 13: Considering UE implementation, feedback overhead, and commonality with Mode 2, Alt 1 is a preferred choice for Mode 1 codebook FD bases selection

Observation 14: FD bases selection Alt 2 does not provide a consistent performance benefit over Alt 1, even in high ISD scenarios.

Observation 15: Quantization Alt 3 does not provide consistent performance benefit over Alt 1, even in high ISD scenarios.

Observation 16: Given the three candidate values for Ln and the previous agreement to support dynamic TRP selection by the UE, the list of combinations for {L1, ..., LN} and N=1,…NTRP would be prohibitively high for specifying a table.

Observation 17: Extensive simulations in different scenarios under various ISDs are required to arrive at a generic, reduced number of combinations of {L1, ..., LN}.

Observation 18: Performance-overhead of mTRP CJT highly depends on the specific combinations of {L1, ..., LN} and (pv,β), implying a link between them.

Proposal 1: For association of X=1 CQI, support Alt 2B, where the CQI is associated with the first/earliest d slots of the CSI reporting window and the first/earliest one of the N4 W2 matrices

Proposal 2: For the Rel-18 Type-II codebook refinement for high/medium velocities, investigate methods to reduce CQI feedback overhead for X>1 time-domain CQIs

Proposal 3: For the Rel-18 Type-II codebook refinement for high/medium velocities, reduce multiple CQI feedback overhead using the concepts of reference CQI and differential CQIs

Proposal 4: For the Rel-18 Type-II codebook refinement for high/medium velocities, aid UCI omission of multiple time domain CQIs by including reference CQI(s) and certain differential CQI(s) in CSI Part 1 and including remaining CQIs in CSI Part 2.

Proposal 5: For the Type-II codebook refinement for high/medium velocities, do not support N4=16,32.

Proposal 6: For the Type-II codebook refinement for high/medium velocities, support d= CSI-RS periodicity.

Proposal 7: For the Type-II codebook refinement for high/medium velocities, do not support additional values of Q other than Q=2.

Proposal 8: For the Type-II codebook refinement for high/medium velocities, support NZC indication by Alt 3A, wherein S delay-Doppler pairs are selected first, and 2LS NZCs are selected for feedback subsequently.

Proposal 9: For the Type-II codebook refinement for high/medium velocities, do not support K>10.

Proposal 10: For the Type-II codebook refinement for high/medium velocities, support δ=1,3.

Proposal 11: For TRS-based TDCP calculation, TRS bursts should allow autocorrelation estimation with time lag periodicity upper bounded by c2vmaxfc, where vmax is the maximum considered UE speed, fc is the carrier frequency and c is the speed of light.

Proposal 12: For TDCP reporting, use Alt. B (autocorrelation-based reporting) as described in R1-2210523.

Proposal 13: On the Type-II codebook refinement for CJT mTRP, for mode-1, support Alt 1.

Proposal 14: For Rel-18 Type II codebook for CJT, revert the working assumption on quantization Alt 3.

Proposal 15: Regarding parameter combination framework for Rel-18 Type II codebook for mTRP CJT, further study is needed

- To arrive a generic, reduced set of combinations of {L1, ..., LN} for potential specification

- To specify a linkage between the combinations of {L1, ..., LN} and (pv,β).

- To specify a list of parameter combinations in increasing order of performance-overhead.

Proposal 16: To limit UE complexity of selecting the best SD basis combination, we prefer NL≤4.

# References

- RP-213598, “MIMO evolution for downlink and uplink,” Samsung.

- Chair’s notes, RAN1#110, Toulouse, France, August 22nd – 26th, 2022.

- Chair’s notes, RAN1#110bis-e, e-Meeting, October 10th – 19th, 2022.

# Appendix I

For TDCP reporting, we conduct more LLS experiments using other configurations to diversify our TDCP study. The results of these experiments are also in line with out earlier findings.

For these experiments, we use the CDL-A channel model. We set the carrier frequency at fc=3.5 GHz and use TRS with periodicity of 2 slots. We run a preliminary simulation that helps us choose appropriate threshold values for all three TDCP metrics. We then run another set of simulations to compare the achieved performance under each of the TDCP metric alternatives. In the following figure, we use NTRS=8 bursts and we plot the relative performance gain compared to a pure Type I codebook scenario.

Figure 12. TDCP metric comparison. % Throughput gain over Type I (8 TRS bursts)

We can see that alternatives A1 and B have much better performance at low UE speed, which is a result of their better ability to identify low speed situations and react accordingly for a better codebook choice. On the other hand, alternative A2 fails to identify such low speed situations and thus is almost always stuck with Type I. The reason behind alternative A2’s shortcoming is the low doppler (speed) resolution, which results in a high probability for the doppler spectrum to be with the two strongest peaks occurring at adjacent doppler shift values. We also see that alternative B performs better than alternative A1 at high speeds. This fact, together with its simplicity compared to A1 makes it a significantly better alternative.

Owing to the simplicity of alternative B in terms of its reliance on a single positive lag, we run another experiment with similar configurations, except this time we use NTRS=2 bursts. This results in the following performance (note that we updated the threshold values based on this new number of TRS bursts).

Figure 13 TDCP metric comparison. % Throughput gain over Type I (2 TRS bursts)

The figure above shows that alternative B was largely not affected by the smaller number of TRS bursts, except at 60 Km/h where it got slightly better, while on the other hand alternative A1 becomes worse.

# Appendix II

SLS assumptions for CSI enhancement of high/medium UE velocities

| Parameter |  | Value |
| --- | --- | --- |
| Scenario |  | Urban macro (UMa) |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Frequency Range |  | FR1 only, 2 GHz. |
| Inter-BS distance |  | 200 m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25 m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9 dB |
| Modulation |  | Up to 256 QAM |
| Numerology | Slot/non-slot | 14 OFDM symbols per slot |
|  | SCS | 15 kHz |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU MIMO with rank adaptation |
| MIMO layers |  | Maximum MU layers 8 |
| CSI feedback |  | CSI feedback periodicity: 5 slots (baseline) Scheduling delay: 4 ms (baseline) |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% |
| UE distribution |  | 100% outdoor |
| UE receiver |  | MMSE-IRC |
| Spatial consistency mobility procedure |  | Disabled |

# Appendix III

SLS assumptions for CSI enhancement of coherent JT

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Outdoor1: Dense urban macro with 4 intra-cell TRPs |
| Frequency Range |  | 2 GHz |
| Inter-site distance |  | 200 m |
| Channel model |  | Based on TR 38.901. Difference in propagation delays between UE and NTRP TRPs is taken into account in the composite Channel Impulse Response (CIR) |
| Antenna setup and port layouts at each TRP |  | 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1) |
| BS Tx power |  | 44 dBm |
| BS antenna height |  | 25 m |
| UE antenna height & gain |  | According to TR36.873 |
| UE receiver noise figure |  | 9 dB |
| Modulation |  | Up to 256QAM |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO adaptation with up to rank 2 |
| CSI feedback |  | CSI feedback periodicity: 5 ms Scheduling delay: 4 ms |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% for MU-MIMO 20 % for SU-MIMO |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Baseline for performance evaluation |  | Single TRP Rel-16 eTypeII |