3GPP TSG RAN WG1 #112 	                                                         R1-2301620 Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.17

Source: 	MediaTek Inc.

Title: 	R18 TEI on BWP without CD-SSB for normal UE

Document for:	Discussion and decision

# Introduction

This contribution discusses the resolution of the BWP without CD-SSB for normal UE, and specifically proposes to specify both “Option C” and “Option B-1-1 without early implementability” as optional features under the Rel-18 TEI, with Option C (NCD-SSB approach) work led by RAN1 and Option B-1-1 work led by RAN2.

# Discussion

## Background and status

It has been discussed for several meetings in both WGs and RAN plenaries about how to support FG6-1a, i.e., BWP operation without CD-SSB, by normal (i.e., non-RedCap) UEs. At RAN#98e, in the final round of discussion, the moderator asked the following questions [1]:

- Can RAN agree that we specify: - both "B-1-1 without early implementability" and C, both as optional features, in Rel 18? - B-1-1 and C are as described in the RAN 4 LS in RP-222725=R4-2220437 - If yes, how to handle the subsequent (small) work?

In Question 1), the proposal with Option B-1-1 and Option C received significant support at the meeting. Therefore, we think TEI proposals are perfect answers to Question 2) for handling subsequent (small) work.

- Option B-1-1: Using larger BW covering SSB outside active BWP without interruptions

- Option C: NCD-SSB approach which would work with existing UE hardware architectures (FG6-1) and be compatible with existing RAN4 specifications for BM/RLM/BFD

For Option B-1-1, the main design change is about UE capability design which is under RAN2’s scope. Hence, in the following section, we focus on Option C (NCD-SSB approach) which we think should be led by RAN1.

## NCD-SSB (Option C) rationale

The concept of Non-cell-defining SSB blocks (NCD-SSB) was introduced in NR Rel-15, but was not specified to be configured to the UE until Rel-17 RedCap was introduced. There is a high commercial interest and rapid deployment plan expected for RedCap, and we expect NCD-SSB to be supported in the field by both RedCap UEs and network infrastructure. Though NCD-SSB is configured to RedCap UEs via dedicated per-UE BWP configuration, NCD-SSB is actually deployed and operated in a per-cell manner. Therefore, once a network has activated NCD-SSB for RedCap UEs in a cell, it is very reasonable to enable normal UEs to also use it.

With a normal UE’s support for FG6-1a, the following combined benefits are identified:

- The network has scheduling flexibility to configure such a UE to operate with a narrowband BWP in any part of the channel (that does not contain CD-SSB) and avoids congestion to the RBs around the CD-SSB.

- Simultaneously, UE power saving gain can be achieved from the network operating a narrowband BWP for this UE.

This combination of benefits allows further gain compared to the operation of a UE only supporting FG6-1, as it no longer requires the network to configure the UE with a wideband BWP if the network wants to allocate the UE to only use Resource Blocks far from the RBs of the CD-SSB.

In addition, as agreed for RedCap UEs (see below), we propose that a normal UE may be configured with multiple NCD-SSBs provided that for each BWP the UE is configured with only one SSB (CD-SSB or NCD-SSB).

RAN2 #117e 1. A RedCap UE may be configured with multiple NCD-SSBs provided that each BWP is configured with at most one SSB

# Conclusion

We propose the following:

Proposal 1: Specify both of the following as Rel-18 TEI as optional features:

- Extension of NCD-SSB applicability to “normal” (non-RedCap) UEs, a.k.a Option C. The work on the details should be led by RAN1.

- Complementary to the above, support Option B-1-1 without early implementability. The work on the details should be led by RAN2.

Proposal 2: For Option C, a normal UE may be configured with multiple NCD-SSBs provided that for each BWP the UE is configured with only one SSB (CD-SSB or NCD-SSB).

# Reference

- RP-223506, “Summary for [98e-30-BWP-WithoutRestriction],” Vodafone, RAN #98e, December 2022