3GPP TSG RAN WG1 #112	 R1-2301724

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:       9.5.2

Source:	IIT Kanpur, CEWiT

Title:	Discussion on solutions for NR carrier phase positioning

Document for:     Discussion and Decision

# Introduction

In Rel-16 native NR, positioning support was standardized, and in Rel-17, enhancements were made. At RAN#94, a new SI “Study on expanded and improved NR positioning” was approved for enhancements for Rel-18 NR positioning. One of the objectives of this enhancement was to introduce the carrier phase-based positioning method for further accurate positioning realizing in sub meters.

The study was concluded in the last meeting RAN1#111, and based on the agreements of the study, the following work item description was endorsed in RAN#98e [1] for normative work from carrier phase-based positioning perspective.

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4].

  - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements.

  - Specify measurements that are limited to a single carrier/PFL.

  - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

This contribution provides our view on physical layer measurements and signaling to support NR DL and UL carrier phase positioning for Rel-18 NR positioning.

# Discussion

## Subcarrier level Phase measurement

In the study, different carrier phase measurement techniques were discussed and evaluated. During study it is found that for standalone CPP, the difference between the phase of subcarriers helps in the swift resolution of integer ambiguity. As it is well understood that phase is a sensitive measurement, reporting the carrier phase of multiple subcarriers will provide robustness against receiver noise by performing noise averaging. The following potential solutions were captured in TR.859[2].

The potential solutions of integer ambiguity resolution for NR carrier phase positioning were investigated in the study item, which include the following:

-	Reporting of the carrier phases of more than one frequency from UE/TRP to LMF

-	NOTE: frequency refers to frequency of carrier or frequency of subcarrier(s)

-	Reporting of the determined integer ambiguity and/or the search range of the integer ambiguity from UE/TRP to LMF

-	Reporting of the carrier phase measurements together with the legacy positioning measurements from UE/TRP to LMF

-	Reporting of the new measurements from UE /TRP to LMF, e.g., based on carrier phase differentials across multiple subcarriers within a carrier

-	NOTE: carrier phase differentials across multiple subcarriers within a carrier can be equivalent to time of arrival

-	LMF configure the integer ambiguity range between the TRP and target UE (for UE-based NR CPP).

Based on our study, along with reporting the phase of the RF carrier or single subcarrier, the reporting of the phase of multiple subcarriers and the phase difference between the subcarriers is important and should also be supported as one of the CPP measurements.

Proposal 1: For carrier-phase based positioning, the phase of the RF carrier or single subcarrier should be reported.

Observation 1: Multiple subcarriers will provide robustness against receiver noise by performing noise averaging.

Observation 2: The difference between the carrier phase of multiple subcarriers helps in swift integer ambiguity resolution.

Proposal 2: For carrier-phase based positioning, the difference between the carrier phase of multiple subcarriers should be reported.

Proposal 3: For carrier-phase based positioning, phase measurement over multiple subcarriers should be reported to help mitigate the effects of noise.

## Carrier Phase measurement with respect to Tx and Rx antenna

Multipath and the NLOS dominant channel inherently deteriorate the positioning performance. It is clearly observed with simulations study that the mitigation of the effect of the multipath improves the performance of positioning methods.

Further, the AOD and AOA measurements over PRS either in DL or UL direction are of great help in identifying the LOS and NLOS signals and mitigate the measurements with NLOS bias. Carrier phase measurements across antennas can give the angle information at the receiver.  Further if the angle of departure w.r.t. to transmitter is known, using the measured angle of arrival at the receiver can help in defining LOS/NLOS confidence parameter. E.g. in DL-AOD based positioning measurements, along with RSRP per beam if the UE is allowed to report carrier phase measurement across each receiving antenna then it can be used to estimate the DL-AOA to with fair accuracy to decide the LOS or NLOS nature of the path experienced by the PRS between gNB/TRP and UE. During the study phase, the benefits of reporting the phase measurements across TRP Rx antennas were discussed. Therefore, reporting of the phase measurements across UE/TRP Rx, and TRP Rx antennas should be supported in Rel 18.

Observation 3: To achieve the desired accuracy in carrier phase-based positioning, multipath mitigation techniques are necessary.

Observation 4: The AOD and AOA measurements with respect to either in DL or UL link will be important to mitigating multipath.

Proposal 4: In Rel 18, for the legacy positioning methods, additional reporting of carrier phase measurements per Rx antennas should be supported.

Further, UEs or TRPs should indicate the LOS-NLOS indicator corresponding to the carrier phase measurements. At least reporting of the carrier phase of the first path, and optionally, the additional paths should be supported Rel 17 framework could be used starting point for the discussion.

The following potential solutions for multipath mitigation were captured in TR.859[2].

Multipath mitigation methods for the carrier phase positioning are recommended to be introduced during normative work, if NR CPP is introduced. The candidate solutions may include, but are not limited to, the following:

-	Reporting of the carrier phase of the first path

-	At least reporting of the carrier phase of the first path, and optionally, the additional paths.

-	The use of LOS/NLOS indication for the carrier phase measurements.

-	NOTE: Rel-17 LOS/NLOS indicator can be considered as a starting point.

-	Reporting of other channel information together with carrier phase measurements, such as existing RSRP/RSRPP.

Proposal 5: For carrier phase-based positioning, reporting of LOS-NLOS indication along with carrier phase measurement should be supported.

Proposal 6: For carrier phase-based positioning, reporting of carrier phase measurement corresponding to the first path should be supported. The measurement corresponding to the additional path could be optionally supported.

## Use of Carrier Phase measurement for legacy measurements

The carrier phase-based positioning method is viewed as a standalone as well as a non-standalone method. In the case of the non-standalone method, the carrier phase will be reported along with the UEs/TRPs measurement of legacy positioning methods e.g., DL/UL-TDOA, M-RTT, DL-AOD, UL-AOA, etc. Once we report the carrier phase along with the legacy measurement, it will inherently act as smoothing of legacy measurement. Therefore, this should be supported.

Proposal 7: In Rel 18, carrier phase positioning using carrier phase information for smoothing existing timing measurements like DL-TDOA, UL-TDOA, and Multi-RTT should be supported for UE-assisted and UE-based positioning.

# Conclusion

This paper provides the following observations and proposals.

Proposal 1: For carrier-phase based positioning, the phase of the RF carrier or single subcarrier should be reported.

Proposal 2: For carrier-phase based positioning, the difference between the carrier phase of multiple subcarriers should be reported.

Proposal 3: For carrier-phase based positioning, phase measurement over multiple subcarriers should be reported to help mitigate the effects of noise.

Proposal 4: In Rel 18, for the legacy positioning methods, additional reporting of carrier phase measurements per Rx antennas should be supported.

Proposal 5: For carrier phase-based positioning, reporting of LOS-NLOS indication along with carrier phase measurement should be supported.

Proposal 6: For carrier phase-based positioning, reporting of carrier phase measurement corresponding to the first path should be supported. The measurement corresponding to the additional path could be optionally supported.

Proposal 7: In Rel 18, carrier phase positioning using carrier phase information for smoothing existing timing measurements like DL-TDOA, UL-TDOA, and Multi-RTT should be supported for UE-assisted and UE-based positioning.

Observation 1: Multiple subcarriers will provide robustness against receiver noise by performing noise averaging.

Observation 2: The difference between the carrier phase of multiple subcarriers helps in swift integer ambiguity resolution.

Observation 3: To achieve the desired accuracy in carrier phase-based positioning, multipath mitigation techniques are necessary.

Observation 4: The AOD and AOA measurements with respect to either in DL or UL link will be important to mitigating multipath.

# References

- RP-223549 New WID on Expanded and Improved NR Positioning

- TR 38.859 V0.3.0 (2022-11) Study on Expanded and Improved NR Positioning; (Release 18)

- RAN1-110 bis e-meeting chairman notes

- RAN1-111 meeting chairman notes