3GPP TSG-RAN WG1 Meeting #112	R1-2301760

# Athens, Greece, February 27 –  March 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.214 | CR | xxxx | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the parameter name alignment for SRS in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The availableSlotOffset-based triggering mechanism for aperiodic SRS was introduced in Rel-17, the enabling condition of which is the existence of specific higher layer parameter in the configuration of at least one SRS resource set. However, the aforementioned specific higher layer parameter is not aligned in 38.214, where both AvailableSlotOffset and availableSlotOffsetList appear in the enabling condition of availableSlotOffset-based triggering mechanism, which may cause misunderstanding. Since AvailableSlotOffset is the entry of availableSlotOffsetList according to 38.331, it’s more appropriate to unify the higher layer parameter in enabling condition to be availableSlotOffsetList. In addition, there is a typo in section 6.2.1.2 that should be modified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To unify the higher layer parameter in the enabling condition of availableSlotOffset-based triggering mechanism to be availableSlotOffsetList. To modify the typo in section 6.2.1.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The higher layer parameter in the enabling condition of availableSlotOffset-based triggering mechanism is not aligned, which may cause misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: No inter-operatability issue is identified. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.2.1	UE sounding procedure

========================= Unchanged parts =========================

-	If the UE receives the DCI triggering aperiodic SRS in slot n and at least one resource set is configured with parameter availableSlotOffsetList across all configured BWPs in a component carrier except when SRS is configured with the higher layer parameter SRS-PosResource,

-	If ca-SlotOffset is configured, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot ,

-	otherwise the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in the (t + 1)-th available slot counting from slot n⋅2μSRS2μPDCCH+k, where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command, respectively;

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot, offset, SRSCA and μoffset,SRS are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. From the first symbol carrying the SRS request DCI to the last symbol of the triggered SRS resource set, UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of available slot.

-	t is configured via higher layer parameter availableSlotOffsetList with up to four different values of AvailableSlotOffset for each triggered SRS resources set and it is based on the subcarrier spacing of the triggered SRS transmission. When one or more SRS resource sets across all configured BWPs in a component carrier are configured, and at least one resource set is configured with availableSlotOffsetList parameter of more than one values, the indicated value of t is indicated by SOI field in DCI scheduling PUSCH/PDSCH and DCI 0_1/0_2 without data and without CSI request described in [5, TS 38.212]. The UE shall apply indicated value t specifically for those sets with configured availableSlotOffsetList parameter. When one or more SRS resource sets across all configured BWPs in a component carrier are configured and at least one resource set is configured with availableSlotOffsetList parameter, and the availableSlotOffsetList parameter for each SRS resource set has only one value, the UE shall apply the configured value specifically for those sets with configured availableSlotOffsetList parameter. For SRS resource set configured with availableSlotOffsetList parameter, each of resource set is configured with K values of AvailableSlotOffset. For SRS resource set configured without availableSlotOffsetList parameter, t = 0 is applied for the resource set.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and none of the resource sets is configured with parameter availableSlotOffsetList across all configured BWPs in a component carrier except when SRS is configured with the higher layer parameter SRS-PosResource

-	if the UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot ,

========================= Unchanged parts =========================

6.2.1.2	UE sounding procedure for DL CSI acquisition

========================= Unchanged parts =========================

-	For 1T4R, if the UE is indicating srs-AntennaSwitching2SP-1Periodic and/or srs-ExtensionAperiodicSRS and/or srs-OneAP-SRS,

-	zero or one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of each resource is associated with a different UE antenna port, and

-	zero or two SRS resource sets each configured with resourceType in SRS-ResourceSet set to 'aperiodic', if the UE is not indicating srs-ExtensionAperiodicSRS or srs-OneAP-SRS, or zero or one or two or four SRS resource sets each configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating srs-ExtensionAperiodicSRS and srs-OneAP-SRS, or zero or two or four SRS resource sets each configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating srs-ExtensionAperiodicSRS only, or zero or one or two SRS resource sets each configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating srs-OneAP-SRS only. In the case of one resource set, a total of four SRS resources are transmitted in different symbols in the same slot, and the SRS port of each resource in the given set is associated with a different UE antenna port. In the case of two resource sets, a total of four SRS resources are transmitted in different symbols of two different slots, and the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. The two sets are each configured with two SRS resources, or one set is configured with one SRS resource and the other set is configured with three SRS resources. In the case of four resource sets, if UE is capable of supporting four sets, a total of four SRS resources are transmitted in different symbols of four different slots, and the SRS port of each SRS resource in the given four sets is associated with a different UE antenna port. The four sets are each configured with one SRS resource. The UE shall expect that the two or four sets are both configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet. The UE shall expect that the value of the higher layer parameter aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet is the same, and the value of the higher layer parameter slotOffset in each SRS-ResourceSet is different. Or,

========================= Unchanged parts =========================