3GPP TSG RAN WG1 #112				                           R1-2301774

Athens, Greece, February 27th – March 3rd, 2023

Source:	Moderator (NTT DOCOMO)

Title:	FL summary on DMRS#1

Agenda item:	9.1.3.1

Document for:	Discussion and Decision

- Introduction

In RAN#94-e meeting, a new Rel-18 WID on MIMO [1] was agreed. From 7 objectives, there are two objectives for DMRS enhancements, as shown below.

- Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM, - Striving for a common design between DL and UL DMRS - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS […] - Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

This document contains summary of the company’s tdocs and FL proposals.

- Objective #3 (increasing DMRS ports)

  - Antenna ports table for PDSCH

2.1.1 eType1, maxLength1

In RAN1#111, following agreement was made.

Agreement - For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data. - FFS: Antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for M-TRP case. Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1 One Codeword: Codeword 0 enabled, Codeword 1 disabled; Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Notes; Value; Number of DMRS CDM group(s) without data; DMRS port(s); Notes. 0; [1]; [0]; Cat. 1; [0]; [2]; [0,1,2,3,8]; [Rank 5-8 with one DMRS symbol]. 1; [1]; [1]; [1]; [2]; [0,1,2,3,8,10]. 2; [1]; [0,1]; [2]; [2]; [0,1,2,3,8,9,10]. 3; 2; 0; [3]; [2]; [0,1,2,3,8,9,10,11]. 4; 2; 1. 5; 2; 2. 6; 2; 3. 7; 2; 0,1. 8; 2; 2,3. 9; [2]; [0-2]. 10; [2]; [0-3]. 11; [2]; [0,2]. 12; [1]; [8]; Cat.2. 13; [1]; [9]. 14; [1]; [8,9]. 15; 2; 8. 16; 2; 9. 17; 2; 10. 18; 2; 11. 19; 2; 8,9. 20; 2; 10,11. 21; [2]; [8-10]. 22; [2]; [8-11]. 23; [2]; [8, 10], [9, 11]. 24; [1]; [0,1,8]; Cat.3. 25; [1]; [0,1,8,9]. 26; 2; 0,1,8. 27; 2; 0,1,8,9. 28; 2; 2,3,10. 29; 2; 2,3,10,11.

In this meeting, we need to discuss which rows should be supported/updated/deleted in the above table.

Some companies propose to remove the rows to avoid increasing the DCI overhead of antenna ports field. However, supporting rows doesn’t automatically mean to increase the DCI size. Whether to increase/keep the DCI size can be discussed in Sect. 2.2.

For at least for S-TRP

FL Proposal 2.1.1A

- For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support the following:

  - For 1 CW,

    - 1) For row 0-2, 12-14, 24-25 (rows with Number of DMRS CDM group(s) without data = 1),

      - Alt.1-1: Support the rows.

      - Alt.1-3: Remove the rows.

    - 2) For row 9-11, 21-22 (rank 3-4 in Cat.1-2, except for row 23),

      - Alt.2-1: Support the rows.

      - Alt.2-2: Modify values in the rows.

      - Alt.2-3: Remove the rows.

    - 3) For row 23 (e.g. To enable 2+2+3 layers in a CDM group),

      - Alt.3-1: Support the rows.

      - Alt.3-2: Modify values in the rows.

      - Alt.2-3: Remove the rows.

    - 4) Additionally include more rows

      - {0,1,2,8,9} for CJT/NCJT

  - 5) For 2 CWs (row 0-3),

      - Alt.3-1: Support the rows.

      - Alt.3-2: Modify values in the rows (e.g. Table 4-1/4-2 below).

FL: Following rows are relatively stable. Let’s try the following rows firstly.

FL Proposal 2.1.1B

- For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support at least support the following rows:

  - For 1 CW,

    - 1) Row 0-2, 12-14, 24-25 (rows with Number of DMRS CDM group(s) without data = 1)

Support (24): HW/HiSilicon, New H3C, Docomo, ZTE Spreadtrum, vivo, Lenovo, LG, CATT, Ericsson, Intel, CMCC, QC, Nokia/NSB, OPPO, Apple, Google, FW, MTK, China Telecom, Sharp (fine with majority), Spreadtrum

Concern (1): Samsung (target of R18 DMRS is for MU-MIMO)

For rows of 2CW, no company propose to delete. Qualcomm [21] mentions that small modification is needed considering the existing CW to layer mapping. ZTE/vivo also propose some update.

For example, with 5 layers PDSCH, current proposal will map the two layers of the first CW to DMRS ports {0,1}, which is fine. But it will map the 3 layers of the second CW to DMRS ports {2,3,8}, which is not preferred because the second CW is distributed on two CDM group. Fig 6: Example of layer to DMRS ports mapping for type 1 single-symbol DMRS [21]

Table 4-1: Modified values for 2 CWs (QC).

| Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 2 | 0,1,2,3,8,10 |
| 1 | 2 | 0,1,8,2,3,8,10 |
| 2 | 2 | 0,1,8,2,3,8,9,10,11 |
| 3 | 2 | 0,1,8,9,2,3,8,9,10,11 |

Table 4-2: Modified values for 2 CWs (ZTE).

| Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 2 | 0,1,2,8,9 |
| 1 | 2 | 0,1,2,3,8,9 |
| 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0,1,2,3,8,9,10,11 |

Table 4-4: Modified values for 2 CWs (Google).

| Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 1 | 2 | 0,8,9,10,11 |
| 1 | 2 | 1,8,9,10,11 |
| 1 | 2 | 2,8,9,10,11 |
| 1 | 2 | 3,8,9,10,11 |
| 1 | 2 | 0,1,8,9,10,11 |
| 1 | 2 | 2,3,8,9,10,11 |
| 1 | 2 | 0,1,2,8,9,10,11 |

Table. Summary of companies’ views for FL Proposal 2.1.1A

|  | Alt.x-1: Support the row(s) | Alt.x-2: Modify values in the rows. | Alt.x-3: Remove the rows. |
| --- | --- | --- | --- |
| 1) Row 0-2, 12-14, 24-25 for 1CW | HW/Hi, New H3C, Docomo, ZTE Spreadtrum, vivo, Lenovo, LG, CATT, Ericsson, Intel, CMCC, QC, Nokia/NSB, OPPO, Apple, Google, FW, MTK, China Telecom |  | Sharp (five with majority), Samsung |
| 2) Row 9-11, 21-22 for 1CW | HW/Hi, New H3C, ZTE, Docomo Spreadtrum, vivo(Support 9-11, but not support 21-22), Lenovo, LG, CATT, Ericsson, Intel, CMCC(remove 21-22) , Nokia/NSB, Apple (rows 21-22 can be removed) , FW, MTK, China Telecom | OPPO: Row 21, 22 is not needed. Google: change [0,1,8] into [0,8,9] | Sharp (fine with majority) , Samsung, QC, HW/Hi |
| 3) Row 23 for 1CW | HW/Hi(9,11), New H3C, Spreadtrum, , Lenovo, LG, CATT, Ericsson,MediaTek (9,11) , Nokia/NSB(9,11), FW(9,11) , China Telecom |  | Sharp, Docomo, ZTE, Intel, CMCC, Samsung, QC, OPPO, Apple, MTK |
| 4) {0,1,2,8,9} for CJT/NCJT | Support: Ericsson, China Telecom (can live) | - | Concern: OPPO |
| 5) Row 0-3 for 2CWs | HW/Hi,New H3C, Docomo, Spreadtrum, , Lenovo, CATT, CMCC, Nokia/NSB, , OPPO, Apple, FW | Support Table 4-1: QC, Docomo Support Table 4-2: ZTE, China Telecom (but fine with majority) Support Table 4-4: Google |  |

For S-DCI based M-TRP

In Rel.17, separate DMRS port table is specified for the case of S-DCI based M-TRP and the case of others. Whether we need separate table for S-DCI based M-TRP and others can be discussed.

FL Proposal 2.1.1B

- For the antenna ports indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH,

  - Alt.1: Separate DMRS port tables are specified for S-DCI based M-TRP and others.

  - Alt.2: Common DMRS port table is specified for S-DCI based M-TRP and others.

Also, for S-DCI based M-TRP, which DMRS ports combinations is needed can be discussed.

FL Proposal 2.1.1C

- For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-DCI based M-TRP, support at least the following row(s):

  - For one CW, row 30-31 in the following table.

Table 7.3.1.2.2-1A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| … | … | … |
| 30 | 2 | 0,2,3 |
| 31 | 2 | 8,10,11 |

Table. Summary of companies’ views for FL Proposal 2.1.1B/2.1.1C

|  | Support/fine | Concern |
| --- | --- | --- |
| 2.1.1B | Alt.1: Docomo, QC? Alt.2: LG, CATT, OPPO, CMCC |  |
| 2.1.1B | Row30 only: LG, QC? OPPO, CMCC Row31only: Both row30/31: CATT, Docomo |  |

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | FL Proposal 2-3A: Support. 1) Support Alt.1-1. Even for MU-MIMO scenario, gNB cannot always find good MU-MIMO UE pair, and gNB dynamically switches SU-MIMO and MU-MIMO for each scheduled PDSCH. Hence, it is better to keep the rows. 2) We don’t think these rows are useful for S-TRP case, but fine to keep the rows. 3) We think MU-MIMO of 2+2+3 layers are too complicated and we don’t see usecase. 5) Support the rows. Also, we can support Table 4-1, which is simple extension, and it allows 1 CW to map to 1 CDM group. The number of DMRS ports for different CDM groups is balanced in this case. We don’t support Table 4-2 because the number of DMRS ports for different CDM groups is not balanced. FL Proposal 2-3B: Support. We slightly prefer table because rows for S-DCI based M-TRP is not useful for other cases. FL Proposal 2-3C: Support. We are fine to have either separate tables for S-TRP and M-TRP, or common table for S-TRP and M-TRP. |
| Google | FL proposal 2.1.1A: We suggest removing row 23 for single CW. For row 24 and 26, we suggest changing [0,1,8] into [0,8,9]. For 2 CW, we suggest removing all the rows except the last one, and adding two rows: [1]; [2]; [0,8,9,10,11]. [1]; [2]; [1,8,9,10,11]. [1]; [2]; [2,8,9,10,11]. [1]; [2]; [3,8,9,10,11]. [1]; [2]; [0,1,8,9,10,11]. [1]; [2]; [2,3,8,9,10,11]. [1]; [2]; [0,1,2,8,9,10,11]. The reason is that there are more legacy UEs. Therefore, we can consider to reserve more ports to the legacy UEs for MU-MIMO. FL proposal 2.1.1B: Support Alt2, which is based on the same principle as legacy spec. We can define some statements on potential restriction for some code-point if necessary. FL proposal 2.1.1C: We can discuss previous DMRS table first to see whether there is enough room for adding such rows. There seems to be no strong motivation to add such rows. |
| OPPO | FL proposal 2.1.1A: We have shown our position in above tables. For Row 23, we cannot see the valid use case. For {0,1,2,8,9}, we don’t think the transmission (e.g. 1+4 layers) is supported by coherent JT or NC-JT. FL proposal 2.1.1B: Prefer Alt.2 which can simplify the specification. The combination for NC-JT can only be applied when two TCI states are configured, as legacy. FL proposal 2.1.1C: Support Row 30 only. What is the use case of Row 31 for NC-JT when Row 30 has been supported? |
| ZTE | FL Proposal 2.1.1 A: - For 1CW, we are supportive all rows so far due to it is good to guarantee the use cases as much as possible, e.g., CJT/NCJT. Regarding row 23, we are open to it as long as the practical use case (e.g., 3+3+2 layer combination in MU-MIMO) can be acceptable to companies. If [9, 11] was valid, [3, 11] should be adopted additionally once [0-2] and [8-10] can be supported, this combination can be used for 3+3+2 in MU-MIMO as well. - For 2CW, we think it is better to support utilize DMRS ports from one CDM group at first (as shown in Table 4-2) to save CDM groups for other co-scheduled UEs, if any. FL Proposal 2.1.1 B: Support Alt 1, which is inline with the DMRS indication tables in the current specification for STRP and MTRP respectively. E.g., tables entitled with “-xA” are used for MTRP SDM PDSCH transmission. FL Proposal 2.1.1 C: Support in principle. It is sufficient to adopt [0, 2, 3] for SDCI MTRP operation. |
| CMCC | FL proposal 2.1.1A: For port combinations [8-10], [8-11], and [8,10], we think they are redundant to legacy port combinations [0-2], [0-3], and [0,2], so row 21-23 can be removed. FL Proposal 2.1.1B: Support Alt2 as legacy scheme. FL Proposal 2.1.1C: Support row 30 only. |
| CATT | FL Proposal 2.1.1 A: Support the rows for all cases. Besides, we are open to Alt.2 (i.e., modify values in the rows) for 2CWs. FL Proposal 2.1.1 B: Support Alt.2. Entries for SU-MIMO/MU-MIMO and other scenarios or features (e.g., M-TRP, UL 8Tx, STxMP, CJT, etc.) can be included in one table to increase the scheduling flexibility of gNB. FL Proposal 2.1.1 C: Support. |
| Lenovo | FL Proposal 2.1.1A: Support. 1) Support Alt.1-1, Alt.2-1, Alt.3-1 for better scheduling flexibility. 2) More discussion and justification are needed for Alt.4 3) Open for discussion but prefer Alt.5-1 for less standard impact. FL Proposal 2.1.1B: Support. We slightly prefer Alt.1 to align with specification for legacy DMRS port indication table separate for single TRP and multiple TRP case. FL Proposal 2.1.1C: Support from view of flexibility. |
| Sharp | FL Proposal 2.1.1 A: We can go with the majority. FL Proposal 2.1.1 B: Support Alt 1. FL Proposal 2.1.1. C: We are fine with the proposal. |
| Xiaomi | FL Proposal 2.1.1 A: - Generally, support. But we are afraid whether dynamic switching between legacy DMRS and R18 DMRS is supported might has impact on this issue. We notice that in section 2.4, some companies want to support the dynamic switching between legacy DMRS and R18 DMRS. When the MU-MIMO switch to SU-MIMO, then, DMRS type can fall back to legacy DMRS. If it is support, then there is no need to include these rows with Number of DMRS CDM group(s) without data = 1 for R18 DMRS antenna port(s) table. - Support alt.2-1 - Support alt.3-1 - Not Support Why should {0,1,2,8,9} for CJT/NCJT (4+1) should be considered for one CW? We do not see any benefit to support this combination. For CJT or NCJT in MTRP, it is better to apply two CWs when (4+1) layers are considered. - Support Alt.3-2. (The modified table by QC) FL Proposal 2.1.1 B: Slightly prefer Alt.2. FL Proposal 2.1.1 C: Fine with proposal 2.1.1 C |
| Huawei, HiSilicon | FL Proposal 2.1.1A: 1) Support Alt.1-1. Can be utilized to improve the spectrum efficiency. 2) Support Alt.2-3 and our position above is updated. Cannot increase the supported layer combination. 3) Support Alt.3-1({9, 11}) and the benefit of the third sub-bullet above is clarified. Among all the layer combinations, 3+3+2 is the only one that is not supported, which can facilitate flexible scheduling. 5) Support Alt.5-1. FL Proposal 2.1.1B: Support Alt.1. No strong motivation to change the legacy way of supporting STRP and MTRP. FL Proposal 2.1.1C: Only support {0, 2, 3}. The motivation of {8, 10, 11} is unclear. |
| Fraunhofer IIS/HHI | FL Proposal 2.1.1A: 1, 2 - Support the rows. 3 - Fine with either option. 5 - Ok with the existing rows or Table 4-1. FL Proposal 2.1.1B: Prefer Alt. 2 FL Proposal 2.1.1C: Support |
| Nokia/NSB | FL Proposal 2.1.1A: 1)2)3)Fine with any options, 4) Not support. (0,1,2,8,9) should be 2 CWs, {8,9,2,3} should be equivalent to {0,1,2,3} 5) We are fine with Alt 3-1 It is not necessary to match with CW. FL Proposal 2.1.1B: Prefer Alt. 2. But we want to clarify if what the use case of Rel-18 DMRS for mTRP. Do we support MU-MIMO with mTRP? FL Proposal 2.1.1C: if we agree with MU-MIMO with mTRP, we are fine. (UE1: 0,2,3 while UE2: 8,10,11 or 8,9,10?) |
| Futurewei | FL proposal 2.1.1A: 1) Support Alt.1-1. 2) Fine with Alt.2-1. 3) Support [9, 11]. It is unclear to us the usage of [8, 10]. 5) Support the rows. FL proposal 2.1.1B: Slightly prefer Alt. 1 to follow the legacy way of supporting sTRP and mTRP. FL proposal 2.1.1C: Support Row 30. It is unclear to us the usage of Row 31. |
| Samsung | FL Proposal 2.1.1A: 1) Support Alt.1-3. 2) Support Alt.2-3. 3) Support Alt.3-3. 4) Not support as we do not see an use case. 5) For rows itself, support Alt.3-1, but as we mentioned in our tdoc, since Rel-18 DMRS is for more MU-MIMO support, we think supporting more than 4 layers is UE optional feature. FL Proposal 2.1.1B: Our view is to support one table which is enough for STRP. BTW, it seems almost every companies consider Rel-18 DMRS can be used even for M-TRP use case. Is it common understanding? |
| QC | FL Proposal 2.1.1A - For row 0-2, 12-14, 24-25 (rows with Number of DMRS CDM group(s) without data = 1), support Alt 1-1 (keep those rows) - For row 9-11, 21-22 (rank 3-4 in Cat.1-2, except for row 23), support Alt 2-3 (remove those rows, they are designed for SU performance optimization, which does not fit into the purpose of Rel-18 DMRS enh for increasing MU capacity) - For row 23 (e.g. To enable 2+2+3 layers in a CDM group), support Alt 2-3 (remove the rows) - If we understand correctly, {0,1,2,8,9} for CJT/NCJT is for M-TRP, which does not fit into this proposal for S-TRP. - Support Alt 3-2. Among the tables, we support 4-1, because it maps layers of 1 CW to 1 CDM group, which can simplify receiver. To decode a CW, UE does not need to touch the channel in other CDM group. FL Proposal 2.1.1B: support Alt 1, which follows legacy. FL proposal 2.1.1C: We prefer adding row 30 {0,2,3} only. We don’t see strong motivation to add {8,10,11}. |
| LGE | FL Proposal 2.1.1 A: 1) Support Alt.1-1, Alt.2-1, Alt.3-1 for better scheduling flexibility. The ports of indication values {0, 1, 2, 9, 10, 11} of Cat. 1 should be included in the new DMRS table because in our view new DMRS table and legacy DMRS table are switched semi-statically. Also, the ports of indication values {12, 13, 14, 21, 22, 23} of Cat. 2 should be included in the new table for MU scheduling. 2) Not support additionally include more rows of {0,1,2,8,9} for CJT/NCJT 3) We are fine with Alt 3-1 for less standard impact. FL Proposal 2.1.1 B: Support Alt.2. FL Proposal 2.1.1 C: Support only {0, 2, 3}. It is sufficient to support ports for MTRP only in Cat. 1 such as legacy. |
| Apple | FL Proposal 2.1.1 A: - For rows 0-2, 12-14, 24-25 (rows with Number of DMRS CDM group(s) without data = 1), we support Alt.1-1: Support the rows. - For rows 9-11, 21-22 (rank 3-4 in Cat.1-2, except for row 23), we suggest removing the rows 21-22. Legacy rows 9-11 can be kept - For row 23, we support Alt 3-1, i.e. to remove this row 5) For 2 CWs (row 0-3), we support Alt.3-1 FL Proposal 2.1.1B: Support Alt.2, as we don’t see any reason to have separate tables because we are designing for both single TRP and multi-TRP with enhanced DMRS type in same Rel. FL Proposal 2.1.1C: It is sufficient to support only {0, 2, 3} for M-TRP (same as legacy) |
| MediaTek | FL Proposal 2.1.1 A: - For rows 0-2, 12-14, 24-25, we support Alt.1-1: Support the rows. - For row 23 we support Alt 2-3 (remove the rows) - Support 2-3 for 2) and 3) - Not support 4) - Support Alt. 3.2 for 5) FL Proposal 2.1.1 B: Support Alt 1. FL Proposal 2.1.1. C: Only support, row 30, i.e., {0, 2, 3}. |
| Ericsson | 2.1.1A: To ensure the scheduling flexibility for a future proof antenna port table, we support Alt.1-1 for cases 1)-3). The traffic pattern in a network varies from slot to slot, the antenna port table shall be designed to server both SU and MU scenarios. It would be unfortunate if entries supported in legacy getting excluded in the Rel-18 antenna port table. On case 4) for the combination {0,1,2,8,9} proposed by us, the thinking is to use it for two CW, or for C-JT. According to the discussion so far, maybe this entry for C-JT is still premature to add. For case 5), we are fine with the principle of table 4-1, to decode one CW within 1 CDM group. On the other hand we think the proposed combination is an optimization coupled with UE implementation or capability, we don’t see why such bundling is necessary for all UE/network implementations, other combinations shall also be allowed if there’s need. 2.1.1B: Support Alt2. We can distinguish the entries for M-TRP instead of defining separate table. 2.1.1C: Support adding both entries. |
| China Telecom | FL Proposal 2.1.1 A: - For 1CW, 1) 2) 3) 5). In principle, we think all the conditions should be considered in the specs even though it can seldom happen. And For 4), we can live with it, but we can agree with the majority’s view. - For 2CW, we agree with ZTE that it is better to support utilize DMRS ports from one CDM group first (Table 4-2). However, we can also live with other tables. FL Proposal 2.1.1 B: Support Alt.1. FL Proposal 2.1.1 C: Support row 30 is enough. |
| vivo | FL Proposal 2.1.1 A: 1)Support Alt.1-1, since they can be used to provide more throughput 2)Support 9-11, but don’t support 21-22. Since 9-10, 21-22 are used for SU-MIMO, they can achieve the same effect, 9-11 is enough. 3)Support Alt.2-3. i.e., don’t support 23, this entry is used for SU-MIMO based on the legacy principle in spec 28.214. 23 is redundant, when 11 is supported. 4)Don’t support 5)Support Alt.3-1. i.e., Table 4-3. We would like to clarify that the entries in Table 4-3 are the same as those in the previous agreement. FL Proposal 2.1.1 B: Prefer Alt.1. In the current spec, separate tables are specified for S-DCI MTRP. FL Proposal 2.1.1 C: 30 and 31 are both for SU-MIMO, support of 30 is enough. |
| Spreadtrum | FL Proposal 2.1.1 A: For 1 CW and 2 CWs, we support all rows to achieve scheduling flexibility. FL Proposal 2.1.1 B: We prefer Alt.1. FL Proposal 2.1.1 C: Support. |

2.1.2 eType1, maxLength2

Goal of RAN1#112 for eType1, maxLength2 is to agree some DMRS port combinations with bracket, similar as RAN1#111 agreement. I added [ ] to controversial rows. Please check whether rows without [ ] are acceptable. The rows with [ ] will be discussed after they are agreed for eType1, maxLength1.

At least for S-TRP case

Table 7.3.1.2.2-2-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| [0 | 1 | 0 | 1] | 0 | 2 | 0-4 | 2 |
| [1 | 1 | 1 | 1] | 1 | 2 | 0,1,2,3,4,6 | 2 |
| [2 | 1 | 0,1 | 1] | 2 | 2 | 0,1,2,3,4,5,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,8 | 1 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,8,10 | 1 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,8,9,10 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| 8 | 2 | 2,3 | 1 | 8 | 1 | 0,1,4,5,8 | 2 |
| [9 | 2 | 0-2 | 1] | 9 | 1 | 0,1,4,5,8,12 | 2 |
| [10 | 2 | 0-3 | 1] | 10 | 1 | 0,1,4,5,8,9,12 | 2 |
| 11 | 2 | 0,2 | 1 | 11 | 1 | 0,1,4,5,8,9,12,13 | 2 |
| 12 | 2 | 0 | 2 | 12 | 2 | 0,1,4,5,8 | 2 |
| 13 | 2 | 1 | 2 | 13 | 2 | 0,1,4,5,8,12 | 2 |
| 14 | 2 | 2 | 2 | 14 | 2 | 0,1,4,5,8,9,12 | 2 |
| 15 | 2 | 3 | 2 | 15 | 2 | 0,1,4,5,8,9,12,13 | 2 |
| 16 | 2 | 4 | 2 | 16 | 2 | 2,3,6,7,10 | 2 |
| 17 | 2 | 5 | 2 | 17 | 2 | 2,3,6,7,10,14 | 2 |
| 18 | 2 | 6 | 2 | 18 | 2 | 2,3,6,7,10,11,14 | 2 |
| 19 | 2 | 7 | 2 | 19 | 2 | 2,3,6,7,10,11,14,15 | 2 |
| 20 | 2 | 0,1 | 2 |  |  |  |  |
| 21 | 2 | 2,3 | 2 |  |  |  |  |
| 22 | 2 | 4,5 | 2 |  |  |  |  |
| 23 | 2 | 6,7 | 2 |  |  |  |  |
| 24 | 2 | 0,4 | 2 |  |  |  |  |
| 25 | 2 | 2,6 | 2 |  |  |  |  |
| [26 | 2 | 0,1,4 | 2] |  |  |  |  |
| [27 | 2 | 2,3,6 | 2] |  |  |  |  |
| [28 | 2 | 0,1,4,5 | 2] |  |  |  |  |
| [29 | 2 | 2,3,6,7 | 2] |  |  |  |  |
| [30 | 2 | 0,2,4,6 | 2] |  |  |  |  |
| [31 | 1 | 8 | 1] |  |  |  |  |
| [32 | 1 | 9 | 1] |  |  |  |  |
| [33 | 1 | 8,9 | 1] |  |  |  |  |
| 34 | 2 | 8 | 1 |  |  |  |  |
| 35 | 2 | 9 | 1 |  |  |  |  |
| 36 | 2 | 10 | 1 |  |  |  |  |
| 37 | 2 | 11 | 1 |  |  |  |  |
| 38 | 2 | 8,9 | 1 |  |  |  |  |
| 39 | 2 | 10,11 | 1 |  |  |  |  |
| [40 | 2 | 8-10 | 1] |  |  |  |  |
| [41 | 2 | 8-11 | 1] |  |  |  |  |
| [42 | 2 | 8,10 | 1] |  |  |  |  |
| 43 | 2 | 8 | 2 |  |  |  |  |
| 44 | 2 | 9 | 2 |  |  |  |  |
| 45 | 2 | 10 | 2 |  |  |  |  |
| 46 | 2 | 11 | 2 |  |  |  |  |
| 47 | 2 | 12 | 2 |  |  |  |  |
| 48 | 2 | 13 | 2 |  |  |  |  |
| 49 | 2 | 14 | 2 |  |  |  |  |
| 50 | 2 | 15 | 2 |  |  |  |  |
| 51 | 2 | 8,9 | 2 |  |  |  |  |
| 52 | 2 | 10,11 | 2 |  |  |  |  |
| 53 | 2 | 12,13 | 2 |  |  |  |  |
| 54 | 2 | 14,15 | 2 |  |  |  |  |
| 55 | 2 | 8,12 | 2 |  |  |  |  |
| 56 | 2 | 10,14 | 2 |  |  |  |  |
| [57 | 2 | 8,9,12 | 2] |  |  |  |  |
| [58 | 2 | 10,11,14 | 2] |  |  |  |  |
| [59 | 2 | 8,9,12,13 | 2] |  |  |  |  |
| [60 | 2 | 10,11,14,15 | 2] |  |  |  |  |
| [61 | 2 | 8,10,12,14 | 2] |  |  |  |  |
| [62 | 1 | 0,1,8 | 1] |  |  |  |  |
| [63 | 1 | 0,1,8,9 | 1] |  |  |  |  |
| 64 | 2 | 0,1,8 | 1 |  |  |  |  |
| 65 | 2 | 0,1,8,9 | 1 |  |  |  |  |
| 66 | 2 | 2,3,10 | 1 |  |  |  |  |
| 67 | 2 | 2,3,10,11 | 1 |  |  |  |  |

For M-TRP case

Table 7.3.1.2.2-2A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |
| --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| … | … | … | … |
| 68 | 2 | 0,2,3 | 1 |
| 69 | 2 | 8,10,11 | 1 |

For S-TRP, please check the rows without [ ] are acceptable. For M-TRP, the same rule as eType1 maxLength1 (select 1st one, 2nd one, or both) will be discussed later.

| Company | Comment |
| --- | --- |
| Docomo | Support |
| Google | It is better to have maximum 64 rows with regard to the DMRS overhead. For single CW, we suggest removing row 26, 27, 30, 42 and not adding 68, 69. For 2 CW and row 62/64 for single CW, similar to our comment in 2.1.1, we think we should reserve more ports for legacy UEs, e.g., changing [0, 1, 8] into [0, 8, 9] |
| OPPO | We agree with google to maintain 6 bits for DMRS signaling overhead. In our understanding, there is no use case for Row 40-42 and 61, which can be deleted. Also, Row 69 for mTRP can also be removed. |
| ZTE | Further discuss it based on the outcome of eType1, maxLength1. |
| CMCC | Support maximum 64 rows. |
| CATT | Support the rows without [ ]. All the legacy port combinations can be included in the new table. |
| Lenovo | We are fine with the rows in general but would like to keep the total row number no larger than 64 for both single TRP case and multiple TRP case. |
| Sharp | We prefer removing Row indexes 26,27,28,29,30,57,58,59,60,61 in yellow parts. In our view, the benefit of the Row indexes is unclear. For example, when the number of CDM groups is 2, both {0,1,8,9} and {0,1,4,5} don’t need to be supported, in our view, {0,1,8,9} is sufficient. For 4 FD-OCC case, FD-OCC-based multiplexing should be prioritized over TD-OCC. |
| Xiaomi | We slightly prefer to remove these rows with Number of DMRS CDM group(s) without data = 1. From our understanding, maxLength=2 will be configured in the case of high load, that is, too many UEs. Otherwise, maxLength will be set to 1. Thus, these rows with Number of DMRS CDM group(s) without data = 1 could be removed. |
| Huawei, HiSilicon | For 1CW, given that the benefit of entry 24 and 25 is that only TD-OCC dispreading is adopted during channel estimation, which means other co-scheduled DMRS ports are not expected to be within the same CDM group. Following this principle, entry 55 and 56 is useless. For 2CWs, considering it is mainly used for SU scenario, it seems that entries 0~11 are enough. |
| Fraunhofer IIS/HHI | Prefer to maintain a maximum of 64 rows. At least 26, 27 and 30 can be removed. |
| Nokia/NSB | We prefer not increasing number of rows beyond 64. Don’t see use case of row 9,10,11,24,25,30, Also, 68,69 is analogous to 2.1.1.C. For two CW, we don’t see gain of having two different combinations for each rank. For DL, we don’t see gain from two DMRS symbols, because gNB is not power limited. So, support only single DMRS symbol. (row 4,5,6,7). |
| Futurewei | We shared the same view as other companies that the maximum number of rows should not exceed 64 such that the size of the antenna ports field is only increased by 1 bit. The detailed entries can be discussed after the case for eType1, maxLength = 1 is settled. |
| QC | For S-TRP 1 CW table, we don’t support row 11. The reason is row 11 is for SU performance optimization, which does not fit into Rel-18 DMRS enhancement. For rank2+other co-scheduled MU, there are a lot of other entries already available. We also don’t support row 24, 25, 55, 56. Again, these rows are used for SU performance optimization, which does not fit into Rel-18 DMRS enhancement. Furthermore, it seems the following entries for 1 CW are missing. 48; 1; 0,1,8; 2. 49; 1; 0,1,8,9; 2. 50; 1; 4,5,12; 2. 51; 1; 4,5,12,13; 2. 52; 2; 0,1,8; 2. 53; 2; 0,1,8,9; 2. 54; 2; 4,5,12; 2. 55; 2; 4,5,12,13; 2. 56; 2; 2,3,10; 2. 57; 2; 2,3,10,11; 2. 58; 2; 6,7,14; 2. 59; 2; 6,7,14,15; 2. For S-TRP, for two CWs entries, we don’t support all of current entries, due to a simple reason. The current entries will result the layers of 1 CW map to two CDM groups. Like we commented for single symbol DMRS, keeping the layers of 1 CW only map to one CDM group would simplify receiver. Based on this principle, we propose the following entries for 2CWs. 0; 2; 0-4; 2. 1; 2; 0,1,2,3,4,6; 2. 2; 2; 0,1,2,3,4,5,6; 2. 3; 2; 0,1,2,3,4,5,6,7; 2. 0; 2; 0,1, 2,3,10; 1. 1; 2; 0,1,8,2,3,10; 1. 2; 2; 0,1,8, 2,3,10,11; 1. 3; 2; 0,1,8,9,2,3,10,11; 1. 4; 1; 0,1,4,5,12; 2. 5; 1; 0,1,8,4,5,12; 2. 6; 1; 0,1,8,4,5,12,13; 2. 7; 1; 0,1,8,9,4,5,12,13; 2. 8; 2; 0,1,4,5,12; 2. 9; 2; 0,1,8,4,5,12; 2. 10; 2; 0,1,8,4,5,12,13; 2. 11; 2; 0,1,8,9,4,5,12,13; 2. 12; 2; 2,3,6,7,14; 2. 13; 2; 2,3,10,6,7,14; 2. 14; 2; 2,3,10,6,7,14,15; 2. 15; 2; 2,3,10,11,6,7,14,15; 2. For M-TRP, we support a separate DMRS port table. In that table, only support row 68 adding for M-TRP. |
| LGE | We are fine with the rows in general but I think we need more discussion not to exceed 64 rows. |
| Apple | 1 CW entries: Our preference is to keep the number of rows not more than 64 (including both S-TRP and M-TRP). Rows 55, 56 can be removed among the rows without brackets. For M-TRP, only row 68 is sufficient and row 69 can be removed Among the rows with brackets, rows 40, 41, 42 and 61 can be removed 2 CW entries: For 2 CW entries, we tend to agree with Nokia that only one combination for each rank is sufficient |
| Ericsson | Support. The table shall be designed to ensure SU MIMO performance unless the UE can dynamically switch to legacy DMRS table. |
| China Telecom | Support in general but the details should be corresponding to that of eType1, maxLength 1. |
| vivo | Prefer maximum number of rows not exceed 64. The similar principle should be used to maxlength=1. We can further discuss the detailed entries after the output of issue 2.1.1. |
| Spreadtrum | We also prefer to have limited size of antenna port field, if possible. The support of rows depends on the outcome of eType1, maxLength1. |

2.1.3 eType2, maxLength1

Goal of RAN1#112 for eType2, maxLength1 is to agree some DMRS port combinations with bracket, similar as RAN1#111 agreement. I added [ ] to controversial rows. Please check whether rows without [ ] are acceptable. The rows with [ ] will be discussed after they are agreed for eType1, maxLength1.

At least for S-TRP case

Table 7.3.1.2.2-3-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| [0 | 1 | 0] | 0 | 3 | 0-4 |
| [1 | 1 | 1] | 1 | 3 | 0-5 |
| [2 | 1 | 0,1] | 2 | 3 | 12-16 |
| 3 | 2 | 0 | 3 | 3 | 12-17 |
| 4 | 2 | 1 | 4 | 2 | 0,1,2,3,12 |
| 5 | 2 | 2 | 5 | 2 | 0,1,2,3,12,14 |
| 6 | 2 | 3 | 6 | 2 | 0-3,12-14 |
| 7 | 2 | 0,1 | 7 | 2 | 0-3,12-15 |
| 8 | 2 | 2,3 | 8 | 3 | 0,1,2,3,12 |
| [9 | 2 | 0-2] | 9 | 3 | 0,1,2,3,12,14 |
| [10 | 2 | 0-3] | 10 | 3 | 0-3,12-14 |
| 11 | 3 | 0 | 11 | 3 | 0-3,12-15 |
| 12 | 3 | 1 |  |  |  |
| 13 | 3 | 2 |  |  |  |
| 14 | 3 | 3 |  |  |  |
| 15 | 3 | 4 |  |  |  |
| 16 | 3 | 5 |  |  |  |
| 17 | 3 | 0,1 |  |  |  |
| 18 | 3 | 2,3 |  |  |  |
| 19 | 3 | 4,5 |  |  |  |
| [20 | 3 | 0-2] |  |  |  |
| [21 | 3 | 3-5] |  |  |  |
| [22 | 3 | 0-3] |  |  |  |
| 23 | 2 | 0,2 |  |  |  |
| [24 | 1 | 12] |  |  |  |
| [25 | 1 | 13] |  |  |  |
| [26 | 1 | 12,13] |  |  |  |
| 27 | 2 | 12 |  |  |  |
| 28 | 2 | 13 |  |  |  |
| 29 | 2 | 14 |  |  |  |
| 30 | 2 | 15 |  |  |  |
| 31 | 2 | 12,13 |  |  |  |
| 32 | 2 | 14,15 |  |  |  |
| [33 | 2 | 12-14] |  |  |  |
| [34 | 2 | 12-15] |  |  |  |
| 35 | 3 | 12 |  |  |  |
| 36 | 3 | 13 |  |  |  |
| 37 | 3 | 14 |  |  |  |
| 38 | 3 | 15 |  |  |  |
| 39 | 3 | 16 |  |  |  |
| 40 | 3 | 17 |  |  |  |
| 41 | 3 | 12,13 |  |  |  |
| 42 | 3 | 14,15 |  |  |  |
| 43 | 3 | 16,17 |  |  |  |
| [44 | 3 | 12-14] |  |  |  |
| [45 | 3 | 15-17] |  |  |  |
| [46 | 3 | 12-15] |  |  |  |
| [47 | 2 | 12,14] |  |  |  |
| [48 | 1 | 0,1,12] |  |  |  |
| [49 | 1 | 0,1,12,13] |  |  |  |
| 50 | 2 | 0,1,12 |  |  |  |
| 51 | 2 | 0,1,12,13 |  |  |  |
| 52 | 2 | 2,3,14 |  |  |  |
| 53 | 2 | 2,3,14,15 |  |  |  |
| 54 | 3 | 0,1,12 |  |  |  |
| 55 | 3 | 0,1,12,13 |  |  |  |
| 56 | 3 | 2,3,14 |  |  |  |
| 57 | 3 | 2,3,14,15 |  |  |  |
| 58 | 3 | 4,5,16 |  |  |  |
| 59 | 3 | 4,5,16,17 |  |  |  |

For M-TRP case

Table 7.3.1.2.2-3A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| … | … | … |
| 60 | 2 | 0,2,3 |
| 61 | 2 | 12,14,15 |

For S-TRP, please check the rows without [ ] are acceptable. For M-TRP, the same rule as eType1 maxLength1 (select 1st one, 2nd one, or both) will be discussed later.

| Company | Comment |
| --- | --- |
| Docomo | Support |
| Google | For 2 CW and row 48//50/54/56/58 for single CW, similar to our comment in 2.1.1, we think we should reserve more ports for legacy UEs, e.g., changing [0, 1, 12] into [0, 12, 13] |
| OPPO | Similarly, we think Row 33-34, 44-47, 61 is not needed. |
| ZTE | Further discuss it based on the outcome of eType1, maxLength1. |
| CATT | Support the rows without [ ]. All the legacy port combinations can be included in the new table. |
| Lenovo | Support in general. We think similar principle can be kept for designing both eType 1 and eType 2 DMRS port indication table. |
| Sharp | We can go with the majority. |
| Xiaomi | Agree with ZTE. |
| Huawei, HiSilicon | For 2CWs, considering it is mainly used for SU scenario, it seems that entries 0~1 & 4~7 are enough. |
| Nokia/NSB | We don’t support yellow highlighted. M-TRP, if supported, we are fine. For 2CWs, we think first 8 rows are enough. Row 8,9,10,11 are not necessary. |
| Futurewei | Further discuss after the case for eType1, maxLength = 1 is settled. |
| QC | For S-TRP, 1CW, we don’t support those entries for SU performance optimization, i.e., row 23. For S-TRP, 2 CW, we don’t support all entries in current proposal, again because the current entries will result the layers of 1 CW map to two CDM groups. Keeping the layers of 1 CW only map to one CDM group would simplify receiver. Based on this principle, we propose the following entries for 2CWs Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 3; 0-4. 1; 3; 0-5. 2; 3; 12-16. 3; 3; 12-17. 0; 2; 0,1,2,3,14. 1; 2; 0,1,12,2,3,14. 2; 2; 0,1,12,2,3,14,15. 3; 2; 0,1,12,13,2,3,14,15. 4; 3; 0,1,2,3,14. 5; 3; 0,1,12,2,3,14. 6; 3; 0,1,12,2,3,14,15. 7; 3; 0,1,12,13,2,3,14,15. For M-TRP, we only support row 60, not row 61. |
| Apple | For 1 CW, M-TRP, only row 60 is sufficient For 2 CWs, rows 0-7 are sufficient |
| MediaTek | Same comment as Apple. |
| Ericsson | Support |
| China Telecom | Support in general but the details should be corresponding to that of eType1, maxLength 1. |
| vivo | Further to discuss it after the output of issue 2.1.1, based on the similar design principle. |
| Spreadtrum | Discuss later and keep the design principle of port combinations the same as for eType 1. |

2.1.4 eType2, maxLength2

Goal of RAN1#112 for eType2, maxLength1 is to agree some DMRS port combinations with bracket, similar as RAN1#111 agreement. I added [ ] to controversial rows. Please check whether rows without [ ] are acceptable. The rows with [ ] will be discussed after they are agreed for eType1, maxLength1.

Table 7.3.1.2.2-4-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| [0 | 1 | 0 | 1] | 0 | 3 | 0-4 | 1 |
| [1 | 1 | 1 | 1] | 1 | 3 | 0-5 | 1 |
| [2 | 1 | 0,1 | 1] | 2 | 2 | 0,1,2,3,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,6,8 | 2 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,6,7,8 | 2 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,12 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0-3,12,14 | 1 |
| 8 | 2 | 2,3 | 1 | 8 | 2 | 0-3,12-14 | 1 |
| [9 | 2 | 0-2 | 1] | 9 | 2 | 0-3,12-15 | 1 |
| [10 | 2 | 0-3 | 1] | 10 | 3 | 0,1,2,3,12 | 1 |
| 11 | 3 | 0 | 1 | 11 | 3 | 0-3,12,14 | 1 |
| 12 | 3 | 1 | 1 | 12 | 3 | 0-3,12-14 | 1 |
| 13 | 3 | 2 | 1 | 13 | 3 | 0-3,12-15 | 1 |
| 14 | 3 | 3 | 1 | 14 | 1 | 0,1,6,7,12 | 2 |
| 15 | 3 | 4 | 1 | 15 | 1 | 0,1,6,7,12,18 | 2 |
| 16 | 3 | 5 | 1 | 16 | 1 | 0,1,6,7,12,13,18 | 2 |
| 17 | 3 | 0,1 | 1 | 17 | 1 | 0,1,6,7,12,13,18,19 | 2 |
| 18 | 3 | 2,3 | 1 | 18 | 2 | 0,1,6,7,12 | 2 |
| 19 | 3 | 4,5 | 1 | 19 | 2 | 0,1,6,7,12,18 | 2 |
| [20 | 3 | 0-2 | 1] | 20 | 2 | 0,1,6,7,12,13,18 | 2 |
| [21 | 3 | 3-5 | 1] | 21 | 2 | 0,1,6,7,12,13,18,19 | 2 |
| [22 | 3 | 0-3 | 1] | 22 | 2 | 2,3,8,9,14 | 2 |
| 23 | 2 | 0,2 | 1 | 23 | 2 | 2,3,8,9,14,20 | 2 |
| 24 | 3 | 0 | 2 | 24 | 2 | 2,3,8,9,14,15,20 | 2 |
| 25 | 3 | 1 | 2 | 25 | 2 | 2,3,8,9,14,15,20,21 | 2 |
| 26 | 3 | 2 | 2 | 26 | 3 | 0,1,6,7,12 | 2 |
| 27 | 3 | 3 | 2 | 27 | 3 | 0,1,6,7,12,18 | 2 |
| 28 | 3 | 4 | 2 | 28 | 3 | 0,1,6,7,12,13,18 | 2 |
| 29 | 3 | 5 | 2 | 29 | 3 | 0,1,6,7,12,13,18,19 | 2 |
| 30 | 3 | 6 | 2 | 30 | 3 | 2,3,8,9,14 | 2 |
| 31 | 3 | 7 | 2 | 31 | 3 | 2,3,8,9,14,20 | 2 |
| 32 | 3 | 8 | 2 | 32 | 3 | 2,3,8,9,14,15,20 | 2 |
| 33 | 3 | 9 | 2 | 33 | 3 | 2,3,8,9,14,15,20,21 | 2 |
| 34 | 3 | 10 | 2 | 34 | 3 | 4,5,10,11,16 | 2 |
| 35 | 3 | 11 | 2 | 35 | 3 | 4,5,10,11,16,22 | 2 |
| 36 | 3 | 0,1 | 2 | 36 | 3 | 4,5,10,11,16,17,22 | 2 |
| 37 | 3 | 2,3 | 2 | 37 | 3 | 4,5,10,11,16,17,22,23 | 2 |
| 38 | 3 | 4,5 | 2 |  |  |  |  |
| 39 | 3 | 6,7 | 2 |  |  |  |  |
| 40 | 3 | 8,9 | 2 |  |  |  |  |
| 41 | 3 | 10,11 | 2 |  |  |  |  |
| [42 | 3 | 0,1,6 | 2] |  |  |  |  |
| [43 | 3 | 2,3,8 | 2] |  |  |  |  |
| [44 | 3 | 4,5,10 | 2] |  |  |  |  |
| [45 | 3 | 0,1,6,7 | 2] |  |  |  |  |
| [46 | 3 | 2,3,8,9 | 2] |  |  |  |  |
| [47 | 3 | 4,5,10,11 | 2] |  |  |  |  |
| [48 | 1 | 0 | 2] |  |  |  |  |
| [49 | 1 | 1 | 2] |  |  |  |  |
| [50 | 1 | 6 | 2] |  |  |  |  |
| [51 | 1 | 7 | 2] |  |  |  |  |
| [52 | 1 | 0,1 | 2] |  |  |  |  |
| [53 | 1 | 6,7 | 2] |  |  |  |  |
| 54 | 2 | 0,1 | 2 |  |  |  |  |
| 55 | 2 | 2,3 | 2 |  |  |  |  |
| 56 | 2 | 6,7 | 2 |  |  |  |  |
| 57 | 2 | 8,9 | 2 |  |  |  |  |
| [58 | 1 | 12 | 1] |  |  |  |  |
| [59 | 1 | 13 | 1] |  |  |  |  |
| [60 | 1 | 12,13 | 1] |  |  |  |  |
| 61 | 2 | 12 | 1 |  |  |  |  |
| 62 | 2 | 13 | 1 |  |  |  |  |
| 63 | 2 | 14 | 1 |  |  |  |  |
| 64 | 2 | 15 | 1 |  |  |  |  |
| 65 | 2 | 12,13 | 1 |  |  |  |  |
| 66 | 2 | 14,15 | 1 |  |  |  |  |
| [67 | 2 | 12-14 | 1] |  |  |  |  |
| [68 | 2 | 12-15 | 1] |  |  |  |  |
| 69 | 3 | 12 | 1 |  |  |  |  |
| 70 | 3 | 13 | 1 |  |  |  |  |
| 71 | 3 | 14 | 1 |  |  |  |  |
| 72 | 3 | 15 | 1 |  |  |  |  |
| 73 | 3 | 16 | 1 |  |  |  |  |
| 74 | 3 | 17 | 1 |  |  |  |  |
| 75 | 3 | 12,13 | 1 |  |  |  |  |
| 76 | 3 | 14,15 | 1 |  |  |  |  |
| 77 | 3 | 16,17 | 1 |  |  |  |  |
| 78 | 3 | 12-14 | 1] |  |  |  |  |
| 79 | 3 | 15-17 | 1] |  |  |  |  |
| 80 | 3 | 12-15 | 1] |  |  |  |  |
| [81 | 2 | 12,14 | 1] |  |  |  |  |
| 82 | 3 | 12 | 2 |  |  |  |  |
| 83 | 3 | 13 | 2 |  |  |  |  |
| 84 | 3 | 14 | 2 |  |  |  |  |
| 85 | 3 | 15 | 2 |  |  |  |  |
| 86 | 3 | 16 | 2 |  |  |  |  |
| 87 | 3 | 17 | 2 |  |  |  |  |
| 88 | 3 | 18 | 2 |  |  |  |  |
| 89 | 3 | 19 | 2 |  |  |  |  |
| 90 | 3 | 20 | 2 |  |  |  |  |
| 91 | 3 | 21 | 2 |  |  |  |  |
| 92 | 3 | 22 | 2 |  |  |  |  |
| 93 | 3 | 24 | 2 |  |  |  |  |
| 94 | 3 | 12,13 | 2 |  |  |  |  |
| 95 | 3 | 14,15 | 2 |  |  |  |  |
| 96 | 3 | 16,17 | 2 |  |  |  |  |
| 97 | 3 | 18,19 | 2 |  |  |  |  |
| 98 | 3 | 20,21 | 2 |  |  |  |  |
| 99 | 3 | 22,23 | 2 |  |  |  |  |
| 100 | 3 | 12,13,18 | 2] |  |  |  |  |
| 101 | 3 | 14,15,20 | 2] |  |  |  |  |
| 102 | 3 | 16,17,22 | 2] |  |  |  |  |
| 103 | 3 | 12,13,18,19 | 2] |  |  |  |  |
| 104 | 3 | 14,15,20,21 | 2] |  |  |  |  |
| 105 | 3 | 16,17,22,23 | 2] |  |  |  |  |
| 106 | 1 | 12 | 2] |  |  |  |  |
| 107 | 1 | 13 | 2] |  |  |  |  |
| 108 | 1 | 18 | 2] |  |  |  |  |
| 109 | 1 | 19 | 2] |  |  |  |  |
| [110 | 1 | 12,13 | 2] |  |  |  |  |
| [111 | 1 | 18,19 | 2] |  |  |  |  |
| 112 | 2 | 12,13 | 2 |  |  |  |  |
| 113 | 2 | 14,15 | 2 |  |  |  |  |
| 114 | 2 | 18,19 | 2 |  |  |  |  |
| 115 | 2 | 20,21 | 2 |  |  |  |  |
| [116 | 1 | 0,1,12 | 1] |  |  |  |  |
| [117 | 1 | 0,1,12,13 | 1] |  |  |  |  |
| 118 | 2 | 0,1,12 | 1 |  |  |  |  |
| 119 | 2 | 0,1,12,13 | 1 |  |  |  |  |
| 120 | 2 | 2,3,14 | 1 |  |  |  |  |
| 121 | 2 | 2,3,14,15 | 1 |  |  |  |  |
| 122 | 3 | 0,1,12 | 1 |  |  |  |  |
| 123 | 3 | 0,1,12,13 | 1 |  |  |  |  |
| 124 | 3 | 2,3,14 | 1 |  |  |  |  |
| 125 | 3 | 2,3,14,15 | 1 |  |  |  |  |
| 126 | 3 | 4,5,16 | 1 |  |  |  |  |
| 127 | 3 | 4,5,16,17 | 1 |  |  |  |  |

For M-TRP case

Table 7.3.1.2.2-4A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |
| --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| … | … | … | … |
| 128 | 2 | 0,2,3 | 1 |
| 129 | 2 | 12,14,15 | 1 |

For S-TRP, please check the rows without [ ] are acceptable. For M-TRP, the same rule as eType1 maxLength1 (select 1st one, 2nd one, or both) will be discussed later.

| Company | Comment |
| --- | --- |
| Docomo | Support |
| Google | For 2 CW and row 116/118/120/122/124/126 for single CW, similar to our comment in 2.1.1, we think we should reserve more ports for legacy UEs, e.g., changing [0, 1, 12] into [0, 12, 13] |
| OPPO | Row67-68，78-81, 129 is not needed. |
| ZTE | Further discuss it based on the outcome of eType1, maxLength1. |
| CATT | Support the rows without [ ]. All the legacy port combinations can be included in the new table. |
| Lenovo | Support in general. We think similar principle can be kept for designing both eType 1 and eType 2 DMRS port indication table. |
| Sharp | We prefer removing Row indexes 42,43,44,45,46,47,100,101,102,103,104,105 in yellow parts. In our view, the benefit of the Row indexes is unclear. |
| Xiaomi | Still, we prefer to remove these rows with Number of DMRS CDM group(s) without data = 1 when maxlength is configured as 2. |
| Huawei, HiSilicon | For 2CWs, considering it is mainly used for SU scenario, it seems that entries 0~9 & 14~17 are enough. |
| Nokia/NSB | We are not happy with such many rows in a table and similar view with Xiaomi, at least. Yellow highlighted are not our preference. For 2 CWs, we prefer to only single symbol only. No need for two symbol for SU-MIMO case, and we support row 0,1,6,7,8,9 and CDM=3, {0,1,2,3,4,5,12}, {0,1,2,3,4,5,12,14}- length=1. |
| Futurewei | Further discuss after the case for eType1, maxLength = 1 is settled. |
| QC | For S-TRP, 1 CW, we don’t support row 23, which are for SU performance optimization. Furthermore, we think the following entries are missing. 104; 1; 0,1,12; 2. 105; 1; 0,1,12,13; 2. 106; 1; 6,7,18; 2. 107; 1; 6,7,18,19; 2. 108; 2; 0,1,12; 2. 109; 2; 0,1,12,13; 2. 110; 2; 6,7,18; 2. 111; 2; 6,7,18,19; 2. 112; 2; 2,3,14; 2. 113; 2; 2,3,14,15; 2. 114; 2; 8,9,20; 2. 115; 2; 8,9,20,21; 2. 116; 3; 0,1,12; 2. 117; 3; 0,1,12,13; 2. 118; 3; 6,7,18; 2. 119; 3; 6,7,18,19; 2. 120; 3; 2,3,14; 2. 121; 3; 2,3,14,15; 2. 122; 3; 8,9,20; 2. 123; 3; 8,9,20,21; 2. 124; 3; 4,5,16; 2. 125; 3; 4,5,16,17; 2. 126; 3; 10,11,22; 2. 127; 3; 10,11,22,23; 2. For S-TRP, 2 CW, again due to the issue of layer to CDM group mapping, we don’t agree with all the entries in the current proposal and we propose the following. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 3; 0-4; 1. 1; 3; 0-5; 1. 2; 2; 0,1,2,3,6; 2. 3; 2; 0,1,2,3,6,8; 2. 4; 2; 0,1,2,3,6,7,8; 2. 5; 2; 0,1,2,3,6,7,8,9; 2. 6; 3; 12-16; 1. 7; 3; 12-17; 1. 8; 2; 12-15,18; 2. 9; 2; 12-15,18,20; 2. 10; 2; 12-15,18-20; 2. 11; 2; 12-15,18-21; 2. 0; 2; 0,1,2,3,14; 1. 1; 2; 0,1,12,2,3,14; 1. 2; 2; 0,1,12,2,3,14,15; 1. 3; 2; 0,1,12,13,2,3,14,15; 1. 4; 1; 0,1,6,7,18; 2. 5; 1; 0,1,12,6,7,18; 2. 6; 1; 0,1,12,6,7,18,19; 2. 7; 1; 0,1,12,13,6,7,18,19; 2. 8; 2; 0,1,6,7,18; 2. 9; 2; 0,1,12,6,7,18; 2. 10; 2; 0,1,12,6,7,18,19; 2. 11; 2; 0,1,12,13,6,7,18,19; 2. 12; 2; 2,3,8,9,20; 2. 13; 2; 2,3,14,8,9,20; 2. 14; 2; 2,3,14,8,9,20,21; 2. 15; 2; 2,3,14,15,8,9,20,21; 2. 16; 3; 0,1,6,7,18; 2. 17; 3; 0,1,12,6,7,18; 2. 18; 3; 0,1,12,6,7,18,19; 2. 19; 3; 0,1,12,13,6,7,18,19; 2. 20; 3; 2,3,8,9,20; 2. 21; 3; 2,3,14,8,9,20; 2. 22; 3; 2,3,14,8,9,20,21; 2. 23; 3; 2,3,14,15,8,9,20,21; 2. 24; 3; 4,5,10,11,22; 2. 25; 3; 4,5,16,10,11,22; 2. 26; 3; 4,5,16,10,11,22,23; 2. 27; 3; 4,5,16,17,11,22,23; 2. For M-TRP, we support row 128 only, not row 129. |
| Apple | For M-TRP, row 128 only is sufficient |
| MediaTek | Agree with Comments made by Qualcomm. |
| Ericsson | Support |
| China Telecom | Support in general but the details should be corresponding to that of eType1, maxLength 1. |
| vivo | Further to discuss it after the output of issue 2.1.3, based on the similar design principle. |
| Spreadtrum | Discuss later and keep the design principle of port combinations the same as for eType 1. |

  - DCI size of antenna ports field for PDSCH

For the size of antenna ports field, views are divergent. Some companies suggest keeping the size of antenna ports field, and the other companies suggest increasing the size of antenna ports field. Nokia/NSB proposes to use TDRA table instead of increasing the DCI size to indicate antenna ports. Ericsson proposes to use RRC configuration to select the actual needed row indexes in the antenna ports table. Some companies propose to introduce new DCI field of “offset indicator” to assist DMRS port indication by the antenna ports field.

FL Proposal 2-2A

- For Rel.18 eType1/eType2 DMRS ports with maxLength=1/2 for PDSCH, the size of DCI field for antenna ports indication in DCI format 1_1/1_2 is down-selected from the following:

  - Alt.0: The DCI size of DMRS port indication is the same as Rel.17.

    - FFS: Whether existing DCI field(s) other than antenna ports field (e.g. TDRA field) is additionally used for antenna ports indication.

  - Alt.1: The DCI size of DMRS port indication is increased by M (M>=1) bit from Rel.17.

    - FFS: Whether existing antenna ports field is increased by M-bit or new M-bit DCI field is added.

  - Alt.3: The DCI size of DMRS port indication is increased by M (M>=0) bit, depending on RRC configuration.

    - RRC signaling can configure the actual rows in the antenna ports table, which can be indicated by the DCI of antenna port indication.

    - Note: antenna ports field can be increased by M-bit, or new M-bit DCI field (e.g. [DMRS port(s) offset indicator]) can be introduced to assist DMRS port indication by the existing antenna ports field.

|  | Support/fine | Concern |
| --- | --- | --- |
| Alt.0 | Nokia/NBS (TDRA field) |  |
| Alt.1 | Fraunhofer IIS/HHI (2nd DCI field), Xiaomi, NEC, Intel, CMCC, Apple (no new field), Docomo, OPPO |  |
| Alt.2 | Google, Docomo, Ericsson, |  |

| Company | Comment |
| --- | --- |
| Docomo | Since the size of antenna ports table is larger than that in Rel.17, it is natural to increase the DCI size of antenna ports field. Hence, we support Alt.1, and we are also file with Alt.2. |
| Google | Since we have more ports, increasing the DCI size might be inevitable. But some codepoints could not always be necessary, e.g., based on the maxRank, some DMRS codepoints would never be indicated. Therefore, Alt3 should be a possible way to reduce the DCI overhead. |
| OPPO | We support Alt.2 with M=1bit for Rel-18 DMRS, which is the most straightforward way. |
| ZTE | It is natural that the bitsize of DCI indication field depends on its related RRC configuration and the newly introduced entries of Rel-18 DMRS ports specially. We can be fine to Alt. 1 or Alt 3 as of now. |
| CMCC | Support Alt.1. Based on the discussion in section 2.1, the codepoints increase with more DMRS ports, and it is natural to increase the DCI size. To consider the overhead, some DMRS port combinations with lower probability to be used can be removed. Besides, the DCI level dynamic indication of DMRS ports is benefit for network scheduling and flexible MU-MIMO pairing. |
| CATT | Support Alt.1. We prefer to keep all the legacy port combinations, and the DCI size can be increased accordingly. |
| Lenovo | Support Alt.1 on account of simple scheme for indicating the rows with increasing number in the antenna port table. |
| Sharp | If the size of antenna ports table is larger than that in Rel-17, we prefer Alt 1 (existing antenna ports field is increased by M-bit or new M-bit DCI field is added). |
| Xiaomi | Fine with proposal 2-2A. And we support alt.2. |
| Huawei, HiSilicon | Support Alt1. |
| Fraunhofer IIS/HHI | Support Alt. 1. Fine with Alt. 2 as a second preference. |
| Nokia/NSB | The DCI size can be determined by table size. Once table is agreed, then we can discuss if alt 2 is supported additionally. Also, with alt 2. |
| Futurewei | Support Alt. 1. As the number of rows in the DMRS port table grows, increasing the size of the antenna ports field by 1 bit is straight forward. |
| Samsung | We Support Alt.1, but it depends on how to decide DMRS port indication table design. |
| QC | We support Alt 1 (without introduce new field). We don’t support Alt 2. The additional flexibility offered by Alt 2 seems unnecessary, which only complicates UE implementation. |
| LGE | We prefer Alt 3. It should be a possible way to reduce the DCI overhead. |
| Apple | We prefer Alt 1 and don’t see any motivation/need to introduce any new/additional field. |
| MediaTek | Support Alt 1. We don’t think new DCI field is necessary. |
| Ericsson | Support Alt 3. We think RRC configuration can be used to decrease the DCI field size by selecting actual used entries that is needed for a typical service model. This would reduce the DCI overhead and reduce the UE implementation effort to store and support all combinations after RRC configuration. |
| China Telecom | Support Alt. 1. But we can also live with Alt. 2. |
| vivo | Support Alt.1 based on increasing the overhead of existing antenna ports field. |
| Spreadtrum | Support Alt.1 for simplicity. |

  - Antenna ports field for PUSCH (rank 1-4)

Similar as PDSCH, we should enhance antenna ports table for PUSCH. Since antenna ports indication for PUSCH (rank 5-8) is under discussion in sect. 3.1, we can fucus on rank 1-4 in this section.

Let’s discuss the case of eType 1 and maxLength = 1 firstly. In this proposal, the number of rows in the tables are enhanced from 8 to 16, but it does not mean that antenna ports field in DCI field in DCI format 0_1/0_2 is increased. Similar as PDSCH, it is possible that RRC can select some rows in the table, so that the DCI size of antenna ports indication field is not increased. We will discuss whether the DCI size of antenna ports field should be increased or not in later. In RAN1#111, some companies proposed to remove rows with Number of DMRS CDM group(s) without data = 1. These rows are marked with [ ] for now.

FL Proposal 2-3A

- For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, following Table 7.3.1.1.2-8-X, Table 7.3.1.1.2-9-X, Table 7.3.1.1.2-10-X, and Table 7.3.1.1.2-11-X are supported.

  - FFS: Whether to increase the size of antenna ports field in DCI format 0_1/0_2 or not.

  - Note: Antenna ports tables for Rel.18 eType2 DMRS ports with maxLength = 1/2 and eType1 DMRS ports with maxLength = 2 for PUSCH are to be discussed separately.

Table 7.3.1.1.2-8-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| [0] | [1] | [0] |
| [1] | [1] | [1] |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| [6] | [1] | [8] |
| [7] | [1] | [9] |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 2 | 11 |
| 12-15 | Reserved | Reserved |

Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| [0] | [1] | [0,1] |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 2 | 0,2 |
| [4] | [1] | [8,9] |
| 5 | 2 | 8,9 |
| 6 | 2 | 10,11 |
| [7] | [2] | [8,10] |
| 8-15 | Reserved | Reserved |

Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| [1] | [2] | [8-10] |
| [2] | [1] | [0,1,8] |
| 3 | 2 | 0,1,8 |
| 4 | 2 | 2,3,10 |
| 5-15 | Reserved | Reserved |

Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| [1] | [2] | [8-11] |
| [2] | [1] | [0,1,8,9] |
| 3 | 2 | 0,1,8,9 |
| 4 | 2 | 2,3,10,11 |
| 5-15 | Reserved | Reserved |

| Company | Comment |
| --- | --- |
| Docomo | Support. For the blue highlighted rows (rows with Number of DMRS CDM group(s) without data = 1), we believe the rows should be included. Even for MU-MIMO scenario, gNB cannot always find good MU-MIMO UE pair, and gNB dynamically switches b/w SU-MIMO and MU-MIMO for each scheduled PDSCH. Opponents commented in RAN1#111 that Rel.15 DMRS ports can be used to indicate “rows with Number of DMRS CDM group(s) without data = 1” in case of SU-MIMO. However, it requires dynamic switching between Rel.15 DMRS ports and Rel.18 DMRS ports, which may not be supported. |
| Google | For rank3 case, we suggest changing [0, 1, 8] into [0, 8, 9] or adding another codepoint for [0, 8, 9], which is to reserve more ports for legacy UEs. |
| OPPO | We support to maintain the highlighted rows for Rel-18 DMRS. |
| ZTE | Support. It is proper to support as much practical rows as possible for real-field and possible use cases. |
| CMCC | Support. The highlighted rows should be included to support dynamic switching between SU and MU-MIMO and support MU-MIMO within a CDM group, such as [0,1] and [8,9]. |
| CATT | For rank3 case, we propose to add entries [0, 2, 3] and [8, 10, 11] for simultaneous transmission of multi-panel (similar to entries for M-TRP in downlink). |
| Lenovo | Support. We are also fine with the values marked as blue in the table. |
| Sharp | We can go with the majority. |
| Xiaomi | Support |
| Huawei, HiSilicon | Generally, the DMRS port combinations for PDSCH can be reused for PUSCH (Notice that some entries are not decided yet during DL discussion). Furthermore, whether to use separate/joint table should be delayed after the indication of TRI has been decided in 9.1.4.2. |
| Fraunhofer IIS/HHI | Fine with the bracketed entries in the tables. |
| Nokia/NSB | Support |
| QC | We support the proposal. We also support include the blue highlighted rows. A quick question: why there are no similar proposals for eType 1 rank 1/2/3/4 dual DMRS, and etype 2 rank 1/2/3/4 single and dual DMRS? FL: It is because there is no baseline agreed table for PDSCH for eType1 maxLength=2 or eType2 yet. |
| LGE | We support the proposal and agree with Docomo. |
| Apple | We support the proposal |
| MediaTek | Support |
| Ericsson | Support. For rank 3 we support CATT’s proposal to add {0,2,3} and {8,10,11} for STxMP case. |
| China Telecom | We support the proposal. |
| vivo | Rank 1: Support the entries. Rank 2: Support value =0, 4, and we don’t support value=7, i.e., {8,10}, which is redundant since we already have {0,2} in the table. Rank 3: Support value=2. i.e., {0,1,8}, and we don’t support value=1, i.e., {8-10}, which is redundant since we already have {0-2} in the table. Rank 4: Support value=2. i.e., {0,1,8,9}, and we don’t support value=1, i.e., {8-11}, which is redundant since we already have {0-3} in the table. Please note that {8,10}, {8-10}, {8-11} are not agreed yet in issue 2.1.1, at lease we should add [] at this stage. |
| Spreadtrum | Support the proposal. |

  - DCI-based dynamic switching between FD-OCC length 2 and 4

In multiple meetings, whether to support the dynamic switching of FD-OCC length has been discussed. Multiple companies (e.g. Figure 1 in [4]) shows performance degradation if FD-OCC length 4 is used for de-spreading compared to the case of FD-OCC length 2. On the other hand, some other companies (e.g. figure 3 in [21]) shows that there is no performance difference between channel estimation with FD-OCC length 2 and 4.

(a) DS=1000ns @eType 2 DMRS for PUSCH     	(b) DS=1000ns @type 2 DMRS for PDSCH

Figure 1 BLER performance gap between the legacy DMRS with length 2 FD-OCC and Rel-18 DMRS with length 4 FD-OCC for both PUSCH and PDSCH [4]

Fig 3: Performance comparison between assuming FD-OCC 2 vs FD-OCC 4 with joint MMSE channel estimation [21]

Ericsson [14] mentioned the dynamic switching by different DCI formats is supported as default.

One thing we should acknowledge about dynamic switching is that we can already achieve it in the downlink by using different DL DCI format. One typical example is using fallback DCI format DCI 1_0 to receive system information update and broadcasting signaling. The UE receiver algorithm implementation for DCI 1_0 should not be changed because DCI 1_1 is configured with Rel-18 DMRS type. - Observation1 Dynamic switching between Rel-18 DMRS and legacy DMRS can be achieved by using different DL DCI formats, e.g, using DCI 1_0 and DCI 1_1.

FL Proposal 2.4A

- To mitigate potential performance degradation in large delay spread in Rel.18 DMRS ports with FD-OCC length 4 for PDSCH, For UE configured with Rel.18 DMRS ports for PDSCH, select one of the following:

  - Alt.1: Scheduling DCI format 1_1/1_2 can indicate FD-OCC length that UE shall assume for receiving DMRS.

  - Alt.2: Scheduling DCI format 1_1/1_2 can indicate the information of DMRS ports of co-scheduled UE in the same CDM group.

    - Note: It is up to UE implementation how to use the information.

  - FFS: Whether additional DCI field is needed, or existing DCI field can be reused for the indication.

Support (21): Google (prefer Alt.2), OPPO (Alt.2 only), Docomo (Both), ZTE (Alt.1), CMCC(Alt.1), CATT(Alt.1), Lenovo(Alt.1), Sharp(Alt.1), Xiaomi(if majority supports), HW/HiSilicon(Alt.1 w/ UE cap.), Fraunhofer IIS/HHI(Alt.2), Nokia/NSC(Alt.1), FW(Alt.1), Samsung(Alt.1 only), Ericsson (both), China Telecom (Alt.1), vivo (Alt.1), Spreadtrum(Alt.1)

Not support (4): QC, LG, Apple, MTK (RRC based only)

Table. Summary of companies’ views for FL Proposal 2.5A

|  | Support/fine | Concern |
| --- | --- | --- |
| Alt.1 | FUTUREWEI (new DCI field), Huawei/HiSilicon (new DCI field), ZTE InterDigital, vivo, CATT, Lenovo, CMCC, Sharp, Docomo, Samsung, Nokia/NSB (by TDRA table), Ericsson, NEC, Spreadtrum | Xiaomi, OPPO Fraunhofer IIS/HHI, Apple, Qualcomm, MediaTek, LG |
| Alt.2 | Qualcomm, Google, Docomo, Ericsson |  |

| Company | Comment |
| --- | --- |
| Docomo | Support either Alt.1 or Alt. 2. Considering the previous discussion, we think Alt.2 is more beneficial. |
| Google | We see benefits from both Alts to improve the performance in UE side. In our view, Alt1 does not actually change the FD-OCC sequence, but provide some information to the UE for FD-OCC despreading length selection. Fundamentally, both can be the same. But from the spec point of view, Alt2 seems to be a better way, which does not define the decoding behavior in the UE side. Therefore, although we are open to Alt1, we slightly prefer Alt2. |
| OPPO | We don’t think dynamic switching (Alt1) is needed. |
| ZTE | Support Alt 1 according to the following elaboration. - First of all, it is common that the performance of length 2 FD-OCC can always be better than that of length 4 FD-OCC (especially in large delay spread scenario) due to the densely allocation in frequency domain of length 2 FD-OCC when compared to length 4 FD-OCC. Naturally, it is reasonable to guarantee the performance of Rel-18 DMRS in case of large delay spread scenario (i.e. DS=1000ns as assumed in EVM) as well. - Secondly, from the perspective of gNB vendors, we do believe dynamic switching between the legacy and Rel-18 DMRS ports is very essential for optimizing system performance via guaranteeing transmission scheduling in SU-MIMO and MU-MIMO scenarios. In a real field network, we observe that it is much more common to schedule DL/UL in ≤8/12-layer SU-MIMO or MU-MIMO when compared to >8/12-layer MU-MIMO which requires a reasonably good channel property as the prerequisite. Besides, it is very common that the payload of the network often changes dynamically. For a given time, the network may have >8/12-layer MU-MIMO and which needs to be supported by Rel-18 DMRS ports. But then due to payload variation, the total layers of MU-MIMO may be reduced into the range of the legacy DMRS ports, i.e. ≤8/12-layer SU-MIMO or MU-MIMO. Consequently, if going with that Rel-18 DMRS ports are semi-statically configured only and then it is mandatory to be used even for ≤8/12-layer MU-MIMO in low-pay-load cases, the performance loss of length 4 FD-OCC will be inevitable as proved before. In other words, Rel-18 DMRS ports with length 4 FD-OCC will be the bottleneck of performance from the perspective of NW. - Last but not the least, we do not agree that due to it can be up to UE implementation to perform de-spreading with length 2/4 FD-OCC and then the indication is not needed anymore. General speaking, the UE can perform any ways regardless of the indication from the NW, and it is a universally applicable theory of the specification. However, it is indeed needed to indicate the length of FD-OCC to assist UE to perform de-spreading well enough to reach out acceptable performance. Otherwise, the benefit of introducing Rel-18 DMRS ports will be quite agnostic. In addition, it should be noticed that Alt 2 is not related to dynamic switching between length 2 2 FD-OCC and length 4 FD-OCC by instinct. Basically, Alt 2 is about co-existence of the legacy and Rel-18 DMRS ports within the same CDM group when MU-MIMO scenario, which belongs to the discussion in section 2.5. Notably, as we analyzed above, the considerable performance loss of length 4 FD-OCC will also be existed in case of SU-MIMO scenario. Regarding the observation from Ericsson, technically, we generally agree that it can be workable. However, two issues cannot be avoided in this way. First, there is no DMRS port indication field in DCI format 1_0, it will extremely restrict the schedule flexibility of DMRS ports indication. Besides, it cannot be able to support MU-MIMO scenario and also >4 layers PUSCH transmission. Second, it will lead to the waste of PDCCH candidates monitoring when considering PDCCH of DCI format 1_1/1_2 have already been monitored and used. Hence utilizing the switching between DCI format 1_0 and DCI format 1_1/1_2 is not a proper way. |
| CMCC | Support Alt.1. For SU-MIMO, since length-4 FD-OCC is more sensitive to the frequency selectivity fading, it is better to use length-2 FD-OCC for channel estimation under large delay spread scenario. While for higher layers MU-MIMO, length-4 FD-OCC is needed. So, dynamic switching of length 2 and length 4 FD-OCC is benefit for SU and MU-MIMO switching. For MU-MIMO scheduling, if the layers of MU-MIMO is less than 8 or 12 layers, then length-2 FD-OCC can be used with better performance. While when network schedules higher layers MU-MIMO, then length-4 FD-OCC is needed. So, dynamic switching of length 2 and length 4 FD-OCC is benefit for MU-MIMO scheduling. |
| CATT | Support Alt.1.A new information field can be adopted to indicate either FD-OCC length or DMRS type. |
| Lenovo | Support Alt. 1. We think even for SU-MIMO, this mechanism is also beneficial. For example, when the delay spread is large, FD-OCC 2 can be used for better performance; when the delay spread is relatively small, FD-OCC 4 can be applied to save some resources since only one CDM group may be enough. |
| Sharp | We have the similar view with ZTE. Support Alt 1. |
| Xiaomi | We support that R18 DMRS can fall back to legacy DMRS port when there is performance degradation because of large delay spread. But whether it should be based on DCI or RRC signaling needs more discussion, at least the issue mentioned in QC’s contribution should be noted. Well, we are open to this issue and we can support proposal 2.4A if DCI based switching is most companies’ preference. |
| Huawei, HiSilicon | Support DCI-based dynamic switching with indicating FD-OCC length. However, considering the potential complexity for preparing different channel estimation algorithms, introducing UE capability can be a good compromise. |
| Fraunhofer IIS/HHI | This proposal is very much intertwined with proposal 2.5A. While alternatives for dynamic FD-OCC length determination are discussed here, proposal 2.5A also discusses its inclusion for a special case of MU-MIMO (MU-MIMO scheduling scenario 4). Does this mean that this proposal is applicable only to MU-MIMO scheduling scenario 2 in proposal 2.5A? A clear separation of the two aspects – method to enable dynamic switching in MU-MIMO scenarios and the possible MU-MIMO scenarios the UE can expect or gNB can schedule – among the two proposals would avoid overlapping discussions. In our perspective, the cons of DCI based switching may outweigh its pros and network implementation may take care of the appropriate scheduling. Hence, RRC-based switching is enough. However, if majority of companies support DCI based switching, we would be open to Alt. 2. |
| Nokia/NSB | We support Alt 1. We have discussed Alt 2 in Rel-15, but it wasn’t agreed due to high complexity. What to be signaled? The signaling of MU-MIMO combinations requires higher complexity. If not, the information is insufficient. Indication of switching FD-OCC2 and FD-OCC4 is much easier than alt 2. |
| Futurewei | Support Alt 1 and we echo other companies’ comments on the advantages of Alt 1. |
| Samsung | Support Alt.1 and not support Alt.2. |
| QC | We don’t support this proposal in general. Because we don’t see performance degradation of using FD-OCC4, even with 1000ns delay spread. Also, there is no justification to let gNB to force UE to do channel estimation assuming size 2 or size 4 OCC. If NW tell a UE this is OCC2, but UE can decode PDSCH with OCC4 assumption, what is wrong about the UE behavior? Therefore, we still object Alt 1. As for Alt 2, since it can potentially indicate more information rather than just 1 bit on OCC size, we are open to discuss this alternative. But this “To mitigate potential performance degradation in large delay spread in Rel.18 DMRS ports with FD-OCC length 4 for PDSCH” in main sentence of the proposal should be removed, we did not observe this performance degradation in our simulation results. |
| LGE | We think RRC based switching can be the base line because the DMRS type is indicated by RRC in current specification. In our view, semi-static switching is sufficient. Since OCC length 2 is used for legacy DMRS, the switching can be done by using RRC signaling to enable or disable Rel-18 DMRS. |
| Apple | We don’t support this proposal and agree to follow the legacy procedure of switching between different DMRS configuration types via RRC signaling. |
| MediaTek | Not Support, we believe RRC based switching is sufficient. |
| Ericsson | Support both Alt.1 and Alt.2. Alt.2 is more UE friendly solution, it provides the information to the UE and let UE to optimize the receiver implementation if it has sufficient capacity. Such information can be provided by using entries in antenna port table or DCI indication. |
| China Telecom | We support Alt.1 for the following reasons. Firstly, it is no doubt that the FD-OCC 2 can have a better performance compared with FD-OCC 4, especially for the SU-MIMO. The FD-OCC 2 should be adopted in the realistic network with higher priority, so that the performance degradation brought by the FD-OCC 4 will only happens in the necessary scenarios such as MU-MIMO where the layer is larger than 8/12. Secondly, it is not a common case that the MU-MIMO is adopted. Considering the mobility of UE, the MU-MIMO network condition can be changed to SU-MIMO very often that the FD-OCC 2 can satisfy the current demand. In such case, the UE should be aware of the change and decode the DMRS with FD-OCC 2 to reduce the performance degradation. Therefore, the indication of FD-OCC type is necessary. Last, we think the Alt.2 is just a comprise of Alt.1, where the motivation is actually the same as Alt.1, to indicate the UE how to decode the DMRS. But when the DMRS ports <=12, the UE won’t be aware of the DMRS is R18 or not. Based on the above reasons, we think the Alt.1 should be supported. |
| vivo | Support Alt 1. One straightforward way is dynamic switching between legacy DMRS table for legacy DMRS with FD-OCC2 and new DMRS table for Rel-18 DMRS with FD-OCC4. Another way is dynamic switching between legacy DMRS port and Rel-18 DMRS port in the same new table including legacy DMRS port with FD-OCC2 and Rel-18 DMRS port with FD-OCC4. |
| Spreadtrum | Support the proposal and prefer Alt1. We don’t think UE should select FD-OCC 4 in any case if FD-OCC2 is configured/indicated. |

  - MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports

In RAN1#110, we agreed to study MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports within a CDM group for PDSCH. Note that for PUSCH, there should be no restriction, as Nokia/NSB commented in RAN1#110. The agreed scope of the study only includes PDSCH. For PDSCH, some proposals are submitted in RAN1#111, and it seems most of companies assume there is no restriction between Rel.18 legacy DMRS ports and Rel.15 DMRS ports. Whether MU-MIMO between Rel.18 new ports and Rel.15 DMRS ports are allowed can be discussed.

FL Proposal 2.5A

- For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

  - 1) For PUSCH, there is no restriction.

  - 2) For PDSCH, there is no additional restriction between Rel.18 UE1 indicated with Rel-18 Legacy ports (eType1: ports 1000-1007, eType2: ports 1000-1011) and Rel.15/18 UE2 indicated with Rel.15 DMRS ports in a CDM group from Rel.17 spec.

    - Note: MU-MIMO restriction in Rel.17 is applied.

  - 3) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.15 UE2 indicated with Rel.15 DMRS ports in a CDM group,

    - UE does not expect such MU-MIMO in a CDM group.

  - 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, down select from the following.

    - Alt.1: UE does not expect such MU-MIMO in a CDM group.

    - Alt.2: Rel.18 UE2 configured with Rel.15 DMRS ports can be signaled, to indicate that there may be another Rel.18 UE1 with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports.

      - Dedicated UE capability is introduced.

      - The signaling is at least by RRC (FFS: whether to support DCI based signaling).

    - Alt.3: It is up to gNB implementation whether to schedule such MU-MIMO in a CDM group (i.e. no spec impact and no additional restriction).

Table. Summary of companies’ views for FL Proposal 2.6A

|  | Support/fine | Concern |
| --- | --- | --- |
| 1) | Docomo, Apple, Google, Nokia/NSB, Huawei/HiSilicon, Futurewei, Samsung, Lenovo, Sharp, vivo, QC, Ericsson, LG, ZTE, Xiaomi, MediaTek, CMCC, Spreadtrum, Intel |  |
| 2) | Docomo, [Apple] , Nokia/NSB, Huawei/HiSilicon, Futurewei, Samsung, Lenovo, Sharp, vivo, [QC], [Ericsson] , LG, ZTE, Xiaomi, MediaTek, CMCC, Spreadtrum | Google (discuss after dynamic switching) |
| 3) | Huawei/HiSilicon, Intel, Docomo, Apple |  |
| 4) | Support Alt.1: Nokia/NSB, Futurewei, Samsung, Lenovo, Sharp, MediaTek, Spreadtrum, Apple (for default UE cap.) Support Alt.2: Huawei/HiSilicon (by DCI), Docomo, Apple (w/ UE cap.), Ericsson, ZTE (by DCI) Support Alt.3: Docomo, OPPO, Nokia/NSB, Samsung, Lenovo, vivo, LG, CMCC, Spreadtrum, | Google (discuss after dynamic switching) |

FL: It seems 1)~3) are stable.

| Company | Comment |
| --- | --- |
| Google | In our view, with the help of information to identify the co-scheduled ports as in 2.4, UE can identify the best FD-OCC despreading scheme. That’s why we think 2), 3) and 4) can be deferred after we see the outcome of 2.4. |
| OPPO | At this stage, we don’t think any restriction is needed in RAN1, as long as the DMRS sequences of Rel-15 UEs and Rel-18 UEs are orthogonal. |
| ZTE | For clarification, as we elaborated in section 2.4, MU-MIMO between the legacy and Rel-18 DMRS ports is part of the discussion of dynamic switching between FD-OCC 2 and FD-OCC 4. For 4) in FL proposal 2.5A, support Alt. 2. Basically, it is beneficial to guarantee schedule flexibility of MU-MIMO and also interference estimation of each co-scheduled UE. |
| CMCC | Based on dynamic switching between length-2 and length-4 FD-OCC, gNB can decide whether to schedule such MU-MIMO in a CDM group. |
| CATT | Support the proposal. For 4), Alt.3 with the following clarification is preferred. - 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, - Alt.1: UE does not expect such MU-MIMO in a CDM group. - Alt.2: Rel.18 UE2 configured with Rel.15 DMRS ports can be signaled, to indicate that there may be another Rel.18 UE1 with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports. - Dedicated UE capability is introduced. - The signaling is at least by RRC (FFS: whether to support DCI based signaling). - Alt.3: It is up to gNB implementation whether to schedule such MU-MIMO in a CDM group (i.e. no spec impact and no additional restriction). |
| Lenovo | Support the proposal. For item 4, support Alt.3. |
| Xiaomi | It seems that we misunderstand the meaning of “Rel.15 DMRS ports”. We thought it was the R15/16 UE(s). Now, it could be - Case1: R18 UE indicated with legacy DMRS port indexes - Case2: R18 UE indicated with legacy DMRS port - Case3: R15/16 UE that can only be indicated with legacy DMRS port. For all these three cases, we think it is up to gNB implementation whether to schedule such MU-MIMO in a CDM group. Considering case 3, assuming Hadamard code shown in table 1 is used, if legacy UE is indicated with DMRS port 0 which applies OCC [+1,+1]T, then for R18 UE, DMRS port(s) using FD-OCC with index 1 and 2 can be indicated. Restriction is needed, but it is up to NW. FD-OCC index; wf(0); wf(1); wf(2); wf(3). 0; +1; +1; +1; +1. 1; +1; -1; +1; -1. 2; +1; +1; -1; -1. 3; +1; -1; -1; +1. Maybe we could make the following conclusion: FL Conclusion 2.5A - For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports, - It is up to gNB implementation whether to schedule such MU-MIMO in a CDM group (i.e. no spec impact). - Note: Rel.15 DMRS ports mean - R18 UE indicated with legacy DMRS port indexes - Or, R18 UE indicated with legacy DMRS port - Or, R15/16 UE that can only be indicated with legacy DMRS port |
| Fraunhofer IIS/HHI | Agree in principle. For MU-MIMO scenario (4), prefer Alt-1 and can live with Alt-3. |
| Nokia/NSB | We prefer Alt-3, but fine with Alt-1. |
| Futurewei | For 4), we prefer Alt. 1 but can live with Alt. 3. |
| Samsung | Support the proposal, and for 4) support Alt3, i.e., no restriction and no additional spec impact. |
| QC | For 1)-3) in the FL Proposal 2.5A, we support. For 4), we suggest discussing it after MU restriction (in section 2.6) is settled. If MU restriction is agreed, we think Alt 3 is acceptable; Otherwise, taking Alt 1 is more reasonable from our perspective. Alt 2 needs clarification. The signaling whether is based on DCI or RRC is related to the discussion in section 2.4. In summary, we think 4) will be automatically settled after discussion in 2.4 and 2.6 are concluded. |
| LGE | Support the proposal. For item 4), We prefer Alt.3. |
| Apple | We support 1) – 3) in the FL proposal. For 4), we think that Alt 1) should be the baseline UE capability i.e. UE don’t expect MU-MIMO within the CDM group. Additionally, Alt2) could be advanced UE capability to indicate support for MU-MIMO within the CDM group |
| MediaTek | FL Proposal 2.5A 1)-3), Support. We Support Alt 1 for 4). |
| Ericsson | 2.5A 1) Support 2) Fine in principle if there’s such need from majority companies. 3) We don’t think we need to limit the gNB scheduling flexibility, it should be allowed for the co-scheduling of Rel-18 DMRS port {8,9} and Rel-18 DMRS {0,1}. What is the meaning of this restriction? 4) Support Alt.2. It makes sense to tell the UE configured with Rel-15 DMRS, that it will not be co-scheduled in same CDM group with Rel-18 DMRS. We don’t support Alt.1, the purpose of Rel-18 DMRS is to allow such MU-MIMO scheduling. |
| China Telecom | For 4), Alt.2 should be supported so that the dynamic switching between R15 and R18 can be guaranteed. |
| vivo | It is unnecessary to introduce additional restriction for co-scheduling legacy port and Rel-18 port in MU-MIMO. Please note there is no such restriction to restrict the orthogonality of co-scheduled DMRS port in the current spec. The network can even configure different scramblingID of DMRS to UEs in MU-MIMO, which can help non-orthogonal MU-MIMIO scheduling in the current network. It is up to the network to ensure the DMRS ports indicated to UEs are orthogonal as much as possible. |
| Spreadtrum | Support 1) – 3) in the FL proposal. For 4), we think it can be achieved by gNB implementation. Alt3 is reasonable, but we are also fine with Alt1. |

  - MU-MIMO scheduling restriction within a CDM group

In section 5.1.6 in TS38.214, MU-MIMO scheduling restriction is specified as following.

For DM-RS configuration type 1, - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 30} in Table 7.3.1.2.2-1 and Table 7.3.1.2.2-2 of Clause 7.3.1.2 of [5, TS 38.212], or - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 12} in Table 7.3.1.2.2-1A and {2, 9, 10, 11, 30 or 31} in Table 7.3.1.2.2-2A of Clause 7.3.1.2 of [5, TS 38.212], or - if a UE is scheduled with two codewords, the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE. For DM-RS configuration type 2, - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10 or 23} in Table 7.3.1.2.2-3 and Table 7.3.1.2.2-4 of Clause 7.3.1.2 of [5, TS38.212], or - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10, 23 or 24} in Table 7.3.1.2.2-3A and {2, 10, 23 or 58} in Table 7.3.1.2.2-4A of Clause 7.3.1.2 of [5, TS 38.212], or - if a UE is scheduled with two codewords, the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.

After Rel.18 eType1/eType2 DMRS ports tables are defined, which row of DMRS port combination requires MU-MIMO restriction will be specified. It is not proper to just reuse the indexes of rows which require MU-restriction in current Rel.17 spec. For example, for Rel.15 Type 1, if row 2 (i.e. {0,1} with number of CDM group without data = 1) is indicated, remaining DMRS ports are not used to another UE. This is because there is no remaining orthogonal DMRS ports in Rel.15 Type 1. However, in Rel.18 eType 1, if row 2 (i.e. {0,1} with number of CDM group without data = 1) is indicated, there are remaining DMRS ports of {8,9}, and it is straightforward to allow DMRS ports combination of {8,9} with number of CDM group without data = 1 to another UE. Hence, we cannot reuse the indexes of rows which require MU-restriction in current Rel.17 spec.

Qualcomm [21] and OPPO [6] propose the following principle. Apple proposes similar proposal. Let’s try the following proposal.

FL Proposal 2.6A

- Adopt the following MU scheduling restriction for Rel.18 DMRS ports for PDSCH:

  - If the DMRS ports of a UE are in more than one CDM groups, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE.

| Company | Comment |
| --- | --- |
| Docomo | OK |
| Google | Similar to our previous comments, we can defer the decision after we reach a consensus in 2.4. In addition, eType1/eType2 is introduced for high order MU-MIMO. We need to be more careful to define such MU-MIMO restrictions. |
| OPPO | Support the proposal. |
| ZTE | Do not support. Note that the double number of DMRS ports can be enabled by Rel-18 DMRS, it is better to support more co-scheduled UEs especially in MU-MIMO scenario. |
| CMCC | Support |
| CATT | Do not support. If multiple ports are not occupied, MU-MIMO can still be performed. |
| Lenovo | It seems it is related with conclusion of section 2.1 i.e. DMRS port table design and conclusion of section 2.4 i.e. dynamic switching between different OCC length. It is preferred to be further discussed after some conclusions are available. |
| Sharp | Support |
| Xiaomi | Support. |
| Huawei, HiSilicon | Do not support FL Proposal 2.6A. The number of MU pairing layers is over-restricted. |
| Fraunhofer IIS/HHI | Support |
| Nokia/NSB | Generally fine with the idea. The Rel-15 restriction is intended to limit application of FD-OCC when high-delay spread channel. If this is applied to Rel-18 DMRS, it should be for no FD-OCC or FD-OCC2. With FD-OCC2, MU-MIMO of UE1 (2+1), UE2 (1) are not supported with such configuration. Dynamic switching is much more clear to support this. |
| Futurewei | Not support. The proposed restriction will reduce the supportable MU-MIMO pairing scenarios. Note that one of the main goals of this agenda item is to better support MU-MIMO with larger number of DMRS ports. |
| Samsung | If the proposal has same principle with Rel-15 DMRS design, we are generally fine. |
| QC | We support Proposal 2.6A. But the proposal only covers the case of single symbol DMRS. For dual symbol DMRS, we need one similar restriction on TD-OCC level, not on CDM group level. As we explained in our Tdoc R1-2301398, for MU port detection and Rhh based noise+interference covariance matrix estimation, in one TD-OCC code of one CDM group, i.e., a box in following figure, even UE is signaled with one DMRS port in a box, UE would need to estimate the channel of all 4 ports in that box. Therefore, if UE is signaled with DMRS ports distributed in two boxes, UE will have to estimate 8 DMRS ports, which is incapable to do for UE with maxNumberMIMO-layersPDSCH = 4 in case of MU. Based on above, we propose add a sub-bullet for two symbol DMRS QC updated FL Proposal 2.6A - Adopt the following MU scheduling restriction for Rel.18 DMRS ports for PDSCH: - If the DMRS ports of a UE are in more than one CDM groups, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE. - The above applies to both single symbol and dual symbol DMRS. - Furthermore, for dual symbol DMRS, if the DMRS ports of a UE are associated with more than one TD-OCC codes in one CDM group, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE. |
| LGE | Support the proposal 2.6A. In addition, regarding DMRS port combination for SU only, we need to consider the maximum number of orthogonal ports per UE in MU-MIMO, first. The below agreement is the maximum number of orthogonal ports per UE in MU-MIMO in Rel-15/16/17 and for the DMRS configuration type 1 with 1-symbol DMRS, rank per UE for MU-MIMO is limited to 2. Since the maximum number of ports is doubled in Rel-18 DMRS, it makes sense that rank per UE for MU-MIMO is limited to 4 for the DMRS configuration type 1 with 1-symbol DMRS in Rel-18. Agreement in RAN1#90bis The maximum number of orthogonal ports per UE in MU-MIMO for - DMRS configuration type 1 with 1-symbol DMRS - 2 for DL - DMRS configuration type 1 with 2-symbol DMRS - 4 for DL - DMRS configuration type 2 with 1-symbol DMRS - 4 for DL - DMRS configuration type 2 with 2-symbol DMRS - 4 for DL. |
| Apple | In principle, we are fine with FL’s proposal and also support the updated proposal from QC |
| MediaTek | Agree with the comments made by Qualcomm and support the updated proposal by them. |
| Ericsson | We don’t support proposal 2.6A. We don’t want to have such restriction for Rel-18 DMRS aiming for MU-MIMO use case. |
| China telecom | Do not support. We wonder why the MU-MIMO can’t be scheduled when the supported DMRS ports for one CDM group has been doubled as long as there is no conflict in the scheduling for different UEs. |
| vivo | Support. What in proposal is following the R15 design principle. In the current spec, the entries of DMRS port combination across multiple CDM groups have been restricted to only be used for SU-MIMO. |
| Spreadtrum | Support the proposal. For MU-MIMO, the scheduled DMRS ports for different UEs should be within different CDM groups. |

  - Other proposals

Following proposals are also proposed.

| Proposals | Companies |
| --- | --- |
| - PTRS-DMRS association for Rel.18 DMRS ports | Lenovo, Huawei/HiSilicon |
| - Study how to support dynamic switching between different number of additional DMRS symbols in Rel-18 | Ericsson |
| - Sequence mapping equation needs to be modified to ensure that Rel.18 DMRS and Rel.15 DMRS have the same DMRS pattern | Lenovo |
| - Study on OCC disabling scheme for new DMRS type (Rel.17 feature in above 52.6GHz). | Samsung |
| - Additional support of TD-OCC schemes (Opt.3/5) | Support: Ericsson Concern: vivo, Spreadtrum, Intel, Fraunhofer IIS/HHI, MediaTek |
| - Reusing low PAPR design for Rel.18 DMRS port(s) | Huawei/HiSilicon |
| - support eType1 DMRS for MsgA PUSCH. | Lenovo |

Please provide your views on the above proposals, or other aspects which are not included in the summary, if any.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | For PTRS-DMRS association, seems it is already included in section 3? For Low PAPR design for Rel.18 DMRS port(s), it should be clarified that what we mentioned is only reusing the low PAPR design in Rel.16 rather than introducing new sequence design. Our previous discussion only focus on the FD-OCC design, while root sequence, which is also an important ingredient of DMRS sequence, is not decided yet. Here we only want to confirm all the design in previous releases is automatically inherited. |

- Specifying objective #5 (>4 layers PUSCH DMRS)

  - Antenna port(s) table for PUSCH (rank 5-8)

In RAN1#111, the following agreement was made.

Agreement - For > 4 layers PUSCH, support new antenna ports tables for rank = 5,6,7,8 for both single-symbol/double-symbol DMRS. - For Type 1/Type 2 Rel.15 DMRS ports, new antenna ports tables are the following: - The same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook. - For Rel.18 eType1/eType2 DMRS ports, - New antenna ports tables with new DMRS port combinations are used for rank = 5,6,7,8 (FFS: details). - Note: Whether the DMRS port combination allows to use single symbol DMRS for rank = 5,6,7,8 should be checked. - FFS: For partial coherent UL codebook, support layers to DMRS port mapping that layers associated to the same antenna port group are multiplexed into the same DMRS CDM group. - FFS: One or more than one DMRS port combination(s) for each rank and TPMI - Note: New DMRS port combinations above does not preclude the new antenna ports tables including the current DMRS port combination(s) for PDSCH for rank = 5,6,7,8 in Rel.15-17. - FFS: Whether the antenna ports combinations for rank = 5,6,7,8 can be indicated by the reserved entries of existing antenna ports tables for rank =1,2,3,4, if the rank is indicated together with DMRS antenna ports.

Open issues for PUSCH DMRS ports table are the following.

- (1) DMRS port combinations

  - Rel.15 DMRS:  Already agreed.

  - Rel.18 DMRS:  Need an agreement (Sect.3.1.2).

- (2) Design principle of separate or joint DMRS ports table for rank = 5-8.

- (3) Whether separate DMRS ports table is needed for partial coherent UL codebook.

(2) Design principle of separate or joint DMRS ports table for rank = 5-8.

FL Proposal 3.1A

- To support PUSCH with rank = 5-8, down select from the following options for enhancement of DMRS port allocation tables.

  - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table).

    - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2.

  - Option 2: A joint table including DMRS ports combinations of rank= 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current DL DMRS ports table).

There are following comments in RAN1#111.

OPPO/Huawei/HiSilicon: Opt.1 or Opt.2 depends on how to indicate the rank for 8 port transmission. If the rank is indicated with TPMI as legacy uplink, Option 1 can be applied. If the rank is indicated with DMRS port as legacy downlink, Option 2 can be applied as downlink. CMCC: Similar method as Rel-15 will reduce the specification effort and complexity, and we could not see the benefit of Option 2 to introduce a joint table. Besides, based on Option 1, the reserved bits for antenna ports indication may be further reused for other purposes, such as MCS, RV and NDI for second codeword.

In AI9.1.4.2 in RAN1#111, the following agreement was made. Opt.2 is valid only if “separate indication of TRI and TPMI” is supported in AI9.1.4.2. Also, it is not clear whether such TRI indication will be supported for NCB based 8Tx.

Agreement For CB-based 8TX PUSCH transmission, for rank indication, down-select among the following - Separate indication of TRI and TPMI - Joint indication of TRI and TPMI

As CMCC’s comment above, whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2. From FL perspective, whether joint indication or separate indication of TRI/TPMI should be discussed in AI9.1.4.2. Even if Opt.1 is agreed in AI9.1.3.1, still AI9.1.4.2 can discuss whether/how to reuse the reserved field of antenna ports field, if needed.

Table. Summary of companies’ views for FL Proposal 3.1A

|  | Support Opt.1 | Support Opt.2 |
| --- | --- | --- |
| 3.1A | Docomo, Google, Nokia/NSB (only one or two combinations), Lenovo, CATT, vivo, Fraunhofer IIS/HHI, LG, ZTE, NEC, CMCC, Spreadtrum , Intel, OPPO, Apple | OPPO, HW/HiS, Xiaomi, Sharp |

(3) Whether separate DMRS ports table is needed for partial coherent UL codebook.

FL Proposal 3.1B

- For > 4 layers PUSCH with Rel.15 Type1/Type2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports, for partial coherent UL codebook, down select from the following:

  - Alt.1: DMRS ports combination(s) that the same antenna group into the same DMRS CDM group.

  - Alt.2: DMRS ports combination(s) for full/non-coherent UL codebook is reused.

    - Note: DMRS ports combination(s) that the same antenna group into the same or different DMRS CDM group.

| Support Alt.1 | Support Alt.2 |
| --- | --- |
| IDC, Nokia/NSB, Sharp, Docomo | OPPO, Xiaomi (current principle), CATT, CMCC |

| Company | Comment |
| --- | --- |
| Docomo | Proposal 3.1A: Support Opt.1. Another problem of Opt.2 is for Rel.15 DMRS ports, existing DMRS ports tables are used for rank = 1-4. It is not reasonable to use joint indication of TRI/TPMI for rank = 1-4 and separate TRP/TPMI for rank 5-8 for R15 DMRS ports. Proposal 3.1B: We are fine with Alt.1. |
| Google | Proposal 3.1A: Support option 1, which is the same principle as legacy. Alt2 would increase the DCI overhead. Proposal 3.1B: Support Alt2. We failed to see the benefit to define the DMRS ports combination based on the indicated precoder. |
| OPPO | Proposal 3.1A: Support option 1 as legacy. Proposal 3.1B: Alt2. |
| ZTE | FL Proposal 3.1A: Support option 1. On the one hand, it is aligned with the legacy rules. On the other hand, rank is indicated by TPMI/SRI field for PUSCH transmission, it is different from that of PDSCH where layer number is indicated by DMRS indication jointly. FL Proposal 3.1B: Support Alt. 2. According to the legacy rules, DMRS port combinations can be from the same or different CDM groups no matter full/partial/non-coherent UL codebook. We fail to see the reason the make the new restriction in Rel-18 specially. |
| CMCC | Proposal 3.1A: Support Opt.1. Proposal 3.1B: Support Alt2. |
| CATT | Proposal 3.1A: Support Opt.1 except for the FFS. We don’t think whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2. If the reserved field in antenna ports field is used to indicate DMRS port associated with PTRS, it should be discussed in this AI. Proposal 3.1B: Support Alt 2. DMRS ports combination(s) that the same antenna group into different DMRS CDM groups is supported in Rel-15, we don’t see more restriction is needed for Rel-18. |
| Lenovo | Proposal 3.1A: Support Opt.2. Proposal 3.1B: Support Alt.1. |
| Sharp | FL Proposal 3.1A: We can go with the majority for progress. FL Proposal 3.1B: Support Alt 1 to mitigate interference between antenna port groups. |
| Xiaomi | Proposal 3.1A: we prefer option 2 to use joint DMRS tables for RANK5-8. Proposal 3.1B: Support alt.2. |
| Huawei, HiSilicon | FL Proposal 3.1A: Similar to the previous discussion under rank 1~4, whether to use separate/joint table should be delayed after the indication of TRI has been decided in 9.1.4.2. FL Proposal 3.1B: Support Alt.2. The benefit of Alt.1 is not clear. |
| Fraunhofer IIS/HHI | FL proposal 3.1A: Prefer Alt. 1 FL proposal 3.1B: Fine with Alt. 2. |
| InterDigital | Proposal 3.1B: Support Alt.1. We’re concerned with the loss of coherency if DMRS ports of the same CDM group are mapped to different antenna groups. This would lead to CHEST degradation and poor demodulation. |
| Nokia/NSB | FL Proposal 3.1A: We don’t think DMRS table is really needed for rank 5-8. According to the discussion in AI9.1.4.2, we can discuss further. FL Proposal 3.1B: Support Alt 1. For partial coherent case, the precoded layers may have different power according to the number antenna ports to layers mapping, and multiplexing non-coherent DMRS ports with different power causes performance degradation. |
| LGE | Proposal 3.1A: Support opt. 1 as legacy. Proposal 3.1B: Support alt. 2. |
| Apple | FL Proposal 3.1A: We support option 1 |
| Ericsson | 3.1A: Support Option1. 3.1B: Support Alt.2. |
| vivo | Proposal 3.1A: Support Option 1. Proposal 3.1B: Support Alt 2. |
| Spreadtrum | Proposal 3.1A: Support Option 1. Proposal 3.1B: Support Alt 2. |

3.1.1 For Rel.15 DMRS ports

In RAN1#111, following was agreed.

- For Type 1/Type 2 Rel.15 DMRS ports, new antenna ports tables are the following: - The same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook.

Based on the agreement for R15 DMRS ports, DMRS ports combination(s) for rank = 5-8 PDSCH is simply reused for PUSCH. Assuming Opt.1 will be agreed in FL proposal 3.1A, following proposal is suggested.

FL Proposal 3.1.1A

- Adopt Table 7.3.1.1.2-12B/13B/14B/15B/16B/17B/20B/21B/22B/23B to support signalling >4 ranks PUSCH with Rel-15 DMRS ports.

- FFS: Whether/how some of bits in the antenna ports field can be reused for other purpose for >4 ranks PUSCH.

Table 7.3.1.1.2-12B: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-4 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-13B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-14B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-16B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-4 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-17B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-5 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-20B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-4 | 1 |
| 1 | 2 | 0,1,2,3,6 | 2 |
| 12-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-21B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-5 | 1 |
| 1 | 2 | 0,1,2,3,6,8 | 2 |
| 2-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Support. Some companies may prefer to select one row for each table, but in our understanding, the one or two DMRS port combination(s) in the above tables are already agreed because of “The same DMRS port combination(s)” in the agreement. |
| Google | For Table 20/21/22, we suggest reserving more ports to legacy UEs, similar to our comments in 2.1. |
| OPPO | Fine with the proposal. |
| ZTE | It is proper to further discussed based on the outcome of section 2.1 due to “The same DMRS port combination(s)” in the agreement. |
| CMCC | Support |
| CATT | Support. We are also fine with removing the tables with only one available entry. |
| Lenovo | Support |
| Sharp | Antenna port(s) table should be defined after the discussion in FL Proposal 3.1A/B. If assuming Option 1 in FL Proposal 3.1A is agreed, we suggest adding “at least for full or non-coherent UL codebook.” into FL Proposal 3.1.1A. |
| Xiaomi | Support the proposal |
| Huawei, HiSilicon | Fine with the DMRS port combinations, but again, whether to use separate/joint table should be delayed after the indication of TRI has been decided in 9.1.4.2. |
| Fraunhofer IIS/HHI | Support |
| Nokia/NSB | Same view with Huawei. Support only the combinations. AI9.1.4.2 is still discussing on the signaling of rank and TPMI. |
| QC | We don’t support this proposal. We acknowledge there is an existing agreement. But we hope RAN1 can reconsider this problem, because it is beneficial to map the layers of 1 CW to DMRS in one CDM group, to simplify gNB receiver for PUSCH decoding. Therefore, we suggest to slightly update the entries as the following. Please notice the changes are very minor. We either change only one index in the DMRS ports combination, or switch the order of two ports in the combination, except that we delete table 16B and 17B. Table 7.3.1.1.2-12B: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 5 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,6; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-13B: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 6 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,4,2,3,6; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-14B: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 7 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,4,2,3,6,7; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-15B: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 8 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,4,5,2,3,6,7; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-16B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=5 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 3; 0-4. 1-15; Reserved; Reserved. Table 7.3.1.1.2-17B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=6 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 3; 0-5. 1-15; Reserved; Reserved. Table 7.3.1.1.2-20B: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=2, rank=5 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 3; 0-4; 1. 1; 2; 0,1,2,3,8; 2. 12-31; Reserved; Reserved; Reserved. Table 7.3.1.1.2-21B: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=2, rank=6 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 3; 0-5; 1. 1; 2; 0,1,6,2,3,8; 2. 2-31; Reserved; Reserved; Reserved. Table 7.3.1.1.2-22B: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=2, rank=7 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,6,2,3,8,9; 2. 1-31; Reserved; Reserved; Reserved. Table 7.3.1.1.2-23B: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=2, rank=8 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,6,7,2,3,8,9; 2. 1-31; Reserved; Reserved; Reserved. |
| LGE | Agree with ZTE. And prefer to select one row for each table. |
| Apple | We are fine with the FL’s proposal |
| MediaTek | Support |
| Ericsson | Fine with the proposal 3.1.1A. |
| China Telecom | Support. |
| vivo | Support. |
| Spreadtrum | Support |

3.1.2 For Rel.18 DMRS ports

Vivo, CATT, CMCC, Docomo, etc. propose to reuse DMRS port combinations for PDSCH with Rel.18 DMRS ports, and it would require waiting the outcome of sect. 2.1 to discuss the detail DMRS port combination for Rel.18 DMRS ports.

FL Proposal 3.1.2A (To be discussed later)

- For > 4 layers PUSCH with Rel.18 eType 1/eType 2 DMRS ports, reuse the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH with Rel.18 eType 1/eType 2 DMRS ports at least for full or non-coherent UL codebook.

  - Max number of PTRS ports

We discussed the following proposal since RAN1#110bis-e, however, there was no consensus. We have to finalize the discussion because it impacts on discussion in sect. 3.3.

FL proposal#3.2A:

- For 8Tx PUSCH, support up to 4 ports PTRS for CP-OFDM.

Support/fine: InterDigital, CATT, Lenovo, Apple, Qualcomm, Xiaomi, LGE Docomo, MediaTek, CMCC

No: Samsung, OPPO, Nokia/NSB, vivo, IDC? (at least 2 ports), Spreadtrum,

If the above proposal is not acceptable, following will be the consequence. It means 2-port PTRS in the current spec.is reused.

FL proposal#3.2B: (for conclusion)

- For 8Tx PUSCH, no consensus to support up to 4 ports PTRS for CP-OFDM.

Support/fine: ZTE, Spreadtrum, OPPO, vivo, Samsung, Nokia/NSB, Docomo,

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Support FL Proposal 3.2A, because Ng (antenna coherent groups) was agreed with 1, 2, 4. If different antenna groups do not share the same PA, different phase noise would be observed for different antenna groups. Hence, each DMRS port(s) should be associated with one PTRS port, and the total number of PTRS ports should be up to 4. But, considering the situation, we can accept FL proposal#3.2B. |
| Google | Support 3.2B. |
| OPPO | Support the proposal 3.2B. We think two PTRS ports are sufficient. Otherwise, we may need 8 PTRS ports for non-coherent UEs. |
| ZTE | Note that 8Tx UL transmission mainly target for FR1, we slightly prefer proposal#3.2B. |
| CMCC | Support Proposal 3.2A. If four antenna groups are assumed for UE’s antenna layouts, the phase noise of different antenna groups may be different, and the maximal PT-RS ports to be configured is 4. There may be some UEs with four antenna groups, but the phase noise of two antenna groups is same, in this case, the maximal PT-RS ports to be configured is 2. So, the maximal PT-RS ports can be UE capability. |
| Lenovo | Support FL Proposal 3.2A |
| Sharp | Support 3.2B. The number of PTRS ports depends on the number of oscillators, and the necessity of 1-to-1 mapping of an oscillator to an antenna port group is unclear in our view. |
| Xiaomi | Support Proposal 3.2A, same view with DCM that this enhancement for PTRS ports is needed since Ng=4 is supported for 8TX UE. In FR2, it would be problematic if limited to only 2 ports. |
| InterDigital | Support Proposal 3.2A. Our preference is to support up to 4 PTRS ports. |
| Nokia/NSB | Support proposal 3.2B. We have already not supported 4 PTRS for rank 4 non-coherent codebook. If we support utpo 4 PTRS for Ng=8, then do we also need to support 8 PTRS for non-coherent 8TX? |
| Samsung | Support Proposal 3.2B. We think that up to 2 PTRS ports as current spec is enough. |
| QC | We support FL Proposal 3.2A, based on the reason as DOCOMO mentioned. And we don’t think it is feasible for antenna groups to share a same PA/Oscillator. We don’t agree with FL Proposal 3.2B, which effectively excluding Ng=4 for 8 Tx in Rel-18. If we have to take the conclusion due to controversial views, we request to add a note to the conclusion. Updated FL proposal#3.2B: (for conclusion) - For 8Tx PUSCH, no consensus to support up to 4 ports PTRS for CP-OFDM. Note: This conclusion effectively excluded the support of Ng=4 for 8 Tx PUSCH with the use of PTRS. |
| LGE | We prefer proposal 3.2A but we open to majority view. |
| Apple | We support FL proposal 3.2A As described in our contribution, antenna port group has its own independent phase noise in the worst case, and if we support up to 8 Tx UL partial-coherent operation over 4 port groups (panels) with up to 8 layers, we need the 4 port PTRS |
| MediaTek | Agree with the comments and updates (added Note) made by Qualcomm to FL proposal#3.2B |
| Ericsson | We are fine with proposal 3.2A |
| China Telecom | Sightly support the proposal 3.2B, but can live with 3.2A. |
| vivo | Support 3.2B. |
| Spreadtrum | Support FL Proposal 3.2A |

  - PTRS-DMRS association

One port PTRS

In RAN1#111 meeting, the following agreement was made. In this meeting, we need to down select one from the following.

Agreement - For full-coherent PUSCH with rank 5-8, UE shall expect only one port PTRS to be configured. - Down select from the following in RAN1#112: - Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2. - FFS: Association with the CW with the higher MCS. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port with the CW with the higher MCS. 1; 2nd scheduled DMRS port the CW with the higher MCS. 2; 3rd scheduled DMRS port the CW with the higher MCS. 3; 4th scheduled DMRS port the CW with the higher MCS. - Alt.2: The size of PTRS-DMRS association field is 3bit in DCI format 0_1/0_2, and the following PTRS-DMRS association for UL PTRS port 0 is specified in TS38.212. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port. 1; 2nd scheduled DMRS port. 2; 3rd scheduled DMRS port. 3; 4th scheduled DMRS port. 4; 5th scheduled DMRS port. 5; 6th scheduled DMRS port. 6; 7th scheduled DMRS port. 7; 8th scheduled DMRS port.

FL Proposal 3.3A

- For full-coherent PUSCH with rank 5-8 with one port PTRS, select [Alt.1 or Alt.2 (To be updated)] in the RAN1#111 agreement.

For Alt.1, OPPO, vivo suggests the following clarification.

- If the MCS is the same for two CWs, the PTRS port is associated with the first CW.

| Support Alt.1 | Support Alt.2 |
| --- | --- |
| HW/Hi, OPPO (with the clarification), Google, vivo (with the clarification), CATT, NEC, Sharp, Samsung | IDC, ZTE, Docomo, Spreadtrum,Lenovo, LG, Xiaomi, Ericsson, CMCC, Nokia/NSB |

Two port PTRS

FL note: The following is only applied to 2-port PTRS. 4-port PTRS (if supported) can be discussed separately.

FL proposal#3.3B: (for two PTRS port)

- For two PTRS ports for partial/non-coherent PUSCH, PTRS-DMRS association for PUSCH with up to 8 layers is determined by 4-bit PTRS-DMRS association field by the following table.

Table: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

Supported by: Lenovo, LG?, Xiaomi, CATT, CMCC, Docomo ….

Nokia/NSB mentioned to postpone the discussion until TPMI design is agreed for partial/non-coherent.

| Company | Comment |
| --- | --- |
| Docomo | Proposal 3.3A: Support Alt.2. PUSCH with rank 5-8 will be scheduled in high RSRP scenario and reducing DCI size seems to be not so important. Proposal 3.3B: Support. |
| Google | Proposal 3.3A: Support Alt1. Actually, this is the same principle as DL. Proposal 3.3B: Probably we do not need to support 2 port PT-RS for >4 layers, since currently STxMP only supports up to 4 layers. |
| OPPO | Proposal 3.3A: Support Alt1. We don’t think the flexibility is worth one more bit overhead. Furthermore, since only single symbol DMRS is supported when PTRS is configured, for full-coherent PUSCH with rank 5-8 based on Rel-15 DMRS, only 4 (type 1) or 6 (type 2) DMRS ports can be indicated. Alt2 would lead to waste of bit. Proposal 3.3B: Support. |
| ZTE | Proposal 3.3A: Support Alt 2. Regarding Alt. 1, one issue is how to indicate the case that two CWs with the same MCS level? One may assume that it can be the DMRS ports associated with first CW. If so, it deviates from the principle of Alt .1 that MCS level is used to down select DMRS ports at first. Otherwise, 1 out of 8 DMRS ports needs to be selected in this case, which is the same to Alt .2 in fact. Proposal 3.3B: Support. |
| CMCC | Proposal 3.3A: Support Alt.2. Proposal 3.3B: Postpone the discussion until 2-ports and/or 4-ports PTRS has been agreed. |
| CATT | Proposal 3.3A: Support Alt1. When the two CWs are with different MCSs, it is natural to transmitting the PTRS port according to a DMRS port for the CW with the higher MCS. Proposal 3.3B: Not support. Since there are lots of reserved entries of antenna ports field, some bits of antenna ports field can be used to indicate DMRS ports for PTRS transmission. The following scheme is preferred: - The size of PTRS-DMRS association field is 2 bits in DCI format 0_1/0_2, - For PUSCH transmission with rank > 4, the PTRS-DMRS association field is for UL PTRS port 0 and 2 bits in Antenna port filed are for UL PTRS port 1; - For PUSCH transmission with rank = 1, 2, 3, 4, the PTRS-DMRS association field is used for DMRS indication for UL PTRS port(s). |
| Lenovo | Proposal 3.3A: Support Alt.2. Proposal 3.3B: Support. |
| Sharp | Proposal 3.3A: Support Alt 2. Alt 1 may have potential large overhead of PTRS when the difference between MCS levels is large. Furthermore, how to select one of two CWs for the same MCS is unclear. Proposal 3.3B: Support. |
| Xiaomi | Proposal 3.3A: Support Alt 2. Proposal 3.3B: support |
| Huawei, HiSilicon | FL Proposal 3.3A: Support Alt1. DCI overhead is always an important issue that need to be carefully treated. Considering layers corresponding to the CW with higher MCS has a large probability to contain the layer of good channel quality, Alt.1 is undoubtedly a good compromise between PTRS performance and DCI overhead. Furthermore, if companies really value the unnecessary high flexibility, the redundant bits in antenna port field can be further combined with the PTRS-DMRS association field to maintain the same overhead. FL Proposal 3.3B: Not support. |
| InterDigital | Proposal 3.3A: Support Alt2 Proposal 3.3B: Support |
| Nokia/NSB | Proposal 3.3A: Support Alt 2. Proposal 3.3B: support |
| Samsung | Proposal 3.3A: Support Alt1 with clarification. Proposal 3.3B: We prefer to discuss after finalizing TPMI design on partial/non-coherent cases. |
| QC | Proposal 3.3A: we prefer Alt 2. Proposal 3.3B: Support the proposal. |
| LGE | Proposal 3.3A: Support Alt.2. Proposal 3.3B: Support. |
| Apple | Proposal 3.3A: Alt 2 Proposal 3.3B: Support |
| MediaTek | Proposal 3.3A: Alt 2 Proposal 3.3B: Support |
| Ericsson | 3.3A: Support Alt.2 3.3B: Support |
| China Telecom | Proposal 3.3A: Support Alt 2. Proposal 3.3B: Support. |
| vivo | Proposal 3.3A: Support Alt.1. When two codewords are transmitted for PUSCH, PTRS port is associated with DMRS port mapped in the target codeword with higher MSC. If two codewords have the same MCS, the first codeword is selected as the target codeword. Besides, we think the legacy table in spec can be reused without the modification of “with the higher MCS” in each row, after the target CW is determined. The table is obviously for the target CW, i.e., “the CW” implies the target CW in Rel-18 and the only one CW (up to rank=4) in Rel-15. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port with the CW with the higher MCS. 1; 2nd scheduled DMRS port the CW with the higher MCS. 2; 3rd scheduled DMRS port the CW with the higher MCS. 3; 4th scheduled DMRS port the CW with the higher MCS. Proposal 3.3B: It should be discussed after finalizing codebook/TPMI design on partial/non-coherent cases. Based on specific codebook/TPMI design, 2 bits can also be kept for partial/non-coherent cases with two PTRS ports. |
| Spreadtrum | Proposal 3.3A: Support Alt 2. Proposal 3.3B: Support |

  - PTRS power boosting

In ZTE[4], Spreadtrum[5], OPPO[6], Ericsson[14], Qualcomm[21], Lenovo[10], PTRS power boosting for PUSCH with rank>4 are discussed.

Qualcomm summarizes principle of PTRS power boosting in Rel.15.

Qualcomm: Based on the above analysis, we then look at Rel-15 PTRS power boost table, which is copied below and verify how it works with the above two sources to borrow power and the associated two criteria. Table 6.2.3.1-3: Factor related to PUSCH to PT-RS power ratio per layer per RE UL-PTRS-power /; The number of PUSCH layers ( ). 1; 2; 3; 4. All cases; Full coherent; Partial and non- coherent and non-codebook based; Full coherent; Partial and non- coherent and non-codebook based; Full coherent; Partial coherent; Non-coherent and non-codebook based. 00; 0; 3; 3Qp-3; 4.77; 3Qp-3; 6; 3Qp; 3Qp-3. 01; 0; 3; 3; 4.77; 4.77; 6; 6; 6. 10; Reserved. 11; Reserved. Note 1: Qp is the number of PTRS ports that the UE is configured with. Note 2: Row 00 is the UE with “small” PAs where every single PA can only generate full power (w.r.t UE’s power class)/# Tx. Row 01 is the UE with “big” PAs where every single PA can generate full power (w.r.t UE’s power class). Whether UE use row 00 or 01 is signaled by RRC parameter ptrs-Power The following are the details of the rationale why Rel-15 PTRS power boost table is defined as in Table 6.2.3.1-3 in spec. - For single layer PUSCH, PTRS power boost is not allowed, as neither of the two sources for power borrowing is available. - For 2-layer or 3-layer PUSCH, - With full coherent precoder, 3dB (with 2-layer PUSCH) or 4.77dB (with 3-layer PUSCH) power boost is always available regardless of the PA capability (i.e., row 00 or 01), because UE can always borrow power from source 1 (other PUSCH layers) by lighting up all antennas to transmit a PTRS port. - With partial or noncoherent precoder, UE cannot light up all Tx antennas to transmit a PTRS port. Therefore, whether power boost is available or not depends on PA capability, i.e., row 00 or row 01. When “big” PAs are used, i.e., row 01, 3dB or 4.77dB power boost is always available because UE can always borrow power from source 1 (other PUSCH layers), even by lighting up one Tx antenna. However, when “small” PAs are used, i.e., row 00, UE cannot borrow power from source 1 (other PUSCH layers). UE can only borrow power from source 2 (other PTRS tone). Therefore, when Qp=1, PTRS power boost is not available, while when Qp=2, PTRS power boost can be 3dB. - For 4-layer PUSCH, - With full coherent precoder, 4dB power boost is always available, based on the same reason as for 2 or 3 layers PUSCH. - With partial coherent precoder - For row 01, 6dB power boost is always available because UE can always borrow power from source 1 (other PUSCH layers), even by lighting up one Tx antenna. - For row 00, with Qp=1, UE can do 3dB power boost for PTRS because the PTRS port can borrow power from 1 PUSCH layer which is coherent with the PUSCH layer that PTRS is associated with. With Qp=2, UE can do 6dB power boost for PTRS because UE can further borrow power from source 2, in addition to the one PUSCH layer in source 1. - With noncoherent precoder - For row 01, 6dB power boost is always available because UE can always borrow power from source 1 (other PUSCH layers), even by lighting up one Tx antenna. - For row 00, UE cannot borrow power from source 1 (other PUSCH layers). UE can only borrow power from source 2 (other PTRS tone). Therefore, when Qp=1, PTRS power boost is not available, while when Qp=2, PTRS power boost can be 3dB. The above rationale for Rel-15 PTRS power boost looks very complicated. However, it can be summarized into the following a few simple principles. - Principle 1: for a UE where each Tx port can support full power transmission w.r.t. UE power class (row 01), the PTRS to PUSCH power ratio is 10log10(L), where L is the total number of PUSCH layers. - Principle 2: for a UE where not each Tx can support full power transmission w.r.t. UE power class (row 00), the PTRS to PUSCH power ratio is determined as the following - Principle 2.1: For fully coherent TPMIs, PTRS to PUSCH power ratio is 10log10(L), where L is the total number of PUSCH layers. - Principle 2.2: For noncoherent TPMIs, PTRS to PUSCH power ratio is 10log10(Qp), where Qp is the number of PTRS ports configured to the UE. - Principle 2.3: For partial coherent TPMIs, PTRS to PUSCH power ratio for PTRS port x is min{10log10(Lx)+10log10(Qp),10log10(L)}, or where Lx is the number of PUSCH layers which are precoded coherently with the PUSCH layer where PTRS port x is associated with.

Based on reviewing the tdocs, I think the principle of row 01 and row 00 with full/con-coherent are reasonable at least. For row 00 with partial-coherent, Ericsson proposes to postpone the discussion. Hence, FL proposal#3.4A is provided.

For partially coherent codebooks, the power boosting is more complicated and can for example depend on the number of antenna groups, the number of PTRS ports, the PA architecture, potential rank-restrictions, and PMI-restrictions introduced in Rel-18 to reduce DCI overhead. Due to that several of these aspects have yet nor been decided in 3GPP, for example the maximum number of PTRS ports for partially coherent UEs (2 or 4), and potential rank/TPMI restrictions to reduce the DCI overhead, the power boosting tables for partially coherent UEs can be postponed until further agreements have been made in the 8 TX UL topic.

FL proposal#3.4A:

- For 8Tx PUSCH, specify PUSCH to PTRS power ratio per layer per RE () based on the following principles.

  - Principle 1: When the ptrs-Power configures 01, the PTRS to PUSCH power ratio is 10log10(L), where L is the total number of PUSCH layers.

  - Principle 2: When the ptrs-Power configures 00, the PTRS to PUSCH power ratio is determined as the following

    - Principle 2.1: For fully coherent TPMIs, PTRS to PUSCH power ratio is 10log10(L), where L is the total number of PUSCH layers.

    - Principle 2.2: For noncoherent TPMIs, PTRS to PUSCH power ratio is 10log10(Qp), where Qp is the number of PTRS ports configured to the UE.

    - FFS:  the PTRS to PUSCH power ratio for partial coherent TPMIs

  - FFS: Whether the transmission power should be consistent within the whole transmission occasion for PUSCH.

  - Send LS to RAN4 to inform that RAN1 made the above agreement.

| Company | Comment |
| --- | --- |
| Docomo | Fine with the proposal. |
| Google | We think we need to add another principle as follows. The UE cannot change the Tx power too frequently, e.g., symbol by symbol. We suggest we add FFS for principle 2.2 as well. Principle 3: The transmission power should be consistent within the whole transmission occasion for PUSCH. |
| OPPO | In Rel-15, the PTRS power boosting is only configured via RRC parameter ptrs-Power with row 00 and 01 but not depended on capability of full power transmission. Hence, we suggest to use “When the ptrs-Power configures 01” and delete “for a UE where each Tx port can support full power transmission w.r.t. UE power class (row 01)” for principle 1. Also for principle 2. Furthermore, since the PTRS to PUSCH power ratio can be directly derived from the principle, we think it should be an agreement instead of principle. |
| ZTE | Support FL proposal#3.4, it is generally correct and in line with the legacy rules to our understanding. |
| CATT | Support all principles in FL proposal#3.4A. We are fine to agree the table on factor related to PUSCH to PT-RS power ratio with PTRS to PUSCH power ratio derived from the principles. |
| Lenovo | Support the proposal and FFS part on Principle 2.3. |
| Xiaomi | Fine with the proposal. |
| Huawei, HiSilicon | Support FL proposal#3.4A. Also fine with OPPO’s modification. In terms of FFS, we think the specific formula should be removed, which should be decided after the partial-coherent TPMI design has been finished in 9.1.4.2. FFS: Principle 2.3: For partial coherent TPMIs, PTRS to PUSCH power ratio should be decided after the partial-coherent TPMI design has been finished in 9.1.4.2 |
| Nokia/NSB | Generally fine with the principle. But, 12 dB boosting can cause issue. So, we propose to send LS to RAN4, if any impact from 12 dB power boosting of PTRS RE may cause any problem? |
| QC | We are supportive of the proposal. We just have a minor comment on “For PUSCH layers = 5,6,7,8,”. We think this proposal even applies to 8 Tx PUSCH with 1,2,3,4 layers, as current Table in spec only cover 4 Tx PUSCH with 1,2,3,4 layers. So, we suggest cover all layers with a generic statement, “For 8 Tx PUSCH, specify…” |
| LGE | Fine with the proposal. |
| Apple | Fine with the proposal |
| MediaTek | Support |
| Ericsson | Support proposal 3.4A |
| vivo | Fine with the proposal. |
| Spreadtrum | Support the proposal. |

  - PTRS RE mapping

Multiple companies propose to enhance PTRS-RE mapping.

In [22], analysis of Rel.15 PTRS-RE mapping table is provided:

Observation of PTRS RE mapping for Rel.15 DMRS: In the above table, offset00~offset11 can be RRC configured to avoid inter-UE collision, and we can see that sub-carrier of PTRS is one of the sub-carriers of DMRS. For example, for Type1 DMRS, PTRS associated with DMRS port#0 and #1 (CDM group#0) is mapped to one of subcarrier 0,2,4,6,8,10. Since we need to map 8 values from 0,2,4,6,8,10 one example is to map (p=0,1) = (0,2), (4,6), (8,10), (0,2) for offset00, offset01, offset10, offset11. However, in this case, both offset00 and offset11 has the same values, and offset11 has no use case. To avoid this, (p=0,1) = (0,2), (2,4), (6,8), (8,10) is supported. Note that other possible option would be (p=0,1) = (0,2), (4,6), (6,8), (8,10) or (p=0,1) = (0,2), (2,4), (4,6), (8,10), but there would be less difference. The same rule is applied to (p=2,3) = (1,3), (3,5), (7,9), (9,11). For Type2 DMRS, PTRS associated with DMRS port#0 and #1 (CDM group#0) is mapped to one of subcarrier 0, 1, 6, 7. Since we need to map 8 values from 0, 1, 6, 7, one example is to map (p=0,1) = (0,1), (6,7), (0,1), (6,7) for offset00, offset01, offset10, offset11. But, in that case, both offset00/offset10 and offset01/offset11 are the same values, and offset10/offset11 have no use case. To avoid this, (p=0,1) = (0,1), (1,6), (6,7), (7,0) is supported. The same rule is applied to (p=2,3) = (2,3), (3,8), (8,9), (9,2) and (p=4,5) = (4,5), (5,10), (10,11), (11,4). a) Type 1 DMRS b) Type 2 DMRS Figure 3-2. Explanation of PTRS RE mapping in Rel.15.

New PTRS RE mapping for Rel.18 DMRS:

Multiple companies provide proposals of new PTRS RE mapping table for R18 DMRS ports [8, 13, 14, 22].

As simple extension of Rel.15 PTRS RE mapping, for Rel.18 eType1 DMRS, PTRS associated with DMRS port#0,#1,#8,#9 (CDM group#0) is mapped to one of subcarrier 0,2,4,6,8,10. For Rel.18 eType2 DMRS, PTRS associated with DMRS port#0,#1,#12,#13 (CDM group#0) is mapped to one of subcarrier 0,1,6,7.

FL proposal#3.5A:

- For Rel.18 eType1/eType2 DMRS ports for PDSCH/PUSCH, select one from the following for PTRS RE mapping. Down select from the following.

  - Alt 1: Different RE offsets set for different Rel.18 DMRS port indexes as shown in Table 4

Table 4 Different RE offsets set for different Rel.18 DMRS port indexes

| DM-RS antenna port p (p for PUSCH, p+1000 for PDSCH) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DM-RS Configuration type 1 |  |  |  | DM-RS Configuration type 2 |  |  |  |
|  | resourceElementOffset |  |  |  | resourceElementOffset |  |  |  |
|  | offset00 | offset01 | offset10 | offset11 | offset00 | offset01 | offset10 | offset11 |
| 0 | 0 | 2 | 6 | 8 | 0 | 1 | 6 | 7 |
| 1 | 2 | 4 | 8 | 10 | 1 | 6 | 7 | 0 |
| 2 | 1 | 3 | 7 | 9 | 2 | 3 | 8 | 9 |
| 3 | 3 | 5 | 9 | 11 | 3 | 8 | 9 | 2 |
| 4 | - | - | - | - | 4 | 5 | 10 | 11 |
| 5 | - | - | - | - | 5 | 10 | 11 | 4 |
| 8 | 4 | 6 | 10 | 0 | - | - | - | - |
| 9 | 6 | 8 | 0 | 2 | - | - | - | - |
| 10 | 5 | 7 | 11 | 1 | - | - | - | - |
| 11 | 7 | 9 | 1 | 3 | - | - | - | - |
| 12 | - | - | - | - | 6 | 7 | 0 | 1 |
| 13 | - | - | - | - | 7 | 0 | 1 | 6 |
| 14 | - | - | - | - | 8 | 9 | 2 | 3 |
| 15 | - | - | - | - | 9 | 2 | 3 | 8 |
| 16 | - | - | - | - | 10 | 11 | 4 | 5 |
| 17 | - | - | - | - | 11 | 4 | 5 | 10 |

  - Alt 3: Different RE offsets set for different Rel.18 DMRS port indexes as shown in Table 6 (CATT)

Table 6 Different RE offsets set for different Rel.18 DMRS port indexes

| DM-RS antenna port p (p for PUSCH, p+1000 for PDSCH) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DM-RS Configuration type 1 |  |  |  | DM-RS Configuration type 2 |  |  |  |
|  | resourceElementOffset |  |  |  | resourceElementOffset |  |  |  |
|  | offset00 | offset01 | offset10 | offset11 | offset00 | offset01 | offset10 | offset11 |
| 0 | 0 | 2 | 6 | 8 | 0 | 1 | 6 | 7 |
| 1 | 2 | 4 | 8 | 10 | 1 | 6 | 7 | 0 |
| 2 | 1 | 3 | 7 | 9 | 2 | 3 | 8 | 9 |
| 3 | 3 | 5 | 9 | 11 | 3 | 8 | 9 | 2 |
| 4 | - | - | - | - | 4 | 5 | 10 | 11 |
| 5 | - | - | - | - | 5 | 10 | 11 | 4 |
| 8 | 6 | 8 | 0 | 2 | - | - | - | - |
| 9 | 8 | 10 | 2 | 4 | - | - | - | - |
| 10 | 7 | 9 | 1 | 3 | - | - | - | - |
| 11 | 9 | 11 | 3 | 5 | - | - | - | - |
| 12 | - | - | - | - | 6 | 7 | 0 | 1 |
| 13 | - | - | - | - | 7 | 0 | 1 | 6 |
| 14 | - | - | - | - | 8 | 9 | 2 | 3 |
| 15 | - | - | - | - | 9 | 2 | 3 | 8 |
| 16 | - | - | - | - | 10 | 11 | 4 | 5 |
| 17 | - | - | - | - | 11 | 4 | 5 | 10 |

| Support Alt.1 | Support Alt.2 |
| --- | --- |
| Vivo, Ericsson, Docomo | vivo |

| Company | Comment |
| --- | --- |
| Docomo | We support Alt.1, which is simple extension of Rel.15. Alt.2 seems not good because different DMRS ports use the same RE for PTRS. |
| Google | Support Alt1 with bracket for RE offset = [10] and [11]. It is better not transmitting PT-RS in orphan REs. |
| OPPO | Fine with Alt.1 |
| ZTE | Support Alt .1, it is the natural extension of the legacy. |
| CATT | The following table (alternative) which is proposed in our contribution [13] can also be considered, where ∆ is equal to 8 and 12 for Rel.18 eType 1 DMRS and eType 2 DMRS, respectively. Table 37: The parameter krefRE for PTRS mapping (example 1) DM-RS antenna port. DM-RS Configuration eType 1; DM-RS Configuration eType 2. resourceElementOffset; resourceElementOffset. offset00; offset01; offset10; offset11; offset00; offset01; offset10; offset11. (1000+)0; 0; 2; 6; 8; 0; 1; 6; 7. (1000+)1; 2; 4; 8; 10; 1; 6; 7; 0. (1000+)2; 1; 3; 7; 9; 2; 3; 8; 9. (1000+)3; 3; 5; 9; 11; 3; 8; 9; 2. (1000+)4; -; -; -; -; 4; 5; 10; 11. (1000+)5; -; -; -; -; 5; 10; 11; 4. (1000+)∆+0; 6; 8; 0; 2; 6; 7; 0; 1. (1000+)∆+1; 8; 10; 2; 4; 7; 0; 1; 6. (1000+)∆+2; 7; 9; 1; 3; 8; 9; 2; 3. (1000+)∆+3; 9; 11; 3; 5; 9; 2; 3; 8. (1000+)∆+4; -; -; -; -; 10; 11; 4; 5. (1000+)∆+5; -; -; -; -; 11; 4; 5; 10. With the same resourceElementOffset configuration, the frequency interval between two different PTRS ports (which are associated with DMRS ports in the same CDM group) with DMRS eType1 configuration is larger than that in Alt.1. |
| Lenovo | Support Alt.1. Also we are fine with CATT’s proposed table on the above. |
| Huawei, HiSilicon | Considering the higher PTRS collision probability caused by increased DMRS ports, remaining same collision probability within each RB by offloading the mapped REs of PTRS ports associated to Rel.18 new DMRS ports to another RB should be considered. For example, the RE mapping formula can be updated as below: k=krefRE+(iKPT-RS+krefRB+knew)NRBSC krefRB=nRNTI mod KPT-RS, if NRB mod KPT-RS=0 nRNTI mod NRB mod KPT-RS, otherwise For KPT-RS=2, knew=1 if krefRB=0; knew=-1 if krefRB=0； For KPT-RS=4, knew=2 if krefRB=0 or 1; knew=-2 if krefRB=2 or 3. Notice that the mapping formula above targets RB-level randomization, i.e., it is compatible with any RE offset table. |
| Nokia/NSB | Fine with Alt 1 in general. But, we can discuss it later. |
| Samsung | Support Alt.3. |
| QC | We are fine with Alt 1. Alt 2 seems does not work because PTRS of UL MU could collide. By the way, there is Alt 1a, which is slightly different than Alt 1. Between Alt 1 and 1a, we don’t have strong preference. DM-RS antenna port p (p for PUSCH, p+1000 for PDSCH). DM-RS Configuration type 1; DM-RS Configuration type 2. resourceElementOffset; resourceElementOffset. offset00; offset01; offset10; offset11; offset00; offset01; offset10; offset11. 0; 0; 2; 6; 8; 0; 1; 6; 7. 1; 2; 4; 8; 10; 1; 6; 7; 0. 2; 1; 3; 7; 9; 2; 3; 8; 9. 3; 3; 5; 9; 11; 3; 8; 9; 2. 4; -; -; -; -; 4; 5; 10; 11. 5; -; -; -; -; 5; 10; 11; 4. 8; 6; 8; 0; 2; -; -; -; -. 9; 8; 10; 2; 4; -; -; -; -. 10; 7; 9; 1; 3; -; -; -; -. 11; 9; 11; 3; 5; -; -; -; -. 12; -; -; -; -; 6; 7; 0; 1. 13; -; -; -; -; 7; 0; 1; 6. 14; -; -; -; -; 8; 9; 2; 3. 15; -; -; -; -; 9; 2; 3; 8. 16; -; -; -; -; 10; 11; 4; 5. 17; -; -; -; -; 11; 4; 5; 10. |
| LGE | Fine with Alt.1 |
| Apple | Alt 1 |
| MediaTek | Support Alt 1 |
| Ericsson | Support Alt.1. |
| vivo | Fine with Alt 1. |
| FL | Alt.2 is removed by companies’ inputs. |
| Spreadtrum | Support Alt.1. |

  - Other proposals

Following proposals are also proposed.

| Proposals | Companies |
| --- | --- |
| - The same antenna port(s) tables for PUSCH for rank 1~4 are appliable to both one codeword case (Codeword 0 enabled, Codeword 1 disabled), and two codewords case (Codeword 0 enabled, Codeword 1 enabled) with rank 1~4 | Docomo |
| - UE indicates the number of PTRS ports associated to Ng as part of its capability | IDC |
| - Rel-15 DMRS with 8 port cannot used with PTRS. At least, we have to agree with 8 port DMRS with PTRS. | Nokia/NSB |

Please provide your views on the above proposals, or other aspects which are not included in the summary, if any.

| Company | Comment |
| --- | --- |
| Nokia/NSB | One more issue we have to resolve is, for Rel-15 DMRS, we cannot use PT-RS with TD-OCC. For Rel-18, we can further discuss on the options. TS38.214 If a UE transmitting PUSCH scheduled by DCI format 0_2 is configured with the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 or dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2, or a UE transmitting PUSCH scheduled by DCI format 0_0 or DCI format 0_1 is configured with the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB, the UE may assume that the following configurations are not occurring simultaneously for the transmitted PUSCH - any DM-RS ports among 4-7 or 6-11 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and PT-RS is transmitted from the UE. |

- Conclusion

Based on the email discussion, following FL proposals are proposed.

1. Dynamic switching b/w R15 DMRS and R18 DMRS

FL Proposal 2.4A

- To mitigate potential performance degradation in large delay spread in Rel.18 DMRS ports with FD-OCC length 4 for PDSCH, For UE configured with Rel.18 DMRS ports for PDSCH, select one of the following:

  - Alt.1: Scheduling DCI format 1_1/1_2 can indicate FD-OCC length that UE shall assume for receiving DMRS.

  - Alt.2: Scheduling DCI format 1_1/1_2 can indicate the information of DMRS ports of co-scheduled UE in the same CDM group.

    - Note: It is up to UE implementation how to use the information.

  - FFS: Whether additional DCI field is needed, or existing DCI field can be reused for the indication.

Support (21): Google (prefer Alt.2), OPPO (Alt.2 only), Docomo (Both), ZTE (Alt.1), CMCC(Alt.1), CATT(Alt.1), Lenovo(Alt.1), Sharp(Alt.1), Xiaomi(if majority supports), HW/HiSilicon(Alt.1 w/ UE cap.), Fraunhofer IIS/HHI(Alt.2), Nokia/NSC(Alt.1), FW(Alt.1), Samsung(Alt.1 only), Ericsson (both), China Telecom (Alt.1), vivo (Alt.1), Spreadtrum(Alt.1)

Not support (4): QC, LG, Apple, MTK (RRC based only)

2. PDSCH DMRS port table for eType1 maxLength=1

FL: Following rows are relatively stable. Let’s try the following rows firstly.

FL Proposal 2.1.1B

- For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support at least support the following rows:

  - For 1 CW,

    - 1) Row 0-2, 12-14, 24-25 (rows with Number of DMRS CDM group(s) without data = 1)

Support (24): HW/HiSilicon, New H3C, Docomo, ZTE Spreadtrum, vivo, Lenovo, LG, CATT, Ericsson, Intel, CMCC, QC, Nokia/NSB, OPPO, Apple, Google, FW, MTK, China Telecom, Sharp (fine with majority), Spreadtrum

Concern (1): Samsung (target of R18 DMRS is for MU-MIMO)

3. PUSCH DMRS port table for eType1 maxLength=1

FL: Based on inputs, I marked [ ] to the rows where some companies prefer to update or remove. For the rows with blue highlighted rows are marked with [ ] because these rows for PDSCH are not agreed yet. If it is agreed for PDSCH, we can try to remove [ ] for PUSCH as well.

FL Proposal 2-3A

- For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, following Table 7.3.1.1.2-8-X, Table 7.3.1.1.2-9-X, Table 7.3.1.1.2-10-X, and Table 7.3.1.1.2-11-X are supported.

  - FFS: Whether to increase the size of antenna ports field in DCI format 0_1/0_2 or not.

  - Note: Antenna ports tables for Rel.18 eType2 DMRS ports with maxLength = 1/2 and eType1 DMRS ports with maxLength = 2 for PUSCH are to be discussed separately.

Table 7.3.1.1.2-8-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| [0] | [1] | [0] |
| [1] | [1] | [1] |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| [6] | [1] | [8] |
| [7] | [1] | [9] |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 2 | 11 |
| 12-15 | Reserved | Reserved |

Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| [0] | [1] | [0,1] |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 2 | 0,2 |
| [4] | [1] | [8,9] |
| 5 | 2 | 8,9 |
| 6 | 2 | 10,11 |
| [7] | [2] | [8,10] |
| 8-15 | Reserved | Reserved |

Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| [1] | [2] | [8-10] |
| [2] | [1] | [0,1,8] |
| 3 | 2 | 0,1,8 |
| 4 | 2 | 2,3,10 |
| 5-15 | Reserved | Reserved |

Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| [1] | [2] | [8-11] |
| [2] | [1] | [0,1,8,9] |
| 3 | 2 | 0,1,8,9 |
| 4 | 2 | 2,3,10,11 |
| 5-15 | Reserved | Reserved |

4. MU MIMO within a CDM group between R15 DMRS and R18 DMRS

FL: No company seem to have concern on 1) and 2). Ericsson/vivo think 3) is not needed (vivo: The network can even configure different scramblingID of DMRS to UEs in MU-MIMO).

At least we can try to agree 1) and 2). For 3) and 4), if it is not agreed, it means up to gNB implementation.

FL Proposal 2.5A

- For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

  - 1) For PUSCH, there is no restriction.

  - 2) For PDSCH, there is no additional restriction between Rel.18 UE1 indicated with Rel-18 Legacy ports (eType1: ports 1000-1007, eType2: ports 1000-1011) and Rel.15/18 UE2 indicated with Rel.15 DMRS ports in a CDM group from Rel.17 spec.

    - Note: MU-MIMO restriction in Rel.17 is applied.

  - 3) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.15 UE2 indicated with Rel.15 DMRS ports in a CDM group,

    - UE does not expect such MU-MIMO in a CDM group.

  - 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, down select from the following.

    - Alt.1: UE does not expect such MU-MIMO in a CDM group.

    - Alt.2: Rel.18 UE2 configured with Rel.15 DMRS ports can be signaled, to indicate that there may be another Rel.18 UE1 with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports.

      - Dedicated UE capability is introduced.

      - The signaling is at least by RRC (FFS: whether to support DCI based signaling).

    - Alt.3: It is up to gNB implementation whether to schedule such MU-MIMO in a CDM group (i.e. no spec impact and no additional restriction).

4. PTRS power boosting for 8Tx PUSCH

FL: Stable proposal for 8Tx.

FL proposal#3.4A:

- For 8Tx PUSCH, specify PUSCH to PTRS power ratio per layer per RE () based on the following principles.

  - Principle 1: When the ptrs-Power configures 01, the PTRS to PUSCH power ratio is 10log10(L), where L is the total number of PUSCH layers.

  - Principle 2: When the ptrs-Power configures 00, the PTRS to PUSCH power ratio is determined as the following

    - Principle 2.1: For fully coherent TPMIs, PTRS to PUSCH power ratio is 10log10(L), where L is the total number of PUSCH layers.

    - Principle 2.2: For noncoherent TPMIs, PTRS to PUSCH power ratio is 10log10(Qp), where Qp is the number of PTRS ports configured to the UE.

    - FFS:  the PTRS to PUSCH power ratio for partial coherent TPMIs

  - FFS: Whether the transmission power should be consistent within the whole transmission occasion for PUSCH.

  - Send LS to RAN4 to inform that RAN1 made the above agreement.

References

| [1] | R1-2300096 | Enhancements on DMRS in Rel-18 | Huawei, HiSilicon |
| --- | --- | --- | --- |
| [2] | R1-2300152 | Discussions on increased number of orthogonal DMRS ports | New H3C Technologies Co., Ltd. |
| [3] | R1-2300159 | Further Details on DMRS Enhancements | InterDigital, Inc. |
| [4] | R1-2300184 | DMRS enhancement for UL/DL MU-MIMO and 8 Tx UL SU-MIMO | ZTE |
| [5] | R1-2300206 | Discussion on increased number of orthogonal DMRS ports | Spreadtrum Communications |
| [6] | R1-2300251 | DMRS enhancement for Rel-18 MIMO | OPPO |
| [7] | R1-2300392 | On DMRS Enhancement | Google |
| [8] | R1-2300439 | Further discussion on DMRS enhancements | vivo |
| [9] | R1-2300504 | On increased number of orthogonal DMRS ports | Fraunhofer IIS, Fraunhofer HHI |
| [10] | R1-2300513 | Discussion of increased number of orthogonal DMRS ports | Lenovo |
| [11] | R1-2300525 | Increased number of orthogonal DMRS ports | LG Electronics |
| [12] | R1-2300548 | Discussion on DMRS enhancement | Xiaomi |
| [13] | R1-2300653 | Discussion on DMRS enhancements in Rel-18 | CATT |
| [14] | R1-2300782 | On increased number of orthogonal DMRS ports for MU-MIMO and 8 Tx UL SU-MIMO | Ericsson |
| [15] | R1-2300813 | Discussion on increased number of orthogonal DMRS ports | NEC |
| [16] | R1-2300848 | Increased number of orthogonal DMRS ports | Sharp |
| [17] | R1-2300935 | DMRS Enhancements for Rel-18 NR | Intel Corporation |
| [18] | R1-2300985 | Discussion on increased number of orthogonal DMRS ports | CMCC |
| [19] | R1-2301250 | Views on DMRS enhancements | Samsung |
| [20] | R1-2301332 | Views on supporting increased number of orthogonal DMRS ports | Apple |
| [21] | R1-2301398 | Design for increased number of orthogonal DMRS ports | Qualcomm Incorporated |
| [22] | R1-2301480 | Discussion on DMRS enhancements | NTT DOCOMO, INC. |
| [23] | R1-2301574 | Increased number of orthogonal DMRS ports | MediaTek Inc. |
| [24] | R1-2301645 | Rel-18 UL and DL DMRS Enhancements | Nokia, Nokia Shanghai Bell |

Appendix

RAN1#109e agreements:

EVM Agreement - LLS is used for objective #3 (increasing DMRS ports for MU-MIMO) in Rel.18 MIMO, while SLS can be used optionally. Agreement - No EVM discussion is needed for objective #5 (>4 layers PUSCH DMRS) in AI 9.1.3.1 (DMRS) in Rel.18. Agreement - LLS for increasing DMRS ports in AI 9.1.3.1 in Rel.18: - Evaluated channel: PDSCH as baseline (Companies can additionally submit evaluation results of PUSCH). - Evaluation metric: - BLER for fixed MCS and rank as baseline - User throughput for adaptive MCS and rank as optional - MSE or NMSE of DMRS as optional - Evaluation baseline (i.e. compared with): - For evaluation of enhanced single-symbol DMRS, baseline refers to Rel.15 single-symbol DMRS or Rel.15 double-symbol DMRS. - For evaluation of enhanced double-symbol DMRS, baseline refers to Rel.15 double-symbol DMRS. Agreement - Following evaluation assumptions are used for LLS for increasing DMRS ports in AI 9.1.3.1 in Rel.18. Parameter; Value. Duplex, Waveform; TDD, OFDM Note: FDD, OFDM is not precluded. Carrier Frequency; 4 GHz. Subcarrier spacing; 30kHz. Channel Model; CDL-B or CDL-C in TR 38.901 with 30ns or 300ns delay spread as baseline for MU-MIMO and SU-MIMO Note: Other delay spread is not precluded. Note: Simulation using TDL-A with 30ns or 300ns for MU-MIMO is not precluded.. Delay spread; Baseline: 30ns, 300ns Optional: 1000ns. UE velocity; Baseline: 3km/h, 30km/h Optional: 60km/h, 120km/h. Allocation bandwidth; 20MHz Note: Other bandwidth smaller than 20MHz is not precluded. MIMO scheme; Baseline: MU-MIMO Optional: SU-MIMO. BS antenna configuration; Companies can select and need to report which option(s) are used between - 32 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded.. UE antenna configuration; Companies can select and need to report which option(s) are used between 4RX: (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded.. MIMO Rank; 1, 2, or 4 per UE (rank fixed or rank adaptation). UE number for MU-MIMO; 1, 2, 4, 8, or 12. Precoding and precoding granularity; For PDSCH: Companies can select and need to report which option(s) are used between - [ZF or SVD] based sub-band precoding (with 4PRB precoding granularity) on ideal channel knowledge - CSI codebook based sub-band precoding (with 4PRB precoding granularity) on ideal CSI feedback. For PUSCH: Companies can select and need to report which option(s) are used between - [ZF or SVD] based wide-band precoding on ideal channel knowledge - Codebook based wide-band precoding on ideal CSI feedback.. Feedback delay for precoding; 5ms. DMRS type; Type 1E and/or Type 2E, which are enhanced DMRS that are based on the legacy RE mappings of DMRS Type 1/2, where the enhanced DMRS support larger DMRS ports. Note: The terminology of Type 1E and/or Type 2E is for discussion purpose.. DMRS configurations; Baseline: - Single symbol DMRS without additional DMRS symbols and 1 additional DMRS symbol - Double symbol DMRS without additional DMRS symbols. Note: evaluation of other additional DMRS symbol(s) are not precluded.. DMRS mapping type; Mapping type A (slot based) for PDSCH. Mapping type A (slot based) for PUSCH.. Link adaptation; - Fixed modulation, coding and rank for BLER evaluation as baseline. - Adaptation of both MCS and rank for throughput evaluation as optional.. HARQ; Baseline: Off Optional: On (HARQ with max. 4 re-transmissions) for throughput evaluation. Channel estimation; Realistic channel estimation with ideal info of frequency sync, SNR, doppler and delay spread. Receiver type; MMSE as baseline. EVM; No radio impairments. Agreement - For LLS assumptions for increasing DMRS ports in AI 9.1.3.1 in Rel.18: - Precoding assumption of PUSCH, “[ZF or SVD]” in RAN1#109e agreement is updated by - Alt.2-2: SVD Agreement For LLS assumptions for increasing DMRS ports in AI 9.1.3.1 in Rel.18: - Precoding assumption of PDSCH, “[ZF or SVD]” in RAN1#109e agreement is updated by SVD. Agreement - For MU-MIMO LLS of PDSCH, for evaluation of SVD/CSI-codebook based sub-band precoding, companies shall report the pre-coding assumption of interference of co-scheduled UEs from the following: - Alt.1: calculated by pre-coder of channel of each co-scheduled UE. - For precoding assumption of PDSCH, precoder of target UE and precoder of co-scheduled UE are generated independently. - Companies can report a set of azimuth and zenith angle offset used for evaluation (For example, azimuth angle offsets from [30 o, 60 o, 90 o] and zenith angle offset from [3o, 6o] can be considered). - Alt.2: calculated by random pre-coder (i.e. precoder selected randomly from a predefined set of precoders) which is different from the pre-coder of target UE. - For precoding assumption of PDSCH, only the channel of one target UE, i.e. Hd, needs to be modelled. Precoder is generated based on Hd to obtain the precoder for this UE only. The interference from co-scheduled UEs can be modelled as, , wherein Wi can be randomly selected from a predefined set of precoders - Companies shall report how to generate the predefined set of precoders for simulation. - Alt.3: the same pre-coder as scheduled UE. - PDSCH interference and interfering DMRS ports are emulated using the same pre-coder as for the scheduled UE. - Power offset of the co-scheduled UE is one value from {0dB, -3dB, -6dB} as fixed evaluation parameter. Other values are not precluded. - For precoding assumption of PDSCH, only the channel of one target UE, i.e. Hd, needs to be modelled. Precoder for the target UE (denoted as Wd) is generated based on Hd only. Denote the precoding matrix/vector of the ith co-scheduled UEs as Wi, and Wi=Wd (Wi for all th co-scheduled UEs are same). Then the interference from co-scheduled UEs can be modelled as .​ For the above Alt.1-3, only PDSCH performance of the target UE is evaluated, while interference of both PDSCH and DMRS of co-scheduled UE(s) is simulated. Agreement - For SLS assumption for increasing DMRS ports in AI 9.1.3.1 in Rel.18, - Scenario: Dense Urban (Macro only) at 4GHz is a baseline. Other scenarios (e.g. Umi, Uma) are not precluded. - Following evaluation assumptions are used for SLS. Parameter; Value. Scenario; Dense Urban (macro only). Carrier frequency; 4GHz. Duplex, Waveform; TDD, OFDM Note: FDD, OFDM is not precluded. Multiple access; OFDMA. Frequency Range; FR1 only.. Inter-BS distance; 200 m. Channel model; According to the TR 38.901. Antenna setup and port layouts at gNB; Companies need to report which option(s) are used between - 32 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded.. Antenna setup and port layouts at UE; 4RX: (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configurations are not precluded.. BS Tx power; 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz. BS antenna height; 25 m. BS noise figure; 5 dB. UE noise figure; 9 dB. UE antenna height & gain; Follow TR36.873. Modulation; Up to 256 QAM. Coding on PDSCH; LDPC Max code-block size=8448bit. Numerology; Slot/non-slot; 14 OFDM symbols per slot. SCS; 30 kHz. Simulation bandwidth; 20 MHz. Number of RBs; 52 for 30 kHz SCS. Frame structure; Slot Format 0 (all downlink) for all slots. MIMO scheme; SU/MU-MIMO with rank adaptation is a baseline For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed. MIMO layers; For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12). CSI feedback; Feedback assumption at least for baseline scheme CSI feedback periodicity (full CSI feedback): 5 ms, Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms. Overhead; Companies shall provide the downlink overhead assumption. Traffic model; Baseline: FTP1 with 50% Resource Utilization Optional: Full buffer. UE distribution; [80%] indoor (3km/h), [20%] outdoor (30km/h). UE receiver; MMSE-IRC as the baseline receiver. Feedback assumption; Realistic. Channel estimation; Realistic. For increasing orthogonal DMRS ports Agreement - Specify to increase the max. number of DMRS ports for PDSCH/PUSCH larger than Rel.15 for CP-OFDM without increasing the DMRS overhead. - Strive to have common design of DMRS enhancement for PDSCH and PUSCH for a given DMRS Type. Agreement - The maximum number of enhanced DMRS ports in Rel.18 is doubled from Rel.15 DMRS ports: - For DMRS type 1, the max. number of enhanced DMRS ports in Rel.18 for PDSCH/PUSCH is - Single symbol DMRS: 8 DMRS ports. - Double symbol DMRS: 16 DMRS ports. - For DMRS type 2, the max. number of enhanced DMRS ports in Rel.18 for PDSCH/PUSCH is - Single symbol DMRS: 12 DMRS ports. - Double symbol DMRS: 24 DMRS ports. Agreement - To increase the number of DMRS ports for PDSCH/PUSCH, evaluate and, if needed, specify one or more from the following options: - Opt.1 (enhance FD-OCC): Introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6). - Study aspect includes potential performance degradation in large delay spread, potential scheduling restriction, backward compatibility. - Opt.2 (enhance TD-OCC): Utilize TD-OCC over non-contiguous DMRS symbols (e.g. TD-OCC across front/additional DMRS symbols) - Study aspect includes potential performance degradation in high UE velocity, potential scheduling restriction (e.g. how to apply freq. hopping), potential DMRS configuration restriction (e.g. restriction of the number of additional DMRS), backward compatibility. - Opt.3 (Sparser frequency allocation): increase the number of CDM groups (e.g. larger number of comb/FDM). - Study aspect includes potential performance degradation in large delay spread, backward compatibility. - Opt.4 (using TDMed DMRS symbol): reusing additional DMRS symbols to increase orthogonal DMRS ports - Study aspect includes potential performance degradation in high UE velocity, potential DMRS configuration restriction (e.g. restriction of the number of additional DMRS), backward compatibility. - Opt.5 TD-OCC over non-contiguous DMRS symbols combined with FD-OCC or FDM: reusing additional DMRS symbol(s) to improve channel estimation performance. - Study aspect includes potential performance degradation in high UE velocity, potential scheduling restriction (e.g. how to apply freq. hopping), potential DMRS configuration restriction (e.g. restriction of the number of additional DMRS), backward compatibility. - The same option can be applied to both single symbol DMRS and double symbol DMRS. Agreement - To increase the max. number of DMRS ports for PDSCH/PUSCH compared to Rel.15 DMRS for CP-OFDM without increasing the DMRS overhead, - Study whether/how to enable MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports, as well as whether/how to enable MU-MIMO among Rel.18 DMRS ports, in the same or different CDM group. Agreement - To increase the max. number of orthogonal DMRS ports for PDSCH/PUSCH larger than Rel.15 - Study whether/how to support DCI-based dynamic antenna ports indication of Rel.18 DMRS ports and/or Rel.15 DMRS ports. - Study whether/how to reuse the antenna port indication table in 38.212 as much as possible for both PDSCH and PUSCH - Study the potential need for MU scheduling restrictions in the design of the enhanced antenna port indication table in 38.212 for DL PDSCH. For 8 Tx UL SU-MIMO Agreement - Study the following potential DMRS enhancement for potential support of more than 4 layers SU-MIMO PUSCH. - Extend DMRS port allocation table for rank 5~8 - Note: DL DMRS table can be a reference - Enhancement for DMRS to PTRS mapping - Study whether to utilize Rel.18 DMRS ports for more than 4 layers SU-MIMO PUSCH. - Note: the above study does not imply more than 4 layers SU-MIMO PUSCH is supported. - Note: other study for potential DMRS enhancement for potential support of more than 4 layers SU-MIMO PUSCH is not precluded.

RAN1#110 agreements:

For increasing orthogonal DMRS ports Working Assumption - To increase the number of DMRS ports for PDSCH/PUSCH, support at least Opt.1 (introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6)). - FFS: FD-OCC length for Rel.18 DMRS type 1 and type 2. - FFS: Whether it is needed to handle potential performance issues of Opt 1. For example, study if there is performance loss in case of large delay spread scenario. If needed, how (e.g. additionally support other options). Agreement - For enhanced FD-OCC length for DMRS of PDSCH/PUSCH, support the following FD-OCC length: - For Rel.18 DMRS type 1, down select from the following in RAN1#110bis-e: - Opt.1-1: Length 6 FD-OCC is applied to 6 REs of DMRS within a PRB within an CDM group - Opt.1-2: Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB or across consecutive PRBs within an CDM group - For Rel.18 DMRS type 2: - Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB within an CDM group - FFS: Support of length 6 FD-OCC Agreement - Support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports. - For MU-MIMO by different CDM groups, no MU-MIMO scheduling restriction of PUSCH/PDSCH (i.e. MU-MIMO between Rel.15 UE and Rel.18 UE is allowed). - For MU-MIMO within a CDM group, study whether and how to support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH. - Note: the study includes MU-MIMO between Rel.15 UE and Rel.18 UE, and between Rel.18 UEs. - Note: PUSCH above is CP-OFDM waveform. Agreement For increased DMRS ports for enhanced FD-OCC, study whether/how to support DCI based switching between DMRS port(s) associated with length 2 FD-OCC and DMRS port(s) associated with length M FD-OCC (where M > 2). For 8 Tx UL SU-MIMO Agreement - For support of more than 4 layers SU-MIMO PUSCH, study the following potential enhancements for PTRS-DMRS association. - Whether to support more than 2-port UL PTRS. - Whether to increase the DCI size of PTRS-DMRS association field in DCI format 0_1/0_2. Agreement For > 4 layers PUSCH, support rank = 5,6,7,8 for both DMRS type 1/2, and for both single-symbol/double-symbol DMRS.

RAN1#110bis-e agreements:

For increasing orthogonal DMRS ports Conclusion - For discussion purpose, definition of Rel.15 DMRS ports and Rel-18 DMRS ports are: - Rel.15 Type 1/Type 2 DMRS ports: DMRS ports with FD-OCC length =2. - Rel.18 eType 1/eType 2 DMRS ports: DMRS ports with FD-OCC length >2. - Following figure as an example shows difference between Rel.15 Type 1 DMRS ports and Rel.18 eType 1 DMRS ports. Agreement Confirm the working assumption in RAN1#110 with the following update: - To increase the number of DMRS ports for PDSCH/PUSCH, support at least Opt.1 (introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6)). - FFS: FD-OCC length for Rel.18 DMRS type 1 and type 2. - FFS: Whether it is needed to handle potential performance issues of Opt 1. For example, study if there is performance loss in case of large delay spread scenario. If needed, how (e.g. additionally support other options). Agreement For enhanced FD-OCC length for DMRS of PDSCH/PUSCH for Rel.18 eType 1 DMRS, support - Opt.1-2: Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB or across consecutive PRBs within an CDM group Agreement For FD-OCC length 4 for DMRS of PDSCH/PUSCH for Rel.18 eType 1/eType 2 DMRS, support one from the following FD-OCCs (to be selected in RAN1#111): - Opt.1-1: Walsh matrix (Hadamard code): FD-OCC index; wf(0); wf(1); wf(2); wf(3). 0; +1; +1; +1; +1. 1; +1; -1; +1; -1. 2; +1; +1; -1; -1. 3; +1; -1; -1; +1. - Opt.1-2: Cyclic shift with {0, π, π/2, 3π/2}: FD-OCC index; wf(0); wf(1); wf(2); wf(3). 0; +1; +1; +1; +1. 1; +1; -1; +1; -1. 2; +1; +j; -1; -j. 3; +1; -j; -1; +j. Agreement For Rel.18 eType 1/eType 2 DMRS ports of PDSCH/PUSCH with FD-OCC length 4, association between DMRS port indexes, CDM group index, FD-OCC index, and TD-OCC index (across consecutive DMRS symbols, if any) are determined by the following Table 1 and Table 2. - The p in Table 1 and Table 2 corresponds to DMRS port index for PUSCH. - DMRS port index for PDSCH is determined by p +1000 in Table 1 and Table 2. Table 1. Rel.18 eType 1 DMRS ports for PUSCH p; CDM group index; FD-OCC index; TD-OCC index. 0; 0; 0; 0. 1; 0; 1; 0. 2; 1; 0; 0. 3; 1; 1; 0. 4; 0; 0; 1. 5; 0; 1; 1. 6; 1; 0; 1. 7; 1; 1; 1. 8; 0; 2; 0. 9; 0; 3; 0. 10; 1; 2; 0. 11; 1; 3; 0. 12; 0; 2; 1. 13; 0; 3; 1. 14; 1; 2; 1. 15; 1; 3; 1. Table 2. Rel.18 eType 2 DMRS ports for PUSCH p; CDM group index; FD-OCC index; TD-OCC index. 0; 0; 0; 0. 1; 0; 1; 0. 2; 1; 0; 0. 3; 1; 1; 0. 4; 2; 0; 0. 5; 2; 1; 0. 6; 0; 0; 1. 7; 0; 1; 1. 8; 1; 0; 1. 9; 1; 1; 1. 10; 2; 0; 1. 11; 2; 1; 1. 12; 0; 2; 0. 13; 0; 3; 0. 14; 1; 2; 0. 15; 1; 3; 0. 16; 2; 2; 0. 17; 2; 3; 0. 18; 0; 2; 1. 19; 0; 3; 1. 20; 1; 2; 1. 21; 1; 3; 1. 22; 2; 2; 1. 23; 2; 3; 1. Agreement For FD-OCC length 4 in Rel.18 eType 1 DMRS for PDSCH, support the following: - Introduce UE capability to report whether UE can be scheduled PDSCH without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS. - If this capability is not supported by the UE, UE expects that gNB shall apply the scheduling restriction for PDSCH for FD-OCC length 4 in Rel.18 eType 1 DMRS. - The scheduling restriction above means satisfying all of the following at least for other than M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - The number of consecutively scheduled PRBs for PDSCH is even. - The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even. - FFS: Restriction on scheduling of different UEs in case of MU-MIMO. - FFS: Scheduling restriction for M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - Note1: Up to UE how to implement DMRS channel estimation. - Note2: No further RAN1 specification enhancement is introduced to handle the orphan REs (e.g. if the total number of REs of DMRS in a CDM group is not multiples of 4, how to handle the remainder of REs) for UE that is scheduled PDSCH without the scheduling restriction. - Note 3: Other scheduling restrictions, if identified in future meetings, are not precluded. Conclusion For FD-OCC length 4 in Rel.18 eType 1 DMRS for PUSCH, - No spec. enhancement is needed to handle orphan RE issue (e.g. if the total number of REs of DMRS in a CDM group is not multiples of 4, how to handle the remainder of REs), because gNB (receiver) can decide whether the scheduling restriction is needed or not. For 8 Tx UL SU-MIMO Agreement For more than 4 layers SU-MIMO PUSCH, support - Both Rel.15 Type 1/Type 2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports. - For UE supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with either of Rel.15 Type 1/Type 2 DMRS ports or Rel.18 eType 1/eType 2 DMRS ports. - RRC based indication is supported as the baseline. FFS whether DCI based indication is further needed. - For UE not supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with Rel.15 Type 1/Type 2 DMRS ports only.

RAN1#111 agreements:

For increasing orthogonal DMRS ports Agreement For FD-OCC length 4 for PDSCH/PUSCH, select the following: - Opt.1-1 (Walsh matrix) for PDSCH - Opt.1-2 (Cyclic shift) for PUSCH Agreement - For the antenna ports indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, all of the following port combinations can be indicated: - Cat. 1) Legacy port indexes (eType 1: p=0~7, eType 2: p=0~11) - Cat. 2) New port indexes (eType 1: p=8~15, eType 2: p=12~23) - Cat. 3) Legacy port indexes and New port indexes at least within a CDM group at least for maxLength=1 (eType 1: up to 4 ports from {0, 1, 8, 9} and/or up to 4 ports from {2, 3, 10, 11}, eType 2: up to 4 ports from {0, 1, 12, 13} and/or up to 4 ports from {2, 3, 14, 15} and/or up to 4 ports from {4, 5, 16, 17}) at least for S-TRP case, - For up to 4 ranks, only one CDM group is used per UE. For larger than 4 ranks, more than one CDM groups can be used per UE. - FFS: Whether to increase the size of antenna ports field in DCI format 1_1/1_2, or introduce new DCI field for antenna ports indication, or not. - FFS: Whether the new antenna port(s) table is specified or not. - FFS: MU restrictions for certain entries. e.g., DMRS ports = {0,2}, or {8,10}, etc. - FFS: Cat.3 for M-TRP case. - Note: DMRS port index for PDSCH is determined by p +1000 Agreement For length 2 TD-OCC (across consecutive DMRS symbols, if any) for DMRS of PDSCH/PUSCH for Rel.18 eType 1/2 DMRS, support Opt.1: - Opt.1: TD-OCC index; Wt(0); Wt(1). 0; +1; +1. 1; +1; -1. Agreement - For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data. - FFS: Antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for M-TRP case. Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1 One Codeword: Codeword 0 enabled, Codeword 1 disabled; Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Notes; Value; Number of DMRS CDM group(s) without data; DMRS port(s); Notes. 0; [1]; [0]; Cat. 1; [0]; [2]; [0,1,2,3,8]; [Rank 5-8 with one DMRS symbol]. 1; [1]; [1]; [1]; [2]; [0,1,2,3,8,10]. 2; [1]; [0,1]; [2]; [2]; [0,1,2,3,8,9,10]. 3; 2; 0; [3]; [2]; [0,1,2,3,8,9,10,11]. 4; 2; 1. 5; 2; 2. 6; 2; 3. 7; 2; 0,1. 8; 2; 2,3. 9; [2]; [0-2]. 10; [2]; [0-3]. 11; [2]; [0,2]. 12; [1]; [8]; Cat.2. 13; [1]; [9]. 14; [1]; [8,9]. 15; 2; 8. 16; 2; 9. 17; 2; 10. 18; 2; 11. 19; 2; 8,9. 20; 2; 10,11. 21; [2]; [8-10]. 22; [2]; [8-11]. 23; [2]; [8, 10], [9, 11]. 24; [1]; [0,1,8]; Cat.3. 25; [1]; [0,1,8,9]. 26; 2; 0,1,8. 27; 2; 0,1,8,9. 28; 2; 2,3,10. 29; 2; 2,3,10,11. For 8 Tx UL SU-MIMO Agreement - For > 4 layers PUSCH, support new antenna ports tables for rank = 5,6,7,8 for both single-symbol/double-symbol DMRS. - For Type 1/Type 2 Rel.15 DMRS ports, new antenna ports tables are the following: - The same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook. - For Rel.18 eType1/eType2 DMRS ports, - New antenna ports tables with new DMRS port combinations are used for rank = 5,6,7,8 (FFS: details). - Note: Whether the DMRS port combination allows to use single symbol DMRS for rank = 5,6,7,8 should be checked. - FFS: For partial coherent UL codebook, support layers to DMRS port mapping that layers associated to the same antenna port group are multiplexed into the same DMRS CDM group. - FFS: One or more than one DMRS port combination(s) for each rank and TPMI - Note: New DMRS port combinations above does not preclude the new antenna ports tables including the current DMRS port combination(s) for PDSCH for rank = 5,6,7,8 in Rel.15-17. - FFS: Whether the antenna ports combinations for rank = 5,6,7,8 can be indicated by the reserved entries of existing antenna ports tables for rank =1,2,3,4, if the rank is indicated together with DMRS antenna ports. Agreement - For full-coherent PUSCH with rank 5-8, UE shall expect only one port PTRS to be configured. - Down select from the following in RAN1#112: - Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2. - FFS: Association with the CW with the higher MCS. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port with the CW with the higher MCS. 1; 2nd scheduled DMRS port the CW with the higher MCS. 2; 3rd scheduled DMRS port the CW with the higher MCS. 3; 4th scheduled DMRS port the CW with the higher MCS. - Alt.2: The size of PTRS-DMRS association field is 3bit in DCI format 0_1/0_2, and the following PTRS-DMRS association for UL PTRS port 0 is specified in TS38.212. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port. 1; 2nd scheduled DMRS port. 2; 3rd scheduled DMRS port. 3; 4th scheduled DMRS port. 4; 5th scheduled DMRS port. 5; 6th scheduled DMRS port. 6; 7th scheduled DMRS port. 7; 8th scheduled DMRS port.