3GPP TSG RAN WG1 #112                                     	       R1-2301848

Athens, Greece, February 27th –March 3rd, 2023

Agenda Item:			9.14.1

Source:			Moderator (China Telecom)

Title:			FL Summary#1 on PRACH coverage enhancements

Document for:		Discussion

- Introduction

In RAN #94 e-meeting, a new Rel-18 work item on further NR coverage enhancements was approved [1] and updated in RAN #96 [2]. The objective of the work item is to specify further uplink coverage enhancements for PRACH, power domain and DFT-S-OFDM. Detailed objectives are listed as follows:

- Specify following PRACH coverage enhancements (RAN1, RAN2) - Multiple PRACH transmissions with same beam for 4-step RACH procedure - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable. - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable. - Study and if necessary specify following power domain enhancements - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1) - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1) - Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

This contribution is a summary of companies’ contributions on PRACH coverage enhancements.

- Summary of contributions

2.1 Multiple PRACH transmissions with same beam

Based on companies’ contributions, sometimes the term “PRACH repetition” is utilized to indicate “multiple PRACH transmissions with same beam”. Thus, it needs to be clarified that the term “PRACH repetition” only indicates “multiple PRACH transmissions with same beam”, it doesn’t put any additional restrictions on multiple PRACH transmissions.

2.1.1 Resource configuration for multiple PRACH transmissions

Issue #1: Support one or multiple PRACH coverage enhancement level(s) (number of PRACH repetitions)

- In RAN1 #111 meeting [3], it was discussed about whether to support one or multiple PRACH coverage enhancement level(s) and the following agreement was achieved:

Agreement For multiple PRACH transmissions with same Tx beam, down-select one option from the following options. - Option 1: gNB can only configure one value for the number of multiple PRACH transmissions. - Option 2: gNB can configure one or multiple values for the number of multiple PRACH transmissions. - FFS: details

- Based on the contributions, the potential pros and cons of the two options are summarized in the following table.

|  | - Option1: only one value can be configured | - Option 2: one or multiple values can be configured |
| --- | --- | --- |
| - Pros | - Simplify system operation. | - Flexible system operation and efficient PRACH resource management. |
| - Cons | - Cannot optimize the trade-off between system overhead and served coverage area. - For area with different coverage status, may cause inefficient resource management. | - Need more gNB effort to do blind detection or suffer from other scheduling restriction. |

- If Option 1 is supported, it indicates that the number of multiple PRACH transmissions is only determined by the network.

- If Option 2 is supported, it indicates that the number of multiple PRACH transmissions is determined at UE side based on some predefined conditions.

- Companies' attitude for each option are summarized as follows:

- Option 1: gNB can only configure one value for the number of multiple PRACH transmissions.

Support: Samsung

- Option 2: gNB can configure one or multiple values for the number of multiple PRACH transmissions.

Support: China Telecom, Spreadtrum (?), ZTE, xiaomi, CATT, Fujitsu, Panasonic, NEC, Lenovo, Sony, Intel ETRI, InterDigital, Qualcomm, NTT DOCOMO, Nokia, MediaTek, Apple

- Notice that this issue may impact the discussion on PRACH resource configurations. FL suggests that we made the down-selection in an early time.

Issue #2: Differentiation between single PRACH transmission and multiple PRACH transmissions

- One important issue for multiple PRACH transmissions is whether/how gNB can differentiate between legacy single PRACH transmissions and multiple PRACH transmissions, and whether/how gNB can differentiate between different numbers of multiple PRACH transmissions if different “coverage enhancement levels” (number of PRACH repetitions) are supported.

- In RAN1 #111 meeting [3], the following agreements were achieved for the differentiation between single PRACH transmission and multiple PRACH transmissions

Agreement For multiple PRACH transmissions with same Tx beam, support to differentiate at least between multiple PRACH transmissions and single PRACH transmissions. Agreement For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, consider one or multiple of the following options. - Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs. - Option 2: Multiple PRACH are transmitted on separate ROs. - Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs. - Other options are not precluded. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

- Based on companies’ contributions, the potential pros and cons of Option 1 and Option 2 are summarized in the following table.

|  | - Pros | - Cons |
| --- | --- | --- |
| - Opt. 1 | - Simple and limited specification impact. - Better preamble utilization than Option 2. | - May result in the increased RACH collision probability and latency, and has a preamble shortage problem. |
| - Opt. 2 | - No need to further partition the preambles and to consider the coexistence of legacy UE. - May realize flexible design of association between SSB and RO. | - Need to configure more PRACH resources. - More specification impact than Option 1, e.g., needs to define separate ROs. |

- For Option 3, companies’ views are quite divergent. Company [vivo] thinks Option 3 can minimize the specification impact, as all repetitions can follow the first repetition with respect to SSB to RO mapping, SSB to RO association period definition, preamble configuration, SSB selection procedures, PRACH resource selection procedures etc. While companies [Huawei, Panasonic, TCL, Intel, Sharp] have the following concerns on Option 3:

- SSB-RO design of additional ROs is very difficult due to following reasons. First, the independent periodic patterns of SSB-RO for legacy and additional ROs cannot be connected seamlessly. Second, the SSB-RO design of additional ROs also has to consider the desired arrangement of TDM-ROs associated with the same SSB for repetition, i.e., additional ROs are located continuously (how many ROs are located continuously) or discretely in time domain.

- Complicated and have large specification impact.

- May introduce unbalanced detection performance for multiple PRACH transmissions, especially when considering the loading conditions for cell edge and cell center UEs may be different.

- Companies' attitude for each option are summarized as follows:

- Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs.

Support: OPPO, ZTE, TCL, Lenovo, Mavenir, Intel, LG, InterDigital, Qualcomm, Ericsson, Samsung, Apple, Panasonic

Deprioritize: vivo, CATT, Chine Telecom

- Option 2: Multiple PRACH are transmitted on separate ROs.

Support: Huawei, HiSilicon, OPPO, CATT, China Telecom, Fujitsu, Lenovo, Sony, Mavenir, Intel, LG, InterDigital, Ericsson, Samsung, Apple, NTT DOCOMO, Sharp, Nokia, NSB, Spreadtrum(?), ETRI, ZTE (based on legacy PRACH configuration with additional new parameters), Panasonic

- Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs.

Support: OPPO, vivo, MediaTek, Spreadtrum(?)

Not support: Panasonic, Mavenir, Intel

It should be also mentioned that, some companies support multiple options, e.g., Companies [Mavenir, Intel, LG, InterDigital, Ericsson, Samsung, Apple, Panasonic] support both Option 1 and Option 2; company [OPPO] supports all of the three options.

Besides, companies [Ericsson, LG, NEC, Apple, ZTE, China Telecom] point out that Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) may be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning. Based on Rel-17 framework, Option 1 can be realized by FeatureCombination-r17 in either legacy configuration or AdditionalRACH-Config-r17, and Option 2 can be realized via AdditionalRACH-Config-r17 and FeatureCombination-r17. Moreover, [Ericsson] provides an illustration as follows:

where Figure (a) shows a legacy Rel-17 RO with two partitions for single PRACH transmission by CBRA and CFRA. Figure (b) illustrates an example of realizing Option 2. Figure (c) and (d) illustrate examples of realizing Option 1, where separate preambles for single PRACH and multiple PRACHs are configured in a legacy RO in Figure (c) and in an additional RO in Figure (d).

In addition, [Ericsson] proposes to send an LS to RAN2, informing RAN2 of RAN1’s decision on the support of separate preamble in a shared RO and separate ROs for multiple PRACH transmissions, and that it is up to RAN2 to decide how to configure RACH resources for Rel-18 multiple PRACH transmissions and possibly other Rel-18 features.

Other companies’ views are summarized as follows:

- [Huawei] Considering that one antenna panel can only generate one analog beam, the resource configuration should be flexible enough to cover all potential gNB RF implementations.

- [ZTE] Completely separating PRACH configuration for multiple PRACH transmissions from legacy PRACH configuration is not preferred.

- [ETRI] The additional sequences can be configured for shared ROs between legacy PRACH transmission and multiple PRACH transmissions.

- [Apple] To support PRACH repetition and reduce the transmission delay, the network should configure the PRACH configuration index with more time domain ROs. There is no need to introduce new PRACH configuration with more time domain ROs for repetition.

- [MediaTek] Consider optimizations to improve the efficiency of system resource utilization by reducing the number of new RACH resources for PRACH repetitions.

Issue #3: Differentiation between different numbers of multiple PRACH transmissions if different “coverage enhancement levels” (number of PRACH repetitions) are supported

Companies also discuss whether/how gNB can differentiate between different numbers of multiple PRACH transmissions. [ZTE, CATT, Intel, NTT DOCOMO] think there is a need to do the differentiation, e.g.., different PRACH resources can be configured per PRACH repetition factors. While [China Telecom, Sony, Panasonic] think PRACH resource can be used for all possible numbers of PRACH transmissions, i.e., not dedicated PRACH resource for a specific number of the PRACH transmissions, or else the required PRACH resources would be too large and it may make the design too complicated.

Considering that this issue is related to the outcome of Issue #1, FL suggests to postpone the discussion.

Issue #4: RO determination for multiple PRACH transmission

Different from legacy single PRACH transmission, where UE can randomly select one of the ROs associated with the selected SSB for PRACH transmission, if UE randomly select a given number of ROs from the ROs assigned for multiple PRACH transmission, gNB may be not able to perform joint detection since gNB is not sure which ROs are utilized by the UE for PRACH transmission. To deal with this issue, it’s necessary to align gNB’s and UE’s understanding about the potential time and frequency position of the corresponding ROs utilized for PRACH repetitions.

- Issue #4-1: RO determination frame work

Based on companies’ contributions, there are at least two methods to deal with the above-mentioned issue.

- Method 1: Define a term of “RO group”

In RAN1 #111, a term of “RO group” was discussed but without consensus, the latest FL proposal is listed as follows:

Proposal 9-new-b For multiple PRACH transmissions with same Tx beam, a term of "RO group" is introduced to facilitate further discussion. For each RACH attempt, UE determines and selects one RO group for multiple PRACH transmissions, where RO(s) for a specific number of multiple PRACH transmissions are configured [, and transmit PRACH in the RO(s) within the RO group]. [Note 1: Determination and selection of RO group (including related configuration) is a separate discussion, including which WG should work on the configuration aspects, details of the signaling structures (if any).] Note 2: Each RO in one RO group is associated with the same SSB(s). Note 3: Whether/How to capture the RO group in the spec. is a separate discussion.

Based on the contribution, companies [OPPO, Spreadtrum, FGI, China Telecom, Fujitsu, Panasonic, Lenovo, ETRI, Ericsson, Apple, Samsung, Nokia, Intel] discuss the details of “RO group/window” and companies views are summarized as follows:

(1) Whether one RO group only consist of valid RO(s)?

Yes: Spreadtrum, ZTE, Nokia, NSB

(2) Whether one RO group consist of TDM-ed and/or FDM-ed ROs?

TDMed ROs: OPPO, Spreadtrum, apple, Nokia, NSB

FDMed ROs: Fujitsu

(3) Whether the number of RO(s) in one RO group is equal to a specific number of multiple PRACH transmissions configured by gNB? In other words, if a UE determines a RO group, does UE also needs to determine the ROs within the RO group?

Equal to a Specific number: OPPO, Spreadtrum, ETRI, Ericsson, Samsung, ZTE, Nokia, NSB

(4) What is the UE behavior if some PRACH transmission(s) is dropped due to e.g., dynamic SFI or other potential collisions? Does UE continue the PRACH transmission in some other ROs to achieve the number of multiple PRACH transmissions configured by the gNB?

The dropped PRACH transmission is not postponed to another RO group: Spreadtrum, Ericsson, Nokia, NSB

Other views are summarized as follows:

- [Ericsson] proposes to send an LS to RAN2, informing RAN2 of RAN1’s agreement on RO group, and that it is up to RAN2 to decide whether/how to capture RO group in specification.

- [Apple] RO is replaced with RO group to perform the association with SSB. Otherwise, there is no strong motivation to introduce a new term.

- [Panasonic] "RO group" needs to be defined from UE perspective or cell perspective, respectively. We call them as a RO UE group or a RO cell group, where a RO UE group includes ROs used in one PRACH attempt for the multi-PRACH transmission and a RO cell group is determined based on clustering ROs located in close proximity to one another in time domain, or frequency domain, or in both time and frequency domains. As illustrated in the following Figure.

Method 2: Define the starting ROs for multiple PRACH transmission

Companies [Lenovo, LG, Apple] propose to configure starting ROs, and PRACH repetition starts from a starting RO and repeats in the following ROs that are associated with a same SSB.

- Issue #4-2: Whether/how the starting RB of ROs can be different at different time instances for multiple PRACH transmissions

The following agreement was achieved in RAN1 110b-e for ROs assignment for multiple PRACH transmissions [4]:

Agreement - For multiple PRACH transmissions with same beam, at least ROs located at different time instances can be utilized for the transmissions. - FFS: whether/how the starting RB of ROs can be different at different time instances for multiple PRACH transmissions. - FFS: whether/how multiple PRACH transmissions located in the same time instance, e.g., for UEs with multiple Tx chains.

Based on the contributions, companies make further discussion on the 1st FFS parts and companies’ views are summarized as follows:

Companies [vivo, Intel, OPPO, CATT, TCL, Apple, LG, NTT DOCOMO, Qualcomm, Nokia, Samsung] propose to consider different starting RB of ROs at different time instances (RO hopping) for multiple PRACH transmissions. While companies [xiaomi, Lenovo] do not support different starting RB of ROs at different time instances for multiple PRACH transmissions. [China Telecom] think whether to introduce frequency hopping for multiple PRACH transmissions depends on whether it will increase the blind detection complexity on gNB side.

Moreover, as pointed out by Lenovo, the ROs for PRACH transmission may be result in the following pattern under some specific configurations. It needs to study RO determination for PRACH transmission when FDMed ROs are configured and are associated with same SSB.

Some simulation results are provided by companies as follows:

- [vivo] The gain of PRACH repetition with RO hopping is about 0.7dB compared to that of PRACH repetition without RO hopping in urban O2I scenario.

- [Intel] For 2 and 4 PRACH repetitions with frequency hopping, ~2.5dB performance gain can be achieved compared to PRACH repetitions without frequency hopping. (@700MHz, PRACH format 0, TDL-C, 300ns)

Other views are summarized as follows:

- [Qualcomm] For multiple PRACH transmissions with same beam, the starting RB of ROs can be different at different time instances for multiple PRACH transmissions. FFS how to handle transmission at time having multiple ROs associated with the same SSB but the RO that would have been use for PRACH transmission is invalid.

- [Nokia] Define partly overlapping groups of ROs with different starting RB for the different time instances of the multiple PRACH transmissions. RAN1 to discuss the multiple PRACH transmissions located in the same time instance, if applicable, after discussions on the feature of RO grouping are completed.

- Issue #4-3: Whether/how multiple PRACH transmissions located in the same time instance

The following agreement was achieved in RAN1 110b-e for ROs assignment for multiple PRACH transmissions [4]:

Agreement - For multiple PRACH transmissions with same beam, at least ROs located at different time instances can be utilized for the transmissions. - FFS: whether/how the starting RB of ROs can be different at different time instances for multiple PRACH transmissions. - FFS: whether/how multiple PRACH transmissions located in the same time instance, e.g., for UEs with multiple Tx chains.

- Based on the contributions in RAN1 #111 and RAN1 #112, companies make further discussion on the 2nd FFS parts and companies’ views are summarized as follows:

- Companies [Huawei, Fujitsu, xiaomi, Intel, ETRI, Lenovo, LG, MediaTek, Nokia, Samsung] propose not to support/ deprioritize multiple PRACH transmissions over FDM-ROs in the same time instance due to the following reasons: small benefit and high complexity; there may be more than one resource multiplexing modes in this network, which will greatly increase the complexity of the non-coherent combining detection at gNB side; only half of the deliverable power is available for each Tx chain.

- While companies [ZTE, CMCC, Ericsson] think multiple PRACH transmissions located in the same time instance can be further studied for UE with 2 or more Tx chains. Moreover, [Ericsson] proposes to support simultaneous FDMed/CDMed PRACH transmissions in Rel-18, including the association of different FDMed ROs/the association of different preambles in an RO within one RACH attempt.

In addition, [Ericsson] provides some simulation results as follow:

- FDM approach of two PRACHs outperforms a single PRACH transmission with one antenna by 1.5 dB and a single PRACH with two antennas by 0.5dB.

- CDM approach of two PRACHs outperforms a single PRACH transmission with one antenna by 1 dB and has nearly the same performance as a single PRACH transmission with two antennas.

Issue #5: Same or different preamble(s) during multiple PRACH transmission

The following agreement was achieved in RAN1 110b-e:

Agreement - For multiple PRACH transmissions with same beam, at least support to use same PRACH preamble during the multiple PRACH transmissions in one RACH attempt. - FFS: whether different preambles can be utilized in different PRACH transmissions during the multiple PRACH transmissions in one RACH attempt.

Regarding the FFS parts, companies’ views are summarized as follows:

Companies [Huawei, vivo, TCL, Fujitsu, CATT, xiaomi, Intel, Lenovo, CMCC, Mavenir, Nokia] propose not to support/ deprioritized utilize different preambles during the multiple PRACH transmissions in one attempt due to the following reasons: there is no additional performance gain and non-coherent combining gain cannot be obtained; the receiver complexity increases when combining PRACH repetitions using multiple preambles; higher standards impact.

While [ZTE] proposes to consider different preambles during the multiple PRACH transmissions in one attempt in order to reduce or randomize the preamble collision with other UEs and reduce the interference. [Samsung] thinks that this issue may be related to RO determination for multiple PRACH transmission, different preambles in different PRACH transmissions can be considered if PRACH transmission is regarded independently to each other. Thus, [Samsung] proposes to defer the determination of whether support different preambles apply to the multiple PRACH transmissions in one attempt until the RO determination design is clearer.

Issue #6: SSB-to-RO mapping

Based on the contributions, companies [xiaomi, Nokia, Huawei, ZTE, Intel, OPPO, Spreadtrum, LG, China Telecom, Panasonic, TCL] discuss the SSB-to-RO mapping for multiple PRACH transmsisions. For legacy SSB-to-RO mapping, SS/PBCH block indexes provided by ssb-PositionsInBurst are mapped to valid PRACH occasions in the following order: preamble indexes, FDMed RO indexes, TDMed RO indexes. The current mapping order leads to long latency between TDMed ROs for multiple PRACH transmission, since TDMed RO indexes has the lowest mapping priority.

Detailed companies views are summarized as follows:

Companies [Panasonic, Intel] propose to reuse the legacy mechanism for SSB-to-RO mapping for the multi-PRACH transmission. Companies [Spreadtrum, LG] propose that for multiple PRACH transmission with separate preamble on shared ROs, support following the legacy mechanism between SSB and RO mapping.

Companies [Spreadtrum, LG, ZTE, xiaomi] propose to consider SSB-to-RO group/set mapping, which replace the RO with a group/set of ROs. The mapping order between SSB and RO groups can follow the current rules. For example, as illustrated by [LG] in the following figure.

Companies [OPPO, China Telecom, Nokia] propose to consider SSB-to-RO mapping enhancement to allow more consecutive TDMed ROs for multiple PRACH transmission, e.g., SSB are mapped to valid ROs for multiple PRACH transmission first in the time domain and then in the frequency domain.

[Huawei] SSB beams should be classified into different coverage enhancement levels, where SSB beams belonging to the same level are associated with the same number of ROs for repetition but SSB beams belonging to different levels are associated with different number of ROs for repetition.

[CATT] propose that validation rule should be performed before determining the ROs used for multiple PRACH transmissions.

[Qualcomm] propose that the counting of PRACH repetitions is based on the valid ROs. Moreover, [Qualcomm] propose that PRACH repetitions are only transmitted in the valid ROs associated with the same SSB at different time with the following order:

- First, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot.

- Second, in increasing order of indexes for PRACH slots.

- Third, in increasing order of indexes for PRACH association period.

2.1.2 RAR window and RA-RNTI calculation

Issue #7: RAR window and RA-RNTI

The following agreement was achieved in RAN1 110b-e for RAR monitoring:

Agreement For multiple PRACH transmissions with same beam, for RAR monitoring, consider the following options. - Option 1: One RAR window per each PRACH transmission, the RAR window follows the legacy design. - FFS: RA-RNTI. - Option 2: Only one RAR window for all of the multiple PRACH transmissions. - FFS: the start position of the RAR window. - FFS: RA-RNTI.

- Issue #7-1: RAR window

Some illustrations for Option 1 and Option 2 are listed as follows for reference.

Illustration of Option 1

Illustration of Option 2

Based on the contributions, companies’ support on Option 1 and Option 2 are summarized as follows:

- Support Option 1: Sony, NTT DOCOMO (If PRACH repetitions cannot be identified)

- Support Option 2: Huawei, HiSilicon, OPPO, Spreadtrum, ZTE, vivo, CATT, China Telecom, Panasonic, TCL, Intel, Lenovo, ETRI, InterDigital, Apple, LG, Qualcomm, Mavenir, MediaTek, NEC, Nokia, NSB, xiaomi, NTT DOCOMO (If PRACH repetitions can be identified)

For Option 2, mainly two alternatives are proposed for the start position of the RAR window:

- Alt. 1: the starting point of RAR window is after the last symbol of the last PRACH occasion corresponding to the multiple PRACH transmissions.

- Support: OPPO, Spreadtrum, ZTE, vivo, CATT, TCL, Intel, Lenovo, InterDigital, LG, MediaTek, NEC, Nokia, NSB, xiaomi, China Telecom, Samsung, Panasonic (for achieving a combined detection of a multi-PRACH transmission), ETRI

- Alt. 2: the start position of the RAR window is after the last symbol of the first PRACH occasion corresponding to the multiple PRACH transmissions.

- Support: Lenovo, Mavenir, Panasonic (for enabling early termination of a multi-PRACH transmission)

Based on the contributions, it can also be known that RAR monitoring issue is related to Issue #3, it depends on whether gNB can differentiate between different numbers of multiple PRACH transmissions if different “coverage enhancement levels” (number of PRACH repetitions) are supported. Meantime, as some company points out, it needs to clarify whether the “PRACH occasion” in the above alternative means the configured one.

[InterDigital] The start of the RAR window is separately configured for each value of the number of multiple PRACH transmissions.

[Spreadtrum] Two RAR windows should be applied if Option 3 in the agreement in Issue#1 is introduced, i.e., one RAR window is used for partial of multiple PRACH transmissions with shared ROs, the other RAR window is used for the remaining of multiple PRACH transmissions with separate ROs. Once UE receives Msg2, UE is able to terminate the follow up PRACH transmission in advance and release the corresponding RO resources for other UEs to access.

- Issue #7-2: RA-RNTI

According to current spec. TS 38.321, RA-RNTI is calculated as follows:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

For multiple PRACH transmissions, the RA-RNTI calculation is related to RAR window design. Based on the companies’ contributions [ZTE, China Telecom, Spreadtrum, CATT, xiaomi, Panasonic, Intel, OPPO, TCL, Lenovo, Apple, LG, NEC, Nokia], there are two options proposed for RA-RNTI calculation as follows:

- Option 1: Multiple RA-RNTI candidates within one RAR window, i.e., UE attempts to detect a DCI format 1_0 with CRC scrambled by one of the multiple RA-RNTI candidates during a RAR window.

- Option 2: Single RA-RNTI within one RAR window, i.e., UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a RAA window.

  - Option 2-1: The corresponding RA-RNTI is calculated based on RO for the last PRACH repetition.

  - Option 2-2: The corresponding RA-RNTI is calculated based on RO for the first PRACH repetitions.

  - Option 2-3: The corresponding RA-RNTI is calculated based on RO for a predefined PRACH repetitions except the last and first one.

  - Option 2-4: The corresponding RA-RNTI is calculated as a function of the sequence of ROs used for the multiple PRACH transmissions

For Option 1, it indicates that UE should assume multiple RA-RNTIs candidates within one RAR window. This may happen for the case that multiple RAR windows are utilized and there is overlapping between RAR windows.

For Option 2, it indicates that UE only expects one RA-RNTI candidate within one RAR window, UE doesn’t need to assume multiple candidates of RA-RNTI and UE will not increase the complexity on the reception of RAR. Option 2 is workable for single RAR window design.

Meantime, as some company points out, it needs to be clarified that whether the “RO” in the above Options means the configured one (consider the last or first PRACH repetition is dropped).

2.1.3 Determine the number of multiple PRACH transmissions

Issue #8: Candidate value

As companies point out, the performance gap for FR2 PRACH channel has been derived based on MIL criterion referring to the coverage range of PUCCH format 1 in TR 38.830 based on the link budget evaluation in Rel-17 as follows:

| Scenarios | Target metrics | Channels | MIL |  |
| --- | --- | --- | --- | --- |
|  |  |  | Number of samples | Relative difference vs. PUCCH Format 1 |
| Urban 28GHz TDD NLOS O2I | Scenario dependent target ISD=200m | PRACH Format B4 | 6 | -1.92 |
| Urban 28GHz TDD NLOS O2O | Scenario dependent target ISD=200m | PRACH Format B4 | 5 | -7.57 |

Besides, companies have the following observations which may facilitate the determination of number of multiple PRACH transmission:

- [ZTE] If the joint detection of the received PRACH repetitions can be performed at gNB side, the simulation results showed about 1.7~3.7 dB and 3.7~5.2 dB gain can be obtained by employing 2 repetitions and 4 repetitions respectively in case of PRACH repetition with same beam (@28GHz). It seems the 4 repetitions can hardly compensate the -7.57dB gap. So at least, up to 8 repetitions should be supported.

- [vivo] In Urban O2O scenario @28GHz (30km/h, DDDSU), the performance gain of PRACH repetition is about 4.31dB for 2 PRACH repetition and 7.95dB for 4 PRACH repetition. In Urban O2I scenario @28GHz (3km/h, DDDSU), the performance gain is about 3.06dB for 2 PRACH repetition, 6.31dB for 4 PRACH repetition

- [Xiaomi] For FR2 in Urban@28GHz O2O scenario, about 2.9dB and 5.1dB performance gain can be obtained with 2 and 4 PRACH repetitions, respectively.

- [Intel] About 2.1dB performance gain can be achieved for PRACH transmission when repetition level is doubled. (@700MHz, PRACH format 0)

- [Ericsson] About 2dB gain is observed when the number of PRACH transmissions doubles, and the gain slightly decreases when the number of PRACH transmissions increases. It is observed for PRACH transmissions with the same best beam, the same wide beam, or with different beams. (@28GHz, PRACH format B4, CDL-A (DS 100ns))

Based on the discussion in RAN1 #110b-e, a working assumption about the candidate values for multiple PRACH transmissions are proposed as follows.

Support at least {2, 4, [8]} for the number of multiple PRACH transmissions with same beams. - FFS other numbers.

During the discussion in past meetings, the majority companies [ZTE, Intel, CATT, FGI, DOCOMO, Panasonic, Qualcomm, LG, Samsung, CMCC, Spreadtrum, Lenovo, Nokia, NSB, Sony, ETRI, InterDigital, Fujitsu, Huawei, HiSilicon, Sharp, OPPO, MediaTek, Apple] support the candidate value of {2, 4, 8}. While [Ericsson] thinks when studying the number of multiple PRACH transmissions to be supported, PRACH transmissions with the same wide beam is the basis to determine the maximum number of multiple PRACH transmissions with the same beam, and the difference in Msg3 and PRACH performance should be evaluated.

Issue #9: Determination of the number of multiple PRACH transmissions

In RAN1 #111, the following agreement was achieved for determination of the number of multiple PRACH transmissions:

Agreement - For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt. - Note: whether to support multiple numbers of PRACH transmissions is separately discussed.

- Issue #9-1: Other factors other than SSB-RSRP threshold(s) for determination the number of multiple PRACH transmissions

Based on the contributions, companies think some other factors than SSB-RSRP needs to be considered when determine the number of PRACH transmissions, as summarized below:

Factor 1: The number of single PRACH attempts exceeds a threshold or the calculated power of single PRACH reaches the maximum output power of UE.

Support companies: ZTE, Nokia, OPPO, vivo, Qualcomm

Factor 2: UE power class.

Support companies: Qualcomm

Factor 3: UE power headroom.

Support companies: Ericsson

Factor 4: MPE or P-MPR.

Support companies: Fujitsu, Samsung

Companies [CATT, Ericsson, NTT DOCOMO, Spreadtrum, FGI, Panasonic] proposes that different methods are applied for CBRA and CFRA, respectively. UE determination of the number of PRACH transmissions by at least SSB-RSRP threshold(s) only applies to CBRA. For CFRA, the number of PRACH transmission(s) is determined and indicated by gNB, e.g., via PDCCH order.

[xiaomi] proposes that whether to transmit PRACH with repetitions and how many repetitions should be transmitted is only determined by SS-RSRP measurement at the UE side.

[CATT] proposes to further study whether to introduce a separate lower RSRP threshold for SSB/CSI-RS determination for PRACH repetitions.

- [Nokia] proposes to investigate aspects of UE selection of the SSB index based on expected UL link budget calculated as a function of e.g., SS-RSRP measurements and expected link gain corresponding to the number of PRACH transmissions.

- Issue #9-2: Determination of the number of multiple PRACH transmissions if multiple PRACH coverage enhancement levels are supported

If multiple PRACH coverage enhancement levels are supported, companies [Huawei, vivo, CATT, Lenovo, Intel, LG, MediaTek] propose to configure several RSRP thresholds for UEs to determine repetition level.

- Companies [OPPO, Apple, Panasonic] propose that Msg3 RSRP threshold is re-used to determine the PRACH repetition.

- [TCL] The number of repetitions of PRACH transmission can be indicated by random access configuration table. Collision factor between UEs should be considered when to determine the maximum number repetition of PRACH transmission.

- [CATT] The RSRP threshold(s) for PRACH repetition request should be separate from that for Msg3 repetition request.

- [Intel] A common SSB-RSRP threshold may be applied for multiple PRACH transmissions and request of Msg3 PUSCH repetitions.

- [Ericsson] Study how to simplify the configuration of SSB-RSRP thresholds for multiple PRACH transmissions and Msg3 repetition and reduce gNB complexity, for instance by using rsrp-ThresholdMsg3 for UEs to determine at least one repetition factor for PRACH transmissions.

2.1.4 Power control

Issue #10: Power ramping

- Based on the discussion in RAN1 #110b-e, power ramping for multiple PRACH transmissions with same beam in one RACH attempt was discussed. The latest FL proposal was listed as follows for reference.

For multiple PRACH transmissions with same beam in one RACH attempt, down-select one option from the following options. - Option 1: Transmission power ramping is not applied during the multiple PRACH transmissions. - Option 2: Transmission power ramping can be applied per PRACH transmission during the multiple PRACH transmissions. - FFS: The initial power and power ramping step. - FFS: The same measurement of the same reference signal to calculate the pathloss is applied for each PRACH transmissions.

Based on the contributions, companies’ views on the above two options are summarized as follows.

- Support Option 1: Huawei, OPPO, Sony, CATT, China Telecom, Panasonic, CMCC, LG, Samsung

- Support Option 2: ZTE

For Option 1, [Huawei, LG, Samsung, China Telecom, NTT DOCOMO, CATT] think the same measurement of the same reference signal is utilized to calculate the path loss for each of the Multiple PRACH transmissions during one RACH attempt.

[ZTE] Separate counter and step size of power ramping for multiple PRACH transmissions in one attempt can be configured. The step size of power ramping for multiple PRACH transmission can also be configured as zero.

[Panasonic] If the multi-PRACH transmission takes long time by configuring a sparse density of ROs in a frame, not using the latest path loss measurement can result too high or too low PRACH transmission power. Therefore, gNB can configure how path loss measurement is taken would be reasonable, e..g, gNB can configure either to compensate path loss or not compensate the path loss.

Issue #11: Power calculation

Companies [Huawei, ZTE, China Telecom] discuss the power calculation for multiple PRACH transmissions. Companies [Huawei, China Telecom] propose that the power control of multiple transmission specified in eMTC PRACH coverage enhancement can be starting point as follows:

Step 1: Calculate the total receive target power.

PREAMBLE_RECEIVED_TARGET_POWER=preambleInitialReceivedTargetPower+DELTA_PREAMBLE + (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep

Step 2: Calculate the target receive power for one single transmission.

PREAMBLE_RECEIVED_TARGET_POWER=PREAMBLE_RECEIVED_TARGET_POWER - 10 * log10(numRepetitionPerPreambleAttempt)

Step 3: Calculate the transmission power for one single transmission.

P_PRACH = min {P_CMAX, PREAMBLE_RECEIVED_TARGET_POWER + PL_c} [dBm]

[ZTE] proposes that an additional power offset for multiple PRACH transmissions should be considered for PREAMBLE_RECEIVED_TARGET_POWER calculation.

2.1.5 Retransmission of multiple PRACH transmissions

- Based on the contributions, companies’ views on the retransmissions of multiple PRACH transmissions are summarized as follows:

- The number of PRACH transmissions in PRACH retransmissions.

- Based on the contributions, companies [Huawei, ZTE, CATT, Nokia, OPPO, China Telecom, Fujitsu, InterDigital, Samsung, Sharp] discuss about the determination of the number of PRACH transmission(s). The potential issues include: the relation between the number of PRACH transmission in Re-attempts and that in first attempt; the relation between power ramping and increasing the number of PRACH transmissions. Detailed companies’ views are summarized as follows:

- [Huawei, ZTE, InterDigital, Sharp] proposes that the selected repetition level should increase in next retransmission when the transmission power of retransmission reaches the maximum transmission power.

- [China Telecom] proposes that if SSB or CSI-RS selected is not changed from the selection in the previous multiple PRACH transmissions, the number of PRACH retransmissions in one RACH attempt is equal to or larger than that of the previous multiple PRACH transmissions. Else, the number of multiple PRACH retransmissions in one RACH attempt is determined in the same way as first RACH attempt.

- [xiaomi] proposes to use the same number of PRACH transmissions for subsequent RACH attempts.

- [CATT, Fujitsu] propose that for subsequent RACH attempt(s), power ramping should be applied first while the number of PRACH repetitions and SSB/CSI-RS are kept unchanged until a certain condition is met.

- [Nokia] Define SSB-RSRP tolerance zone to allow a UE to increase the number of PRACH transmissions in case of PRACH re-attempt. Define a procedure for increasing the number of the multiple PRACH transmissions at different RACH attempts based on adapting the value of the measured SSB-RSRP.

- [Samsung] In the PRACH re-attempt in current RACH procedure, UE is allowed to update/change the selected SSB. Thus, there is no pre-information at gNB on which UE it’s going to detect. It seems natural to also support UE to re-determine whether support multiple PRACH transmission during the whole RACH procedure or not.

- [OPPO] proposes two options: Opt.1: Power ramping can be applied for the next attempt of multiple PRACH. The power of PRACH is ramped with the increase of multiple PRACH attempt; Opt.2: Power ramping is not applied for the next attempt of multiple PRACH. The number of multiple PRACH is ramped with the increase of multiple PRACH attempt.

- The maximum number of retransmissions

- Companies [Huawei, China Telecom, NEC] propose that the maximum retransmission time of large repetition levels should be reduced to deal with the harmful retransmission.

- Power ramping

- [Huawei, NEC] propose to configure a separate power ramping step for multiple PRACH transmission in Re-attempts.

2.1.6 Others

- Timing of UE starts the monitoring PDCCH for BFR

[Lenovo] When there are multiple ROs used for PRACH transmissions, the timing of the UE starts the monitoring PDCCH for BFR should also be decided. Same method as RAR could be considered.

2.2 Multiple PRACH transmissions with different beams

Based on the contributions, companies’ general views on multiple PRACH transmissions with different beams are summarized as follows:

Companies [Spreadtrum, ZTE, Intel, TCL, Lenovo, Nokia, NTT DOCOMO (if specification impact is not large), Ericsson, Samsung] propose to support multiple PRACH transmissions with different beams, while [vivo] thinks PRACH repetition with different beams can be up to UE implementation and no specification work is needed on this in Rel-18 coverage enhancement work item. [Panasonic, LG, CMCC] propose to prioritize the work on multi-PRACH transmission with the same beam. Based on companies’ contributions, some Pros and Cons

of multiple PRACH transmissions with different beams are summarized in the following table.

| - Pros | - Cons |
| --- | --- |
| - Latency reduces in multiple transmission with different beams as ROs of different beams are used for repetition. - Multiple PRACH transmissions with different beams increases robustness even for UE with beam correspondence. - PRACH transmission with different beams could increase the possibility of being detected by any of the TRPs which may locate in different directions. - Msg3 transmission may be transmitted with the best narrow beam observed during PRACH. | - BS has to indicate the association groups of beams, UEs select association group for repetition correspondingly, and extra operation is required to determine the RAR beam, which results in an increased complexity and signaling cost. - The benefits and target scenarios are not clear. - UE complexity will increase obviously. - Larger spec. impact. |

2.2.1 Potential use cases

In RAN1 #111, the following agreement was achieved about the potential use case for multiple PRACH transmissions with different beams:

Agreement Study at least the following case for multiple PRACH transmissions with different Tx beams. - UE uses different TX beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS - FFS: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs /CSI-RSs, where the different SSBs/CSI-RSs are not associated with the same RO. - Note: not related to decision on CFRA Note: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs/CSI-RSs, where the different SSBs/CSI-RSs are associated with the same RO is not considered.

Based on the contributions, companies [InterDigital, NTT DOCOMO, ZTE, CATT, Intel] propose to focus on/support UE using different Tx beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS, [Qualcomm] proposes to focus on its benefit for UL beam refinement in RACH and the potential necessary specification impact, including the selection of UL Tx beam for Msg3. Companies [Mavenir, China Telecom] propose that the use case of UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs /CSI-RSs (the different SSBs/CSI-RSs are not associated with the same RO) is deprioritized/not supported.

Other companies’ views are summarized as follows:

- [Ericsson] Firstly, UEs not supporting beamCorrespondenceWithoutUL-BeamSweeping can choose to transmit multiple PRACHs with the same wide beam repeatedly or by sweeping different narrow beams with a possibly better directivity. Secondly, UEs supporting beamCorrespondenceWithoutUL-BeamSweeping can also take advantage of the same or different beams for multiple PRACH transmissions. Repeating PRACH transmission with the refined beam can maximize beamforming gain.

- [MediaTek] Multiple PRACH transmissions using different beams should be studied for the two following cases separately: A) For UEs without beamCorrespondence feature support, and B) For UEs with beamCorrespondence feature support.

- [Spreadtrum] Multiple PRACH transmissions with same beam can be used as a baseline for different beams.

- [CATT] The benefit and target scenario of multiple PRACH transmissions with different beams should be provided to justify the support of multiple PRACH transmissions with different beams.

- [Sharp] RAN1 should study if any triggering condition is necessary for multiple PRACH transmissions with different UL Tx beams.

- [Panasonic] Multiple PRACH transmissions using different beams can be same as the design for multiple PRACH transmissions using same beams with early termination, i.e., not to have a combined detection of a multi-PRACH transmission.

2.2.2 Performance gain

In RAN1 #111, the simulation assumptions about multiple PRACH transmissions were discussed and the following working assumption was achieved:

Working Assumption Simulation results for multiple PRACH transmissions with different beam(s) and same beam(s) (baseline) to be discussed in the next meeting. - Simulation assumptions in TR 38.830 are used as the starting point for the simulation. - Focus on FR2. - UE antenna configuration 2-2-2(baseline), 1-4-1(optional) - Performance metric: 0.1% false alarm, 1% miss-detection - Companies report the number of beams, the beam widths, beam correspondence assumption, and the boresights. - Channel model for link-level simulation: CDL-A defined in table 7.7.1-1 in TR 38.901. - Both that UE fulfills beamCorrespondence requirements Without UL-BeamSweeping and UE fulfils beamCorrespondence requirements With UL-BeamSweeping can be considered in the simulation are used as starting point for simulation

Based on the contributions, companies’ simulation results are summarized as follows:

- [vivo] Coverage performance of PRACH repetition with beam sweeping and rough beam direction:
The performance gain of PRACH repetition with single beam is 3dB better than that of PRACH repetition with beam sweeping among beams far apart. 
The performance gain of PRACH repetition with single beam is about 1 dB better than that of PRACH repetition with beam sweeping among beams in the directions close to the best Tx beam. 
Even if the UE can not find the best beam direction, when the TX beam sweeps around the direction of best Tx beam, there could be close to 6dB gain from PRACH repetition. 
(@28GHz, 30km/h, 100MHz, UE Tx chains [2 2 2], Azimuth angle set 1 = [-3*pi/8, -pi/8, pi/8, 3*pi/8 ] vs. Azimuth angle set 2 = [x-3*pi/32, x-pi/32, x+pi/32, x+3*pi/32 ] where x is determined such that set2 are around the angle in set1 corresponding to best TX beam, Zenith angle set = [pi/4, 3*pi/4])

- [vivo] Coverage performance of PRACH repetition with beam sweeping and without rough beam direction:
Compared to the single PRACH transmission with omni beam, the performance gain of three configuration cases is 5.28dB (case 1: 4 PRACH repetitions with the same omni beam), 7.29dB (case 2: 4 BeamSweeping-BestUEdirection) and 4.43dB (case 3: 4 BeamSweeping-RandomUEdirection).
The evaluation results demonstrate that the performance of beam sweeping is highly related to the direction of UE panel in practice, and the average gain of beam sweeping is smaller than that of omni beam.
(@28GHz, 30km/h, 100MHz, UE Tx chains [2 2 2], Azimuth angle set = [-pi/4, pi/4 ], Zenith angle set =  [pi/4, -pi/4])

- [Ericsson] For the same number of PRACH transmissions, the transmission with different beams (beam sweeping) has a loss of about 5dB compared with transmissions with the same best beam, which is generated by four times number of antennas. A single PRACH transmission with the best beam out of eight candidate beams has a performance between PRACH transmissions by sweeping four beams and eight beams. (@28GHz, 3km/h, UE antennas [2 2 2], BS antennas [1 2 2], Azimuth angle set = a wide beam, [-pi, 0], [-pi, -pi/2, 0, pi/2], Zenith angle set = [0, pi/2] for 2,4,8 PRACH transmissions)

- [Ericsson] About 2dB gain is observed when the number of PRACH transmissions doubles, and the gain slightly decreases when the number of PRACH transmissions increases. It is observed for PRACH transmissions with the same best beam, the same wide beam, or with different beams. (@28GHz, PRACH format B4, CDL-A (DS 100ns))

- [Nokia] Beam sweeping to 4 different directions provides an advantage of ~1.8dB over 4 PRACH repetitions using a single wide beam. (@28GHz, 3km/h, UE antenna [2 2 2], BS antenna [4 8 2 2 2], Azimuth [deg], Elevation [deg] = (-45,45), (-45,135), (45,45), (45,135); Optimal direction: Azimuth [deg], Elevation [deg] = (-4.2,93.2))

- [Nokia] Beam sweeping to 4 different directions provides an advantage of ~3dB over 4 PRACH repetitions using a single wide beam (@28GHz, 3km/h, UE antenna [1 4 1], BS antenna [4 8 2 2 2], Azimuth [deg], Elevation [deg] = (0,22.5), (0,67.5), (0,112.5), (0,157.5); Optimal direction: Azimuth [deg], Elevation [deg] = (-4.2,93.2))

- [ZTE] For PRACH transmission, about 0.83~1.18 dB gain can be obtained by employing 2, 4, or 8 repetitions with fine beam sweeping compared with 2, 4 and 8 repetitions with same rough beam respectively. About 0.54~1.13 dB gain can be obtained by employing 2, 4, or 8 repetitions with beam correspondence compared with 2, 4 and 8 repetitions with fine beam sweeping respectively. (@30GHz, 3km/h, 100MH, UE Tx chains [1 1 2] for rough beam, [2 2 2] for 2/4/8 repetitions with fine beam and with beam correspondence, Zenith angle set [0, pi/6], Azimuth angel set [0], [0, 7*pi/6], [0, pi/6, pi, 7*pi/6] for 2,4,8 repetitions, respectively.).

- [Huawei] Compared to the same wide beam for multiple PRACH transmission, if different Tx beams are finer beams, then 3.9-5 dB gains are observed assuming that only one PRACH occasion with the best detected SINR is selected at the gNB reception, where the beam gain of fine beam is 4 times that of wide beam.

- [Mavenir] PRACH transmissions with best beam (w/o repetition) has about 5.5dB gain over transmissions with beam sweeping (8 beams). (@28GHz, 3km/h, UE antennas [2 2 2], BS antennas [1 2 2], 8 beams with boresight [0, -10, -21, -32, 42, 32, 21, 10])

Other companies’ views are summarized as follows:

- [HW] The fading channel of PRACH over different beams can be extremely different, especially for phase rotation, which makes coherent detection impossible. Without coherent detection, the detection performance upper bound can be observed by the maximum of beam gains among different beams.

- [vivo] Beam width of PRACH transmission is restricted by UE antenna configuration and Tx beams of PRACH can hardly to be assumed to be narrower than SSB beams. No UE capability has been introduced for UE beam correspondence in RRC inactive/idle UEs. i.e., it should be up to UE implementation to determine the Tx beam.

2.3 Interaction between multiple PRACH transmissions and other transmission

When multiple PRACH transmissions is enabled, it may have some interaction with other transmissions, e.g., Msg3 repetitions. Companies [Huawei, ZTE, vivo, CATT, Spreadtrum, OPPO, vivo, Intel, Qualcomm, Ericsson, CMCC, Panasonic, Sharp, Apple] think the coupling/interaction between PRACH repetitions, Msg.3 repetitions (and PUCCH repetitions for HARQ-ACK of Msg4) should be investigated.

- [ZTE, Panasonic, Qualcomm, CATT, vivo] The coupling between PRACH repetitions and Msg3 repetitions should be investigated. E.g., Msg3 repetition is mandatory if multiple PRACH transmissions is applied.

Besides, [Ericsson] has the following observations based on link-level simulation and propose to study how Msg3 performance can be improved by PRACH transmissions with different beams:

- In FR2, the required SNR for Msg3 with 8 repetitions and inter-slot frequency hopping at 10% BLER is 1.7 dB higher than that of a single PRACH transmission with a wide beam and 8 dB higher than a single PRACH transmission with the best beam for 1% missed detection. The gap could be 4.5 dB more for 10% mis-detection rate.

- With Rel-18 PRACH enhancement, the performance gap between Msg1 and Msg3 would grow. Msg3 needs further enhancement to be on par with Rel-18 PRACH.

- [OPPO] For legacy Msg3 transmission, the power is calculated based on the total preamble power ramping of performed PRACH transmission. When multiple PRACH transmissions is applied, how to calculate the total preamble power ramping during multiple PRACH transmissions attempts should be studied.

- [Panasonic] When multi-PRACH transmission is triggered, the mechanism to enable more repetitions and/or lower MCS index than the Rel.17 configured set for Msg3 repetition should be supported.

- [Huawei] The following beam indication methods can be studied, including associating beam candidates with the time or frequency locations of PDCCH or RAR, associating beam candidates with RA-RNTIs, and introducing a new field for beam indication.

2.4 CBRA and CFRA

Based on the contributions, companies [Spreadtrum, ZTE, vivo, NTT DOCOMO, Ericsson, Qualcomm, Panasonic] propose to support multiple PRACH transmissions for both CBRA and CFRA. For CFRA, applying multiple PRACH transmissions can improve PRACH detection rate in SNR limited scenarios, which is essential to the cases of handover and beam failure recovery. Moreover, for CFRA, it is more flexible for network to configure the PRACH resources for PRACH repetition as dedicated signalling can be applied.

2.5 Others

- RACH Procedure

[Samsung] The multiple PRACH transmissions is kept as one RACH procedure

- Switching Tx filter within RO boundaries

[Nokia] Investigate mechanisms for switching Tx filter within RO boundaries for short PRACH formats. E.g., more than 1 Tx filter would be used per RO, where the Tx filters would be changed in a TDM manner.

- Frequency domain allocation of PRACH preamble

[Nokia] RAN1 to investigate mechanisms for transmission of subsets of the frequency representation of the PRACH preamble in ROs located at different time instances. For example, the mechanism is based on splitting the PRACH samples in frequency domain into two subsets, the two subsets being transmitted in different ROs in time domain. As shown below, UE1 and UE2 are transmitting the two subsets in two different portions of two different ROs, creating a sort of frequency hopping behavior within two TDMed ROs centred on the same frequency location.

- Clarification of “same Tx beam”

[NTT DOCOMO] Confirm that when multi-PRACH transmissions with the same beam are configured, a single spatial domain Tx filter is applied to all the multi-PRACH transmissions. Support to specify that the spatial domain filter applied for the multi-PRACH transmissions with the same beam is associated with the SSB/CSI-RS selected to determine PRACH resources.

- Draft Proposals

3.1 Multiple PRACH transmissions with same beam

3.1.1 Resource configuration for multiple PRACH transmissions

Proposal 1

For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions.

- FFS: details

| Companies | Comments |
| --- | --- |
| LG | Support. |
| Sony | Support |
| Panasonic | Support |
| ETRI | Support |
| Apple | Support |
| ZTE | Support |
| Qualcomm | Support |
| DOCOMO | Support |
| Ericsson | Support |
| TCL | Support |
| OPPO | Support |
| Spreadtrum | Support |
| Fujitsu | Support |
| Sharp | Support |
| Xiaomi | Support |

Proposal 2

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs.

- Note: Separate RO means that the RO is separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

| Companies | Comments |
| --- | --- |
| LG | Support. We can further discuss whether the separated RO will only be used for multiple PRACH transmissions. |
| Sony | Support Additional ROs can be configured to support multi PRACH transmissions. |
| Panasonic | Support |
| ETRI | Support |
| Apple | Support |
| ZTE | Separate preamble to differentiate the multiple PRACH transmissions with single PRACH transmission should not be precluded especially when RO resources are not enough for multiple PRACH transmissions. Similarly, combination of shared RO and separate RO is also helpful for saving the RO resources. |
| Qualcomm | Agree with ZTE. We prefer option 1 (shared ROs) and Option 3 (partially shared ROs). |
| DOCOMO | Support |
| Ericsson | - Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs. - Option 2: Multiple PRACH are transmitted on separate ROs. - Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. Regarding Option 1 and Option 2, we think both can be supported with Rel-17 framework, as the below the figure (b) and (d) show. RA resource efficiency of Option 2 separate ROs is worth considering, namely whether at a moment there are 64 UEs in a cell requesting multiple PRACH transmissions. Option 1 is better than Option 2 in this perspective. In addition, as can be seen from the figure (b) and (d) below, Option 2 separate RO is a subset configuration of Option 1 separate preambles in a shared RO, if all preambles in the additional RO are configured to the feature combination of multiple PRACHs. In a summary, we think separate preamble in a shared RO can be supported. Furthermore, regarding the FFS, it is in RAN2’s scope. We advise the following change. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning is up to RAN2. |
| TCL | Support |
| OPPO | Support |
| Spreadtrum | Support |
| Fujitsu | Support |
| Sharp | Support |
| Xiaomi | Support |

Proposal 3

Proposed working assumption

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

| Companies | Comments |
| --- | --- |
| LG | Agree, but we prefer to make it an agreement (not a working assumption) because it can be also realized using the Rel-17 feature combination. |
| Sony | Not support. If we support Proposal 2 we don’t need Proposal 3. |
| Panasonic | Support |
| ETRI | Support |
| Apple | Support |
| ZTE | Support. Regarding proposal 2 and 3, gNB can freely select the differentiation approach. |
| Qualcomm | Support |
| DOCOMO | We prefer Proposal 2 to Proposal 3. In our understanding, separate preamble on shared ROs may bring impact on legacy RO configuration (e.g. if addressing multi-PRACH latency when determining RO configuration parameter, it implies impact to legacy single PRACH transmission). If majority companies prefer to support both Proposal 2 and Proposal 3, we want to clarify that only one is selected for gNB configuration. |
| Ericsson | Support. This can be merged with Proposal 2. We have the same comment on the FFS as in Proposal 2. |
| TCL | Not support Proposal 2 is enough. |
| OPPO | Support. Alternatives of proposal 2 and 3 can be based on gNB configuration. |
| FL | @ all, thanks for the comments, the reason for discussing proposal 2 and 3 separately is that some companies prefer to deprioritize original Option 1, i.e., separate preamble on shared ROs. From FL’s perspective, the separately discussion may benefit for the progress. |
| Spreadtrum | Support. Another point to be considered is how to determine whether UE should use Proposal 2 or Proposal 3 to transmit multiple PRACHs. |
| Sharp | We share same view with DOCOMO. |
| Xiaomi | Share the same view with DOCOMO and slightly prefer Proposal 2. |

Proposal 4

For multiple PRACH transmissions with same Tx beam, to align gNB’s and UE’s understanding about the potential time and frequency position of the corresponding ROs utilized for PRACH repetitions, considering the following options:

- Option 1:

"RO group" is defined, and one RO group consists of valid TDMed RO(s) for a specific number of multiple PRACH.

For each RACH attempt, UE determines and selects one RO group for multiple PRACH transmissions.

Note 1: All ROs in one RO group is associated with the same SSB(s).

FFS: details.

- Option 2:

For each RACH attempt, multiple PRACH transmissions are transmitted in the starting RO configured by gNB and in the subsequent TDMed ROs that are associated with the same SSB.

FFS: details.

| Companies | Comments |
| --- | --- |
| LG | We prefer to support Option 2 with following change: “subsequent TDMed ROs” “subsequent TDMed valid ROs”. Regarding Option 1, it is necessary to further discuss how gNB indicates the time/frequency position of the RO groups. |
| Sony | Option 1 and Option 2 read the same, just that Option 1 introduces a new term “RO Group”. Functionally is there anything different between the two options? |
| Panasonic | We think that "RO group" needs to be defined from UE perspective or cell perspective, respectively. We define them as a RO UE group, or a RO cell group as follows in our contribution. - A RO UE group includes ROs used in one PRACH attempt for the multi-PRACH transmission, and - A RO cell group is determined based on clustering ROs located in close proximity to one another in time domain, or frequency domain, or in both time and frequency domains - The understanding of RO cell groups is common among UEs regardless of the number of N PRACH transmissions (or repetitions). - This allows the resource usage alignment among UEs. The above means that a UE can choose one RO UE group from M RO cell groups for one PRACH attempt for a multi-PRACH transmission. With above understanding, a “RO group” in Option 1 is proposed as the meaning of our usage of a RO UE group. We support Option 1 as the meaning to define it from a UE perspective. As described we see the need of “RO group” from the cell perspective for the efficient multiplexing among different repetition numbers of PRACHs when the resource are different for each repetition. |
| ETRI | Option 2 may share the start RO with PRACHs using different repetition factors, while Option 1 explicitly introduce RO group. We would like to ask whether different RO groups can have intersections of some ROs or disjoint. We think two options seem not very different for now. |
| ZTE | We agree to discuss this issue from the two options. |
| Qualcomm | We prefer Option 1. |
| Ericsson | If gNB supports TDMed PRACHs and FDMed PRACHs, it can configure some RO groups with TDMed ROs and other RO groups with FDMed ROs. In other words, an RO group is a group of ROs, which can be TDMed or FDMed. A UE selects an RO according to its preferred multiplexing method. We suggest removing “TDMed” in the proposal of RO group and also considering FDMed PRACH transmissions. |
| TCL | Both options are ok for us. |
| OPPO | RO group means that ‘a large RO’ including a specific number of valid TDMed RO(s) can be selected by UE for multiple PRACH transmission. A RO group consists of ROs in time and frequency domain. It is necessary for gNB to differentiate the multiple PRACH transmission with different number of repetitions. RO groups corresponding to different number of repetitions should be configured. We prefer option 1 to allow the differentiation of multiple PRACH transmission with different repetition number. For option 2, it is not clear for us how to realize the similar purpose with e.g. a shared starting RO configured by gNB for multiple PRACH transmission with different repetition number. |
| Spreadtrum | We prefer to support Option 1. Regarding Option 2, one point need to be clarified is whether 2 number of PRACH repetitions and 4 number of PRACH repetitions have the same starting point or not. If Proposal 1 is supported, gNB may be not able to perform joint detection since gNB is not sure the number of ROs are utilized by the UE for PRACH transmission. |
| Fujitsu | We prefer Option 1. |
| Sharp | We are OK with the FL proposal to discuss the issue. For RO group, not only how to apply valid/invalid but also whether/how RO mask is applied needs to be clarified. (esp, shared RO case with single PRACH) |
| Xiaomi | We prefer Option 1. We can’t see the necessary to introduce the concept of “RO group”. |

Proposal 5

For multiple PRACH transmissions with same Tx beam, the starting RB of ROs can be different at different time instances.

FFS: details.

| Companies | Comments |
| --- | --- |
| LG | Support. |
| Sony | Support. This can facilitates frequency hoping. |
| Panasonic | Support |
| Apple | If the intention of this proposal is to support frequency hopping, we support this proposal. |
| ZTE | From my point of view, this proposal is not related to FH, but it may happen under some mapping configurations. For example, if FDM=2 and number of actually SSBs is 3, it is possible that ROs mapping to SSB0 may occur in different RBs. |
| Qualcomm | Support (with intention to support frequency hopping) |
| DOCOMO | Support |
| TCL | Support |
| OPPO | Support. It is helpful for frequency hopping and configuration flexibility. |
| FL | @Apple, ZTE, Qualcomm, from FL’s understanding, on one hand, “RO hopping” is supported if this proposal is agreed; on the other hand, it can deal with the issue mentioned by ZTE. |
| Spreadtrum | Support. |
| Sharp | Support |
| Xiaomi | If FDMed ROs are supported for multiple PRACH transmissions, it means that different multiplexing patterns may be introduced for different number of PRACH transmissions, number of FDMed ROs as well as SSB-to-RO mapping, which should be FFS and the workload will be increased. Of course, we can support the proposal for progress. |

Proposal 6

Proposed conclusion

Multiple PRACH transmissions over FDM-ROs in the same time instance is not supported in Rel-18.

| Companies | Comments |
| --- | --- |
| LG | Agree. |
| Sony | Can you please clarify whether this means: - We cannot have repetitions in frequency domain. That is if we have 4 FDM-ed ROs, then we can perform 4x PRACH repetitions in a single time instance - We can only have one “RO Group” for PRACH repetition. That is, we cannot have 2 different set of “RO group” starting at different frequency. If this proposal refers to interpretation 1 above, then we support it. |
| Panasonic | Support |
| ETRI | Agree |
| Apple | Support |
| ZTE | Support |
| Qualcomm | Support |
| DOCOMO | Support |
| Ericsson | As observed in our simulation, 2 FDMed PRACH transmissions, each with half power and a Tx chain, has an ~1.5dB gain over a single PRACH transmission with full power and one Tx chain. This performance gain comes from ~1dB spatial diversity and ~0.5dB frequency diversity. This spatial diversity gain is applicable to all UEs with two Tx chains with FDM transmissions. In addition, a specification impact of simultaneous FDMed PRACH transmissions different from TDMed PRACH transmissions is the association of multiple FDMed ROs with a RACH attempt, which can be solved by supporting an RO group of FDMed ROs. |
| TCL | Support |
| OPPO | Support |
| FL | @Sony, I think it is your interpretation 1, e.g., “if we have 4 FDM-ed ROs, then we cannot perform 4 PRACH repetitions on the 4 FDM-ed ROs in a single time instance.” |
| Spreadtrum | Support. |
| Fujitsu | Support |
| CMCC | Do not support. As commented from E///, we share a similar view that FDMed PRACH transmission would offer gains for the PRACH coverage. We encourage companies to study and look into the details. |
| Sharp | Support |
| Xiaomi | Support |

Proposal 7

Proposed conclusion

Multiple PRACH transmissions with different preambles is not supported in Rel-18.

| Companies | Comments |
| --- | --- |
| LG | Agree. |
| Sony | Support |
| Panasonic | We think that the proposal 7 is not clear whether it targets to (i) a case of one PRACH attempt, or (ii) a case of both first PRACH attempt and re-attempt. We support the proposal 7 with case (i), therefore we suggest updating the proposal 7 as follows Proposed conclusion Multiple PRACH transmissions with different preambles is not supported in one PRACH attempt in Rel-18. |
| ETRI | Support |
| Apple | Support |
| ZTE | Not support. We propose to use different preambles to mitigate the same preamble collision. In case the starting ROs of multiple PRACH transmission from different UEs are aligned, the collision of preambles during multiple PRACH transmissions will not always be fixed to the same preamble and the collision rate will be reduced. In case the starting ROs of multiple PRACH transmission from different UEs are not aligned, the collisions of preambles during multiple PRACH transmissions will be dispersed to much more UEs, and the total interference will be reduced as the arriving time and energy from more UEs are more random. |
| Qualcomm | We think that preambles for We think that preambles for different PRACH transmissions (as part of PRACH repetition) should be clearly linked with some predefined rule. However, as ZTE mentioned, there may be some benefit in not precluding different (but linked) preambles. |
| DOCOMO | Support |
| TCL | Support |
| OPPO | Support |
| Spreadtrum | We share the same view with Panasonic and we support that multiple PRACH transmissions with different preambles is not supported in one PRACH attempt in Rel-18. |
| Fujitsu | Support |
| Sharp | Support |
| Xiaomi | Support |

About sending an LS to RAN2

Based on the contributions, Ericsson proposes to send an LS to RAN2, informing RAN2 of RAN1’s decision about how to differentiate the multiple PRACH transmissions with single PRACH transmission, and that it is up to RAN2 to decide how to configure RACH resources for Rel-18 multiple PRACH transmissions and possibly other Rel-18 features.

FL comment: Companies are welcome to provide your views about whether to send an LS to RAN2.

| Companies | Comments |
| --- | --- |
| LG | Agree in principle. It can be further discussed together when discussing Proposal 2 and/or 3. |
| Sony | Agree to let RAN2 decide on the configuration. |
| Panasonic | We are fine about sending an LS to RAN2 as Ericsson proposed. |
| ETRI | Support |
| Apple | We support to send the LS to RAN2. |
| ZTE | We can discuss whether we need a LS after the agreement or consensus of how to differentiate the multiple PRACH transmissions with single PRACH transmission as now we are not clear about is there any issue to be solved by RAN1 further. |
| Ericsson | Support |
| TCL | Support |
| OPPO | Support |
| Spreadtrum | We share the same view with ZTE that we should defer to decide whether to send an LS to RAN2 after reaching an agreement or consensus on the issue of distinguishing multiple PRACH transmissions with single PRACH transmission. |
| Xiaomi | Support |

3.1.2 RAR window and RA-RNTI calculation

Proposal 8

For multiple PRACH transmissions with same beam, only one RAR window is supported for RAR monitoring for all of the multiple PRACH transmissions.

  - FFS: the start position of the RAR window.

  - FFS: RA-RNTI.

| Companies | Comments |
| --- | --- |
| LG | Agree. Regarding FFS points, we prefer to support the last RO among multiple ROs configured for multiple PRACH transmissions for one RACH attempt can be used for both the starting position of RAR window and for the RA-RNTI calculation. |
| Sony | I think we can discuss this after we agree whether to support multiple levels of PRACH repetitions, i.e. after we agree on Proposal 1. |
| Panasonic | Support the proposal. Regarding FFS points, we support the following two schemes for RAR window and RA-RNTI calculation - Scheme 1: For enabling early termination of a multi-PRACH transmission, - A RAR window for PRACH transmissions can start after the first RO in the RO UE group, - A RA-RNTI candidate is calculated based on the indexes of time and frequency resources of the first RO in the RO UE group. - Scheme 2: For achieving a combined detection of a multi-PRACH transmission, - A RAR window for PRACH transmissions can start after the last RO in the RO UE group, - A RA-RNTI candidate is calculated based on the indexes of time and frequency resources of the last RO in the RO UE group. We think that one of two schemes (Schemes 1 and 2) can be configured for RAR window and RA-RNTI calculation in a semi-static manner depending on the purpose (early termination or a combined detection of a multi-PRACH transmission). Although we understand the majority view seems to support scheme 2 above, it requires large number of PRACH resources, and it can increase the latency when the number of PRACH resource is limited. Therefore, we also propose to support scheme 1 above. Scheme 1 can be also utilized for the multiple PRACH transmissions with different beam |
| ETRI | Support |
| Apple | OK with this proposal |
| ZTE | Fine with the proposal. It does not mean the same approach will be used for multiple PRACH transmissions with different beams |
| Qualcomm | Support |
| DOCOMO | Support the proposal. One clarification question, does this proposal imply that gNB identifying multiple PRACH transmissions is supported? |
| Ericsson | Support |
| TCL | Support |
| OPPO | Fine with the proposal. |
| Spreadtrum | Support. |
| Fujitsu | We support this proposal. Also, for the RAR window, we prefer the RAR window starts after “the last RO”. But we may need to define “the last RO” as it can be interpreted as two ways as below - Last RO within the configured RO - Last RO where the UE transmitted currently there is only maximum number of transmissions but no restriction how many times UE will transmit within the configured maximum number of transmissions (e.g., UE can transmit 3 times of PRACH transmissions within maximum 4 of transmissions). |
| Sharp | Support |
| Xiaomi | Support |

3.1.3 Power control

Proposal 9

For multiple PRACH transmissions with same Tx beam in one RACH attempt, transmission power ramping is not applied within one RACH attempt.

  - The same measurement of the same reference signal to calculate the pathloss is applied for each PRACH transmissions.

  - FFS: the initial power.

| Companies | Comments |
| --- | --- |
| LG | Agree. |
| Sony | Support |
| Panasonic | Support |
| ETRI | Support |
| Apple | Support |
| ZTE | Not support. We support power ramping in one RACH attempt as our intention is to enhanced the coverage. It is weird that the transmission power cannot achieved the highest level when multiple PRACH transmissions is applied. If we are the only one to support the ramping during one attempt, we can compromise if any approach can make sure the initial transmission power should be the maximum level, e.g., set an additional power offset for multiple PRACH transmissions can also be considered for PREAMBLE_RECEIVED_TARGET_POWER calculation. It needs more clarifications on “the same measurement”, does it mean the same measure RSRP result? Or in the other words, only once measurement during the attempt? |
| Qualcomm | Support |
| DOCOMO | Support |
| Ericsson | Support the first sentence. The same power ramping counter is applied to all PRACH transmissions of one RACH attempt. |
| TCL | Support |
| OPPO | Support. If we consider the number of PRACH repetition can be based on the measured RSRP, it seems that the number of PRACH repetition and transmission power are determined by the initial measured RSRP results, and do not change during the multiple PRACH transmission. |
| Spreadtrum | Support. |
| Fujitsu | Support |
| Sharp | Support |
| Xiaomi | Support |

3.2 Multiple PRACH transmissions with different beams

FL comments

Based on companies’ contributions, it can be seen that multiple PRACH transmission with different Tx beams has potential gains. Companies are welcome to share views about whether to support multiple PRACH transmission with different Tx beams over ROs associated with the same SSB/CSI-RS.

| Companies | Comments |
| --- | --- |
| Panasonic | We propose to prioritize the work on multi-PRACH transmission with the same beam, and then come back to discuss this case. We think that the design other than a combined detection of the multi-PRACH transmission can be used for multi-PRACH transmission with different beams. The choice of beam can be up to UE implementation. |
| ZTE | We support multiple PRACH transmission with different Tx beams over ROs associated with the same SSB/CSI-RS. |
| Qualcomm | We think the main benefit of transmission with different UL Tx beams is using it for choosing the best beam for Msg3 (and reception of Msg4). Therefore, If we use PRACH repetition with different beams, we should also specify a mechanism to indicate the best beam (for Msg3) to the UE. Also, in case of using different UL Tx beams for PRACH repetition, there should be a common understanding between UE and gNB about which PRACH transmissions are done using the same beam and which where can UE change the beam, i.e. RO bundles with the same beam (inside the RO group used for the whole PRACH repetition) should be clearly configured/defined. |
| DOCOMO | We are supportive of multiple PRACH transmissions with different Tx beams over ROs associated with the same SSB/CSI-RS. We think most specification impact design related with PRACH resource and RAR reception for multi-PRACH with same beam can be reused. Moreover, we agree with Qualcomm’s comment that indication of best UL beam for Msg 3 would be beneficial. |
| Ericsson | Our simulation shows PRACH transmissions with different beams (beam sweeping) outperforms the transmissions with the same wide beam by 1dB for the same number of transmissions. It should be noted that our simulation assumption of different Tx beams sweeping -pi~pi horizontally is based on an assumption that a UE has no any prior information of gNB direction. It is the worst case and provides the lower bound of beam sweeping's performance. However, in reality, if a UE has multiple panels, it is possibly able to select a panel with the best orientation before initiating random access. This is true even for a UE not supporting beam correspondence. Consider there are two antenna panels on the left and right size of a mobile phone, each generating Tx beams sweeping a 180-degree range, which is half of the angular range in our simulation assumption. After UE antenna panel selection, beam sweeping in a smaller angular range can reduce the unnecessary PRACH transmissions, which energy fail to reach gNB, and improve beam sweeping’s performance. In short, more than 1dB gain is expected for beam sweeping given UE has some prior knowledge of gNB direction. |
| OPPO | It is advised to deprioritize the discussion on the multiple PRACH transmission with different Tx beams. We can further discuss whether there are potential gains and specification impacts to support the it. |
| Spreadtrum | We support multiple PRACH transmission with different Tx beams over ROs associated with the same SSB/CSI-RS In addition, we should prioritize the work on multiple PRACH transmission with the same beam, which can be used as a baseline for different beams. |
| Sharp | We are OK to consider multiple PRACH transmission with different Tx beams but the case of same Tx beam should be prioritized. If different Tx beams case is supported. available ROs should be clarified. In our view, unlike same Tx beam case, gNB are not required to differentiate with single PRACH case and the preambles/ROs for different Tx beams can be shared with the preambles/ROs for legacy single PRACH transmission. |
| Xiaomi | We share the same view with OPPO. |

Proposal 10

Proposed conclusion

Multiple PRACH transmission with different Tx beams over ROs associated with different SSBs/CSI-RSs, where the different SSBs/CSI-RSs are not associated with the same RO, is not considered in Rel-18.

| Companies | Comments |
| --- | --- |
| LG | Agree. |
| Sony | Agree. |
| Panasonic | Support. |
| ETRI | Support |
| Qualcomm | Support |
| DOCOMO | OK with the proposal. |
| Ericsson | Support |
| TCL | Support |
| OPPO | Support |
| Spreadtrum | Support. |
| Sharp | Support |
| Xiaomi | Support |

- Proposals for online discussion (Mon.)

Proposal 1

For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions.

- If multiple values are configured, RACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported.

- FFS: details

Combined Proposal 2 & 3

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs.

- Note: Separate RO means that the RO is separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

At least support the main bullet: Huawei, HiSilicon, OPPO, CATT, China Telecom, Fujitsu, Lenovo, Sony, Mavenir, Intel, LG, InterDigital, Ericsson, Samsung, Apple, NTT DOCOMO, Sharp, Nokia, NSB, Spreadtrum(?), ETRI, ZTE (based on legacy PRACH configuration with additional new parameters), Panasonic

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Backup proposal

For multiple PRACH transmissions with same Tx beam, Option 3 (Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs) is not supported.

Proposal 5

For multiple PRACH transmissions with same Tx beam, the starting RB of ROs can be different at different time instances.

FFS: details.

Proposal 9

For multiple PRACH transmissions with same Tx beam in one RACH attempt, transmission power ramping is not applied within one RACH attempt.

  - The same measurement of the same reference signal to calculate the pathloss is applied for each PRACH transmissions.

  - FFS: the initial power.

Proposal 10

Proposed conclusion

Multiple PRACH transmission with different Tx beams over ROs associated with different SSBs/CSI-RSs, where the different SSBs/CSI-RSs are not associated with the same RO, is not considered in Rel-18.

- Agreements at RAN1#111

Agreement

For multiple PRACH transmissions with same Tx beam, support to differentiate at least between multiple PRACH transmissions and single PRACH transmissions.

Agreement

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, consider one or multiple of the following options.

- Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs.

- Option 2: Multiple PRACH are transmitted on separate ROs.

- Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs.

- Other options are not precluded.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

Agreement

Study at least the following case for multiple PRACH transmissions with different Tx beams.

- UE uses different TX beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS

- FFS: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs /CSI-RSs, where the different SSBs/CSI-RSs are not associated with the same RO.

- Note: not related to decision on CFRA

Note: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs/CSI-RSs, where the different SSBs/CSI-RSs are associated with the same RO is not considered.

Working Assumption

Simulation results for multiple PRACH transmissions with different beam(s) and same beam(s) (baseline) to be discussed in the next meeting.

- Simulation assumptions in TR 38.830 are used as the starting point for the simulation.

  - Focus on FR2.

    - UE antenna configuration 2-2-2(baseline), 1-4-1(optional)

  - Performance metric: 0.1% false alarm, 1% miss-detection

  - Companies report the number of beams, the beam widths, beam correspondence assumption, and the boresights.

- Channel model for link-level simulation: CDL-A defined in table 7.7.1-1 in TR 38.901.

- Both that UE fulfills beamCorrespondence requirements Without UL-BeamSweeping and UE fulfils beamCorrespondence requirements With UL-BeamSweeping can be considered in the simulation are used as starting point for simulation.

Agreement

For multiple PRACH transmissions with same Tx beam, down-select one option from the following options.

- Option 1: gNB can only configure one value for the number of multiple PRACH transmissions.

- Option 2: gNB can configure one or multiple values for the number of multiple PRACH transmissions.

  - FFS: details

Agreement

- For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt.

  - Note: whether to support multiple numbers of PRACH transmissions is separately discussed.

- Agreements at RAN1#110b-e

Agreement

- For multiple PRACH transmissions with same beam, at least support to use same PRACH preamble during the multiple PRACH transmissions in one RACH attempt.

  - FFS: whether different preambles can be utilized in different PRACH transmissions during the multiple PRACH transmissions in one RACH attempt.

Agreement

- For multiple PRACH transmissions with same beam, at least ROs located at different time instances can be utilized for the transmissions.

- FFS: whether/how the starting RB of ROs can be different at different time instances for multiple PRACH transmissions.

- FFS: whether/how multiple PRACH transmissions located in the same time instance, e.g., for UEs with multiple Tx chains.

Agreement

For multiple PRACH transmissions with same beam, for RAR monitoring, consider the following options.

- Option 1: One RAR window per each PRACH transmission, the RAR window follows the legacy design.

  - FFS: RA-RNTI.

- Option 2: Only one RAR window for all of the multiple PRACH transmissions.

  - FFS: the start position of the RAR window.

  - FFS: RA-RNTI.

- Reference

- 3GPP RP-213579, “New WI: Further NR coverage enhancements”, China Telecom, RAN#94e, December 6-17, 2021.

- 3GPP RP-221858, “Revised WID on Further NR coverage enhancements”, China Telecom, RAN #96, Budapest, Hungary, June 6-9, 2022.

- 3GPP RAN1 Chairman’s Notes, RAN1 meeting #111, Toulouse, France, November 14th – 18th, 2022.

- 3GPP RAN1 Chairman’s Notes, RAN1 meeting #110b-e, October 10th – 19th, 2022.

- R1-2300089	Discussion on PRACH coverage enhancements	Huawei, HiSilicon

- R1-2300244	Discussion on PRACH coverage enhancements	Spreadtrum Communications

- R1-2300276	PRACH coverage enhancements	OPPO

- R1-2300344	Discussion on PRACH coverage enhancements	ZTE

- R1-2300479	Discussions on PRACH coverage enhancements	vivo

- R1-2300495	Discussion on Resource Allocation for Multiple PRACH Transmission	FGI

- R1-2300561	Discussion on PRACH coverage enhancements	xiaomi

- R1-2300667	PRACH coverage enhancements	CATT

- R1-2300728	Discussion on PRACH coverage enhancement	China Telecom

- R1-2300758	Discussion on PRACH coverage enhancements	Fujitsu

- R1-2300796	Discussion on PRACH coverage enhancements	Panasonic

- R1-2300828	Discussion on PRACH coverage enhancement	NEC

- R1-2300838	PRACH coverage enhancements	TCL Communication Ltd.

- R1-2300860	PRACH coverage enhancements	Lenovo

- R1-2300894	PRACH Coverage Enhancement using Multi PRACH Transmissions	Sony

- R1-2300904	Discussion on PRACH coverage enhancement	Mavenir

- R1-2300972	Discussions on PRACH coverage enhancement	Intel Corporation

- R1-2301027	Discussion on PRACH coverage enhancements	CMCC

- R1-2301053	PRACH coverage enhancements	ETRI

- R1-2301074	Discussion on PRACH repeated transmission for NR coverage enhancement	LG Electronics

- R1-2301171	Discussion on PRACH coverage enhancements 	InterDigital Communications

- R1-2301185	Discussion on PRACH coverage enhancement	Ericsson

- R1-2301292	PRACH coverage enhancements	Samsung

- R1-2301374	Discussion on PRACH coverage enhancement	Apple

- R1-2301441	PRACH Coverage Enhancements	Qualcomm Incorporated

- R1-2301519	Discussion on PRACH coverage enhancements	NTT DOCOMO, INC.

- R1-2301550	Views on multiple PRACH transmission for coverage enhancement	Sharp

- R1-2301777	On PRACH coverage enhancements	MediaTek Inc.

- R1-2301654	PRACH coverage enhancements	Nokia, Nokia Shanghai Bell