3GPP TSG RAN WG1 #112                                                 R1- 2301970

Athens, Greece, February 27th – March 3rd, 2023

Source:          Moderator (ZTE)

Title:            Summary of positioning SRS power control corrections

Agenda item:     7.2

Document for:	Discussion and Decision

- Introduction

This document provides the moderator summary of the following contribution.

- R1-2300798 Draft CR on positioning SRS power control in 38.213 ZTE

- R1-2300799 Discussion on positioning SRS power control in 38.213 ZTE

- Discussion

| Company | Proposals |
| --- | --- |
| ZTE [R1-2300798] [R1-2300799] | Reason for change:; - In the current 38.213, there is no description for the case when serving cell SSB index is provided as pathloss RS for positioning SRS. But there is the descriptions for the other two cases. - The parameter ‘ssb-IndexNcell’ is not aligned with 38.331. - In Rel-16 for positioning SRS power control, if PRS is configured as the pathloss RS of the positioning SRS resource set by RRC parameter dl-PRS-r16 in 38.331, the power of PRS resource set can be known based on PRS resource set ID and TRP ID informed in dl-PRS-r16. - In 37.355, the power of PRS is configured in PRS resource set basis. And in 38.331, dl-PRS-ResourceId in dl-PRS-r16 is optionally provided. That is, even dl-PRS-ResourceId is not provided, UE can still get the PRS power which is pathloss RS of positioning RS. However, in the current 38.213, the condition to provide PRS power value is ‘a dl-PRS-ResourceId is provided’ which is not correct.. Summary of change:; - Add the pathloss description when serving cell SSB index is provided - Change the parameter name of ssb-IndexNcell according to 38.331 - Change ‘dl-PRS-ResourceId’ to ‘dl-PRS-r16’. - 7.3.1 UE behaviour < Unchanged parts are omitted > If a UE transmits SRS based on a configuration by SRS-PosResourceSet on active UL BWP b of carrier f of serving cell c, the UE determines the SRS transmission power PSRS,b,f,ci,qs in SRS transmission occasion i as [dBm] where, - PO_SRS,b,f,cqs and αSRS,b,f,cqs are provided by p0-r16 and alpha-r16 respectively, for active UL BWP b of carrier f of serving cell c, and SRS resource set qs is indicated by SRS-PosResourceSetId from SRS-PosResourceSet, and - PLb,f,cqd is a downlink pathloss estimate in dB calculated by the UE, as described in clause 7.1.1 in case of an active DL BWP of a serving cell c, using RS resource indexed qd in a serving or non-serving cell for SRS resource set qs [6, TS 38.214]. A configuration for RS resource index qd associated with SRS resource set qs is provided by pathlossReferenceRS-Pos - if a ssb-IndexServing-r16 is provided, referenceSignalPower is provided by ss-PBCH-BlockPower - if a ssb-Ncell-r16 is provided, referenceSignalPower is provided by ss-PBCH-BlockPower-r16 - if a dl-PRS-r16 is provided, referenceSignalPower is provided by dl-PRS-ResourcePower If the UE determines that the UE is not able to accurately measure PLb,f,cqd, or the UE is not provided with pathlossReferenceRS-Pos, the UE calculates PLb,f,cqd using a RS resource obtained from the SS/PBCH block of the serving cell that the UE uses to obtain MIB The UE may indicate a capability for a number of pathloss estimates that the UE can simultaneously maintain for all SRS resource sets provided by SRS-PosResourceSet in addition to the up to four pathloss estimates that the UE maintains per serving cell for PUSCH/PUCCH transmissions and for SRS transmissions configured by SRS-Resource. < Unchanged parts are omitted > |

Moderator comments

The first two changes are more like alignment CR. The third change is essential because UE gets PRS power value based on dl-PRS-r16 rather than dl-PRS-ResourceId. More details can be found in the discussion paper R1-2300799.

- Proposal 1-1

The changes in the draft CR R1-2300798 are agreed.

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT | Yes | The changes are needed to align the descriptions about pathlossReferenceRS for SRS-Pos in TS 38.331 as follows: pathlossReferenceRS-Pos-r16 CHOICE { ssb-IndexServing-r16 SSB-Index, ssb-Ncell-r16 SSB-InfoNcell-r16, dl-PRS-r16 DL-PRS-Info-r16 } OPTIONAL, -- Need M |
| OPPO | Yes | We are general fine with the modifications. One editorial suggestion: according to RAN2’s guidance, the suffix “-r16” of the RRC parameters should be deleted if there is no ambiguity |
| ZTE | Yes | Remove ‘-16’ as OPPO suggested. So, the following is suggested - if a ssb-IndexServing is provided, referenceSignalPower is provided by ss-PBCH-BlockPower - if a ssb-Ncell is provided, referenceSignalPower is provided by ss-PBCH-BlockPower-r16 - if a dl-PRS is provided, referenceSignalPower is provided by dl-PRS-ResourcePower |

- Conclusion

Based on the comments, moderator suggests the following change for 38.213 for both Rel-16 and Rel-17

- 7.3.1 UE behaviour < Unchanged parts are omitted > If a UE transmits SRS based on a configuration by SRS-PosResourceSet on active UL BWP b of carrier f of serving cell c, the UE determines the SRS transmission power PSRS,b,f,ci,qs in SRS transmission occasion i as [dBm] where, - PO_SRS,b,f,cqs and αSRS,b,f,cqs are provided by p0-r16 and alpha-r16 respectively, for active UL BWP b of carrier f of serving cell c, and SRS resource set qs is indicated by SRS-PosResourceSetId from SRS-PosResourceSet, and - PLb,f,cqd is a downlink pathloss estimate in dB calculated by the UE, as described in clause 7.1.1 in case of an active DL BWP of a serving cell c, using RS resource indexed qd in a serving or non-serving cell for SRS resource set qs [6, TS 38.214]. A configuration for RS resource index qd associated with SRS resource set qs is provided by pathlossReferenceRS-Pos - if a ssb-IndexServing is provided, referenceSignalPower is provided by ss-PBCH-BlockPower - if a ssb-Ncell is provided, referenceSignalPower is provided by ss-PBCH-BlockPower-r16 - if a dl-PRS is provided, referenceSignalPower is provided by dl-PRS-ResourcePower If the UE determines that the UE is not able to accurately measure PLb,f,cqd, or the UE is not provided with pathlossReferenceRS-Pos, the UE calculates PLb,f,cqd using a RS resource obtained from the SS/PBCH block of the serving cell that the UE uses to obtain MIB The UE may indicate a capability for a number of pathloss estimates that the UE can simultaneously maintain for all SRS resource sets provided by SRS-PosResourceSet in addition to the up to four pathloss estimates that the UE maintains per serving cell for PUSCH/PUCCH transmissions and for SRS transmissions configured by SRS-Resource. < Unchanged parts are omitted >