3GPP TSG-RAN WG1 Meeting #112	R1-2300136

Athens, Greece, February 27 – March 3, 2023

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	On RB set configuration for IAB

Document for:	Discussion and Decision

Introduction

In this paper, we discuss the issue raised in RAN3 LS [1], i.e. how to define the start RB index for the Rel-17 IAB-DU HSNA resource configuration of an IAB-DU cell.

RB set configuration for IAB DU

RAN1 has made the following agreements related to the start RB index for the Rel-17 IAB-DU HSNA resource configuration of an IAB-DU cell.

Agreement (RAN1#106-e)
A Reference SCS is configured for frequency domain H/S/NA configuration.

Agreement (RAN1#108-e)
The start RB index of the first RB set for the Rel-17 IAB-DU HSNA resource configuration is the lowest index of RB of the IAB-DU cell

Agreement (RAN1#106-e)
A Reference SCS is configured for frequency domain H/S/NA configuration.

Agreement (RAN1#108-e)
The start RB index of the first RB set for the Rel-17 IAB-DU HSNA resource configuration is the lowest index of RB of the IAB-DU cell

As pointed out in [1], there could be one or multiple numerologies in one NR cell. F1AP and XnAP allow defining a separate frequency offset for each numerology via the NR Carrier List, wherein each numerology is referred as “SCS-specific carrier”. RAN3 has provided two different understandings:

- Understanding 1: the start RB index for the Rel-17 IAB-DU HSNA resource configuration of the IAB-DU cell is point A plus the SCS-specific offset defined in the NR Carrier List IE based on the SCS included in the RB Set Configuration IE.

- Understanding 2: the start RB index for the Rel-17 IAB-DU HSNA resource configuration of the IAB-DU cell is point A plus the smallest SCS-specific frequency offset across all numerologies provided by the NR Carrier List IE for this cell. Note that in this understanding, the start RB index does not depend on the SCS value included in the RB Set Configuration IE in TS 38.473.

In both understandings, the start RB is aligned with point A in case the NR Carrier List IE is not present.

It can be seen that one value is applied for all SCS values for an IAB-DU cell in both understandings. As shown in Figure 1, understanding 1 may result in undefined H/S/NA resources for a given SCS configurated by NR Carrier List since the start RB index for the Rel-17 IAB-DU HSNA resource configuration of the IAB-DU cell is point A plus the SCS-specific offset defined in the NR Carrier List IE based on the SCS included in the RB Set Configuration IE. There is no such issue for understanding 2. However, understanding 2 may result in H/S/NA resources that will never be used for a given SCS. Therefore, another possible way is to define the start RB index as the first RB for each SCS provided in NR Carrier List IE.

Observation: Understanding 1 may result in undefined H/S/NA resources for a given SCS configurated by NR Carrier List. There is no such issue for understanding 2. However, understanding 2 may result in H/S/NA resources that will never be used for a given SCS.

(a)

(b)

Figure 1: (a) Understanding 1 results in undefined H/S/NA resource for IAB-DU, (b) Understanding 2 results in H/S/NA resources that will never be used

Therefore, another possible way is to define the start RB index as the first RB for each SCS provided in NR Carrier List IE as shown in Figure 2.

Figure 2: The start RB index is defined as the first RB for each SCS provided in NR Carrier List IE

Proposal: Define the start RB index as the first RB for each SCS provided in NR Carrier List IE.

One draft reply LS is provided in [2].

Summary and conclusion

In this contribution, we discussed the issue in RAN3 LS on on RB set configuration for IAB and have the following observation

Observation: Understanding 1 may result in undefined H/S/NA resources for a given SCS configurated by NR Carrier List. There is no such issue for understanding 2. However, understanding 2 may result in H/S/NA resources that will never be used for a given SCS.

Proposal: Define the start RB index as the first RB for each SCS provided in NR Carrier List IE.

References

- R1-2300006, “LS on RB set configuration”, RAN3

- R1-2301756, “Draft reply LS on RB set configuration for IAB”, Huawei, HiSilicon