3GPP TSG RAN WG1 #112   	                           R1-2300530

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.2.2.1

Source:	LG Electronics

Title:	Evaluation on AI/ML for CSI feedback enhancement

Document for:	   Discussion and Decision

- Introduction

In the RAN1#111 meeting, several agreements related to evaluation methodology and assumptions were made. Also, working assumption on CSI prediction This contribution discusses on evaluation methodology for AI/ML based CSI feedback enhancement.

- Discussions on evaluation methodology

Agreement@RAN1#110bis

For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, at least Method 3 is adopted, FFS whether additionally adopt a down-selected metric between Method 1 and Method 2.

- Method 1: Average over all layers

- Method 2: Weighted average over all layers

SGCS=E1Ni=1NjKλijk=1Kλik||wijHwij ||||wij||  ||wij||2

where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij.

- Method 3: SGCS is separately calculated for each layer (e.g., for K layers, K SGCS values are derived respectively, and comparison is performed per layer)

Intermediate KPI

Currently, SGCS and NMSE is agreed as intermediate KPIs for AI/ML based CSI compression. Other KPIs including relative achievable rate (RAR) or numerical spectral efficiency were listed as candidate intermediate KPIs during the discussion. For these metrics, it may have a merit that it can be helpful to see whole MIMO layer impacts and there is no layer disorder issue. In our view, if we want to see the impact of whole MIMO layers Method 1 and Method 2 can alternatively considered. Also, it is recommended to ensure to avoid layer disorder issue, so another intermediate KPI seems not necessary.

For additional support of SGCS between Method 1 and 2, we think either method can reflect whole MIMO layer impacts. Specifically, method 2 can be more appropriate when the cost function is weighted MSE or weighted sum rate as it considers weighted sum of layers. Thus, if needed, either Method 1 or 2 can be supported optionally.

Proposal 1. Intermediate KPI other than SGCS and NMSE is not necessary.

Proposal 2. For generating SGCS when rank>1, Method 1 or method 2 can be optionally reported by company.

- UPT upper bound

In the last meeting, there was a discussion on whether provision of an additional UPT upper bound assuming ideal CSI is needed. For AI/ML based CSI compression, Type II CSI is chosen as a baseline and Type I CSI which may be a lower bound can be optional. Although the most of gain of AI/ML based CSI compression comes from the overhead reduction, it will be helpful to have an UPT upper bound in order to see how AI/ML can go far. Therefore, UPT upper bound assuming ideal CSI can be optional and can be reported by company.

Proposal 3. UPT upper bound assuming ideal CSI can be optionally reported by company.

- CSI Payload size

For two-sided AI/ML based CSI compression, one of KPI is payload. The issue is how to calculate the payload for AI/ML based CSI compression and Type II codebook. For this issue, two options are listed.

- Option 1: Payload size is calculated based on the maximum rank.

- Option 2: Payload size is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE. FFS the following understandings

  - Option 2a: The above-mentioned “CSI payload” is calculated as each CSI reported payload with a given rank

  - Option 2b: The above-mentioned “CSI payload” is calculated as max allowed bits at the given rank

Option 1 is simple and widely used in MIMO CSI evaluation. Option 2 is more accurate payload calculation compared to option 1. However, it is more complicated and seems hard to visualize the payload vs. throughput curve as UEs dropped in the evaluation experiences different ranks. Also, between option 2a and 2b, option 2b is more realistic than option 2a. Based on above discussion, in our view, option 1 seems sufficient.

Proposal 4. Payload size is calculated based on the maximum rank.

- AI/ML model setting for rank >1

For AI/ML model setting for rank>1, following four candidates were listed during the discussion.

- Option 1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference.

- Option 2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference.

- Option 3 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference.

- Option 4 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference.

Among 4 options, option 4 can be considered as a baseline as it is simple and straightforward. For option 2, in our view, it is unclear what is difference compared to option 4. Option 1 and 3 may provide better AI/ML performance as those approaches are kind of optimization at the expense of increased complexity. Therefore, option 4 can be considered as a baseline.

Proposal 5. For AI/ML model setting for rank>1, option 4 (layer common) can be a baseline.

- Temporal domain CSI prediction

In the last meeting, it was agreed that UE-sided AI/ML based CSI prediction is selected as sub-use case for CSI feedback. For this CSI prediction, following evaluation methodology can be further considered.

One is whether or not to consider spatial consistency. Spatial consistency is the procedure of channel generation where the cluster-specific and ray-specific random variables are generated to be spatially consistency. CSI predication is useful when there is some level of mobility at UE. To have realistic channel assumption on this scenario, spatial consistency is needed. Otherwise, generated channels during UE movement are not well correlated, so prediction performance may be degraded. In this regard, BM case-2 adopted spatial consistency procedure and companies are required to report one of spatial consistency procedures, Procedure A and/or B in TR 38.901. Therefore, for UE-sided AI/ML based CSI prediction, spatial consistency can be considered and companies can report one of spatial consistency procedures used for their simulation.

Proposal 6. For UE-sided AI/ML based CSI prediction, consider spatial consistency and companies can report one of spatial consistency procedures, Procedure A and/or B in TR 38.901.

Another one is model generalization. There were several agreements on model generalization performance evaluation in CSI feedback. As mentioned above, CSI prediction is useful when UE has some level of mobility, and UE speed will impact on the prediction performance. Also, in real world, every UE has different level of mobility and it would be very hard to have UE-specific AI/ML model. In this regard, it would be good to study on generalization performance according to various UE speed. Hence, various UE speed can be considered for model generalization performance of CSI prediction.

Proposal 7. Consider various UE speed for model generalization performance of CSI prediction.

- Conclusion

This contribution discussed on evaluation methodology for AI/ML based CSI feedback enhancement. Based on the above discussion, following proposals are proposed.

Proposal 1. Intermediate KPI other than SGCS and NMSE is not necessary.

Proposal 2. For generating SGCS when rank>1, Method 1 or method 2 can be optionally reported by company.

Proposal 3. UPT upper bound assuming ideal CSI can be optionally reported by company.

Proposal 4. Payload size is calculated based on the maximum rank.

Proposal 5. For AI/ML model setting for rank>1, option 4 (layer common) can be a baseline.

Proposal 6. For UE-sided AI/ML based CSI prediction, consider spatial consistency and companies can report one of spatial consistency procedures, Procedure A and/or B in TR 38.901.

Proposal 7. Consider various UE speed for model generalization performance of CSI prediction.