3GPP TSG RAN WG1 #112                                       	       R1-2300545

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.1.1.1

Source:	Xiaomi

Title:                     Unified TCI framework extension for multi-TRP

Document for:	Discussion and Decision

Introduction

The WI for R18 MIMO was approved in RAN-94 e-meeting, and the detailed objectives [1] are as follows:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows:

  - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis

  - UE reporting of time-domain channel properties measured via CSI-RS for tracking

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

- Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM,

  - Striving for a common design between DL and UL DMRS

  - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

    - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

  - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

- Study, and if justified, specify the following

  - Two TAs for UL multi-DCI for multi-TRP operation

  - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

While in this contribution, we will focus on the unified TCI framework extension for M-TRP, which including the object 2, the second bullet of objective 6 and the first bullet of objective 7.

Indication of multiple DL/UL TCI states

CC list configuration for simultaneous TCI state activation

In Rel-16, there are two MAC CEs for PDSCH TCI state activation [38.321]. The first one is TCI state activation with CORESETPoolindex and it can be used for S-TRP CC(s) or M-DCI based M-TRP CC. But the S-TRP CC(s) can’t be configured in a same cc list as M-DCI based M-TRP CC. Since “If the Serving Cell in the MAC CE is configured in a cell list that contains more than one Serving Cell, the CORESET Pool ID field shall be ignored when receiving the MAC CE.” That means M-DCI based M-TRP CC can’t be configured in any CC list. And the second one is enhanced TCI state activation with two activated TCI state for one TCI codepoint, which can be used for S-DCI based M-TRP CC(s).

So for unified TCI state activation, it is straightforward and simple to support separate TCI state activation for S-TRP CC, S-DCI based M-TRP CC and M-DCI based M-TRP CC respectively. If a CC list comprised of a mix of STRP CC(s) and MTRP CC(s) will be supported, it need to be discussed M-TRP CC(s) is S-DCI based or M-DCI based or both. While for S-DCI based MTRP CC and M-DCI based MTRP CC, it can be configured in one CC list for simultaneous TCI state activation. But it need some more spec effort to define a common MAC CE and additional indication or default rule is necessary to indicate the mapping between activated TCI states and S-DCI MTRP CC or the mapping between activated TCI states and M-DCI MTRP CC.

Observation 2-1: Additional indication or default rule to indicate the mapping between activated TCI states and S-DCI based MTRP CC(s) or M-DCI based MTRP CC(s) is needed to support a CC list comprised of a mix of S-DCI based MTRP CC(s) and M-DCI based MTRP CC(s).

Unified TCI state indication in S-DCI case

For TCI state indication for S-DCI based M-TRP, the agreement in RAN1-111 e-meeting [5].

Agreement

On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list

- FFS: Increase on the size of the TCI field

- Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS

TCI state mapping/association for PDCCH in S-DCI case

For PDCCH reception, the agreement in RAN1-110bis e-meeting [4].

Agreement

On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following:

- Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported)

For PDCCH-SFN, it is possible that a CORESET will be configured to apply two indicated joint/DL TCI states. For PDCCH reception of such CORESET, two TCI states will be applied when the number of indicated joint/DL TCI states is 2. But if the number of indicated joint/DL TCI states is 1, it is necessary to define the UE behavior on the PDCCH reception of such CORESET.

There are some alternatives on UE behavior when the number of indicated joint/DL TCI states is 1:

- Alt 1: UE stops monitoring the PDCCH candidates of CORESET configured to apply two indicated joint/DL TCI states.

- Alt 2: UE monitors the PDCCH candidates of CORESET configured to apply two indicated joint/DL TCI states by the one indicated joint/DL TCI state.

- Alt 3: UE monitors the PDCCH candidates of the CORESET configured to apply two indicated joint/DL TCI states by the one indicated joint/DL TCI state and the non-changed indicated joint/DL TCI state.

Proposal 2-1: For PDCCH reception with PDCCH SFN, suggest to down-select one from the following alternatives on the behavior if the UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP.

- Alt 1: UE stops monitoring the PDCCH candidates of CORESET configured to apply two indicated joint/DL TCI states.

- Alt 2: UE monitors the PDCCH candidates of CORESET configured to apply two indicated joint/DL TCI states by the one indicated joint/DL TCI state.

- Alt 3: UE monitors the PDCCH candidates of the CORESET configured to apply two indicated joint/DL TCI states by the one indicated joint/DL TCI state of the first TRP and the non-changed indicated joint/DL TCI state of the second TRP.

For PDCCH-repetition, two linked search space set will be configured to transmit same DCI information. While for the corresponding CORESET of each search space, one indicated joint/DL TCI states will be configured to apply. When the number of indicated joint/DL TCI states is 2, the PDCCH reception for PDCCH-repetition will be based on two indicated joint/DL TCI states, i.e., each PDCCH candidate will be monitored based on the corresponding TCI state. And in Rel-17, it was agreed that individual PDCCH candidate is not allowed in two linked search space set.  So when the number of indicated joint/DL TCI states is 1, it is much more likely to stop monitoring the PDCCH candidates of two linked search space set. It is also possible to monitor the PDCCH candidates of only one search space set corresponding to the one indicated joint/DL TCI state. Or to monitor the PDCCH candidates of the first search space set with the one indicated joint/DL TCI state and monitor the PDCCH candidates of the second search space set with the non-changed indicated joint/DL TCI state.

Proposal 2-2: For PDCCH reception with PDCCH repetition, suggest to down-select one from the following alternatives on the behavior if the UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP.

- Alt 1: UE stops monitoring the PDCCH candidates of two linked search space set.

- Alt 2: UE monitors the PDCCH candidates of only one search space set corresponding to the one indicated joint/DL TCI state of the one TRP.

- Alt 3: UE monitors the PDCCH candidates of the first search space set by the one indicated joint/DL TCI state of the first TRP and monitors the PDCCH candidates of the second search space set by the non-updated indicated joint/DL TCI state of the second TRP.

TCI state mapping/association for PDSCH in S-DCI case

For PDSCH reception, the agreements were archived in RAN1-111 meeting.

Agreement

On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception

- The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception

  - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception

- FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field)

- FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold

FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0.

Above applies for the case where PDSCHs scheduled by the same DCI.

Agreement

On unified TCI framework extension for S-DCI based MTRP, use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2

Agreement

On unified TCI framework extension for S-DCI based MTRP, a new indicator field is supported as the DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception

- FFS: Detail design of the new indicator field

Default TCI state for PDSCH in S-DCI case

As for the default indicated joint/DL TCI states to PDSCH reception when DCI field is not present in DCI format 1_1/1_2 or when the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold, or when after receiving the first MAC CE/DCI for the indicated TCI states and before the application time of the first DCI for informing the TCI association, we prefer a fixed rule than RRC configured to determine the default TCI state.

Proposal 2-3: Support to define default TCI state(s) applied to the PDSCH reception for following cases:

- When the TCI association filed is not present in DCI format 1_1/1_2 (if the presence of the TCI association is configurable by RRC).

- When the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold.

- After receiving the first MAC CE/DCI for the indicated TCI states and before the application time of the first DCI for informing the TCI association.

As for what is the default TCI state(s), there were some alternatives been discussed in last meeting.

- Alt1: Use RRC to configure that the UE shall apply which one or both of the indicated joint/DL TCI states

- Alt2: The UE applies the first indicated joint/DL TCI state

- Alt3: The UE applies both indicated joint/DL TCI state

- Alt4: The UE applies the same indicated joint/DL TCI state(s) configured to the CORESET used for PDCCH reception if the CORESET follows the indicated joint/DL TCI state(s)

- Alt.5: The UE continuously applies the applicable joint/DL TCI state(s) for PDSCH after receiving DCI without TCI field

- Alt 6: UE does not expect the DCI field is absent after the BAT when joint/DL/UL TCI states for both of the two TRPs are indicated in a CC/BWP or a set of CCs/BWPs in a CC list. How to ensure the restriction is FFS.

While for different cases, it is better to use different default TCI state. For example, if the PDSCH is configured with fdmschemeA/ fdmschemeB/ tdmschemeA/ two CDM groups, the default TCI state can be both indicated joint/DL TCI states directly. If the PDSCH is configured by the higher layer parameter repetitionNumber in PDSCH-TimeDomainResourceAllocation, the default TCI state can be the first indicated joint/DL TCI state or   both indicated joint/DL TCI states based on enabletwodefaultTCIstates configured by RRC.

Proposal 2-4: For PDSCH configured with fdmschemeA/ fdmschemeB/ tdmschemeA/ two CDM groups/ repetitionNumber, the default TCI state for PDSCH reception can be both indicated joint/DL TCI states directly or based on enabletwodefaultTCIstates configured by RRC.

For PDSCH configured with SFN, it should depend on UE capability of supporting dynamic SFN or not. If UE doesn’t support dynamic SFN, the default TCI state can be both indicated joint/DL TCI states directly. If UE supports dynamic SFN, the default TCI state can be both indicated joint/DL TCI states directly or based on enabletwodefaultTCIstates configured by RRC. While for STRP PDSCH, the default TCI state can be the first indicated joint/DL TCI states directly.

Proposal 2-5: For PDSCH configured with SFN scheme, the default TCI state for PDSCH reception will be both indicated joint/DL TCI states directly if UE doesn’t support dynamic SFN. Else，it can be both indicated joint/DL TCI states based on enabletwodefaultTCIstates configured by RRC.

TCI state association/mapping for PDSCH in S-DCI case with SFN scheme

In addition, for PDSCH configured with SFN scheme, if UE doesn’t support dynamic switching between SFN and S-TRP, UE is not expected to receive a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states indicating one indicated joint/DL TCI state.

Proposal 2-6: For PDSCH configured with SFN scheme, UE is not expected to receive a DCI field in DCI format 1_1/1_2 indicating that PDSCH is associated with only one indicated joint/DL TCI state if UE doesn’t support dynamic switching between SFN and S-TRP.

Also if PDCCH configured with SFN scheme B, or PDCCH configured with SFN scheme A and UE doesn’t support sfn-SchemeA-PDCCH-only， UE is not expected to receive a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states indicating one indicated joint/DL TCI state.

Proposal 2-7: For PDCCH configured with SFN scheme B, or PDCCH configured with SFN scheme A and UE doesn’t support sfn-SchemeA-PDCCH-only, UE is not expected to receive a DCI field in DCI format 1_1/1_2 indicating that PDSCH is associated with only one indicated joint/DL TCI state.

TCI state determination for PDSCH when beam indication DCI indicating joint/DL/UL TCI state(s) for one TRP

If the UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP, it is also necessary to decide how to determine the TCI state for PDSCH. Alt 1 is that UE is not expected to receive a DCI field in DCI format 1_1/1_2 indicating that PDSCH is associated with two indicated joint/DL TCI state in this case.  Or it is allowed to indicate that PDSCH is associated with two indicated joint/DL TCI state in this case, but Alt 2 will be applied. Alt 2 is to apply one new indicated joint/DL TCI state and the other one indicated joint/DL TCI state will be not updated.

Proposal 2-8: If UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP, there are two Alternatives for PDSCH.

- Alt 1: UE is not expected to receive a DCI for the TCI association to associate PDSCH with two TCI states

- Alt 2: UE applies one new indicated joint/DL TCI state and the other one indicated joint/DL TCI state will be not updated for PDSCH if it receives a DCI for the TCI association to associate PDSCH with two TCI states

TCI state determination for PDSCH scheduled/activated by DCI format 1_0

How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0, there are following alternatives.

- Alt 1: same as the CORESET of the scheduling PDCCH

- Alt 2: based on default rule, e.g., use the first TCI state of the indicated joint/DL TCI state(s) or both

- Alt 3: Use RRC/MAC CE to inform the mapping/association

- Alt 4: same as the latest PDSCH scheduled/activated by DCI format 1_1/1_2

Proposal 2-9: On unified TCI framework extension for S-DCI based MTRP, for PDSCH transmission scheduled/activated by a DCI format 1_0, down-selection one alternative from the followings

- Alt 1: Same as that of the CORESET with the scheduling PDCCH.

- Alt 2: Based on default rule, e.g., use the first TCI state of the indicated joint/DL TCI state(s) or both.

- Alt 3: Use RRC/MAC CE to inform the mapping/association.

- Alt 4: Based on the mapping/association informed by the latest DCI format 1_1/1_2.

TCI state mapping/association for PUCCH in S-DCI case

According to the agreement in RAN1-111 meeting [5], RRC configuration is agreed to inform the association between indicated joint/UL TCI states and PUCCH resource/group.

Agreement

On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group

- Note: Detail of the RRC configuration is left to RAN2 design

In addition, if UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP, for PUCCH repetition, it is much likely to transmit the PUCCH with the one indicated TCI state. It also possible to apply one new indicated joint/UL TCI state and keep the other one not updated.

Proposal 2-10: Support to transmit PUCCH associated with two TCI states by the one indicated TCI state if it receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP, or apply one new indicated joint/UL TCI state and keep the other one not updated.

TCI state mapping/association for PUSCH in S-DCI case

For Rel-17 DG M-TRP PUSCH repetition scheme, dynamic switching between S-TRP and M-TRP is supported based on SRS resource set indicator in DCI format 0_1/0_2. While for M-TRP unified TCI framework, SRS resource set indicator in DCI can be reused to inform the UE which indicated joint/UL TCI state(s) should apply to PUSCH by the scheduling DCI.

According to the agreement in RAN1-111 meeting, an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 will be used to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2.

Agreement

On unified TCI framework extension for S-DCI based MTRP, use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2

As for the default TCI state for PUSCH scheduled/activated by a DCI format 0_1/0_2, a fixed rule can also be used to define a default TCI state, e.g., the first indicated TCI state. And the default TCI state can be used when after receiving the first MAC CE/DCI for the indicated TCI states and before the application time of the first DCI for informing the TCI association.

Proposal 2-11: Support to define a default TCI state by a fixed rule, and the default TCI state can be applied to the PUSCH transmission scheduled/activated by DCI format 0_1/0_2 after receiving the first MAC CE/DCI for the indicated TCI states and before the application time of the first DCI for informing the TCI association.

If the UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP, it is also necessary to decide how to determine the TCI state for PUSCH. Alt 1 is that UE is not expected to receive a DCI field in DCI format 0_1/0_2 indicating that PUSCH is associated with two indicated joint/UL TCI state in this case.  Or it is allowed to indicate that PUSCH is associated with two indicated joint/UL TCI state in this case, but Alt 2 will be applied. Alt 2 is to apply one new indicated joint/UL TCI state and the other one indicated joint/UL TCI state will be not updated.

Proposal 2-12: If UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP, there are two Alternatives for PUSCH.

- Alt 1: UE is not expected to receive a DCI for the TCI association to associate PUSCH with two TCI states

- Alt 2: UE applies one new indicated joint/UL TCI state and the other one indicated joint/UL TCI state will be not updated for PUSCH if it receives a DCI for the TCI association to associate PUSCH with two TCI states

For Type 1 CG M-TRP PUSCH repetition scheme, for retransmission, similar mechanism for DG PUSCH repetition scheme can be used. Otherwise, RRC configuration is preferred to inform the mapping/association between a configured or indicated joint/UL TCI state and SRS resource set. In addition, the SRS resource set is determined by the number of field for ‘pathlossReferenceIndex’, 'srs-ResourceIndicator', 'precodingAndNumberOfLayers', 'p0-PUSCH-Alpha' and 'powerControlLoopToUse'. If only one field for above indicator, the first indicated joint/UL TCI state will be applied. Else, both two indicated joint/UL TCI state will be applied.

Proposal 2-13: On unified TCI framework extension for S-DCI based MTRP, for Type 1 CG PUSCH repetition, support to use RRC configuration or default rule to inform the mapping/association.

In addition, if UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP, for PUSCH repetition of CG type 1 PUSCH, it is much likely to transmit the PUSCH with the one indicated TCI state.  But it is also possible to transmit it with the one indicated TCI state and keep the other one not changed.

Proposal 2-14: Support to transmit CG type 1 PUSCH associated with two TCI states by the one indicated TCI state, or by the one indicated TCI state and keep the other one not changed, if it receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP.

For PUSCH transmission scheduled/activated by a DCI format 0_0, there are some alternatives to inform the mapping/association between indicated joint/UL TCI state(s) and PUSCH transmission scheduled/activated by the DCI format 0_0.

- Alt 1: The TCI state of PUSCH transmission scheduled/activated by the DCI format 0_0 is same as that of the CORESET with the scheduling PDCCH.

- Alt 2: Based on default rule that use the first TCI state of the indicated joint/UL TCI state(s).

- Alt 3: Use RRC/MAC CE to inform the mapping/association between indicated joint/UL TCI state(s) and PUSCH transmission scheduled/activated by the DCI format 0_0.

- Alt 4: Based on the mapping/association informed by the latest DCI format 0_1/0_2.

Proposal 2-15: On unified TCI framework extension for S-DCI based MTRP, for PUSCH transmission scheduled/activated by a DCI format 0_0, down-selection one alternative from the followings

- Alt 1: Same as that of the CORESET with the scheduling PDCCH.

- Alt 2: Based on default rule, e.g., use the first TCI state of the indicated joint/UL TCI state(s).

- Alt 3: Use RRC/MAC CE to inform the mapping/association.

- Alt 4: Based on the mapping/association informed by the latest DCI format 0_1/0_2.

TCI state mapping/association for AP CSI-RS for CSI/BM in S-DCI case

In R17 with STRP unified TCI state, the AP CSI-RS for CSI/BM can be configured with followunifiedTCIstate. If configured, it will apply the indicated TCI state for AP CSI-RS. Else, legacy R15/16 signalling will be used for TCI state configuration for AP CSI-RS.

While for R18 with MTRP unified TCI state, for AP CSI-RS for CSI/BM, RRC configuration with followunifiedTCIstate can be reused and enhanced. It means that RRC can configure which unified TCI state will be followed.

Proposal 2-16: Use RRC to configure the association between indicated TCI states and AP CSI-RS for CSI/BM.

As for the default TCI state for AP CSI-RS, it also can reuse the legacy mechanism, apply the TCI state of CORESET or PDSCH with overlapping symbols. If there are two TCI states for CORESET for PDSCH, apply the first TCI state for AP CSI-RS.

Proposal 2-17: For default TCI state for AP CSI-RS for CSI/BM, to apply the first indicated TCI state of CORESET when with two TCI states.

TCI state mapping/association for SRS in S-DCI case

In R17 with STRP unified TCI state, the SRS can be configured with followunifiedTCIstate. If configured, it will apply the indicated TCI state for SRS. Else, legacy R15/16 signalling will be used for spatialrelationinfo configuration for SRS.

While for R18 with MTRP unified TCI state, for SRS for CSI/BM, RRC configuration with followunifiedTCIstate can be reused and enhanced. It means that RRC can configure which unified TCI state will be followed.

Proposal 2-18: Use RRC to configure the association between indicated TCI states and SRS.

As for the default TCI state for SRS for CB/NCB, if there are two SRS resource set, the first indicated TCI state can be applied to the first SRS resource set and the second indicated TCI state can be applied to the second SRS resource set. If there is only one SRS resource set, the first indicated TCI state can be applied.

Proposal 2-19: For default TCI state for SRS for CB/NCB, 1 to 1 mapping can be used in the case of two SRS resource sets. And the first indicated TCI state can be applied in the case of only one SRS resource set.

Application time for TCI state mapping/association information for PDSCH/PUSCH

For activated TCI state by MAC CE, the application time is the first slot that is after slot n+3Nslotsubframe,µ , where n is the slot for a PUCCH with HARQ-ACK information corresponding to the activation command. For TCI state of PDSCH in Rel-15/16, the application time is timeDurationForQCL after the scheduling DCI. For indicated joint/DL/UL TCI state in Rel-17, the application time is BeamAppTime_r17 after the last symbol of a PUCCH or PUSCH with HARQ-ACK information to the DCI carrying the TCI State indication and without DL assignment, or corresponding to the PDSCH scheduled by the DCI carrying the TCI State indication.

Based on the above analysis, it is to use DCI to inform the mapping/association between an indicated joint/DL/UL TCI state and PDSCH/PUSCH or to inform which indicated joint/DL/UL TCI state should apply to PDSCH/PUSCH. Thus it is also necessary to define the application time for the TCI state mapping/association information. If it is in the same DCI with the indicated joint/DL/UL TCI state, the application time can be same as the indicated joint/DL/UL TCI state. Else, independent application time should be defined. For PDSCH/PUSCH, we suggest the application time can be timeDurationForQCL after the scheduling DCI.

Proposal 2-20: The application time for TCI state mapping/association information for PDSCH/PUSCH should be defined.

Unified TCI state indication in M-DCI case

In RAN1-110b e-meeting [4], the agreement for M-DCI based MTRP can be seen as below.

Agreement

On unified TCI framework extension for M-DCI based MTRP:

- The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET that is associated with the same coresetPoolIndex value

- The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDSCH scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value

- FFS: Other channel(s)/signal(s) that has explicit or implicit association with a coresetPoolIndex value

- FFS: Other channel(s)/signal(s) that doesn’t have association with a coresetPoolIndex value

Above are applicable to the CORESET(s) that is configured/allowed to follow the indicated joint/DL TCI state

FFS: The configuration/rule to configure/allow CORESET(s) to follow the indicated joint/DL TCI state, including the option to reuse the same configuration/rule as in Rel-17 unified TCI framework

In RAN1-111 meeting [5], the agreement for PUSCH  in M-DCI based MTRP can be seen as below

Agreement

On unified TCI framework extension for M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH transmission scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value

While for CG type I PUSCH, since there is no association between the PUSCH and the CORESETPooIndex, UE will not know which indicated TCI state will be applied. Thus it is straightforward to configure a CORESETPoolIndex to Type I CG PUSCH.

Proposal 2-21: For M-DCI M-TRP scenario, support to configure a CORESETPoolIndex for Type I CG PUSCH.

While for PUCCH, if it is scheduled by DCI, it also can apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PUCCH scheduled by PDCCH on a CORESET that is associated with the same coresetPoolIndex value. But for other PUCCH resource, it is necessary to configure a CORESETPoolIndex for TCI state association.

Proposal 2-22: For M-DCI M-TRP scenario, for PUCCH resource scheduled by DCI, UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUCCH transmission scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value. For other PUCCH resource, support to configure a CORESETPoolIndex to PUCCH resource.

For AP CSI-RS for CSI/BM in M-DCI based MTRP, RRC configuration with followunifiedTCIstate can be reused. And the TCI state specific to a coresetPoolIndex value can be applied to AP CSI-RS scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value if the AP CSI-RS is configured with followunifiedTCIstate.

Proposal 2-23: For M-DCI M-TRP scenario, for AP CSI-RS, UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to AP CSI-RS scheduled by PDCCH on a CORESET that is associated with the same coresetPoolIndex value if it configured with followunifiedTCIstate.

Unified TCI state indication for CJT

According to the agreement in RAN1-110b e-meeting, up to 2 joint TCI states can be applied to CJT-based PDSCH reception.

Agreement

On unified TCI framework extension, up to 2 joint TCI states can be indicated by MAC-CE/DCI and applied to CJT-based PDSCH reception (PDSCH-CJT) in a BWP/CC configured with joint DL/UL TCI mode

- Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability

- FFS: QCL type(s)/assumption(s) of the indicated joint TCI state(s) applied to PDSCH-CJT

Note: On how to inform UE to apply which indicated joint TCI state(s) to target channel(s)/signal(s) in the BWP/CC, it is discussed individually in AI 9.1.1.1

The following agreement were archived in RAN1-111 meeting.

Agreement

On unified TCI framework extension, PDSCH-CJT is supported as a S-DCI based MTRP scheme

Note: Above does not preclude discussions specific to PDSCH-CJT design in the unified TCI framework

Agreement

On unified TCI framework extension, down-select at least one of the following alternatives for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT):

- Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA

- Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state

- Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB

If one joint TCI state is supported, the parameters in QCL Type A including {average delay, delay spread, Doppler shift, Doppler spread} should be all applied. While if two joint TCI states are supported, for the 2nd Joint TCI state, which parameters should be applied?

As for the QCL type, we think the QCL parameters for SFN-PDSCH can be reused. That means {average delay, delay spread, Doppler shift, Doppler spread} can be considered. If there are more than 2 sets of {Doppler shift, Doppler spread} for N coordinated TRP, gNB side pre-compensation of SFN-PDSCH can be reused.

In addition, whether {average delay, delay spread} can also be pre-compensated by gNB should also be considered. If {average delay, delay spread} can be pre-compensated by gNB, one TCI state with QCL Type A and the other TCI state with QCL Type B can be supported.

Proposal 2-24: For two TCI states of CJT based PDSCH reception, at least support Alt 1 and Alt 2.

Enhancement on BFR

The following agreements were archived in RAN1-110b e-meeting on TRP-specific BFR:

Agreement

On unified TCI framework extension, study the following enhancements for TRP-specific BFR:

- Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP

- Enhancement to beam update after NW response to TRP-specific BFR request

In Rel-17, the TRP specific BFR was considered. But the implicit BFD-RS set determination for M-DCI based MTRP was agreed. Since for S-DCI based MTRP, there is no CORESETPoolIndex, UE doesn’t know which CORESETs belong to a same TRP. Thus in Rel-18, if implicit BFD-RS set determination for S-DCI based MTRP will be supported, it is necessary to configure a CORESET group for each CORESET of S-DCI based MTRP or to determine the implicit BFD-RS set determination for S-DCI based MTRP based on the association between CORESET and indicated joint/DL TCI state.

And according to the discussion in RAN1-111 meeting, the following potential proposal was discussed. It means that UE can determine one BFD-RS for the BFD-RS set from an indicated joint/DL TCI state if the indicated joint/DL TCI state is configured to apply for PDCCH reception. We think the potential proposal makes sense but it is not completed. Since for UE can support 2 BFD-RS for the BFD-RS set of one TRP, it is also necessary to define how to determine the second BFD-RS for the BFD-RS set.

Potential Proposal: On unified TCI framework extension for S-DCI based MTRP, if a BFD-RS set is not explicitly provided to a UE for TRP-specific BFR, the UE determines one BFD-RS for the BFD-RS set from an indicated joint/DL TCI state if the indicated joint/DL TCI state is configured to apply for PDCCH reception

- FFS: Determine from which one of the indicated joint/DL TCI states

- FFS: The case if CORESET(s) is configured to apply both indicated joint/DL TCI states for PDCCH-SFN

In order to discuss it in detail, we consider all possible types of CORESET in the scenario of S-DCI MTRP:

- Type 1 CORESET: apply the first indicated TCI state

- Type 2 CORESET: apply the second indicated TCI state

- Type 3 CORESET: apply the first and the second indicated TCI state

- Type 4 CORESET: apply the third TCI state different from the first and the second indicated TCI state, i.e., not follow unified TCI state.

And we agree that the above potential proposal is reasonable only for following special cases

- Case 1: configured with Type 1 and Type 2 CORESET.

- Case 2: configured with Type 1 and Type 3 CORESET.

- Case 3: configured with Type 2 and Type 3 CORESET.

- Case 4: configured with Type 1, Type 2 and Type 3 CORESET.

But for other cases, such as following cases with type 4 CORESET, it is not reasonable. Since for some of them, it is unnecessary to support TRP-specific BFD-RS set, such as Case 5, 6 and 7. And for others of them, BFD-RS set should also include the RS of the TCI state different from the unified TCI state, such as Case 8-14.

- Case 5: configured with Type 1 CORESET.

- Case 6: configured with Type 2 CORESET.

- Case 7: configured with Type 3 CORESET.

- Case 8: configured with Type 4 CORESET.

- Case 9: configured with Type 1 and Type 4 CORESET.

- Case 10: configured with Type 2 and Type 4 CORESET.

- Case 11: configured with Type 3 and Type 4 CORESET.

- Case 12: configured with Type 1, Type 2 and Type 4 CORESET.

- Case 13: configured with Type 1, Type 3 and Type 4 CORESET.

- Case 14: configured with Type 2, Type 3 and Type 4 CORESET.

So we propose the following improved protentional proposal

Proposal 2-27: On unified TCI framework extension for S-DCI based MTRP, if a BFD-RS set is not explicitly provided to a UE for TRP-specific BFR, the UE determines one BFD-RS for the first BFD-RS set from the first indicated joint/DL TCI state if the first indicated joint/DL TCI state is configured to apply for PDCCH reception on the first CORESET, and determines one BFD-RS for the second BFD-RS set from the second indicated joint/DL TCI state if the second indicated joint/DL TCI state is configured to apply for PDCCH reception on the second CORESET

- FFS: The case if CORESET(s) is configured to apply both indicated joint/DL TCI states for PDCCH-SFN

- FFS: If CORESET is configured to not follow unified TCI state, which BFD-RS set will contain the BFD-RS from the TCI state of such CORESET.

While for the enhancement to beam update after NW response, only the beam update for PDCCH of M-DCI based MTRP is supported in Rel-17. Even for the beam update for PDCCH of S-DCI based MTRP, there is no conclusion for TRP specific BFR. If enhancement for S-DCI based MTRP will be supported, it is also necessary to support the association between CORESET and BFD-RS set.

Simultaneous multi-panel UL transmission

The Rel-18 WID for MIMO Evolution for Downlink and Uplink is approved [1], which includes the following objective mainly targeting UL enhancements:

Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

- The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

  - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

The following agreement was made to consider involve STxMP into the extension of unified TCI framework in RAN1#109 [2].

Agreement

On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17

    - Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP

In RAN1#110 meeting, the following WA was approved [3].

Working assumption

Support the following schemes for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18:

- SDM scheme

- In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e.

- FFS: FDM-A scheme

- FFS: FDM-B scheme

- FFS: SFN-based transmission scheme

For schemes other than SDM, final decision to support or not will be made in RAN1#110bis-e.

In RAN1#110bis meeting, the following agreement was approved [4].

Agreement

Support SFN-based transmission scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18

If PUSCH/PUCCH is transmitted in SDM/SFN scheme, multiple beams are needed to transmit to different TRPs simultaneously and each beam is associated with one TRP of the two TRPs. However, the PL RS associated with each beam is also different for each TRP. Therefore, the power of both transmissions from different panels would have to be different in the same OFDM symbols within a slot.

Proposal 3-1: For STxMP, support the extension of unified TCI to the transmission schemes for both single DCI based and multi-DCI based PUSCH/PUCCH.

Several issues according to the beam indication for the simultaneous transmission of multi-panel/multi-TRP based on the unified TCI framework can be discussed.

For STxMP based PUSCH/PUCCH transmissions, 2 beams are needed to be indicated (via DCI or MAC-CE) for the transmissions towards different TRPs from different UE panels. TCIs or SRIs can be used for the detailed indication for the 2 beams. For the purpose of scheduling, the first problem need to be solved is how to identify two UL beams to support the simultaneous UL transmission. The association between the indicated TCI(s)/SRI(s) and specific TRP(s)/panel(s) should take the UE capability value set index into consideration. To support this simultaneous multi-panel transmission, UE has to let gNB aware whether the selected two beams from UE beam reporting can be simultaneously transmitted or not, and whether they can be transmitted from different panels.

In R17, the beam reporting process is enhanced with the index of UE capability value set to indicate the maximum supported number of SRS ports related to each pair of CRI/SSBRI and L1-RSRP/L1-SINR.But the gNB cannot select two UE panels with same UE capability value set for simultaneous UL transmission, and also the gNB cannot separate the case of the beam reporting for the DL-only panel case.

To determine the simultaneous uplink transmission beams, the methods based on current CSI measurement and reporting mechanism can be further enhanced.

UE can report the beam pair with an additional indication to provide information about the supported UL/DL transmission types. Three different corresponding UL/DL transmission types are:

- Type 1: DL-only simultaneous transmission

- Type 2: DL simultaneous transmission and UL non-simultaneous transmission

- Type 3: DL&UL simultaneous transmission

And an example of codepoints of the beam group index mapping to the supported UL/DL transmission types. 2bits are needed for one reported beam pair. If DL-only case is precluded, then 1bit indication is enough for each beam pair.

Table 1.example of the indication

| Codepoints indicating the transmition type of the Beam pair | UL/DL transmission types supported |
| --- | --- |
| 00 | DL-only simultaneous transmission |
| 01 | DL simultaneous transmission and UL non-simultaneous transmission |
| 10 | DL simultaneous transmission and UL simultaneous transmission including STxMP |
| 11 | Reserved |

Proposal 3-2: Enhancements can be based on group-based beam reporting for the beam report of STxMP.

Another approach is to enhance the beam reporting mechanism. Currently, either group based beam reporting or non-group based beam reporting is supported for the CSI measurement and reporting of beam management.  For the simultaneous uplink transmission, a new configuration of the beam reporting like groupBasedUplinkBeamReporting targeting STxMP can be considered for the report of beams/beam pair(s) for the STxMP transmissions.  UE would guarantee all the supported beams/beam pair(s) can support the STxMP scheduling, the beams/beam pair(s) reported can be differentiated at least for whether to support the STxMP or the TDM transmission mode for the MTRP operation.

Proposal 3-3: To determine the simultaneous uplink transmission beams, introduce a new beam reporting configuration “groupBasedUplinkBeamReporting” targeting STxMP for the report of beams/beam pair(s) for the STxMP transmissions.

For the current group-based beam reporting, only 2 beams (N=2) or one beam pair can be configured to be reported. From the analysis above, we can see that some beam pair selected cannot even be used for STxMP. Even though one pair of beams is supportive of STxMP, it may not be appropriate for the scheduling of uplink transmission because of the interference situation estimated at the gNB. So more beam pairs should be allowed to be configured for the selection to provide more flexibility for the gNB scheduler and make the schedule of STxMP much more feasible.

Proposal 3-4: Support to report in a single reporting instance of more than two (N>=2) different beams/beam pairs that UE can receive/transmit simultaneously.

For the enhanced beam reporting, it can be applied to all types of CSI reporting types, i.e. P-CSI/SP-CSI/AP-CSI.

Proposal 3-5: For the enhanced CSI measurement and reporting targeting STxMP, the enhancement can be applied to all CSI types, i.e. P-CSI/SP-CSI/AP-CSI.

For the simultaneous uplink transmission, there can be different combinations of simultaneous transmission such as PUSCH/PUSCH or PUCCH/PUCCH that can be considered for reliability enhancements. A tight coordination with potentially CSI exchange between the TRPs for the co-scheduling is needed to reduce interference between the simultaneous transmissions for SDM solutions. In these operations, uplink beams needs to be carefully selected to mitigate interference towards the other TRP.

Proposal 3-6: Beam selection enhancement needs to be further considered for the coordination between panels/TRPs to mitigate the interference for the SDM/SFN schemes.

Power control for UL single DCI

Power control for mTRP UL TDM transmission

In RAN1 109-e meeting, there is an agreement about the indication of power control parameters for S-DCI based PUSCH/PUCCH repetition with TDM.

Agreement

On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH /PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH /PUCCH (including P0, alpha for PUSCH , and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion.

- FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported

- FFS: UL PC enhancement for CB and non-CB SRS in above case

- FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH

If the power control parameters are associated with joint/UL TCI state, the UE should apply the UL PC parameter setting (including P0, alpha, and closed loop index) and the PL-RS corresponding to the indicated joint/UL TCI state for the PUSCH /PUCCH transmission occasion. While, for the case described in third FFS, in which the joint/UL TCI state(s) is not associated with an UL PC parameter setting, the indication of UL PC parameter setting is not clear yet.

Observed from TS 38.331 [1], there is a parameter, ul-powerControl-r17, in UL BWP configuration and it is configured with Uplink-powerControl-r17 which includes power control parameters { P0, alpha, closed loop index } as shown in table 1. ul-powerControl-r17 in UL BWP configuration is optional, and it only will be configured when no TCI state is associated with Uplink-powerControl-r17. Then the power control parameter corresponding to ul-powerControl-r17 will be used to determine UL transmission power.

Tab.1 UL power control parameter Uplink-powerControl-r17 in UL BWP configuration

BWP-UplinkDedicated ::= SEQUENCE { … ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL, -- Cond NoTCI-PC … ]] } Uplink-powerControl-r17 ::= SEQUENCE { ul-powercontrolId-r17 Uplink-powerControlId-r17, p0AlphaSetforPUSCH-r17 P0AlphaSet-r17 OPTIONAL, -- Need R p0AlphaSetforPUCCH-r17 P0AlphaSet-r17 OPTIONAL, -- Need R p0AlphaSetforSRS-r17 P0AlphaSet-r17 OPTIONAL -- Need R } P0AlphaSet-r17 ::= SEQUENCE { p0-r17 INTEGER (-16..15) OPTIONAL, -- Need R alpha-r17 Alpha OPTIONAL, -- Need R closedLoopIndex-r17 ENUMERATED { i0, i1 } } Uplink-powerControlId-r17 ::= INTEGER(1.. maxUL-TCI-r17)

However, based on Table 1, there is only one set of {P0, Alpha, closed Loop Index} for each UL Channel/signal. Then, to support single DCI based multi-TRP UL transmission, two sets of PC parameters {P0, alpha, closed loop index} need to be configured in UL BWP configuration or other RRC IE when these parameters are not associated with joint/UL TCI state.

Proposal 4-1: Two sets of PC parameters {P0, alpha, closed loop index} can be configured via RRC to support single DCI based multi-TRP UL transmission if both indicated joint/UL TCI states are not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH

Power control for STxMP

In RAN1 109-e meeting, the definition of Pcmax was discussed and it should be decided first. Therefore, we sent a LS, in which three possible assumptions were listed as shown following[1], to RAN4.

109-e-Agreement

On UE power limitation for STxMP for FR2, send LS to RAN4 to check the followings:

- Whether it is feasible to assume power limitation per panel for STxMP (Assumption 1)

- Whether it is feasible to assume a total power limitation per UE over all UE panels used for STxMP (Assumption 2)

- In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class?

- If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)?

FFS: Detail of exact LS if agreed

Note: Scenarios of above include at least single carrier scenario for FR2

Note: Above power limitation includes both total radiated power and EIRP

LS to RAN4 is endorsed in R1-2205639.

For the perspective of handset manufacturers, the excessive transmit power is unbearable because of the limited battery capacity. Therefore, a total power limitation per UE over all panel should be defined. Thus, we prefer to support assumption 2 or both assumption 1 and assumption 2.

Proposal 4-2: About the definition of power limitation for STxMP for FR2, support Assumption 2 or both Assumption 1 and Assumption 2.

In addition, based on the following agreements in agenda 9.1.4.1, both single DCI based STxMP and multi-DCI based STxMP should be supported[1][2].

109-e-Agreement

For STxMP PUSCH in single-DCI based mTRP system, study and evaluate the following schemes for PUSCH:

- SDM scheme: different layers/DMRS ports of one PUSCH are separately precoded and transmitted from different UE panels simultaneously.

  - Study and evaluate whether to support 2 CWs in SDM manner and transmitted from two different panel simultaneously.

- FDM-B scheme: two PUSCH transmission occasions with same/different RV of the same TB are transmitted from different UE panels on non-overlapped frequency domain resources and the same time domain resources.

- FDM-A scheme: different parts of the frequency domain resource of one PUSCH transmission occasion are transmitted from different UE panels.

- SFN-based transmission scheme: all of the same layers/DMRS ports of one PUSCH are transmitted from two different UE panels simultaneously.

- SDM repetition scheme: two PUSCH transmission occasions with different RV of the same TB are transmitted from two different UE panels simultaneously.

Note: Companies are encouraged to evaluate the different schemes for possible down-selection in RAN1#110.

110-bis-Agreement

Support STxMP PUSCH+PUSCH transmission in multi-DCI based system in Rel-18.

- Two independent PUSCHs associated with different TRPs can be transmitted by a UE simultaneously in same active BWP.

- The total number of layers of these two PUSCHs is up to 4.

FFS: whether the number of layers of each of these two PUSCHs is up to 2.

In this part, the power control for single DCI based STxMP (Fig.1(a)) and Multi-DCI based STxMP (Fig.1(b)) will be discussed separately.

Fig.1 Single DCI and M-DCI based STxMP

Single DCI based STxMP

According to TS38.213, UE will split the power of PUSCH equally across the antenna ports. Then, TRP/panel specific power control cannot be support in current power control framework for single DCI based STxMP.

There are two possible methods to deal with this issue. Firstly, the power control for PUSCH in STxMP is still channel specific and the UE splits the transmit power across panels considering the number of antenna ports, channel conditions and other aspects for each panel. But the exact way to split the power and indicate power control parameters needs further discussion. Secondly, the power control is panel specific, in which UE calculates the transmit power for each panel according to the parameters indicated for each panel. Then the PUSCH transmission power PPUSCH is PPUSCH=Ppanel1+Ppanel2.

Proposal 4-3: There are two possible power control methods for STxMP

- Option 1: UE determines a total PUSCH transmission power and splits the power across the multiple panels.

- Option 2: UE determines the transmit power for each panel independently

Because for single DCI based STxMP, two TCI state should be indicated, and correspondingly two sets of power control parameters are indicated too. It is nature to decide the transmit power for each panel based on the two sets of power control parameters. What’s more, how to determine a total transmit power is hard to decide when two sets of power control parameters are indicated. Therefore, option 2 is a more feasible power control method for STxMP.

Proposal 4-4: Option 2 is a more feasible power control method for STxMP.

The same with S-DCI based PUSCH/PUCCH repetition with TDM, two joint/UL TCI states should be indicated. And in unified TCI framework in R17, the power control parameters are associated with joint/UL TCI state. Then, UE should apply the UL PC parameters for the PUSCH /PUCCH transmission occasion

Proposal 4-5: For single DCI based STxMP, the UE should apply the UL PC parameters associated with the two indicated joint/UL TCI states for the PUSCH/PUCCH transmission occasion.

Multi-DCI based STxMP

For Multi-DCI based STxMP, each PUSCH transmission occasion (TO) is scheduled by corresponding PDCCH and the transmit beam for each PUSCH TO is indicated by its own PDCCH. Then UE should apply the UL PC parameters associated with joint/UL TCI state indicated by the corresponding PDCCH for each PUSCH transmission occasion.

Proposal 4-6: For Multi-DCI based STxMP, UE should apply the UL PC parameters associated with joint/UL TCI state indicated by the corresponding PDCCH for each PUSCH transmission occasion.

In addition, based on the agreement about multi-DCI based STxMP in agenda 9.1.4.1 [3], the PUSCHs can be fully/partially overlapped in time domain.

110-bis-Agreement

The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain.

- FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are fully/partially overlapped in time domain (Depending on RAN4’s input on Pcmax requirements).

- Note: No symbol-level power adjustment within a PUSCH transmission occassion in the case of fully/partially overlapping in time domain

As we discussed in section 4.2 about the definition of power limitation, a total power limitation over all UE panels should be defined, which can be denoted as Pcmax. Then, as shown in Figure 1, the transmit power of PUSCH 1 between t1 and t2, PUSCH1t1-t2 , should be less than Pcmax. While the total transmit power of PUSCH1 and PUSCH2 between t2 and t3, PUSCH1t2-t3+PUSCH2t2-t3 , should be less than Pcmax. Thus, the transmit power of the PUSCH transmission occasion between t2 and t3 might be different from that between t1 and t2 or t3 and t4. Then, whether UE can change the transmit power within the one PUSCH transmission occasion should be clarified. To keep the calculation of transmit power simpler and have the same rules with current specification, UE should not change the transmit power within one PUSCH transmission.

Fig.2 Partially overlapping PUSCHs in time domain

Proposal 4-7: For Multi-DCI based STxMP, when the PUSCHs form multiple panels are partially overlapped in time domain, UE should not change the transmit power within one PUSCH transmission if a total power limitation over all UE panels is defined.

Conclusion

In this contribution, we discuss about the unified TCI framework extension for M-TRP. Based on above discusses, we provide the following proposals.

Indication of multiple DL/UL TCI states

Observation 2-1: Additional indication or default rule to indicate the mapping between activated TCI states and S-DCI based MTRP CC(s) or M-DCI based MTRP CC(s) is needed to support a CC list comprised of a mix of S-DCI based MTRP CC(s) and M-DCI based MTRP CC(s).

Proposal 2-1: For PDCCH reception with PDCCH SFN, suggest to down-select one from the following alternatives on the behavior if the UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP.

- Alt 1: UE stops monitoring the PDCCH candidates of CORESET configured to apply two indicated joint/DL TCI states.

- Alt 2: UE monitors the PDCCH candidates of CORESET configured to apply two indicated joint/DL TCI states by the one indicated joint/DL TCI state.

- Alt 3: UE monitors the PDCCH candidates of the CORESET configured to apply two indicated joint/DL TCI states by the one indicated joint/DL TCI state of the first TRP and the non-changed indicated joint/DL TCI state of the second TRP.

Proposal 2-2: For PDCCH reception with PDCCH repetition, suggest to down-select one from the following alternatives on the behavior if the UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP.

- Alt 1: UE stops monitoring the PDCCH candidates of two linked search space set.

- Alt 2: UE monitors the PDCCH candidates of only one search space set corresponding to the one indicated joint/DL TCI state of the one TRP.

- Alt 3: UE monitors the PDCCH candidates of the first search space set by the one indicated joint/DL TCI state of the first TRP and monitors the PDCCH candidates of the second search space set by the non-updated indicated joint/DL TCI state of the second TRP.

Proposal 2-3: Support to define default TCI state(s) applied to the PDSCH reception for following cases:

- When the TCI association filed is not present in DCI format 1_1/1_2 (if the presence of the TCI association is configurable by RRC).

- When the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold.

- After receiving the first MAC CE/DCI for the indicated TCI states and before the application time of the first DCI for informing the TCI association.

Proposal 2-4: For PDSCH configured with fdmschemeA/ fdmschemeB/ tdmschemeA/ two CDM groups/ repetitionNumber, the default TCI state for PDSCH reception can be both indicated joint/DL TCI states directly or based on enabletwodefaultTCIstates configured by RRC.

Proposal 2-5: For PDSCH configured with SFN scheme, the default TCI state for PDSCH reception will be both indicated joint/DL TCI states directly if UE doesn’t support dynamic SFN. Else，it can be both indicated joint/DL TCI states based on enabletwodefaultTCIstates configured by RRC.

Proposal 2-6: For PDSCH configured with SFN scheme, UE is not expected to receive a DCI field in DCI format 1_1/1_2 indicating that PDSCH is associated with only one indicated joint/DL TCI state if UE doesn’t support dynamic switching between SFN and S-TRP.

Proposal 2-7: For PDCCH configured with SFN scheme B, or PDCCH configured with SFN scheme A and UE doesn’t support sfn-SchemeA-PDCCH-only, UE is not expected to receive a DCI field in DCI format 1_1/1_2 indicating that PDSCH is associated with only one indicated joint/DL TCI state.

Proposal 2-8: If UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP, there are two Alternatives for PDSCH.

- Alt 1: UE is not expected to receive a DCI for the TCI association to associate PDSCH with two TCI states

- Alt 2: UE applies one new indicated joint/DL TCI state and the other one indicated joint/DL TCI state will be not updated for PDSCH if it receives a DCI for the TCI association to associate PDSCH with two TCI states

Proposal 2-9: On unified TCI framework extension for S-DCI based MTRP, for PDSCH transmission scheduled/activated by a DCI format 1_0, down-selection one alternative from the followings

- Alt 1: Same as that of the CORESET with the scheduling PDCCH.

- Alt 2: Based on default rule, e.g., use the first TCI state of the indicated joint/DL TCI state(s) or both.

- Alt 3: Use RRC/MAC CE to inform the mapping/association.

- Alt 4: Based on the mapping/association informed by the latest DCI format 1_1/1_2.

Proposal 2-10: Support to transmit PUCCH associated with two TCI states by the one indicated TCI state if it receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP, or apply one new indicated joint/UL TCI state and keep the other one not updated.

Proposal 2-11: Support to define a default TCI state by a fixed rule, and the default TCI state can be applied to the PUSCH transmission scheduled/activated by DCI format 0_1/0_2 after receiving the first MAC CE/DCI for the indicated TCI states and before the application time of the first DCI for informing the TCI association.

Proposal 2-12: If UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP, there are two Alternatives for PUSCH.

- Alt 1: UE is not expected to receive a DCI for the TCI association to associate PUSCH with two TCI states

- Alt 2: UE applies one new indicated joint/UL TCI state and the other one indicated joint/UL TCI state will be not updated for PUSCH if it receives a DCI for the TCI association to associate PUSCH with two TCI states

Proposal 2-13: On unified TCI framework extension for S-DCI based MTRP, for Type 1 CG PUSCH repetition, support to use RRC configuration or default rule to inform the mapping/association.

Proposal 2-14: Support to transmit CG type 1 PUSCH associated with two TCI states by the one indicated TCI state, or by the one indicated TCI state and keep the other one not changed, if it receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP.

Proposal 2-15: On unified TCI framework extension for S-DCI based MTRP, for PUSCH transmission scheduled/activated by a DCI format 0_0, down-selection one alternative from the followings

- Alt 1: Same as that of the CORESET with the scheduling PDCCH.

- Alt 2: Based on default rule, e.g., use the first TCI state of the indicated joint/UL TCI state(s).

- Alt 3: Use RRC/MAC CE to inform the mapping/association.

- Alt 4: Based on the mapping/association informed by the latest DCI format 0_1/0_2.

Proposal 2-16: Use RRC to configure the association between indicated TCI states and AP CSI-RS for CSI/BM.

Proposal 2-17: For default TCI state for AP CSI-RS for CSI/BM, to apply the first indicated TCI state of CORESET when with two TCI states.

Proposal 2-18: Use RRC to configure the association between indicated TCI states and SRS.

Proposal 2-19: For default TCI state for SRS for CB/NCB, 1 to 1 mapping can be used in the case of two SRS resource sets. And the first indicated TCI state can be applied in the case of only one SRS resource set.

Proposal 2-20: The application time for TCI state mapping/association information for PDSCH/PUSCH should be defined.

Proposal 2-21: For M-DCI M-TRP scenario, support to configure a CORESETPoolIndex for Type I CG PUSCH.

Proposal 2-22: For M-DCI M-TRP scenario, for PUCCH resource scheduled by DCI, UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUCCH transmission scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value. For other PUCCH resource, support to configure a CORESETPoolIndex to PUCCH resource.

Proposal 2-23: For M-DCI M-TRP scenario, for AP CSI-RS, UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to AP CSI-RS scheduled by PDCCH on a CORESET that is associated with the same coresetPoolIndex value if it configured with followunifiedTCIstate.

Proposal 2-24: For two TCI states of CJT based PDSCH reception, at least support Alt 1 and Alt 2.

Simultaneous multi-panel UL transmission

Proposal 3-1: For STxMP, support the extension of unified TCI to the transmission schemes for both single DCI based and multi-DCI based PUSCH/PUCCH.

Proposal 3-2: Enhancements can be based on group-based beam reporting for the beam report of STxMP.

Proposal 3-3: To determine the simultaneous uplink transmission beams, introduce a new beam reporting configuration “groupBasedUplinkBeamReporting” targeting STxMP for the report of beams/beam pair(s) for the STxMP transmissions.

Proposal 3-4: Support to report in a single reporting instance of more than two (N>=2) different beams/beam pairs that UE can receive/transmit simultaneously.

Proposal 3-5: For the enhanced CSI measurement and reporting targeting STxMP, the enhancement can be applied to all CSI types, i.e. P-CSI/SP-CSI/AP-CSI.

Proposal 3-6: Beam selection enhancement needs to be further considered for the coordination between panels/TRPs to mitigate the interference for the SDM/SFN schemes.

Power control for UL single DCI

Proposal 4-1: Two sets of PC parameters {P0, alpha, closed loop index} can be configured via RRC to support single DCI based multi-TRP UL transmission if both indicated joint/UL TCI states are not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH

Proposal 4-2: About the definition of power limitation for STxMP for FR2, support Assumption 2 or both Assumption 1 and Assumption 2.

Proposal 4-3: There are two possible power control methods for STxMP

- Option 1: UE determines a total PUSCH transmission power and splits the power across the multiple panels.

- Option 2: UE determines the transmit power for each panel independently

Proposal 4-4: Option 2 is a more feasible power control method for STxMP.

Proposal 4-5: For single DCI based STxMP, the UE should apply the UL PC parameters associated with the two indicated joint/UL TCI states for the PUSCH/PUCCH transmission occasion.

Proposal 4-6: For Multi-DCI based STxMP, UE should apply the UL PC parameters associated with joint/UL TCI state indicated by the corresponding PDCCH for each PUSCH transmission occasion.

Proposal 4-7: For Multi-DCI based STxMP, when the PUSCHs form multiple panels are partially overlapped in time domain, UE should not change the transmit power within one PUSCH transmission if a total power limitation over all UE panels is defined.

References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink

- TS 38.321 h30, Medium Access Control (MAC) protocol specification

- RAN1-109 e-meeting Chairman notes.

- RAN1-110 meeting Chairman notes.

- RAN1-110b e-meeting Chairman notes.

- RAN1-111 meeting Chairman notes.