3GPP TSG RAN WG1 #112			R1-2300574

Athens, Greece, February 27th – March 3rd, 2023

Source: 	     Xiaomi

Title:		     Discussion on subband non-overlapping full duplex

Agenda item:    9.3.2

Document for:  Discussion

Introduction

In RAN1#111 meeting, subband non-overlapping full duplex was discussed and the following agreements were achieved: [1]

| Agreement For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived - DL receptions within DL subband(s) are allowed in the symbol - Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol Agreement For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid. Agreement Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following: - PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols - Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols - Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition - Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols - Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered. Examples of potential enhancements include: - Resource allocation in frequency domain including frequency hopping - Resource allocation in time domain - Power domain - Spatial domain FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured. Agreement For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs, Option 1: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol - FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol Option 2: - UL transmissions within UL subband are allowed in the symbol - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same - FFS: SBFD aware UE behaviours - FFS: Whether or not signalling of guardband(s) is needed - FFS: Whether or not the symbol can be converted to a DL-only symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously Agreement Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following: - RBG for PDSCH RA type 0 - CSI reporting configuration - CSI-RS resource configuration - PRG of PDSCH |  |
| --- | --- |

During RAN1#111 meeting, we heatedly discussed behaviour for SBFD aware UE on both semi-static DL symbol and semi-static flexible symbol. Besides, RAN1 also reached the consensus that we need to further study the case where UL transmission and DL transmission across different resource type, i.e. across SBFD symbol and non-SBFD symbol, across UL RB and DL RB on the same SBFD symbol.

In this contribution, we provide our analyses and views on the subband non-overlapping full duplex.

Discussion

Time domain resource allocation for UL subband

In previous meeting, it was agreed that explicit configuration of SBFD subband time locations within a period is the baseline. [2] Accordingly, we need to clarify how to determine the period. A straightforward direction is to configure SBFD subband time location within a period equal to one or multiple slot configuration periods determined by TDD UL DL configuration. Basically, there are different cases of TDD frame structure determination, which are summarized as below:

- Case#1: TDD frame structure with single periodicity is configured by tdd-UL-DL-ConfigurationCommon only

- Case#2: TDD frame structure with two periodicities is configured by tdd-UL-DL-ConfigurationCommon only

- Case#3: TDD frame structure is configured by tdd-UL-DL-ConfigurationCommon, and  tdd-UL-DL-ConfigurationDedicated additionally

  - Case#3-1: single periodicity

  - Case#3-2: dual periodicity

- Case#4: Network doesn’t provide any TDD UL-DL configuration, i.e. all the symbols are regarded as semi-static flexible symbol

For case#1, gNB provides UE a slot configuration within a periodicity, i.e. P milliseconds, via SIB1.  Generally speaking, the first N consecutive slot within period P are DL slots and the last M consecutive slots within period P are UL slots. Additionally, several DL symbols can be configured immediately after N DL slots and several UL symbols can be configured before the M UL slots. The left symbols are semi-static flexible symbols. In this case, the period of configuring SBFD subband time location can be equal to TDD periodicity. For example, a SLIV-like mechanism can be used to indicate time location of SBFD subband, i.e. indicate the starting position and the length.

Figure 1 Example of SBFD subband time location based on single TDD periodicity

For case#2, gNB configures dual TDD periodicity which are denoted as P and P2. For each periodicity, the configuration is same as the single periodicity illustrated in case#1. It should be noted that there is no restriction on the length of each periodicity. Any {P, P2} combination satisfies the condition that P+P2 divides 20 msec is supported. In the other words, the TDD frame structure corresponding to each period can be totally different from each other. Consequently, it is quite difficult to use single SLIV indication to indicate the time location of SBFD subband. For example, assuming the reference SCS=15 kHz with P=3 msec and P2=2 msec. The specific TDD frame structure is shown in Figure 2.  Obviously, the situation is much more complicated compared with case#1.  Here we still take SLIV-like mechanism as example. It is not clear whether a time domain indication is applied to two periodicity or it is applied to each periodicity respectively. If we follow the former understanding, a single SLIV is not sufficient to indicate time location of a UL subband in two TDD periodicities as the DL regions across two periodicity are discrete. On the other hand, the latter understanding bring restriction on the time domain location, as gNB has to guarantee the indicated time domain location does not exceed each TDD periodicity.

Figure 2 Example of SBFD subband time location based on dual TDD periodicity

For case#3, a slot level TDD structure can be indicated by tdd-UL-DL-ConfigurationDedicated on top of tdd-UL-DL-ConfigurationCommon. The UE-dedicated signalling is used to configure the slot structure of each semi-static flexible slot. One example is shown in Figure 3, where two switching points are configured. Similar issues as case#2, a SLIV-like method is incapable to indicate the discrete time domain resources for subband.

Figure 3 Example of SBFD subband time location based on common TDD+UE-dedicated TDD

Case#4 is a special case wherein all the slots are semi-static flexible. There is no reference TDD periodicity. Therefore, gNB needs to configure a period within which SBFD subband is configured if SLIV-like method or bitmap-like method is enployed to indicate time domain allocation of UL subband.

As analysed above, existing TDD periodicity provided by tdd-UL-DL-ConfigurationCommon cannot be directly reused as the period for configuring SBFD subband time domain location.

Observation 1: There are several mechanisms to realize TDD UL-DL configuration which leads to different TDD periodicity:

- Single TDD periodicity provided by tdd-UL-DL-ConfigurationCommon

- Double TDD periodicity provided by tdd-UL-DL-ConfigurationCommon

- Additional TDD UL-DL configuration provided by tdd-UL-DL-ConfigurationDedicated on top of common configuration

- No TDD periodicity is provided by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated

Based on the aforementioned observation, there can be single TDD periodicity, double TDD perioidcity or none TDD periodicity. A unified solution to indicate the SBFD subabnd time domain location should be pursued in order to simplify specification and gNB operation. Otherwise, gNB has to use different mechanisms to configure SBFD subband time domain location for different TDD UL-DL configurations.

Observation 2: A unified solution to indicate the SBFD subband time domain location should be enployed in order to simplify specification and gNB operation.

Regarding to how to indicate SBFD subband time domain location within a time period, there are several candidate solutions:

- Option 1: gNB jointly indicates the starting position and duration of SBFD subband time domain location, i.e. SLIV-like mechanism.

- Option 2: gNB indicates SBFD subband time domain location via a bitmap, e.g. 1 denotes SBFD subband occupies the related time unit while 0 denotes no SBFD subband exists on the related time unit.

- Option 3: gNB indicates SBFD subband time domain location via a subband-specific TDD UL-DL configuration.

First of all, it can be also observed that discrete DL region in time domain is a typical case, i.e. TDD frame structure with two periodicity or UE-dedicated configuration. SLIV-like mechanism is not sufficient for indicating SBFD time location for discrete DL region.

Observation 3: SLIV-like method is not sufficient for indicating SBFD time location for discrete DL region hence a unified indication is impossible across different TDD configuration scenarios.

Bitmap based method is sufficient to indicate SBFD subband time domain location, despite of consecutive time domain resources or discrete time domain resources. Accordingly, we need to further discuss the time unit for indication. For example, 1 bit contained by the bitmap can be used to indicate the occurance of SBFD subband on a slot, several OFDM symbols or a OFDM symbol. From our perspective, a time unit as small as single OFDM symbol is not necessary which will bring huge signaling overhead. Slot level granularity is sufficient which provides good trade-off between overhead and flexibility. As aforementioned, TDD frame structure is divergent and there may be single, double or none TDD periodicity is configured. Hence, time period during which the bitmap is applicable needs to be discussed. One straigthforward way is to use individual bitmap to indicate the SBFD subband time domain location for each TDD periodicity. Single bitmap is configured if there is only one TDD periodicity. If two TDD patterns are configured, two bitmaps are needed in order to indicate the SBFD subband time domain location for eachh TDD periodicity. There is a possible case that neither common TDD configuration nor UE-dedicated TDD configuration is configured. Theoretically, every OFDM symbol is regarded as semi-static flexible. There is no TDD periodicity which can be directly used as time period for configuring SBFD subband time domain location. In order to resolve the ambiguity on time period, a default time period can be adopted once no TDD configuration is provided by network, e.g. 20 ms.

Observation 4: Bitmap based method can be used to indicate SBFD time location within a time period.

- The time period is equal to TDD periodicity provided by tdd-UL-DL-ConfigurationCommon

- FFS: the time unit of bitmap

- FFS: how to determine time period if gNB doesn’t provide any TDD configuration

Basically, the bitmap-based indication provides the time domain location of SBFD subband. One example is shown in Figure 4 as below. In theory, guard period is needed between DL transmission and UL transmission in order to avoid inter-slot interference. Otherwise, UE-to-UE CLI will jeopardize the DL transmission for both SBFD aware UE and none SBFD aware UE which receives DL transmission on the previous DL slot. Therefore, it needs attention on how to define or indicate guard period between DL symbol and the first OFDM symbol occupied by UL subband.

Figure 4 Example of SBFD subband time location based on bitmap indication

Observation 5: If bitmap-based mechanism is used to indicate SBFD subband time domain location, how to define guard period between DL symbol and first symbol occupied by UL subband needs further study.

Option 3 provide a TDD UL-DL configuration for SBFD subband so as to indicate the time domain location. Current TDD UL-DL configuration can be fully reused for subband TDD structure configuration, i.e. common TDD configuration or UE dedicated TDD configuration. Take UE-dedicated TDD configuration as example, assuming TDD UL-DL configuration of the carrier is DDDDDDFFUU, gNB configures a subband-specific TDD UL-DL configuration for UL subband as DFUUUUUFU. Accordingly, the SBFD subband time domain allocation is shwon in Figure 5. In this example, TDD periodicity is equal to the one provided by tdd-UL-DL-ConfigurationCommon for carrier. Exactly same signaling structure as tdd-UL-DL-ConfigurationCommon is reused to indicate the subband-specific TDD structure. As gNB has for power to determine the TDD structure for each slot, the guard period between DL symbol and first OFDM symbol occupied by SBFD subband can be easily configured. Furthermore, it is also friendly to the scenario with UE-dedicated TDD frame structure for a carrier.

Figure 5 Example of SBFD subband time location based on SBFD-specific TDD UL-DL configuration

Observation 6: A subband-specific TDD UL-DL configuration can be used to indicate SBFD subband time domain location.

Based on the previous analyses, we think either bitmap-based solution or TDD UL-DL configuration based solution can be employed to indicate SBFD subband time domain location. Hence, we have the following proposal:

Proposal 1: For semi-static configuration of subband time locations for SBFD operation, the following mechanisms can be considered:

- gNB indicates SBFD subband time domain location via a bitmap

- gNB indicates SBFD subband time domain location via a subband-specific TDD UL-DL configuration

In generic, gNB can indicate SBFD subband time location via either cell-specific signalling or UE-dedicated signalling. For cell-specific signalling, all SBFD capable UEs belonging to the same cell has unified subband in time domain. It is friendly to configuration from network perspective as gNB only needs to deliver same indication to all the UEs. However, it loses flexibility to some extent. Given different UE may be interested in different traffic, SBFD UE may have different requirement on subband. UE-dedicated signalling is expected to carter to UE-specific requirement. Furthermore, UE behaviour on SBFD symbol is still under discussion, i.e. whether it is allowed to receive DL within UL subband. If the answer is yes, there is no difference on flexibility perspective between cell-specific manner and UE-dedicated manner.

On the other hand, even SBFD subband is configured via cell-specific signaling, UE can still receive DL transmission within the DL subband on SBFD slots for sure. In this sense, the bonus from UE-dedicated subband configuration might be trivial compared with the penalty of more complicated configuration coming along with.

Proposal 2: Cell-specific signalling is sufficient to configure SBFD subband time location.

It was discussed whether dynamic indication is supported for SBFD subband time location. The motivation is to improve scheduling flexibility and to adapt traffic load. For example, SBFD subband can be indicated with a longer duration in time domain once if uplink traffic load goes up. It is beneficial to improve UL performance. However, gNB can also configure a SBFD subband with sufficient time domain resources at the first place via semi-static signalling.  One example is shown in Figure 6, wherein a UL subband is configured to locate on all the DL symbols within a period. UL scheduling flexibility and performance is guaranteed. From this perspective, dynamic indication for subband time location does not provide additional benefits. On the other hand, it may impact the scheduling flexibility of DL for a SBFD aware UE in the other way around if we assumes SBFD UE only expects transmission on the UL subband in SBFD slot. However, the UE behaviour on SBFD slot is still under discussion, wherein several options are raised. For example, if SBFD aware UE can receive DL in the UL subband according to DL assignment, it is not an issue for DL scheduling flexibility.

Figure 6 Example of SBFD subband existing in all DL symbols

Proposal 3: Before deciding whether dynamic indication is supported for SBFD subband time location, we need to firstly clarify UE behaviour in SBFD slot.

No matter which mechanism is used to indicate SBFD subband time domain location, SBFD subband can be configured on semi-static flexible symbol. It is well known that gNB can adjust the transmission direction of semi-static flexible symbol according to traffic load at least. Assuming the TDD UL-DL configuration for carrier is DFFFU configured via tdd-UL-DL-ConfigurationCommon, gNB configures SBFD subband time location via a cell-specific signaling, either via a bitmap or subband-specific TDD UL-DL configuration. Here we assume the UL subband occupies slot#1, slot#2, slot#3 in time domain, which is shown in Figure 7. As slot#1, slot#2 and slot#3 are semi-statically flexible slots configured via cell-specific signaling, gNB can further indicate the slot direction via UE-dedicated signaling or SFI. To be specific, the flexible symbols can be either indicated as DL symbol or UL symbol. It can be summarized as below:

- If semi-flexible symbol is indicated as DL symbol on which UL subband is configured, what is the expected behaviour for SBFD aware UE?

- If semi-flexible symbol is indicated as UL symbol on which UL subband is configured, what is the expected behaviour for SBFD aware UE?

- If semi-flexible symbol is indicated as SFI flexible symbol on which UL subband is configured, what is the expected behaviour for SBFD aware UE?

Figure 7 Example of SBFD subband locates on semi-static flexible slots

Proposal 4: If UL subband is configured on semi-static flexible symbol and gNB further adjust direction for the symbols, clarify behaviour of SBFD aware UE at least for the following cases:

- Semi-flexible symbol is indicated as DL symbol on which UL subband is configured

- Semi-flexible symbol is indicated as UL symbol on which UL subband is configured

- Semi-flexible symbol is indicated as SFI flexible symbol on which UL subband is configured

Frequency domain resource allocation for UL subband

In RAN1#110bis e-meeting, the following agreement was achieved.

Agreement For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined.

Furthermore, it was agreed that frequency location of UL/DL subband is with reference to CRB grid in RAN1#111 meeting, which is shown as below. In this section, we further discuss the remaining issues on determining the frequency location of UL/DL subband.

Agreement For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid.

There are three types of frequency resource in SBFD symbol, i.e. UL subband, DL subband and guard band. gNB at most needs to indicate or determine two out of these three subbands. The frequency location of the third subband can be automatically determined on SBFD symbols. For example, DL subbands within an active DL BWP can be derived from the information of UL subband and guard band. Then we can focus on one issue: whether explicit configuration/indication is needed for determining guard band. If the answer is yes, gNB needs to explicitly indicate either DL subbands or guard band. Otherwise, explicit indication of frequency location of UL subband is sufficient.

Generally speaking, the functionality of guard band can be summarized as below:

- Suppress self-interference via frequency isolation together with spatial isolation, Beam nulling /isolation and Digital IC.[3]

- Suppress inter-subband gNB-to-gNB CLI assuming aligned UL subband configuration is applicable across gNBs.

Regarding inter-subband gNB-to-gNB CLI suppression, it can be regarded as a sub-case for that studied in dynamic TDD. Considering DTDD needs to handle intra-subband gNB-to-gNB CLI, inter-subband gNB-to-gNB CLI should not be a concern anymore when we discuss guard band for SBFD operation.

Regarding self-interference suppression, as mentioned by the reply LS from RAN4, RAN4 is discussing how much frequency isolation is needed. In the other word, the definition and configuration of guard band needs RAN4 involvement.

From UE perspective, it needs to have full information of UL subband in order to behave in the correct way. Besides UL subband, we don’t see strong motivation of explicit configuration. For a SBFD ware UE, it doesn’t need to know the subband type beside UL subband if it transmit uplink within UL subband. On the other hand, it doesn’t need to know the subband type beside UL subband if it receives downlink either. The reason is UE only has half duplex capability, it doesn’t care the resources can not be used. Hence, guard band actually can be implicitly determined by scheduling.

One issue related to guard band is it cannot be used for either UL transmission or DL reception, SBFD aware UE may needs to recognize guard band in order to perform rate matching once it’s DL channel/RS overlaps with it.  Actually, RMR can be reused here in order to facilitate rate matching.

As aforementioned, guard band provides frequency isolation between UL subband and DL subband. From this perspective, it is not always needed if there is no UL transmission within UL subband or there is already sufficient GAP between DL transmission and UL reception. Figure 8 provides three examples, denoted as case-a, case-b and case-c respectively.

Figure 8 Examples for different scheduling status within UL subband

For case-a, the UL subband is fully occupied by uplink transmission. Guard band is needed in order to guarantee there is sufficient isolation for SI suppression. For case-b, uplink is scheduled in the middle of UL subband. In this case, the unused RBs within UL subband may be sufficient for frequency isolation and hence guard band is not needed. For case-c, there is no uplink within UL subband. Guard band is not needed as gNB only needs to transmit downlink and there is no self-interference issue. If guard band is always assumed between DL subband and UL subband, it will jeopardize downlink performance.

Observation 7: Guard band between DL subband and UL subband is not aways needed.

Proposal 5: Study whether and how to use guard band for downlink transmission.

Basically, the definition of guard band needs RAN4 involvement. For example, what guard band size is required to suppress SI sufficiently and where the guard band locates, i.e. out of UL subband or included in UL subband, depends on RAN4. As one example, guard bands are included in UE channel bandwidth which cannot be used for transmission/reception. No matter which direction is adopted in the end, it seems no explicit indication is needed as it is totally RAN4 work.

Based on the minimum requirement of guard band, gNB schedules DL in DL subband for legacy UE and UL in UL subband for SBFD aware UE with guaranteeing the guard band is sufficient.

Proposal 6: Send LS to RAN4 and consult RAN4 how guard band between DL subband and UL subband is determined.

Currently, the frequency resources of UL BWP and DL BWP is indicated via SLIV, i.e. the starting RB index and BWP size provided by locationAndBandwidth. In the other words, gNB indicates the starting RB index and the number of RBs of a BWP within a carrier. The reference point is CRB#0 and the offset to the reference point is indicated via offsetToCarrier. Considering the frequency location of SBFD subband is contained within a carrier, it is straightforward to reuse the same mechanism indicating frequency resource of a BWP.

On the other hand, it may be unncessary to configure a UL subband with different sizes from that of active UL BWP. Especially considering we are still discussing the UE behaviour within UL subband, i.e. whether it is allowed to receive DL transmission within UL subband, flexibility of DL transmission can be guaranteed even a UL subband as large as active UL BWP is configured in DL slot. Hence, the frequency location of UL subband can be exactly same as one of UL BWP. It can significantly reduce RRC signaling complexity on frequency location configuration, i.e. gNB doesn’t need to additionally indicate the frequency location of a UL subband. However, gNB has to guarantee all of SBFD aware UE has same UL BWP in order to achieve a common UL subband configuration with same frequency location. Consdiering UE supports up to 4 UL BWPs within a carrier, the UL BWP with the smallest index can be used to determine the frequency location of UL subband if gNB doesn’t provide frequency location configuration for UL subband.

Proposal 7: For configuring frequency location of UL subband, the following mechanism can be considered:

- Reuse the same mechanism of configuring BWP to configure the frequency locations of subband for SBFD operation, i.e. the starting position and length are indicated.

- If gNB doesn’t provide any configuration for UL subband frequency location, a default BWP is used to determine the frequency location of the UL subband.

In addition to semi-static configuration of subband frequency locations for SBFD operation, dynamic mechanism was also discussed. There is concern on gNB filter design if the frequency range can dynamically change. Dynamic indication can adjust the size of a UL subband so that it can alleviate the impacts on legacy DL transmission on the DL subband. For example, if UL traffic becomes low, gNB can indicate a smaller UL subband. Consequently, the DL subband is enlarged and more resources can be used for DL transmission. However, it again highly depends on the UE behaviour in SBFD symbols, which is still open. For example, if a SBFD aware UE supports DL reception within UL subband, dynamic UL subband size changing doesn’t provide any additional benefits for DL performance. Also, dynamic indication is not a free meal and we have to spend lots of efforts on standardization.

Proposal 8: Before deciding whether dynamic indication is supported for SBFD subband frequency location, we need to firstly clarify UE behaviour in SBFD slot.

Similar to time domain location indication, there are two directions for semi-static configuration of subband frequency locations for SBFD operation, i.e. via cell-specific signalling or via UE-dedicated signalling. If UL subband is configured via UE-deidcated RRC signaling, unaligned UL subband is inevitable across different gNB and across different UE. It is against the agreement that we should consider same subband frequency resources across different SBFD symbols as baseline. [2] On the other hand, unaligned UL subband brings intra-subband UE-to-UE CLI and gNB-to-gNB CLI, which certainly degrade system performance.

Proposal 9: Cell-specific RRC signalling is sufficient to configure SBFD subband frequency location.

UE behaviour on SBFD symbols

In RAN1#111 meeting, UE behaviour on SBFD symbols was heatedly discussed. Regarding UE behavior on semi-static DL symbols, i.e. UL subband is configured on semi-static DL symbol, the only open issue is whether to allow DL reception within UL subband for SBFD aware UE. While situation is much more controversial for UE behaviour on semi-static flexible symbol, there are two directions in summary and the detail description can be found in introduction section:

- Option 1: Same behaviour as DL symbol, i.e. UL transmission is confined in UL subband while whether DL reception is allowed in UL subband depends on further discussion.

- Option 2: Basic idea is allowing UL transmission outside UL subband so that the flexibility of semi-static flexible symbol is sustained.

Option 1 guarantees the same UE behaviour for SBFD aware UE across different OFDM symbol type. To be specific, UE behaviour in SBFD symbol configured with DL symbol and in SBFD symbol configured with semi-static flexible symbol is exactly same. It significantly cuts down workload from standardization perspective. More importantly, option 1 is helpful to reduce complexity for SBFD aware UE as it doesn’t need to implement different solutions on different SFBD symbol. Correspondingly, it can also alleviate burden at gNB side as it is much easier to schedule UE with simple UE behaviour. Option 1 would expedite the commercialization thanks to its simplication and coherency compared to option 2.

On the other hand, option 2 defines different UE behaviour compared with that on DL symbols. It brings additional complexity on both UE side and gNB side. Major motivation of introducing option 2 is flexible symbol can be scheduled or configured any transmission up to gNB in nature.  However, it is quite questionable that option 2 can bring any flexibility in reality. Compared with option 2, if flexibility does matters and gNB wants to sustain flexibility, gNB can avoid configuring UL subband on flexible symbols at the very beginning. From this perspective, it is meaningless to define some new behaviour on flexible symbol to accommodate flexibility, while full flexibility is already achieved by flexible symbol in fact.

Proposal 10: For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, the following behaviours are applicable for SBFD aware UEs,

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

- DL receptions within DL subband(s) are allowed in the symbol

- FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol

Based on agreements achieved in RAN1#111 meeting and the analyses provided in this section, the following UE behaviours can be concluded:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- DL receptions within DL subband(s) are allowed in the symbol

However, whether DL receptions outside DL subband(s) are allowed or not in the symbol is still open. Opponents think UL subband is configured for UL transmission intentionally and DL reception should not be allowed within a UL subband. However, UL subband is configured in DL symbol or flexible symbol on which any RB can be used for DL transmission in nature. If DL reception is not allowed within UL subband, it means the DL performance of Rel-18 SBFD aware UE is much worse than that of a Rel-15/16/17 UE. It doesn’t make sense that an advanced UE experience much worse performance.

One concern for allowing DL reception within UL subband is that it additionally introduces Intra-subband gNB-to-gNB CLI among gNBs. Actually we encounter the same issue for dynamic TDD and the mechanisms managing intra-subband CLI can be reused. Regarding to collision between DL transmission and UL transmission within UL subband, it can be properly handled by gNB.

Proposal 11: DL reception is allowed within UL subband in SBFD symbol.

As mentioned in section 2.1, guard band between DL subband and UL subband can be implicitly determined according to requirement defined in RAN4. The functionality of guard band is to provide sufficient frequency isolation between DL transmission and UL transmission so as to guarantee transmission performance. From gNB perspective, guard band is not needed if there is no UL transmission to be protected. In this case, guard band can be used for DL reception. gNB has full power to decide how to schedule/configure UL transmission and DL reception, it should be totally gNB’s decision.

Proposal 12: DL reception is allowed within guard band in SBFD symbol.

Transmission or reception across normal symbol and SBFD symbol

In RAN1#111 meeting, we discussed the potential issues on transmission/reception across non-SBFD symbol and SBFD symbol. The key point is that the available frequency resources in different slot type, i.e. non-SBFD symbol and SBFD symbol, are different. As a starting point, the following agreement was achieved:

Agreement Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following: - PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols - Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols - Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition - Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols - Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered. Examples of potential enhancements include: - Resource allocation in frequency domain including frequency hopping - Resource allocation in time domain - Power domain - Spatial domain FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured.

Firstly, it should be noticed that whether we need to introduce new mechanism to handle the case wherein DL receptions are across SBFD symbols and non-SBFD symbols highly depends on UE behaviour. For example, if DL reception is allowed within UL subband, there would be no issue for DL recception, even it is across SBFD symbols and non-SBFD symbols.

Observation 8: For SBFD aware UE, whether enhancement is needed for DL receptions across SBFD symbols and non-SBFD symbols depends UE behaviour discussion.

In the following part, we assume that DL reception is not allowed within UL subband for generic discussion.

For dynamically scheduled downlink channel with or without repetition, the starting symbol and duration is fully controlled by gNB via TDRA indication and/or higher layer configured periodicity. Even UL subband is configured on partial DL symbols within a DL slot, gNB can avoid different frequency resource allocation on different symbols types via proper scheduling strategy. An example is shown in Figure 9 and Figure 10, assuming UL subband occupies symbol#3 – symbol#13 on DL slot#0 and slot#1 respectively. Three strategies can be used, which totally follows current specification:

- PDSCH is scheduled on non-SBFD symbols only

- PDSCH is scheduled on SBFD symbols only

- PDSCH is scheduled across SBFD symbols and non-SBFD symbols

Figure 9 Examples for PDSCH w/o repetition in SBFD slot containing SBFD symbol and non-SBFD symbol

Figure 10 Example for PDSCH w/ repetition across SBFD symbols and non-SBFD symbols

Besides, PDSCH RA type0 and RMR can be enployed to address uncontinuous frequency resource issue on SBFD symbols.

Observation 9: For DG PDSCH with or without repetition, current specification is sufficient for DL receptions across SBFD symbols and non-SBFD symbols.

For PUSCH, RBs belonging to active UL BWP are available in UL symbol while only RBs belonging to UL subband are available in SBFD symbol. Different from DL resources, UL resources either on UL symbol or SBFD symbol are continous. The issue comes from the number of available UL RBs on different OFDM symbol is different. Consequently, single FDRA indication may not be suitable once PUSCH is allocated with UL symbol and SBFD symbol. If frequency hopping is enabled, the issue is even more severe as PUSCH hop may exceed UL subband due to the more restricted available UL resources.

Figure 11examples for PUSCH transmission across SBFD symbol and non-SBFD symbol

PUSCH can be scheduled by UL grant, or be activated by an activation DCI, or be configured via CG configuration. No matter how a PUSCH is indicated, we believe current specification is sufficient to avoid the case where a PUSCH is across SBFD symbol and non-SBFD symbol if repetition is not enabled.

Observation 10: For PUSCH without repetition, current specification is sufficient to avoid the case where a PUSCH is across SBFD symbol and non-SBFD symbol.

Regarding to PUSCH repetition without frequency hopping, same resource allocation in frequency domain for different repetitions can be achieved up to implementation. At least the following several options can be considered:

- PUSCH repetition is scheduled on UL slot only, depending whether there are continuous UL slots

- PUSCH repetition is scheduled on SBFD slot only

- PUSCH repetition is scheduled on UL slot and SBFD slot, with confining frequency resources in UL subband

- PUSCH repetition is scheduled on UL slot and SBFD slot, the frequency allocation on UL slot and SBFD slot can be different

For the last mechanism, enhanced FDRA for PUSCH is required. However, the motivation and benefits are unclear.

Regarding to PUSCH repetition with frequency hopping, it is too restrictive if gNB has to always guarantee two PUSCH hops are located within UL subband. Currently, a frequency hopping offset list is configured for determining each PUSCH hop. In order to avoid PUSCH hop is out of UL subband, conservative scheduling decision is the only choice at gNB side. It obviously degrades potential performance gain harvested from frequency hopping. Similar to PUSCH repetition without frequency hopping, the following several options can be considered:

- PUSCH repetition is scheduled on UL slot only, depending whether there are continuous UL slots

- PUSCH repetition is scheduled on SBFD slot only

- PUSCH repetition is scheduled on UL slot and SBFD slot, and gNB guarantees PUSCH hop on SBFD slot doesn’t exceed UL subband with the same frequency hopping offset list

- PUSCH repetition is scheduled on UL slot and SBFD slot, separate frequency offsets list are applied to UL slot and SBFD slot respectively.

For the last mechanism, additional consideration on how to use single FH offset indication to indicate proper FH offset for UL slot and SBFD slot respectively is needed.

Regarding whether it is possible to map PUSCH on SBFD symbol and non-SBFD within slot, it is possible for special slot consists of flexible symbol and UL symbol. However, there is no strong motivation as in this case several flexible symbols can be used for uplink transmission based on scheduling.

Proposal 13: For PUSCH repetition without frequency hopping, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- PUSCH repetition is mapped on UL slot only

- PUSCH repetition is mapped on SBFD slot only

- PUSCH repetition is scheduled on UL slot and SBFD slot with confining frequency resources in UL subband

- PUSCH repetition is scheduled on UL slot and SBFD slot, the frequency allocation on UL slot and SBFD slot can be different

Proposal 14: For PUSCH repetition with frequency hopping, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- PUSCH repetition is mapped on UL slot only

- PUSCH repetition is mapped on SBFD slot only

- PUSCH repetition is scheduled on UL slot and SBFD slot, same frequency hopping offset list is applied

- PUSCH repetition is scheduled on UL slot and SBFD slot, separate frequency offsets list are applied to UL slot and SBFD slot respectively

PUCCH can be transmitted with periodic manner or aperiodic manner. The time and frequency resource for PUCCH transmission is indicated by PUCCH resource indicator and/or PUCCH-config. Furthermore, PUCCH repetition is supported, which is indicated either via DCI or RRC signaling.

For a DCI indicated PUCCH transmission without repetition, gNB can indicate a PUCCH resource anywhere within UL subband based on pre-configured PUCCH resource list. Considering the available bandwidth for uplink transmission is quite different on UL slot and SBFD slot, gNB has to configure at least one PUCCH resource which fits into UL subband. As there are at least 8 PUCCH resources can be configured for a PUCCH resource set, it should be sufficient to reuse current mechanism to indicate the PUCCH resource on UL slot or on SBFD slot.

For a periodic PUCCH transmission, its time/frequency resource is totally configured by RRC signaling. The minimum periodicity is 4 slots and the offset within periodicity can also be configured. Theoritically, periodic PUCCH transmission can be always allocated on UL slot or SBFD slot. Even if different repetitions are located on both SBFD slot and UL slot, there is no issue if a proper configuration is provided, e.g. the assigned PUCCH resource is within UL subband in frequency domain. From this perspective, the current mechanism to configure periodic PUCCH transmission can be reused when UL subband is configured on some OFDM symbols.

Observation 11: For PUCCH without repetition, current specification is sufficient for avoiding PUCCH transmission across SBFD symbols and non-SBFD symbols.

For PUCCH transmission with repetition, it is quite similar to PUSCH. Accordingly, we have the following proposals:

Proposal 15: For PUCCH repetition without frequency hopping, the following mechanisms can be considered for PUCCH across SBFD slot and non-SBFD slot:

- PUCCH repetition is mapped on UL slot only

- PUCCH repetition is mapped on SBFD slot only

- PUCCH repetition is indicated on UL slot and SBFD slot with confining frequency resources in UL subband

- PUCCH repetition is indicated on UL slot and SBFD slot, the frequency allocation on UL slot and SBFD slot can be different

Proposal 16: For PUCCH repetition with frequency hopping, the following mechanisms can be considered for PUCCH across SBFD slot and non-SBFD slot

- PUCCH repetition is mapped on UL slot only

- PUCCH repetition is mapped on SBFD slot only

- PUCCH repetition is indicated on UL slot and SBFD slot, same frequency hopping offset list is applied

- PUCCH repetition is indicated on UL slot and SBFD slot, separate frequency offsets list are applied to UL slot and SBFD slot respectively

If TBoMS is enabled for PUSCH transmission, it will be transmitted on N available slots as defined in TS38.214. However, different from PUSCH repetition, a single TB is transmitted across N slots. The TBS is determined not only by the time/frequency within a slot but also the number of N slots. The procedure of determining NRE for a PUSCH with TBoMS is excerpted as below. [4].  Obviously, the allocated RB across different slot should be same. For TBoMS PUSCH across SBFD slot and non-SBFD slot, gNB should guarantee the same time/frequency resource allocation is applied.

TS38.214, section 6.1.4.2 - A UE determines the total number of REs allocated for PUSCH as follows - For TB processing over multiple slots, NRE=N*min156,NRE'⋅nPRB where is the total number of allocated PRBs for the UE and N is the number of slots used for TBS determination indicated by numberOfSlotsTBoMS. - Otherwise, NRE=min156,NRE'⋅nPRB.

Proposal 17: For PUSCH with TBoMS, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- PUSCH repetition is mapped on UL slot only

- PUSCH repetition is mapped on SBFD slot only

- PUSCH repetition is indicated on UL slot and SBFD slot with confining frequency resources in UL subband

For multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols, same frequency resources are indicated for different PUSCH transmission on different slots. Although the TBs carried by different PUSCH are different, the resource allocation is quite similar to the case of PUSCH/PDSCH repetition. Accordingly, the same mechanisms as PDSCH and PUSCH without inter-slot frequency hopping can be considered. For multi-PDSCH scheduled by a single DCI, we don’t see any issue. On the other hand, we have the following proposal for multi-PUSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols.

Proposal 18: For multi-PUSCH scheduled by a single DCI, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- All PUSCHs are mapped on UL slot only

- All PUSCHs are mapped on SBFD slot only

- Multi-PUSCH is scheduled on UL slot and SBFD slot with confining frequency resources in UL subband

- Multi-PUSCH is scheduled on UL slot and SBFD slot, the frequency allocation on UL slot and SBFD slot can be different

PDCCH search space is configured in a semi-static manner and transmitted periodically. It is possible that search space occurs on SBFD symbols or non-SBFD symbols on different monitoring occasion. As the frequency resource of CORESET is configured via a bitmap with 6 RB granularity, gNB can avoid collision between UL subband and CORESET in frequency domain with proper configuraiton.

Observation 12: For PDCCH, it is up to gNB to avoid collision between UL subband and CORESET in frequency domain.

Overlapping between DL reception and UL subband in frequency domain

Active DL BWP consists of contiguous RBs in frequency domain. However, there will be two segmentations of DL resources in frequency domain if a UL subband is configured in between. Several issues were identified and further study is required, including:

- RBG for PDSCH RA type 0

- CSI reporting configuration

- CSI-RS resource configuration

- PRG of PDSCH

First of all, whether frequency enhancement for DL transmission/reception is needed highly depends on UE behaviour discussion. Although it is still open on whether DL transmission is needed within UL subband, we assume RB belongs to UL subband is unavailable for DL in the following discussion.

PDSCH resource allocation

Considering unaligned boundaries between resource block group(s) and SBFD subbands, further enhancement is needed to improve spectrum efficiency.  In order to utilize the RBs belongs to a RBG which partially overlaps with UL subband or guard band if configured, the following three mechanisms can be considered:

- Option 1: Leave it to gNB implementation. gNB configures a UL subband which has aligned boundary with RBG. If the guranularity of UL subband is one RB, there is no problem to configure guarantee boundaries between RBG and UL subband are aligned.

Figure 12 Aligned boundaries between RBG and UL subband

- Option 2: RBG is mapped to the RBs not contained by UL subband only. UL subband RBs are treated as unavailable and should be excluded when determining the DL resource assignment. Consequently, the RBG at DL subband boundary may be across two DL subbands.

Figure 13 RBG is allowed to mapped to uncontiguous RBs

Option 1 is simpler as it is up to proper gNB configuration. However, it may not be suitable for FDRA type 1 for which contiguous DL resources is assumed, no matter VRB-to-PRB is enabled or not. On the other hand, option 2 is a unified solution for different RA type, which is slightly preferred.

Proposal 19: For PDSCH frequency resource allocation on SBFD symbols, the following mechanisms can be considered:

- Option 1: UL subband boundary is aligned with that of RBG

- Option 2: Only RBs not contained by UL subband is available for DL transmission in a RBG

- Option 3: RBG is mapped to the RBs not contained by UL subband only.

CSI reporting configuration

CSI reporting is carried by PUSCH or PUCCH, i.e. aperiodic CSI report is carried by PUSCH which is scheduled by DCI and periodic CSI report is carried by PUCCH which is configured by PUCCH. It should be discussed under the umbrella of PUCCH and PUSCH. We don’t see the necessity of separately discussing CSI reporting configuration.

CSI-RS resource configuration

A CSI-RS resource has to contain multiple of 4 RBs. The smallest configurable number is the minimum of 24 and the width of the associated BWP. Hence, we need to consider the impacts on CSI-RS resource configuration if it is associated with a SBFD symbol. For UL subband with {DU} or {UD} pattern, we believe there is no issue as the frequency resources of DL subband is consecutive and the boundaries of DL subband and CSI-RS subband can be aligned with each other. For {DUD} SBFD subband pattern, following options have been raised so far：

- Option 1: Two CSI-RS resources are configured

- Option 2: Non-contiguous CSI-RS resource via explicit or implicit mechanism is allowed

- Option 3: CSI-RS resource is allowed to overlap with UL subband and guardband(s), the corresponding CSI reporting is not required

Option 1 consumes the limited number of CSI-RS resources, which may have negative impacts on the CSI-RS measurement and reporting on legacy slot. Option 3 makes it impossible to report CSI based on the CSI-RS located on SBFD symbol once the CSI-RS resource is partially overlaps with UL subband and guardbands. Option 2 makes it possible for CSI measurement on SBFD slot. It can also resolve the problem that partial RBs cannot be used at DL subband boundary. Hence we slightly prefer option 2.

Proposal 20: For CSI-RS resource configuration on SBFD slot, non-contiguous CSI-RS resource configuration via explicit or implicit mechanism should be further studied.

PRG of PDSCH

There are three sizes of PRG, i.e. {2,4, wideband}, which can be semi-statically configured or dynamically indicated. It is possible that the PRG located at DL subband boundary is partially overlaps with UL subband or guard band if exists. Therefore, we need to consider how to define PRG at DL subband boundary. Considering the boundaries among PRB, RBG and VRB group is aligned with each other, the potential enhancement for PDSCH resource allocation can also resolve the issues relevant to PRB as well. We have the following proposal accordingly.

Proposal 21: For PRG determination on SBFD symbols, the following mechanisms can be considered:

- Option 1: UL subband boundary is aligned with that of PRG

- Option 2: For PRG partially overlapped with UL subband or guard band, the actual PRG is determined by the left RBs

- Option 3: PBG is determined on the RBs not contained by UL subband and guard band only.

CORESET on SBFD symbols

TBD

Conflict of transmission direction on SFBD symbols

SBFD aware UE is only capable of half duplex, i.e. it supports either uplink transmission or downlink reception at one time. Accordingly, gNB can schedule/configure a SBFD aware UE to transmit uplink in UL subband or receive DL in DL subband. Considering UE can only transmit uplink or receive downlink at one time, we should consider how to handle conflicts between UL and DL. There are several potential conflicting cases on SBFD slot for a SBFD aware UE, which are listed as below:

- Type 1: Dynamic DL on SBFD symbol conflicts with Dynamic UL on the UL subband.

- Type 2: Semi-static DL on SBFD symbol conflicts with Dynamic UL on the UL subband.

  - The semi-static DL includes periodic reference signal, semi-persistent reference signal, SPS PDSCH without DL assignment, etc.

- Type 3: Broadcast DL on SBFD symbol conflicts with UL on the UL subband.

  - The broadcast DL includes Type0/0A/1/2-PDCCH CSS and SSB

- Type 4: Dynamic DL on SBFD symbol conflicts with semi-static UL on the UL subband, including CG PUSCH, periodic uplink reference signal and periodic PUCCH.

- Type 5: Semi-static DL on SBFD symbol conflicts with semi-static UL on the UL subband.

Theoretically, gNB has full power to avoid collision between dynamic DL and dynamic UL on the same symbol. Therefore, it may be a reasonable hypothesis that UE doesn’t expect gNB dynamically schedules uplink and downlink at the same time. On the other hand, dynamic downlink channel and dynamic uplink channel may have different priority, i.e. one is associated with eMBB while the other is associated with URLLC. gNB may firstly schedule a PUSCH or PDSCH carrying eMBB traffic and later schedule schedule a PDSCH or PUSCH carrying URLLC on the same symbol. In this sense, it make sense to consider collision between dynamic downlink and dynamic uplink with different priority respectively.

Observation 13: The conflicted dynamic downlink and dynamic uplink on the same SBFD symbol may have different priorities.

For type 2 collision, gNB may transmits semi-static downlink channel or reference signal on the DL subband(s) whilst gNB may schedule SBFD aware UE to transmit uplink channel or reference signal on the UL subband. One straightforward mechanism is to make dynamic scheduling deserve higher priority, which is the generic principle since Rel-15. However, the aforementioned dynamic channel and the semi-static channel may have different priorities. For example, gNB schedules a PUSCH with R=4 while the SPS periodicity equates two, which can be explained in Figure 14. On slot#0, SPS PDSCH collides with PUSCH repetition. If SPS PDSCH is configured with a high priority, i.e. its harq-CodebookID points to the HARQ codebook with high priority, it is hard to say the dynamic PUSCH should have high priority.

Figure 14 Example of collision between semi-static DL and dynamic UL

Observation 14: Semi-static DL and dynamic UL may collide on SBFD symbol.

For type 3 collision, gNB transmits broadcast channels, i.e. type 0/0A/0B/1/2-PDCCH CSS and SSB, while UE is required to transmit uplink channel or signal on the same SBFD symbol. Both type 0/0A/0B/1/2-PDCCH CSS and SSB target for group of UEs, including SBFD aware UE and non-SBFD aware UE. Their configuration and transmission should not be impacted by SBFD operation. On the other hand, gNB may or may not schedule UL transmission within UL subband in a SBFD slot containing aforementioned DL broadcast channels. There is a concern that intra-cell UE-to-UE CLI may jeopardize DL performance, particularly SSB is fundamental for system. Actually SSB is quite robust in nature, with low coding rate, robust physical structure, special RS design, etc. Even in the legacy network, SSB suffers intra-subband gNB-to-UE interference from neighbouring cell. Intra-gNB UE-to-UE CLI may be not as significant as legacy gNB-to-UE interference. Furthermore, frequency gap between SSB and UL transmission within UL subband can provide additional protection for SSB.

Observation 15: The impacts on SSB/CSS from UL transmission within UL subband may not always be an issue.

For type 4 collision, it is similar to type 2 collision, i.e. gNB may transmits dynamic downlink channel or reference signal on the DL subband(s) whilst inidcate SBFD aware UE to transmit semi-static uplink channel or reference signal on the UL subband.

For type 5 collision, we need to discuss the semi-static DL transmission and semi-staitc UL transmission case by case. We summarize the procedure of how to determine the time location of each semi-static channel or reference signal as below:

Table 1  Determination of time domain resource for semi-static channel and reference signal

| Semi-static channel/signal | Procedure of determining time location for each transmission |
| --- | --- |
| Type-1 CG PUSCH | The periodicity, time reference SFN, time offset and time domain resource allocation are configured by RRC signaling. |
| Type-2 CG PUSCH | The priodicity of CG PUSCH occasion is provided by RRC signaling. The time domain resource allocation within a slot is provided via activation DCI. |
| Periodic PUCCH | PUCCH resource, periodicity and offset are configured by RRC signaling. |
| Periodic SRS | SRS resource, periodicity and offset are configured by RRC signaling. |
| Semi-persistent SRS | SP SRS is triggered via MAC CE. Its periodicity and offset are configured by RRC signaling. |
| SPS PDSCH | SPS PDSCH is activated via DCI format with CRC scrambled by CS-RNTI. The periodicity is configured via RRC signaling. |
| Periodic CSI RS | The periodicity and offset for NZP CSI-RS and ZP CSI-RS are configured via RRC signaling. |
| Semi-persistent CSI RS | Both SP ZP CSI RS and SP NZP CSI RS are triggered by MAC CE, i.e. with individual MAC CE respectively. The periodicity and offset are configured via RRC signaling. |

Generally speaking, gNB can avoid collision between semi-static uplink and semi-static downlink via proper configuration. On the other hand, the additional consideration on avoiding collision between DL and UL may degrade the performance for one or another. For example, Type-1 CG PUSCH carrying URLLC traffic with high priority and SPS PDSCH are configured for UE simultaneous. If gNB has to guarantee there is no overlapping between SPS PDSCH and CG PUSCH in time domain, it introduces restrictions on the configuration for both configuration, which is not expected.

Observation 16: The conflict between semi-static DL and semi-static UL may occur.

Based on the aforementioned analyses, we have the following proposal:

Proposal 22: Further study whether the following cases are allowed or not:

- Type 1: Dynamic DL on SBFD symbol conflicts with Dynamic UL on the UL subband.

- Type 2: Semi-static DL on SBFD symbol conflicts with Dynamic UL on the UL subband.

- Type 3: Broadcast DL on SBFD symbol conflicts with UL on the UL subband.

- Type 4: Dynamic DL on SBFD symbol conflicts with semi-static UL on the UL subband, including CG PUSCH, periodic uplink reference signal and periodic PUCCH.

- Type 5: Semi-static DL on SBFD symbol conflicts with semi-static UL on the UL subband.

Mechanisms of realizing SBFD operation

In RAN1#110bis e-meeting, we discussed SBFD operation with achieving the following agreement:

Agreement For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline.

Additionally, another potential solution for SBFD operation within a TDD carrier is BWP-based SBFD. The basic idea is to introduce a UE capability of simultaneously supporting multiple active BWP pairs. However, it would be a stringent requirement on UE capability which needs careful and comprehensive study. Secondly, it will introduce BWP switching delay inevitable. The delay comes from switching between the normal DL BWP and the duplex-DL BWP, vice versa. One example is shown as below. The delay components need further study as it is different from DL-DL BWP switching or UL-UL BWP switching. At least it needs more time on transmitter and receiver transition additionally.

Figure 15 Delay of BWP based SBFD

Last but not least, one basic idea behind BWP based SBFD is to configure TDD UL DL configuration per BWP. It is a fundamental change on TDD UL DL configuration as currently it is configured per cell. UE firstly determines the slot type and direction, i.e. DL, UL or flexible, then it determines UL or DL BWP is applied. If TDD UL-DL configuration is configured per BWP, the standard impacts may be uncontrollable.

Based on the aforementioned analyses, BWP-based SBFD operation should be precluded:

Proposal 23: BWP-based SBFD operation is not supported in Rel-18 duplex enhancement.

Beyond SBFD operation within a TDD carrier, it was also discussed that SBFD operation across carriers, called HDCA-SBFD. The nature of HDCA-SBFD is to regard carrier as subband and allow directional conflicts between carriers within the same band. However, we note that there is nothing about HDCA-SBFD in the SID of Rel-18 duplex enhancement. From this perspective, it is out of scope. On the other hand, we don’t see any bonus compared to single carrier mechanism.  Furthermore, we need to define new UE behaviour for supporting HDCA-SBFD, e.g. currently it is not allowed different directions on carriers within the same band. We may need to study/define the UE behaviour case by case, e.g. Semi-SFI+Semi-SFI, Semi-SFI+Dynamic SFI, Dynamic SFI + Semi-SFI, Dynamic SFI + Dynamic SFI. It will introduce jumbo standard workload.  Furthermore, the carrier used as UL subband basically has an UL dominated TDD-UL-DL configuration. Considering the carrier is also configured to the other UEs, including legacy UE, it will introduce improper TDD configuration for the non-target UE.

Proposal 24: Half duplex CA based SBFD operation is not supported in Rel-18 duplex enhancement.

DL-to-UL switching and TA determination

For unpaired spectrum, guard period is needed between DL region and UL region. Basically, the guard period is expected to provide sufficient time between DL region and UL region for DL-to-UL switching and ICI interference avoidance. For example, UE needs time to conduct DL-to-UL switching and TA adjustment. Currently the guard period is covered by flexible slot and the length of guard period is up to implementation, i.e. a value not smaller than NTA+NTA,offset. Similarly, guard period is also needed between the last OFDM symbol in DL region and the first OFDM symbol in UL subband. One example is shown in Figure 16, wherein DL-to-UL switching point between DL slot and UL subband is needed in addition to the legacy guard period. The additional DL-to-UL switching point can be configured explicitly or implicitly. However, the overhead should also be carefully evaluated as the guard period cannot be used for transmission and reception. Additional DL-to-UL switching point would reduce the spectrum utilization.

Figure 16 Examples of DL to UL switching point between DL and UL subband

For legacy TDD, uplink frame number  for transmission from the UE shall start TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc before the start of the corresponding downlink frame at the UE. The purpose is to achieve an aligned TDD timing at gNB side and provide sufficient time for Rx-to-Tx switching at gNB side.[5] The Uplink-downlink time relation can be explained by Figure 17. As there is always flexible symbol between DL symbol and UL symbol, inter-slot interference caused by time advance at UE side can be avoided and Rx-to-Tx switching gap NRx-TxTc can be guaranteed.

Figure 17 Uplink-downlink timing relation

For SBFD aware UE, UL subband is configured in DL slots. An example is shown in Figure 18, wherein assuming UL subband is configured in DL slot#2 to DL slot#5. There is a back-to-back transmission between DL slot#2 and UL subband in slot#3. As a timing advance is applied to UE, i.e. TTA defined in TS38.211, UE needs to transmit uplink in advance with TTA relative to its DL timing. As there is no guard period between DL symbol and UL subband, we have the following issues can be observed:

- Issue#1: UE needs to transmit uplink on the resources potentially scheduled with downlink, i.e. there is collision between DL and UL for SBFD UE. Considering UE only half duplex capability, gNB has to avoid such kind of collision via scheduling. In the other words, some resources are unavailable for DL reception or UL transmission, which degrades system performance.

- Issue#2: If gNB schedules uplink transmission on the conflicting symbols, gNB suffers intra-subband self-interference as gNB needs to schedule DL for the other UEs. This case is out of scope as Rel-18 only study SBFD operation, instead of SSFD operation.

- Issue#3: The uplink transmission on the conflicting resources introduces intra-subband CLI for the other UEs belonging to the same serving cell. It will significantly degrade legacy UE’s DL performance, which is not preferred.

Figure 18 Timing alignment due to back-to-back DL and UL

Some companies propose to configure a zero value for NTA,offset. Theoretically, NTA is used to compensate propagation delay between UE and serving cell while NTA,offset is used to provide sufficient gap for Rx-to-Tx switching at gNB side. For SBFD gNB, it is capable to receive and transmit at the same time. From this perspective, gNB doesn’t need switching time for Rx-to-Tx and Tx-to-Rx. An example with NTA,offset = 0 is shown in Figure 19 as below.

Figure 19 Timing of UL transmission assuming NTA,offset = 0

However, as UE still needs to transmit uplink within UL subband with NTA×Tc in advance, intra-subband UE-to-UE CLI to another UE belonging to the same serving cell is inevitable. It is same as issue#3 for fully reusing the mechanism determing TTA as analyzed above. The last UL symbol suffers DL interference from neighbouring gNB because of synchronization error among gNBs. Most importantly, the last DL symbol in the DL region cannot be used for the UE as UE cannot transmit and receive at the same time. From this perspective, we don’t think simply setting NTA,offset = 0 really resolves the issue.

Figure 20 ISI introduced by zero NTA,offset

Another solution proposed in the last meeting is only setting NTA,offset = 0 for SBFD symbol. Although it can avoid DL interference to UL symbol due to early DL transmission from neighbouring gNB, it doesn’t resolve issue#1 and issue#2 as well. In essence, TA is introduced to compensate the bi-derection propagation delay between gNB and UE, and provide sufficient guard period for gNB to conduct Rx-to-Tx switching. There is no reason to define two TA values for an individual UE.

Observation 17: TA enhancement is not necessary as it doesn’t resolve inter-slot interference issue.

Currently, flexible symbols needs to be configured between DL region and UL region. Thanks to the flexible symbols in between, NTA,offset and NTA can be employed to guarantee aligned timing at gNB side and provide sufficient gap for Rx-to-Tx switching. Hence the key point is a guard period must be provided between the last DL symbol in DL slot and the first UL symbol in UL subband. At least the followig mechanisms can be considered in the further study:

- Option 1: The guard period is configured by gNB for SBFD aware UE.

- Option 2: UE does not expect to transmit in the uplink on the first G symbols within UL subband.

- Option 3: The first OFDM symbol of UL subband is always adjacent UL symbol or flexible symbol.

Option 3 is the simplest mechanism and can fully enjoy the bonus of semi-static flexible OFDM symbols. No or trivial standard impact is expected. One example is shown in Figure 21. UL subband is configured right behand semi-static UL symbol, i.e. UL region is contiguous from SBFD aware UE point of view. The flexible symbols can be used for the guard period between DL region and UL region. Unified TA value, i.e. determined by the current mechanism, is applied to SBFD symbol and UL symbol. Hence we slightly prefer option 3.

Figure 21 UL subband is adjacent to UL symbol

Proposal 25: Guard period between DL region and UL subband is needed and at least the following mechanism can be considered.

- Option 1: The guard period is configured by gNB for SBFD aware UE.

- Option 2: UE does not expect to transmit in the uplink on the first G symbols within UL subband.

- Option 3: The first OFMD symbol of UL subband is always adjacent UL symbol or flexible symbol.

Interference management for SBFD

Self-interference handling

On SBFD symbol, gNB can transmit in DL subband and receive in UL subband simultaneously. Therefore, gNB suffers inter-subband self-interference in SBFD slot, which would affect UL performance. One concern raised by companies is that the unaligned symbol/slot boundary at gNB side may deteriorate performance of digital interference cancellation. However, self-interference can be suppressed by spatial isolation, frequency isolation, beam nulling /isolation, RF domain mitigation and digital interference cancellation according to the reply from RAN4. [3] The value range of RSIC provided by RAN4 is shown in Table 2.  It can be seen that digital IC is not a solid mechanism to suppress self-interference and even RAN4 thinks that we may not obtain any bonus from digital domain. Even digital IC is not available, we still can harvest sufficient isolation from the other aspects. Actually how much isolation is needed to make SBFD operation feasible depends on the SLS evaluation results. If RSIC capability provided by spatial isolation, frequency isolation and beam nulling/isolation is sufficient, digital IC may not be something has to be present. On the other hand, even the SBFD symbol and DL symbol is not aligned at gNB side, the performance of digital IC can still be guaranteed as gNB has the information of NTA,offset.

Table 2 value range of RSIC

| Parameter | FR1(Frequency Range 1) | FR2(Frequency Range 2) |
| --- | --- | --- |
| Spatial isolation | 50~80dBc | 80-120 dBc |
| Frequency isolation | 45 dBc | 22.5~30 dBc |
| Beam nulling /isolation | 0~40 dBc | 0~40 dBc |
| Digital IC | 0~50 dBc | 0~50 dBc |
| Overall RSIC capability | 95 ~185 dBc | 102.5~ 205 dBc |
| NOTE1: Other isolation schemes could be discussed further. NOTE 2: Both transmitter leakage to the RX sub-band and interference arising from receiver imperfections need to be considered. Receiver imperfections may reduce the RSIC to be lower than the RSIC considering transmitter leakage alone. RAN4 will assess impact of Rx impairments on the RSIC capability. But the RSIC model can potentially be simplified to address impact from both aspects together. |  |  |

Observation 18: The impacts on digital IC from unaligned symbol/slot boundary at gNB side needs further study.

Interference handling at UE

The UE-UE CLI measurement has been studied in Rel-16, which can be used to obtain the RSSI and RSRP of intra-subband UE-UE CLI. However, considering that power of UL signal leaked to DL subband is non-uniform with the assumption of in-band emissions, strength of inter-subband UE-UE CLI differs in frequency, which is shown in Figure 21. It is not suitable to measure the UE-UE CLI in the whole DL subband and report the average RSSI of inter-subband UE-UE CLI. Some enhancements would be needed to obtain accurate inter-subband UE-UE CLI strength in frequency. Similar to subband based CSI measurement and report, subband based inter-subband UE-UE CLI measurement and report could be considered.

Figure 22 IBE model at UE sideProposal 26: Subband based inter-subband UE-UE CLI measurement and report can be discussed in AI 9.3.2.

Interference handling at gNB

Similar to inter-subband UE-UE CLI, inter-subband gNB-gNB CLI is also non-uniform in frequency, which is shown in Figure 22. Thus, it is preferred to apply subband based inter-subband gNB-gNB CLI measurement and information exchange to represent more accurate strength of gNB-gNB CLI in the frequency domain.

Figure 23 Interference leakage at gNB sideProposal 27: Subband based inter-subband UE-UE CLI measurement and information exchange can be discussed in AI 9.3.2.

- 2.10 Others

SBFD operation in UL symbols

In RAN#96 plenary meeting, it was concluded that ‘UL symbol as 2nd priority is accepted, no intended suspension of continuation of work in WGs’. From our understanding, the intention of the conclusion is to deprioritize the discussion on whether to support DL subband on UL symbols. When we finish the discussion or the design of UL subband on DL symbols becomes mature, we can discuss whether to support DL subband on UL symbols.

As discussed during preparation phase of Rel-18 NR duplex operation, the motivation of supporting full duplex within a slot is to enhance the uplink transmission, in terms of coverage, latency and capacity. It is also captured in the justification section of SID, which is excerpted as below:

TDD is widely used in commercial NR deployments. In TDD, the time domain resource is split between downlink and uplink. Allocation of a limited time duration for the uplink in TDD would result in reduced coverage, increased latency and reduced capacity. As a possible enhancement on this limitation of the conventional TDD operation, it would be worth studying the feasibility of allowing the simultaneous existence of downlink and uplink, a.k.a. full duplex, or more specifically, subband non-overlapping full duplex at the gNB side within a conventional TDD band. < -----omitted text----> This study aims to identify the feasibility and solutions of duplex evolution in the areas outlined above to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operations in unpaired spectrum. In addition, the regulatory aspects need to be examined for deploying identified duplex enhancements in TDD unpaired spectrum considering potential constraints.

Based on previous discussion and the SID, the common understanding on the type of subband seems to be UL subband, which exists in DL slot and is used for uplink transmission. On the other hand, the motivation of configuring/indicating a DL subband in an uplink slot is unclear.  Considering uplink subband is already on the table, a DL dominated TDD UL-DL configuration is feasible which can certainly guarantee DL performance.  More importantly, configuring/indicating DL subband in an uplink slot has significant impacts on legacy UEs, which is not desired. One example is shown in Figure1 and the interference from UL/DL subband is summarized as below:

- For case a), a UL subband is configured or indicated in a DL slot. For a legacy UE which receives downlink channel or signal, it suffers uplink interference from other UEs in the same cell.

- For case b), a DL subband is configured or indicated in a UL slot. For a legacy UE which transmits uplink channel or signal, gNB suffers downlink interference from cellist own transmitter.

From perspective of legacy UE, the DL interference from same serving cell is much stronger than UL interference from other UE. In the other words, DL subband in a UL slot introduces more significant impacts for legacy UE, which needs carefully study.

Figure 24 Examples of subband type for duplex operation

Proposal 28: For subband non-overlapping full duplex, it cannot be applied to UL symbols.

Valid RO determination on SBFD symbols

Currently, the time-frequency resources used for RACH is configured by SIB1. Additionally, UE determines which ROs are valid as below in TS38.213:

For unpaired spectrum, - if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last SS/PBCH block reception symbol, where Ngap is provided in Table 8.1-2 and, if channelAccessMode = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213]. - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon , as described in clause 4.1 - If a UE is provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if - it is within UL symbols, or - it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2, and if channelAccessMode = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213] - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, as described in clause 4.1.

In summary, a valid RO cannot exist in DL symbols. It makes it impossible for a non-SBFD UE to transmit preamble on DL symbols. Even for a SBFD UE with transparent UL subband, it is not possible to transmit preamble on DL symbols as it would be an invalid RO according to the above definition. The following figure could be an example.

Figure 25 Example of multiplexing between UL and DL on SBFD slots

For non-transparent SBFD operation, SBFD UE has full knowledge on which resources are reserved for UL transmission. Therefore, it is confident to transmit preamble on the UL subband when random access is triggered, even if the RO is on the DL symbols. However, it needs to be further clarified whether it is possible or necessary to allow UE to transmit preamble on the UL subband.

For transparent SBFD operation, SBFD UE has to rely on the UL grant from gNB to transmit UL on SBFD slot. However, CBRA is event-based. gNB has no idea when and where a UE needs to transmit preamble. Hence it is not possible to introduce more valid RO for SBFD UE if UL subband is transparently determined.

Observation 19: If a RO exists in DL slots configured by tdd-UL-DL-ConfigurationCommon, it cannot be used for preamble transmission even if the RO locates in a transparent UL subband.

Proposal 29: Clarify whether to allow a SBFD UE to transmit preamble within the UL subband in a SBFD slot.

Conclusion

In this contribution, we provide our views on subband non-overlapping full duplex. We have the following observations:

Observation 1: There are several mechanisms to realize TDD UL-DL configuration which leads to different TDD periodicity:

- Single TDD periodicity provided by tdd-UL-DL-ConfigurationCommon

- Double TDD periodicity provided by tdd-UL-DL-ConfigurationCommon

- Additional TDD UL-DL configuration provided by tdd-UL-DL-ConfigurationDedicated on top of common configuration

- No TDD periodicity is provided by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated

Observation 2: A unified solution to indicate the SBFD subband time domain location should be enployed in order to simplify specification and gNB operation.

Observation 3: SLIV-like method is not sufficient for indicating SBFD time location for discrete DL region hence a unified indication is impossible across different TDD configuration scenarios.

Observation 4: Bitmap based method can be used to indicate SBFD time location within a time period.

- The time period is equal to TDD periodicity provided by tdd-UL-DL-ConfigurationCommon

- FFS: the time unit of bitmap

- FFS: how to determine time period if gNB doesn’t provide any TDD configuration

Observation 5: If bitmap-based mechanism is used to indicate SBFD subband time domain location, how to define guard period between DL symbol and first symbol occupied by UL subband needs further study.

Observation 6: A subband-specific TDD UL-DL configuration can be used to indicate SBFD subband time domain location.

Observation 7: Guard band between DL subband and UL subband is not aways needed.

Observation 8: For SBFD aware UE, whether enhancement is needed for DL receptions across SBFD symbols and non-SBFD symbols depends UE behaviour discussion.

Observation 9: For DG PDSCH with or without repetition, current specification is sufficient for DL receptions across SBFD symbols and non-SBFD symbols.

Observation 10: For PUSCH without repetition, current specification is sufficient to avoid the case where a PUSCH is across SBFD symbol and non-SBFD symbol.

Observation 11: For PUCCH without repetition, current specification is sufficient for avoiding PUCCH transmission across SBFD symbols and non-SBFD symbols.

Observation 12: For PDCCH, it is up to gNB to avoid collision between UL subband and CORESET in frequency domain.

Observation 13: The conflicted dynamic downlink and dynamic uplink on the same SBFD symbol may have different priorities.

Observation 14: Semi-static DL and dynamic UL may collide on SBFD symbol.

Observation 15: The impacts on SSB/CSS from UL transmission within UL subband may not always be an issue.

Observation 16: The conflict between semi-static DL and semi-static UL may occur.

Observation 17: TA enhancement is not necessary as it doesn’t resolve inter-slot interference issue.

Observation 18: The impacts on digital IC from unaligned symbol/slot boundary at gNB side needs further study.

Observation 18: If a RO exists in DL slots configured by tdd-UL-DL-ConfigurationCommon, it cannot be used for preamble transmission even if the RO locates in a transparent UL subband.

Based on the aforementioned discussion and observation, we have the following proposals:

Proposal 1: For semi-static configuration of subband time locations for SBFD operation, the following mechanisms can be considered:

- gNB indicates SBFD subband time domain location via a bitmap

- gNB indicates SBFD subband time domain location via a subband-specific TDD UL-DL configuration

Proposal 2: Cell-specific signalling is sufficient to configure SBFD subband time location.

Proposal 3: Before deciding whether dynamic indication is supported for SBFD subband time location, we need to firstly clarify UE behaviour in SBFD slot.

Proposal 4: If UL subband is configured on semi-static flexible symbol and gNB further adjust direction for the symbols, clarify behaviour of SBFD aware UE at least for the following cases:

- Semi-flexible symbol is indicated as DL symbol on which UL subband is configured

- Semi-flexible symbol is indicated as UL symbol on which UL subband is configured

- Semi-flexible symbol is indicated as SFI flexible symbol on which UL subband is configured

Proposal 5: Study whether and how to use guard band for downlink transmission.

Proposal 6: Send LS to RAN4 and consult RAN4 how guard band between DL subband and UL subband is determined.

Proposal 7: For configuring frequency location of UL subband, the following mechanism can be considered:

- Reuse the same mechanism of configuring BWP to configure the frequency locations of subband for SBFD operation, i.e. the starting position and length are indicated.

- If gNB doesn’t provide any configuration for UL subband frequency location, a default BWP is used to determine the frequency location of the UL subband.

Proposal 8: Before deciding whether dynamic indication is supported for SBFD subband frequency location, we need to firstly clarify UE behaviour in SBFD slot.

Proposal 9: Cell-specific RRC signalling is sufficient to configure SBFD subband frequency location.

Proposal 10: For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, the following behaviours are applicable for SBFD aware UEs,

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

- DL receptions within DL subband(s) are allowed in the symbol

- FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol

Proposal 11: DL reception is allowed within UL subband in SBFD symbol.

Proposal 12: DL reception is allowed within guard band in SBFD symbol.

Proposal 13: For PUSCH repetition without frequency hopping, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- PUSCH repetition is mapped on UL slot only

- PUSCH repetition is mapped on SBFD slot only

- PUSCH repetition is scheduled on UL slot and SBFD slot with confining frequency resources in UL subband

- PUSCH repetition is scheduled on UL slot and SBFD slot, the frequency allocation on UL slot and SBFD slot can be different

Proposal 14: For PUSCH repetition with frequency hopping, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- PUSCH repetition is mapped on UL slot only

- PUSCH repetition is mapped on SBFD slot only

- PUSCH repetition is scheduled on UL slot and SBFD slot, same frequency hopping offset list is applied

- PUSCH repetition is scheduled on UL slot and SBFD slot, separate frequency offsets list are applied to UL slot and SBFD slot respectively

Proposal 15: For PUCCH repetition without frequency hopping, the following mechanisms can be considered for PUCCH across SBFD slot and non-SBFD slot:

- PUCCH repetition is mapped on UL slot only

- PUCCH repetition is mapped on SBFD slot only

- PUCCH repetition is indicated on UL slot and SBFD slot with confining frequency resources in UL subband

- PUCCH repetition is indicated on UL slot and SBFD slot, the frequency allocation on UL slot and SBFD slot can be different

Proposal 16: For PUCCH repetition with frequency hopping, the following mechanisms can be considered for PUCCH across SBFD slot and non-SBFD slot

- PUCCH repetition is mapped on UL slot only

- PUCCH repetition is mapped on SBFD slot only

- PUCCH repetition is indicated on UL slot and SBFD slot, same frequency hopping offset list is applied

- PUCCH repetition is indicated on UL slot and SBFD slot, separate frequency offsets list are applied to UL slot and SBFD slot respectively

Proposal 17: For PUSCH with TBoMS, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- PUSCH repetition is mapped on UL slot only

- PUSCH repetition is mapped on SBFD slot only

- PUSCH repetition is indicated on UL slot and SBFD slot with confining frequency resources in UL subband

Proposal 18: For multi-PUSCH scheduled by a single DCI, the following mechanisms can be considered for PUSCH across SBFD slot and non-SBFD slot:

- All PUSCHs are mapped on UL slot only

- All PUSCHs are mapped on SBFD slot only

- Multi-PUSCH is scheduled on UL slot and SBFD slot with confining frequency resources in UL subband

- Multi-PUSCH is scheduled on UL slot and SBFD slot, the frequency allocation on UL slot and SBFD slot can be different

Proposal 19: For PDSCH frequency resource allocation on SBFD symbols, the following mechanisms can be considered:

- Option 1: UL subband boundary is aligned with that of RBG

- Option 2: Only RBs not contained by UL subband is available for DL transmission in a RBG

- Option 3: RBG is mapped to the RBs not contained by UL subband only.

Proposal 20: For CSI-RS resource configuration on SBFD slot, non-contiguous CSI-RS resource configuration via explicit or implicit mechanism should be further studied.

Proposal 21: For PRG determination on SBFD symbols, the following mechanisms can be considered:

- Option 1: UL subband boundary is aligned with that of PRG

- Option 2: For PRG partially overlapped with UL subband or guard band, the actual PRG is determined by the left RBs

- Option 3: PBG is determined on the RBs not contained by UL subband and guard band only.

Proposal 22: Further study whether the following cases are allowed or not:

- Type 1: Dynamic DL on SBFD symbol conflicts with Dynamic UL on the UL subband.

- Type 2: Semi-static DL on SBFD symbol conflicts with Dynamic UL on the UL subband.

- Type 3: Broadcast DL on SBFD symbol conflicts with UL on the UL subband.

- Type 4: Dynamic DL on SBFD symbol conflicts with semi-static UL on the UL subband, including CG PUSCH, periodic uplink reference signal and periodic PUCCH.

- Type 5: Semi-static DL on SBFD symbol conflicts with semi-static UL on the UL subband.

Proposal 23: BWP-based SBFD operation is not supported in Rel-18 duplex enhancement.

Proposal 24: Half duplex CA based SBFD operation is not supported in Rel-18 duplex enhancement.

Proposal 25: Guard period between DL region and UL subband is needed and at least the following mechanism can be considered.

- Option 1: The guard period is configured by gNB for SBFD aware UE.

- Option 2: UE does not expect to transmit in the uplink on the first G symbols within UL subband.

- Option 3: The first OFMD symbol of UL subband is always adjacent UL symbol or flexible symbol.

Proposal 26: Subband based inter-subband UE-UE CLI measurement and report can be discussed in AI 9.3.2.

Proposal 27: Subband based inter-subband UE-UE CLI measurement and information exchange can be discussed in AI 9.3.2.

Proposal 28: For subband non-overlapping full duplex, it cannot be applied to UL symbols.

Proposal 29: Clarify whether to allow a SBFD UE to transmit preamble within the UL subband in a SBFD slot.

Reference

  - RAN1#111 meeting chairman notes

  - RAN1#110bis e-meeting chairman notes

  - R4-2214376,  Reply LS on interference modelling for duplex evolution, RAN WG4

  - TS38.214, v17.3.0

  - R4-1804119, On NR NTA offset, Intel Corporation