3GPP TSG RAN WG1 #112			R1-2300601

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item: 	9.11.1

Source: 	Baicells

Title:	Discussion on coverage enhancement for NR NTN

Document for:	Discussion and Decision

- Introduction

It has been concluded that PUCCH for Msg4 HARQ-ACK should be enhanced to meet the coverage requirement from RAN1#110.

In RAN#97, the NR NTN enhancements WI was updated.

The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarization loss (per antenna port).

Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO.

The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps.

The detailed objectives are for NTN:

- To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4]

- To study DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) and, if necessary, specify enhancements to the Rel-17 procedures [RAN1]

In RAN1#110bis-e, some major agreements and conclusions have been achieved:

Agreement

For PUCCH for Msg4 HARQ-ACK,

- Support PUCCH repetition

  - Further discuss the specification impact for at least the following

    - Procedure and signaling (e.g., cell-specific configuration, request to gNB and dynamic indication from gNB, UE capability indication before Msg4, etc.)

    - Repetition factor

    - Repetition slot counting for FDD

  - Further study whether to enhance or support the following

    - Frequency hopping

    - DMRS bundling

Agreement

For PUCCH repetition for Msg4 HARQ-ACK,

- Discuss the following options of procedure to perform repetitions

  - Option 1: UE always performs repetition if configured in cell-specific manner

    - FFS: details of cell-specific configuration

    - FFS: behavior of UE being incapable of repetition

  - Option 2: UE requests repetition and is dynamically indicated to perform repetition

    - FFS: details of repetition request

    - FFS: details of configuration and dynamic repetition indication

  - Option 3: UE indicates repetition capability and is dynamically indicated to perform repetition

  - How UE indicates repetition capability before Msg4

Conclusion

For PUCCH repetition for Msg4 HARQ-ACK,

- The existing mechanism on repetition slot counting (as in section 9.2.6 of TS 38.213) can be applied.

  - FFS: whether specification update to apply the existing mechanism to PUCCH repetition for Msg4 HARQ-ACK is needed.

Agreement

For NTN-specific PUSCH DMRS bundling,

- Discuss further the need of enhancement in consideration of at least the following:

  - Phase difference due to timing drift and/or doppler shift.

    - e.g., whether/how long a UE can meet phase continuity requirement specified as Table 6.4.2.5-1 in 38.101-1 in consideration of frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-5 and timing error specified in Table 7.1C.2-1 of 38.133, whether RAN1 should introduce enhancement to meet the requirement and/or recommend RAN4 to update the requirement or UE should pre-compensate phase difference by UE implementation, etc.

  - An event which causes power consistency and phase continuity not to be maintained.

    - e.g., whether the new event is necessary to determine actual TDW(s) from each nominal TDW or the existing specification can work without any specification change or whether such event may not occur depending on implementations, etc.

  - Note: baseline performance for legacy UEs can include antenna switching

Agreement

For PUCCH transmission for Msg4 HARQ-ACK, supported number of transmissions are 1, 2, 4, 8.

- Note: single PUCCH transmission is performed as in the existing specification, and/or (if supported for single PUCCH transmission) according to configuration/indication e.g., in signaling with respect to number of transmissions.

- FFS: whether larger number of transmissions is supported

- FFS: whether/how single PUCCH transmission can be configured and/or indicated

In RAN1#111, more conclusions and working assumption have been achieved.

Conclusion

For the study of NTN-specific PUSCH DMRS bundling, RAN1’s understanding is that Phase variation due to constant frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 does not have impact on the phase continuity requirement for two adjacent slots specified as Table 6.4.2.5-1 in 38.101-1, according to annex F.9 and F.4 of 38.101-1.

Conclusion

RAN1 concluded that PUSCH DMRS bundling with sufficient TDW size should be applicable in NTN to meet the performance requirement for VoIP

- FFS: How to determine TDW size, including UE capability.

- Note: The above does not mean the performance requirements will be satisfied with DMRS bundling

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- One or more repetition factors may be configured via SIB

  - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor

    - FFS: whether UE requests repetition or indicates repetition capability

  - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB

    - FFS: whether UE requests repetition or indicates repetition capability

    - FFS: whether repetition factor is indicated by UE

  - FFS: UE behavior when repetition factor is not configured via SIB

  - FFS: whether one or more UE capabilities are needed for the above is for further discussion

In this contribution, we will discuss in more details regarding PUCCH enhancement for Msg4 HARQ-ACK and DMRS bundling for PUSCH for NTN.

- 2. Discussion

- 2.1 PUCCH enhancement for Msg4 HARQ-ACK

- For PUCCH repetition for Msg4 HARQ-ACK, it has been agreed that one or more repetition factors may be configured via SIB in RAN1#111. The repetition factors are basically agreed to be {1, 2, 4, 8}.

- Four different cases are summarized in our view:

- Case 1. If repetition factor is not configured via SIB, it is implied that no PUCCH repetition for Msg4 HARQ-ACK is applied for all UEs (same as the legacy specifications).

- Case 2. If only one repetition factor is configured via SIB, and the factor is NOT 1, then UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor.  If an UE is not capable of repetition, UE does not make repetition, and gNB make blind detection for it.

- Case 3. If only one repetition factor is configured via SIB, and the factor is 1, no repetition is required for all UEs, as same as Case1. This case is basically not recommended in our view.

- Case 4. If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB. UE can indicate whether or not repetition is required from UE point of view. gNB determines the factor to be used, and dynamically indicate the factor via DCI.

- Proposal 1 : If repetition factor is not configured via SIB, it is implied that no PUCCH repetition for Msg4 HARQ-ACK is applied for all UEs (same as the legacy specifications).

- Proposal 2:  If only one repetition factor is configured via SIB, and the factor is not 1, then UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor.  For an UE not capable of repetition, no repetition is taken, and gNB make blind detection for it.

- Proposal 3:  If multiple factors from {1, 2, 4, 8} are configured via SIB, UE can indicate whether or not repetition is required from UE point of view. gNB determines and dynamically indicate the factor to be used by UE.

For PUCCH for Msg4 HARQ ACK, when intra-slot frequency hopping is applied, the performance is worse than not applied. As illustrated in the figure below.

Note: The simulation presumptions are summarized in the following table.

| Number of symbols | 14 |
| --- | --- |
| Number of Tx and Rx | 1T1R |
| Channel model | NTN-TDL-C |
| Scenario | Rural,LOS |
| Elevation angle | 30deg |

Therefore, mechanism for disabling intra-slot FH for PUCCH for Msg4 HARQ ACK may be needed to enhance performance.

Observation 1: For Msg4 HARQ ACK in NTN, intra-slot frequency hopping worsen the performance compared to no intra-slot frequency hopping.

Proposal 4: For Msg4 HARQ ACK in NTN, intra-slot frequency hopping should be disabled.

- 2.2 DMRS bundling for PUSCH

In Table 7.1C.2-1 of 38.133, timing error limit is specified.  When SCS = 15kHz, Te =  29 Ts  = 0.944 µs.

Table 7.1C.2-1: Te_NTN Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

For LEO-600, the trip time between satellite and ground has a changing rate as illustrated in the figure below. The maximum drift rate = 23ppm. At elevation angle of 30deg, drift rate = 20ppm. For both UL and DL, feeder link and service link, the total drift rate is 4 times of that, i.e., 80ppm at 30degree.

Therefore, to comply with the 29Ts RAN4 timing error requirement, the actual TDW should be within  0.944us/80ppm = 11.8ms for LEO-600.

Observation 2:  For LEO-600, elevation angle of 30deg, the trip time between satellite and ground has a changing rate of 20ppm.  For both UL and DL, feeder link and service link, the total drift rate is 80ppm.

Proposal 5: To comply with the 29Ts timing error requirement, the actual TDW for DMRS bundling shall be within 11.8ms for LEO-600 at elevation angle of 30deg.

In Table 6.4.2.5-1 of 38.101-1, the maximum allowable phase difference for DMRS bundling is specified. Phase difference should not exceed 30 degrees.

Table 6.4.2.5-1: Maximum allowable phase difference for DMRS bundling

| UL channel | Modulation order | Phase difference between any slot p-1 and slot p (NOTE 2) | Phase difference between slot 0 and any slot p (NOTE 3) |
| --- | --- | --- | --- |
| PUSCH | Pi/2 BPSK, QPSK | [25] degrees | [30] degrees |
| PUCCH | Pi/2 BPSK, BPSK, QPSK |  |  |
| NOTE 1: The UE capability of the length of maximum duration refers to the maximum time duration during which UE is able to meet the phase continuity requirements, assuming no phase consistency violating events defined in TS 38.214 in between. NOTE 2: This requirement applies for FDD and TDD bands, for supported DMRS bundling configurations ≤ 8 slots. NOTE 3: This requirement applies only for FDD bands, for supported DMRS bundling configurations of 16 slots. |  |  |  |

For a time duration T for DMRS bundling, the accumulated phase difference is:

φ =360deg ∙0Tf(t) dt,

in which f is the frequency offset.

Assume that fixed frequency error (± 0.1 PPM) and doppler frequency shift at time 0 have been compensated, then

f(t) =r∗t,

where r is the changing rate of doppler frequency shift.

Therefore,

φ =360deg∙0Tf(t) dt=360deg∙0Tr∙t dt = 180deg∙r∙T2.

T = φ/(180∙r) .

For LEO-600, when carrier frequency is 2GHz, the doppler frequency varies with a changing rate illustrated in the figure below. The maximum absolute changing rate  r = 580Hz/s. For UL and DL, the total changing rate is 2 times of that, i.e., 1160Hz/s.

Let r =1160,  φ = 30,

then T = φ/(180∙r)= 30/(180∙1160)  = 0.012 (s) = 12 (ms)

Therefore, actual TDW for DMRS bundling should be within 12 ms.

Observation 3:  For LEO-600, 2GHz carrier frequency, the maximum doppler changing rate is 580Hz/s (absolute value) .  For both UL and DL, the total changing rate is 1160 Hz/s.

Proposal 6: To comply with the 30 degree phase difference requirement, the actual TDW for DMRS bundling shall be within 12 ms for LEO-600.

- 3. Conclusions

- Proposal 1 : If repetition factor is not configured via SIB, it is implied that no PUCCH repetition for Msg4 HARQ-ACK is applied for all UEs (same as the legacy specifications).

- Proposal 2:  If only one repetition factor is configured via SIB, and the factor is not 1, then UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor.  For an UE not capable of repetition, no repetition is taken, and gNB make blind detection for it.

- Proposal 3:  If multiple factors from {1, 2, 4, 8} are configured via SIB, UE can indicate whether or not repetition is required from UE point of view. gNB determines and dynamically indicate the factor to be used by UE.

Observation 1: For Msg4 HARQ ACK in NTN, intra-slot frequency hopping worsen the performance compared to no intra-slot frequency hopping.

Proposal 4: For Msg4 HARQ ACK in NTN, intra-slot frequency hopping should be disabled.

Observation 2:  For LEO-600, elevation angle of 30deg, the trip time between satellite and ground has a changing rate of 20ppm.  For both UL and DL, feeder link and service link, the total drift rate is 80ppm.

Proposal 5: To comply with the 29Ts timing error requirement, the actual TDW for DMRS bundling shall be within 11.8ms for LEO-600 at elevation angle of 30deg.

Observation 3:  For LEO-600, 2GHz carrier frequency, the maximum doppler changing rate is 580Hz/s (absolute value) .  For both UL and DL, the total changing rate is 1160 Hz/s.

Proposal 6: To comply with the 30 degree phase difference requirement, the actual TDW for DMRS bundling shall be within 12 ms for LEO-600.

References

R1-2212849 Chair's notes RAN1#111 9.11 R18 NTN v05

RP-222812 SR R18 WID NR-NTN-enh for RAN98-e_v07

R1-2210694 Chair's notes RAN1#110bis-e 9.11 R18 NTN v06 (eom).

TS 38.133 V17.8.0 (2022-12)

R1-2212213 Discussion on coverage enhancement for NR NTN