3GPP TSG RAN WG1 #112		                                                      R1-2300677

Athens, Greece, February 27th – March 3rd, 2023

Source:	CATT

Title:	Discussion on evaluation on NR duplex evolution

Agenda Item:	9.3.1

Document for:	Discussion and Decision

- Introduction

During RAN1#111, evaluation scenarios, methodologies and assumptions on NR duplex evolution were further discussed and many consensuses were reached on this topic [1].

In this contribution, views on remaining issues for evaluation methodologies and evaluation assumptions are further provided. And initial SLS results are updated based on the agreed simulation assumptions in RAN1#111 meeting.

- Discussion

  - Evaluation methodology

    - LLS purpose

The following agreement was made in RAN1#111 with a couple of FFS points.

Agreement RAN1 agrees link-level simulations (LLS) may be performed for various purposes related to SBFD performance and feasibility in both FR1 and FR2, interested companies may perform LLS at least for the following purpose: - To evaluate coverage performance - Option 1: Take link level evaluation methodology in TR 38.830 (i.e., LLS + Link budget analysis) as starting point to evaluate the coverage performance (e.g., MPL, MCL, MIL) for SBFD considering inter-gNB/sector interference and self interference. - Other options (e.g. SLS as a tool to obtain the coverage metric) are not precluded - Details on LLS including but not limited to impact of different BS antennas to channel reciprocity / BF - FFS: - To evaluate advanced receivers and realistic demodulation performance - To evaluate UE-UE CLI mitigation performance - To evaluate gNB-gNB CLI mitigation performance - To evaluate feasibility and performance of self-IC accounting for realistic non-linearities in the gNB transmit and receive chains - Details on LLS including but not limited to impact of different BS antennas to channel reciprocity / BF

For the CLI mitigation performance evaluation, it depends more on CLI handling scheme itself. Some CLI mitigation solutions should be evaluated via LLS, e.g. advanced receiver based solution, while some CLI mitigation solutions should be evaluated via SLS, e.g. scheduling coordination based solution. From our point of view, self-interference mitigation performance mainly depends on RAN4’s input other than RAN1’s LLS evaluation. Therefore, whether LLS is used for other purposes in addition to coverage performance evaluation can be up to companies’ interests.

Proposal 1: LLS for other purpose besides coverage performance evaluation is up to companies’ interests.

    - UE-UE CLI modeling

Agreement For SLS in RAN1, regarding Tx leakage model of UE-UE co-channel inter-subband CLI modelling, Option 1 is used as starting point. - Option 1: RAN1 to take in-band emission (IBE) defined in TS38.101-1 and TS38.101-2 as starting point. - Send LS to RAN4 to ask them whether it can be modelled as an equivalent frequency flat model (e.g., IBEUE,ave) based on RAN4 IBE requirement, and if possible, what is the value of IBEUE,ave

IBE based method was agreed for Tx leakage model of UE-UE co-channel inter-subband CLI modelling in last meeting while Rx impairment model of UE-UE co-channel inter-subband CLI modelling is still open. The following two options were proposed in RAN1#111 meeting.

- Option 1: Reuse UE ACS in absence of further RAN4 inputs.

- Option 2: RAN1 assumes no UE in-band selectivity in absence of further RAN4 inputs, i.e., 0 dB without any rejection/attenuation on interference in Rx model.

Besides those two options, ICS based solution was proposed [2] in RAN4. The issue was extensively discussed in RAN4. From our view, RAN1 can wait for RAN4’s conclusion on this issue.

Proposal 2: Wait for RAN4 conclusion on Rx impairment model of UE-UE co-channel inter-subband CLI modelling.

  - Evaluation assumptions for SBFD deployment case 4

For deployment case 4, i.e. adjacent-channel co-existence case, reusing the traditional inter-cell interference (including BS-UE and UE-BS) modelling mothed defined in TR38.828 can be considered. And ACIR based method was agreed for BS-BS/ UE-UE adjacent-channel CLI modelling. With respect to ACIR for FR1 for deployment case 4, the parameters defined in Table 5.2.1.2-1 in TR38.828 can be used.

Table 1  (Table 5.2.1.2-1 in TR 38.828: ACIR for FR1)

| Parameter | Assumption/Value |
| --- | --- |
| ACIR BS-BS | 43 dB |
| ACIR BS-UE | 33 dB |
| ACIR UE-BS | 30 dB |
| ACIR UE-UE | 28 dB |

With respect to ACLR and ACS for FR2 for deployment case 4, the parameters defined in Table 5.2.2.2-1 in TR38.828 can be used.

Table 2: (Table 5.2.2.2-1 in TR 38.828: ACLR and ACS for FR2)

| Parameter | Assumption/Value |
| --- | --- |
| BS ACLR | 28 dB |
| UE ACLR | 17 dB |
| BS ACS | 23.5 dB |
| UE ACS | 23 dB |

Proposal 3: Adopt the ACIR defined in Table 5.2.1.2-1 in TR38.828 for FR1 for SBFD deployment case 4.

Proposal 4: Adopt the ACLR and ACS defined in Table 5.2.2.2-1 in TR38.828 for FR2 for SBFD deployment case 4.

- Initial SLS results

- Alt 1 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#1 (DXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 4 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth.

In RAN1#109-e meeting, Alt 1 and Alt 4 were agreed as two of the four SBFD configurations alternatives. The initial simulation results of Alt 1 and Alt 4 for deployment Case 1 are given respectively in the following sections.

  - Simulation results of SBFD configuration Alt 1 with large packet (0.5Mbyte for DL and 0.125 Mbytes for UL)

    - Indoor office

Table 3: Latency and RU results for indoor office

| Feature |  | DL latency (ms) |  |  |  |  |  | UL latency (ms) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) |
| Low load | Legacy TDD | 6.97 | 11.4 | 17.32 | 11.8 | 6.02 | 7.52 | 4.79 | 6.2 | 12.01 | 7.2 | 1.53 | 7.66 |
|  | SBFD | 9.81 | 12.78 | 19.1 | 13.41 | 5.6 | 8.36 | 3.43 | 5.58 | 8.68 | 5.91 | 2.15 | 6.53 |
| Medium load | Legacy TDD | 12.03 | 16.47 | 26.77 | 17.47 | 19.89 | 24.86 | 8.09 | 11.32 | 21.45 | 12.47 | 5.74 | 28.69 |
|  | SBFD | 14.67 | 18.6 | 28.79 | 19.89 | 17.53 | 26.17 | 5.59 | 8.51 | 12.81 | 9.42 | 7.01 | 21.24 |
| High load | Legacy TDD | 33.91 | 44.94 | 60.82 | 46.22 | 40.94 | 51.18 | 20.76 | 35.26 | 58.53 | 37.38 | 10.52 | 52.61 |
|  | SBFD | 37.99 | 47.897 | 70.99 | 50.67 | 34.7 | 51.79 | 18.39 | 25.26 | 35.04 | 26 | 16.75 | 50.76 |

| Low load | Median load | High load |
| --- | --- | --- |
| Low load | Median load | High load |

Figure 1: Latency results for indoor office

As can be seen from Table 3 and Figure 1, UL latency of SBFD is reduced at all three load conditions for indoor office scenario at the cost of slightly increased DL latency.

Observation 1: For indoor office, compared to legacy TDD, SBFD with Alt 1 can significantly reduce the UL latency at the cost of slightly increased DL latency.

Table 4: UPT results for indoor office

| Feature |  |  | DL UPT (Mbps) |  |  |  |  | UL UPT (Mbps) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | Mean | 5% |  | 50% | 95% | Mean |
| Low load | Average-UPT | Legacy TDD | 245.65 | 327.14 | 428.14 | 336.84 | 73.16 |  | 130.54 | 208.59 | 130.31 |
|  |  | SBFD | 191.63 | 291.2 | 345.52 | 281.55 | 125.62 |  | 168.33 | 209.38 | 170.73 |
|  | Tail-UPT | Legacy TDD | 136.23 | 221.85 | 277.19 | 218.6 | 50.68 |  | 82.43 | 103.99 | 84.98 |
|  |  | SBFD | 134.7 | 208.18 | 242.8 | 200.15 | 93.87 |  | 131.55 | 166.55 | 132.94 |
|  | Median-UPT | Legacy TDD | 249.77 | 332.67 | 442.41 | 347.6 | 75.3 |  | 126.07 | 188.76 | 122.04 |
|  |  | SBFD | 198.67 | 296.2 | 360.52 | 286.42 | 114.77 |  | 168.33 | 209.2 | 168.66 |
| Median load | Average-UPT | Legacy TDD | 162.62 | 224.89 | 272.74 | 225.82 | 53 |  | 82.37 | 106.58 | 79.45 |
|  |  | SBFD | 127.24 | 199.36 | 241.02 | 194.25 | 85.09 |  | 116.4 | 150.67 | 117.53 |
|  | Tail-UPT | Legacy TDD | 99.2 | 151.03 | 180.03 | 144.89 | 29.44 |  | 46.97 | 61.45 | 47.04 |
|  |  | SBFD | 84.48 | 138.45 | 158.45 | 132.48 | 46.64 |  | 75.28 | 87.68 | 73.58 |
|  | Median-UPT | Legacy TDD | 164.45 | 239.84 | 304.27 | 238.66 | 46.26 |  | 83.93 | 107.23 | 77.76 |
|  |  | SBFD | 128.73 | 207.21 | 251.65 | 200.41 | 78.47 |  | 113.25 | 152.75 | 117.55 |
| High load | Average-UPT | Legacy TDD | 73.87 | 89.54 | 105.83 | 89.78 | 16.8 |  | 26.89 | 37.29 | 27.25 |
|  |  | SBFD | 58.71 | 82.91 | 99.18 | 82.47 | 22.26 |  | 39.08 | 53.19 | 39.52 |
|  | Tail-UPT | Legacy TDD | 50.27 | 65.71 | 75.16 | 65.33 | 11.22 |  | 18.61 | 20.61 | 17.8 |
|  |  | SBFD | 45.86 | 60.31 | 69.13 | 59.39 | 18.67 |  | 29.61 | 36.44 | 29.07 |
|  | Median-UPT | Legacy TDD | 72.21 | 89.04 | 103.03 | 87.99 | 16.45 |  | 24.12 | 32.83 | 24.82 |
|  |  | SBFD | 59.35 | 83.29 | 98.53 | 82.99 | 20.62 |  | 39.16 | 54.25 | 39.77 |

Low load

Median load

High load

Figure 2: UPT results for indoor office

As can be seen from Table 4 and Figure 2, SBFD achieves better UL user throughput at all load conditions for indoor office than that of the legacy TDD at the cost of decreased DL user throughput due to more resources are allocated for UL in SBFD.

Observation 2: For indoor office, compared to legacy TDD, SBFD with Alt 1 achieves better UL user throughput at all three load conditions at the cost of decreased DL user throughput.

    - Urban macro

Table 5: Latency and RU results for urban macro

| Feature |  | DL latency (ms) |  |  |  |  |  | UL latency (ms) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) |
| Low load | Legacy TDD | 11.05 | 16.52 | 24.58 | 17.07 | 5.32 | 7.02 | 5.84 | 10.74 | 18.39 | 11.37 | 1.75 | 8.76 |
|  | SBFD | 15.18 | 18.45 | 25.63 | 19.33 | 5.78 | 8.63 | 4.5 | 7.88 | 10.89 | 7.97 | 2.39 | 7.24 |
| Medium load | Legacy TDD | 21.64 | 27.92 | 39.34 | 28.89 | 19.62 | 24.52 | 10.81 | 18.14 | 28.06 | 18.05 | 5.95 | 29.75 |
|  | SBFD | 24.78 | 29.43 | 45.92 | 31.26 | 17.48 | 26.09 | 9.31 | 12.68 | 17.33 | 12.99 | 9.66 | 29.27 |
| High load | Legacy TDD | 51.67 | 65.04 | 89.27 | 67.04 | 41.93 | 52.41 | 34.35 | 44.85 | 70.62 | 47.58 | 10.39 | 51.94 |
|  | SBFD | 60.36 | 72.58 | 91.19 | 73.83 | 34.07 | 50.85 | 21.87 | 28.08 | 37.66 | 28.94 | 16.73 | 50.69 |

| Low load | Median load | High load |
| --- | --- | --- |

| Low load | Median load | High load |
| --- | --- | --- |

Figure 3: Latency results for urban macro

As can be seen from Table 5 and Figure 3, UL latency is significantly reduced in SBFD at all three load conditions for urban macro at the cost of slightly increased DL latency.

Observation 3: For urban macro, compared to legacy TDD, SBFD with Alt 1 can significantly reduce the UL latency at all three load conditions with the cost of slightly increased DL latency.

Table 6: UPT results for urban macro

| Feature |  |  | DL UPT (Mbps) |  |  |  | UL UPT (Mbps) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | Mean | 5% | 50% | 95% | Mean |
| Low load | Average-UPT | Legacy TDD | 163.63 | 233.54 | 307.13 | 233.86 | 44.14 | 94.8 | 128.73 | 92.01 |
|  |  | SBFD | 153.86 | 218.63 | 246.88 | 212.14 | 27.49 | 53.32 | 64.35 | 50.6 |
|  | Tail-UPT | Legacy TDD | 118.31 | 161.09 | 201.31 | 161.44 | 58.03 | 89.53 | 108.14 | 86.96 |
|  |  | SBFD | 110.24 | 149.26 | 165.24 | 144.98 | 58.89 | 85.07 | 118.75 | 83.78 |
|  | Median-UPT | Legacy TDD | 156.56 | 233.31 | 303.99 | 232.67 | 42.51 | 88.01 | 122.91 | 85.34 |
|  |  | SBFD | 150.33 | 218.36 | 246.01 | 211.69 | 91.45 | 118.25 | 150.34 | 121.65 |
| Median load | Average-UPT | Legacy TDD | 98.96 | 139.87 | 173.25 | 138.13 | 28.89 | 54.03 | 72.68 | 52.71 |
|  |  | SBFD | 90.58 | 132.70 | 154.35 | 128.54 | 53.54 | 70.39 | 87.93 | 70.68 |
|  | Tail-UPT | Legacy TDD | 73.74 | 104.62 | 130.15 | 103.25 | 23.03 | 36.16 | 48.04 | 36.15 |
|  |  | SBFD | 72.29 | 98.16 | 109.45 | 95.28 | 36.3 | 54.86 | 67.8 | 53.78 |
|  | Median-UPT | Legacy TDD | 96.63 | 138.52 | 173.54 | 137.66 | 29.87 | 54.03 | 65.58 | 50.68 |
|  |  | SBFD | 86.21 | 129.78 | 150.24 | 127.37 | 52.62 | 74.48 | 89.76 | 74.01 |
| High load | Average-UPT | Legacy TDD | 42.08 | 60.09 | 69.62 | 58.74 | 13.52 | 22.29 | 30.4 | 22.3 |
|  |  | SBFD | 42.29 | 54.31 | 61.96 | 53.74 | 26.43 | 34.51 | 41.6 | 34.68 |
|  | Tail-UPT | Legacy TDD | 32.92 | 44.15 | 51.69 | 43.39 | 10.18 | 14.45 | 17.83 | 14.45 |
|  |  | SBFD | 32 | 42.46 | 46.75 | 41.31 | 14.31 | 26.33 | 31.02 | 25.64 |
|  | Median-UPT | Legacy TDD | 42.44 | 60.16 | 69.74 | 58.39 | 14.16 | 22.51 | 30.22 | 22.19 |
|  |  | SBFD | 40.57 | 53.88 | 61.11 | 52.35 | 23.63 | 34.67 | 46.72 | 34.59 |

Low load

Median load

High load

Figure 4: UPT results for urban macro

As can be seen from Table 6 and Figure 4, SBFD achieves better UL user throughput at all three load conditions for urban macro than that of the legacy TDD at the cost of slightly decreased DL user throughput due to more resources are allocated for UL in SBFD. It is observed that improvement of UL performance is much more than the degradation of DL performance.

Observation 4: For urban macro, compared to legacy TDD, SBFD with Alt 1 achieves better UL user throughput at all three load conditions at the cost of slightly decreased DL user throughput.

  - Simulation results of SBFD configuration Alt 1 with small packet (4Kbytes for DL and 1Kbyte for UL)

    - Indoor office

Table 7: Latency and RU results for indoor office

| Feature |  | DL latency (ms) |  |  |  |  |  | UL latency (ms) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) |
| Low load | Legacy TDD | 0.52 | 0.74 | 0.97 | 0.8 | 6.29 | 7.86 | 0.52 | 0.75 | 0.98 | 0.77 | 1.61 | 8.04 |
|  | SBFD | 0.52 | 0.75 | 0.97 | 0.77 | 5.32 | 7.94 | 0.52 | 0.74 | 0.98 | 0.76 | 2.42 | 7.32 |
| Medium load | Legacy TDD | 0.52 | 0.74 | 0.97 | 0.8 | 19.85 | 24.81 | 0.51 | 0.75 | 0.97 | 0.77 | 5.72 | 28.58 |
|  | SBFD | 0.52 | 0.75 | 0.97 | 0.77 | 17.06 | 25.47 | 0.52 | 0.74 | 0.97 | 0.76 | 7.23 | 21.9 |
| High load | Legacy TDD | 0.52 | 0.76 | 0.99 | 0.88 | 40.9 | 51.12 | 0.52 | 0.75 | 0.97 | 0.76 | 10.47 | 52.34 |
|  | SBFD | 0.52 | 0.8 | 1.4 | 0.9 | 34.64 | 51.7 | 0.52 | 0.75 | 0.97 | 0.76 | 16.8 | 50.91 |

| Low load | Median load | High load |
| --- | --- | --- |

| Low load | Median load | High load |
| --- | --- | --- |

Figure 5: Latency results for indoor office

As can be seen from Table 7 and Figure 5, SBFD has comparable DL and UL latency performances with legacy TDD besides DL latency at high load (due to less resources are allocated for DL compared to legacy TDD) for indoor office.

Observation 5: For indoor office, SBFD has comparable DL and UL latency performance with legacy TDD besides DL latency at high load for Alt 1 with small packet.

Table 8: UPT results for indoor office

| Feature |  |  | DL UPT (Mbps) |  |  |  | UL UPT (Mbps) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | Mean | 5% | 50% | 95% | Mean |
| Low load | Average-UPT | Legacy TDD | 40.77 | 42.63 | 44.03 | 42.61 | 10.09 | 10.69 | 11.31 | 10.69 |
|  |  | SBFD | 41.23 | 42.59 | 43.94 | 42.66 | 10.08 | 10.66 | 11.53 | 10.69 |
|  | Tail-UPT | Legacy TDD | 32.34 | 32.87 | 33.51 | 32.62 | 8.05 | 8.23 | 8.63 | 8.22 |
|  |  | SBFD | 32.45 | 32.83 | 33.44 | 32.66 | 8.04 | 8.21 | 8.62 | 8.23 |
|  | Median-UPT | Legacy TDD | 32.62 | 40.78 | 42.88 | 45.46 | 10.09 | 10.69 | 11.31 | 10.69 |
|  |  | SBFD | 32.66 | 41.08 | 42.87 | 45.5 | 9.96 | 10.61 | 11.36 | 10.66 |
| Median load | Average-UPT | Legacy TDD | 40.77 | 42.63 | 44.03 | 42.61 | 10.09 | 10.69 | 11.31 | 10.69 |
|  |  | SBFD | 41.2 | 42.69 | 44.03 | 42.66 | 10.13 | 10.65 | 11.46 | 10.71 |
|  | Tail-UPT | Legacy TDD | 32.29 | 32.85 | 33.48 | 32.62 | 8.04 | 8.23 | 8.63 | 8.23 |
|  |  | SBFD | 32.39 | 32.86 | 33.45 | 32.66 | 8.02 | 8.22 | 8.63 | 8.22 |
|  | Median-UPT | Legacy TDD | 40.68 | 42.85 | 45.56 | 42.94 | 9.78 | 10.63 | 11.88 | 10.71 |
|  |  | SBFD | 41.05 | 42.99 | 45.53 | 43.09 | 9.77 | 10.52 | 12.08 | 10.72 |
| High load | Average-UPT | Legacy TDD | 14.76 | 40.93 | 43.63 | 38.8 | 10.11 | 10.67 | 11.37 | 10.68 |
|  |  | SBFD | 24.17 | 37.29 | 39.1 | 36.16 | 10.21 | 10.66 | 11.41 | 10.71 |
|  | Tail-UPT | Legacy TDD | 4.53 | 32.66 | 33.51 | 30.55 | 8.02 | 8.24 | 8.6 | 8.22 |
|  |  | SBFD | 9.05 | 23.21 | 25.36 | 22.03 | 8.01 | 8.23 | 8.62 | 8.23 |
|  | Median-UPT | Legacy TDD | 35.65 | 42.25 | 44.68 | 41.46 | 9.79 | 10.63 | 11.86 | 10.71 |
|  |  | SBFD | 33.59 | 39.62 | 41.7 | 39.28 | 9.67 | 10.59 | 11.95 | 10.72 |

Low load

Median load

High load

Figure 6: UPT results for indoor office

As can be seen from Table 8 and Figure 6, SBFD shows comparable UL/DL user throughput with legacy TDD for indoor office except that it shows worse DL user throughput performance at high load (as the consequence of reduced DL resources).

Observation 6: For indoor office, SBFD with Alt 1 with small packet shows comparable UL/DL user throughput with legacy TDD except that it shows worse DL user throughput performance at high load.

    - Urban macro

Table 9: Latency and RU results for urban macro

| Feature |  | DL latency (ms) |  |  |  |  |  | UL latency (ms) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) |
| Low load | Legacy TDD | 0.52 | 0.75 | 0.97 | 0.81 | 6.36 | 7.95 | 0.52 | 0.75 | 0.97 | 0.76 | 1.79 | 8.96 |
|  | SBFD | 0.52 | 0.75 | 0.98 | 0.78 | 5.86 | 8.75 | 0.52 | 0.75 | 0.97 | 0.76 | 2.47 | 7.49 |
| Medium load | Legacy TDD | 0.52 | 0.75 | 0.97 | 0.81 | 19.32 | 24.15 | 0.52 | 0.74 | 0.97 | 0.76 | 5.79 | 28.97 |
|  | SBFD | 0.52 | 0.77 | 1.15 | 0.82 | 17.96 | 26.76 | 0.52 | 0.75 | 0.98 | 0.76 | 9.26 | 28.06 |
| High load | Legacy TDD | 0.52 | 0.78 | 5.58 | 1.23 | 41.56 | 51.95 | 0.52 | 0.77 | 3.31 | 1.1 | 10.3 | 51.48 |
|  | SBFD | 0.53 | 0.85 | 3.12 | 1.07 | 33.98 | 50.72 | 0.52 | 0.79 | 1.64 | 0.88 | 16.68 | 50.54 |

| Low load | Median load | High load |
| --- | --- | --- |

| Low load | Median load | High load |
| --- | --- | --- |

Figure 7: Latency results for urban macro

As can be seen from Table 9 and Figure 7, SBFD shows comparable DL and UL latency with legacy TDD except that it shows significant performance gain at 95%-CDF of DL/UL latency (multiple HARQ retransmissions are needed where SBFD can achieve significant gain) at high load condition.

Observation 7: For urban macro, SBFD with Alt 1 with small packet shows comparable DL and UL latency with legacy TDD except that it shows significant performance gain at 95%-CDF of DL/UL latency at high load condition.

Table 10: UPT results for urban macro

| Feature |  |  | DL UPT (Mbps) |  |  |  | UL UPT (Mbps) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | Mean | 5% | 50% | 95% | Mean |
| Low load | Average-UPT | Legacy TDD | 37.85 | 42.38 | 43.79 | 41.93 | 10 | 10.63 | 11.32 | 10.63 |
|  |  | SBFD | 37.73 | 42.39 | 43.84 | 42.02 | 10.01 | 10.62 | 11.3 | 10.64 |
|  | Tail-UPT | Legacy TDD | 32.28 | 32.82 | 33.61 | 32.59 | 8.05 | 8.23 | 8.58 | 8.22 |
|  |  | SBFD | 32.2 | 32.83 | 33.62 | 32.63 | 8.05 | 8.24 | 8.62 | 8.23 |
|  | Median-UPT | Legacy TDD | 40.5 | 42.74 | 44.54 | 42.65 | 9.56 | 10.58 | 11.79 | 10.65 |
|  |  | SBFD | 40.56 | 42.56 | 44.54 | 42.62 | 9.68 | 10.66 | 11.82 | 10.72 |
| Median load | Average-UPT | Legacy TDD | 30.14 | 42.36 | 43.63 | 41.04 | 10.05 | 10.66 | 11.35 | 10.69 |
|  |  | SBFD | 32.71 | 39.81 | 42.54 | 38.88 | 9.99 | 10.65 | 11.31 | 10.68 |
|  | Tail-UPT | Legacy TDD | 6.7 | 32.79 | 33.58 | 31.51 | 8.03 | 8.24 | 8.65 | 8.25 |
|  |  | SBFD | 10.18 | 27.01 | 32.61 | 25.32 | 7.97 | 8.29 | 8.66 | 8.27 |
|  | Median-UPT | Legacy TDD | 40.27 | 42.61 | 44.66 | 42.63 | 9.62 | 10.7 | 11.72 | 10.72 |
|  |  | SBFD | 39.08 | 41.5 | 43.79 | 41.49 | 9.62 | 10.67 | 11.95 | 10.74 |
| High load | Average-UPT | Legacy TDD | 7.38 | 38.06 | 43.6 | 31.65 | 1.49 | 9.53 | 10.86 | 8.32 |
|  |  | SBFD | 15.4 | 34.84 | 38.08 | 31.22 | 6.81 | 9.94 | 11.08 | 9.45 |
|  | Tail-UPT | Legacy TDD | 4.15 | 32.24 | 33.28 | 21.23 | 0.75 | 2.47 | 8.36 | 4.74 |
|  |  | SBFD | 7.84 | 21.85 | 24.07 | 17.55 | 3.36 | 5.3 | 8.26 | 5.97 |
|  | Median-UPT | Legacy TDD | 6.03 | 41.29 | 44.39 | 37.52 | 1.74 | 9.75 | 11.22 | 8.99 |
|  |  | SBFD | 22.69 | 38.11 | 40.83 | 36.11 | 7.74 | 10.06 | 11.25 | 9.89 |

Low load

Median load

High load

Figure 8: UPT results for urban macro

As can be seen from Table 10 and Figure 8, SBFD achieves better UL user throughput at high load condition for urban macro than that of the legacy TDD, as the consequence of more UL resource allocation in SBFD. SBFD shows worse DL user throughput except 5% CDF (multiple HARQ retransmissions are needed where SBFD can achieve significant gain) of DL UPT at high load condition than that of the legacy TDD, as the consequence of less DL resource allocation in SBFD. And SBFD shows comparable DL and UL throughput with legacy TDD at low and medium load conditions for indoor office.

Observation 8: For urban macro, compared to legacy TDD, SBFD with Alt 1 with small packet achieves better UL user throughput at high load condition at the cost of decreased DL user throughput except 5% CDF of DL UPT.

  - Simulation results of SBFD configuration Alt 4 with large packet (0.5Mbyte for DL and 0.125 Mbytes for UL)

    - Indoor office

Table 11: Latency and RU results for indoor office （0.5Mbyte for DL and 0.125 Mbytes for UL）

| Feature |  | DL latency (ms) |  |  |  |  |  | UL latency (ms) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) |
| Low load | Legacy TDD | 6.97 | 11.40 | 17.32 | 11.80 | 6.02 | 7.52 | 4.79 | 6.20 | 12.01 | 7.20 | 1.53 | 7.66 |
|  | SBFD | 6.84 | 9.68 | 16.80 | 10.52 | 5.62 | 7.03 | 3.53 | 5.57 | 8.75 | 5.79 | 1.64 | 8.19 |
| Medium load | Legacy TDD | 12.03 | 16.47 | 26.77 | 17.47 | 19.89 | 24.86 | 8.09 | 11.32 | 21.45 | 12.47 | 5.74 | 28.69 |
|  | SBFD | 11.70 | 15.65 | 25.52 | 16.82 | 19.34 | 24.18 | 6.56 | 10.84 | 15.37 | 11.00 | 5.78 | 28.91 |
| High load | Legacy TDD | 33.91 | 44.94 | 60.82 | 46.22 | 40.94 | 51.18 | 20.76 | 35.26 | 58.53 | 37.38 | 10.52 | 52.61 |
|  | SBFD | 32.93 | 42.71 | 55.98 | 42.59 | 41.43 | 51.79 | 28.07 | 38.01 | 45.62 | 37.86 | 10.19 | 50.95 |

| Low load | Median load | High load |
| --- | --- | --- |

| Low load | Median load | High load |
| --- | --- | --- |

Figure 9: Latency results for indoor office

As can be seen from Table 11 and Figure 9, it can be observed that DL latency at all three load conditions for indoor office are reduced slightly compared to legacy TDD (due to increased HARQ-ACK transmission occasions). UL latency at low and median load conditions are reduced compared to that of legacy TDD (due to increased UL transmission occasions).

Observation 9: For indoor office, compared to legacy TDD, SBFD with Alt 4 can reduce DL latency slightly at all three low loads and reduce UL latency at low and median loads.

Table 12: UPT results for indoor office

| Feature |  |  | DL UPT (Mbps) |  |  |  | UL UPT (Mbps) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | Mean | 5% | 50% | 95% | Mean |
| Low load | Average-UPT | Legacy TDD | 245.65 | 327.14 | 428.14 | 336.84 | 73.16 | 130.54 | 208.59 | 130.31 |
|  |  | SBFD | 265.55 | 403.19 | 483.49 | 393.48 | 108.95 | 153.79 | 225.22 | 160.32 |
|  | Tail-UPT | Legacy TDD | 136.23 | 221.85 | 277.19 | 218.60 | 50.68 | 82.43 | 103.99 | 84.98 |
|  |  | SBFD | 164.80 | 262.76 | 300.60 | 248.72 | 79.25 | 106.22 | 126.43 | 106.38 |
|  | Median-UPT | Legacy TDD | 249.77 | 332.67 | 442.41 | 347.60 | 75.30 | 126.07 | 188.76 | 122.04 |
|  |  | SBFD | 281.10 | 392.67 | 483.72 | 389.82 | 126.48 | 173.96 | 187.57 | 169.22 |
| Median load | Average-UPT | Legacy TDD | 162.62 | 224.89 | 272.74 | 225.82 | 53.00 | 82.37 | 106.58 | 79.45 |
|  |  | SBFD | 186.69 | 235.65 | 288.11 | 235.37 | 65.13 | 96.41 | 108.75 | 93.25 |
|  | Tail-UPT | Legacy TDD | 99.20 | 151.03 | 180.03 | 144.89 | 29.44 | 46.97 | 61.45 | 47.04 |
|  |  | SBFD | 105.37 | 163.57 | 193.47 | 155.45 | 46.09 | 59.37 | 69.58 | 59.09 |
|  | Median-UPT | Legacy TDD | 164.45 | 239.84 | 304.27 | 238.66 | 46.26 | 83.93 | 107.23 | 77.76 |
|  |  | SBFD | 194.09 | 255.23 | 316.09 | 256.80 | 71.73 | 96.10 | 113.78 | 95.10 |
| High load | Average-UPT | Legacy TDD | 73.87 | 89.54 | 105.83 | 89.78 | 16.80 | 26.89 | 37.29 | 27.25 |
|  |  | SBFD | 75.59 | 92.43 | 111.09 | 93.95 | 23.31 | 26.50 | 29.82 | 26.49 |
|  | Tail-UPT | Legacy TDD | 50.27 | 65.71 | 75.16 | 65.33 | 11.22 | 18.61 | 20.61 | 17.80 |
|  |  | SBFD | 51.09 | 70.45 | 82.48 | 69.91 | 17.58 | 21.44 | 23.04 | 21.10 |
|  | Median-UPT | Legacy TDD | 72.21 | 89.04 | 103.03 | 87.99 | 16.45 | 24.12 | 32.83 | 24.82 |
|  |  | SBFD | 75.36 | 90.87 | 108.48 | 92.01 | 22.34 | 25.05 | 27.86 | 25.07 |

Low load

Median load

High load

Figure 10: UPT results for indoor office

As can be seen from Table 12 and Figure 10, compared to legacy TDD, SBFD with Alt 4 can significantly improve the UL UPT at low/medium load conditions and DL UPT at all load conditions for indoor office.

Observation 10: For indoor office, compared to legacy TDD, compared to legacy TDD, SBFD with Alt 4 can improve the UL UPT at low/medium load conditions and DL UPT at all load conditions.

    - Urban macro

Table 13: Latency and RU results for Urban macro

| Feature |  | DL latency (ms) |  |  |  |  |  | UL latency (ms) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) |
| Low load | Legacy TDD | 11.05 | 16.52 | 24.58 | 17.07 | 5.32 | 7.02 | 5.84 | 10.74 | 18.39 | 11.37 | 1.75 | 8.76 |
|  | SBFD | 10.68 | 15.41 | 22.09 | 15.77 | 5.85 | 7.31 | 5.92 | 9.33 | 14.91 | 9.59 | 1.68 | 8.39 |
| Medium load | Legacy TDD | 21.64 | 27.92 | 39.34 | 28.89 | 19.62 | 24.52 | 10.81 | 18.14 | 28.06 | 18.05 | 5.95 | 29.75 |
|  | SBFD | 22.21 | 26.37 | 37.63 | 27.56 | 19.72 | 24.65 | 9.77 | 16.35 | 19.20 | 15.50 | 5.83 | 29.17 |
| High load | Legacy TDD | 51.67 | 65.04 | 89.27 | 67.04 | 41.93 | 52.41 | 34.35 | 44.85 | 70.62 | 47.58 | 10.39 | 51.94 |
|  | SBFD | 51.60 | 63.02 | 90.02 | 65.02 | 42.16 | 52.70 | 31.14 | 47.41 | 52.33 | 45.84 | 10.04 | 50.18 |

| Low load | Median load | High load |
| --- | --- | --- |

| Low load | Median load | High load |
| --- | --- | --- |

Figure 11: Latency results for urban macro

As can be seen from Table 13 and Figure 11, when comparing UL latency with the legacy TDD, it can be observed that SBFD shows slightly reduced DL latency at all three load conditions for urban macro and significant reduced UL latency at low/medium load condition.

Observation 11: For urban macro, compared to legacy TDD, SBFD with Alt 4 can reduce the DL latency at all three load conditions for urban macro and UL latency at low/medium load condition.

Table 14: UPT results for urban macro

| Feature |  |  | DL UPT (Mbps) |  |  |  | UL UPT (Mbps) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | Mean | 5% | 50% | 95% | Mean |
| Low load | Average-UPT | Legacy TDD | 163.63 | 233.54 | 307.13 | 233.86 | 44.14 | 94.80 | 128.73 | 92.01 |
|  |  | SBFD | 170.01 | 257.83 | 325.53 | 250.59 | 80.42 | 114.14 | 131.30 | 113.63 |
|  | Tail-UPT | Legacy TDD | 118.31 | 161.09 | 201.31 | 161.44 | 27.49 | 53.32 | 64.35 | 50.60 |
|  |  | SBFD | 133.43 | 182.20 | 200.50 | 175.94 | 51.92 | 68.71 | 75.10 | 66.97 |
|  | Median-UPT | Legacy TDD | 156.56 | 233.31 | 303.99 | 232.67 | 42.51 | 88.01 | 122.91 | 85.34 |
|  |  | SBFD | 171.47 | 256.06 | 305.65 | 250.09 | 81.82 | 104.69 | 128.54 | 106.66 |
| Median load | Average-UPT | Legacy TDD | 98.96 | 139.87 | 173.25 | 138.13 | 28.89 | 54.03 | 72.68 | 52.71 |
|  |  | SBFD | 107.63 | 149.16 | 182.69 | 145.37 | 46.49 | 60.39 | 72.82 | 60.23 |
|  | Tail-UPT | Legacy TDD | 73.74 | 104.62 | 130.15 | 103.25 | 23.03 | 36.16 | 48.04 | 36.15 |
|  |  | SBFD | 81.34 | 110.45 | 135.40 | 107.60 | 55.73 | 69.51 | 76.10 | 68.10 |
|  | Median-UPT | Legacy TDD | 96.63 | 138.52 | 173.54 | 137.66 | 29.87 | 53.73 | 65.58 | 50.68 |
|  |  | SBFD | 103.93 | 155.87 | 178.09 | 149.57 | 50.33 | 59.73 | 69.12 | 60.42 |
| High load | Average-UPT | Legacy TDD | 42.08 | 60.09 | 69.62 | 58.74 | 13.52 | 22.29 | 30.40 | 22.30 |
|  |  | SBFD | 43.47 | 61.98 | 70.32 | 60.57 | 18.24 | 21.03 | 23.75 | 21.24 |
|  | Tail-UPT | Legacy TDD | 32.92 | 44.15 | 51.69 | 43.39 | 10.18 | 14.45 | 17.83 | 14.45 |
|  |  | SBFD | 35.31 | 45.49 | 53.13 | 44.49 | 16.88 | 18.49 | 20.04 | 18.50 |
|  | Median-UPT | Legacy TDD | 42.44 | 60.16 | 69.74 | 58.39 | 14.16 | 22.51 | 30.22 | 22.19 |
|  |  | SBFD | 44.25 | 62.27 | 71.46 | 59.88 | 19.88 | 21.73 | 26.24 | 22.28 |

Low load

Median load

High load

Figure 12: UPT results for urban macro

As can be seen from Table 14 and Figure 12, compared to legacy TDD, SBFD with Alt 4 can slightly improve the DL UPT at all the three load conditions and improve UL UPT at low/medium load conditions for urban macro. There is a decrease for the 95%-CDF of UL UPT (in case of low HARQ retransmission number) at high load condition.

Observation 12: For urban macro, compared to legacy TDD, SBFD with Alt 4 can significantly improve the DL UPT at all the three load conditions and improve UL UPT at low/medium load conditions for urban macro while there is a decrease for the 95%-CDF of UL UPT at high load condition.

  - Simulation results of SBFD configuration Alt 4 with small packet (4Kbytes for DL and 1Kbyte for UL)

    - Indoor office

Table 15: Latency and RU results for indoor office （0.5Mbyte for DL and 0.125 Mbytes for UL）

| Feature |  | DL latency (ms) |  |  |  |  |  | UL latency (ms) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) |  | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) |
| Low load | Legacy TDD | 0.52 | 0.74 | 0.97 | 0.80 | 6.12 | 7.65 |  | 0.52 | 0.75 | 0.98 | 0.77 | 1.51 | 7.57 |
|  | SBFD | 0.52 | 0.74 | 0.97 | 0.76 | 6.05 | 7.57 |  | 0.52 | 0.74 | 0.97 | 0.76 | 1.61 | 8.03 |
| Medium load | Legacy TDD | 0.52 | 0.74 | 0.97 | 0.80 | 19.87 | 24.84 |  | 0.51 | 0.75 | 0.97 | 0.77 | 5.68 | 28.40 |
|  | SBFD | 0.52 | 0.75 | 0.97 | 0.77 | 19.49 | 24.31 |  | 0.52 | 0.75 | 0.97 | 0.76 | 5.79 | 28.93 |
| High load | Legacy TDD | 0.52 | 0.76 | 0.99 | 0.88 | 41.34 | 51.67 |  | 0.52 | 0.75 | 0.97 | 0.76 | 10.35 | 51.75 |
|  | SBFD | 0.52 | 0.75 | 0.98 | 0.82 | 41.50 | 51.87 |  | 0.50 | 0.75 | 0.97 | 0.76 | 10.15 | 50.77 |

| Low load | Median load | High load |
| --- | --- | --- |

| Low load | Median load | High load |
| --- | --- | --- |

Low load                                                                Median load                                                           High load

Figure 13: Latency results for indoor office

As can be seen from Table 15 and Figure 13, it can be observed that SBFD has comparable DL/UL latency performance to legacy TDD at all the three load conditions.

Observation 13: For indoor office, SBFD has comparable DL/UL latency performance with legacy TDD at all the three load conditions.

Table 16: UPT results for indoor office

| Feature |  |  | DL UPT (Mbps) |  |  |  | UL UPT (Mbps) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | Mean | 5% | 50% | 95% | Mean |
| Low load | Average-UPT | Legacy TDD | 40.77 | 42.63 | 44.03 | 42.61 | 10.09 | 10.69 | 11.31 | 10.69 |
|  |  | SBFD | 41.67 | 42.74 | 44.07 | 42.77 | 10.05 | 10.66 | 11.46 | 10.71 |
|  | Tail-UPT | Legacy TDD | 32.34 | 32.87 | 33.51 | 32.62 | 8.05 | 8.23 | 8.63 | 8.22 |
|  |  | SBFD | 32.46 | 32.83 | 33.49 | 32.69 | 7.99 | 8.23 | 8.62 | 8.23 |
|  | Median-UPT | Legacy TDD | 40.77 | 42.63 | 44.03 | 42.61 | 10.09 | 10.69 | 11.31 | 10.69 |
|  |  | SBFD | 40.71 | 42.85 | 44.72 | 42.80 | 10.01 | 10.72 | 11.45 | 10.72 |
| Median load | Average-UPT | Legacy TDD | 40.77 | 42.63 | 44.03 | 42.61 | 10.09 | 10.69 | 11.31 | 10.69 |
|  |  | SBFD | 41.59 | 42.69 | 43.64 | 42.64 | 10.06 | 10.65 | 11.35 | 10.69 |
|  | Tail-UPT | Legacy TDD | 32.29 | 32.85 | 33.48 | 32.62 | 8.04 | 8.23 | 8.63 | 8.23 |
|  |  | SBFD | 32.36 | 32.88 | 33.51 | 32.67 | 8.02 | 8.22 | 8.65 | 8.22 |
|  | Median-UPT | Legacy TDD | 40.68 | 42.85 | 45.56 | 42.94 | 9.78 | 10.63 | 11.88 | 10.71 |
|  |  | SBFD | 41.06 | 42.95 | 44.68 | 42.92 | 9.78 | 10.62 | 11.82 | 10.72 |
| High load | Average-UPT | Legacy TDD | 14.76 | 40.93 | 43.63 | 38.80 | 10.11 | 10.67 | 11.37 | 10.68 |
|  |  | SBFD | 23.25 | 42.49 | 43.66 | 40.76 | 9.92 | 10.63 | 11.38 | 10.67 |
|  | Tail-UPT | Legacy TDD | 4.53 | 32.66 | 33.51 | 30.55 | 8.02 | 8.24 | 8.60 | 8.22 |
|  |  | SBFD | 9.60 | 32.75 | 33.30 | 30.44 | 8.01 | 8.25 | 8.63 | 8.24 |
|  | Median-UPT | Legacy TDD | 35.65 | 42.25 | 44.68 | 41.46 | 9.79 | 10.63 | 11.86 | 10.71 |
|  |  | SBFD | 36.48 | 42.44 | 44.42 | 41.85 | 9.83 | 10.65 | 12.07 | 10.72 |

Low load

Median load

High load

Figure 14: UPT results for indoor office

As can be seen from Table 16 and Figure 14, SBFD has comparable DL/UL UPT performance to legacy TDD at all the three load conditions.

Observation 14: For indoor office, SBFD has comparable DL/UL UPT performance with legacy TDD at all the three load conditions.

    - Urban macro

Table 17: Latency and RU results for Urban macro

| Feature |  | DL latency (ms) |  |  |  |  |  | UL latency (ms) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) |  | 5% | 50% | 95% | Mean | Type-1 RU (%) | Type-2 RU (%) |
| Low load | Legacy TDD | 0.52 | 0.75 | 0.97 | 0.81 | 6.15 | 7.69 |  | 0.52 | 0.75 | 0.97 | 0.76 | 1.72 | 8.62 |
|  | SBFD | 0.52 | 0.75 | 0.97 | 0.77 | 5.82 | 7.27 |  | 0.52 | 0.75 | 0.97 | 0.76 | 1.81 | 9.04 |
| Medium load | Legacy TDD | 0.52 | 0.75 | 0.97 | 0.81 | 19.27 | 24.09 |  | 0.52 | 0.74 | 0.97 | 0.76 | 5.77 | 28.83 |
|  | SBFD | 0.52 | 0.75 | 0.97 | 0.77 | 18.97 | 23.71 |  | 0.52 | 0.75 | 0.98 | 0.77 | 5.65 | 28.25 |
| High load | Legacy TDD | 0.52 | 0.78 | 5.58 | 1.23 | 41.18 | 51.48 |  | 0.52 | 0.77 | 3.31 | 1.10 | 10.26 | 51.31 |
|  | SBFD | 0.52 | 0.78 | 3.00 | 0.93 | 41.35 | 51.69 |  | 0.52 | 0.80 | 1.48 | 0.89 | 10.14 | 50.71 |

| Low load | Median load | High load |
| --- | --- | --- |

| Low load | Median load | High load |
| --- | --- | --- |

Figure 15: Latency results for urban macro

As can be seen from Table 17Table 13 and Figure 15, when comparing with the legacy TDD, it can be observed that SBFD shows significantly reduced DL/UL latency in case of 95%-CDF of DL/UL latency at high load condition and comparable UL/DL latency for the other case.

Observation 15: For urban macro, compared to legacy TDD, SBFD shows significantly reduced DL/UL latency in case of 95%-CDF of DL/UL latency at high load condition and comparable UL/DL latency for the other case.

Table 18: UPT results for urban macro

| Feature |  |  | DL UPT (Mbps) |  |  |  | UL UPT (Mbps) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5% | 50% | 95% | Mean | 5% | 50% | 95% | Mean |
| Low load | Average-UPT | Legacy TDD | 37.85 | 42.38 | 43.79 | 41.93 | 10.00 | 10.63 | 11.32 | 10.63 |
|  |  | SBFD | 41.46 | 42.67 | 44.03 | 42.63 | 9.83 | 10.61 | 11.38 | 10.62 |
|  | Tail-UPT | Legacy TDD | 32.28 | 32.82 | 33.61 | 32.59 | 8.05 | 8.23 | 8.58 | 8.22 |
|  |  | SBFD | 32.35 | 32.81 | 33.49 | 32.65 | 7.92 | 8.23 | 8.65 | 8.23 |
|  | Median-UPT | Legacy TDD | 40.50 | 42.74 | 44.54 | 42.65 | 9.56 | 10.58 | 11.79 | 10.65 |
|  |  | SBFD | 40.67 | 42.59 | 44.78 | 42.67 | 9.45 | 10.56 | 11.78 | 10.63 |
| Median load | Average-UPT | Legacy TDD | 30.14 | 42.36 | 43.63 | 41.04 | 10.05 | 10.66 | 11.35 | 10.69 |
|  |  | SBFD | 41.28 | 42.59 | 43.90 | 42.53 | 9.90 | 10.66 | 11.37 | 10.68 |
|  | Tail-UPT | Legacy TDD | 6.70 | 32.79 | 33.58 | 31.51 | 8.03 | 8.24 | 8.65 | 8.25 |
|  |  | SBFD | 32.36 | 32.82 | 33.44 | 32.65 | 7.93 | 8.26 | 8.76 | 8.26 |
|  | Median-UPT | Legacy TDD | 40.27 | 42.61 | 44.66 | 42.63 | 9.62 | 10.70 | 11.72 | 10.72 |
|  |  | SBFD | 40.60 | 42.68 | 44.81 | 42.65 | 9.57 | 10.75 | 11.89 | 10.73 |
| High load | Average-UPT | Legacy TDD | 7.38 | 38.06 | 43.60 | 31.65 | 1.49 | 9.53 | 10.86 | 8.32 |
|  |  | SBFD | 16.55 | 41.27 | 43.53 | 36.14 | 6.36 | 9.29 | 10.87 | 9.11 |
|  | Tail-UPT | Legacy TDD | 4.15 | 32.24 | 33.28 | 21.23 | 0.75 | 2.47 | 8.36 | 4.74 |
|  |  | SBFD | 9.00 | 32.45 | 33.23 | 24.54 | 3.23 | 5.58 | 8.29 | 5.97 |
|  | Median-UPT | Legacy TDD | 6.03 | 41.29 | 44.39 | 37.52 | 1.74 | 9.75 | 11.22 | 8.99 |
|  |  | SBFD | 26.31 | 41.36 | 44.17 | 39.46 | 6.69 | 9.96 | 11.08 | 9.66 |

Low load

Median load

High load

Figure 16: UPT results for urban macro

As can be seen from Table 18 and Figure 16, compared to legacy TDD, it can be observed that SBFD achieves DL/UL UPT performance gain at high load condition, especially in the case of 5%-CDF of DL UPT (multiple HARQ retransmissions needed). And SBFD shows comparable UL/DL UPT performance at low/medium load condition besides the 5%-CDF of DL UPT.

Observation 16: SBFD achieves DL/UL UPT performance gain at high load condition and comparable UL/DL UPT at low/medium load condition beside the 5%-CDF of DL UPT.

- Conclusions

In this contribution, further analyses and views on deployment scenarios, evaluation methodologies and evaluation assumptions for Rel-18 duplex enhancement are provided. Our proposals and observations are:

Proposal 1: LLS for other purpose besides coverage performance evaluation is up to companies’ interests.

Proposal 2: Wait for RAN4 conclusion on Rx impairment model of UE-UE co-channel inter-subband CLI modelling.

Proposal 3: Adopt the ACIR defined in Table 5.2.1.2-1 in TR38.828 for FR1 for SBFD deployment case 4.

Proposal 4: Adopt the ACLR and ACS defined in Table 5.2.2.2-1 in TR38.828 for FR2 for SBFD deployment case 4.

Observation 1: For indoor office, compared to legacy TDD, SBFD with Alt 1 can significantly reduce the UL latency at the cost of slightly increased DL latency.

Observation 2: For indoor office, compared to legacy TDD, SBFD with Alt 1 achieves better UL user throughput at all three load conditions at the cost of decreased DL user throughput.

Observation 3: For urban macro, compared to legacy TDD, SBFD with Alt 1 can significantly reduce the UL latency at all three load conditions with the cost of slightly increased DL latency.

Observation 4: For urban macro, compared to legacy TDD, SBFD with Alt 1 achieves better UL user throughput at all three load conditions at the cost of slightly decreased DL user throughput.

Observation 5: For indoor office, SBFD has comparable DL and UL latency performance with legacy TDD besides DL latency at high load for Alt 1 with small packet.

Observation 6: For indoor office, SBFD with Alt 1 with small packet shows comparable UL/DL user throughput with legacy TDD except that it shows worse DL user throughput performance at high load.

Observation 7: For urban macro, SBFD with Alt 1 with small packet shows comparable DL and UL latency with legacy TDD except that it shows significant performance gain at 95%-CDF of DL/UL latency at high load condition.

Observation 8: For urban macro, compared to legacy TDD, SBFD with Alt 1 with small packet achieves better UL user throughput at high load condition at the cost of decreased DL user throughput except 5% CDF of DL UPT.

Observation 9: For indoor office, compared to legacy TDD, SBFD with Alt 4 can reduce DL latency slightly at all three low loads and reduce UL latency at low and median loads.

Observation 10: For indoor office, compared to legacy TDD, compared to legacy TDD, SBFD with Alt 4 can improve the UL UPT at low/medium load conditions and DL UPT at all load conditions.

Observation 11: For urban macro, compared to legacy TDD, SBFD with Alt 4 can reduce the DL latency at all three load conditions for urban macro and UL latency at low/medium load condition.

Observation 12: For urban macro, compared to legacy TDD, SBFD with Alt 4 can significantly improve the DL UPT at all the three load conditions and improve UL UPT at low/medium load conditions for urban macro while there is a decrease for the 95%-CDF of UL UPT at high load condition.

Observation 13: For indoor office, SBFD has comparable DL/UL latency performance with legacy TDD at all the three load conditions.

Observation 14: For indoor office, SBFD has comparable DL/UL UPT performance with legacy TDD at all the three load conditions.

Observation 15: For urban macro, compared to legacy TDD, SBFD shows significantly reduced DL/UL latency in case of 95%-CDF of DL/UL latency at high load condition and comparable UL/DL latency for the other case.

Observation 16: SBFD achieves DL/UL UPT performance gain at high load condition and comparable UL/DL UPT at low/medium load condition beside the 5%-CDF of DL UPT.

- Reference

- Chairman's Notes RAN1#110bis-e, 3GPP TSG RAN WG1 Meeting #110bis-e, October 10th – 19th, 2022.

- R4-2218481,The remaining issues for UE modelling in SBFD SLS, CATT, 3GPP TSG RAN WG4 Meeting #105, Toulouse, France, November 14 – November 18, 2022

- Annex

Table Annex-1. System-level simulation assumption for NR Full Duplex

| Parameters | Indoor office | Urban macro |  |
| --- | --- | --- | --- |
| Layout | Single layer 12BSs per 120m x 50m | Single layer Macro layer: Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |  |
| Carrier frequency | 4G |  |  |
| Deployment case | Case 1 |  |  |
| BS antenna configuration | SBFD antenna configuration option-2 : = (4,4,2,1,1; 4,4) = (0.5, 0.5)λ, +45°/-45° polarization | SBFD antenna configuration option-2 : = (8,8,2,1,1;2,8) = (0.5, 0.8)λ, +45°/-45° polarization |  |
| BS Tx power | 24dBm (38.901) | 49 dBm (38.901) |  |
| UE antenna height | 1.0m | Same as 3D-UMa in TR36.873 |  |
| Traffic model | FTP traffic model 3 Each UE is assigned both UL traffic and DL traffic Asymmetric packet size: 0.5Mbyte for DL and 0.125 Mbytes for UL UL Traffic load: low UL RU (<10%), medium UL RU (20%-30%), and high UL RU ([≥50%]). DL Traffic load: low DL RU (<10%), medium DL RU (20%-30%), and high DL RU (≥50%). Note: Type-2 RU definition (calculated per link direction) is used |  |  |
| UE-to-UE Channel model | Large-scale channel parameters: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA |  | Large-scale channel parameters: UMi-Street canyon in TR 38.901(hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 in TR38.802, ASD and ZSD statistics updated to be the same as ASA and ZSA. |
| Transmission mode | SU-MIMO |  |  |
| Channel estimation | Realistic |  |  |
| Overhead | No extra overhead |  |  |