3GPP TSG-RAN WG1 Meeting #112	R1-2300914

Athens, Greece, February 27th – March 3rd, 2022

Agenda Item:	8.2

Source:	Ericsson

Title:	Discussion on TCI state indication for multiple PDSCHs scheduled by single DCI

Document for:	Discussion, Decision

# Introduction

In this contribution we discuss a correction under "Maintenance on Supporting NR from 52.6GHz to 71 GHz." The corresponding draft CR is in [1].

# 2	Discussion

In 38.214 Section 5.1.5, square brackets still exist around the RRC parameter pdsch-TimeDomainAllocationListForMultiPDSCH-r17. In our understanding, there is no open issue regarding TCI state indication for multiple PDSCHs scheduled by single DCI, and the RRC parameter name has been stable in 38.331 for multiple updates of the specification during maintenance phase. Without removing the square brackets, the UE behavior is unclear for determing applicable TCI state in the case of multiple-PDSCHs scheduled by single DCI.

Based on the above discussion, we propose a correction to 38.214 Section 5.1.5. The text of the CR is copied into the Appendix of this document for convenience.

Adopt draft CR to 38.214 Section 5.1.5 in [1]

# Conclusion

In this paper we propose the following:

Proposal 1	Adopt draft CR to 38.214 Section 5.1.5 in [1]

# References

R1-2300915, “Draft CR on TCI state indication for multiple PDSCHs scheduled by single DCI”, Ericsson, RAN1#112, February 2022.

# Appendix

*** Unchanged text omitted ***

### 5.1.5	Antenna ports quasi co-location

*** Unchanged text omitted ***

If a PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. For a single slot PDSCH, the indicated TCI state(s) should be based on the activated TCI states in the slot with the scheduled PDSCH. For a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state(s) should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH(s). When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL.

*** Unchanged text omitted ***