3GPP TSG-RAN WG1 Meeting #112	R1-2300916

Athens, Greece, February 27th – March 3rd, 2022

Agenda Item:	8.2

Source:	Ericsson

Title:	Discussion on TDRA for multiple PUSCHs scheduled by single DCI

Document for:	Discussion, Decision

# Introduction

In this contribution we discuss a correction under "Maintenance on Supporting NR from 52.6GHz to 71 GHz." The corresponding draft CR is in [1].

# 2	Discussion

In Table 6.1.2.1.1-1A in 38.214, reference is made to the non-existent RRC parameter pusch-TimeDomainAllocationListForMultiPUSCH-17 for the case of multiple PUSCHs scheduled by a single DCI. This is due to the fact that RAN2 decided to reuse the parameter pusch-TimeDomainAllocationListForMultiPUSCH-r16 for multi-PUSCH scheduling with single DCI for both Rel-16 and Rel-17 functionality instead of introducing a new Rel-17 parameter.

In addition in the field descriptions for the PUSCH-Config IE, 38.331 states that the parameter  pusch-TimeDomainAllocationList for single-PUSCH scheduling shall not be configured simultaneously with pusch-TimeDomainAllocationListForMultiPUSCH. This conflicts with the final row of Table 6.1.2.1.1-1A which indicates “No/Yes” for the configuration of  pusch-TimeDomainAllocationList.

Both issues result in unclear UE behavior for determining applicable TDRA table in the case of multi-PUSCHs scheduled by single DCI.

Based on the above discussion, we propose corrections to Table 6.1.2.1.1-1A in 38.214. The text of the CR is copied into the Appendix of this document for convenience.

Adopt draft Rel-17 CR to 38.214 Section 6.1.2.1.1 in [1]

We observe that the same “No/Yes” problem in Table 6.1.2.1.1-1A also exists in Rel-16 specifications (see 38.214 v16.12.0), hence we think  a Rel-16 CR is needed to have consistency between 38.331 and 38.214.

RAN1 to discuss  a Rel-16 CR for adopting similar changes as the proposed Rel-17 CR.

# Conclusion

In this paper we propose the following:

Proposal 1	Adopt draft Rel-17 CR to 38.214 Section 6.1.2.1.1 in [1]

Proposal 2	RAN1 to discuss  a Rel-16 CR for adopting similar changes as the proposed Rel-17 CR.

# References

R1-2300917, “Draft CR on TDRA for multiple PUSCHs scheduled by single DCI”, Ericsson, RAN1#112, February 2022.

# Appendix

*** Unchanged text omitted ***

##### 6.1.2.1.1	Determination of the resource allocation table to be used for PUSCH

*** Unchanged text omitted ***

Table 6.1.2.1.1-1A: Applicable PUSCH time domain resource allocation for DCI format 0_1 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI

Table 6.1.2.1.1-1A: Applicable PUSCH time domain resource allocation for DCI format 0_1 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationListDCI-0-1 | pusch-Config includes pusch-TimeDomainAllocationList-ForMultiPUSCH or pusch-Config includes pusch-TimeDomainAllocationListForMultiPUSCH-17 | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| No | No | No | No | Default A |
| Yes | No | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No | Yes | - | pusch-TimeDomainAllocationListDCI-0-1 provided in pusch-Config |
| No/Yes | No/Yes | - | Yes | pusch-TimeDomainAllocationList-ForMultiPUSCH or pusch-TimeDomainAllocationListForMultiPUSCH -17 provided in pusch-Config |

*** Unchanged text omitted ***