3GPP TSG RAN WG1 #112		                                                    R1-2300932

Athens, Greece, February 27th – March 3rd, 2023

Source:	Intel Corporation

Title:	On Unified TCI Framework for multi-TRP

Agenda item:	9.1.1.1

Document for:	Discussion and Decision

# Introduction

In this paper, we present a discussion on extension of Rel-17 unified TCI framework to multi-TRP transmission/reception based on the MIMO WI objectives [1].

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In Rel-17 NR, unified TCI framework was introduced to unify beam indication for downlink and uplink using joint DL/UL TCI states for cases when downlink and uplink use the same beam and with separate DL/UL TCI for the case when beam correspondence is not assumed with different beams for downlink and uplink. The unified TCI framework assumes a common beam operation in DL and UL where the same beam is used across all DL/UL signals and channels unless specific RS and channels are configured not to apply the indicated TCI state. In Rel-17, the framework was specified only for single TRP case with M=1 downlink QCL and N=1 uplink spatial filter being indicated by each TCI codepoint i.e., MAC-CE maps a single joint TCI state or a single DL and/or a single UL TCI state to a TCI codepoint.

In Rel-18 unified TCI framework should be extended to multi-TRP transmission schemes. In this paper, we discuss views on extension of unified TCI framework to Rel-16 and Rel-17

# Issue 2 – TCI State Update and Activation

## Single-DCI Multi-TRP

The following agreements were made for TCI state update of S-DCI in RAN1#111

Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. Agreement On unified TCI framework extension for S-DCI based MTRP, a new indicator field is supported as the DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - FFS: Detail design of the new indicator field

Based on the agreements, a new DCI field will be configured by RRC and if present, this field will indicate which or both of the indicated TCI states to apply for S-DCI mTRP. In Rel-17, S-DCI based mTRP supported dynamic switching between schemes based on the number of indicated TCI states and the number of DM-RS CDM groups and it can be illustrated as follows.

Figure 1: Dynamic switching in legacy Rel-16 NR between sTRP and mTRP schemes

It would be preferable to keep the same dynamic switching functionality in Rel-18 as well. Therefore, the function of the indicator field and S-DCI based switching between single-TRP and multi-TRP schemes need to be considered together.

- For S-DCI based PDSCH, dynamic switching between sTRP and different mTRP schemes as specified in Rel-17 should be supported.

### DCI Indicator Field is Present

When the DCI field is configured to be present in the scheduling DCI, the field may indicate which one or both of the indicated TCI states are to be applied. This agreement then implies that switching between single TRP and mTRP schemes are handled using this new DCI field when the indicated TCI codepoint is mapped to two TCI states. On the other hand, if the UE is indicated with a TCI codepoint mapped to a single TCI state, then there are two alternatives i.e., (i) the DCI indicator field (if configured) is ignored and sTRP operation is assumed as in legacy case (ii) the new DCI indicator field values are reinterpreted such that the field indicates if the indicated single TCI state is the first indicated TCI, the second indicated TCI for mTRP operation or sTRP is assumed. The first alternative is illustrated in Figure 2.

Figure 2: Dynamic switching of sTRP and mTRP schemes using number of indicated TCI states and new DCI indicator field

If the second alternative is assumed, then the sTRP vs. mTRP switching can always be handled by the new DCI indicator field i.e., if single TCI state is indicated, the UE can update one of the two TCI states for mTRP operation based on the value of the field.

- If the new DCI indicator field is configured to be present by RRC for S-DCI based mTRP, dynamic switching between sTRP and mTRP schemes can be supported based on the number of indicated TCI states and the value of the indicator field with one of the following alternatives

  - When a codepoint with two TCI states is indicated, the value of the indicator field and number of CDM groups switched between sTRP and mTRP schemes

  - When a codepoint with single TCI state is indicated

    - Alt-1: sTRP operation is assumed

    - Alt-2: The value of the DCI codepoint is reinterpreted to switch between sTRP and mTRP operation where in mTRP only one of the TCI states are updated with the new indicated TCI state based on the value of DCI indicator field

### DCI Indicator Field is not Present

For this case, the switching can again have a few options. The first option is to fall back to legacy switching mechanism as shown in Figure 1.

- If the DCI indicator field is not present, legacy switching mechanism for S-DCI based mTRP can be used.

In this case, if the DCI indicates a codepoint mapped with two TCI states, the mapping of TCI states to MAC-CE codepoints can implicitly determine whether the codepoint indicated by DCI is applicable to one or both TRPs i.e., codepoints with single TCI states correspond to sTRP. In addition, in order to specify the TRP to which the beam indication is applicable in the case when a codepoint is mapped to a single TCI state, the MAC-CE can also include an ID similar to coresetPoolIndex or a TCI group ID, which can specify the TRP (first or second) to which the activated TCI state may apply.

Figure 3: TCI State Grouping for S-DCI based MTRP (left) Implicit grouping according to activated TCI states (right) explicit TCI group ID configured by TCI activation MAC-CE which support single or two TCI states configured to a codepoint

The two alternatives are shown in Figure 3. This functionality can also be useful, as discussed in Section 5, for beam failure detection and recovery.

- The MAC-CE for TCI state activation can configure one or two TCI states to each codepoint with an additional index (0 or 1) corresponding to a TCI state group which can indicate which TRP the TCI codepoint applicable to in the case of beam indication for one of the two TRPs for S-DCI based MTRP

### Beam Application Time

When unified TCI framework is used for beam indication for single DCI multi-TRP, for the case when the UE is configured with joint DL/UL beam indication with the value of unifiedtci-StateType set to “JointULDL”, the UE expects to be indicated with a TCI codepoint which is mapped to two joint DL/UL TCI states, one for each TRP. The legacy Rel-16/17 MAC-CE can be used to map the joint DL/UL TCI states to the TCI codepoints. When configured for separate DL/UL beam indication with the value of unifiedtci-StateType set to “SeparateULDL”, the UE expects to be indicated with a TCI codepoint which is mapped to two (M=2) DL TCI states or two (N=2) UL TCI states or two DL + two UL TCI states. For the beam indication, the beam application time needs to be determined based on configured BAT across the TRPs and CCs.

- The beam application time for single-DCI multi-TRP can be configured per CC for all TRPs and is determined based on the smallest SCS among the CCs from the TRPs which apply the indicated beams.

## Multi-DCI Multi-TRP

A default beam for each CORESETPoolIndex is applicable after initial access and before the first slot boundary after beam application time after the UE receives a first beam indication DCI for the corresponding CORESETPoolIndex. Once the UE receives a beam application DCI corresponding to a CORESETPoolIndex, the indicated beam, once applied is assumed to be active until a different TCI state is indicated either by DCI or MAC-CE.

- Default beam for multi-DCI multi-TRP is applicable after initial access and before a first beam indication. After BAT of first beam indication, the indicated TCI per CORESETPoolIndex remains active until a different TCI is indicated per CORESETPoolIndex.

# Issue 3 – TCI State Association With Target Channels/Signals

## Multi-DCI Multi-TRP

### PUCCH Transmission for Multi-DCI Multi-TRP

On this note, for PUCCH transmission, in Rel-17 the indicated joint/UL TCI state is applicable to all configured PUCCH resources within a CC. But for M-DCI based MTRP operation, some more clarifications are needed especially for the case when, for example, joint HARQ feedback is configured for M-DCI. The scheduling DCI should update the PUCCH beam for only a subset of PUCCH resources corresponding to a particular TRP. Otherwise the PUCCH beam would always follow the indicated TCI corresponding to the last received DCI. To solve this issue, a PUCCH resource or resource group can be configured by RRC to be associated with a specific value of the coresetPoolIndex. When a DCI provides a new TCI indication, the PUCCH resources associated with the coresetPoolIndex corresponding to the DCI are the ones for which the indicated TCI state is applicable. The PUCCH resources associated with the other coresetPoolIndex continues to follow the previously indicated TCI state or default TCI state whichever is applicable.

- For PUCCH transmission in M-DCI based MTRP, PUCCH resource/group should be configured by RRC to be associated with a specific value of coresetPoolIndex and when a beam indication DCI is received, the PUCCH resources corresponding to coresetPoolIndex associated with the DCI should apply the indicated TCI state.

## Single-DCI Multi-TRP

### PDSCH Reception for Single-DCI Multi-TRP

The following proposal has been discussion offline

FL Proposal 3.1: On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the 1st indicated joint/DL TCI state to the scheduled/activated PDSCH reception - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the 2nd indicated joint/DL TCI state to the scheduled/activated PDSCH reception If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both the 1st and the 2nd indicated joint/DL TCI states to the scheduled/activated PDSCH reception

While this proposal is OK in principle, if dynamic switching between sTRP and mTRP is considered for the case of single indicated TCI state, the fields of the indicator might need to be reinterpreted when single TCI state is indicated.

- FL Proposal 3.1 should apply when the indicated TCI codepoint is mapped to two DL/joint TCI states. When the indicated codepoint is mapped to single TCI state, DCI indicator field value ‘00’ indicates that the TCI is the first indicated TCI in mTRP, ‘01’ indicates the TCI is 2nd indicated TCI in mTRP and ‘10/11’ can indicate sTRP mode.

If UE is indicated with two TCI states and the scheduling DCI does not have the new indicator field or if the PDSCH is received before a threshold, the default behaviour needs to be specified. In this case, since two indicated TCI states are active, it may be reasonable for the UE to apply both the TCI states for buffering the received signal.

- If the DCI field is not present or if the PDSCH is received before a threshold, the UE should apply both the indicated joint/DL TCI states to buffer the received signal.

### PUSCH Transmission for Single-DCI Multi-TRP

For PUSCH transmission, firstly DCI formats 0_1/0_2 should be enabled for beam indication which was a missing functionality in Rel-17 unified TCI design. Using a separate DCI for beam indication and then the scheduling DCI to signal association of the indicated TCI the transmission incurs unnecessary latency. This is true especially for the case when separate DL/UL TCI mode is in use.

- For single-DCI mTRP PUSCH transmission, use DCI format 0_1/0_2 for both beam indication and for informing association of TCI with PUSCHs.

### Multi-TRP Repetition Schemes

#### PDSCH Repetition Scheme 3 (TDMSchemeA) & Scheme 4 (TDMSchemeB)

When the time offset between the DCI and the 1st PDSCH transmission occasion is below the BAT when the scheduling DCI indicates the beams, and before the UE has received the first beam indication DCI with a TCI codepoint which is mapped to two joint DL/UL TCI states, or 2 DL + 2 UL TCI states, two default beams are applied where the default beams for each TRP are determined based on the activated TCI codepoints in the slot with the first PDSCH transmission and the lowest indexed TCI codepoint which is mapped to 2 joint DL/UL or 2 DL + 2 UL TCI states is considered to be the default beam.

- For PDSCH Repetition Schemes 3 and 4, default beam is based on the activated TCI codepoints in the slot with the first PDSCH transmission and is the lowest indexed TCI codepoint which is mapped to 2 joint DL/UL TCI states (joint DL/UL beam indication) or 2 DL and 2 UL TCI states (separate DL/UL beam indication).

#### PDCCH Repetition for Multi-TRP

Figure 4: PDCCH Repetition for Rel-17 Multi-TRP Operation with SS set linking

Rel-17 PDCCH repetition for multi-TRP operation is illustrated in Figure 4. For PDCCH repetition in Rel-17 multi-TRP, a DCI is repeated from two TRPs wherein each repetition is from a SS set linked to CORESET which associated with a TCI state. In Figure 1, PDCCH-1 and PDCCH-2 are repetitions of the same DCI. Candidate m of SS set-1 is linked with candidate m of SS-set-2 for a given AL, using RRC signaling.

For support of PDCCH repetition using unified TCI framework, CORESET-1 and CORESET-2 can be configured to apply the indicated joint or DL TCI state(s) only for the case when the indicated TCI codepoint maps to 2 joint or DL TCI states. The first TCI state in the codepoint applies to CORESET-1 and second TCI state in the codepoint maps to CORESET-2.

- For PDCCH repetition, the two CORESETs apply indicated TCI state(s) only when the TCI codepoint is mapped to two joint or DL TCI states. The first TCI state applies to CORESET 1 and the second TCI state applies to CORESET-2.

#### PUCCH Repetition for Multi-TRP

Intra-Slot Repetition

Inter-Slot Repetition

Figure 5: PUCCH Repetition Schemes for Rel-17 Multi-TRP operation

In Rel-17 multi-TRP, PUCCH repetition schemes were specified with both intra and inter slot repetition. When unified TCI framework is used to support PUCCH repetitions, PUCCH resources or resource sets can be configured to follow the indicated Rel-17 joint or UL TCI state and a beam indication DCI with format 1_1/1_2 (with or without data assignment) or 0_1/0_2 with UL data assignment can indicate a TCI codepoint mapped to two joint or UL TCI states and the first TCI state of the codepoint maps to the first PUCCH repetition and the second TCI state in the codepoint maps to the second PUCCH repetition and this pattern repeats for every subsequent repetition

- For PUCCH repetition, DCI 0_1/0_2 can indicate a TCI codepoint mapped to two joint or UL TCI states and the first TCI state of the codepoint maps to the first PUCCH repetition and the second TCI state in the codepoint maps to the second PUCCH repetition.

# Issue 5 – Unified TCI for Coherent JT

For support of CJT, it was agreed in RAN1#110bis-e that only up to 2 joint TCI states will be supported based on UE capability with FFS on QCL assumptions for the two joint TCI states. In general, we prefer to support per-TRP TRS transmissions and for joint 2 TCI States, Rel-17 PDSCH-SFN scheme A can be used. When more than 1 unified TCI is applied to CJT schemes, further discussion is needed in RAN1 on assumptions for channels other than PDSCH, since unified TCI states are applicable to multiple channels in DL and/or UL.

- Support TRS transmission from up to 2 TRPs corresponding to two joint TCI states for CJT

- For CJT operation with 2 joint DL/UL or separate DL TCI states, further discussion is need on TCI assumptions for other physical channels than PDSCH for the indicated TCI states.

# Issue 6 – Beam Reporting And BFR

For study of MTRP BFR under the unified TCI framework, the following agreement was made in RAN1#110bis-e

Agreement On unified TCI framework extension, study the following enhancements for TRP-specific BFR: - Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP - Enhancement to beam update after NW response to TRP-specific BFR request

Implicit BFD-RS may be supported for S-DCI based MTRP. It has been previously discussed if the concept of coresetPoolIndex should also be extended to S-DCI for this purpose. To this end, we think that instead of coresetPoolIndex, a new ID may be used e.g., a TCI state group ID or another explicit ID (0 or 1) which the TCI state activation MAC-CE associates with each TCI codepoint. In case every TCI codepoint is mapped to two TCI states, the set of first TCI states can be considered as one TCI state group and the set of second TCI states can be considered as a second TCI state group. For codepoint with single TCI state mapped, the value of ID identifies the TCI state group. The UE can assume that the TCI states in the TCI state groups form BFD-RS sets corresponding to each TRP. In general, in order to not violate the UE capability of BFD-RS tracking, the first N TCI states in each group can be chosen as implicit BFD-RSs.

- Allow implicit BFD-RS determination based on activated joint/DL TCI states for S-DCI based MTRP.

# Unified TCI Framework for STxMP

## Panel-ID reporting:

| Figure 6: UL STxMP Tx - L1-RSRP with panel selection (2Rx) | Figure 7: UL single TRP Tx - L1-RSRP using all panels (4Rx) |
| --- | --- |

As listed above, different STxMP schemes and transmission scenarios have been identified to study. In order to decide whether to schedule STxMP UL transmission or single-panel UL transmission and also to decide pairs of TCI-states that are compatible for STxMP, the gNB should be informed about the association between the UE panels and the gNB transmission beams and the corresponding L1-RSRPs. This can be done by the following steps.

- Step-0: gNB transmits SSBs/CSI-RSs in different CSI resource sets using different TRPs.

- Step-1: In a groupBasedBeamReporting setting (Figure 6), the UE measures the SSB/CSI-RS received from the best selected panel. Then, the UE reports L1-RSRPs in a CSI report, where each L1-RSRP is associated with a panel-ID indicating which panel is used for L1-RSRP measurement. In a non- groupBasedBeamReporting setting (Figure 7), the UE measures the received the SSB/CSI-RS from all panels. Then, the UE reports L1-RSRP in a CSI report

- Step-2: According to the L1-RSRP value pairs with panel IDs, the gNB can determine pairs of TCI states that are compatible for STxMP. Comparing L1-RSRP reports for the same Tx beam with groupBasedBeamReporting switched on or off, it can decide whether to schedule a STxMP transmission or a single-panel UL transmission.

Furthermore, as agreed in Rel-17 unified TCI framework discussion, a list of UE capability value sets can be reported by the UE to facilitate UE-initiated panel activation and selection, where each UE capability value set comprises the max supported number of SRS ports and any two capability values sets are different. To be specific, for L1-RSRP reporting, when the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRPCapability[Set]Index' or 'ssb-Index-RSRP-Capability[Set]Index' an index of UE capability value set, indicating the maximal supported number of SRS antenna ports, is reported along with the pair of SSBRI/CRI and L1-RSRP. And L1-SINR reporting follows the same mechanism.

This mechanism can be enhanced to support the UE panel indication for STxMP transmission. For example, different UE capability indices can be mapped to different SRS port groups which may have the same or different maximal supported number of SRS antenna ports. And such a UE capability index pair can be reported along with a pair of SSBRI/CRI and L1-RSRP.

- To support STxMP operation with panel-id reporting, the Rel-17 UE capability value sets can be extended to the case of same maximal supported number of SRS ports (per panel).

# Conclusion

In this paper, unified TCI extension to multi-TRP operation and simultaneous multi-panel UL transmission was discussed. The main proposals from this paper are outlined here:

- For S-DCI based PDSCH, dynamic switching between sTRP and different mTRP schemes as specified in Rel-17 should be supported.

- If the new DCI indicator field is configured to be present by RRC for S-DCI based mTRP, dynamic switching between sTRP and mTRP schemes can be supported based on the number of indicated TCI states and the value of the indicator field with one of the following alternatives

  - When a codepoint with two TCI states is indicated, the value of the indicator field and number of CDM groups switched between sTRP and mTRP schemes

  - When a codepoint with single TCI state is indicated

    - Alt-1: sTRP operation is assumed

    - Alt-2: The value of the DCI codepoint is reinterpreted to switch between sTRP and mTRP operation where in mTRP only one of the TCI states are updated with the new indicated TCI state based on the value of DCI indicator field

- If the DCI indicator field is not present, legacy switching mechanism for S-DCI based mTRP can be used.

- The MAC-CE for TCI state activation can configure one or two TCI states to each codepoint with an additional index (0 or 1) corresponding to a TCI state group which can indicate which TRP the TCI codepoint applicable to in the case of beam indication for one of the two TRPs for S-DCI based MTRP

- The beam application time for single-DCI multi-TRP can be configured per CC for all TRPs and is determined based on the smallest SCS among the CCs from the TRPs which apply the indicated beams.

- Default beam for multi-DCI multi-TRP is applicable after initial access and before a first beam indication. After BAT of first beam indication, the indicated TCI per CORESETPoolIndex remains active until a different TCI is indicated per CORESETPoolIndex.

- For PUCCH transmission in M-DCI based MTRP, PUCCH resource/group should be configured by RRC to be associated with a specific value of coresetPoolIndex and when a beam indication DCI is received, the PUCCH resources corresponding to coresetPoolIndex associated with the DCI should apply the indicated TCI state.

- FL Proposal 3.1 should apply when the indicated TCI codepoint is mapped to two DL/joint TCI states. When the indicated codepoint is mapped to single TCI state, DCI indicator field value ‘00’ indicates that the TCI is the first indicated TCI in mTRP, ‘01’ indicates the TCI is 2nd indicated TCI in mTRP and ‘10/11’ can indicate sTRP mode

- If the DCI field is not present or if the PDSCH is received before a threshold, the UE should apply both the indicated joint/DL TCI states to buffer the received signal

- For single-DCI mTRP PUSCH transmission, use DCI format 0_1/0_2 for both beam indication and for informing association of TCI with PUSCHs

- For PDSCH Repetition Schemes 3 and 4, default beam is based on the activated TCI codepoints in the slot with the first PDSCH transmission and is the lowest indexed TCI codepoint which is mapped to 2 joint DL/UL TCI states (joint DL/UL beam indication) or 2 DL and 2 UL TCI states (separate DL/UL beam indication).

- For PDCCH repetition, the two CORESETs apply indicated TCI state(s) only when the TCI codepoint is mapped to two joint or DL TCI states. The first TCI state applies to CORESET 1 and the second TCI state applies to CORESET-2

- For PUCCH repetition, DCI 0_1/0_2 can indicate a TCI codepoint mapped to two joint or UL TCI states and the first TCI state of the codepoint maps to the first PUCCH repetition and the second TCI state in the codepoint maps to the second PUCCH repetition

- Support TRS transmission from up to 2 TRPs corresponding to two joint TCI states for CJT

- For CJT operation with 2 joint DL/UL or separate DL TCI states, further discussion is need on TCI assumptions for other physical channels than PDSCH for the indicated TCI states.

- Allow implicit BFD-RS determination based on activated joint/DL TCI states for S-DCI based MTRP.

- To support STxMP operation with panel-id reporting, the Rel-17 UE capability value sets can be extended to the case of same maximal supported number of SRS ports (per panel).

# References

- RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, TSG RAN Meeting #94e, Samsung

- Chairman’s Notes, 3GPP RAN1#110bis-e Meeting, October 2022.

- Chairman’s Notes, 3GPP RAN1#111 Meeting, November 2022.

- 3GPP TS 38.211 v17.0.0, NR Physical Channels and Modulation (Release 17)

- 3GPP TS 38.212 v17.0.0, NR Multiplexing and Channel Coding (Release 17)

- 3GPP TS 38.213 v17.0.0, NR Physical Layer Procedures for Control (Release 17)

- 3GPP TS 38.214 v17.0.0, NR Physical Layer Procedures for Data (Release 17)