3GPP TSG RAN WG1 #112                                                                 R1-2300934

Athens, Greece, February 27th – March 3rd, 2023

Source:	Intel Corporation

Title:	On CSI enhancements

Agenda item:	9.1.2

Document for:	Discussion and Decision

# Introduction

At the 3GPP TSG RAN #94e meeting the new WI on MIMO Evolution for Downlink and Uplink [1] were agreed. Two objectives of the WI correspond to the CSI enhancements including CSI enhancements for high/medium UE velocities and CSI enhancements for Coherent Joint Transmission (CJT).

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

In this contribution aspects related to the CSI enhancements for high/medium UE velocities and CSI enhancements for CJT are discussed.

# Discussion

## CSI enhancements for high/medium UE velocities

At the previous RAN1 meetings significant progress was achieved on new PMI codebook design with Doppler Domain (DD) compression. In this section, remaining issues on the PMI codebook are discussed including parameters for PMI codebook with Doppler-Domain (DD) compression, coefficient selection and reporting, CQI design, etc. Also, our view on the Time Domain Channel Properties (TDCP) reporting is presented.

### Parameters for PMI codebook with Doppler-Domain (DD) compression

New PMI codebook with DD compression has many different parameters including legacy parameters (e.g., L, pv, beta) as well as new parameters (e.g., N4, m, delta, d, Q, K). The proper configuration of parameters is critical to achieve the desired performance-overhead trade-off considering different aspects of the system. Thus, it is important to support reasonable range of values for the PMI codebook parameters.

For the PMI codebook with DD compression, PMI is calculated for N4 DD units. At RAN1#111 it was agreed that the range for the parameter N4 values is {1, 2, 4, 8} with additional candidate values {3, 5, 16, 32}. Since channel prediction is used to calculate PMI, larger N4 values correspond to more significant channel prediction errors comparing to lower N4 values assuming the same DD unit size (d). Considering that the DD unit size (d) and CSI-RS periodicity are selected to approach the Nyquist limit considering the maximum UE speed in the deployment, it may be hard to do accurate channel prediction for larger number of DD units for a practical channel prediction algorithm. Thus, the maximum value of N4 equal to 8 is reasonable in our view. For the intermediate N4 values (N4 = {3, 5}), we don’t see a need to support it since lager N4 values can be considered with omission of DD units at the gNB, if needed.

Proposal 1:

- The range of supported values for N4 is {1, 2, 4, 8} DD units

  - Support of larger values should be justified considering performance loss due to non-ideal channel prediction

It may be important to align the assumption on the slot indexes for the channel prediction at the UE and the application of the corresponding precoder from the predicted channel at the gNB. There is a delay between the CSI report transmission from the UE and usage of the corresponding RI, PMI and CQI which is needed for decoding of the UCI and reconstruction of the precoding matrix based on the received PMI bits. This delay can be different for different gNBs. Hence, configuration of parameter which reflects the delay is required. Parameter delta corresponds to the offset of CSI reporting window (time window for channel prediction) from the CSI report. It was agreed that the range of supported values for parameter delta is {0, 2} with additional candidate values {1, 3, 4, and 5}. Considering that parameter delta provides information which may be used by a UE to improve the performance (by alignment of slots for channel prediction and the corresponding precoding matrix usage), but it doesn’t have direct impact on the UE complexity (depending on the implementation for channel prediction) and PMI overhead, we don’t see the need to restrict the range of supported values. In our view, at least delta = {0, 1, 2} shall be supported, further extension of the value range can be considered (e.g., to 5 slots).

Proposal 2:

- The range of supported values for parameter delta is {0, 1, 2} slots, extension of the value range can be considered (e.g., to 5 slots)

One of the important parameters for the PMI codebook with DD compression is the size of DD unit (d). Selection of proper d value allows to achieve optimal overhead/performance trade-off by merging multiple slots with similar channels. The concept of DD unit is similar to the concept of subband, where multiple PRBs are merged together and has the same precoding matrix. Optimal DD unit size is correlated with the channel coherence time which is determined by the Doppler spread of the channel. To properly sample the channel in time, CSI-RS periodicity shall be also related to the channel coherence time. Hence, it can be concluded that the optimal DD unit size is correlated with the optimal CSI-RS periodicity.

At the last RAN1 meeting it was agreed that size of DD unit is equal to m (separation between two consecutive CSI-RS resources for aperiodic CSI-RS). There is no agreement on DD unit size for periodic and semi-persistent CSI-RS, the starting point of the discussion is DD unit size equal to the CSI-RS periodicity.

To check if performance gains can be achieved for DD unit size lower than CSI-RS periodicity. SLS evaluations were done for different CSI-RS periodicity (P_RS = {4, 8} slots) with the CSI reporting periodicity equal to 4 * P_RS, where the size of DD unit is equal to P_RS or P_RS/2. Impact of PMI quantization is not considered for these simulations; precoding based on eigenvectors of the predicted channel per DD unit is used for precoding and CQI/RI determination. The detailed evaluation assumptions are presented in the Appendix of the tdoc.

Figure 1: Average UE throughput and Cell-edge UE throughput gain for different CSI-RS periodicity (P_RS) and DD unit size (d)

As it can be observed from the above simulation results, there is a significant performance gain for decreased CSI-RS periodicity and decreased CSI reporting periodicity. However, the performance gains for decreased DD unit size (d) are not significant (3% – 5% in average UE throughput and -1% - 3% in cell-edge UE throughput). Thus, we propose to support DD unit size equal to CSI-RS periodicity for periodic and semi-persistent CSI-RS and distance between adjacent CSI-RS resources for aperiodic CSI (m) for aperiodic CSI-RS.

Observation 1:

- The performance gains observed for decreased DD unit size d = P_RS/2 comparing to DD unit size equal to the CSI-RS periodicity d = P_RS are not significant

Proposal 3:

- The value of DD unit size is equal to CSI-RS periodicity for periodic and semi-persistent CSI-RS and distance between adjacent CSI-RS resources for aperiodic CSI (m) for aperiodic CSI-RS

### Coefficient selection and reporting

One of the important overhead reduction mechanisms supported since Rel-16 eType II PMI codebook is omission of not significant coefficients for the report. Selection of the coefficients is done via bitmap with size 2LM bits for the Rel-16 PMI codebook. Indication of coefficient selection for the new PMI codebook can be done in different ways which are described in the following proposal discussed at RAN1#111 meeting [2].

Proposal 2.E.2: For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select only one from the following alternatives: - Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv - Alt2. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design and further compressed using source-coding (e.g Huffman code - Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector. - Alt3B: A single 2-dimensional bitmap of size 2LQ to report the selected S pairs of SD components and DD basis vector and a single 2-dimensional bitmap of size MS for indicating the location of the NZCs, where each row corresponds to a selected FD basis vector and each column corresponds to one of the selected S pairs of SD component and DD basis vector. - Alt3C: A single 2-dimensional bitmap of size 2LM to report the selected S pairs of SD component and FD basis vector and a single 2-dimensional bitmap of size QS for indicating the location of the NZCs, where each row corresponds to a selected DD basis vector and each column corresponds to one of the selected S pairs of SD component and FD basis vector. - Alt4. A bitmap that includes bits associated with the set of {(f1, s1, t1)} with d(f1, s1, t1)<=d, where d is the threshold that can be configured by gNB, df1, s1,t1= minf1-f0, Mv-f1-f0+ mins1-s0, L - s1-s0+ mint1-t0, Q - t1-t0. s0, f0 and t0 denotes a reference SD basis index and a reference FD basis index and a reference DD basis index associated with SCI, respectively.

Alt1 from the above proposal corresponds to the legacy coefficient selection design extension. In this case Q bitmaps with size 2LM bits are reported from the UE. Other alternatives are used to decrease the coefficient selection overhead. For Alt3A it is assumed that K0 coefficients can be localized in S FD-DD vector pairs (i.e., multiple coefficients corresponding to different SD vectors share the same delay and Doppler values). Thus, the number of bits for Alt3A is MQ + 2LS.

SLS evaluations were done for Alt1 and Alt3A coefficient selection. Special case of Alt3 is used with S = M.  All legacy parameter combinations with L = 4 are considered, number of DD vectors is Q = 2, number of DD units is N4 = 4, size of DD unit is d = 4 slots, CSI periodicity is 16 slots, CSI delay is 4 slots. The detailed evaluation assumptions are captured in the Appendix.

Figure 2: Average UE throughput and Cell-edge UE throughput gain for different PMI overhead for Alt1 and Alt3A coefficient selection

Based on the above evaluation results, small performance degradation (up to 0.8% in average UE throughput and up to 2% for cell-edge UE throughput) is observed for Alt3A comparing to Alt1. At the same time, 48 bits can be saved for configurations with M = 4 and 84 bits can be saved for configuration with M = 7.

Observation 2:

- Performance degradation of up to 0.8% in average UE throughput and up to 2% for cell-edge UE throughput is observed for Alt3A comparing to Alt1.

- 48 bits can be saved for configurations with M = 4 and 84 bits for configuration with M = 7 for Alt3A comparing to Alt1

There is no agreement on whether the number of non-zero coefficients is limited across all DD basis vectors or per DD basis vector. In our view there is no need to limit the number of non-zero coefficients per DD basis vector. This constraint has no impact on the overhead while without this constraint UE may consider implementation for the selection of the non-zero coefficients with improved performance by allocating the coefficients across the DD basis vectors according to the predicted channel in optimal way. Constrained allocation of DD basis vectors can be considered for UE implementation, if needed.

Proposal 4:

- Maximum number of non-zero coefficients in W2 is limited across the W2 matrix. Additional constraint on the maximum number of non-zero coefficients per DD vector is not needed

At the last RAN1 meeting it was agreed to fully reuse the legacy quantization for the new PMI codebook with one DD unit (N4 = 1). This includes SD and FD compression design as well as coefficient quantization. For cases with multiple DD units (N4 > 1), the change of coefficient quantization design should be justified by performance gains or overhead reduction. Based on our study so far, we don’t see the need to change the coefficient quantization scheme for the new PMI codebook with DD compression.

Proposal 5:

- Support legacy coefficient quantization design (same as for Rel-16 eType II PMI) for new PMI codebook with DD compression (with N4 > 1)

### CQI design

Since for the new PMI codebook it is assumed that precoding matrixes related to several time instances are calculated using predicted channel matrixes, legacy CQI calculation and reporting design can not be reused directly. At the last RAN1 meeting the following agreement was made on CQI reporting.

Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, decide by RAN1#112 whether including X>1 CQIs in one sub-band and one CSI reporting instance are supported - If supported, also decide the value(s) of X and the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots)

The number of reported CQIs corresponding to one or multiple (up to N4) precoding matrixes is discussed in the above agreement. To check whether larger number of CQIs provides significant performance gains, SLS evaluations were done for X = 1 CQI (calculated for the first DD unit) and X = N4 = 4 CQIs (calculated per each DD unit). All legacy parameter combinations with L = 4 are considered, number of DD vectors is Q = 2, number of DD units is N4 = 4, size of DD unit is d = 4 slots, CSI periodicity is 16 slots, CSI delay is 4 slots. The overhead on the X-axis for the below figures corresponds to PMI reporting overhead. The detailed evaluation assumptions are captured in the Appendix.

Figure 3: Average UE throughput and Cell-edge UE throughput gain for different PMI overhead for X = 1 CQI and X = 4 CQIs

As it can be observed from the above evaluation results, the performance gain for X = 4 CQIs comparing to X = 1 CQI is not significant (up to 1% for average UE throughput and up to 5% for cell-edge UE throughput). Considering the SLS performance results, we propose to support X = 1 CQI for the new PMI codebook with DD compression.

Observation 3:

- Up to 1% for average UE throughput and up to 5% for cell-edge UE throughput performance gains can be observed for X = 4 CQIs comparing to X = 1 CQI

Proposal 6:

- Support X = 1 CQI

Also, at the last RAN1 meeting the following agreement was made on the assumption for CQI calculation.

Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, down-select (by RAN1#112) one from the following alternatives: - Alt1. The CQI is associated with the entire duration of the CSI reporting window and all the N4 W2 matrices - Alt2A. The CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Alt2B. The CQI is associated with the first/earliest d slots of the CSI reporting window and the first/earliest one of the N4 W2 matrices Note: The N4 W2 matrices represent the combining coefficients before DD compression at the UE, or after DD de-compression at the gNB

Assumption on CQI calculation may be important to align PMI and the corresponding CQI. Considering that there is no significant performance gain observed for the X = 4 CQIs calculated per each time unit comparing to X = 1 CQI calculated for the first DD unit, significant performance gains of Alt1 comparing to Alt2A/2B are not expected. Also, the difference between Alt2A and Alt2B significantly depends on the exact implementation of channel prediction. Thus, it may be not possible to check the UE implementation for CQI assumption and distinguish it between Alt2A and Alt2B via RAN4 tests. So, we propose to support Alt2A or Alt2B for the CQI assumption.

Proposal 7:

- The CQI is associated with the first slot (or d slots) of the CSI reporting window and the first/earliest of the N4 W2 matrices

### Time Domain Channel Properties (TDCP) reporting

At the last RAN1 meeting the following agreement was made on Time Domain Channel Properties reporting.

Agreement For the Rel-18 TRS-based TDCP reporting, down select only one of the following alternatives by RAN1#112: - AltA.1 (Doppler spread) as described in R1-2210523 - AltA.2 (Doppler shift): A UE is configured to report the Doppler shifts corresponding to the M strongest peaks of the wideband Doppler spectrum, for each of the N≥1 configured TRS resources - A TDCP report can be configured with N periodic TRS resources (e.g., N=2 with one TRS resource per TRP) - Parameter M is RRC configured with candidate values TBD, e.g. M=1,2,3,… - Wideband Doppler spectrum is calculated from the wideband time correlation function, given, as an example, by At,τ=ct,τct,0 , where ct,τ=n=0N-1hnt+τ∙hn*t and hn is the channel for subcarrier n. - AltB (TD correlation profile) as described in R1-2210523 Down-selection is to done based on, at least, the (single-)user throughput (LLS) performance comparison among the alternatives assuming: - Three special cases of an agreed use case (companies can select only one or more): aiding gNB to determine switching between Type-I and Rel-16 eType-II codebooks, or to determine SRS periodicity in the UL-SRS reciprocity-based precoding scheme; or aiding the gNB implementation in CSI prediction for TDD - In their simulations on switching between Type-I and Rel-16 eType-II codebooks, companies should state how to calculate the metric for the determination and how to set the threshold, and what the UE reports. - In their simulations on UL-SRS reciprocity-based precoding scheme, companies should state how to set the SRS periodicity based on the reported metrics, and what the UE reports; and the results should be displayed in terms of user throughput vs SRS overhead - In their simulations on CSI prediction for TDD, the results should be the correlation between real channel and predicted channel, and what the UE reports; aided by the reported metric. - Other scenarios of the agreed use cases can optionally be simulated - Based on the agreed EVM for sTRP and mTRP Note: Different alternatives may or may not apply to different use cases FFS: The need for a measure of confidence level in the TDCP report, and/or UE behaviour when the quality of TDCP measurement is not sufficiently high FFS: TDCP parameter(s) signalled with respect to each alternative

The TDCP report based on Doppler profile and time-domain correlation profile are considered based on the above agreement. The main use cases of TDCP reporting can be supported by any type of TDCP reporting including Doppler profile-based TDCP and correlation profile-based TDCP. In our view TDCP reporting based on Time-domain correlation profile corresponds to simpler UE implementation. Thus, we prefer to support TDCP reporting based on Time-domain correlation profile.

Proposal 8:

- Support TDCP reporting based on Time-domain correlation profile

### Partial CSI omission

Partial CSI omission feature is supported for PMI codebooks from Rel-15. Since the actual PMI payload size is different depending on RI and number of non-zero coefficients, gNB may allocate lower number of resources for the CSI transmission comparing to the actual PMI size. Thus, a mechanism is specified which allows to omit a part of CSI with small performance loss. For the partial CSI omission, the CSI component correspond to different priorities. CSI components with lower priority are omitted first. Thus, it is important to define priorities for PMI component of the newly introduced PMI codebook.

For the PMI codebook with Doppler-domain compression, the legacy priorities for partial CSI omission can be reused from Enhanced Type II PMI codebook except priority of coefficients with different DD vectors. We propose to consider the following alternatives for priority of coefficients corresponding to different DD vectors.

- Coefficients corresponding to a DD vector with a lower index have lower priority irrespective of the corresponding index of SD vector and FD vector

- Coefficients corresponding to a DD vector with a lower index may have different priority depending on indexes of the corresponding SD and FD vectors

Proposal 9:

- Consider the following alternatives for priority of coefficients corresponding to different DD vectors

  - Alt1: Coefficients corresponding to a DD vector with a lower index have lower priority irrespective of the corresponding index of SD vector and FD vector

  - Alt2: Coefficients corresponding to a DD vector with a lower index may have different priority depending on indexes of the corresponding SD and FD vectors

### Other

In some cases, PMI report with DD compression may require longer period of PMI update comparing to the legacy PMI codebooks since legacy PMI codebooks are sensitive to channel variation due to UE speed (Doppler). However, CQI and RI depend not only on the channel but also on the interference observed at the UE receiver. Thus, considering dynamic interference for non-full buffer traffic it may happen that CQI and RI require more frequent reporting comparing to PMI with DD compression. Thus, we propose to study separate reporting of PMI and CQI/RI with different periodicities.

Proposal 10:

- Study separate reporting of PMI and CQI/RI with different periodicities

## CSI enhancements for CJT

At the previous RAN1 meetings significant progress was achieved on new PMI codebook design for Coherent Joint Transmission (CJT). In this section, remaining issues on the PMI codebook are discussed including remaining details on Spatial Domain (SD) basis selection, Frequency Domain (FD) basis selection, Codebook Subset Restriction (CBSR) and partial CSI omission.

### Details on Spatial Domain basis selection

Optimal number of SD vectors for precoding matrix compression may be different for different TRPs. Since the total number of SD vectors is limited, the SD vectors allocation problem is important to consider for increased efficiency of the PMI reporting. At the last RAN1 meeting the following agreement was made on SD basis selection for CJT PMI codebook.

Agreement On the Type-II codebook refinement for CJT mTRP, regarding the SD basis selection, for a configured value of NTRP, a set of NL combinations of values for {L1, ..., LNTRP} is gNB-configured via higher-layer (RRC) signaling - When NL>1, the selected combination of values for {L1, ..., LNTRP} is reported in CSI part 1 using an indicator, selected from the NL configured combinations - NL =1 is one of the supported candidate values - FFS: Other supported value(s) of NL, and its respective UE capability - FFS: The supported combinations of values for {L1, ..., LNTRP} - Following the legacy design, the SD basis selection for the n-th (n=1,...,N) selected CSI-RS resource is indicated in CSI part 2 using a combinatorial indicator selected from a set of PCSI-RS/2Ln codepoints where, for Rel-16-based refinement PCSI-RS = 2*N1N2. - The supported candidate values for each of the Ln parameters include the legacy candidate values, i.e. {2,4,6} for Rel-16-based refinement, and - for Rel-17-based refinement, the gNB configures a set of N_L combinations for {alpha1, ..., alphaNTRP} Ln=αnPCSI-RS/2 where αn=12,34, 1 FFS: Whether the set of NL combinations of values for {L1, ..., LNTRP} can be implicitly derived Following the legacy design, for all the selected N CSI-RS resources, the SD basis oversampling group for each CSI-RS resource is indicated in CSI part 2 using an indicator selected from a set of O1O2 codepoints.

There are multiple points which require further discussion for the SD basis selection such as values of NL and supported combinations of {L1, L2, …, LNTRP}. However, since the total number of SD vectors Lmax defines the PMI complexity and overhead mainly, we propose to prioritize the discussion on the Lmax parameter. For the Enhanced Type II PMI codebook the maximum value for L parameter is equal to 6 with separate UE capability. For the CJT PMI codebook, at least Lmax = 6 should be supported. Lmax values larger than 6 can be considered assuming that the number of CPUs (CSI processing unit) is expected to be larger than one since multiple CSI-RS resources are supported for the CJT CSI.

Proposal 11:

- The range of Lmax parameter should be defined as the complexity of PMI search and the maximum overhead is mainly determined by Lmax parameter

  - Consider Lmax > 6 (up to UE capability)

### Details on Frequency Domain basis selection

For the Frequency Domain (FD) basis selection for the CJT PMI codebook 2 modes are supported, where mode-1 corresponds to independent selection of FD vectors across TRPs while mode-2 corresponds to common selection of FD vectors across TRPs. At the last RAN1 meeting it was agreed to clarify the independent selection of FD basis for mode-2 CJT PMI codebook.

Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, study and down select (no later than RAN1#112) only one from the following schemes: - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) - Alt3. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf,n where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf,n is independently selected across N CSI-RS resources For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FS basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

Alt1 from the above agreement corresponds to the rotation of FD basis (which is equivalent to shift of FD vectors) commonly selected across all the TRPs, where the rotation factor (φn) is reported per TRP. In our view the Alt1 is very close to the mode-2 with the difference of up to 15 bits for 4 TRPs and 26 subbands.

Alt2 from the above agreement corresponds to independent selection of FD basis per TRP.

Alt3 from the above agreement is combination of Alt1 and Alt2 with independent selection of FD basis vectors per TRP and additional rotation factor reporting per TRP. Integer rotation factor (φn) which corresponds to cyclic shift of FD vectors in this case doesn’t make any difference in performance. However, if rotation factor value is fractional then FD vectors across TRP are becoming non-orthogonal and are selected from a set of oversampled DFT vectors.

In our view Alt2 should be supported for mode-1 FD basis selection for CJT PMI codebook as the simplest alternative which corresponds to independent selection of FD vectors. As was mentioned above, Alt1 is very close to the mode-2 with the difference of up to 15 bits. There is no significant benefit to support both mode-1 with Alt1 and mode-2. Thus, if Alt1 is supported for mode-1, RAN1 should consider merging of mode-1 and mode-2.

Proposal 12:

- Support independent FD basis selection (Alt2) for mode-1

  - If Alt1 is supported for mode-1, consider merging of mode-1 and mode-2

### Codebook Subset Restriction

Codebook subset restriction (CBSR) is supported for Regular Type II and Type I PMI codebooks which allow to avoid undesired beamforming directions for the PMI. Also, CBSR is supported for the NCJT CSI report with separate configuration of the CBSR per TRP. For CJT CSI, RAN1 can consider similar approach as for NCJT and support CBSR configuration per TRP (per CSI-RS resource).

Proposal 13:

- Support Codebook Subset Restriction (CBSR) configuration per CSI-RS resource (per TRP)

### Partial CSI omission

Partial CSI omission feature is supported for PMI codebooks from Rel-15. Since the actual PMI payload size is different depending on RI and number of non-zero coefficients, gNB may allocate lower number of resources for the CSI transmission comparing to the actual PMI size. Thus, a mechanism which allow to omit a part of CSI with small performance loss is specified. For the partial CSI omission, the CSI component corresponds to different priorities. CSI components with lower priority are omitted first. Thus, it is important to define priorities for PMI component of the newly introduced PMI codebook.

For the CJT PMI codebook, the legacy priorities for partial CSI omission can be reused from Enhanced Type II PMI codebook except priority of coefficients corresponding to different TRPs. We propose to consider the following alternatives for priority of coefficients corresponding to different TRPs.

- Coefficients corresponding to a TRP with a lower index have lower priority irrespective of the corresponding index of SD vector and FD vector

- Coefficients corresponding to a TRP with a lower index may have different priority depending on indexes of the corresponding SD and FD vectors

Proposal 14:

- Consider the following alternatives for priority of coefficients corresponding to different DD vectors

  - Alt1: Coefficients corresponding to a TRP with a lower index have lower priority irrespective of the corresponding index of SD vector and FD vector

  - Alt2: Coefficients corresponding to a TRP with a lower index may have different priority depending on indexes of the corresponding SD and FD vectors

### Other

Parameter R (number of PMI subbands per CQI subband) has significant impact for UE complexity due to the larger number of SVD operations for larger R value for Rel-16 Type II PMI codebook. For Rel-16 Type II PMI feedback R = 1 is supported as basic feature and R = 2 is supported as optional feature. For CJT PMI codebook due to large delay for signal propagation corresponding to different TRPs for inter-TRP scenario, larger R value may be beneficial. Thus, we propose to support at least R =1 and R = 2 for the new PMI codebook.

Proposal 15:

- Support R = 1 and R = 2 for the number of PMI subbands per CQI subband

Since Type II PMI codebook for CJT contains precoder for multiple TRPs, it may be beneficial to report CQIs for single-TRP transmission assuming part of precoding matrix reported by the UE for CJT corresponding to the TRP. I.e. in that case PMI reported for CJT can be applied for single-TRP transmission.

Proposal 16:

- Reporting of single-TRP CQI can be considered together with PMI for CJT

# Conclusion

In this contribution aspects related to the CSI enhancements for high/medium UE velocities and CSI enhancements for CJT were discussed. The following observations and proposals were made.

Proposal 1:

- The range of supported values for N4 is {1, 2, 4, 8} DD units

  - Support of larger values should be justified considering performance loss due to non-ideal channel prediction

Proposal 2:

- Gap between CSI report and CSI reporting window (delta): The range of supported values for delta is {0, 1, 2}, extension of the value range can be considered (e.g., to 5 slots)

Observation 1:

- The performance gains observed for decreased DD unit size d = P_RS/2 comparing to DD unit size equal to the CSI-RS periodicity d = P_RS are not significant

Proposal 3:

- The value of DD unit size is equal to CSI-RS periodicity for periodic and semi-persistent CSI-RS and distance between adjacent CSI-RS resources for aperiodic CSI (m) for aperiodic CSI-RS

Observation 2:

- Performance degradation of up to 0.8% in average UE throughput and up to 2% for cell-edge UE throughput is observed for Alt3A comparing to Alt1.

- 48 bits can be saved for configurations with M = 4 and 84 bits for configuration with M = 7

Proposal 4:

- Maximum number of non-zero coefficients in W2 is limited across the W2 matrix. Additional constraint on the maximum number of non-zero coefficients per DD vector is not needed

Proposal 5:

- Support legacy coefficient quantization design (same as for Rel-16 eType II PMI) for new PMI codebook with DD compression (with N4 > 1)

Observation 3:

- Up to 1% for average UE throughput and up to 5% for cell-edge UE throughput performance gains can be observed for X = 4 CQIs comparing to X = 1 CQI

Proposal 6:

- Support X = 1 CQI

Proposal 7:

- The CQI is associated with the first slot (or d slots) of the CSI reporting window and the first/earliest of the N4 W2 matrices

Proposal 8:

- Support TDCP reporting based on Time-domain correlation profile

Proposal 9:

- Consider the following alternatives for priority of coefficients corresponding to different DD vectors

  - Alt1: Coefficients corresponding to a DD vector with a lower index have lower priority irrespective of the corresponding index of SD vector and FD vector

  - Alt2: Coefficients corresponding to a DD vector with a lower index may have different priority depending on indexes of the corresponding SD and FD vectors

Proposal 10:

- Study separate reporting of PMI and CQI/RI with different periodicities

Proposal 11:

- The range of Lmax parameter should be defined as the complexity of PMI search and the maximum overhead is mainly determined from the Lmax parameter

  - Consider Lmax > 6 (up to UE capability)

Proposal 12:

- Support independent FD basis selection for mode-1 (Alt2)

  - If Alt1 is supported for mode-1, consider merging of mode-1 and mode-2

Proposal 13:

- Support Codebook Subset Restriction (CBSR) configuration per CSI-RS resource (per TRP)

Proposal 14:

- Consider the following alternatives for priority of coefficients corresponding to different DD vectors

  - Alt1: Coefficients corresponding to a TRP with a lower index have lower priority irrespective of the corresponding index of SD vector and FD vector

  - Alt2: Coefficients corresponding to a TRP with a lower index may have different priority depending on indexes of the corresponding SD and FD vectors

Proposal 15:

- Support R = 1 and R = 2 for the number of PMI subbands per CQI subband

Proposal 16:

- Reporting of single-TRP CQI can be considered together with PMI for CJT

# References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, 3GPP TSG RAN #94e, Electronic Meeting, Dec. 6 - 17, 2021.

- R1-2212819, Moderator Summary#3 on Rel-18 CSI enhancements: Round 2, Moderator (Samsung)

# Appendix

Table 1. Evaluation assumptions for CSI enhancements for high/medium UE velocities

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban (Macro only) |
| Layout | Hexagonal grid with 2 tiers (19 sites) |
| ISD | 500 m |
| Carrier frequency | 2 GHz |
| Simulation bandwidth | 20 MHz with 30 kHz subcarrier spacing, 52 PRB |
| Tx power | 41 dBm |
| UE distribution | Uniform, 100% Outdoor with 60 kmph speed |
| UE antenna configuration | 2 Rx X-pol slant 0/90 degrees |
| BS antenna configuration | 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8) λ |
| Traffic model | FTP 1 with 0.5 Mbytes packet size, high traffic load (~65% resource utilization) |
| TRP association | RSRP based, Handover margin = 0 dB |
| Transmission mode | MU-MIMO with rank adaptation |
| Scheduling | Proportional Fair |
| OLLA | 10% BLER target |
| MU-MIMO precoding | MMSE, 8 BS layers max |
| Elevation beamforming | One vertical beam per TXRU electrically down-tilted to 100 degrees |
| UE receiver | MMSE-IRC |
| UE noise figure | 9 dB |
| HARQ | 4 HARQ transmissions max |